3GPP TSG RAN WG1 Meeting #92	bis                                        R1-1805280

Sanya, China, April 16 –20, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0429 | rev | 1 | Current version: | 14.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DMRS scrambling ID for semi-open loop transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eFDMIMO-Core |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 6.10.3.1 of TS36.211, nCSID for DMRS geneartion is given by Table 5.3.3.1.5C-1 and Table 5.3.3.1.5C-2 in TS36.212. However, when higher layer parameter semiolEnabled is configured, Table 5.3.3.1.5C-1 and Table 5.3.3.1.5C-2 is not applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When higher layer parameter semiolEnabled is configured, nCSID for DMRS generation is given by Table 5.3.3.1.5C-6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | nCSID for DMRS generation is missing when higher layer parameter semiolEnabled is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 6.10.3.1	Sequence generation

For antenna port 5, the UE-specific reference-signal sequence  is defined by

where  denotes the assigned bandwidth in resource blocks of the corresponding PDSCH transmission. The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with  at the start of each subframe where  is as described in clause 7.1 3GPP TS 36.213 [4].

For any of the antenna ports , the reference-signal sequence  is defined by

.

The pseudo-random sequence  is defined in clause 7.2. The pseudo-random sequence generator shall be initialised with

at the start of each subframe.

For BL/CE UEs, the same scrambling sequence is applied per subframe to the UE-specific reference-signal sequence for a given block of  subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies . For the block of  subframes, the scrambling sequence generator shall be initialised with

where

and  is the absolute subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmissions span  consecutive subframes, including non-BL/CE DL subframes where the PDSCH transmission is postponed. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured with CEModeB,  for frame structure type 1 and  for frame structure type 2.

The quantities , , are given by

-	 if no value for  is provided by higher layers or if DCI format 1A, 2B or 2C is used for the DCI associated with the PDSCH transmission

-	 otherwise

The value of  is zero unless specified otherwise. For a PDSCH transmission on ports 7 or 8,  is given by the DCI format 2B, 2C, 2D or 6-1A in 3GPP TS 36.212 [3] associated with the PDSCH transmission. 
In the case of DCI format 2B,  is indicated by the scrambling identity field according to Table 6.10.3.1-1. In the case of DCI format 2C or 2D,  is given by Table 5.3.3.1.5C-1, Table 5.3.3.1.5C-2 or Table 5.3.3.1.5C-6 in 3GPP TS 36.212 [3]. For a PDSCH transmission on ports 11 or 13,  is given by the DCI format 2C or 2D in 3GPP TS 36.212 [3] associated with the PDSCH transmission where  is given by Table 5.3.3.1.5C-2 in 3GPP TS 36.212 [3].

Table 6.10.3.1-1: Mapping of scrambling identity field in DCI format 2B to values for antenna ports 7 and 8

| Scrambling identity field in DCI format 2B (3GPP TS 36.212 [3]) |  |
| --- | --- |
| 0 | 0 |
| 1 | 1 |