3GPP TSG-RAN WG2 Meeting #102	R2-1808294

# Busan, Korea, 21st May – 25th May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0014 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on EN-DC bearer type changes in TS 37.340 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-06 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The bearer type change from split to split bearer with key change (i.e. PDCP termination point change) is allowed without MAC reset in TS 36.331 and 38.331. But the informative table in the annex of TS 37.340 indicates MAC reset is required for the case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that MAC reset is not necessary for split to split bearer change with key change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

Annex A (informative):
Layer 2 handling for bearer type change

This subclause provides for information an overview on L2 handling for bearer type change in EN-DC, with and without security key change (from KeNB to S-KgNB and from S-KgNB to KeNB), i.e. with and without a change of the termination point.

Table A-1: L2 handling for bearer type change with and without security key change

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: Re-establish SCG MAC: See Note | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Re-establish SCG MAC: See Note |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-establish SCG MAC: See Note | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: Re-establish SCG MAC: See note |

NOTE:	MAC behaviour depends on the solution selected by the network, e.g. MAC reset, change of LCID, etc.

CHANGE END