3GPP TSG RAN WG4 Meeting #87	R4-1808532

# Busan, KR, 21-25 May 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0013 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-3: Implementation of endorsed draft CRs from RAN4 #87 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2017-05-31 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Endorsed draft CRs from RAN4#87 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Endorsed draft CRs from RAN4#87 R4-1806418 Draft CR for TS 38.101-3: to correct reqirements for some EN-DC combinations, Samsung R4-1806667 Draft CR for introducing of LTE(xDL/1UL)+NR(2DL/1UL) DC band combinations, LG Electronics R4-1806817 Clarification of EN-DC UE requirements with NR in FR2. OPPO R4-1806834 Draft CR to 38.101-3: Corrections on MSD items for EN-DC_1A-3A_n78A in section 7.3B.2.3.5.2, China Telecommunications R4-1806987 TS 38.101-3 clause 5 corrections, Nokia, Nokia Shanghai Bell R4-1807054 Draft CR for introduction of completed EN-DC with LTE 5CC + NR 1band in TS 38.101-3, Samsung R4-1807104 Draft CR 38.101-3 for completed DC combinations LTE 3CC + NR 1 CC, Ericsson. R4-1807171 Draft CR for RF requirements of 3UL EN-DC for TS 38.101-3, NTT DOCOMO, INC. R4-1807235 Draft CR into TS 38.101-3 Some Corrections for SUL, Huawei, HiSilicon R4-1807241 Draft CR for completed DC of LTE 2CC + NR 1band for TS 38.101-3, Huawei, HiSilicon R4-1807824 Draft CR to TS 38.101-3: Introduction of B41/n41 intra-band contiguous EN-DC, Sprint Corporation R4-1807825 Draft CR to TS 38.101-3: Introduction of B41/n41 non-contiguous intra-band EN-DC, Sprint Corporation, Nokia, Ericsson R4-1807827 Draft CR to TS 38.101-3: Channel spacing for intra-band EN-DC, Sprint Corporation, Nokia, Nokia Shanghai Bell R4-1807847 Draft CR for TS 38.101-3: OOBE for inter-band EN-DC, Sprint Corporation R4-1807924 General requirements for intra-band contiguous EN-DC, Qualcomm Incorporated R4-1808092 Draft CR to TS 38.101-3: Inclusion of Simultaneous RxTx UE capability for some band combinations, Ericsson, Vodafone, Orange R4-1808108 Draft CR to TS38.101-3 on completion of DC_3A_n3A with single tx switched UL mode in Rel.15, CHTTL R4-1808115 Draft CR to TS38.101-3_introduction of completed band combinations for FR1 plus FR2 inter-band 2UL CA, ZTE Corporation R4-1808119 Completion of transmitter and receiver requirements for DC_(n)71B, Ericsson R4-1808145 Draft CR for 38.101-3 DC_(n)41AA A-MPR, Sprint Corporation R4-1808146 Draft CR for 38.101-3 DC_41A_n41A A-MPR, Sprint Corporation R4-1808220 Introduction of 4LTECC+1NR to 38.101-3, Nokia, Nokia Shanghai Bell R4-1808261 Draft CR for TS38.101-3 on addition of new 90MHz UE CBW for n78, LG Electronics Inc., LG Uplus, Samsung R4-1808536 Draft CR for completed DC of LTE 1CC + NR 1band for TS 38.101-3, NTT DOCOMO, NC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not updated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 38.101-3 V15.1.0 (2018-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

User Equipment (UE) radio transmission and reception;

Part 3: Range 1 and Range 2 Interworking operation with other radios

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2018, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Symbols	7

3.3	Abbreviations	8

4	General	8

4.1	Relationship between minimum requirements and test requirements	8

4.2	Applicability of minimum requirements	8

4.3	Specification suffix information	8

5	Operating bands and channel arrangement	9

5.1	General	9

5.2	Operating bands	9

5.2A	Operating bands for CA	9

5.2A.1	Inter-band CA between FR1 and FR2	9

5.2B	Operating bands for DC	10

5.2B.1	General	10

5.2B.2	Intra-band contiguous EN-DC	10

5.2B.2.1	EN-DC (two bands)	10

5.2B.3	Intra-band non-contiguous EN-DC	10

5.2B.3.1	EN-DC (two bands)	10

5.2B.3.2	EN-DC (three bands)	10

5.2B.4	Inter-band EN-DC within FR1	10

5.2B.4.1	EN-DC (two bands)	11

5.2B.4.2	EN-DC (three bands)	12

5.2B.4.3	EN-DC (four bands)	13

5.2B.4.4	EN-DC (five bands)	14

5.2B.5	Inter-band EN-DC including FR2	14

5.2B.5.1	EN-DC (two bands)	14

5.2B.5.2	EN-DC (three bands)	15

5.2B.5.3	EN-DC (four bands)	16

5.2B.5.4	EN-DC (five bands)	16

5.3	UE Channel bandwidth	16

5.3A	UE Channel bandwidth for CA	16

5.3A.1	Inter-band CA between FR1 and FR2	16

5.4	Channel arrangement	16

5.5	Configuration	17

5.5A	Configuration for CA	17

5.5B	Configuration for DC	17

5.5B.1	General	17

5.5B.2	Intra-band contiguous EN-DC	17

5.5B.3	Intra-band non-contiguous EN-DC	17

5.5B.4	Inter-band EN-DC within FR1	18

5.5B.4.1	Inter-band EN-DC configurations (two bands)	18

5.5B.4.2	Inter-band EN-DC configurations (three bands)	19

5.5B.4.3	Inter-band EN-DC configurations (four bands)	21

5.5B.4.4	Inter-band EN-DC configurations (five bands)	23

5.5B.4.5	Inter-band EN-DC configurations (six bands)	24

5.5B.5	Inter-band EN-DC including FR2	24

5.5B.5.1	Inter-band EN-DC configurations (two bands)	24

5.5B.5.2	Inter-band EN-DC configurations (three bands)	25

5.5B.5.3	Inter-band EN-DC configurations (four bands)	27

5.5B.5.4	Inter-band EN-DC configurations (five bands)	28

5.5B.5.5	Inter-band EN-DC configurations (six bands)	28

6	Transmitter characteristics	28

6.1	General	28

6.2	Transmitter power	29

6.2A	Transmitter power for CA	29

6.2A.1	UE maximum output power for CA	29

6.2A.1.1	Inter-band CA between FR1 and FR2	29

6.2A.2	UE maximum output power reduction for CA	29

6.2A.2.1	Inter-band CA between FR1 and FR2	29

6.2A.3	UE additional maximum output power reduction for CA	29

6.2A.4	Configured output power for CA	29

6.2A.4.1	Configured output power level	29

6.2A.4.2	ΔTIB,c for CA	29

6.2A.4.2.1	Inter-band CA between FR1 and FR2	29

6.2B	Transmitter power for DC	30

6.2B.1	UE maximum output power for EN-DC	30

6.2B.1.1	Intra-band contiguous EN-DC	30

6.2B.1.2	Intra-band non-contiguous EN-DC	30

6.2B.1.3	Inter-band EN-DC within FR1	30

6.2B.1.4	Inter-band EN-DC including FR2	31

6.2B.2	UE maximum output power reduction and EN-DC	32

6.2B.2.1	Intra-band contiguous EN-DC	32

6.2B.2.2	Intra-band non-contiguous EN-DC	32

6.2B.2.3	Inter-band EN-DC within FR1	32

6.2B.2.4	Inter-band EN-DC including FR2	32

6.2B.3	UE additional maximum output power reduction for EN-DC	32

6.2B.3.1	Intra-band contiguous EN-DC	32

6.2B.3.2	Intra-band non-contiguous EN-DC	32

6.2B.3.3	Inter-band EN-DC within FR1	32

6.2B.3.4	Inter-band EN-DC including FR2	32

6.2B.4	Configured output power for EN-DC	32

6.2B.4.1	Configured output power level	32

6.2B.4.1.1	Intra-band contiguous EN-DC	32

6.2B.4.1.2	Intra-band non-contiguous EN-DC	32

6.2B.4.1.3	Inter-band EN-DC within FR1	33

6.2B.4.1.4	Inter-band EN-DC including FR2	33

6.2B.4.2	ΔTIB,c for EN-DC	33

6.2B.4.2.1	Intra-band contiguous EN-DC	34

6.2B.4.2.2	Intra-band non-contiguous EN-DC	34

6.2B.4.2.3	Inter-band EN-DC within FR1	34

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands	34

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands	36

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands	39

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands	41

6.2B.4.2.4	Inter-band EN-DC including FR2	43

6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands	43

6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands	44

6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands	45

6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands	46

6.3	Output power dynamics	46

6.4	Transmit signal quality	46

6.5	Output RF spectrum emissions	46

6.5A	Output RF spectrum emissions for CA	46

6.5A.1	Occupied bandwidth for CA	47

6.5A.2	Out-of-band emissions for CA	47

6.5A.3	Spurious emissions for CA	47

6.5B	Output RF spectrum emissions for DC	47

6.5B.1	Occupied bandwidth for EN-DC	47

6.5B.2	Out-of-band emissions for EN-DC	47

6.5B.2.1	Intra-band contiguous EN-DC	47

6.5B.2.1.1	Spectrum emissions mask	47

6.5B.2.1.2	Additional spectrum emissions mask	47

6.5B.2.1.2.1	DC_(n)71B	47

6.5B.2.1.3	Adjacent channel leakage ratio	47

6.5B.2.2	Intra-band non-contiguous EN-DC	48

6.5B.2.3	Inter-band EN-DC within FR1	48

6.5B.2.4	Inter-band EN-DC including FR2	48

6.5B.3	Spurious emissions for EN-DC	48

6.5B.3.1	Intra-band contiguous EN-DC	48

6.5B.3.2	Intra-band non-contiguous EN-DC	48

6.5B.3.3	Inter-band EN-DC within FR1	48

6.5B.3.3.1	Spurious emission band UE co-existence	48

6.5B.3.4	Inter-band EN-DC including FR2	53

6.5B.3.4.1	Spurious emission band UE co-existence	53

7	Receiver characteristics	56

7.1	General	56

7.2	Diversity characteristics	56

7.3	Reference sensitivity	56

7.3A	Reference sensitivity for  CA	56

7.3A.1	General	56

7.3A.2	Reference sensitivity power level for CA	56

7.3A.3	ΔRIB,c for CA	56

7.3A.3.1	Inter-band CA between FR1 and FR2	57

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA	57

7.3B	Reference sensitivity level for DC	57

7.3B.1	General	57

7.3B.2	Reference sensitivity for EN-DC	57

7.3B.2.1	Intra-band contiguous EN-DC	57

7.3B.2.2	Intra-band non-contiguous EN-DC	58

7.3B.2.3	Inter-band EN-DC within FR1	58

7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1	58

7.3B.2.3.2	MSD due to receiver harmonic mixing for EN-DC in NR FR1	60

7.3B.2.3.3	Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1	61

7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1	61

7.3B.2.3.5	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1	62

7.3B.2.3.5.1	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands	63

7.3B.2.3.5.2	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands	64

7.3B.2.4	Inter-band EN-DC including FR2	67

7.3B.2.4.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR2	67

7.3B.3	ΔRIB,c for EN-DC	67

7.3B.3.1	Intra-band contiguous EN-DC	68

7.3B.3.2	Intra-band non-contiguous EN-DC	68

7.3B.3.3	Inter-band EN-DC within FR1	68

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands	68

7.3B.3.3.2	ΔRIB,c for EN-DC three bands	69

7.3B.3.3.3	ΔRIB,c for EN-DC four bands	72

7.3B.3.3.4	ΔRIB,c for EN-DC five bands	74

7.3B.3.4	Inter-band EN-DC including FR2	75

7.3B.3.4.1	ΔRIB,c for EN-DC in two bands	75

7.3B.3.4.2	ΔRIB,c for EN-DC three bands	76

7.3B.3.4.3	ΔRIB,c for EN-DC four bands	76

7.3B.3.4.4	ΔRIB,c for EN-DC five bands	77

Annex A (normative):	Dual uplink interferer	77

Annex B (informative):	Change history	78

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document establishes the minimum RF characteristics and minimum performance requirements for NR User Equipment (UE) Interworking operation with other radios. This includes but is not limited to additional requirements for carrier aggregation between Range 1 and Range 2 and additional requirements due to NR non-standalone (NSA) operation mode with E-UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[5]	3GPP TS 38.521-3: “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

SUL	Supplementary uplink

ULSUP                Uplink sharing from UE perspective

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The present document is interwork specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-3 [5].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-3 [5] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [6].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level subclause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) with and without SUL including UL sharing from UE perspective |
| C |  |
| D | UL MIMO |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specifications are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 450 MHz – 6000 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers band combinations including

-	at least one FR1 operating band and one FR2 operating band for carrier aggregation and dual connectivity operations;

-	at least one E-UTRA operating band for dual connectivity operations.

## 5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [2] and FR2 operating bands defined in TS 38.101-2 [3]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [4].

Editor’s note: The lists of specific NR operating bands and band combinations is maintained in TR 38.817-01 and will be merged into TS 38.101-3 in a future version.

## 5.2A	Operating bands for CA

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation are designed to operate in the operating bands defined in Table 5.2A.1-1. The band combinations include at least one FR1 operating band and one FR2 operating band.

Table 5.2A.1-1: Band combinations for NR CA

| NR CA Band | NR Band |
| --- | --- |
| CA_n8-n258 | n8, n258 |
| CA_n71-n2571 | n71, n257 |
| CA_n77-n2571 | n77, n257 |
| CA_n78-n2571 | n78, n257 |
| CA_n79-n2571 | n79, n257 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

## 5.2B	Operating bands for DC

### 5.2B.1	General

The operating bands are specified for operation with EN-DC or NGEN-DC configured. The band combinations include at least one E-UTRA operating band.

For EN-DC configurations indicated by column “Single Uplink allowed” (e.g., problematic band combinations as defined in TS38.306) in tables in this section the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own downlink band if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are below 1 GHz or between 1695 MHz – 2690 MHz. In case for the EN-DC configurations listed in tables in this section for which the intermodulation products caused by the dual and triple uplink operation fall into the recive band but do not interfere with the own downlink transmission as defined in Annex-A the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulations product could create difficulty for meeting emission requirements.

### 5.2B.2	Intra-band contiguous EN-DC

<Editor’s note: conducted requirements>

#### 5.2B.2.1	EN-DC (two bands)

Table 5.2B.2.1-1: Band combinations for EN-DC (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_(n)71 | 71 | n71 | No3 |
| DC_(n)41 | 41 | n41 | Yes1 |
| Note 1: Single UL allowed due to potential emission issues, not self-interference. Note 2: The minimum requirements apply for 15 kHz subcarrier spacing on the SCG. Note 3: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

### 5.2B.3	Intra-band non-contiguous EN-DC

<Editor’s note: conducted requirements>

#### 5.2B.3.1	EN-DC (two bands)

Table 5.2B.3.1-1: Band combinations EN-DC (two bands)

| EN-DC Band Uplink Combination | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_3_n3 | 3 | n3 | Yes1 |
| DC_41_n41 | 41 | n41 | Yes |
| Note 1: Only single switched UL is supported in Rel.15 |  |  |  |

5.2B.3.2

### 5.2B.4	Inter-band EN-DC within FR1

<Editor’s note: conducted requirements>

#### 5.2B.4.1	EN-DC (two bands)

Table 5.2B.4.1-1: Band combinations for EN-DC (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n28 | 1 | n28 | No |
| DC_1_n40 | 1 | n40 | No |
| DC_1_n51 | 1 | n51 | No |
| DC_1_n773 | 1 | n77 | DC_1_n77 |
| DC_1_n783 | 1 | n78 | No |
| DC_1_n793 | 1 | n79 | No |
| DC_2_n5 | 2 | n5 | No |
| DC_2_n66 | 2 | n66 | No |
| DC_2_n71 | 2 | n71 | No |
| DC_2_n78 | 2 | n78 | No |
| DC_3_n7 | 3 | n7 | No |
| DC_3_n28 | 3 | n28 | No |
| DC_3_n40 | 3 | n40 | No |
| DC_3_n51 | 3 | n51 | No |
| DC_3_n773 | 3 | n77 | DC_3_n77 |
| DC_3_n783 | 3 | n78 | DC_3_n78 |
| DC_3_n793 | 3 | n79 | No |
| DC_5_n40 | 5 | n40 | No |
| DC_5_n66 | 5 | n66 | No |
| DC_5_n783 | 5 | n78 | No |
| DC_7_n28 | 7 | n28 | No |
| DC_7_n51 | 7 | n51 | No |
| DC_7_n783 | 7 | n78 | No |
| DC_8_n40 | 8 | n40 | No |
| DC_8_n773 | 8 | n77 | No |
| DC_8_n783 | 8 | n78 | No |
| DC_8_n793 | 8 | n79 | No |
| DC_11_n773 | 11 | n77 | No |
| DC_11_n783 | 11 | n78 | No |
| DC_11_n793 | 11 | n79 | No |
| DC_12_n5 | 12 | n5 | No |
| DC_12_n66 | 12 | n66 | No |
| DC_18_n773 | 18 | n77 | No |
| DC_18_n783 | 18 | n78 | No |
| DC_18_n793 | 18 | n79 | No |
| DC_19_n773 | 19 | n77 | No |
| DC_19_n783 | 19 | n78 | No |
| DC_19_n793 | 19 | n79 | No |
| DC_20_n8 | 20 | n8 | No |
| DC_20_n284 | 20 | n28 | No |
| DC_20_n51 | 20 | n51 | No |
| DC_20_n77 | 20 | n77 | No |
| DC_20_n783 | 20 | n78 | No |
| DC_21_n773 | 21 | n77 | No |
| DC_21_n783 | 21 | n78 | No |
| DC_21_n793 | 21 | n79 | No |
| DC_25_n41 | 25 | n41 | No |
| DC_26_n413 | 26 | n41 | No |
| DC_26_n773 | 26 | n77 | No |
| DC_26_n783 | 26 | n78 | No |
| DC_26_n793 | 26 | n79 | No |
| DC_28_n51 | 28 | n51 | No |
| DC_28_n773 | 28 | n77 | No |
| DC_28_n783 | 28 | n78 | No |
| DC_28_n793 | 28 | n79 | No |
| DC_30_n5 | 30 | n5 | No |
| DC_30_n66 | 30 | n66 | No |
| DC_38_n78 | 38 | n78 | No |
| DC_39_n781,3 | 39 | n78 | No |
| DC_39_n793 | 39 | n79 | No |
| DC_40_n77 | 40 | n77 | No |
| DC_41_n77 | 41 | n77 | No |
| DC_41_n78 | 41 | n78 | No |
| DC_41_n792 | 41 | n79 | No |
| DC_42_n51 | 42 | n51 | No |
| DC_42_n77 | 42 | n77 | No |
| DC_42_n78 | 42 | n78 | No |
| DC_42_n79 | 42 | n79 | No |
| DC_66_n71 | 66 | n71 | No |
| DC_66_n5 | 66 | n5 | No |
| DC_66_n78 | 66 | n78 | No |
| NOTE 1: The frequency range above 3600MHz for Band n78 is not used in this combination. NOTE 2: The frequency range below 2545MHz for Band 41 is not used in this combination. NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 4: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.2B.4.2	EN-DC (three bands)

Table 5.2B.4.2-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n28 | CA_1-3 | n28 | No |
| DC_1-3_n772 | CA_1-3 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3_n782 | CA_1-3 | n78 | DC_3_n78 |
| DC_1-3_n792 | CA_1-3 | n79 | No |
| DC_1-5_n782 | CA_1-5 | n78 | No |
| DC_1-5_n782 | CA_1-5 | n78 | No |
| DC_1-7_n282 | CA_1-7 | n28 | No |
| DC_1-7_n78 2 | CA_1-7 | n78 | No |
| DC_1-7-7_n782 | CA_1-7-7 | n78 | No |
| DC_1-8_n782 | CA_1-8 | n78 | No |
| DC_1-18_n772 | CA_1-18 | n77 | No |
| DC_1-18_n782 | CA_1-18 | n78 | No |
| DC_1-18_n79 | CA_1-18 | n79 | No |
| DC_1-19_n772 | CA_1-19 | n77 | DC_1_n77 |
| DC_1-19_n782 | CA_1-19 | n78 | No |
| DC_1-19_n792 | CA_1-19 | n79 | No |
| DC_1-20_n283 | CA_1-20 | n28 | No |
| DC_1-20_n782 | CA_1-20 | n78 | No |
| DC_1-21_n772 | CA_1-21 | n77 | DC_1_n77 |
| DC_1-21_n782 | CA_1-21 | n78 | No |
| DC_1-21_n792 | CA_1-21 | n79 | No |
| DC_1-28_n772 | CA_1-28 | n77 | No |
| DC_1-28_n782 | CA_1-28 | n78 | No |
| DC_1-28_n79 | CA_1-28 | n79 | No |
| DC_1_n28-n782 | 1 | CA_n28-n78 | No |
| DC_1_n77-n79 | 1 | CA_n77-n79 | No |
| DC_1_n78-n79 | 1 | CA_n78-n79 | No |
| DC_1-41_n77 | CA_1-41 | n77 | No |
| DC_1-41_n79 | CA_1-41 | n79 | No |
| DC_1-42_n77 | CA_1-42 | n77 | DC_1_n77 |
| DC_1-42_n78 | CA_1-42 | n78 | No |
| DC_1-42_n79 | CA_1-42 | n79 | No |
| DC_1_SUL_n78-n842 | 1 | SUL_n78-n84 | No |
| DC_2-5_n66 | CA_2-5 | n66 | No |
| DC_2-12_n66 | CA_2-12 | n66 | No |
| DC_2-30_n66 | CA_2-30 | n66 | No |
| DC_2-(n)71 | CA_2-71 | n71 | No |
| DC_2-66_n71 | CA_2-66 | n71 | No |
| DC_3_n3-n77 | 3 | CA_n3-n77 | DC_3_n3 |
| DC_3_n3-n78 | 3 | CA_n3-n78 | DC_3_n3 |
| DC_1-28_n77 | CA_1-28 | n77 | No |
| DC_3-5_n782 | CA_3-5 | n78 | DC_3_n78 |
| DC_3-7_n28 | CA_3-7 | n28 | No |
| DC_3-7_n782 | CA_3-7 | n78 | DC_3_n78 |
| DC_3-19_n772 | CA_3-19 | n77 | No |
| DC_3-19_n782 | CA_3-19 | n78 | No |
| DC_3-19_n792 | CA_3-19 | n79 | No |
| DC_3-20_n282,3 | CA_3-20 | n28 | No |
| DC_3-20_n782 | CA_3-20 | n78 | No |
| DC_3-21_n772 | CA_3-21 | n77 | No |
| DC_3-21_n782 | CA_3-21 | n78 | No |
| DC_3-21_n792 | CA_3-21 | n79 | No |
| DC_3-28_n782 | CA_3-28 | n78 | No |
| DC_3_n28-n782 | 3 | CA_n28-n78 | DC_3_n78 |
| DC_3-38_n78 | CA_3-38 | n78 | DC_3_n78 |
| DC_3-42_n77 | CA_3-42 | n77 | DC_3_n77 |
| DC_3-42_n78 | CA_3-42 | n78 | DC_3_n78 |
| DC_3-42_n79 | CA_3-42 | n79 | No |
| DC_3_n77-n79 | 3 | CA_n77-n79 | No |
| DC_3_n78-n79 | 3 | CA_n78-n79 | No |
| DC_3_SUL_n78-n802 | 3 | SUL_n78-n80 | DC_3_n78 |
| DC_3_SUL_n78-n822 | 3 | SUL_n78-n821 | DC_3_n78 |
| DC_3_SUL_n79-n802 | 3 | SUL_n79-n80 | No |
| DC_5-7-7_n78 | CA_5-7-7 | n78 | No |
| DC_5-7_n78 | CA_5-7 | n78 | No |
| DC_5-30_n66 | CA_5-30 | n66 | No |
| DC_7-7_n78 | CA_7-7 | n78 | No |
| DC_7-20_n283 | CA_7-20 | n28 | No |
| DC_7-20_n782 | CA_7-20 | n78 | No |
| DC_7-28_n782 | CA_7-28 | n78 | No |
| DC_7_n28-n782 | 7 | CA_n28-n78 | No |
| DC_7_n782 | CA_7 | n78 | No |
| DC_7-46_n78 | CA_7-46 | n78 | No |
| DC_8_SUL_n78-n812 | 8 | SUL_n78-n81 | No |
| DC_8_SUL_n79-n812 | 8 | SUL_n79-n81 | No |
| DC_12-30_n66 | CA_12-30 | n66 | No |
| DC_18-28_n772 | CA_18-28 | n77 | No |
| DC_18-28_n782 | CA_18-28 | n78 | No |
| DC_18-28_n792 | CA_18-28 | n79 | No |
| DC_19-21_n772 | CA_19-21 | n77 | No |
| DC_19-21_n782 | CA_19-21 | n78 | No |
| DC_19-21_n792 | CA_19-21 | n79 | No |
| DC_19-42_n77 | CA_19-42 | n77 | No |
| DC_19-42_n78 | CA_19-42 | n78 | No |
| DC_19-42_n79 | CA_19-42 | n79 | No |
| DC_19_n77-n79 | 19 | CA_n77-n79 | No |
| DC_19_n78-n79 | 19 | CA_n78-n79 | No |
| DC_20_n8-n75 | 20 | CA_n8-n75 | DC_20_n8 |
| DC_20_n28-n753 | 20 | CA_n28-n75 | No |
| DC_20_n28-n782,3 | 20 | CA_n28-n78 | No |
| DC_20_n75-n782 | 20 | CA_n75-n78 | No |
| DC_20_n76-n782 | 20 | CA_n76-n78 | No |
| DC_20_SUL_n78-n822 | 20 | SUL_n78-n82 | No |
| DC_20_SUL_n78-n832 | 20 | SUL_n78-n831 | No |
| DC_21-42_n77 | CA_21-42 | n77 | No |
| DC_21-42_n78 | CA_21-42 | n78 | No |
| DC_21-42_n79 | CA_21-42 | n79 | No |
| DC_21_n77-n79 | 21 | CA_n77-n79 | No |
| DC_21_n78-n79 | 21 | CA_n78-n79 | No |
| DC_28-42_n77 | CA_28-42 | n77 | No |
| DC_28-42_n78 | CA_28-42 | n78 | No |
| DC_28-42_n79 | CA_28-42 | n79 | No |
| DC_41-42_n77 | CA_41-42 | n77 | No |
| DC_41-42_n78 | CA_41-42 | n78 | No |
| DC_41-42_n79 | CA_41-42 | n79 | No |
| DC_41_n77 | CA_41 | n77 | No |
| DC_41_n78 | CA_41 | n78 | No |
| DC_41_n79 | CA_41 | n79 | No |
| DC_42_n77 | CA_42 | n77 | No |
| DC_28_SUL_n78-n832 | 20 | SUL_n78-n83 | No |
| DC_42_n77 | CA_42 | n77 | No |
| DC_42_n78 | CA_42 | n78 | No |
| DC_42_n79 | CA_42 | n79 | No |
| DC_66_(n)71 | CA_66-71 | n71 | No |
| DC_66_SUL_n78-n862 | 20 | SUL_n78-n86 | DC_66_n78 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.2B.4.3	EN-DC (four bands)

Table 5.2B.4.3-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5_ n781 | CA_1-3-5 | n78 | DC_3_n78 |
| DC_1-3-7_n28 | CA_1-3-7 | n28 | No |
| DC_1-3-7-7_n781 | CA_1-3-7-7 | n78 | DC_3_n78 |
| DC_1-3-7_n781 | CA_1-3-7 | n78 | DC_3_n78 |
| DC_1-3-8_n781 | CA_1-3-8 | n78 | No |
| DC_1-3-28_n771 | CA_1-3-28 | n77 | No |
| DC_1-3-28_n781 | CA_1-3-28 | n78 | No |
| DC_1-3_n28-n781 | CA_1-3 | CA_n28-n78 | DC_3_n78 |
| DC_1-3-28_n791 | CA_1-3-28 | n79 | No |
| DC_1-3-19_n771 | CA_1-3-19 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-19_n781 | CA_1-3-19 | n78 | DC_3_n78 |
| DC_1-3-19_n791 | CA_1-3-19 | n79 | No |
| DC_1-3-20_n282 | CA_1-3-20 | n28 | No |
| DC_1-3-20_n781 | CA_1-3-20 | n78 | DC_3_n78 |
| DC_1-3-21_n771 | CA_1-3-21 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-21_n781 | CA_1-3-21 | n78 | DC_3_n78 |
| DC_1-3-21_n791 | CA_1-3-21 | n79 | No |
| DC_1-3-42_n77 | CA_1-3-42 | n77 | DC_1_n77 |
| DC_1-3-42_n78 | CA_1-3-42 | n78 | No |
| DC_1-3-42_n79 | CA_1-3-42 | n79 | No |
| DC_1-5-7_n78 | CA_1-5-7 | n78 | No |
| DC_1-5-7-7_n78 | CA_1-5-7-7 | n78 | No |
| DC_1-7-20_n282 | CA_1-7-20 | n28 | No |
| DC_1-7-20_n781 | CA_1-7-20 | n78 | No |
| DC_1-7_n28-n781 | CA_1-7 | CA_n28-n78 | No |
| DC_1-18-28_n77 | CA_1-18-28 | n77 | No |
| DC_1-18-28_n78 | CA_1-18-28 | n78 | No |
| DC_1-18-28_n791 | CA_1-18-28 | n79 | No |
| DC_1-19-42_n77 | CA_1-19-42 | n77 | DC_1_n77 |
| DC_1-19-42_n78 | CA_1-19-42 | n78 | No |
| DC_1-19-42_n79 | CA_1-19-42 | n79 | No |
| DC_1-20_n28-n781 | CA_1-20 | CA_n28-n78 | No |
| DC_1-21-28_n771 | CA_1-21-28 | n77 | No |
| DC_1-21-28_n781 | CA_1-21-28 | n78 | No |
| DC_1-21-28_n791 | CA_1-21-28 | n79 | No |
| DC_1-21-42_n77 | CA_1-21-42 | n77 | DC_1_n77 |
| DC_1-21-42_n78 | CA_1-21-42 | n78 | No |
| DC_1-21-42_n79 | CA_1-21-42 | n79 | No |
| DC_1-28-42_n77 | CA_1-28-42 | n77 | No |
| DC_1-28-42_n78 | CA_1-28-42 | n78 | No |
| DC_1-28-42_n79 | CA_1-28-42 | n79 | No |
| DC_1-41-42_n77 | CA_1-41-42 | n77 | No |
| DC_1-41-42_n78 | CA_1-41-42 | n78 | No |
| DC_1-41-42-n79 | CA_1-41-42 | n79 | No |
| DC_2-66-(n)71 | CA_2-66-71 | n71 |  |
| DC_3-5-7-7_n78 | CA_3-5-7-7 | n78 | DC_3_n78 |
| DC_3-7_n28-n781 | CA_3-7 | CA_n28-n78 | DC_3_n78 |
| DC_3-20_n28-n781,2 | CA_3-20 | CA_n28-n78 | DC_3_n78 |
| DC_3-21-42_n77 | DC_3-21-42 | n77 | DC_3_n77 |
| DC_3-21-42_n78 | DC_3-21-42 | n78 | DC_3_n78 |
| DC_3-21-42_n79 | DC_3-21-42 | n79 | No |
| DC_19-21-42_n77 | CA_19-21-42 | n77 | No |
| DC_19-21-42_n78 | CA_19-21-42 | n78 | No |
| DC_19-21-42_n79 | CA_19-21-42 | n79 | No |
| DC_3-5-7_n78 | CA_3-5-7 | n78 | DC_3_n78 |
| DC_3-7-20_n282 | CA_3-7-20 | n28 | No |
| DC_3-7-28_n781 | CA_3-7-28 | n78 | No |
| DC_3-7-20_n781 | CA_3-7-20 | n78 | DC_3_n78 |
| DC_3-19-21_n771 | CA_3-19-21 | n77 | DC_3_n77 |
| DC_3-19-21_n781 | CA_3-19-21 | n78 | DC_3_n78 |
| DC_3-19-21_n791 | CA_3-19-21 | n79 | No |
| DC_3-19-42_n77 | CA_3-19-42 | n77 | No |
| DC_3-19-42_n78 | CA_3-19-42 | n78 | No |
| DC_3-19-42_n791 | CA_3-19-42 | n79 | No |
| DC_3-28-42_n77 | CA_3-28-42 | n77 | No |
| DC_3-28-42_n78 | CA_3-28-42 | n78 | No |
| DC_3-28-42_n79 | CA_3-28-42 | n79 | No |
| DC_7-20_n28-n781 | CA_7-20 | CA_n28-n78 | No |
| DC_21-28-42_n77 | CA_21-28-42 | n77 | No |
| DC_21-28-42_n78 | CA_21-28-42 | n78 | No |
| DC_21-28-42_n79 | CA_21-28-42 | n79 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability NOTE 2: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. |  |  |  |

#### 5.2B.4.4	EN-DC (five bands)

Table 5.2B.4.4-1: Band combinations EN-DC (five bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-7_n78 | CA_1-3-5-7 | n78 | DC_3_n78 |
| DC_1-3-5-7-7_n78 | CA_1-3-5-7-7 | n78 | DC_3_n78 |
| DC_1-3-7-20_n282 | CA_1-3-7-20 | n28 | No |
| DC_1-3-7-20_n781 | CA_1-3-7-20 | n78 | DC_3_n78 |
| DC_1-3-7_n28-n781 | CA_1-3-7 | CA_n28-n78 | DC_3_n78 |
| DC_1-3-19-21_n771 | CA_1-3-19-21 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-19-21_n781 | CA_1-3-19-21 | n78 | DC_3_n78 |
| DC_1-3-19-21_n791 | CA_1-3-19-21 | n79 | No |
| DC_1-3-19-42_n77 | CA_1-3-19-42 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-19-42_n78 | CA_1-3-19-42 | n78 | DC_3_n78 |
| DC_1-3-19-42_n79 | CA_1-3-19-42 | n79 | No |
| DC_1-3-20_n28-n781,2 | CA_1-3-20 | CA_n28-n78 | DC_3_n78 |
| DC_1-3-21-42_n77 | CA_1-3-21-42 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-21-42_n78 | CA_1-3-21-42 | n78 | DC_3_n78 |
| DC_1-3-21-42_n79 | CA_1-3-21-42 | n79 | No |
| DC_1-7-20_n28-n781,2 | CA_1-7-20 | CA_n28-n78 | No |
| DC_1-19-21-42_n77 | DC_1-19-21-42 | n77 | DC_1_n77 |
| DC_1-19-21-42_n78 | DC_1-19-21-42 | n78 | No |
| DC_1-19-21-42_n79 | DC_1-19-21-42 | n79 | No |
| DC_3-7-20_n28-n781,2 | CA_3-7-20 | CA_n28-n78 | DC_3_n78 |
| DC_1-3-5-7_n78 | CA_1-3-5-7 | n78 | DC_3_n78 |
| DC_1-3-7-20_n28 | CA_1-3-7-20 | n28 | No |
| DC_1-3-28-42_n77 | CA_1-3-28-42 | n77 | DC_1_n77, DC_3_n77 |
| DC_1-3-28-42_n78 | CA_1-3-28-42 | n78 | DC_3_n78 |
| DC_1-3-28-42_n79 | CA_1-3-28-42 | n79 | No |
| DC_1-21-28-42_n77 | CA_1-21-28-42 | n77 | DC_1_n77 |
| DC_1-21-28-42_n78 | CA_1-21-28-42 | n78 | No |
| DC_1-21-28-42_n79 | CA_1-21-28-42 | n79 | No |
| Note 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability Note 2: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

#### 5.2B.4.5	EN-DC (six bands)

Table 5.2B.4.5-1: Band combinations EN-DC (six bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-7-20_n28-n781,2 | CA_1-3-7-20 | CA_n28-n78 | DC_3_n78 |
| Note 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability Note 2: The frequency range in band 28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL |  |  |  |

### 5.2B.5	Inter-band EN-DC including FR2

<Editor’s note: OTA requirements>

#### 5.2B.5.1	EN-DC (two bands)

Table 5.2B.5.1-1: Band combinations for EN-DC (two bands)

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n257 | 1 | n257 | No |
| DC_2-2_n257 | CA_2-2 | n257 | No |
| DC_2_n257 | CA_2 | n257 | No |
| DC_2_n260 | 2 | n260 | No |
| DC_2_n260 | CA_2 | n260 | No |
| DC_2-2_n260 | CA_2-2 | n260 | No |
| DC_3_n257 | 3 | n257 | No |
| DC_3_n258 | 3 | n258 | No |
| DC_5-5_n257 | CA_5-5 | n257 | No |
| DC_5-5_n260 | CA_5-5 | n260 | No |
| DC_5_n257 | 5 | n257 | No |
| DC_5_n260 | 5 | n260 | No |
| DC_5_n261 | 5 | n261 | No |
| DC_7-7_n257 | CA_7-7 | n257 | No |
| DC_7_n257 | 7 | n257 | No |
| DC_7_n258 | 7 | n258 | No |
| DC_8_n257 | 8 | n257 | No |
| DC_8_n258 | 8 | n258 | No |
| DC_11_n257 | 11 | n257 | No |
| DC_12_n260 | 12 | n260 | No |
| DC_18_n257 | 18 | n257 | No |
| DC_19_n257 | 19 | n257 | No |
| DC_20_n258 | 20 | n258 | No |
| DC_21_n257 | 21 | n257 | No |
| DC_26_n257 | 26 | n257 | No |
| DC_28_n257 | 28 | n257 | No |
| DC_28_n258 | 28 | n258 | No |
| DC_30_n260 | 30 | n260 | No |
| DC_39_n258 | 39 | n258 | No |
| DC_41_n257 | 41 | n257 | No |
| DC_41_n258 | 41 | n258 | No |
| DC_42_n257 | 42 | n257 | No |
| DC_48-48_n257 | CA_48-48 | n257 | No |
| DC_48_n257 | CA_48 | n257 | No |
| DC_48-48_n260 | C_48-48 | n260 | No |
| DC_48_n260 | CA_48 | n260 | No |
| DC_66-66_n257 | CA_66-66 | n257 | No |
| DC_66_n257 | 66 | n257 | No |
| DC_66-66_n260 | C_66-66 | n260 | No |
| DC_66_n260 | 66 | n260 | No |
| DC_66_n261 | 66 | n261 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |  |  |

#### 5.2B.5.2	EN-DC (three bands)

Table 5.2B.5.2-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n2571 | CA_1-3 | n257 | No |
| DC_1-3_n2571 | CA_1-3 | n257 | No |
| DC_1-5_n2571 | CA_1-5 | n257 | No |
| DC_1-7_n2571 | CA_1-7 | n257 | No |
| DC_1-7-7_n2571 | CA_1-7-7 | n257 | No |
| DC_1-8_n257 | CA_1-8 | n257 | No |
| DC_1-18_n2571 | CA_1-18 | n257 | No |
| DC_1-19_n2571 | CA_1-19 | n257 | No |
| DC_1-21_n2571 | CA_1-21 | n257 | No |
| DC_1-41_n257 | CA_1-41 | n257 | No |
| DC_1-42_n257 | CA_1-42 | n257 | No |
| DC_2-5_n2571 | CA_2-5 | n257 | No |
| DC_2-5_n260 | CA_2-5 | n260 | No |
| DC_2-12_n260 | CA_2-12 | n260 | No |
| DC_2-13_n2571 | CA_2-13 | n257 | No |
| DC_2-30_n260 | CA_2-30 | n260 | No |
| DC_2-66_n2571 | CA_2-66 | n257 | No |
| DC_2-66_n260 | CA_2-66 | n260 | No |
| DC_2-13_n2601 | CA_2-13 | n260 | No |
| DC_3-5_n2571 | CA_3-5 | n257 | No |
| DC_3-7_n2571 | CA_3-7 | n257 | No |
| DC_3-19_n2571 | CA_3-19 | n257 | No |
| DC_3-21_n2571 | CA_3-21 | n257 | No |
| DC_3-28_n2571 | CA_3-28 | n257 | No |
| DC_3-41_n257 | CA_3-41 | n257 | No |
| DC_3-42_n2571 | CA_3-42 | n257 | No |
| DC_5-7-7_n2571 | CA_5-7-7 | n257 | No |
| DC_5-7_n2571 | CA_5-7 | n257 | No |
| DC_5-30_n260 | CA_5-30 | n260 | No |
| DC_5-66_n260 | CA_5-66 | n260 | No |
| DC_12-30_n260 | CA_12-30 | n260 | No |
| DC_12-66_n260 | CA_12-66 | n260 | No |
| DC_13-66_n2571 | CA_13-66 | n257 | No |
| DC_13-66_n2601 | CA_13-66 | n260 | No |
| DC_18-28_n2571 | CA_18-28 | n257 | No |
| DC_19-21_n2571 | CA_19-21 | n257 | No |
| DC_19-42_n2571 | CA_19-42 | n257 | No |
| DC_21-42_n2571 | CA_21-42 | n257 | No |
| DC_21-28_n2571 | CA_21-28 | n257 | No |
| DC_28-42_n2571 | CA_28-42 | n257 | No |
| DC_30-66_n260 | CA_30-66 | n260 | No |
| DC_41-42_n257 | CA_41-42 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2B.5.3	EN-DC (four bands)

Table 5.2B.5.3-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5_n2571 | CA_1-3-5 | n257 | No |
| DC_1-3-7_n2571 | CA_1-3-7 | n257 | No |
| DC_1-3-7-7_n257 | CA_1-3-7-7 | n257 | No |
| DC_1-3-19_n2571 | CA_1-3-19 | n257 | No |
| DC_1-3-21_n2571 | CA_1-3-21 | n257 | No |
| DC_1-3-28_n2571 | CA_1-3-28 | n257 | No |
| DC_1-3-42_n257 | CA_1-3-42 | n257 | No |
| DC_1-5-7_n2571 | CA_1-5-7 | n257 | No |
| DC_1-5-7-7_n257 | CA_1-5-7-7 | n257 | No |
| DC_1-18-28_n2571 | CA_1-18-28 | n257 | No |
| DC_1-19-42_n257 | CA_1-19-42 | n257 | No |
| DC_1-21-28_n2571 | CA_1-21-28 | n257 | No |
| DC_1-21-42_n257 | CA_1-21-42 | n257 | No |
| DC_1-28-42_n257 | CA_1-28-42 | n257 | No |
| DC_1-41-42_n257 | CA_1-41-42 | n257 | No |
| DC_3-5-7-7_n257 | CA_3-5-7-7 | n257 | No |
| DC_3-5-7_n2571 | CA_3-5-7 | n257 | No |
| DC_3-19-21_n2571 | CA_3-19-21 | n257 | No |
| DC_3-19-42_n257 | CA_3-19-42 | n257 | No |
| DC_3-21-42_n257 | DC_3-21-42 | n257 | No |
| DC_3-28-42_n257 | CA_3-28-42 | n257 | No |
| DC_19-21-42_n2571 | CA_19-21-42 | n257 | No |
| DC_21-28-42_n2571 | CA_21-28-42 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

#### 5.2B.5.4	EN-DC (five bands)

Table 5.2B.5.4-1: Band combinations EN-DC (five bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-7_n2571 | CA_1-3-5-7 | n257 | No |
| DC_1-3-5-7-7_n2571 | CA_1-3-5-7-7 | n257 | No |
| DC_1-3-19-21_n2571 | CA_1-3-19-21 | n257 | No |
| DC_1-3-19-42_n257 | CA_1-3-19-42 | n257 | No |
| DC_1-3-21-42_n257 | CA_1-3-21-42 | n257 | No |
| DC_1-3-28-42_n257 | CA_1-3-28-42 | n257 | No |
| DC_1-19-21-42_n257 | DC_1-19-21-42 | n257 | No |
| DC_1-21-28-42_n257 | DC_1-21-28-42 | n257 | No |
| DC_3-5-7-7_n257 | CA_3-5-7-7 | n257 | No |
| DC_1-3-28-42_n257 | CA _1-3-28-42 | n257 | No |
| DC_1-21-28-42_n257 | CA _1-21-28-42 | n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |

### 5.2B.6	Inter-band EN-DC including both FR1 and FR2

<Editor’s note: OTA requirements>

#### 5.2B.6.1	EN-DC (two bands)

This section is N/A

#### 5.2B.6.2	EN-DC (three bands)

Table 5.2B.6.2-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_n77-n257 | 1 | CA_n77-n257 | No |
| DC_1_n78-n257 | 1 | CA_n78-n257 | No |
| DC_1_n79-n257 | 1 | CA_n79-n257 | No |
| DC_3_n77-n257 | 3 | CA_n77-n257 | No |
| DC_3_n78-n257 | 3 | CA_n78-n257 | No |
| DC_3_n79-n257 | 3 | CA_n79-n257 | No |
| DC_5_n78-n2571 | 5 | CA_n78-n257 | No |
| DC_7-7_n78-n257 | CA_7-7 | CA_n78-n257 | No |
| DC_7_n78-n257 | 7 | CA_n78-n257 | No |
| DC_19_n77-n257 | 19 | CA_n77-n257 | No |
| DC_19_n78-n257 | 19 | CA_n78-n257 | No |
| DC_19_n79-n257 | 19 | CA_n79-n257 | No |
| DC_21_n77-n257 | 21 | CA_n77-n257 | No |
| DC_21_n78-n257 | 21 | CA_n78-n257 | No |
| DC_21_n79-n257 | 21 | CA_n79-n257 | No |

#### 5.2B.6.3	EN-DC (four bands)

Table 5.2B.6.3-1: Band combinations EN-DC (four bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n78-n257 | CA_1-3 | CA_n78-n257 | No |
| DC_1-5_n78-n257 | CA_1-5 | CA_n78-n257 | No |
| DC_1-7-7_n78-n257 | CA_1-7-7 | CA_n78-n257 | No |
| DC_1-7_n78-n257 | CA_1-7 | CA_n78-n257 | No |
| DC_3-5_n78-n257 | CA_3-5 | CA_n78-n257 | No |
| DC_3-7-7_n78-n257 | CA_3-7-7 | CA_n78-n257 | No |
| DC_3-7_n78-n257 | CA_3-7 | CA_n78-n257 | No |
| DC_5-7-7_n78-n257 | CA_5-7-7 | CA_n78-n257 | No |
| DC_5-7_n78-n257 | CA_5-7 | CA_n78-n257 | No |

#### 5.2B.6.4	EN-DC (five bands)

Table 5.2B.6.4-1: Band combinations EN-DC (five bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5_n78-n257 | CA_1-3-5 | CA_n78-n257 | No |
| DC_1-3-7-7_n78-n257 | CA_1-3-7-7 | CA_n78-n257 | No |
| DC_1-3-7_n78-n257 | CA_1-3-7 | CA_n78-n257 | No |
| DC_1-5-7-7_n78-n257 | CA_1-5-7-7 | CA_n78-n257 | No |
| DC_1-5-7_n78-n257 | CA_1-5-7 | CA_n78-n257 | No |
| DC_3-5-7-7_n78-n257 | CA_3-5-7-7 | CA_n78-n257 | No |
| DC_3-5-7_n78-n257 | CA_3-5-7 | CA_n78-n257 | No |

#### 5.2B.6.5	EN-DC (six bands)

Table 5.2B.6.5-1: Band combinations EN-DC (six bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3-5-7_n78-n257 | CA_1-3-5-7 | CA_n78-n257 | No |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory |  |  |  |

## 5.3	UE Channel bandwidth

## 5.3A	UE Channel bandwidth for CA

### 5.3A.1	Inter-band CA between FR1 and FR2

## 5.3B	UE Channel bandwidth for EN-DC

For intra-band contiguous EN-DC, the aggregated channel bandwidth is sum of the individual NR and E-UTRA channel bandwidths assuming nominal EN-DC channel with 0 kHz offset spacing as specified in sub-clause 5.4.

ENBW = BWNR_Channel + BWE-UTRA Channel

In the case where the NR sub-block and/or the E-UTRA sub-block itself is composed of intra-band contiguous CA carriers, the EN-DC aggregated channel bandwidth is the sum of the aggregated channel bandwidths of the NR and E-UTRA sub-blocks assuming nominal EN-DC channel spacing between the NR sub-block and E-UTRA sub-block.

ENBW = BWNR_Channel_CA + BWE-UTRA Channel_CA

### 5.3B.1	Intra-band EN-DC in FR1

#### 5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

The DL component carrier combinations for a given EN-DC configuration shall be symmetrical in relation to channel centre unless stated otherwise in Table 5.3B.1-1.

#### 5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, a n EN-DC configuration is a single operating band supporting a carrier aggregation bandwidth class.

Requirements for intra-band contiguous carrier aggregation are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.2-1.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |  |  |
|  |  | Channel bandwidths for LTE carrier (MHz) |  | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |  |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 |  | 40, 60, 80,100 |  |  | 120 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20 |  |  |  |  |  |
| DC_(n)41CA | DC_(n)41AA1, DC_41A_n41A2 | 20+20 |  | 40, 60, 80,100 |  |  | 140 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20+20 |  |  |  |  |  |
| DC_(n)41DA | DC_(n)41AA1, DC_41A_n41A2 | 20+20+20 |  | 40, 60, 80,100 |  |  | 160 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |  |  |  |
| DC_(n)71B | DC_(n)71B | 15 |  | 5 |  |  | 20 | 0 |  |  |
|  |  | 10 |  | 5, 10 |  |  |  |  |  |  |
|  |  | 5 |  | 5, 10, 15 |  |  |  |  |  |  |
|  |  |  |  | 5 | 15 |  |  |  |  |  |
|  |  |  |  | 5, 10 | 10 |  |  |  |  |  |
|  |  |  |  | 5, 10, 15 | 5 |  |  |  |  |  |
| Note 1: Contiguous intra-band EN-DC uplink requirements shall apply. Note 2: LTE and NR ACLR requirements and non-contiguous intra-band EN-DC uplink requirements shall apply. |  |  |  |  |  |  |  |  |  |  |

#### 5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is a single operating band supporting an EN-DC bandwidth class.

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

|  |  | E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |  |  |
|  |  | Channel bandwidths for LTE carrier (MHz) |  | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for LTE carrier (MHz) |  |  |  |  |  |
| DC_3A_n3A | DC_3A_n3A(1) |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 |  | 50 | 0 |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 |  | 40, 60, 80,100 |  |  | 120 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20 |  |  |  |  |  |
| DC_41C_n41A | DC_41A_n41A | 20+20 |  | 40, 60, 80,100 |  |  | 140 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20+20 |  |  |  |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 |  | 40, 60, 80,100 |  |  | 160 | 0 |  |  |
|  |  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |  |  |  |
| Note 1: Only single switched UL is supported in Rel.15 |  |  |  |  |  |  |  |  |  |  |

## 5.4	Channel arrangement

## 5.4A	Channel arrangement for CA

The channel arrangement for CA operations in FR1 and FR2 as specified in 38.101-1 and 38.101-2, respectively.

## 5.4B	Channel arrangement for DC

The channel arrangement for intra-band EN-DC operations in FR1 is specified in sub-clause 5.4B.1 of TS 38.101-1.

### 5.4B.1	Channel spacing for intra-band EN-DC carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between and E-UTRA carrier and an adjacent NR carrier for intra-band contiguous EN-DC is defined as following:

Nominal Channel spacing = (BWLTE_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz}

where BWLTE_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

For intra-band non-contiguous EN-DC the channel spacing between E-UTRA and NR carriers shall be larger than the nominal channel spacing defined in this subclause.

## 5.5	Configuration

## 5.5A	Configuration for CA

#### 5.5A.1	Inter-band CA configurations between FR1 and FR2

Table 5.5A.1-1: Inter-band CA configurations (two bands)

| NR CA configuration | Uplink NR CA configuration | NR configuration for FR1 | NR configuration for FR2 |
| --- | --- | --- | --- |
| CA_n8A-n258A | CA_n8A-n258A | n8A | n258A |
| CA_n71A-n257A | - | n71A | n257A |
| CA_n77A-n257A | CA_n77A-n257A | n77A | n257A |
| CA_n77A-n257D |  | n77A | n257D |
| CA_n77A-n257E |  | n77A | n257E |
| CA_n77A-n257F |  | n77A | n257F |
| CA_n77C-n257A |  | n77C | n257A |
| CA_n77C-n257D |  | n77C | n257D |
| CA_n77C-n257E |  | n77C | n257E |
| CA_n77C-n257F |  | n77C | n257F |
| CA_n78A-n257A | CA_n78A-n257A | n78A | n257A |
| CA_n78A-n257D |  | n78A | n257D |
| CA_n78A-n257E |  | n78A | n257E |
| CA_n78A-n257F |  | n78A | n257F |
| CA_n78C-n257A |  | n78C | n257A |
| CA_n78C-n257D |  | n78C | n257D |
| CA_n78C-n257E |  | n78C | n257E |
| CA_n78C-n257F |  | n78C | n257F |
| CA_n79A-n257A | CA_n79A-n257A | n79A | n257A |
| CA_n79A-n257D |  | n79A | n257D |
| CA_n79A-n257E |  | n79A | n257E |
| CA_n79A-n257F |  | n79A | n257F |
| CA_n79C-n257A |  | n78C | n257A |
| CA_n79C-n257D |  | n78C | n257D |
| CA_n79C-n257E |  | n78C | n257E |
| CA_n79C-n257F |  | n78C | n257F |
| NOTE 1: NR configuration for FR1 and FR2 are defined in TS 38.101-1 and TS 38.101-2 respectively. |  |  |  |

## 5.5B	Configuration for DC

### 5.5B.1	General

The channel bandwidth and bandwidth classes are specified for operation with EN-DC or NGEN-DC configured.

### 5.5B.2	Intra-band contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in [4] and for NR operating bands in TS 38.101-1.

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_(n)41AA | DC_(n)41AA | 41A | n41A |
| DC_(n)41CA | DC_(n)41AA, DC_41A_n41A | 41C | n41A |
| DC_(n)41DA | DC_(n)41AA, DC_41A_n41A | 41D | n41A |
| DC_(n)71B | DC_(n)71B | 71A | n71A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.101 and for NR operating bands in TS 38.101-1.

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_n3A | DC_3A_n3A2 | 3 | n3A |
| DC_41A_n41A | DC_41A_n41A | 41A | n41A |
| DC_41C_n41A | DC_41A_n41A | 41C | n41A |
| DC_41D_n41A | DC_41A_n41A | 41D | n41A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel.15 |  |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

#### 5.5B.4.1	Inter-band EN-DC configurations (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n28A | DC_1A_n28A | 1 | n28A |
| DC_1A_n40A | DC_1A_n40A | 1A | n40A |
| DC_1A_n51A | DC_1A_n51A | 1A | n51A |
| DC_1A_n77A DC_1A_n77C | DC_1A_n77A | 1A | n77A CA_n77C |
| DC_1A_n78A DC_1A_n78C | DC_1A_n78A | 1A | n78A CA_n78C |
| DC_1A_n79A DC_1A_n79C | DC_1A_n79A | 1A | n79A CA_n79C |
| DC_2A_n5A | DC_2A_n5A | 2A | n5A |
| DC_2A_n66A | DC_2A_n66A | 2A | n66A |
| DC_2A_n71A | DC_2A_n71A | 2 | n71A |
| DC_2A_n78A | DC_2A_n78A | 2A | n78A |
| DC_3A_n7A | DC_3A_n7A | 3 | n7A |
| DC_3A_n28A | DC_3A_n28A | 3 | n28A |
| DC_3A_n40A | DC_3A_n40A | 3A | n40A |
| DC_3A_n51A | DC_3A_n51A | 3A | n51A |
| DC_3A_n77A DC_3A_n77C | DC_3A_n77A | 3A | n77A CA_n77C |
| DC_3A_n78A DC_3A_n78C | DC_3A_n78A | 3A | n78A CA_n78C |
| DC_3A_n79A DC_3A_n79C | DC_3A_n79A | 3A | n79A CA_n79C |
| DC_3C_n78A | DC_3A_n78A | CA_3C | n78A |
| DC_5A_n40A | DC_5A_n40A | 5A | n40A |
| DC_5A_n66A | DC_5A_n66A | 5A | n66A |
| DC_5A_n78A | DC_5A_n78A | 5 | n78A |
| DC_7A-7A_n78A | DC_7A_n78A | CA _7A-7A | n78A |
| DC_7A_n28A | DC_7A_n28A | 7 | n28A |
| DC_7A_n51A | DC_7A_n51A | 7A | n51A |
| DC_7A_n78A | DC_7A_n78A | 7 | n78A |
| DC_7C_n78A | DC_7C_n78A | CA _7C | n78A |
| DC_8A_n40A | DC_8A_n40A | 8A | n40A |
| DC_8A_n77A | DC_8A_n77A | 8 | n77A |
| DC_8A_n78A | DC_8A_n78A | 8 | n78A |
| DC_11A_n77A | DC_11A_n77A | 11 | n77A |
| DC_11A_n78A | DC_11A_n78A | 11 | n78A |
| DC_11A_n79A | DC_11A_n79A | 11 | n79A |
| DC_12A_n5A | DC_12A_n5A | 12A | n5A |
| DC_12A_n66A | DC_12A_n66A | 12A | n66A |
| DC_18A_n77A | DC_18A_n77A | 18 | n77A |
| DC_18A_n78A | DC_18A_n78A | 18 | n78A |
| DC_18A_n79A | DC_18A_n79A | 18 | n79A |
| DC_19A_n77A DC_19A_n77C | DC_19A_n77A | 19A | n77A CA_n77C |
| DC_19A_n78A DC_19A_n78C | DC_19A_n78A | 19A | n78A CA_n78C |
| DC_19A_n79A DC_19A_n79C | DC_19A_n79A | 19A | n79A CA_n79C |
| DC_20A_n8A | DC_20A_n8A | 20A | n8A |
| DC_20A_n28A | DC_20A_n28A | 20 | n28A |
| DC_20A_n51A | DC_20A_n51A | 20A | n51A |
| DC_20A_n77A | DC_20A_n77A | 20A | n77A |
| DC_20A_n78A | DC_20A_n78A | 20A | n78A |
| DC_21A_n77A DC_21A_n77C | DC_21A_n77A | 21A | n77A CA_n77C |
| DC_21A_n78A DC_21A_n78C | DC_21A_n78A | 21A | n78A CA_n78C |
| DC_21A_n79A DC_21A_n79C | DC_21A_n79A | 21A | n79A CA_n79C |
| DC_25A_n41A | DC_25A_n41A | 25 | n41A |
| DC_26A_n41A | DC_26A_n41A | 26A | n41A |
| DC_26A _n77A | DC_26A _n77A | 26 | n77A |
| DC_26A _n78A | DC_26A _n78A | 26 | n78A |
| DC_26A _n79A | DC_26A _n79A | 26 | n79A |
| DC_28A n51A | DC_28A_n51A | 28A | n51A |
| DC_28A_n77A DC_28A_n77C | DC_28A_n77A | 28A | n77A CA_n77C |
| DC_28A_n78A DC_28A_n78C | DC_28A_n78A | 28A | n78A CA_n78C |
| DC_28A_n79A DC_28A_n79C | DC_28A_n79A | 28A | n79A CA_n79C |
| DC_30A_n5A | DC_30A_n5A | 30 | n5A |
| DC_30A_n66A | DC_30A_n66A | 30A | n66A |
| DC_38A_n78A | N/A | 38 | n78A |
| DC_39A_n78A | DC_39A_n78A | 39 | n78A |
| DC_39A_n79A | DC_39A_n79A | 39 | n79A |
| DC_40A_n77A | N/A | 40A | n77A |
| DC_41A_n77A | DC_41A_n77A | 41A | n77A |
| DC_41A_n78A | DC_41A_n78A | 41A | n78A |
| DC_41A_n79A | DC_41A_n79A | 41 | n79A |
| DC_41C_n77A | DC_41C_n77A | CA _41C | n77A |
| DC_41C_n78A | DC_41C_n78A | CA _41C | n78A |
| DC_41C_n79A | DC_41C_n79A | CA _41C | n79A |
| DC_42A_n51A | DC_42A_n51A | 42A | n51A |
| DC_42A_n77A DC_42A_n77C | N/A | 42A | n77A CA_n77C |
| DC_42A_n78A DC_42A_n78C | N/A | 42A | n78A CA_n78C |
| DC_42A_n79A DC_42A_n79C | N/A | 42A | n79A CA_n79C |
| DC_42C_n77A | N/A | CA _42C | n77A |
| DC_42C_n78A | N/A | CA _42C | n78A |
| DC_42C_n79A | N/A | CA _42C | n79A |
| DC_42C_n79A | DC_42C_n79A | CA _42C | n79A |
| DC_42C_n77C | N/A | CA _42C | CA n77C |
| DC_42C_n78C | N/A | CA _42C | CA n78C |
| DC_42C_n79C | N/A | CA _42C | CA n79C |
| DC_42D_n77A | N/A | 42 | n77A |
| DC_42D_n78A | N/A | 42 | n78A |
| DC_42D_n79A | DC_42A_n79A | 42 | n79A |
| DC_42E_n77A | N/A | 42 | n77A |
| DC_42E_n78A | N/A | 42 | n78A |
| DC_42E_n79A | N/A | 42 | n79A |
| DC_46D_n78A2 |  |  |  |
| DC_46E_n78A2 |  |  |  |
| DC_66A _n5A | DC_66A _n5A | 66A | n5A |
| DC_66A _n71A | DC_66A _n71A | 66 | n71A |
| DC_66A _n78A | DC_66A _n78A | 66A | n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A | CA_1A-3A | n28A |
| DC_1A-3A_n77A DC_1A-3A_n77C | DC_1A_n77A DC_3A_n77A | CA_1A-3A | n77A |
| DC_1A-3A_n78A DC_1A-3A_n78C | DC_1A_n78A DC_3A_n78A | CA _1A-3A | n78A |
| DC_1A-3A_n79A DC_1A-3A_n79C | DC_1A_n79A DC_3A_n79A | CA _1A-3A | n79A |
| DC_1A-3C_n78A | DC_1A_n78A DC_3A_n78A | CA _1A-3C | n78A |
| DC_1A-3A_n77A | DC_1A_n77A DC_3A_n77A | CA_1A-3A | n77A |
| DC_1A-3A_n78A | DC_1A_n78A DC_3A_n78A | CA _1A-3A | n78A |
| DC_1A-3A_n79A | DC_1A_n79A DC_3A_n79A | CA _1A-3A | n79A |
| DC_1A-5A_n78A | DC_1A_n78A DC_5A_n78A | CA _1A-5A | n78A |
| DC_1A-7A_n28A | DC_1A_n28A DC_7A_n28A | CA _1A-7A | n28A |
| DC_1A-7A_n78A | DC_1A_n78A DC_7A_n78A | CA _1A-7A | n78A |
| DC_1A-7A-7A_n78A | DC_1A_n78A DC_7A_n78A | CA _1A-7A-7A | n78A |
| DC_1A-8A_n78A | DC_1A_n78A DC_8A_n78A | CA _1A-8A | n78A |
| DC_1A-18A_n77A | DC_1A_n77A DC_18A_n77A | CA _1A-18A | n77A |
| DC_1A-18A_n78A | DC_1A_n78A DC_18A_n78A | CA _1A-18A | n78A |
| DC_1A-19A_n77A DC_1A-19A_n77C | DC_1A_n77A DC 19A_n77A | CA _1A-19A | n77A CA_n77C |
| DC_1A-19A_n78A DC_1A-19A_n78C | DC_1A_n78A DC_19A_n78A | CA _1A-19A | n78A CA_n78C |
| DC_1A-19A_n79A DC_1A-19A_n79C | DC_1A_n79A DC_19A_n79A | CA _1A-19A | n79A CA_n79C |
| DC_1A-19A_n77A | DC_1A_n77A DC 19A_n77A | CA _1A-19A | n77A |
| DC_1A-19A_n78A | DC_1A_n78A DC_19A_n78A | CA _1A-19A | n78A |
| DC_1A-19A_n79A | DC_1A_n79A DC_19A_n79A | CA _1A-19A | n79A |
| DC_1A-20A_n28A | DC_1A_n28A DC_20A_n28A | CA _1A-20A | N28A |
| DC_1A-20A_n78A | DC_1A_n78A DC_20A_n78A | CA _1A-20A | n78A |
| DC_1A-21A_n77A DC_1A-21A_n77C | DC_1A_n77A DC_21A_n77A | CA _1A-21A | n77A CA_n77C |
| DC_1A-21A_n78A DC_1A-21A_n78C | DC_1A_n78A DC_21A_n78A | CA _1A-21A | n78A CA_n78C |
| DC_1A-21A_n79A DC_1A-21A_n79C | DC_1A_n79A DC_21A_n79A | CA _1A-21A | n79A CA_n79C |
| DC_1A-21A_n77A | DC_1A_n77A DC_21A_n77A | CA _1A-21A | n77A |
| DC_1A-21A_n78A | DC_1A_n78A DC_21A_n78A | CA _1A-21A | n78A |
| DC_1A-21A_n79A | DC_1A_n79A DC_21A_n79A | CA _1A-21A | n79A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A DC_41C_n77A | CA_1A-41A CA_1A-41C | n77 |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A DC_41C_n78A | CA_1A-41A CA_1A-41C | n78 |
| DC_1A-41C_n79A | DC_1A_n79A DC_41C_n79A | CA_1A-41C | n79 |
| DC_1A-28A_n77A DC_1A-28A_n77C | DC_1A_n77A DC_28A_n77A | CA _1A-28A | n77A CA_n77C |
| DC_1A-28A_n78A DC_1A-28A_n78C | DC_1A_n78A DC_28A_n78A | CA _1A-28A | n78A CA_n78C |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A | CA _1A-28A | n79A CA_n79C |
| DC_1A_n28A-n78A | DC_1A_n28A, DC_1A_n78A | 1A | CA_n28A-n78A |
| DC_1A-42A_n77A DC_1A-42A_n77C | DC_1A_n77A | CA _1A-42A | n77A CA_n77C |
| DC_1A-42A_n78A DC_1A-42A_n78C | DC_1An78A | CA _1A-42A | n78A CA_n78C |
| DC_1A-42A_n79A DC_1A-42A_n79C | DC_1A_n79A | CA _1A-42A | n79A CA_n79C |
| DC_1A-42C_n77A | DC_1A_n77A | CA_1A-42C | n77A |
| DC_1A-42C_n78A | DC_1A_n78A | CA_1A-42C | n78A |
| DC_1A-42C_n79A | DC_1A_n79A | CA_1A-42C | n79A |
| DC_1A-42D_n77A | DC_1A_n77A | CA_1A-42C | n77A |
| DC_1A-42D_n78A | DC_1A_n78A | CA_1A-42C | n78A |
| DC_1A-42D_n79A | DC_1A_n79A | CA_1A-42C | n79A |
| DC_1A-42E_n77A | DC_1A_n77A | CA_1A-42E | n77A |
| DC_1A-42E_n78A | DC_1A_n78A | CA_1A-42E | n78A |
| DC_1A-42E_n79A | DC_1A_n79A | CA_1A-42E | n79A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A | 1A | CA_n77A-n79A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A | 1A | CA_n78A-n79A |
| DC_1A_SUL_n78A-n84A | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A, DC_1A_n84A_ULSUP-FDM_n78A | 1 | SUL_n78A-n84A |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A | CA_2A-5A | n66 |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A | CA_2A-12A | n66 |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-30A | n66 |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A | CA_2A-66A | n71 |
| DC_2A-(n)71B | DC_2A_n71A DC_(n)71B | CA _2A-71A | n71A |
| DC_3A_n3A-n77A | DC_3A_n77A | 3A | CA_n3A-n77A |
| DC_3A_n3A-n78A | DC_3A_n78A | 3A | CA_n3A-n78A |
| DC_3A-5A_n78A | DC_3A_n78A DC_5A_n78A | CA _3A-5A | n78A |
| DC_3A-7A-7A_n78A | DC_3A_n78A DC_7A_n78A | CA_3A-7A-7A | n78A |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A | CA _3A-7A | n28A |
| DC_3A-7A_n78A | DC_3A_n78A DC_7A_n78A | CA _3A-7A | n78A |
| DC_3A-7C_n78A | DC_3A_n78A DC_7C_n78A | CA _3A-7C | n78A |
| DC_3C-7C_n78A | DC_3A_n78A DC_7A_n78A | CA _3C-7C | n78A |
| DC_3C-7A_n78A | DC_3A_n78A DC_7A_n78A | CA _3C-7A | n78A |
| DC_3A-8A_n78A | DC_3A_n78A DC_8A_n78A | CA _3A-8A | n78A |
| DC_3A-19A_n77A DC_3A-19A_n77C | DC_3A_n77A DC_19A_n77A | CA _3A-19A | n77A CA_n77C |
| DC_3A-19A_n78A DC_3A-19A_n78C | DC_3A_n78A DC_19A_n78A | CA _3A-19A | n78A CA_n78C |
| DC_3A-19A_n79A DC_3A-19A_n79C | DC_3A_n79A DC_19A_n79A | CA _3A-19A | n79A CA_n79C |
| DC_3A-20A_n28A | DC_3A_n28A DC_20A_n28A | CA _3A-20A | n28A |
| DC_3A-20A_n78A | DC_3A_n78A DC_20A_n78A | CA _3A-20A | n78A |
| DC_3C-20A_n78A | DC_3A_n78A DC_20A_n78A | CA _3C-20A | n78A |
| DC_3A-21A_n77A DC_3A-21A_n77C | DC_3A_n77A DC_21A_n77A | CA _3A-21A | n77A CA_n77C |
| DC_3A-21A_n78A DC_3A-21A_n78C | DC_3A_n78A DC_21A_n78A | CA _3A-21A | n78A CA_n78C |
| DC_3A-21A_n79A DC_3A-21A_n79C | DC_3A_n79A DC_21A_n79A | CA _3A-21A | n79A CA_n79C |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A | CA _3A-28A | n77A CA_n77C |
| DC_3A-28A_n78A DC_3A-28A_n78C | DC_3A_n78A DC_28A_n78A | CA _3A-28A | n78A CA_n78C |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A | CA _3A-28A | n79A CA_n79C |
| DC_3A_n28A-n78A | DC_3A_n28A, DC_3A_n78A | 3A | CA_n28A-n78A |
| DC_3A-42A_n77A DC_3A-42A_n77C | DC_3A_n77A | CA _3A-42C | n77A CA_n77C |
| DC_3A-42A_n78A DC_3A-42A_n78C | DC_3A_n78A | CA _3A-42A | n78A CA_n78C |
| DC_3A-42A_n79A DC_3A-42A_n79C | DC_3A_n79A | CA _3A-42A | n79A CA_n79C |
| DC_3A-42C_n77A | DC_3A_n77A | CA_3A-42C | n77A |
| DC_3A-42C_n78A | DC_3A_n78A | CA_3A-42C | n78A |
| DC_3A-42C_n79A | DC_3A_n79A | CA_3A-42C | n79A |
| DC_3A-42D_n77A | DC_3A_n77A | CA _3A-42A | n77A |
| DC_3A-42D_n78A | DC_3A_n78A | CA _3A-42A | n78A |
| DC_3A-42D_n79A | DC_3A_n79A | CA _3A-42A | n79A |
| DC_3A-42E_n77A | DC_3A_n77A | CA_3A-42E | n77A |
| DC_3A-42E_n78A | DC_3A_n78A | CA_3A-42E | n78A |
| DC_3A-42E_n79A | DC_3A_n79A | CA_1A-42E | n79A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A | 3A | CA_n77A-n79A |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A | 3A | CA_n78A-n79A |
| DC_3A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_3A_n80A_ULSUP-FDM_n78A | 3 | SUL_n78-n80 |
| DC_3A_SUL_n78A-n82A | DC_3A_n78A DC_3A_n82A | 3 | SUL_n78A-n82A |
| DC_3A_SUL_n79A-n80A | DC_3A_n79A, DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A | 3 | SUL_n79A-n80A |
| DC_5A-7A-7A_n78A | DC_5A_n78A DC_7A_n78A | CA_5A-7A-7A | n78A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A | CA _5A-7A | n78A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A | CA _5A-30A | n66A |
| DC_7A-20A_n28A | DC_7A_n28A DC_20A_n28A | CA _7A-20A | n28A |
| DC_7A-20A_n78A | DC_7A_n78A DC_20A_n78A | CA _7A-20A | n78A |
| DC_7A-28A_n78A | DC_7A_n78A DC_28A_n78A | CA _7A-28A | n78A |
| DC_7A_n28A-n78A | DC_7A_n28A, DC_7A_n78A | 7A | CA_n28A-n78A |
| DC_7C-28A_n78A | DC_7C_n78A DC_28A_n78A | CA _7C-28A | n78A |
| DC_7A-46A_n78A | DC_7A_n78A DC_46A_n78A | CA _7A-46A | n78A |
| DC_7A-46C_n78A | DC_7A_n78A DC_46C_n78A | CA _7A-46C | n78A |
| DC_7A-46D_n78A | DC_7A_n78A | CA_7A-46D | n78 |
| DC_7A-46E_n78A | DC_7A_n78A | CA_7A-46E | n78 |
| DC_8A_SUL_n78A-n81A | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A | 8 | SUL_n78A-n81A |
| DC_8A_SUL_n79A-n81A | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A | 8 | SUL_n79A-n81A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A | CA _12A-30A | n66A |
| DC_18A-28A_n77A | DC_18A_n77A DC_28A_n77A | CA _18A-28A | n77A |
| DC_18A-28A_n78A | DC_18A_n78A DC_28A_n78A | CA _18A-28A | n78A |
| DC_18A-28A_n79A | DC_18A_n79A DC_28A_n79A | CA _18A-28A | n79A |
| DC_19A-42A_n77A DC_19A-42A_n77C | DC_19A_n77A | CA _19A-42A | n77A CA_n77C |
| DC_19A-42A_n78A DC_19A-42A_n78C | DC_19A_n78A | CA _19A-42A | n78A CA_n78C |
| DC_19A-42A_n79A DC_19A-42A_n79C | DC_19A_n79A | CA _19A-42A | n79A CA_n79C |
| DC_19A-21A_n78A DC_19A-21A_n78C | DC_19A_n78A DC_21A_n78A | CA _19A-21A | n78A CA_n78C |
| DC_19A-21A_n79A DC_19A-21A_n79C | DC_19A_n79A DC_21A_n79A | CA _19A-21A | n79A CA_n79C |
| DC_19A-21A_n77A DC_19A-21A_n77C | DC_19A_n77A DC_21A_n77A | CA _19A-21A | n77A CA_n77C |
| DC_19A-42C_n77A | DC_19A_n77A | CA_19A-42C | n77A |
| DC_19A-42C_n78A | DC_19A_n78A | CA_19A-42C | n78A |
| DC_19A-42C_n79A | DC_19A_n79A | CA_19A-42C | n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A | 19A | CA_n77A-n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A | 19A | CA_n78A-n79A |
| DC_20A_n8A-n75A | DC_20A_n8A | 20A | CA_n8A-n75A |
| DC_20A_n28A-n75A | DC_20A_n28A | 20A | CA_n28A-n75A |
| DC_20A_n28A-n78A | DC_20A_n28A DC_20A_n78A | 20A | CA_n28A-n78A |
| DC_20A_n75A-n78A | DC_20A_n78A | 20A | CA_n75A-n78A |
| DC_20A_n76A-n78A | DC_20A_n78A | 20A | CA_n76A-n78A |
| DC_20A_SUL_n78A-n82A | DC_20A_n78A, DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A | 20 | SUL_n78A-n82A |
| DC_20A_SUL_n78A-n83A | DC_20A_n78A DC_20A_n83A | 20 | SUL_n78A-n83A |
| DC_21A-42A_n77A DC_21A-42A_n77C | DC_21A_n77A | CA _21A-42A | n77A CA_n77C |
| DC_21A-42A_n78A DC_21A-42A_n78C | DC_21A_n78A | CA _21A-42A | n78A CA_n78C |
| DC_21A-42A_n79A DC_21A-42A_n79C | DC_21A_n79A | CA _21A-42A | n79A CA_n79C |
| DC_21A-42C_n77A | DC_21A_n77A | CA_21A-42C | n77A |
| DC_21A-42C_n78A | DC_21A_n78A | CA_21A-42C | n78A |
| DC_21A-42C_n79A | DC_21A_n79A DC_42A_n79A | CA_21A-42C | n79A |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A | 21A | CA_n77A-n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A | 21A | CA_n78A-n79A |
| DC_28A_SUL_n78A-n83A | DC_28A_n78A, DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A | 28 | SUL_n78A-n83A |
| DC_28A-42A_n77A DC_28A-42A_n77C | DC_28A_n77A | CA_28A-42A | n77A |
| DC_28A-42A_n78A DC_28A-42A_n78C | DC_28A_n78A | CA_28A-42A | n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C | DC_28A_n79A | CA_28A-42A | n79A |
| DC_28A-42C_n77A | DC_28A_n77A | CA_28A-42C | n77A |
| DC_28A-42C_n78A | DC_28A_n78A | CA_28A-42C | n78A |
| DC_28A-42C_n79A | DC_28A_n79A | CA_28A-42C | n79A |
| DC_41A-42A_n77A | DC_41A_n77A | CA_41A-42A | n77A |
| DC_41C-42C_n77A | DC_41A_n77A | CA_41C-42C | n77A |
| DC_41A-42C_n77A | DC_41A_n77A | CA_41A-42C | n77A |
| DC_41C-42A_n77A | DC_41C_n77A | CA_41C-42A | n77A |
| DC_41A-42A_n78A | DC_41A_n78A | CA_41A-42A | n78A |
| DC_41C-42A_n78A | DC_41C_n78A | CA_41C-42A | n78A |
| DC_41C-42C_n78A | DC_41A_n78A | CA_41C-42C | n78A |
| DC_41A-42C_n78A | DC_41A_n78A | CA_41A-42C | n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A | DC_41A_n79A DC_42A_n79A | CA_41A-42A CA_41A-42C | n79A |
| DC_41C-42C_n79A | DC_41A_n79A | CA_41C-42C | n79 |
| DC_41C-42A_n79A | DC_41C_n79A | CA_41C-42A | n79A |
| DC_66A_(n)71B | DC_66A_71A DC_(n)71B | CA_66A_71A | n71A |
| DC_66A_SUL_n78A-n86A | DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A | 66 | SUL_n78A-n86A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.4.3	Inter-band EN-DC configurations (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A | CA_1A-3A-5A | n78A |
| DC_1A-3A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A | CA_1A-3A-7A | n28A |
| DC_1A-3A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3A-7A | n78A |
| DC_1A-3C-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3C-7A | n78A |
| DC_1A-3A-7A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A | CA_1A-3A-7A-7A | n78A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A | CA_1A-3A-8A | n28A |
| DC_1A-3A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A | CA_1A-3A-8A | n78A |
| DC_1A-3A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A | CA_1A-3A-20A | n78A |
| DC_1A-3A-28A_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A | n77A |
| DC_1A-3A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A | n78A |
| DC_1A-3A-28A_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A | n79A |
| DC_1A-3A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A | CA_1A-3A | CA_n28A-n78A |
| DC_1A-3A-19A_n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A | n77A |
| DC_1A-3A-19A_n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A | n78A |
| DC_1A-3A-19A_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A | n79A |
| DC_1A-3A-21A_n77A | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A | n77A |
| DC_1A-3A-21A_n78A | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A | n78A |
| DC_1A-3A-21A_n79A | DC_1A_n79A DC_3A_n79A DC_21A_n79A | CA_1A-3A-21A | n79A |
| DC_1A-3A-42C_n77A | DC_1A_n77A DC_3A_n77A | CA_1A-3A-42C | n77A |
| DC_1A-3A-42C_n78A | DC_1A_n78A DC_3A_n78A | CA_1A-3A-42C | n78A |
| DC_1A-3A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_42A_n79A | CA_1A-3A-42C | n79A |
| DC_1A-3A-42C_n77C | DC_1A_n77A DC_3A_n77A | CA_1A-3A-42C | n77A |
| DC_1A-3A-42C_n78C | DC_1A_n78A DC_3A_n78A | CA_1A-3A-42C | n78A |
| DC_1A-3A-42C_n79C | DC_1A_n79A DC_3A_n79A | CA_1A-3A-42C | n79A |
| DC_1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-5A-7A | n78A |
| DC_1A-5A-7A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-5A-7A-7A | n78A |
| DC_1A-7A-20A_n28A | DC_1A_n28A DC_7A_n28A DC_20A_n28A | CA_1A-7A-20A | n28A |
| DC_1A-7A-20A_n78A | DC_1A_n78A DC_7A_n78A DC_20A_n78A | CA_1A-7A-20A | n78A |
| DC_1A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A | CA_1A-7A | CA_n28A-n78A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A | CA_1A-18A-28A | n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A | CA_1A-18A-28A | n78A |
| DC_1A-18A-28A_n79A | DC_1A_n79A DC_18A_n79A DC_28A_n79A | CA_1A-18A-28A | n79A |
| DC_1A-19A-42A_n77A | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42A | n77A |
| DC_1A-19A-42A_n78A | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42A | n78A |
| DC_1A-19A-42A_n79A | DC_1A_n79A DC_19A_n79A DC_42A_n79A | CA_1A-19A-42A | n79A |
| DC_1A-19A-42C_n77A | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42C | n77A |
| DC_1A-19A-42C_n78A | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42C | n78A |
| DC_1A-19A-42C_n79A | DC_1A_n79A DC_19A_n79A DC_42A_n79A | CA_1A-19A-42C | n79A |
| DC_1A-19A-42C_n77C | DC_1A_n77A DC_19A_n77A | CA_1A-19A-42C | n77A |
| DC_1A-19A-42C_n78C | DC_1A_n78A DC_19A_n78A | CA_1A-19A-42C | n78A |
| DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A | CA_1A-19A-42C | n79A |
| DC_1A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-20A | CA_n28A-n78A |
| DC_1A-21A-28A_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A | n77A |
| DC_1A-21A-28A_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A | n78A |
| DC_1A-21A-28A_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A | n79A |
| DC_1A-21A-42A_n77A | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42A | n77A |
| DC_1A-21A-42A_n78A | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42A | n78A |
| DC_1A-21A-42A_n79A | DC_1A_n79A DC_21A_n79A DC_42A_n79A | CA_1A-21A-42A | n79A |
| DC_1A-21A-42C_n77A | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42C | n77A |
| DC_1A-21A-42C_n78A | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42C | n78A |
| DC_1A-21A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_42A_n79A | CA_1A-21A-42C | n79A |
| DC_1A-21A-42C_n77C | DC_1A_n77A DC_21A_n77A | CA_1A-21A-42C | n77A |
| DC_1A-21A-42C_n78C | DC_1A_n78A DC_21A_n78A | CA_1A-21A-42C | n78A |
| DC_1A-21A-42C_n79C | DC_1A_n79A DC_21A_n79A | CA_1A-21A-42C | n79A |
| DC_1A-28A-42A_n77A | DC_1A_n77A DC_28A_n77A DC_42A_n77A | CA_1A-28A-42A | n77A |
| DC_1A-28A-42A_n78A | DC_1A_n78A DC_28A_n78A DC_42A_n78A | CA_1A-28A-42A | n78A |
| DC_1A-28A-42A_n79A | DC_1A_n79A DC_28A_n79A DC_42A_n79A | CA_1A-28A-42A | n79A |
| DC_1A-28A-42C_n77A | DC_1A_n77A DC_28A_n77A | CA_1A-28A-42A | n77A |
| DC_1A-28A-42C_n78A | DC_1A_n78A DC_28A_n78A | CA_1A-28A-42A | n78A |
| DC_1A-28A-42C_n79A | DC_1A_n79A DC_28A_n79A | CA_1A-28A-42A | n79A |
| DC_1A-41A-42A_n77A | DC_1A_n77A DC_41A_n77A DC_42A_n77A | CA_1A-41A-42A | n77A |
| DC_1A-41A-42C_n77A | DC_1A_n77A DC_41A_n77A DC_42A_n77A | CA_1A-41A-42C | n77A |
| DC_1A-41C-42A_n77A | DC_1A_n77A DC_41A_n77A DC_42A_n77A | CA_1A-41C-42A | n77A |
| DC_1A-41A-42A_n78A | DC_1A_n78A DC_41A_n78A DC_42A_n78A | CA_1A-41A-42A | n78A |
| DC_1A-41A-42C_n78A | DC_1A_n78A DC_41A_n78A DC_42A_n78A | CA_1A-41A-42C | n78A |
| DC_1A-41C-42A_n78A | DC_1A_n78A DC_41A_n78A DC_42A_n78A | CA_1A-41C-42A | n78A |
| DC_1A-41A-42A_n79A | DC_1A_n79A DC_41A_n79A DC_42A_n79A | CA_1A-41A-42A | n79A |
| DC_1A-41A-42C_n79A | DC_1A_n79A DC_41A_n79A DC_42A_n79A | CA_1A-41A-42C | n79A |
| DC_1A-41C-42A_n79A | DC_1A_n79A DC_41A_n79A DC_42A_n79A | CA_1A-41C-42A | n79A |
| DC_1A-41C-42C_n77A | DC_1A_n77A DC_41A_n77A | CA_1A-41C-42C | n77A |
| DC_1A-41C-42C_n78A | DC_1A_n78A DC_41A_n78A | CA_1A-41C-42C | n78A |
| DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A | CA_1A-41C-42C | n79A |
| DC_2A-66A-(n)71B | DC_2A_n71A DC_66A_n71A DC_(n)71B | CA _2A-66A-71A | n71A |
| DC_3A-5A-7A-7A_n78A | DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_3A-5A-7A-7A | n78A |
| DC_3A-5A-7A_n78A | DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_3A-5A-7A | n78A |
| DC_3A-7A-20A_n28A | DC_3A_n28A DC_7A_n28A DC_20A_n28A | CA_3A-7A-20A | n28A |
| DC_3A-7A-20A_n78A | DC_3A_n78A DC_28A_n78A DC_7A_n78A | CA_3A-7A-20A | n78A |
| DC_3A-7A-28A_n78A | DC_3A-7A_n78A DC_3A-28A_n78A DC_7A-28A_n78A | CA_3A-7A-28A | n78A |
| DC_3A-7C-28A_n78A | DC_3A_n78A DC_7A_n78A DC_28A_n78A | CA_3A-7C-28A | n78A |
| DC_3A-7A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A | CA_3A-7A | CA_n28A-n78A |
| DC_3A-19A-21A_n77A | DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_3A-19A-21A | n77A |
| DC_3A-19A-21A_n78A | DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_3A-19A-21A | n78A |
| DC_3A-19A-21A_n79A | DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_3A-19A-21A | n79A |
| DC_3A-19A-42A_n77A | DC_3A_n77A DC_19A_n77A DC_42A_n77A | CA_3A-19A-42A | n77A |
| DC_3A-19A-42C_n77A | DC_3A_n77A DC_19A_n77A DC_42A_n77A | CA_3A-19A-42C | n77A |
| DC_3A-19A-42C_n77C | DC_3A_n77A DC_19A_n77A | CA_3A-19A-42C | n77A |
| DC_3A-19A-42A_n78A | DC_3A_n78A DC_19A_n78A DC_42A_n78A | CA_3A-19A-42A | n78A |
| DC_3A-19A-42C_n78A | DC_3A_n78A DC_19A_n78A DC_42A_n78A | CA_3A-19A-42C | n78A |
| DC_3A-19A-42C_n78C | DC_3A_n78A DC_19A_n78A | CA_3A-19A-42C | n78A |
| DC_3A-19A-42A_n79A | DC_3A_n79A DC_19A_n79A DC_42A_n79A | CA_3A-19A-42A | n79A |
| DC_3A-19A-42C_n79A | DC_3A_n79A DC_19A_n79A DC_42A_n79A | CA_3A-19A-42C | n79A |
| DC_3A-19A-42C_n79C | DC_3A_n79A DC_19A_n79A | CA_3A-19A-42C | n79A |
| DC_3A-20A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A | CA_3A-20A | CA_n28A-n78A |
| DC_3A-21A-42C_n77A | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | n77A |
| DC_3A-21A-42C_n78A | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | n78A |
| DC_3A-21A-42C_n79A | DC_3A_n79A DC_21A_n79A DC_42A_n79A | CA_3A-21A-42C | n79A |
| DC_3A-21A-42C_n77C | DC_3A_n77A DC_21A_n77A | CA_3A-21A-42C | n77A |
| DC_3A-21A-42C_n78C | DC_3A_n78A DC_21A_n78A | CA_3A-21A-42C | n78A |
| DC_3A-21A-42C_n79C | DC_3A_n79A DC_21A_n79A DC_42A_n79A | CA_3A-21A-42C | n79A |
| DC_3A-28A-42A_n77A | DC_3A_n77A DC_28A_n77A DC_42A_n77A | CA_3A-28A-42A | n77A |
| DC_3A-28A-42A_n78A | DC_3A_n78A DC_28A_n78A DC_42A_n78A | CA_3A-28A-42A | n78A |
| DC_3A-28A-42A_n79A | DC_3A_n79A DC_28A_n79A DC_42A_n79A | CA_3A-28A-42A | n79A |
| DC_3A-28A-42C_n77A | DC_3A_n77A DC_28A_n77A | CA_3A-28A-42A | n77A |
| DC_3A-28A-42C_n78A | DC_3A_n78A DC_28A_n78A | CA_3A-28A-42A | n78A |
| DC_3A-28A-42C_n79A | DC_3A_n79A DC_28A_n79A | CA_3A-28A-42A | n79A |
| DC_7A-20A_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_7A-20A | CA_n28A-n78A |
| DC_19A-21A-42A_n77A | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42A | n77A |
| DC_19A-21A-42A_n78A | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42A | n78A |
| DC_19A-21A-42A_n79A | DC_19A_n79A DC_21A_n79A DC_42A_n79A | CA_19A-21A-42A | n79A |
| DC_19A-21A-42C_n77A | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42C | n77A |
| DC_19A-21A-42C_n78A | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42C | n78A |
| DC_19A-21A-42C_n79A | DC_19A_n79A DC_21A_n79A DC_42A_n79A | CA_19A-21A-42C | n79A |
| DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A | CA_19A-21A-42C | n77A |
| DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A | CA_19A-21A-42C | n78A |
| DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A DC_42A_n79A | CA_19A-21A-42C | n79A |
| DC_21A-28A-42A_n77A | DC_21A_n77A DC_28A_n77A DC_42A_n77A | CA_21A-28A-42A | n77A |
| DC_21A-28A-42A_n78A | DC_21A_n78A DC_28A_n78A DC_42A_n78A | CA_21A-28A-42A | n78A |
| DC_21A-28A-42A_n79A | DC_21A_n79A DC_28A_n79A DC_42A_n79A | CA_21A-28A-42A | n79A |
| DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A | CA_21A-28A-42A | n77A |
| DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A | CA_21A-28A-42A | n78A |
| DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A | CA_21A-28A-42A | n79A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.4.4	Inter-band EN-DC configurations (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-3A-5A-7A | n78A |
| DC_1A-3A-7A-20A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A | CA_1A-3A-7A-20A | n28A |
| DC_1A-3A-5A-7A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A | CA_1A-3A-5A-7A-7A | n78A |
| DC_1A-3A-7A-20A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A | CA_1A-3A-7A-20A | n78A |
| DC_1A-3A-7A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A | CA_1A-3A-7A | CA_n28A-n78A |
| DC_1A-3A-19A-21A_n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-3A-19A-21A | n77A |
| DC_1A-3A-19A-21A_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-3A-19A-21A | n77A |
| DC_1A-3A-19A-21A_n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-3A-19A-21A | n78A |
| DC_1A-3A-19A-21A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-3A-19A-21A | n78A |
| DC_1A-3A-19A-21A_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-3A-19A-21A | n79A |
| DC_1A-3A-19A-21A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-3A-19A-21A | n79A |
| DC_1A-3A-19A-42A _n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42A | n77A |
| DC_1A-3A-19A-42A _n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42A | n77A |
| DC_1A-3A-19A-42C _n77A | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42C | n77A |
| DC_1A-3A-19A-42C _n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A | CA_1A-3A-19A-42A | n77C |
| DC_1A-3A-19A-42A _n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42A | n78A |
| DC_1A-3A-19A-42A_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78 | CA_1A-3A-19A-42A | n78A |
| DC_1A-3A-19A-42C _n78A | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42C | n78A |
| DC_1A-3A-19A-42C _n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A | CA_1A-3A-19A-42C | n78C |
| DC_1A-3A-19A-42A _n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_42A_n79A | CA_1A-3A-19A-42A | n79A |
| DC_1A-3A-19A-42A_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42A | n79A |
| DC_1A-3A-19A-42C _n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42C | n79A |
| DC_1A-3A-19A-42C _n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-19A-42C | n79C |
| DC_1A-3A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-3A-20A | CA_n28A-n78A |
| DC_1A-3A-21A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A-42C | n77A |
| DC_1A-3A-21A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A | CA_1A-3A-21A-42C | n77C |
| DC_1A-3A-21A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A-42C | n78A |
| DC_1A-3A-21A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A | CA_1A-3A-21A-42C | n78C |
| DC_1A-3A-21A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_42A_n79A | CA_1A-3A-21A-42C | n79A |
| DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A | CA_1A-3A-21A-42C | n79C |
| DC_1A-3A-28A-42A_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A-42A | n77A |
| DC_1A-3A-28A-42A_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A-42A | n78A |
| DC_1A-3A-28A-42A_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A-42A | n79A |
| DC_1A-3A-28A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_28A_n77A | CA_1A-3A-28A-42C | n77A |
| DC_1A-3A-28A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_28A_n78A | CA_1A-3A-28A-42C | n78A |
| DC_1A-3A-28A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_28A_n79A | CA_1A-3A-28A-42C | n79A |
| DC_1A-7A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-7A-20A | CA_n28A-n78A |
| DC_1A-19A-21A-42A_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42A | n77A |
| DC_1A-19A-21A-42A_n78A | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42A | n78A |
| DC_1A-19A-21A-42A_n79A | DC_1A_n79A DC_19A_n79A DC_21A_n79A DC_42A_n79A | CA_1A-19A-21A-42A | n79A |
| DC_1A-19A-21A-42A_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42A | n77A |
| DC_1A-19A-21A-42A_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42A | n78A |
| DC_1A-19A-21A-42A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42A | n79A |
| DC_1A-19A-21A-42C_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42C | n77A |
| DC_1A-19A-21A-42C_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A | CA_1A-19A-21A-42C | n77C |
| DC_1A-19A-21A-42C_n78A | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42C | n78A |
| DC_1A-19A-21A-42C_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A | CA_1A-19A-21A-42C | n78C |
| DC_1A-19A-21A-42C_n79A | DC_1A_n79A DC_19A_n79A DC_21A_n79A DC_42A_n79A | CA_1A-19A-21A-42C | n79A |
| DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A | CA_1A-19A-21A-42C | n79C |
| DC_1A-21A-28A-42A_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A-42A | n77A |
| DC_1A-21A-28A-42A_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A-42A | n78A |
| DC_1A-21A-28A-42A_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A-42A | n79A |
| DC_1A-21A-28A-42C_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A | CA_1A-21A-28A-42C | n77A |
| DC_1A-21A-28A-42C_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A | CA_1A-21A-28A-42C | n78A |
| DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A | CA_1A-21A-28A-42C | n79A |
| DC_3A-7A-20A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_3A-7A-20A | CA_n28A-n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.4.5	Inter-band EN-DC configurations (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A | CA_1A-3A-7A-20A | CA_n28A-n78A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.5B.5	Inter-band EN-DC including FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 and for NR operating bands and CA configurations in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

#### 5.5B.5.1	Inter-band EN-DC configurations (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F | DC_1A_n257A | 1A | n257A CA_n257D CA_n257E CA_n257F |
| DC_2A_n257A DC_2A_n257(2A) | DC_2A_n257A | 2A | n257A CA_n257(2A) |
| DC_2A-2A_n257A | DC_2A-2A_n257A | CA _2A-2A | n257A |
| DC_2A_n257A | DC_2A_n257A | 2 | n257A |
| DC_2C_n257A | DC_2C_n257A | CA _2C | n257A |
| DC_2A_n260 DC_2A_n260(2A) | DC_2A_n260A | 2A | n260A CA_n260(2A) |
| DC_2A-2A_n260A | DC_2A_n260A | CA _2A-2A | n260A |
| DC_2C_n260A | DC_2C_n260A | CA _2C | n260A |
| DC_3A_n257A DC_3A_n257D DC_3A_n257E DC_3A_n257F | DC_3A_n257A | 3A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A_n258A | DC_3A_n258A | 3 | n258A |
| DC_5A-5A_n257A | DC_5A_n257A | CA _5A-5A | n257A |
| DC_5A-5A_n260A | DC_5A_n260A | CA _5A-5A | n260A |
| DC_5A_n257A | DC_5A_n257A | 5 | n257A |
| DC_5A_n260A DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q | DC_5A_n260A | 5 | n260A CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260B CA_n260C CA_n260D CA_n260E CA_n260F CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260O CA_n260P CA_n260Q |
| DC_5A_n261A DC_5A_n261(2A) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A | 5 | n261A CA_n261(2A) CA_n261(3A) CA_n261(4A) CA_n261B CA_n261C CA_n261D CA_n261E CA_n261F CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M CA_n261O CA_n261P CA_n261Q |
| DC_5B_n257A | DC_5B_n257A | CA _5B | n257A |
| DC_5B_n260A | DC_5B_n260A | CA _5B | n260A |
| DC_7A-7A_n257A | DC_7A_n257A | CA _7A-7A | n257A |
| DC_7A_n257A | DC_7A_n257A | 7 | n257A |
| DC_7A_n258A | DC_7A_n258A | 7 | n258A |
| DC_8A_n257A | DC_8A_n257A | 8 | n257A |
| DC_8A_n258A | DC_8A_n258A | 8 | n258A |
| DC_11A_n257A | DC_11A_n257A | 11 | n257A |
| DC_12A_n260A | DC_12A_n260A | 12A | n260A |
| DC_13A_n257A | DC_13A_n257A | 13 | n257A |
| DC_13A_n260A | DC_13A_n260A | 13 | n260A |
| DC_18A_n257A | DC_18A_n257A | 18 | n257A |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F | DC_19A_n257A | 19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_20A_n258A | DC_20A_n258A | 20A | n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F | DC_21A_n257A | 21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_26A _n257A | DC_26A _n257A | 26 | n257A |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F | DC_28A_n257A | 28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_28A_n258A | DC_28A_n258A | 28 | n258A |
| DC_30A_n260A | DC_30A_n260A | 30A | CA_n260A |
| DC_41A_n257A DC_41C_n257A | DC_41A_n257A | 41 | n257A |
| DC_41A_n258A | DC_41A_n258A | 41A | CA_n258A |
| DC_41C_n257A | DC_41C_n257A | CA _41C | n257A |
| DC_42A_n257A DC_42C_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F | DC_42A_n257A | 42 | n257A CA_n257D CA_n257E CA_n257F |
| DC_42C_n257A | DC_42C_n257A | CA _42C | n257A |
| DC_42E_n257A | DC_42A_n257A | 42 | n257A |
| DC_48A-48A_n257A | DC_48A_n257A | CA _48A-48A | n257A |
| DC_48A-48A_n260A | DC_48A_n260A | CA _48A-48A | n260A |
| DC_48A_n257A | DC_48A_n257A | 48 | n257A |
| DC_48C_n257A | DC_48C_n257A | CA _48C | n257A |
| DC_48C_n260A | DC_48C_n260A | CA _48C | n260A |
| DC_48A_n260A | DC_48A_n260A | 48 | n260A |
| DC_66A-66A_n257A | DC_66A_n257A | CA _66A-66A | n257A |
| DC_66A-66A_n260A | DC_66A_n260A | CA _66A-66A | n260A |
| DC_66A_n257A DC_66A_n257(2A) DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M | DC_66A_n257A | 66 | n257A CA_n257(2A) CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M |
| DC_66A_n260A DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A | 66 | n260A CA_n260(2A) CA_n260(3A) CA_n260(4A) CA_n260D CA_n260E CA_n260F CA_n260G CA_n260H CA_n260I CA_n260J CA_n260K CA_n260L CA_n260M CA_n260O CA_n260P CA_n260Q |
| DC_66C_n257A | DC_66C_n257A | CA _66C | n257A |
| DC_66A_n261A DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A | 66A | n261A CA_n261(2A) CA_n261(3A) CA_n261(4A) CA_n261D CA_n261E CA_n261F CA_n261G CA_n261H CA_n261I CA_n261J CA_n261K CA_n261L CA_n261M CA_n261O CA_n261P CA_n261Q |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.5.2	Inter-band EN-DC configurations (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A_n257A DC_1A-3A_n257D DC_1A-3A_n257E DC_1A-3A_n257F | DC_1A_n257A DC_3A_n257A | CA _1A-3A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-5A_n257A | DC_1A_n257A DC_5A_n257A | CA _1A-5A | n257A |
| DC_1A-7A_n257A | DC_1A_n257A DC_7A_n257A | CA _1A-7A | n257A |
| DC_1A-7A-7A_n257A | DC_1A_n257A DC_7A-7A_n257A | CA _1A-7A-7A | n257A |
| DC_1A-8A_n257A | DC_1A-257A DC_8A_n257A | CA _1A-8A | n257A |
| DC_1A-18A_n257A | DC_1A-257A DC_18A_n257A | CA _1A-18A | n257A |
| DC_1A-19A_n257A DC_1A-19A_n257D DC_1A-19A_n257E DC_1A-19A_n257F | DC_1A-257A DC_19A_n257A | CA _1A-19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-21A_n257A DC_1A-21A_n257D DC_1A-21A_n257E DC_1A-21A_n257F | DC_1A_n257A DC_21A_n257A | CA _1A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-28A_n257A DC_1A-28A_n257D DC_1A-28A_n257E DC_1A-28A_n257F | DC_1A_n257A DC_28A_n257A | CA _1A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-41A_n257A | DC_1A_n257A DC_41A_n257A | CA _1A-41A | n257A |
| DC_1A-41C_n257A | DC_1A_n257A DC_41C_n257A | CA _1A-41C | n257A |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F | DC_1A_n257A DC_42A_n257A | CA _1A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_1A-42C_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42C | n257A |
| DC_1A-42D_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42C | n257A |
| DC_1A-42E_n257A | DC_1A_n257A DC_42A_n257A | CA_1A-42E | n257A |
| DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A | 1A | CA_n77A-n257A |
| DC_1A_n78A-n257A | DC_1A_n78A DC_1A_n257A | 1A | CA_n78A-n257A |
| DC_1A_n79A-n257A | DC_1A_n79A DC_1A_n257A | 1A | CA_n79A-n257A |
| DC_2A-13A_n260A | DC_2A_n260A DC_13A_n260A | CA _2A-13A | n260A |
| DC_2A-5A_n257A | DC_2A_n257A DC_5A_n257A | CA _2A-5A | n257A |
| DC_2A-5A_n260A | DC_2A_n260A DC_5A_n260A | CA _2A-5A | n260A |
| DC_2A-12A_n260A | DC_2A_n260A DC_12A_n260A | CA _2A-12A | n260A |
| DC_2A-13A_n257A | DC_2A_n257A DC_13A_n257A | CA _2A-13A | n257A |
| DC_2A-30A_n260A | DC_2A_n260A DC_30A_n260A | CA _2A-30A | n260A |
| DC_2A-66A_n257A | DC_2A_n257A DC_66A_n257A | CA _2A-66A | n257A |
| DC_2A-66A_n260A | DC_2A_n260A DC_66A_n260A | CA _2A-66A | n260A |
| DC_3A-5A_n257A | DC_3A_n257A DC_5A_n257A | CA _3A-5A | n257A |
| DC_3A-7A-7A_n257A | DC_3A_n257A DC_7A_n257A | CA _3A-7A-7A | n257A |
| DC_3A-7A_n257A | DC_3A_n257A DC_7A_n257A | CA _3A-7A | n257A |
| DC_3A-19A_n257A DC_3A-19A_n257D DC_3A-19A_n257E DC_3A-19A_n257F | DC_3A_n257A DC_19A_n257A | CA _3A-19A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-21A_n257A DC_3A-21A_n257D DC_3A-21A_n257E DC_3A-21A_n257F | DC_3A_n257A DC_21A_n257A | CA _3A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-28A_n257A DC_3A-28A_n257D DC_3A-28A_n257E DC_3A-28A_n257F | DC_3A_n257A DC_28A_n257A | CA _3A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-41A_n257A | DC_3A_n257A DC_41A_n257A | CA _3A-41A | n257A |
| DC_3A-42A_n257A DC_3A-42A_n257D DC_3A-42A_n257E DC_3A-42A_n257F | DC_3A_n257A DC_42A_n257A | CA _3A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_3A-42C_n257A | DC_3A_n257A DC_42A_n257A | CA_3A-42C | n257A |
| DC_3A-42D_n257A | DC_3A_n257A DC_42A_n257A | CA _3A-42A | n257A |
| DC_3A-42E_n257A | DC_3A_n257A DC_42A_n257A | CA_3A-42E | n257A |
| DC_3A_n77A-n257A | DC_3A_n77A DC_3A_n257A | 3A | CA_n77A-n257A |
| DC_3A_n78A-n257A | DC_3A_n78A DC_3A_n257A | 3A | CA_n78A-n257A |
| DC_3A_n79A-n257A | DC_3A_n79A DC_3A_n257A | 3A | CA_n79A-n257A |
| DC_5A-30A_n260A | DC_5A_n260A DC_30A_n260A | CA _5A-30A | n260A |
| DC_5A-66A_n257A | DC_5A_n257A DC_66A_n257A | CA _5A-66A | n257A |
| DC_5A-66A_n260A | DC_5A_n260A DC_66A_n260A | CA _5A-66A | n260A |
| DC_5A-7A-7A_n257A | DC_5A_n257A DC_7A_n257A | CA _5A-7A-7A | n257A |
| DC_5A-7A_n257A | DC_5A_n257A DC_7A_n257A | CA _5A-7A | n257A |
| DC_5A_n78A-n257A | DC_5A_n78A DC_5A_n257A | 5A | CA_n78A-n257A |
| DC_5B_n260A | DC_5B_n260A | CA _5B | n260A |
| DC_7A-7A_n257A | DC_7A_n257A | CA _7A-7A | n257A |
| DC_7A_n78A-n257A | DC_7A_n78A DC_7A_n257A | 7A | CA_n78A-n257A |
| DC_12A-30A_n260A | DC_12A_n260A DC_30A_n260A | CA _12A-30A | n260A |
| DC_12A-66A_n260A | DC_12A_n260A DC_66A_n260A | CA _12A-66A | n260A |
| DC_13A-66A_n257A | DC_13A_n257A DC_66A_n257A | CA _13A-66A | n257A |
| DC_13A-66A_n260A | DC_13A_n260A DC_66A_n260A | CA _13A-66A | n260A |
| DC_18A-28A-n257A | DC_18A_n257A DC_28A_n257A | CA _18A-28A | n257A |
| DC_19A-42A_n257A DC_19A-42A_n257D DC_19A-42A_n257E DC_19A-42A_n257F | DC_19A_n257A DC_42A_n257A | CA _19A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_19A-21A_n257A DC_19A-21A_n257D DC_19A-21A_n257E DC_19A-21A_n257F | DC_19A_n257A DC_21A_n257A | CA _19A-21A | n257A CA_n257D CA_n257E CA_n257F |
| DC_19A-42C_n257A | DC_19A_n257A DC_42A_n257A | CA_19A-42C | n257 |
| DC_19A_n77A-n257A | DC_19A_n77A DC_19A_n257A | 19A | CA_n77A-n257A |
| DC_19A_n78A-n257A | DC_19A_n78A DC_19A_n257A | 19A | CA_n78A-n257A |
| DC_19A_n79A-n257A | DC_19A_n79A DC_19A_n257A | 19A | CA_n79A-n257A |
| DC_21A-28A_n257A DC_21A-28A_n257D DC_21A-28A_n257E DC_21A-28A_n257F | DC_21A_n257A DC_28A_n257A | CA _21A-28A | n257A CA_n257D CA_n257E CA_n257F |
| DC_21A-42A_n257A DC_21A-42A_n257D DC_21A-42A_n257E DC_21A-42A_n257F | DC_21A_n257A DC_42A_n257A | CA _21A-42A | n257A CA_n257D CA_n257E CA_n257F |
| DC_21A-42C_n257A | DC_21A_n257A DC_42A_n257A | CA_21A-42C | n257A |
| DC_21A_n77A-n257A | DC_21A_n77A DC_21A_n257A | 21A | CA_n77A-n257A |
| DC_21A_n78A-n257A | DC_21A_n78A DC_21A_n257A | 21A | CA_n78A-n257A |
| DC_21A_n79A-n257A | DC_21A_n79A DC_21A_n257A | 21A | CA_n79A-n257A |
| DC_28A-42C_n257A | DC_28A_n257A DC_42A_n257A | CA_28A-42C | n257 |
| DC_28A-42A_n257A | DC_28A_n257A DC_42A_n257A | CA_28A-42A | n257 |
| DC_30A-66A_n260A | DC_30A_n260A DC_66A_n260A | CA _30A-66A | n260A |
| DC_41A-42A_n257A | DC_41A_n257A DC_42A_n257A | CA _41A-42A | n257A |
| DC_41A-42C_n257A | DC_41A_n257A DC_42C_n257A | CA _41A-42C | n257A |
| DC_41C-42A_n257A | DC_41C_n257A DC_42A_n257A | CA _41C-42A | n257A |
| DC_41C-42C_n257A | DC_41A_n257A DC_42A_n257A | CA_41C-42C | n257A |
| DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F | DC_42C_n257A | CA _42C | n257A CA_n257D CA_n257E CA_n257F |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.5.3	Inter-band EN-DC configurations (four bands)

Table 5.5B.5.3-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A | CA_1A-3A-5A | n257A |
| DC_1A-3A-7A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_7A_n257A | CA_1A-3A-7A-7A | n257A |
| DC_1A-3A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_7A_n257A | CA_1A-3A-7A | n257A |
| DC_1A-3A-19A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A | CA_1A-3A-19A | n257A |
| DC_1A-3A-21A_n257A | DC_1A_n257A DC_3A_n257A DC_21A_n257A | CA_1A-3A-21A | n257A |
| DC_1A-3A-28A_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A | CA_1A-3A-28A | n257A |
| DC_1A-3A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | n257A |
| DC_1A-3A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | n257A |
| DC_1A-3A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | n257A |
| DC_1A-3A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_42A_n257A | CA_1A-3A-42C | n257A |
| DC_1A-3A_n78A-n257A | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A | CA_1A-3A | CA_n78A-n257A |
| DC_1A-5A-7A-7A_n257A | DC_1A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-5A-7A-7A | n257A |
| DC_1A-5A-7A_n257A | DC_1A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-5A-7A | n257A |
| DC_1A-5A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_5A_n78A DC_5A_n257A, | CA_1A-5A | CA_n78A-n257A |
| DC_1A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-7A-7A | CA_n78A-n257A |
| DC_1A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-7A | CA_n78A-n257A |
| DC_1A-18A-28A_n257A | DC_1A_n257A DC_18A_n257A DC_28A_n257A | CA_1A-18A-28A | n257A |
| DC_1A-19A-42A_n257A | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42A | n257A |
| DC_1A-19A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42C | n257A |
| DC_1A-19A-42C_n257D | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42A | n257A |
| DC_1A-19A-42C_n257E | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42A | n257A |
| DC_1A-19A-42C_n257F | DC_1A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-19A-42A | n257A |
| DC_1A-21A-28A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A | CA_1A-21A-28A | n257 |
| DC_1A-21A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42A | n257A |
| DC_1A-21A-42C_n257A | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | n257A |
| DC_1A-21A-42C_n257D | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | n257A |
| DC_1A-21A-42C_n257E | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | n257A |
| DC_1A-21A-42C_n257F | DC_1A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-21A-42C | n257A |
| DC_1A-28A-42A_n257A | DC_1A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-28A-42A | n257 |
| DC_1A-28A-42C_n257A | DC_1A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-28A-42A | n257 |
| DC_1A-41A-42A_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41A-42A | n257 |
| DC_1A-41A-42C_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41A-42C | n257A |
| DC_1A-41C-42A_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41C-42A | n257A |
| DC_1A-41C-42C_n257A | DC_1A_n257A DC_41A_n257A DC_42A_n257A | CA_1A-41C-42C | n257A |
| DC_3A-5A-7A-7A_n257A | DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_3A-5A-7A-7A | n257A |
| DC_3A-5A-7A_n257A | DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_3A-5A-7A | n257A |
| DC_3A-5A_n78A-n257A | DC_3A_n78A DC_3A_n257A, DC_5A_n78A DC_5A_n257A, | CA_3A-5A | CA_n78A-n257A |
| DC_3A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A | CA_3A-7A-7A | CA_n78A-n257A |
| DC_3A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A, DC_7A_n78A DC_7A_n257A, | CA_3A-7A | CA_n78A-n257A |
| DC_3A-19A-21A_n257A | DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_3A-19A-21A | n257A |
| DC_3A-19A-42A_n257A | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42A | n257A |
| DC_3A-19A-42C_n257A | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | n257A |
| DC_3A-19A-42C_n257D | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | n257A |
| DC_3A-19A-42C_n257E | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | n257A |
| DC_3A-19A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_3A-19A-42C | n257A |
| DC_3A-21A-42C_n257A | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | n257A |
| DC_3A-21A-42C_n257D | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | n257A |
| DC_3A-21A-42C_n257E | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | n257A |
| DC_3A-21A-42C_n257F | DC_3A_n257A DC_21A_n257A DC_42A_n257A | CA_3A-21A-42C | n257A |
| DC_3A-28A-42A_n257A | DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_3A-28A-42A | n257A |
| DC_3A-28A-42C_n257A | DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_3A-28A-42A | n257A |
| DC_5A-7A-7A_n78A-n257A | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_5A-7A-7A | CA_n78A-n257A |
| DC_5A-7A_n78A-n257A | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A | CA_5A-7A | CA_n78A-n257A |
| DC_19A-21A-42A_n257A | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42A | n257A |
| DC_19A-21A-42C_n257D | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | n257A |
| DC_19A-21A-42C_n257E | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | n257A |
| DC_19A-21A-42C_n257F | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | n257A |
| DC_19A-21A-42C_n257A | DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_19A-21A-42C | n257A |
| DC_21A-28A-42A_n257A | DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_21A-28A-42A | n257A |
| DC_21A-28A-42C_n257A | DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_21A-28A-42A | n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.5.4	Inter-band EN-DC configurations (five bands)

Table 5.5B.5.4-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A | n257A |
| DC_1A-3A-5A-7A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A | CA_1A-3A-5A-7A-7A | n257A |
| DC_1A-3A-5A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_3A_n78A DC_3A_n257A, DC_5A_n78A DC_5A_n257A, | CA_1A-3A-5A | CA_n78A-n257A |
| DC_1A-3A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_3A_n78A DC_3A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-3A-7A-7A | CA_n78A-n257A |
| DC_1A-3A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_3A_n78A DC_3A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-3A-7A | CA_n78A-n257A |
| DC_1A-3A-19A-21A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | n257A |
| DC_1A-3A-19A-21A_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | n257A |
| DC_1A-3A-19A-21A_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | n257A |
| DC_1A-3A-19A-21A_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A | CA_1A-3A-19A-21A | n257A |
| DC_1A-3A-19A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | n257A |
| DC_1A-3A-19A-42A_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | n257A |
| DC_1A-3A-19A-42A_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | n257A |
| DC_1A-3A-19A-42A_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42A | n257A |
| DC_1A-3A-19A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | n257A |
| DC_1A-3A-19A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | n257D |
| DC_1A-3A-19A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | n257E |
| DC_1A-3A-19A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-19A-42C | n257F |
| DC_1A-3A-21A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | n257A |
| DC_1A-3A-21A-42C_n257D | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | n257D |
| DC_1A-3A-21A-42C_n257E | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | n257E |
| DC_1A-3A-21A-42C_n257F | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_42A_n257A | CA_1A-3A-21A-42C | n257F |
| DC_1A-3A-28A-42A_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-3A-21A-42A | n257A |
| DC_1A-3A-28A-42C_n257A | DC_1A_n257A DC_3A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-3A-28A-42C | n257A |
| DC_1A-5A-7A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_5A_n78A DC_5A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-5A-7A-7A | CA_n78A-n257A |
| DC_1A-5A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_5A_n78A DC_5A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-5A-7A | CA_n78A-n257A |
| DC_1A-19A-21A-42A_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257D | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257E | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42A_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42A | n257A |
| DC_1A-19A-21A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257A |
| DC_1A-19A-21A-42C_n257D | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257D |
| DC_1A-19A-21A-42C_n257E | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257E |
| DC_1A-19A-21A-42C_n257F | DC_1A_n257A DC_19A_n257A DC_21A_n257A DC_42A_n257A | CA_1A-19A-21A-42C | n257F |
| DC_1A-19A-28A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-19A-28A-42C | n257A |
| DC_1A-21A-28A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A DC_42A_n257A | CA_1A-21A-28A-42A | n257A |
| DC_3A-5A-7A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A, DC_5A_n78A DC_5A_n257A, DC_7A_n78A DC_7A_n257A, | CA_3A-5A-7A-7A | CA_n78A-n257A |
| DC_3A-5A-7A_n78A-n257A | DC_3A_n78A DC_3A_n257A, DC_5A_n78A DC_5A_n257A, DC_7A_n78A DC_7A_n257A, | CA_3A-5A-7A | CA_n78A-n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

#### 5.5B.5.5	Inter-band EN-DC configurations (six bands)

Table 5.5B.5.5-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-5A-7A_n78A-n257A | DC_1A_n78A DC_1A_n257A, DC_3A_n78A DC_3A_n257A, DC_5A_n78A DC_5A_n257A, DC_7A_n78A DC_7A_n257A, | CA_1A-3A-5A-7A | CA_n78A-n257A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

### 5.5B.6	Inter-band EN-DC including FR1 and FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 and for NR operating bands and CA configurations in TS 38.101-1, TS 38.101-2 and TS 38.101-3.

#### 5.5B.6.1	Inter-band EN-DC configurations (two bands)

This section is N/A.

#### 5.5B.6.2	Inter-band EN-DC configurations (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257A |
| DC_1A_n77A-n257D | DC_1A_n77A DC_1A-n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257D |
| DC_1A_n77A-n257E | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257E |
| DC_1A_n77A-n257F | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77A-n257F |
| DC_1A_n77C-n257A | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257A |
| DC_1A_n77C-n257D | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257D |
| DC_1A_n77C-n257E | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257E |
| DC_1A_n77C-n257F | DC_1A_n77A DC_1A_n257A DC_1A_n77A-n257A | 1A | CA_n77C-n257F |
| DC_3A_n77A-n257A | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257A |
| DC_3A_n77A-n257D | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257D |
| DC_3A_n77A-n257E | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257E |
| DC_3A_n77A-n257F | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77A-n257F |
| DC_3A_n77C-n257A | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257A |
| DC_3A_n77C-n257D | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257D |
| DC_3A_n77C-n257E | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257E |
| DC_3A_n77C-n257F | DC_3A_n77A DC_3A_n257A DC_3A_n77A-n257A | 3A | CA_n77C-n257F |
| DC_19A_n77A-n257A | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257A |
| DC_19A_n77A-n257D | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257D |
| DC_19A_n77A-n257E | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257E |
| DC_19A_n77A-n257F | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77A-n257F |
| DC_19A_n77C-n257A | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257A |
| DC_19A_n77C-n257D | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257D |
| DC_19A_n77C-n257E | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257E |
| DC_19A_n77C-n257F | DC_19A_n77A DC_19A_n257A DC_19A_n77A-n257A | 19A | CA_n77C-n257F |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. |  |  |  |

# 6	Transmitter characteristics

## 6.1	General

Detailed structure of the subclause is TBD.

Unless otherwise stated the transmitter characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with LTE connecting to the network by OTA without calibration.

## 6.2	Transmitter power

## 6.2A	Transmitter power for CA

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.1	Inter-band CA between FR1 and FR2

<Editor’s notes: errors to be updated.>

For inter-band NR CA in FR1, the UE Power Classes in Table 6.2A.1.1-1 define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2A.1.1-1: Maximum output power for inter-band NR CA

| CA configuration |  | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- |

<Editor’s notes: chapter numbers to be updated.>

For inter-band NR CA in FR1 and FR2 combined, the UE shall meet each transmitter power requirement specified in clause 6.2.1 of TS 38.101-1 and clause 6.2.1 TS 38.101-2 independently.

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	Inter-band CA between FR1 and FR2

### 6.2A.3	UE additional maximum output power reduction for CA

### 6.2A.4	Configured output power for CA

#### 6.2A.4.1	Configured output power level

<Editor’s note: The title of 6.2A.4.1 to be updated by later RAN4 decision>

#### 6.2A.4.2	ΔTIB,c for CA

##### 6.2A.4.2.1	ΔTIB,c for Inter-band CA between FR1 and FR2

ΔTIB,c for NR CA For the UE which supports inter-band NR CA configuration, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2A.4.2-1: ΔTIB,c due to NR CA (two bands)

| Inter-band EN-DC configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |

## 6.2B	Transmitter power for DC

### 6.2B.1	UE maximum output power for EN-DC

#### 6.2B.1.1	Intra-band contiguous EN-DC

< conducted requirements >

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71B |  |  | 23 | +2/-3 |
| DC_(n)41AA | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |

#### 6.2B.1.2	Intra-band non-contiguous EN-DC

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3A_n3A(2) |  |  | 23 | +2/-3 |
| DC_41A_n41A | 26 | +2/-21 | 23 | +2/-21 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Only single switched UL is supported in Rel.15 |  |  |  |  |

#### 6.2B.1.3	Inter-band EN-DC within FR1

< conducted requirements >

For inter-band EN-DC of LTE and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration |  | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- |
| DC_1A_n28A |  | 23 | +2/-3 |
| DC_1A_n40A |  | 23 | +2/-3 |
| DC_1A_n51A |  | 23 | +2/-3 |
| DC_1A_n77A |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A |  | 23 | +2/-3 |
| DC_1A_n79A |  | 23 | +2/-3 |
| DC_2A_n5A |  | 23 | +2/-3 |
| DC_2A_n66A |  | 23 | +2/-3 |
| DC_2A_n71A |  | 23 | +2/-3 |
| DC_2A_n78A |  | 23 | +2/-3 |
| DC_3A_n7A |  | 23 | +2/-3 |
| DC_3A_n28A |  | 23 | +2/-3 |
| DC_3A_n40A |  | 23 | +2/-3 |
| DC_3A_n51A |  | 23 | +2/-3 |
| DC_3A_n77A |  | 23 | +2/-3 |
| DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A, DC_3A_n80A_ULSUP-FDM_n78A |  | 23 | +2/-3 |
| DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A |  | 23 | +2/-3 |
| DC_3A_n82A |  | 23 | +2/-3 |
| DC_5A_n40A |  | 23 | +2/-3 |
| DC_5A_n66A |  | 23 | +2/-3 |
| DC_5A_n78A |  | 23 | +2/-3 |
| DC_7A_n28A |  | 23 | +2/-3 |
| DC_7A_n51A |  | 23 | +2/-3 |
| DC_7A_n78A |  | 23 | +2/-3 |
| DC_8A_n40A |  | 23 | +2/-3 |
| DC_8A_n77A |  | 23 | +2/-3 |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A |  | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A |  | 23 | +2/-3 |
| DC_11A_n77A |  | 23 | +2/-3 |
| DC_11A_n78A |  | 23 | +2/-3 |
| DC_11A_n79A |  | 23 | +2/-3 |
| DC_12A_n5A |  | 23 | +2/-3 |
| DC_12A_n66A |  | 23 | +2/-3 |
| DC_18A_n77A |  | 23 | +2/-3 |
| DC_18A_n78A |  | 23 | +2/-3 |
| DC_18A_n79A |  | 23 | +2/-3 |
| DC_19A_n77A |  | 23 | +2/-3 |
| DC_19A_n78A |  | 23 | +2/-3 |
| DC_19A_n79A |  | 23 | +2/-3 |
| DC_20A_n8A |  | 23 | +2/-3 |
| DC_20A_n28A DC_20A_n83A |  | 23 | +2/-3 |
| DC_20A_n51A |  | 23 | +2/-3 |
| DC_20A_n77A |  | 23 | +2/-3 |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A |  | 23 | +2/-3 |
| DC_21A_n77A |  | 23 | +2/-3 |
| DC_21A_n78A |  | 23 | +2/-3 |
| DC_21A_n79A |  | 23 | +2/-3 |
| DC_25A_n41A |  | 23 | +2/-3 |
| DC_26A_n41A |  | 23 | +2/-3 |
| DC_26A_n77A |  | 23 | +2/-3 |
| DC_26A_n78A |  | 23 | +2/-3 |
| DC_26A_n79A |  | 23 | +2/-3 |
| DC_28A n51A |  | 23 | +2/-3 |
| DC_28A_n77A |  | 23 | +2/-3 |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A |  | 23 | +2/-3 |
| DC_28A_n79A |  | 23 | +2/-3 |
| DC_30A_n5A |  | 23 | +2/-3 |
| DC_30A_n66A |  | 23 | +2/-3 |
| DC_38A_n78A |  | N/A | N/A |
| DC_39A_n78A |  | 23 | +2/-3 |
| DC_39A_n79A |  | 23 | +2/-3 |
| DC_40A_n77A |  | N/A | N/A |
| DC_41A_n77A |  | 23 | +2/-3 |
| DC_41A_n78A |  | 23 | +2/-3 |
| DC_41A_n79A |  | 23 | +2/-3 |
| DC_42A_n51A |  | 23 | +2/-3 |
| DC_42A_n77A |  | N/A | N/A |
| DC_42A_n78A |  | N/A | N/A |
| DC_42A_n79A |  | 23 | +2/-3 |
| DC_66A _n5A |  | 23 | +2/-3 |
| DC_66A _n71A |  | 23 | +2/-3 |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A |  | 23 | +2/-3 |

#### 6.2B.1.4	Inter-band EN-DC including FR2

< OTA requirements >

<Editor’s notes: chapter numbers to be updated.>

For inter-band EN-DC of LTE and NR in FR2, the UE shall meet each transmitter power requirement specified in clause 6.2.2 of TS 36.101 and clause 6.2.1 TS 38.101-2 independently.

#### 6.2B.1.5	Inter-band EN-DC including both FR1 and FR2

< OTA requirements >

<Editor’s notes: chapter numbers to be updated.>

For inter-band EN-DC of LTE and NR in both FR1 and FR2, the UE shall meet each transmitter power requirement for inter-band EN-DC of LTE and NR in FR1specified in clause 6.2B.1.3 of TS 38.101-3 and for NR in FR2 clause 6.2.1 of TS 38.101-2 independently.

### 6.2B.2	UE maximum output power reduction and EN-DC

#### 6.2B.2.1	Intra-band contiguous EN-DC

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

#### 6.2B.2.3	Inter-band EN-DC within FR1

< conducted requirements >

#### 6.2B.2.4	Inter-band EN-DC including FR2

< OTA requirements >

#### 6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

< OTA requirements >

### 6.2B.3	UE additional maximum output power reduction for EN-DC

#### 6.2B.3.1	Intra-band contiguous EN-DC

For EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.1-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_(n)71B | 6.5B.2.1.2.1 | NS_35 | NS_35 | 6.2B.3.1.1 |
| DC_(n)41AA1 | 6.5B.2.1.2.2 | NS_04 | NS_04 | 6.2B.3.1.2 |
| Note 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. Note 2: The network signalling value for NR is mapped to configured FBI and AdditionalSpecrumEmission values as specified in [4]. |  |  |  |  |

##### 6.2B.3.1.1	A-MPR for DC_(n)71B

For DC_(n)71B with configured with network signaling values as per Table 6.2B.3.1.1-1 the allowed A-MPR is defined by

- for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

AMPRDC=CEILMA,DC(A), 0.5

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	10.00 - 11.67*A;		0.00 < A ≤ 0.30

7.10 -  2.00*A;					0.30 < A ≤ 0.80

5.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	10.00 - 13.33*A;  	0.00 < A ≤ 0.30

7.00 - 3.33*A;   					0.30 < A ≤ 0.60

5.00;								0.60 < A ≤ 1.00

where

A=LCRB,LTE+LCRB,NRNRB,LTE+NRB,NR

with LCRB and NRB the number of allocated PRB and transmission bandwidth for the respective CG,

- for UE not indicating support of dynamicPowerSharing

AMPRLTE=CEIL{MA,LTE, 0.5}

AMPRNR=CEIL{MA,NR, 0.5}

where A-MPR is the total power reduction allowed per CG with

MA,LTE=MA,DC(ALTE,wc)-LTE

MA,NR=MA,DC(ANR,wc)-NR

ALTE,wc=LCRB,LTE+1NRB,LTE+NRB,NR

ANR,wc=1+LCRB,NRNRB,LTE+NRB,NR

∆LTE=10 log10LCRB,LTELCRB,LTE+NRB,NR

∆NR=10 log10LCRB,NRNRB,LTE+LCRB,NR

where ÑRB,NR is the transmission bandwidth configuration of the SCG channel for SCS = 15 kHz.

##### 6.2B.3.1.2	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Channel Configuration Case B. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

Else

Channel Configuration Case A. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

where

-	FIM3,low_block,low = (2 * Flow_channel,low_edge) – Fhigh_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

A-MPRLTE = MAX( A-MPRsingle,LTE, A-MPRIM3 )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3 )

For UEs supporting dynamic power sharing , with backoff applied equally to LTE and NR

A-MPR = MAX(A-MPRsingle,LTE, A-MPRsingle,NR, A-MPRIM3 )

where

-	A-MPRsingle,LTE is the A-MPR defined for the LTE transmission in [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2]

6.2B.3.1.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz

For the UE is configured with channel configurations Case A or Case C, the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3,-13 = MA

Where MA is defined as follows

MA = 	14	;	 0 ≤ A < 0.5

9	;	 0.5 ≤ A < 1.0

7	; 	1.0 ≤ A < 2.0

5	; 	2.0 < A

Where:

For UEs supporting dynamic power sharing,

A = NRB_alloc,LTE * 12* SCSLTE + NRB_alloc,NR * 12 * SCSNR

For UEs not supporting dynamic power sharing,

For LTE

A = NRB_alloc,LTE * 12* SCSLTE + 12 * SCSNR

For NR

A = 12* SCSLTE + NRB_alloc,NR * 12 * SCSNR

6.2B.3.1.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz

For the UE is configured with channel configurations Case B or Case D, the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3,-13 = MA

Where MA is defined as follows

MA = 	14	; 	0 ≤ A < 1.0

13	; 	1.0 ≤ A < 2.0

12	; 	2.0 ≤ A < 5.0

11	; 	5.0 < A

Where:

For UEs supporting dynamic power sharing,

A = NRB_alloc,LTE * 12* SCSLTE + NRB_alloc,NR * 12 * SCSNR

For UEs not supporting dynamic power sharing,

For LTE

A = NRB_alloc,LTE * 12* SCSLTE + 12 * SCSNR

For NR

A = 12 * SCSLTE + NRB_alloc,NR * 12 * SCSNR

< any intermediate sub-clauses omitted >

#### 6.2B.3.2	Intra-band non-contiguous EN-DC

For intra-band EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.1-1 for combinations of network signalling values indicated in E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR allowed below is in addition to the MPR requirements specified in sub-clause 6.2B.2.

Table 6.2B.3.1-1: Allowed power reduction for EN-DC

| DC configuration | Requirement (sub-clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (subclause) |
| --- | --- | --- | --- | --- |
| DC_41A_n411 | 6.6.3.3.19 and 6.6.2.2.2 of [4] and 6.5.2.3.2 and 6.5.3.3.1 of [2] | NS_04 | NS_04 | 6.2B.3.2.2 |
| Note 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. |  |  |  |  |

##### 6.2B.3.2.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this subclause.

The UE determines the Channel Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Channel Configuration Case C. A-MPRIM3 defined in Subclause 6.2B.3.1.2.1

Else

Channel Configuration Case D. A-MPRIM3 defined in Subclause 6.2B.3.1.2.2

where

-	FIM3,low_block,high = (2 * Flow_channel,high_edge ) – Fhigh_channel,low_edge

-	FIM3,high_block,low = (2 * Fhigh_channel,low_edge) – Flow_channel,high_edge

-	Flow_channel,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Flow_channel,high_edge is the uppermost frequency of lower transmission bandwidth configuration.

-	Fhigh_channel,low_edge is the lowermost frequency of upper transmission bandwidth configuration.

-	Fhigh_channel,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2750 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Subclause 6.5B.2.1.2.2.

The UE determines the value of A-MPRACLRoverlap as follows:

If  Wgap < BWchannel,LTE + BWchannel,NR

A-MPRACLRoverlap = 4 dB

Else

A-MPRACLRoverlap = 0 dB

where

-	Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge

The UE determines the total allowed maximum output power reduction as follows:

For UEs not supporting dynamic power sharing, with backoff applied independently

A-MPRLTE = MAX( A-MPRsingle,LTE, A-MPRIM3, A-MPRACLRoverlap )

A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap )

For UEs supporting dynamic power sharing , with backoff applied equally to LTE and NR

A-MPR = MAX(A-MPRsingle,LTE, A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap )

where

-	A-MPRsingle,LTE is the A-MPR defined for the LTE transmission in [4].

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in [2].

#### 6.2B.3.3	Inter-band EN-DC within FR1

#### 6.2B.3.4	Inter-band EN-DC including FR2

#### 6.2B.3.5	Inter-band EN-DC including both FR1 and FR2

### 6.2B.4	Configured output power for EN-DC

#### 6.2B.4.1	Configured output power level

<Editor’s note: The title of 6.2B.4.1 to be updated by later RAN4 decision>

##### 6.2B.4.1.1	Intra-band contiguous EN-DC

< equations for Pcmax >

##### 6.2B.4.1.2	Intra-band non-contiguous EN-DC

< equations for Pcmax >

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

< equations for Pcmax >

##### 6.2B.4.1.4	Inter-band EN-DC including FR2

< equations for Pcmax >

##### 6.2B.4.1.5	Inter-band EN-DC including both FR1 and FR2

< equations for Pcmax >

#### 6.2B.4.2	ΔTIB,c for EN-DC

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in Tables below applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

##### 6.2B.4.2.1	Intra-band contiguous EN-DC

##### 6.2B.4.2.2	Intra-band non-contiguous EN-DC

##### 6.2B.4.2.3	Inter-band EN-DC within FR1

###### 6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2A_n78A | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_5A_n40A | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5A_n66A | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12A_n5A | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12A_n66A | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n77 | 0.8 |
| DC_25_n41 | 25 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26A_n77A | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30A_n5A | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30A_n66A | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n77 | n77 | 0.5 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

###### 6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-1A_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-18_n78 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-28_n77 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n78 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_SUL_n78-n84 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n84 | 0.3 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_2-(n)71B | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_2-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
| DC_2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2-66_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
| DC_3_n3-n77 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n3-n78 | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |
| DC_3-7_n78, DC_3-7-7_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-19_n79 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-20_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-21_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_3A_n28-n78 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n78 | 0.8 |
| DC_3-38_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3-41_n78 | 3 | 0.6 |
|  | 41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| DC_3-42_n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n787 | 0.8 |
| DC_3-42_n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-42_n79 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n79 | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_3_n78-n79 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_3_SUL_n78-n80 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |
| DC_3A_SUL_n78A-n82A | 3 | 0.5 |
|  | n78 | 0.8 |
|  | n82 | 0.3 |
| DC_5-7_n78, DC_5-7-7_n78 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_5_30_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_7-20_n28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_7-20_n78 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_7-28_n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |
| DC_7-46_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_SUL_n78- n81 | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n81 | 0.6 |
| DC_18-28_n77 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_18-28_n78 | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_18-28_n79 | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_19-21_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-21_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
| DC_19-42_n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-42_n79 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n79 | 19 | 0.3 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_19_n78-n79 | 19 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_20_n8-n75 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28-n75 | 20 | 0.5 |
|  | n28 | 0.7 |
| DC_20_n28-n78 | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_20_n75-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20_n76-n78 | 20 | 0.5 |
|  | n78 | 0.8 |
| DC_20A_SUL_n78A-n82A | 20 | 0.6 |
|  | n78 | 0.8 |
|  | n82 | 0.6 |
| DC_20A_SUL_n78A-n83A | 20 | 0.8 |
|  | n78 | 0.8 |
|  | n83 | 0.8 |
| DC_21-42_n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-42_n79 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n79 | 21 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0 |
| DC_21_n78-n79 | 21 | 0.4 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_28-42_n77 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_28-42_n78 | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_28-42_n79 | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_28_SUL_n78-n83 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n83 | 0.5 |
| DC_41-42_n77 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_41-42_n78 | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_41-42_n79 | 41 | 0. |
|  | 42 | 0.8 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_66_(n)71 | 66 | 0.3 |
|  | 71 | 0.3 |
|  | n71 | 0.3 |
| DC_66_SUL_n78-n86 | 66 | 0.6 |
|  | n78 | 0.8 |
|  | n86 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

###### 6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-7_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-8_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3-28_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
| DC_1-3-19_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-19_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_1-3-20_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_1-3-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-20_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7-20_n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18-28_n77 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_1-18-28_n78 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_1-18-28_n79 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-20_n28-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-21-28_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| DC_1-21-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-28-42_n77 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-28-42_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-28-42_n79 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-41-42_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-41-42_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-41-42_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| DC_2-66-(n)71B | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_3-5-7_n78, DC_3-5-7-7_n78A | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-20_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n28 | 0.5 |
| DC_3-7-20_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-7-28_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19-21_n77 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-19-21_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-19-21_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_3-19-42_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-19-42_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-19-42_n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_3-20_n28-n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-28-42_n77 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-28-42_n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-28-42_n79 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_3-21-42_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-21-42_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-21-42_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_7-20_n28-n78 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_19-21-42_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-21-42_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-21-42_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21-28-42_n77 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-28-42_n78 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-28-42_n79 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |

###### 6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.6 |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-19-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_1-3-19-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-19-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-19-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-20_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-21-42C_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.6 |
| DC_1-3-21-42C_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.6 |
| DC_1-3-21-42C_n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n79 | 0 |
| DC_1-3-28-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-28-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-28-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-7-20_n28-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-21-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-21-28-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-28-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-28-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_3-7-20_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

###### 6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

##### 6.2B.4.2.4	Inter-band EN-DC including FR2

###### 6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.4.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |

###### 6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.4.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n257 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1-7-7_n257 | 1 | 0.5 |
|  | 7 | 0.6 |
| DC_1-8_n257 | 1 | 0.3 |
|  | 8 | 0.3 |
| DC_1-18_n257 | 1 | 0.3 |
|  | 18 | 0.3 |
| DC_1-19_n257 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-21_n257 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-28_n257 | 1 | 0.3 |
|  | 28 | 0.6 |
| DC_1-41_n257 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-42_n257 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_n77-n257 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78-n257 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_1_n79-n257 | 1 | 0 |
|  | n79 | 0 |
| DC_2-5_n257 | 2 | 0.3 |
|  | 5 | 0.3 |
| DC_2-5_n260 | 2 | 0.3 |
|  | 5 | 0.3 |
| DC_2-12_n260 | 2 | 0.3 |
|  | 12 | 0.3 |
| DC_2-13_n257 | 2 | 0.3 |
|  | 13 | 0.3 |
| DC_2-13_n260 | 2 | 0.3 |
|  | 13 | 0.3 |
| DC_2-30_n260 | 2 | 0.5 |
|  | 30 | 0.5 |
| DC_2-66_n257 | 2 | 0.5 |
|  | 66 | 0.5 |
| DC_2-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
| DC_3-19_n257 | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_3-21_n257 | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_3-28_n257 | 3 | 0.3 |
|  | 28 | 0.3 |
| DC_3-41_n257 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
| DC_3-42_n257 | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_3_n77-n257 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n79-n257 | 3 | 0 |
|  | n79 | 0 |
| DC_5-30_n260 | 5 | 0.5 |
|  | 30 | 0.5 |
| DC_5-66_n257 | 5 | 0.3 |
|  | 66 | 0.3 |
| DC_5-66_n260 | 5 | 0.3 |
|  | 66 | 0.3 |
| DC_5_n78-n257 | 5 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| DC_7_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n257 | 0 |
| DC_12-30_n260 | 12 | 0.5 |
|  | 30 | 0.5 |
| DC_12-66_n260 | 12 | 0.3 |
|  | 66 | 0.3 |
| DC 13-66_n260 | 13 | 0.3 |
|  | 66 | 0.3 |
| DC_18-28_n257 | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_19-21_n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_19-42_n257 | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_19_n77-n257 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78-n257 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n79-n257 | 19 | 0 |
|  | n79 | 0 |
| DC_21-28_n257 | 21 | 0.4 |
|  | 28 | 0.3 |
| DC_21-42_n257 | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_21_n77-n257 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78-n257 | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_21_n79-n257 | 21 | 0 |
|  | n79 | 0 |
| DC_28-42_n257 | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_41-42_n257 | 41 | 0.5 |
|  | 42 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

###### 6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1A-3A-7A-7A_n257A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| DC_1-3-19_n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_1-3-21_n257 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_1-3-28_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
| DC_1-3-42_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_1-3_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5-7-7_n257 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| DC_1-5_n78-n257 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18-28_n257 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_1-19-42_n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-21-28_n257 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| DC_1-21-42_n257 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-28-42_n257 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-41-42_n257 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| DC_19-21-42_n257 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_3-5_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19-21_n257 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_3-19-42_n257 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_3-21-42_n257 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_3-28-42_n257 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_5-7_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_7-7_n78-n257 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_21-28-42_n257 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |

###### 6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.4.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7-7_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| DC_1-3-5_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n257 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_1-3-19-42_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-21-42C_n257 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_1-3-28-42_n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-5-7_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-7_n78-n257 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n257 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-21-28-42_n257 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_3-5-7_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-7_n78-n257 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_5-7-7_n78-n257 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

###### 6.2B.4.2.4.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.4.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78-n257 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7-7_n78-n257 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-5-7-7_n78-n257 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5-7-7_n78-n257 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

##### 6.2B.4.2.5	Inter-band EN-DC including both FR1 an FR2

###### 6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.5.1-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n77-n257 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78-n257 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_3_n77-n257 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78-n257 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_19_n77-n257 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78-n257 | 19 | 0.3 |
|  | n78 | 0.8 |

## 6.3	Output power dynamics

Output power dynamics for CA operations in FR1 and FR2 as specified in 38.101-1 and 38.101-2, respectively.

Output power dynamics for EN-DC operations in FR1 and FR2 as specified in 38.101-1 and 38.101-2, respectively. E-UTRA as specified in 36.101. For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in sub-clause 6.3.1 of 38.101-1 and sub-clause 6.3.2 of 36.101 shall only apply when the power of all NR and E-UTRA carriers are set to minimum value.  Similarly, OFF power requirements specified in sub-clause 6.3.2 of 38.101-1 and sub-clause 6.3.3 of 36.101 shall only apply when the power of all NR and E-UTRA carriers are OFF.  The OFF power condition in transmit ON/OFF time mask requirements specified in sub-clause 6.3.3 of 38.101-1 and sub-clause 6.3.4 of 36.101 is applicable only when all NR and E-UTRA carriers are OFF.  If both E-UTRA and NR transition between ON and OFF states simultaneously, the longer transient time shall apply to both.  If either E-UTRA or NR is OFF and the other carrier transitions from OFF to ON, then the transiet time associated with that carrier applies.

6.3B	Output power dynamics for DC

### 6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

#### 6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective

The E-UTRA and NR switching time mask is only applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective over the shared bandwidth of a carrier.

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time <0.5us, time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time <20us, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective

### 6.3B.2	Output power dynamics for intra-band non-contigious switching time

For DC_3A_n3A single switched UL operation in Rel.15, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching.

## 6.4	Transmit signal quality

Transmit signal quality for CA operations in FR1 and FR2 as specified in 38.101-1 and 38.101-2, respectively.

Transmit signal quality for EN-DC operations in FR1 and FR2 as specified in 38.101-1 and 38.101-2, respectively. E-UTRA as specified in 36.101.

## 6.4B	Transmit signal quality for DC

### 6.4B.2	Transmit modulation quality for EN-DC

#### 6.4B.2.1	Intra-band contiguous EN-DC

##### 6.4B.2.1.1	Error Vector Magnitude

For the intra-band contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of [4] for the MCG and 6.4.2 of [2] for the SCG with EN-DC configured.

##### 6.4B.2.1.2	Carrier leakage

The carrier leakage requirements for each CG are according to clause 6.5.2 of [4] for the MCG and 6.4.2 of [2] for the SCG with EN-DC configured.

##### 6.4B.2.1.3	In-band emissions

For the MCG the requirements the in-band emission requirments in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in [4] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the MCG at the edge of the said aggregated transmission bandwidth configuration.

For the SCG the requirements the in-band emission requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in [4] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth  within the SCG at the edge of the aggregated transmission bandwidth configuration.

## 6.5	Output RF spectrum emissions

## 6.5A	Output RF spectrum emissions for CA

<Editor’s note: carrier aggregation of bands FR1 and FR2>

### 6.5A.1	Occupied bandwidth for CA

### 6.5A.2	Out-of-band emissions for CA

### 6.5A.3	Spurious emissions for CA

#### 6.5A.3.1	Inter-band CA between FR1 and FR2

Detailed structure of the subclause is TBD.

Table 6.5A.X.X-1: Requirements for uplink inter-band CA (two bands)

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8A-n258A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41,42,n78,n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 6 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 6 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
| CA_n77A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
| CA_n78A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
| CA_n79A-n257A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 26, 21, 28, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1/2 or Table 5.5 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 in TS 38.101-1 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: 15KHz SCS is assumed when RB is mentioned in the note. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. |  |  |  |  |  |  |  |

Note:	To simplify above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

## 6.5B	Output RF spectrum emissions for DC

### 6.5B.1	Occupied bandwidth for EN-DC

For intra-band contiguous EN-DC the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum.  The OBW shall be less than the aggregated channel bandwidth for EN-DC, denoted as ENBW in sub-clause 5.3B.

### 6.5B.2	Out-of-band emissions for EN-DC

#### 6.5B.2.1	Intra-band contiguous EN-DC

Unless otherwise stated, the OOBE limits specified for the DC combination in this sub-clause supercede any OOBE requirements specified for each sub-block in the respective TS [4] and [2].

The requirements apply to the sum of transmissions across all antenna connectors.

##### 6.5B.2.1.1	Spectrum emissions mask

The general spectrum emission for intra-band contiguous EN-DC is specified in Table 6.5B.2.1.1-1.

Table 6.5B.2.1.1-1.  General spectrum emission mask for intra-band contiguous EN-DC

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0 - 1 | Max(Round(10*log(0.15/ENBW)),-24) | 30 kHz |
| ± 1 - 5 | -10 | 1 MHz |
| ± 5 - ENBW | -13 | 1 MHz |
| ± ENBW – (ENBW+5) | -25 | 1 MHz |
| Note: ENBW refers to the aggregated channel bandwidth in MHz as defined in sub-clause 5.3B. |  |  |

##### 6.5B.2.1.2	Additional spectrum emissions mask

###### 6.5B.2.1.2.1	Requirements for network signalled value "NS_35"

When NS_35 is indicated in the MCG and NS_35 is indicated in the SCG, the requirements in Table 6.5B.2.1.2.1-1 apply in the frequency ranges immediately adjacent and outside the aggregated sub-blocks of the EN-DC configuration for DC_(n)71B.

Table 6.5B.2.1.2.1-1: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

###### 6.5B.2.1.2.2	Requirements for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The Band 41/n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power.  Since the 26 dB emission bandwidth is implementation dependent, the transmission bandwidths occupied by RBs is used for the SEM. The emission bandwidth for LTE carriers is document in 36.101 [4], and the emission bandwidth for NR carriers is documented in 38.101-1 [2]. The total emission bandwidth for contiguous intra-band EN-DC is the sum of the emission bandwidth for each CC plus the guard band between contiguous CCs.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.2.2-1.

Table 6.5B.2.1.2.2-1: n41 SEM with NS_04

|  | Spectrum emission limit (dBm)/ measurement bandwidth for each channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 | -18 | -20 | -21 | -24 | -25 |  | 30 kHz |
| ± 1 - 5 | -10 |  |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) | -25 |  |  |  |  |  |  |
| Note 1: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |  |

##### 6.5B.2.1.3	Adjacent channel leakage ratio

For EN-DC operation with an E-UTRA sub-block immediately adjacent to an NR sub-block, the ACLR is defined as the ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW at nominal channel spacing. The UE shall meet the ACLR minimum requirement EN-DCACLR specified in Table 6.5B.2.1.3-1 with ENBW the sum of the sub-block bandwidths.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in 6.5B.2.1.3-1.

Table 6.5B.2.1.3-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR | dBc | 30 |
| Measurement bandwidth of EN-DC channel |  | 1.00*ENBW |
| Measurement bandwidth of adjacent channel |  | 0.95*ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz as defined in sub-clause 5.3B. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

#### 6.5B.2.2	Intra-band non-contiguous EN-DC

##### 6.5B.2.2.1	Spectrum emissions mask

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used.

##### 6.5B.2.2.2	Additional spectrum emissions mask

When additional spectrum emission mask or masks apply, the additional SEM(s) shall be used to calculate the composite SEM described in 6.5B.2.2.1.

##### 6.5B.2.2.3	Adjacent channel leakage ratio

For intra-band non-contiguous EN-DC when all UL sub-blocks consist of one component carrier the EN-DC Adjacent Channel Leakage power Ratio (EN-DCACLR) is the ratio of the sum of the filtered mean powers centred on the assigned sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than of the sub-block bandwidth than for that sub-block no EN-DCACLR requirement is set for the gap. In case the sub-block gap bandwidth Wgap is smaller than either of the sub-block bandwidths then no EN-DCACLR requirement is set for the gap. The assigned EN-DC sub-block power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in in [4] for the E-UTRA sub-block, and [2],[3] for the NR sub-block. If the measured adjacent channel power is greater than –50dBm then the EN-DCACLR shall be higher than the value specified in for E-UTRAACLR and NRACLR .

#### 6.5B.2.3	Inter-band EN-DC within FR1

Unless otherewise stated, the OOBE requirements specified in sub-clause 6.6.2.1 of [4], sub-clause 6.5.2.2 of [2] and the additional requirments in 6.6.2.2 of [4] and 6.5.2.3 of [2] apply for each component carrier.

The requirements apply to each antenna connector.

#### 6.5B.2.4	Inter-band EN-DC including FR2

#### 6.5B.2.5	Inter-band EN-DC including both FR1 and FR2

### 6.5B.3	Spurious emissions for EN-DC

#### 6.5B.3.1	Intra-band contiguous EN-DC

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4] and sub-clause 6.5.3.1 of [2] apply beyond any frequencies for which the out-of-band emissions requirements in sub-clause 6.5.2 apply.

##### 6.5B.3.1.1	General spurious emissions

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4] and sub-clause 6.5.3.1 of [2] apply beyond any frequencies for which the out-of-band emissions requirements in sub-clause 6.5B.2.1apply.

##### 6.5B.3.1.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.1.2-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.1-1: Requirements for intraband carrier aggregation

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| DC_(n)71B | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_lowFDL_low | -- | FDL_highFDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| … |  |  |  |  |  |  |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 [4] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 6.5B.3.2	Intra-band non-contiguous EN-DC

#### 6.5B.3.3	Inter-band EN-DC within FR1

< conducted requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

##### 6.5B.3.3.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.1-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.3.1-1: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n28A | E-UTRA Band 18, 19, 27, 31, 32, 72 NR band n5, n7, n8, n20, n26, n38, n40, n41, n50, n51, n74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band42, 43 NR band n78, n75, n76 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n3, n34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | E-UTRA Band 65 NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 18 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,17 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 17 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 10, 16 |
| DC_1A_n40A | Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |
| DC_1A_n51A | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5, 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 17 |
|  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A DC_1A_n84A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n79A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_2A_n5A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2A_n66A | Bands 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, n71, 74, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Bands 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2A_n71A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2A_n78A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band 78 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n1, n5, n7, n8, n20, n28, n50, n51, n74, n75, n76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3A_n28A | E-UTRA Band 42, 43, 65 NR band n1, n50, n51, n74, n75, n76, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | NR band n3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 27, 31, 72 NR band n5, n7, n8, n20, n26, n34, n38, n40, n41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10 |
| DC_3A_n40A | Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n51A | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 65, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3A_n77A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A, DC_3A_n80A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n79A DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_3A_n82A | E-UTRA Band 1, 3 7, 8, 20，22, 31, 32, 33, 34, 38, 40, 43, 50, 51, 65, 67, 68, 69, 72,74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5A_n40A | Band 1, 3, 5, 7, 8, 28, 31, 34, 38, 42, 43, 45, 65, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n66A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_7A_n28A | E-UTRA Band 27, 31, 72 NR band n2, n3, n5, n7, n8, n20, n26, n34, n40 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 42, 43, 65 NR band n1, n50, n51, n66, n74, n75, n76, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR band n1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7A_n51A | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 17 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 21 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7A_n78A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
|  | Frequency range | 3800 | - | 3805 | +1.6 | 5 | 5, 7, 8 |
|  | Frequency range | 3805 | - | 3825 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 3825 | - | 3850 | -40 | 1 | 5, 8 |
|  | Frequency range | 3850 | - | 4200 | -50 | 1 |  |
| DC_8A_n40A | Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 7, 22, 41, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | ３, 13 |
| DC_8A_n77A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A, DC_8A_n81A_ULSUP-FDM_n78A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | -5 | 100 |  |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A | E-UTRA Band 1,8,28,34,39,40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3,41,42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
|  | NR Band n258 | 24250 | - | 27500 | -5 | 100 |  |
| DC_11A_n77A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_11A_n78A | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_11A_n79A | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_12A_n5A | Bands 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 4, 10, 41, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_12A_n66A DC_12A_n5A | Bands 2, 4, 5, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 70, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 4, 10, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Bands 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Bands 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, n71, 74, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_18A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_18A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_18A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_19A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_20A_n8A | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n28A DC_20A_n83A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n51A | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20A_n77A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20A_n78A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  |  |  |  |  |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_21A_n77A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_21A_n78A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_21A_n79A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_25A_n41A | NR band n5, n28, n66, n71 E-UTRA/NR Band 4, 10, 12, 13 , 14, 17, 24, 26, 27, 29, 30, 42, 45, 48, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n2 E-UTRA/NR Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | EUTRA/NR Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_26A_n41A | E-UTRA/NR Band 1, 2, 3, 4, 5, 10, 12, 13 , 14, 17, 24, 25, 26, 28, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 20 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26A_n77A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_26A_n78A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_26A_n79A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n51A | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 66, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 18 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28A_n77A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_28A_n79A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_30A_n5A | Bands 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_30A_n66A | Bands 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38A_n78A | N/A |  |  |  |  |  |  |
| DC_39A_n78A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 19 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 19 |
|  | NR Band n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_39A_n79A | E-UTRA Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 19 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 19 |
|  | NR Band n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_40A_n77A | N/A |  |  |  |  |  |  |
| DC_41A_n77A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 20 |
|  | NR Band n257 | 26500 | - | 29500 | -5 | 100 |  |
| DC_41A_n78A | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 or NR Band n1, n8, n34, n40, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_(n)41AA | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 20 |
| DC_41A_n41A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 20 |
| DC_41A_n79A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 or NR Band n1, n3, n8, n28, n34, n40, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | NR Band n257, n258 | FDL_low | - | FDL_high | -5 | 100 |  |
| DC_42A_n51A | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 5, 6, 7, 10, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_42A_n77A | N/A |  |  |  |  |  |  |
| DC_42A_n78A | N/A |  |  |  |  |  |  |
| DC_42A_n79A | N/A |  |  |  |  |  |  |
| DC_66A_n5A | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, n71, 85, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66A_n71A | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP-FDM_n78A | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range TBD – 3792.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range TBD - 3790 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to TBD RB. NOTE 9: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE14: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 15: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 16: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 17: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 18: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 19: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 20: This requirement applies when the E-UTRA and NR carriers are confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz |  |  |  |  |  |  |  |

#### 6.5B.3.4	Inter-band EN-DC including FR2

< OTA requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

##### 6.5B.3.4.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.4.1-1 apply on each component carrier separately.

Table 6.5B.3.4.1-1: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_2A_n257A | Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 66, 70, 71, 74, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 |
|  | Band 43 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2A_n260A | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 42, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n258A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n257A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_5A_n260A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | Band 41 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 |
| DC_5A_n261A | Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | 26 | FDL_low | - | FDL_high | -27 | 1 |  |
|  | 41 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 17 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 21 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7A_n257A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |
| DC_7A_n258A | E-UTRA Band 1, 3, 5, 7, 8, 26, 28, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 |  |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 |  |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 |  |
| DC_8A_n257A | E-UTRA Band 1, 28, 34, 38, 69, 74, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41, 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
| DC_8A_n258A | E-UTRA Band 1,8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7,41,42,n78,n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_11A_n257A | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12A_n260A | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 48, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_13A_n257A | Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 66, 70, 71, 74, n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 |
| DC_18A_n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19A_n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20A_n258A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 38, 42, 52, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_21A_n257A | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26A_n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28A_n257A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28A_n258A | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_30A_n260A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_39A_n258A | Band 1, 8, 34, 40, 41, 44, 45 or NR Band n1, n8, n34, n40, n41, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 19 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 19 |
| DC_41A_n257A | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 26, 28, 33, 34, 39, 40, 44, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1839.9 |  | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_41A_n258A | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_42A_n257A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_48A_n257A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_48A_n260A | Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66A_n257A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 of TS38.101 |
| DC_66A_n260A | Bands 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, n71, n257 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Bands 48 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_66A_n261A | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | Table 6.5.3.2-1 of TS38.101 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range TBD – 3792.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range TBD - 3790 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to TBD RB. NOTE 9: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE14: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 15: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 16: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 17: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 18: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 19: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

#### 6.5B.3.5	Inter-band EN-DC including both FR1 and FR2

<OTA requirements >

< Editor’s note: Chapter numbers to be updated >

The general spurious emissions requirements specified in sub-clause 6.6.3.1 of [4], sub-clause 6.5.3.1 of [2] and [3] apply for each component carrier.

##### 6.5B.3.5.1	Spurious emission band UE co-existence

< Editor’s note: Chapter numbers to be updated >

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.4.1-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.5.1-1: Requirements

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1A_n77A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n78A-n257 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_1A_n79A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 9 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 9 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 9 |
| DC_3A_n77A-n257 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n78A-n257A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n79A-n257A | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_19A_n77A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19A_n78A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19A_n79A-n257A | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 3300 - 3800 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range TBD – 3792.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range TBD - 3790 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to TBD RB. NOTE 9: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE14: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 15: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 16: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 17: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 18: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 19: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

### 6.5B.4	Additional spurious emissions

#### 6.5B.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5B.4.1.1	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5B.4.1.1 -1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW for contiguous BW up to 100 MHz, 1 MHz for contiguous BW > 100 MHz |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

# 7	Receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with LTE connecting to the network by OTA without calibration.

The requirements defined in this clause are the extra requirements compared with the single carrier requirements defined in [2] and [3].

Unless otherwise stated, the UL and DL reference measurement channels are the same with the configurations specified in [2] and [3].

## 7.2	Diversity characteristics

## 7.3	Reference sensitivity

## 7.3A	Reference sensitivity for  CA

### 7.3A.1	General

<Editor’s note: Table number to be updated>

For NR CA operation NR single carrier REFSENS requirements defined in [2] and [3] apply to all downlink bands part of NR CA configurations listed in Tables 5.2.2.1-1 unless sensitivity degradation is allowed as defined in clause 7.3A.

### 7.3A.2	Reference sensitivity power level for CA

### 7.3A.3	ΔRIB,c for CA

<Editor’s note: Chapter number to be updated>

For the UE which supports inter-band NR CA configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in TS 36.101, Table 7.3-1 in TS 38.101-1 and Table 7.3.1-1 in TS 38.101-2 shall be increased by the amount given in ΔRIB,c in Tables below. Unless otherwise stated, ΔRIB,c is set to zero.

#### 7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2

Table 7.3A.3.1-1: ΔRIB,c due to NR CA (two bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for a band in frequency range 2 if it is impacted by UL harmonic interference from the band in frequency range 1 of the same CA configuration. Reference sensitivity exceptions are specified in Table 7.3A.4-1 with uplink configuration specified in Table 7.3A.4-2.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA of FR1+FR2

| UL Band | DL Band | 50 MHz (dBm) | 100 MHz (dBm) | 200 MHz (dBm) | 400 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |

Table 7.3A.2-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA of FR1+FR2

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |  |

## 7.3B	Reference sensitivity level for DC

### 7.3B.1	General

<Editor’s note: Table number to be updated>

For EN-DC, E-UTRA and NR single carrier REFSENS requirements defined in [2], [3] and [4] apply to all downlink bands of EN-DC configurations listed in TablesTBD unless sensitivity degradation is allowed as defined in clause 7.3.2.1 and 7.3.2.2.

### 7.3B.2	Reference sensitivity for EN-DC

#### 7.3B.2.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels.

For each CG, the reference sensitivity is specified as a maximum allowed degradation MSD of the reference sensitivity level as specified for the applicable carrier bandwidths in accordance with [4] for the E-UTRA CG and [2] for the NR CG.

For DC configurations of DC bandwidth class B, the throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in TBD with parameters specified in Table 7.3B.2.1-1.

Table 7.3B.2.1-1: Reference sensitivity (MSD) for intra-band DC bandwidth class

| MSD / DC bandwidth class B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71B | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71B | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71B | 71 | 668 | 10 | 10 (RBend = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 51) | 632 | 29.4 |  |

#### 7.3B.2.2	Intra-band non-contiguous EN-DC

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in rel.15, no MSD is required.

#### 7.3B.2.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

<Editor’s note: FFS how to clarify the issues of 1Tx may also exist for 2Tx mode, for example harmonic, etc.>

##### 7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3B.2.3.1-1 with uplink configuration specified in Table 7.3B.2.3.1-2.

Table 7.3B.2.3.1-1: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n771,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 |  |  |  |  |  |
|  | n773 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 2 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 |  |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 3 | n781,2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 |  |  |  |  |  |
|  | n783 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 8 | n776,7 n786,7 | NA | 10.8 | 9.1 | 8 | 5.1 | 4.2 | 3.5 | 2.3 | 1.4 |  |  |  |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 | 4.9 |  | 4.4 |
| 18， 19 | n774,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 |  | 0.7 |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 |  |  |  |  |  |
| 26 | n41 | NA | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 | 3.1 | 2.7 |  |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 |  |  |  |  |  |
| 26 | n774,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 |  | 0.7 |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |
|  | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |
| 66 | n781,2 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 |  |  |  |  |  |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 1 | n77 | 12 | 25 | 36 | 50 |  |  | 100 |  |  |  |  |  |
| 2 | n78 | 12 | 26 | 39 | 531 1002 |  |  |  |  |  |  |  |  |
| 3 | n77 | 12 | 25 | 36 | 50 |  |  | 50 |  |  |  |  |  |
| 3 | n78 | 12 | 25 | 36 | 50 |  |  | 50 |  |  |  |  |  |
| 8 | n77 n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 | 25 |  | 25 |
| 18 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |  |  |
| 19 | n77 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 |  | 25 |
| 20 | n77 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |  |  |
| 20 | n78 |  | 12 | 18 | 20 |  |  | 20 |  |  |  |  |  |
| 26 | n77 n78 | 8 | 16 | 25 | 251, 252 |  |  |  |  |  |  |  |  |
| n28 | 1 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| n28 | n75 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |
| 28 | n77 n78 |  | 10 | -15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 66 | n78 |  | 26 | 39 | 53 |  |  | 100 |  |  |  |  |  |
| n71 | 2 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |
| NOTE 1: The configuration is used for measurement of MSD for NR channel bandwidth of 20MHz. NOTE 2: The configuration is used for measurement of MSD for NR channel bandwidth of 40MHz. NOTE 3: The RB allocation is at the lower edge of the lowest channel of UL band. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.2	MSD due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3B.2.3.2-1 with uplink configuration specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 26 | n414 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| 41 | n777 | N/A | 8.3 | 8.0 | 6.9 | N/A | 3.9 | 3 | 2.3 | 1.2 | 0.4 |  |
| 41 | n787 | N/A | 8.3 | 8.0 | 6.9 | N/A | 3.9 | 3 | 2.3 | 1.2 | 0.4 |  |
| n71 | 25 | 4.6 | 1 | 0.7 | 0.6 |  |  |  |  |  |  |  |
|  | 26 | 1.7 | 1 | 0.7 | 0.6 |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 213 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 | 25 | 25 | 25 | 25 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| 41 | n78 | 15 | 25 | 25 | 25 | 25 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 41 | 30 | N/A | 50 | 50 | 50 | N/A | 50 | 50 | 50 | 50 | 50 |  |
| n78 | 41 | 30 | N/A | 50 | 50 | 50 | N/A | 50 | 50 | 50 | 50 | 50 |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.3	Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by close proximity of an UL of another band part of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3B.2.3.3-1 with uplink configuration specified in Table 7.3B.2.3.3-2.

Table 7.3B.2.3.3-1: Reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3-2: Uplink configuration for reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same DC configuration due to cross band isolation issues. Reference sensitivity exceptions are specified in Table 7.3B.2.3.4-1 with uplink configuration specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 411 | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
| n78 | 411 | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n77 | 41 | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A |
| n78 | 41 | 30 | 273 | 273 | 273 | 273 | N/A | N/A | N/A | N/A | N/A | N/A |

##### 7.3B.2.3.5	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency with its own downlink transmissions if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when the operating bands of the configuration are either confined below 1 GHz or confined within the frequency range 1695 MHz – 2690 MHz.

In case for the EN-DC in NR FR1 configurations the intermodulation products caused by dual uplink operation do not interfere with the own downlink transmission as defined in Annex-A the UE is mandated to operate in dual uplink mode.

For EN-DC in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in  [4] and 7.3.2.1 of [2] for the corresponding channel bandwidths or in clause 7.3.1 of [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in … with parameters specified in Table 7.3B.2.3.5-1 with dual UL transmissions overlapping in time unless otherwise stated.

For EN-DC configurations in Table 7.3B.2.3.5-1 with UL/DL channel assignments such that Single UL is allowed, the MSD requirement is verified with non-simultaneous uplink transmissions on the two CGs for UEs only indicating support of Single UL.

###### 7.3B.2.3.5.1	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD23 |  |
|  |  |  |  |  |  | 32.54 |  |  |  |
|  | n77 | 4090 | 10 | 25 | 4090 | N/A | TDD | N/A |  |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD |  |  |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |  |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | TDD | N/A |  |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |  |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | TDD | IMD5 |  |
| DC_2A_n78A | 2 | 1855 | 5 | 25 | 1940 | 26 | FDD | IMD23 |  |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3795 | 10 | 25 | 3795 | N/A | TDD | N/A |  |
| DC_2A_n78A | 2 | 1885 | 5 | 25 | 1955 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3700 | 10 | 25 | 3700 | N/A | TDD | N/A |  |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |  |
|  | n7 | 2535 | 10 | 52 | 2655 | 10.25 | FDD | IMD4 |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 |  |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n77, n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
| DC_3A_n77A DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n77, n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_3A_n78A | 3 | 1712.5 | 5 | 25 | 1807.5 | TBD5 | FDD | IMD2 | Yes |
|  | n78 | 3515 | 10 | 50 | 3515 | N/A | TDD | N/A |  |
|  | 3 | 1762.5 | 5 | 25 | 1857.5 | N/A | FDD | N/A | No |
|  | n78 | 3465 | 10 | 50 | 3465 | N/A | TDD | N/A |  |
| DC_3A-SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 | Yes |
|  |  |  |  |  |  | 28.74 |  |  | Yes |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A | Yes |
| DC_3A_SUL_n78A-n80A, DC_66A-SUL_n78A-n86A | 3, 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 | No |
|  |  |  |  |  |  | 10.74 |  |  | No |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A | No |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |  |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |  |
|  | n77, n78 | 3635 | 10 | 52 | 3635 | N/A | TDD | H4 |  |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |  |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |  |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | FDD | IMD3 | Yes |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | FDD | IMD3 |  |
| DC_20A_n77A | 20 | 850 | 5 | 25 | 810 | 11 | FDD | IMD4 |  |
|  | n77 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
|  | 20 | 840 | 5 | 25 | 800 | 6.5 | FDD | IMD5 |  |
|  | n77 | 4160 | 10 | 50 | 4150 | N/A | TDD | N/A |  |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | 850 | 5 | 25 | 810 | 21.7 | FDD | IMD44 |  |
|  | n78 | 3360 | 10 | 50 | 3360 | N/A | TDD | N/A |  |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | FDD | IMD3 |  |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | TDD | N/A |  |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD3 |  |
|  | n41 | 2562 | 10 | 52 | 2562 | N/A | TDD | N/A |  |
| DC_28A_n51A | 28 | 725.5 | 20 | 25 | 765.5 | 5 | FDD | IMD 4, 5 | Yes |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | 5 | TDD | IMD 4, 5 |  |
| DC_26A_n77A DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |  |
|  | n77, n78 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |  |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |  |
|  | n77, n78 | 3582.5 | 10 | 25 | 3582.5 | N/A | TDD | N/A |  |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD23 |  |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |  |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |  |
| DC_66A_n78A | 66 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD23 |  |
|  |  |  |  |  |  | 28.74 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | 66 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD43 |  |
|  |  |  |  |  |  | 10.74 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A. In case Single UL is allowed and the UE only indicates support of “Single UL” the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs |  |  |  |  |  |  |  |  |  |

###### 7.3B.2.3.5.2	Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A-3A_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |  |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |  |
| DC_1A-3A_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |  |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |  |
| DC_1A-7A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |  |
|  | 7 | 2533 | 10 | 52 | 2653 | 30.0 | FDD | IMD2 |  |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 |  | IMD2 |  |
|  | n77 | 3757.5 | 10 | 52 | 3757.5 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 |  | IMD4 |  |
|  | n77 | 3980 | 10 | 52 | 3980 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | FDD | IMD2 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  | n77 | 3915 | 10 | 52 | 3915 | N/A | TDD | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 |  | IMD2 ¦fB78-fB1¦ |  |
|  | n78 | 3757.5 | 10 | 52 | 3757.5 | N/A | TDD | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | FDD | IMD5 ¦2*fB78-3*fB3¦ |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A |  | N/A |  |
|  | n78 | 3725 | 10 | 52 | 3725 | N/A | TDD | N/A |  |
| DC_1A-5A_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |  |
|  | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |  |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦ |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
|  | 1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 ¦fB78-2*fB5¦ |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |  |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |  |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | FDD | IMD5 ¦2*fB78-3*fB1¦ |  |
|  | n78 | 3405 | 10 | 52 | 3405 | N/A | TDD | N/A |  |
| DC_1A-7A_n78A | 1 | 1930 | 5 | 25 | 2120 | 8.3 | FDD | IMD4 ¦fB78-3*fB1¦ |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |  |
|  | n78 | 3670 | 10 | 52 | 3670 | N/A | TDD | N/A |  |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |  |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | FDD | IMD4 ¦fB78-3*fB1¦ |  |
|  | n78 | 3305 | 10 | 52 | 3305 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | FDD | IMD4 ¦2*fB78-2*fB7¦ |  |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | FDD | N/A |  |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |  |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | FDD | IMD5 |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |  |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | TDD | N/A |  |
| DC_1A-18A_n77A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | FDD | IMD3 |  |
|  | 18 | 825 | 5 | 25 | 870 | N/A |  | N/A |  |
|  | n77 | 3770 | 10 | 52 | 3770 | N/A | TDD | N/A |  |
| DC_1A-18A_n78A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | FDD | IMD3 |  |
|  | 18 | 819 | 5 | 25 | 864 | N/A |  | N/A |  |
|  | n78 | 3758 | 10 | 52 | 3758 | N/A | TDD | N/A |  |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | FDD | N/A |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | FDD | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |  |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | FDD | IMD4 |  |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | TDD | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | FDD | IMD4 |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | FDD | N/A |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | FDD | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A |  | N/A |  |
|  | n77, n78 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |  |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 |  | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | FDD | IMD4 |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A |  | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | TDD | N/A |  |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | FDD | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | FDD | N/A |  |
|  | n78 | 3790 | 10 | 52 | 3790 | N/A | TDD | N/A |  |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | FDD | IMD5 |  |
|  | n78 | 3330 | 10 | 52 | 3330 | N/A | TDD | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | FDD | IMD2 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  | n77, n78 | 3605 | 10 | 52 | 3605 | N/A | TDD | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 |  | IMD5 |  |
|  | n77, n78 | 3675 | 10 | 52 | 3675 | N/A | TDD | N/A |  |
| DC_2A-66A-(n)71B | 66 | 1750 | 5 | 25 | 2150 | 5.0 | FDD | IMD4 |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | FDD | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A |  | N/A |  |
|  | n77 | 3630 | 10 | 52 | 3630 | N/A | TDD | N/A |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 |  | IMD5 |  |
|  | n77 | 3330 | 10 | 52 | 3330 | N/A | TDD | N/A |  |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | FDD | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A |  | N/A |  |
|  | n78 | 3630 | 10 | 52 | 3630 | N/A | TDD | N/A |  |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |  |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 |  | IMD5 |  |
|  | n78 | 3352 | 10 | 52 | 3352 | N/A | TDD | N/A |  |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A |  | N/A |  |
|  | n78 | 3416 | 10 | 52 | 3416 | 15.7 | TDD | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n78 | 3320 | 10 | 52 | 3320 | N/A | TDD | N/A |  |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | FDD | IMD5 |  |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |  |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | FDD | IMD3 |  |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | TDD | N/A |  |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | FDD | N/A |  |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | FDD | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | TDD | N/A |  |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | FDD | IMD4 |  |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | FDD | IMD5 |  |
|  | 28 | 718 | 5 | 25 | 773 | N/A | FDD | N/A |  |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | TDD | N/A |  |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |  |
|  | n77 | 3400 | 10 | 52 | 3400 |  | TDD |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | TDD | IMD4 |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |  |
|  | n77 | 4150 | 10 | 52 | 4150 |  | TDD |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | TDD | IMD5 |  |
| DC_1A-41A_n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |  |
|  | 41 |  | 5 | 25 | 2515 | 12 | TDD | IMD4 |  |
|  | n78 | 3410 | 10 | 52 | 3410 | N/A | TDD | N/A |  |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |  |
|  | n79 | 4500 | 40 | 216 | 4500 |  | TDD |  |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | TDD | IMD2 |  |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |  |
|  | n79 | 4980 | 40 | 216 | 4980 |  | TDD |  |  |
|  | 41 | 2687.5 | 5 | 25 | 2687.5 | 0.0 | TDD | IMD5 |  |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | FDD | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 |  | TDD |  |  |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | TDD | IMD5 |  |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | TDD | N/A |  |
|  | n79 | 4640 | 40 | 216 | 4640 |  | TDD |  |  |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | FDD | IMD3 |  |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | TDD | N/A |  |
|  | n79 | 4520 | 40 | 216 | 4520 |  | TDD |  |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | FDD | IMD4 |  |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n78 | 3410 | 10 | 52 | 3410 | N/A | TDD | N/A |  |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |  |
|  | n78 | 3490 | 10 | 52 | 3490 | 4.6 | TDD | IMD5 |  |
| DC_2A-66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5.0 | FDD | IMD4 |  |
|  | n71 | 675 | 5 | 25 | 629 | N/A |  | N/A |  |
| DC_3A-5A_n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |  |
|  | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦4 |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
|  | 3 | 1740 | 5 | 25 | 1835 | 26.0 | FDD | IMD2 ¦fB78-fB3¦ |  |
|  |  |  |  |  |  | 28.75 |  |  |  |
|  | 5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
|  | 3 | 1770 | 5 | 25 | 1865 | 8.0 | FDD | IMD4 ¦fB78-3*fB3¦4 |  |
|  |  |  |  |  |  | 10.75 |  |  |  |
| DC_3A-7A_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |  |
|  | 7 | 2562 | 10 | 52 | 2682 | 16.9 | FDD | IMD3 |  |
|  | 7 | 2543 | 10 | 52 | 2663 | N/A | FDD | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |  |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | FDD | IMD2 |  |
| DC_3C-7C_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fB78-2*fB7¦ |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fB78-2*fB7¦ |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n78 | 3475 | 10 | 52 | 3475 | N/A | TDD | N/A |  |
| DC_3A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | FDD | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |  |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | FDD | IMD4 |  |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |  |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |  |
|  | n77 | 4195 | 10 | 52 | 4195 | N/A | TDD | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |  |
|  | 28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |  |
|  | n77 | 3320 | 10 | 52 | 3320 | N/A | TDD | N/A |  |
| DC_3A-28A_n78A | 3 | 1750 | 5 | 25 | 1850 | 26.0 | FDD | IMD2 |  |
|  | 28 | 760 | 5 | 25 | 760 | N/A |  | N/A |  |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 17.3 | FDD | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |  |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | TDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1845 | 8.0 | FDD | IMD4 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A |  | N/A |  |
|  | n78 | 3480 | 10 | 25 | 3480 | N/A | TDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1875 | 8.0 | FDD | IMD5 |  |
|  | 28 | 740 | 5 | 25 | 760.5 | N/A |  | N/A |  |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |  |
|  | 28 | 705 | 5 | 25 | 780 | 8.3 |  | IMD5 |  |
|  | n78 | 3600 | 10 | 25 | 3600 | N/A | TDD | N/A |  |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | FDD | IMD4 |  |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | TDD | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | FDD | IMD5 |  |
|  | 28 | 725 | 5 | 25 | 780 | N/A | FDD | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |  |
| DC_3A_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A |  | N/A |  |
|  | n78 | 3764 | 10 | 52 | 3764 | 4.5 | TDD | IMD5 |  |
| DC_3C_n78A | 3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |  |
|  |  |  |  |  |  | 28.75 |  |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 25 | 3710 | N/A | TDD | N/A |  |
| DC_3C_n78A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |  |
|  |  |  |  |  |  | 10.75 |  |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |  |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |  |
|  | n78 | 3340 | 10 | 52 | 3340 | N/A | TDD | N/A |  |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | TDD | IMD3 |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |  |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | TDD | N/A |  |
|  | n78 | 3710 | 10 | 52 | 3710 | 4.2 | TDD | IMD5 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | FDD | IMD3 ¦fB78-2*fB7¦ |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | FDD | IMD4 ¦2*fB78-2*fB7¦ |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n78 | 3475 | 10 | 52 | 3475 | N/A | TDD | N/A |  |
| DC_3A-19A_n79A | 5 | 840 | 5 | 25 | 885 | N/A | FDD TDD | N/A |  |
|  | n78 | 3445 | 10 | 52 | 3445 | N/A |  | N/A |  |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | TDD | N/A |  |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | FDD | IMD4 |  |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A |  | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | FDD | IMD3 ¦fB78-2*fB20¦ |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |  |
|  | n78 | 3510 | 10 | 52 | 3510 | N/A | TDD | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | FDD | N/A |  |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 |  | IMD4 |  |
|  | n77, n78 | 3795 | 10 | 52 | 3795 | N/A | TDD | N/A |  |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | FDD | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  | n77 | 3935 | 10 | 52 | 3935 | N/A | TDD | N/A |  |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | FDD | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | TDD | N/A |  |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 ¦fB78-3*fB5¦ |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |  |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
|  | 5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | FDD | N/A |  |
|  | n78 | 3489 | 10 | 52 | 3489 | N/A | TDD | N/A |  |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | FDD | IMD2 ¦fB78-fB7¦ |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | FDD | N/A |  |
|  | n78 | 3429 | 10 | 52 | 3429 | N/A | TDD | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | FDD | IMD5 ¦2*fB78-3fB7¦ |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | FDD | N/A |  |
|  | n78 | 3350 | 10 | 52 | 3350 | N/A | TDD | N/A |  |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | FDD | IMD2 |  |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |  |
|  | n78 | 3500 | 10 | 52 | 3500 | N/A | TDD | N/A |  |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | FDD | IMD5 |  |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | TDD | N/A |  |
|  | n78 | 3490 | 10 | 52 | 3490 | N/A | TDD | N/A |  |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | FDD | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |  |
|  | 7 | 2550 | 10 | 52 | 2670 | 5.9 | FDD | IMD5 |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | FDD | IMD2 ¦fB78-fB7¦ |  |
|  | n78 | 3370 | 10 | 52 | 3370 | N/A | TDD | N/A |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | FDD | IMD5 ¦2*fB78-3*fB7¦ |  |
|  | n78 | 3435 | 10 | 52 | 3435 | N/A | TDD | N/A |  |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | FDD | IMD2 ¦fB78-fB20¦ |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | FDD | N/A |  |
|  | n78 | 3520 | 10 | 52 | 3520 | N/A | TDD | N/A |  |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |  |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 |  | IMD2 |  |
|  | n78 | 3350 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | FDD | N/A |  |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 |  | IMD5 |  |
|  | n78 | 3460 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | FDD | IMD2 |  |
|  | 28 | 740 | 5 | 25 | 768 | N/A |  | N/A |  |
|  | n78 | 3390 | 10 | 52 | 3421 | N/A | TDD | N/A |  |
| DC_7A_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A |  | N/A |  |
|  | n78 | 3310 | 10 | 52 | 3310 | 29.7 | TDD | IMD2 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | FDD | N/A |  |
|  | n78 | 3365 | 10 | 52 | 3365 | N/A | TDD | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | FDD | IMD2 |  |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | TDD | IMD2, IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |  |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 |  | IMD5 |  |
|  | n77 | 4058 | 10 | 52 | 4058 | N/A | TDD | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | FDD | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A |  | N/A |  |
|  | n77 | 3757 | 10 | 52 | 3757 | N/A | TDD | N/A |  |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | FDD | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A |  | N/A |  |
|  | n78 | 3756 | 10 | 52 | 3756 | N/A | TDD | N/A |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | FDD | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A |  | N/A |  |
|  | n77, n78 | 3783.3 | 10 | 52 | 3783.3 | N/A | TDD | N/A |  |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | FDD | N/A |  |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 |  | IMD4 |  |
|  | n77 | 4015 | 10 | 52 | 4015 | N/A | TDD | N/A |  |
| DC_19A-21A_n79A | 19 | 837.5 | 5 | 25 | 882.2 | N/A | FDD | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 |  | IMD5 |  |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |  |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | FDD | N/A |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | FDD | IMD3 |  |
|  | n77 | 3689.5 | 10 | 52 | 3689.5 | N/A | TDD | N/A |  |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | FDD | IMD4 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | FDD | N/A |  |
|  | n77 | 3690 | 10 | 52 | 3690 | N/A | TDD | N/A |  |
| DC_28A-42A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | FDD | IMD5 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | TDD | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |  |
| DC_66A_(n)71B | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |  |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |  |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |  |
|  | n78 | 3680 | 10 | 52 | 3680 | N/A | TDD | N/A |  |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | TDD | IMD2 |  |
|  | 19 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |  |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | TDD | N/A |  |
|  | n78 | 3715 | 10 | 52 | 3715 | 28.8 | TDD | IMD2 |  |
| DC_20A_n28A-n78A | 20 | 857 | 5 | 25 | 816 | N/A | FDD | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |  |
|  | n78 | 3314 | 10 | 52 | 3314 | 8.7 | TDD | IMD4 |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |  |
|  | n78 | 3310 | 10 | 52 | 3310 | N/A | TDD | N/A |  |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | FDD | IMD4 |  |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | FDD | N/A |  |
|  | n78 | 3420 | 10 | 52 | 3420 | N/A | TDD | N/A |  |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | TDD | IMD2 |  |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | FDD | N/A |  |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | TDD | N/A |  |
|  | n78 | 3487 | 10 | 52 | 3487 | 29.8 | TDD | IMD2 |  |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A. In case Single UL is allowed and the UE only indicates support of “Single UL” the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of 36.101 apply). |  |  |  |  |  |  |  |  |  |

###### 7.3B.2.3.5.3	MSD exceptions due to Tx leakage issue

Table 7.3B.2.3.5.3-1: MSD exceptions due to Tx leakage issue (three bands)

| MSD due to Tx leakage issue exception for the DL band |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA and NR DC Configuration | E-UTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL FC (MHz) | MSD (dB) | Duplex mode |
| DC_2A-66A-(n)71B | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |

#### 7.3B.2.4	Inter-band EN-DC including FR2

##### 7.3B.2.4.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR2

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

Sensitivity degradation is allowed for a band in frequency range 2 if it is impacted by UL harmonic interference from the band in frequency range 1 of the same DC configuration. Reference sensitivity exceptions are specified in Table 7.3B.2.4.1-1 with uplink configuration specified in Table 7.3B.2.4.1-2.

Table 7.3B.2.4.1-1: Reference sensitivity exceptions due to UL harmonic for EN-DC in NR FR2

| UL Band | DL Band | 50 MHz (dBm) | 100 MHz (dBm) | 200 MHz (dBm) | 400 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |

Table 7.3B.2.4.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Y |  |  |  |  |  |  |  |  |  |  |  |

#### 7.3B.2.5	Inter-band EN-DC including both FR1 and FR2

##### 7.3B.2.5.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC including both FR1 and FR2

For inter-band EN-DC of LTE and NR in both FR1 and FR2, the UE is allowed to apply each sensitivity degradation for EN-DC in FR1 specified in clause 7.3B.2.3 TS 38.101-3 and for EN-DC including FR2 specified in clause 7.3B.2.3 of TS 38.101-3 independently.

### 7.3B.3	ΔRIB,c, ΔRIBNC for EN-DC

<Editor’s note: Table number to be updated>

For the UE which supports inter-band EN-DC configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in [4], Table 7.3-1 in [2] and Table 7.3.1-1 in [3] shall be increased by the amount given in ΔRIB,c,ΔRIBNC in Tables below where unless otherwise stated, the same ΔRIB,c, ΔRIBNC are applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔRIB,c or ΔRIBNC is set to zero.

#### 7.3B.3.1	Intra-band contiguous EN-DC

#### 7.3B.3.2	Intra-band non-contiguous EN-DC

Table 7.3B.3.2-1: Intra-band non-contiguous EN-DC with one uplink configuration for reference sensitivity

| DC configuration | Aggregated channel bandwidth (LTE+NR) | Wgap / (MHz) | UL LTE allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| DC_3A_n3A | 5MHz+5MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5MHz+10MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5MHz+15MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5MHz+20MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz+25MHz | 25.0 < Wgap ≤ 45.0 | 121 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz+30MHz | 20.0 < Wgap ≤ 40.0 | 121 | 3.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 10MHz+5MHz | 30.0 < Wgap ≤ 60.0 | 125 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10MHz+10MHz | 25.0 < Wgap ≤ 55.0 | 125 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10MHz+15MHz | 20.0 < Wgap ≤ 50.0 | 125 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10MHz+20MHz | 15.0 < Wgap ≤ 45.0 | 125 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz+25MHz | 10.0 < Wgap ≤ 40.0 | 125 | 3.2 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz+30MHz | 5.0 < Wgap ≤ 35.0 | 125 | 2.8 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+5MHz | 25.0 < Wgap ≤ 55.0 | 126 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15MHz+10MHz | 20.0 < Wgap ≤ 50.0 | 126 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15MHz+15MHz | 15.0 < Wgap ≤ 45.0 | 126 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz+20MHz | 10.0 < Wgap ≤ 40.0 | 126 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15MHz+25MHz | 5.0 < Wgap ≤ 35.0 | 126 | 3.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15MHz+30MHz | 0.0 < Wgap ≤ 30.0 | 126 | 3.3 |  |
|  | 20MHz+5MHz | 15.0 < Wgap ≤ 50.0 | 167 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20MHz+10MHz | 10.0 < Wgap ≤ 45.0 | 167 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz+15MHz | 5.0 < Wgap ≤ 40.0 | 167 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20MHz+20MHz | 0.0 < Wgap ≤ 35.0 | 167 | 4.1 |  |
|  | 20MHz+25MHz | 0.0 < Wgap ≤ 30.0 | 167 | 3.8 |  |
|  | 20MHz+30MHz | 0.0 < Wgap ≤ 25.0 | 167 | 3.6 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. |  |  |  |  |  |

#### 7.3B.3.3	Inter-band EN-DC within FR1

##### 7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n51 | n51 | 0.1 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n51 | 3 | 0.2 |
|  | n51 | 0.2 |
| DC_3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n51 | n51 | 0.2 |
| DC_7_n77 | n78 | 0.5 |
| DC_7_n78 | n78 | 0.5 |
| DC_8_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12A_n5A | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12A_n66A | 12 | 0.5 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n51 | n51 | 0.2 |
| DC_20_n77 | n77 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41 | n41 | 0f |
|  |  | 0.52 |
| DC_26A_n77A | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28A_n51 | n51 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_39_n78 | n78 | 0.5 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n77 | 40 | 0.4 |
|  | n77 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_42_n51 | n51 | 0.2 |
| DC_66A_n78A | 66 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

##### 7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n78 DC_1-7-7_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-18_n78 | n78 | 0.5 |
| DC_1-19_n77 |  |  |
|  | n77 | 0.5 |
| DC_1-19_n78 |  |  |
|  | n78 | 0.5 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 |  |  |
|  | n77 | 0.5 |
| DC_1-21_n78 | 1 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n78 | 1 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | 28 | 0.3 |
| DC_1-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n78 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-42_n79 |  |  |
|  | 42 | 0.5 |
| DC_1_n77-n79 | 1 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_1_n78-n79 | 1 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_1-SUL_n78-n84 | n78 | 0.5 |
| DC_2_5_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_30_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-66_n71B | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3_n3-n77 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n3-n78 | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n78, DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-19_n79 |  |  |
| DC_3-20_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-28_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n28-n78 | 3 | 0.2 |
|  | n28 | 0 |
|  | n78 | 0.5 |
| DC_3-38_n78 | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_3-41_n78 | 3 | 0.2 |
|  | 41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| DC_3-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_3_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_3-SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n80 | 0.2 |
| DC_3-SUL_n78-n82 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_5_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_7-7_n78 | 7 | 0.0 |
|  | n78 | 0.5 |
| DC_7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_7-20_n78 |  |  |
|  | n78 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 |  |  |
|  | n78 | 0.5 |
| DC_7-46_n78 |  |  |
|  | n78 | 0.5 |
| DC_8A-SUL_n78-n81 | 8 | 0.2 |
|  | n78 | 0.2 |
|  | n81 | 0.2 |
| DC_18-28_n77 | n77 | 0.5 |
| DC_18-28_n78 | n78 | 0.5 |
| DC_19-21_n77 |  |  |
|  | n77 | 0.5 |
| DC_19-21_n78 |  |  |
|  | n78 | 0.5 |
| DC_19-42_n77 |  |  |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78 |  |  |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-42_n79 |  |  |
|  | 42 | 0.5 |
| DC_19_n77-n79 | 19 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_19_n78-n79 | 19 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_20_n8-n75 | 20 | 0.0 |
|  | n8 | 0.0 |
|  | n75 | 0.0 |
| DC_20_n28-n75 | 20 | 0.0 |
|  | n28 | 0.2 |
|  | n75 | 0.0 |
| DC_20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_20_n75-n78 | 20 | 0.0 |
|  | n75 | 0.0 |
|  | n78 | 0.5 |
| DC_20_n76-n78 | 20 | 0.0 |
|  | n76 | 0.0 |
|  | n78 | 0.5 |
| DC_20-SUL_n78-n82 | n78 | 0.5 |
| DC_20-SUL_n78-n83 | 20 | 0.2 |
|  | n78 | 0.5 |
|  | n83 | 0.2 |
| DC_21-42_n77 |  |  |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78 |  |  |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-42_n79 |  |  |
|  | 42 | 0.5 |
| DC_21_n77-n79 | 21 | 0.0 |
|  | n77 | 0.5 |
|  | n79 | 0.0 |
| DC_21_n78-n79 | 21 | 0.0 |
|  | n78 | 0.5 |
|  | n79 | 0.0 |
| DC_28-SUL_n78-n83 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n83 | 0.2 |
| DC_28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_41-42_n79 | 42 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_66-SUL_n78-n86 | 66 | 0.2 |
|  | n78 | 0.5 |
|  | n86 | 0.2 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

##### 7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28 | n28 | 0.2 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-8_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
| DC_1-3-19_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-21_n79 |  |  |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-7-20_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18-28_n77 | n77 | 0.5 |
| DC_1-18-28_n78 | n78 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-42_n79 | 42 | 0.5 |
| DC_1-20_n28-n78 | 1 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-42_n79 | 42 | 0.5 |
| DC_1-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_2-66-(n)71B | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3-5-7_n78, DC_3-5-7-7_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.1 |
| DC_3-7-20_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-28_n78 DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-19-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-42_n78 | 0.2 | 0.2 |
|  | 0.5 | 0.5 |
|  | 0.5 | 0.5 |
| DC_3-19-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-20_n28-n78 | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21-42_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_3-28-42_n77 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-28-42_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-28-42_n79 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_5-7-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_7-20_n28-n78 | 7 | 0.0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_19-21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-21-42_n79 | 42 | 0.5 |
| DC_21-28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |

##### 7.3B.3.3.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-7-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-19-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-28-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-28-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-28-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21-42_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.2 |
| DC_1-3-21-42_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.2 |
| DC_1-3-21-42_n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_1-7-20_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n79 | 42 | 0.5 |
| DC_1-21-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_3-7-20_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

##### 7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

#### 7.3B.3.4	Inter-band EN-DC including FR2

##### 7.3B.3.4.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.4.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |

##### 7.3B.3.4.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.4.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-18_n257 | 1 | 0.3 |
|  | 18 | 0.3 |
| DC_1-28_n257 | 28 | 0.2 |
| DC_1-41_n257 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-42_n257 | 1 | 0 |
|  | 42 | 0.5 |
| DC_1-77_n257 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1-78_n257 | 1 | 0 |
|  | n78 | 0.5 |
| DC_1-79_n257 | 1 | 0.0 |
|  | n79 | 0.0 |
| DC_2-66_n257 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_3-21_n257 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-28_n257 | n257 | 0.5 |
| DC_3-41_n257 | 41 | 01/0.52 |
| DC_3-42_n257 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-77_n257 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-78_n257 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-79_n257 | 3 | 0.0 |
|  | n79 | 0.0 |
| DC_5_n78-n257 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_7_n78-n257 | 7 | 0 |
|  | n78 | 0.5 |
| DC_13-66_n260 | 13 | 0.3 |
|  | 66 | 0.3 |
| DC_19-42_n257 |  |  |
|  | 42 | 0.5 |
| DC_19-77_n257 | 19 | 0.0 |
|  | n77 | 0.5 |
| DC_19-78_n257 | 19 | 0.0 |
|  | n78 | 0.5 |
| DC_19-79_n257 | 19 | 0.0 |
|  | n79 | 0.0 |
| DC_21-42_n257 |  |  |
|  | 42 | 0.5 |
| DC_21-77_n257 | 21 | 0.0 |
|  | n77 | 0.5 |
| DC_21-78_n257 | 21 | 0.0 |
|  | n78 | 0.5 |
| DC_21-79_n257 | 21 | 0.0 |
|  | n79 | 0.0 |
| DC_28-42_n257 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_41-42_n257 |  |  |
|  | 42 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

##### 7.3B.3.4.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.4.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-21_n257 |  |  |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-28_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
| DC_1-3-42_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-42_n257 | 42 | 0.5 |
| DC_1-21-28_n257 | 28 | 0.2 |
| DC_1-21-42_n257 | 42 | 0.5 |
| DC_1-28-42_n257 | 28 | 0.2 |
| DC_1-41-42_n257 | 42 | 0.5 |
| DC_3-5_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_19-21-42_n257 | 42 | 0.5 |
| DC_3-19-21_n257 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-19-42_n257 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-21-42_n257 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_3-28-42_n257 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_5-7_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_7-7_n78-n257 | n78 | 0.5 |
| DC_21-28-42_n257 | 28 | 0.2 |
|  | 42 | 0.5 |

##### 7.3B.3.4.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.4.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n257 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-19-42_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-21-42_n257 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_1-3-28-42_n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1A-3A-28A-42C_n257A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-5-7_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-7_n78-n257 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n257 | 42 | 0.5 |
| DC_1-21-28-42_n257 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_3-5-7_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-7_n78-n257 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_5-7-7_n78-n257 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

##### 7.3B.3.4.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.4.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78-n257 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-7_n78-n257 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-5-7-7_n78-n257 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5-7-7_n78-n257 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

#### 7.3B.3.5	Inter-band EN-DC including both FR1 and FR2

##### 7.3B.3.5.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.5.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n77-n257 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78-n257 | n78 | 0.5 |
| DC_3_n77-n257 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78-n257 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_19_n77-n257 | n77 | 0.5 |
| DC_19_n78-n257 | n78 | 0.5 |

# Annex A (normative):
Dual uplink interferer

UE is mandated to support operation in dual uplink mode also in EN-DC configuration for FR1 listed in Table 7.3.2.1.5-1 and indicated by column single uplink allowed if the intermodulation products caused by the dual uplink operation do not interfere own downlink transmission.

Formula for determining if the EN-DC in NR FR1 configuration with dual uplink operation interferes own downlink reception.

Interference bandwidth: IBW = |a| * CBW1 + |b| * CBW2

-	|a| + |b| = 2 (or 3)

-	CBW1 and CBW2 are the transmission bandwidth configurations of the UL channels

Center frequency of IBW:  fIBW = |a * f1 + b * f2|

-	f1 and f2 are center frequency of the transmission bandwidth configurations of each UL channel

The range of IMD 2 (or 3): [fIBW – IBW/2, fIBW + IBW/2]

Note 1: UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing on the transmission bandwidth.

Note 2: For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in rel.15.

# Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN4#84 |  |  |  |  | Initial Skeleton | 0.0.1 |
| 2017-11 | RAN4#84Bis | R4-1711980 |  |  |  | Number TPs from editors | 0.1.0 |
| 2017-12 | RAN4#85 | R4-1713807 |  |  |  | Approved TPs in RAN4#85 R4-1714444, CA BW classes, TP, Ericsson R4-1714170, How to list DC configurations into TS 38.101-3, Nokia R4-1714530, TP on introducing operating bands for NR-LTE DC including SUL band combinations in 38.101-3 , Qualcomm R4-1714098, TP to TS 38.101-3: UE RF requirements for non-standalone SUL, Huawei R4-1713206, TP on general parts for 38.101-3 NR interwork, Ericsson R4-1714443, TP to TS 38.101-3: On dual uplink operation for EN-DC in NR FR1 and single uplink, Nokia R4-1714450, TP to 38.101-3: maximum output power and unwanted emissions for EN-DC, Ericsson R4-1714346, TP to 38.101-3: REFSENS for intra-band EN-DC, Ericsson R4-1714345, TP for TS 36.101-3: clause 7 receiver requirements, Huawei Band list according to R4-1714542, List of bands and band combinations to be introduced into RAN4 NR core requirements by December 2017, RAN4 Chairmen | 0.2.0 |
| 2017-12 | RAN4#85 | R4-1714571 |  |  |  | Further corrections after email review | 0.3.0 |
| 2017-12 | RAN#78 | RP-172477 |  |  |  | v1.0.0 submitted for plenary approval. Contents same as 0.3.0 | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180264 | 0005 |  | F | Implementation of endorsed CRs to 38.101-3 Endorsed draft CR F: R4-1801267, Draft CR on UE RF requirements for SUL in TS 38.101-3, Huawei B: R4-1801111, Draft CR for completed LTE 1CC + NR 1band for TS 38.101-3, NTT DOCOMO, INC. B: R4-1800716, Draft CR for introduction of completed band combinations from 37.863-03-01 into 38.101-3, Ericsson B: R4-1800063, Draft CR for completed EN-DC of LTE 4CC + NR 1band for TS 38.101-3, Nokia B: R4-1800717, Draft CR for introduction of completed band combinations from 37.865-01-01 into 38.101-3, Ericsson F: R4-1800049, Modification for TS38.101-3, CATT F: R4-1800287, 38.101-3 DC_(n)71B draft CR for section 6.2.4.1 - A-MPR for intra-band EN-DC - NS value, T-Mobile USA Inc. F: R4-1800288, 38.101-3 DC_(n)71B draft CR for section 7.3.3 Reference sensitivity for DC_(n)71B - MSD values, T-Mobile USA Inc. F: R4-1801139 Draft CR to 38.101-3: MSD for inter-band EN-DC, Ericsson | 15.1.0 |
| 2018-06 | RN#80 |  |  |  | F | Endorced drft CRs from RN4#86Bis R4-1804550, Drft CR for Delt RIB tble improvement, NTT DOCOMO, INC. R4-1804551, Draft CR for Delta TIB table improvement, NTT DOCOMO, INC. R4-1804838, Draft CR to 38.101-3: Update of section 5.5, Rohde & Schwarz R4-1805903, Draft CR into TS 38.101-3 Correction on DC_3_SUL_n78-n80, Huawei, HiSilicon R4-1805905, Draft CR into TS 38.101-3 Introduction of band combinations for SUL, Huawei, HiSilicon R4-1806005, Draft CR to TS38.101-3 on introduction of ULSUP EN-DC band combination, Nokia R4-1806008, Draft CR on time mask for EN-DC with TDM-based UL sharing from UE perspective, Huawei R4-1805937, Draft CR for introduction of completed EN-DC with LTE 5CC + NR 1band in TS 38.101-3, Samsung R&D Institute UK R4-1805938, Draft CR for completed DC of LTE 2CC + NR 1band for TS 38.101-3, Huawei R4-1803979, Draft CR for LTE(xDL/1UL)+NR(2DL/1UL) DC band combinations, LG Electronics France R4-1805940, draft CR introduction completed band combinations 37.865-03-01 -> 38.101-3, Ericsson R4-1805939, Draft CR for completed DC of LTE 1CC + NR 1band for TS 38.101-3, NTT DoCoMo R4-1806030, Draft CR of completed DC of LTE 4CC + NR 1 band, Nokia | 15.2.0 |