3GPP TSG RAN Meeting #80                                                                    RP-181381

La Jolla, USA, 11th June – 14th June 2018				rev from RP-180660

Source:	Thales, HNS, Inter Digital, Nokia and Nokia Shanghai Bell, ESA, Heron, IAESI, Fraunhofer IIS

Title:	NR-NTN: TP for Chap 8 Recommendations on the way forward

TDOC Type:	pCR

Agenda Item:	9.4.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification:	TR 38.811 (SID = sFS_NR_nonterr_nw)

# Abstract

This document proposes a draft text for the clause “recommendations on the way forward” of the 3GPP TR 38.811.

It includes the text proposed in R1-1807795

# Proposed Text

We suggest adding section 8, with the text below, in order to briefly describe the complexity and the constraints:

* * * Start of changes * * * * (new text)

# 8	Recommendations on the way forward

## 8.1	General outcomes

This report has

- provided detailed descriptions of some deployment scenarios for non-terrestrial networks and the related system parameters such as altitude, orbit, etc.

- defined a way to generate Non-Terrestrial network channel models leveraging on the models defined in TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”

- identified potential key impact areas on the NR to support Non-Terrestrial networks

In line with 3GPP TR 38.901, the study considered channel models in Non-Terrestrial networks operating in any frequency bands between 0.5 and 100 GHz.

## 8.2	Reference deployment scenarios

A number of scenarios is considered in clause 5.1 “Scenarios overview” including GEO, Non-GEO as well as HAPS based access networks. The intent is to assess relatively worst-case scenarios with respect to Doppler shift, Doppler variation rate, Delay and Delay variation rate. For non-GEO deployment scenarios, the altitude has been set to 600 km, which is pessimistic with respect to the Doppler, and Doppler rate, related effects.

Non-Terrestrial access networks can be used to serve directly User Equipment or indirectly via a relay node.

## 8.3	Non-Terrestrial network channel modelling

Leveraging on the models defined in TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 14)”, complementary features and specific parameters have been identified and proposed in chapter 6 of this report to generate Non-Terrestrial network channel models for all possible deployment scenarios characterised by altitude/orbit of space/aerial vehicles, user environments (rural, sub-urban, urban and dense urban), targeted user equipment type (omni or directive antennas, LOS/NLOS conditions) as well as frequency bands.

## 8.4	NR impacts to support Non-Terrestrial networks

8.4.1 Type of potential NR impacts

The potential NR impacts are characterised according to the classification in the table below:

Table 8.3-1: Type of Potential area of NR impacts to support Non-Terrestrial networks

| Type of Potential area of impact | Modifications of 3GPP NR specifications |
| --- | --- |
| Higher layers impact | Layer 2/3 protocols/architecture |
| Physical layer impact | Parameters settings, procedures or physical channels and signals |

8.4.2 Assessment of potential NR impacts to support Non-Terrestrial networks

The table below identifies the potential area of impacts on the NR specification. This will depend on the deployment scenarios. The intent is to identify the main studies to be carried out taking into account the defined NTN channel model to enable operation of NR in Non-Terrestrial networks. Other areas of impact are identified which could lead to performance improvement.

Table 8.3-2: Evaluation of NR impacts to support Non-Terrestrial networks

| Non-Terrestrial network specifics | Effects | Impacted NR features | Potential areas of impact to be further studied | Comment |
| --- | --- | --- | --- | --- |
| Motion of the space/aerial vehicles | Moving cell pattern | Hand-over/paging | Higher layers impact | Paging and Hand-over procedures should be adapted to take into account the relative motion of the cell pattern with respect to the tracking area. Further analysis on tracking area design may need to be carried out. Mobility management is also to be considered (Note 1) |
|  | Delay variation | TA adjustment | Physical layer impact | Alignment of uplink signals may need to be considered |
|  | Doppler | Initial downlink synchronization | No impact | The preferred SCS values for Non-Terrestrial Networks may be respectively 60 KHz for frequency bands lower than 6 GHz and 240 KHz for frequency bands above 6 GHz. However it can also operate with lower SCS value |
|  |  | DMRS time density | No impact | The prefered DM-RS configuration may be type 1 to cope with Doppler variation rate |
| Altitude | Long latency | HARQ | Higher Layers & physical layer Impact | Need to adapt the HARQ specification. Deactivation and/or enhancements of NR HARQ can be considered |
|  |  | Physical layer Procedures (ACM, power control) | Physical layer impact | The operation/configuration of Adaptive power and coding/modulation control loop protocols may have to be adapted. |
|  |  | MAC/RLC Procedures | Higher layers impact | Timers limit of MAC/RLC and higher layers loop protocols may have to be extended |
| Cell size | Differential delay | TA in Random access response message | Physical layer impact | Doppler/Delay (Note 2) compensation technique (Note 3) can be implemented. Further analysis/simulations using the NTN channel model is needed. Adaptations of PRACH format and random access procedure may have to be considered. |
|  |  | Random access | Physical layer impact |  |
| Propagation channel | Impairments | DMRS frequency density | No impact | Non terrestrial network propagation channel may feature a frequency selective at most comparable with cellular channel |
|  | Impairments | Cyclic prefix | No impact | Non terrestrial network propagation channel may feature a worse delay spread at most comparable to cellular channels. |
| Duplex scheme | Regulatory constraints | Duplexing mode (TDD/FDD) | Higher layers impact | FDD is prefered especially for most satellite systems. TDD can be considered for HAPS and for LEO |
| Satellite or aerial Payload performance | Phase noise impairment | PT-RS | Potential constraint on the operation to be further studied | Satellite Radio links are typically operated with relatively low order modulation scheme, in most of the cases up to 16QAM |
|  | Back-off | PAPR | Physical layer impact | Uplink: It is recommended to use DFT-S OFDM Downlink: Low PAPR scheme may improve performance. Howver not mandatory to support non-terrestrial networks |

Note 1: Some of the considered deployment scenarios assume indirect access based on fixed or even mobile relays.

For NGSO-based deployment scenarios NTN and/or relays mounted on a moving platform such as a train, mobility management also needs further studies.

Note 2: The common delay, the Differential delay as well as their time variation may need to be compensated.

Note 3: Doppler/Delay compensation techniques can be implemented especially for Non GEO satellites:

- GNSS based techniques: The User Equipment equipped with a GNSS receiver determines its position and the universal time. Thanks to pre-loaded / updated ephemeris of the satellite constellation which can be theoretical or actual, the UE is able to compute the position and motion of the possible serving satellites enabling to determine the Doppler shift and variation rate as well as the absolute Delay and Delay variation.

- Non GNSS based techniques can also be envisaged and implemented.

* * * End of changes * * * *