3GPP TSG RAN Meeting #80								RP-181466

La Jolla, USA, June 11 - 14, 2018

Agenda Item:		9.3.1

Source:			NTT DOCOMO, INC.

Title:				WI Summary of New Radio Access Technology

WI code(s):			NR_newRAT

leading WG:		RAN WG1

Release:			Rel-15

1	Introduction

According to the requirements and scenarios for new radio (NR) described in [TS38.913], the objectives of the Work Item are to specify the NR functionalities for enhanced mobile broadband (eMBB) and ultra-reliable low-latency-communication (URLLC) considering frequency ranges up to 52.6 GHz. While the NR is not required to be backward compatible with LTE, the NR functionalities are forward compatible and allow for smooth introduction of additional technology components and support for new use cases.

2	Description

2.1	Architecture and functionality split

There are two types of architecture; NSA (Non-Standalone) and SA(Standalone). In following sections, the details are shown. (Note that SA aspects will be added later.)

2.1.1	NSA overall architecture and functionality split

NSA architecture

The NSA architecture is illustrated in following figure.

Figure 1  NSA Architecture (in section 4.1.2 of 3GPP TS 37.340[X])

In the NSA architecture, the NR base station (logical node “en-gNB”) connects to the LTE base station (logical node “eNB”) via the X2 interface. Although the X2 interface has been used up to now to connect eNBs, Release 15 extends the interface to also support connecting an eNB and en-gNB in case of NSA operation. In addition, E-UTRAN for NSA architecture connects to the EPC network using an S1 interface. Dual connectivity between eNB (as master node) and en-gNB (as secondary node) is called EN-DC.

2.1.2	NSA radio protocol architecture

2.1.2.1	Control Plane

In EN-DC, the UE has a single RRC state, based on the MN RRC and a single C-plane connection towards the Core Network. Figure 2 illustrates the Control plane architecture for EN-DC. Each radio node has its own RRC entity (E-UTRA version if the node is an eNB or NR version if the node is a gNB) which can generate RRC PDUs to be sent to the UE. RRC PDUs generated by the SN can be transported via the MN to the UE. The MN always sends the initial SN RRC configuration via MCG SRB (SRB1), but subsequent reconfigurations may be transported via MN or SN. When transporting RRC PDU from the SN, the MN does not modify the UE configuration provided by the SN.

Figure 2:	Control plane architecture for EN-DC.

2.1.2.2	User Plane

In EN-DC, from a UE perspective, three bearer types exist: MCG bearer, SCG bearer and split bearer. These three bearer types are depicted in Figure 3 for EN-DC. For EN-DC, the network can configure either E-UTRA PDCP or NR PDCP for MCG bearers while NR PDCP is always used for SCG and split bearers.

Figure 3: Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in EN-DC

2.2	Physical layer aspects

Numerologies, waveform and frame structure

Similar to LTE, OFDM using a CP is used as downlink (DL) waveform. In contrast to LTE, OFDM can also be used also in the NR uplink (UL) direction. As a complement, DFT-s-OFDM (OFDM with DFT precoding) can be used in the uplink although limited to single-layer transmission only.

In order to support various deployment scenarios and wide range of carrier frequencies, NR supports multiple subcarrier spacings . More specifically, 15kHz, 30kHz, and 60kHz subcarrier spacing can be used for network deployments operating in frequency bands below 6 GHz, referred to as frequency range 1 (FR1). For network deployments above 24 GHz (FR2), 60 kHz and 120 kHz subcarrier spacing can be used.

For the 15 kHz numerology, the CP is approximately 4.7 µs, with the CP scaling inversely with the sub-carrier spacing. In addition, for the 60 kHz numerology, there is a possibility for an extended CP of length approximately four times the normal CP.

One resource block (RB) is defined as consecutive twelve subcarriers.

In the time domain, a subframe of 1ms consists of a number of slots, each slot being of length 14 symbols. The length, in ms, of a slot and thus also the number of slots per subframe hence depends on the numerology.

NR supports both FDD and TDD operation with the same frame structure. In case of TDD, the direction of each OFDM symbol in a slot can be downlink or uplink for flexible traffic adaptation. The transmission direction can be semi-statically configured or dynamically changes as part of the scheduling decision.  Furthermore, transmissions can be carried out over only a fraction of a slot with the minimum transmission consisting of only two symbols. Such very short transmissions mainly targets usage cases requiring low latency, such as some URLLC services.

Channel coding and modulation

In NR, low density parity check (LDPC) coding is used for data (PDSCH/PUSCH) while, for L1 control signalling (PDCCH/PUCCH), polar coding is used in case of more than 11 information bits. For modulation, QPSK, 16QAM, 64QAM and 256QAM are supported for both DL and UL. In addition, when DFT-s-OFDM is selected as a UL waveform, pi/2-BPSK modulation can be used to further reduce peak-to-average power ratio (PAPR) of the UL transmission.

Initial access and mobility

NR synchronization signals i.e., PSS and SSS, are transmitted over 127 sub carriers and are designed to carry the physical cell ID (PCID) selected from 1008 candidates. PSS and SSS are transmitted together with PBCH and DMRS for PBCH as an SS/PBCH block for a carrier within four OFDM symbols in time domain and 240 subcarriers, corresponding to 20 resource blocks, in frequency domain as shown in Figure 4. PBCH carries only minimum system information required to receive system information block 1 (SIB1) necessary for initial access, such as system frame number (SFN), initial PDCCH configuration, initial PDSCH and PUSCH. For SS/PBCH block transmission, the same or different subcarrier spacing from that for other DL transmissions can be utilized according to default subcarrier spacing for the frequency band or higher layer indication.

Figure 4: SS/PBCH block design in NR

SIB1 and other system information (SIB2 and onwards) are carried by PDSCH which is scheduled by PDCCH. RMSI contains necessary information to perform random access procedure for initial access. Paging message is carried by PDSCH which is scheduled by PDCCH. Numerology used for RMSI on a NR cell is indicated in MIB carried by PBCH and is commonly applied to OSI, Paging, Msg.2 and Msg.4 transmissions on the same cell.

For random access in NR, a four-step procedure (Msg.1, Msg.2, Msg.3 and Msg.4) similar to LTE is defined. For Msg.1 transmission in random access, NR supports two types of PRACH sequences, one is based on long Zadoff-Chu (ZC) sequences of length 839 and another is based on short ZC sequences of length 139. Four different PRACH formats based on long ZC sequence are defined and corresponding numerology is defined for each format. Nine different PRACH formats based on short ZC sequence are defined and numerology used for the PRACH format based on short ZC sequence is indicated in RMSI. Msg.2 and Msg.4 for random access are carried by PDSCH that are scheduled by PDCCH. Msg.3 for random access is carried by PUSCH which is scheduled by random access response (RAR) in Msg.2 or PDCCH in case of retransmission.

For mobility measurement and radio link monitoring, SS/PBCH block and/or CSI-RS can be utilized in NR. Configurations of SS/PBCH block and/or CSI-RS including numerology are provided by higher layer signalling.

MIMO aspects

NR supports multi-layer transmission (single-user MIMO) with a maximum of eight and four transmission layers for the downlink and uplink transmission directions, respectively. NR is also particularly designed to support a larger number of multi-user MIMO scheduled users. RSs are specified assuming multi-antenna transmission. Demodulation RS (DM-RS), channel state information RS (CSI-RS) and sounding RS (SRS) are supported for PDSCH/PUSCH demodulation, DL CSI acquisition and UL CSI acquisition, respectively. CSI-RS can be also used for fine frequency/time tracking, a.k.a. Tracking RS (TRS), mobility measurements and beam management measurements. In addition, phase tracking RS (PT-RS) can be used in both DL and UL to compensate for the increased phase noise for the higher frequency ranges.

Beamforming/precoding is an important technique for achieving higher throughput and sufficient coverage. In the DL, beam management is performed based on SSB and CSI-RS, while SRS can be used for the UL beam management. Tx and Rx beams are selected based on L1-RSRP measurement and it is possible to utilize beam sweeping spatial filters at transmitter and receiver to improve beam directions. Information on Tx beams for PDCCH, PDSCH and PUCCH are indicated to a UE and it enables to apply the appropriate Rx beam at a receiver side. In addition, beam failure recovery is supported to achieve quick recovery from misalignment of Tx/Rx beams. UE can identify the misalignment and informs gNB of index of new candidate beam. For DL CSI acquisition, NR supports two levels of PMI definitions, the type I and II codebooks, providing different levels of CSI granularity. On the other hand, for the UL, NR supports a codebook that can be applied for different antenna implementations in terms of transmitter phase coherence across antennas and time. Non-codebook based UL transmission is also supported, where precoded SRS resources are transmitted by the UE and then gNB selects the desired transmission layers for PUSCH by selecting among these SRS resources.

DL channels

The basic way of controlling data transmission in NR is scheduling in a similar way as in LTE. Each device monitors a number of physical downlink control channels (PDCCHs), typically once per slot although it is possible to configure more frequent monitoring to support traffic requiring very low latency. Upon detection of a valid PDCCH, the device follows the scheduling decision and receives (or transmits) accordingly.

The PDCCHs are transmitted in one or more control resource sets (CORESETs), each of length one to three OFDM symbol(s). Unlike LTE, where control channels span the full carrier bandwidth, the bandwidth of a CORESET can be configured. This is needed in order to handle devices with different bandwidth capabilities and also beneficial from a forward-compatibility perspective.

There are different formats for downlink control information (DCI) transmitted on a PDCCH as shown in Table 1. A UE monitors one or more PDCCH candidates for DCI with CRC scrambled by a certain RNTI in PDCCH common search space (CSS) set and/or UE-specific search space (USS) set.

Table1	NR DCI formats.

| DCI format | RNTI | Notes |
| --- | --- | --- |
| DCI format 0_0 | RA-RNTI, TC-RNTI, C-RNTI, CS-RNTI | Monitored on CSS or USS Scheduling PUSCH |
| DCI format 0_1 | C-RNTI, CS-RNTI | Monitored in USS Scheduling PUSCH |
| DCI format 1_0 | SI-RNTI, RA-RNTI, P-RNTI, C-RNTI, CS-RNTI | Monitored in CSS or USS Scheduling PDSCH |
| DCI format 1_1 | C-RNTI, CS-RNTI | Monitored in USS Scheduling PDSCH |
| DCI format 2_0 | SFI-RNTI | Monitored in CSS Indicating slot format for slot(s) |
| DCI format 2_1 | INT-RNTI | Monitored in CSS Indicating pre-emption of DL resource |
| DCI format 2_2 | TPC-PUSCH-RNTI, TPC-PUCCH-RNTI | Monitored in CSS Group-TPC command for PUSCH/PUCCH |
| DCI format 2_3 | TPC-SRS-RNTI | Monitored in CSS Group-command for SRS |

Figure 5: General description of NR PDCCH.

UL channels

In NR, PUCCH delivers UCI (Uplink Control Information) which consists of HARQ-ACK (Hybrid Automatic Repeat Request Acknowledgement), SR (Scheduling Request), or CSI (Channel State Information). Various PUCCH formats are specified as in figure 6. NR PUCCH supports durations of 1 to2 symbols, or durations of 4 to 14 symbols. PUCCH formats 0/2 is called short-PUCCH, which can deliver UCI by 1 to 2 symbols; PUCCH formats 1/3/4 is called long-PUCCH, which can deliver UCI by 4 to 14 symbols.

Figure 6: NR PUCCH formats.

Figure 7 describes general use-cases for PUCCH in NR. Long-PUCCH is used to improve coverage; on the other hand, short-PUCCH is used to reduce latency. Unlike LTE, where the PUCCH is always located at the uplink carrier edges, the frequency/time-domain resources for PUCCH transmission in NR are flexibly configurable. For a UE (User Equipment), TDM (Time Division Multiplexing) between long-PUCCH and short-PUCCH is also supported; for example, UCI of large payload, e.g. CSI, is transmitted by long-PUCCH, and UCI of small payload, e.g. HARQ-ACK, is transmitted by short-PUCCH.

Figure 7: General description of NR PUCCH.

Scheduling/HARQ

DCI formats 0_0/0_1 and 1_0/1_1 schedule PUSCH and PDSCH, respectively. For each DCI format, frequency-domain and time-domain resource allocation fields are included. For frequency-domain resource allocation, resource block group (RBG)-level bit-map resource allocation (resource allocation Type 0) and contiguous resource allocation (resource allocation Type1) are supported. A UE can be configured with either or both of them. For time-domain resource allocation, the time-domain resource allocation field in the DCI jointly indicates scheduled slot, starting symbol, and the duration (number of consecutive symbols to use for the channel).

For PUSCH and PDSCH, other than DCI-based scheduling, configured grant for PUSCH and semi-persistent scheduling (SPS) for PDSCH are supported. For configured grant PUSCH, once PUSCH resource is configured and activated, the UE can transmit a PUSCH without DCI format 0_0/0_1. There are two types of configured grant PUSCH; with the configured grant Type1, UE can transmit PUSCH on the configured grant resource once RRC configuration is available, while with the configured grant Type2, UE can transmit PUSCH on the configured grant resource after a DCI with CRC scrambled by CS-RNTI activates the configured grant resource. SPS for PDSCH also requires activation DCI with CRC scrambled by CS-RNTI.

For NR, only asynchronous and adaptive HARQ is supported for both PDSCH and PUSCH. For re-transmission of a PDSCH or PUSCH, DCI formats 0_0/0_1 or 1_0/1_1 is used. HARQ process number and redundancy version are indicated in the DCI scheduling the re-transmission.

UE reports HARQ-ACK feedback for a decoded PDSCH. UE can be configured with reporting one HARQ-ACK bit for each transport block (TB), while if a UE is configured with CBG-based PDSCH transmission, the UE reports one HARQ-ACK bit for each code block group (CBG), where a CBG is a group of code-blocks consisting of a TB. If a UE is configured with CBG-based PDSCH transmission, the DCI scheduling PDSCH includes CBG transmission information (CBGTI) field which indicates which CBG(s) is/are re-transmitted, and the DCI can also include CBG flushing out information (CBGFI) field, which indicates the CBGs being retransmitted can be combined with the earlier received instances of the same CBGs. For PUSCH, CBG-based transmission using CBGTI is also supported.

For reporting multiple HARQ-ACK bits at one time, two types of HARQ-ACK codebook construction are supported; Type 1 HARQ-ACK codebook constructs the codebook based on semi-static configuration only, while Type 2 HARQ-ACK codebook constructs the codebook based on both semi-static configuration and DCI field named downlink assignment index (DAI).

Carrier Aggregation, Bandwidth Parts, and LTE/NR dual connectivity

In NR, the maximum bandwidth of a NR carrier is 100MHz for carrier frequencies below 6 GHz and 400MHz for carrier frequencies above 24 GHz, respectively. In order to achieve wider bandwidth, carrier aggregation (CA) of up to 16 NR carriers is further supported. Both intra-band CA and inter-band CA are supported. Both self-carrier scheduling and cross-carrier scheduling are supported. For the case of inter-band CA, CA with different numerologies, i.e., CA with NR carrier below 6 GHz and NR carrier above 24 GHz, is also enabled.

NR newly defines the concept of bandwidth part (BWP). Up to four BWPs can be configured for a UE per NR carrier for the DL and UL respectively. The usage cases for BWP include 1) support of different UE BW capabilities in a wideband NR carrier, 2) UE power saving resulted from BW adaptation, and 3) scheduling different UEs with different numerologies in different frequency parts of a carrier. In BWP adaptation, for example, a BWP with a smaller bandwidth is used if there is no data and is dynamically switched to BWP with a wider bandwidth when data transmission occurs. Also, BWP switching between BWPs with different numerologies is specified.

For EN-DC, simultaneous UL transmissions across LTE and NR is supported; however, a single UL transmission is also enabled in some cases for difficult band combinations, e.g., band combinations where inter-modulation issue happens.

NR-LTE co-existence

NR is designed to allow co-existence with LTE. To achieve this, for instance, higher-layer signaling can be used to configure reserved resources and LTE CRS-related information to be rate-matched around. It is further possible to configure an LTE carrier that overlaps with an NR carrier with MBSFN subframe configurations to minimize the impact on NR. The corresponding backhaul signaling between eNodeB and gNodeB is also specified. For the UL it is possible to operate with a 7.5 kHz shift so that LTE and NR subcarriers are aligned. The above enables operations of NR and LTE on the same frequency. In addition, PDSCH resource mapping in NR allows LTE, NB-IoT and LTE-M to operate on the same frequency as NR.

Supplementary Downlink

Similar as within LTE the concept of SDL is supported wherein a carrier is pure DL carrier without any associated UL carrier. An SDL carrier can be aggregated with CA together with another DL and UL carrier.

Supplementary Uplink

In case of Supplementary Uplink (SUL), the UE is configured with 2 ULs for one DL of the same cell, and uplink transmissions on those two ULs are controlled by the network to avoid overlapping PUSCH/PUCCH transmissions in time. Overlapping transmissions on PUSCH are avoided through scheduling while overlapping transmissions on PUCCH are avoided through configuration (PUCCH can only be configured for only one of the 2 ULs of the cell). In addition, initial access is supported in each of the uplink.

UL TPC

NR UL power control is designed to allow dynamic power adjustment and multiple power control processes, e.g., for switching. gNB executes it by signalling an index, which has pre-configured linkage with a reference signal (RS) for pathloss calculation, a power control parameter set, and a closed-loop. In addition, for LTE-NR coexistence, power sharing mechanism between RATs was introduced. When UE supports dynamic power sharing, power allocation is dynamically adjusted on condition that the total transmission power never exceeds allowed value. Otherwise, the total power is semi-statically split for the two RATs by gNB configuration.

2.3	Higher layer aspects

2.3.1	Layer 2 related aspects

2.3.1.1	MAC sublayer

The main services and functions of the MAC sublayer include:

-	Mapping between logical channels and transport channels;

-	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

-	Scheduling information reporting;

-	Error correction through HARQ (one HARQ entity per cell in case of CA);

-	Priority handling between UEs by means of dynamic scheduling;

-	Priority handling between logical channels of one UE by means of logical channel prioritisation;

-	Padding.

2.3.1.2	RLC sublayer

The main services and functions of the RLC sublayer depend on the transmission mode and include:

-	Transfer of upper layer PDUs;

-	Sequence numbering independent of the one in PDCP (UM and AM);

-	Error Correction through ARQ (AM only);

-	Segmentation (AM and UM) and re-segmentation (AM only) of RLC SDUs;

-	Reassembly of SDU (AM and UM);

-	Duplicate Detection (AM only);

-	RLC SDU discard (AM and UM);

-	RLC re-establishment;

-	Protocol error detection (AM only).

2.3.1.3	PDCP sublayer

The main services and functions of the PDCP sublayer for the user plane include:

-	Sequence Numbering;

-	Header compression and decompression: ROHC only;

-	Transfer of user data;

-	Reordering and duplicate detection;

-	PDCP PDU routing (in case of split bearers);

-	Retransmission of PDCP SDUs;

-	Ciphering, deciphering;

-	PDCP SDU discard;

-	PDCP re-establishment and data recovery for RLC AM;

-	Duplication of PDCP PDUs.

The main services and functions of the PDCP sublayer for the control plane include:

-	Sequence Numbering;

-	Ciphering, deciphering and integrity protection;

-	Transfer of control plane data;

-	Reordering and duplicate detection;

-	Duplication of PDCP PDUs.

2.3.2	RRC related aspects

The main services and functions of the RRC sublayer include:

-	Broadcast of System Information related to AS and NAS;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

2.3.3	Network interface related aspects

Additional functions of X2AP

The X2AP protocol provides the following functions additionally:

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another en-gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Secondary RAT Data Usage Report. This function allows eNB to get the uplink and downlink data volumes for the Secondary RAT on a per E-RAB basis.

Additional functions of S1 AP

The S1AP protocol provides the following functions additionally:

-	Report of Secondary RAT data volumes function. The functionality enables the eNB to report Secondary RAT data usage information in case of EN-DC.

gNB-CU/gNB-DU Architecture and F1 interface

F1 interface was specified for the case where en-gNB is consisted of gNB-CU and gNB-DU; gNB-CU is defined as a logical node hosting RRC and PDCP protocols of the en-gNB. And, gNB-DU is defined as a logical node hosting RLC, MAC and PHY layers of the en-gNB.

Functions of F1 AP

The F1AP protocol provides the following functions:

-	F1 interface management function

-	System Information management function

-	F1 UE context management function

-	RRC message transfer function

-	Paging function

2.4	Frequency aspects

While the physical and higher layers are designed as frequency agnostic, two separate radio performance requirements are specified for two frequency ranges (FRs) which are FR 1 is sub-6 GHz range (450 - 6000 MHz) and FR2 is mmWave range (24250 - 52600 MHz). Both ranges were identified in Rel.15 NR SI. The RF and RRM requirements are developed for respective frequency ranges. One big difference between the requirements in FR1 and FR2 is testing methodology. Both conducted and over-the-air (OTA) methodologies can be utilized in FR1, but only OTA methodology can be utilized in FR2. With respect to bands for NR, the NR bands are specified considering market demands and categorized in the following four types in the WI. Note that all the NR bands are defined with a prefix “n” to distinguish them from the bands for the other RATs.

- LTE “refarming” band: The bands have the corresponding LTE bands. For example, NR band n7 is corresponding to LTE band 7. Hence, the bands would be likely to be used by “reframing” the exiting LTE bands.

- NR new bands in FR1: Completely new frequency bands for NR in FR1 whose corresponding LTE bands do not exist.

- NR new bands in FR2: This is new frequency band for NR in FR2. Note that there are no specified LTE bands in FR2.

- Supplemental uplink (SUL) / downlink (SDL) band: SUL/SDL band has only uplink/downlink frequency, and it can be deployed with other type of NR bands as described in section 2.2.

Note that the ranges {65 - 256} and {257-512} are reserved as band number for NR new bands in FR1 and FR2, respectively. A band number will be assigned to a new frequency range on a “first come first served” basis from the reserved frequency range. During Rel.15 NR WI, 3 bands in FR1 and 4 bands in FR2 were defined as NR new bands considering the spectrum allocation plan in each region/country as shown in Fig. 8. In addition, some LTE bands were defined as LTE refarming band, and uplink frequencies of some LTE bands were defined as SUL/SDL band. All NR bands specified in this WI are summarized in Table 2. Especially for above new NR bands which have wider bandwidth than LTE, wider channel bandwidths, i.e. 100MHz in FR1 and 400MHz in FR2 at maximum, were defined to improve the spectrum efficiency and reduce the number of component carriers in case of NR CA operation. In addition, the RF requirements for NR bands and band combinations of EN-DC and NR CA were developed based on the market demands.

Figure 8: New NR bands in FR1 and FR2 in Rel.15 NR

Table 2 NR bands specified in this WI

| NR operating band | Uplink (UL) operating band | Downlink (DL) operating band | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780MHz | N/A | SUL |
| n257 | 26500 MHz – 29500 MHz | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | 27500 MHz – 28350 MHz | TDD |

3	References

[1]	3GPP RP-180536, “Revision of WI: New Radio Access Technology,” NTT DOCOMO, May 2018.

[2]	3GPP RP-181172	CRs to 38.2xx NR specs capturing RAN1#92b & RAN1#93 agreements	RAN1

### Annex: Abbreviation

NR	NR Radio Access

NSA	Non Stand-Alone

PBCH	Physical Broadcast Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PRACH	Physical Random Access Channel

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

SSS	Secondary Synchronisation Signal