3GPP TSG RAN meeting #81										RP-181618

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.16

| WI / SI Name | Increased number of E-UTRAN data bearers |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | No |  | Testing part: | No |
| Acronym | INOBEARRAN |  |  |  |  |  |  |
| Unique ID | 780070 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-180569 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: |  |  |

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | SAYENKO, Alexander |
|  | Company | Samsung |
|  | Email | alex.sayenko@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

## 2.2	RAN2

#### 2.2.1	Agreements

-	MAC header structure was agreed to accommodate more logical channel IDs to support up to 15 DRBs

-	ASN.1 extensions to support up to 15 DRBs were introduced

-	UE capability signalling to support up to 15 DRBs was added

-	RAN WG2 CRs for TS 36.321 and 36.306 were agreed already at RAN#80 meeting, while TS 36.331 CR was postponed.

-	During the RAN#103 meeting, ASN.1 was further reviewed and some minor changes were introduced

#### 2.2.2	Remaining Open issues

There are no remaining open issues in RAN2.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN WG3 discussed and concluded that no further specification changes are needed. Existing specifications can support up to 15 DRBs and existing signalling can handle error cases when a target eNB does not support as many DRBs as were configured at the source eNB.

#### 2.3.2	Remaining Open issues

There are no remaining open issues in RAN3.

## 2.4	RAN4

## 2.5	RAN5

## 2.6	RAN6

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| Increased number of radio bearers | INOBEAR | SA2: a.bennett@samsung.com | INOBEARRAN | RAN2: alex.sayenko@samsung.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1]	R2-1804493	Workplan for increased number of E-UTRAN data bearers	Samsung

[2]	R2-1804494	MAC specification impact analysis	Samsung

[3]	R2-1804495	RRC specification impact analysis	Samsung

[4]	R2-1804498	Introduction of increased number of E-UTRAN data bearers 	Samsung

[5]	R2-1804499	Introduction of increased number of E-UTRAN data bearers 	Samsung

[6]	R2-1804500	Introduction of increased number of E-UTRAN data bearers	Samsung

[7]	R2-1804890	Supporting larger number of DRBs for LTE	Intel Corporation

[8]	R2-1805001	Draft Response LS on INOBEAR	Samsung	LS out

[9]	R2-1805118	The impact on user plane to extend the DRB number	ZTE, Sanechips	e

[10]	R2-1805119	Introduction of DRB number extension	ZTE, Sanechips

[11]	R2-1805372	LCID space extension	Ericsson

[12]	R2-1805373	LCID space extension	Ericsson

[13]	R2-1805374	Introduction of INOBEAR in 36.331	Ericsson

[14]	R2-1805375	Introduction of INOBEAR in 36.306	Ericsson

[15]	R2-1805376	Draft LS reply on INOBEAR	Ericsson

[16]	R2-1806036	Extended number of bearers for LTE	LG Electronics Inc.

[17]	R2-1806037	CR to 36.321 on introduction of extended LCID field	LG Electronics Inc.

[18]	R3-182414 Response LS on INOBEAR

[19]	R3-182410 Reply LS on INOBEAR

[20]	R3-182366 Rply LS on INOBEAR

[21]	R3-181859 Homogeneous Issue in INOBEAR Samsung

[22]	R2-1806602	Reply LS on INOBEAR

[23]	R2-1806661	Reply LS on INOBEAR

[24]	R2-1807065	Remaining issues for INOBEAR	Ericsson

[25]	R2-1807939	List of open issues	Samsung

[26]	R2-1807940	Introduction of increased number of E-UTRAN data bearers	Ericsson, Samsung

[27]	R2-1807941	Introduction of increased number of E-UTRAN data bearers	Samsung

[28]	R2-1807942	Introduction of increased number of E-UTRAN data bearers	Samsung

[29]	R2-1808636	Remaining issues on extended LCID	LG Electronics Inc.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template