3GPP TSG RAN meeting #81										RP-181675

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.7.4

| WI / SI Name | DL MIMO efficiency enhancements for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_DL-MIMO_EE |  |  |  |  |  |  |
| Unique ID | 800086 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181485 |  |  |  |  |  |  |
| Target Completion Date | Core part: 12/2019 |  |  |  | Performance part: 12/2019 |  |  |
| Is the WI/SI complete? | Core part: No |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Yubo YANG |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94:

31 ([1-31]) contributions were submitted to RAN1#94 meeting and following were achieved:

For additional SRS symbols:

Agreement

The work for additional SRS symbols in this WI should consider the following scenarios

- TDD for non-CA

- TDD only CA

- FDD-TDD CA

RAN1 will strive for a common framework for all scenarios

Agreement

The time location of possible additional SRS symbols in one normal UL subframe for a cell include the following

- Option 1: All symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Option 2: All symbols in one subframe can be used for SRS from cell perspective

- Option 3: A subset of symbols in one slot can be used for SRS from cell perspective

  - E.g., The other slot in the subframe may be used for PUSCH transmission for sTTI-capable UE.

- Other options are not precluded

- FFS whether it has spec impacts or not for cell-specific configuration

- Companies are encouraged to study and evaluate considering

  - Performance (eg, DL performance improvement, degradation on UL performance including legacy UL for legacy UEs)

  - Specification impact

  - UE complexity

  - RF impact

- RAN1 will finalize on the performance metrics and evaluation methodology in RAN1#94bis

- System level parameters, evaluation methodology, and the need for system level evaluation can be discussed in the next RAN1 meeting

Conclusion

For link level evaluation, companies are encouraged to submit evaluations results based on following assumptions as a starting point, which is to be checked in next RAN1 meeting.

| ​Parameter | Value |
| --- | --- |
| Carrier frequency​ | 2 GHz​ |
| Bandwidth​ | 20 MHz​ ( or 10MHz) |
| Duplex​ | TDD, LTE Configuration 2 |
| Subcarrier spacing​ | 15 kHz​ |
| Polarization​ | eNB: X pole (+/-45°)​ UE: + pol (0/90°)​ |
| Macro eNB antenna array | (M,N,P) = (2,4,2)(M, N, P) = (2, 4, 2) with vertical spacing 0.8 and horizontal spacing 0.5 |
| Macro eNB antenna element model​ (only applicable to CDL and other details FFS) | Elevation beamwidth = 65º​ |
|  | Azimuth beamwidth = 65º​ |
| UE Speed​ | 3km/h, 30km/h, 60km/h​ |
| UE antenna | 2 receive antennas Isotropic (0 dBi) ​ |
| CP | Normal CP |

And for virtual cell ID for SRS:

Agreement

For SRS, nIDRS=nIDSRS if nIDSRS nIDSRS is configured by higher layer, otherwise nIDRS=NIDcell nIDRS=NIDcell, where nIDRS is within the range from 0 to 503.

#### 2.1.2	Remaining Open issues

- Decide the time location of possible additional SRS symbols in one normal UL subframe for a cell.

- The configuration and transmission of additional SRS symbols.

- Collision handling of additional SRS symbols.

- Configuration granularity of virtual cell ID for SRS.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

- Specify higher layer support of enhancements for additional SRS symbols and virtual cell ID for SRS.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

- Specify RF requirement of UE for additional SRS symbols and virtual cell ID for SRS.

- Specify the necessary UE performance requirements.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1808138	Work plan of DL MIMO efficiency enhancements for LTE	Huawei, HiSilicon

- R1-1808368	Discussion on SRS capacity and coverage enhancement	CATT

- R1-1809584	Feature summary on additional SRS symbols for LTE		Huawei, HiSi

- R1-1809659	Outcome of offline discussion for additional SRS symbols	Huawei, HiSi

- R1-1808136	Discussion on SRS transmission on more than one symbols of normal UL subframe	Huawei, HiSilicon

- R1-1808192	Discussion on additional SRS symbols	ZTE

- R1-1808227	Discussion on SRS enhancements	vivo

- R1-1808369	Consideration on multiple SRS symbols in a subframe	CATT

- R1-1808445	Views on additional symbols for SRS	Mitsubishi Electric Co.

- R1-1808480	Enhancements on SRS resource allocation	LG Electronics

- R1-1808547	Additional SRS symbols	Lenovo, Motorola Mobility

- R1-1808662	Additional SRS symbols	Intel Corporation

- R1-1808742	Discussions on additional SRS symbols	Samsung

- R1-1809037	Additional SRS symbols	Qualcomm Incorporated

- R1-1809191	On multi-symbol SRS enhancements	Ericsson

- R1-1809215	A potential scenario for additional SRS symbols	SoftBank Corp.

- R1-1809376	Discussion on introduction of additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-1809585	Feature summary on virtual cell ID for SRS	Huawei, HiSilicon

- R1-1808137	On virtual cell ID for SRS sequence generation	Huawei, HiSilicon

- R1-1808193	Discussion on virtual cell ID for SRS	ZTE

- R1-1808446	Views on SRS virtual cell ID	Mitsubishi Electric Co.

- R1-1808481	Discussion on VCID for SRS sequence	LG Electronics

- R1-1808548	Virtual cell ID for SRS	Lenovo, Motorola Mobility

- R1-1808663	Virtual cell ID support for SRS	Intel Corporation

- R1-1808743	Discussions on virtual cell ID for SRS	Samsung

- R1-1808957	Virtual Cell ID for SRS Capacity Enhancements	MediaTek Inc.

- R1-1809038	Virtual cell ID for SRS	Qualcomm Incorporated

- R1-1809192	On virtual cell ID for SRS	Ericsson

- R1-1809377	Introduction of virtual cell id for SRS	Nokia, Nokia Shanghai Bell

- R1-1809193	Simulation assumptions and evaluation methodology for SRS enhancements	Ericsson

- R1-1809194	Possible enhancements to Advanced CSI reporting	Ericsson

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template