| Features | # | Feature group |  |  | Components |  |  | Prerequisite feature groups (listed in this sheet only) |  |  | Need for gNB to know whether the feature is supported by the UE (what happens if gNB does not know?) |  | Consequences if the feature is not supported by the UE |  |  |  | Type (see R2-1712078) |  | Need of FDD/TDD differentiation |  |  |  | Need of FR1/FR2 differentiation |  |  | RAN5 implication |  |  | Note |  |  | Responsible WG |  | RAN WG recommendation |  | TSG-RAN decision / [Intel’s view] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL |  |  | 1) CP-OFDM for DL 2) CP -OFDM for UL |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 0-2 | DFT-S-OFDM waveform for UL |  |  | Transform precoding for single-layer PUSCH |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 0-3 | DL modulation scheme |  |  | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | RAN4 will check 64QAM modulation for FR2 |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 0-4 | UL modulation scheme |  |  | 1) QPSK modulation 2) 16QAM modulation |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 0-5 | pi/2-BPSK for PUSCH |  |  | pi/2-BPSK for PUSCH |  |  | 0-2 |  |  | Yes |  | pi/2-BPSK for PUSCH is not possible |  |  |  | [Type 3] RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | RAN4 will discuss if it is per band, common for all bands or FR1/2 |  |  | RAN4 |  |  |  |  |
|  | 0-6 | 64QAM for PUSCH |  |  | 64QAM for PUSCH |  |  |  |  |  | Yes |  | 64QAM for PUSCH is not possible |  |  |  | [Type 3] RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | RAN4 will discuss if it is per band or common for all bands |  |  | RAN4 |  |  |  |  |
|  | 0-7 | 256QAM for PDSCH |  |  | 256QAM for PDSCH |  |  |  |  |  | Yes |  | 256QAM for PDSCH is not possible |  |  |  | [Type 3] RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | RAN4 will discuss if it is per band or common for all bands |  |  | RAN4 |  |  |  |  |
|  | 0-8 | 256QAM for PUSCH |  |  | 256QAM for PUSCH |  |  |  |  |  | Yes |  | 256QAM for PUSCH is not possible |  |  |  | [Type 3] RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | RAN4 will discuss if it is per band or common for all bands |  |  | RAN4 |  |  |  |  |
|  | 0-9 | Subcarrier spacings and FFT size in conjunction with supportable BW with normal CP |  |  | 1) 15kHz 2) 30 kHz 3) 60 kHz 4) 120 kHz |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 [Type 3] RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | It is up to RAN4 decision Baseband processing (memory) in CA combination related as well as RF (SCS support is per band between sub6 and mmWave) |  |  | RAN4 |  |  |  |  |
|  | 0-10 | Extended CP |  |  | Extended CP |  |  | 0-9 (component 3; SCS60) |  |  | Yes |  |  |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 0-11 | pi/2-BPSK for PUCCH format 3/4 |  |  | pi/2-BPSK for PUCCH format 3/4 |  |  | 0-2 |  |  | Yes |  | pi/2-BPSK for PUCCH format 3/4 is not possible |  |  |  | Type 3 RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | RAN4 will discuss if it is per band, common for all bands, or FR1/2 |  |  | RAN4 |  |  |  |  |
|  | 0-12 | Non-contiguous UL PRB CP-OFDM per CC |  |  | When VRB-to-PRB mapping is used, PRB is after interleaving |  |  | 0-1 |  |  | Yes |  |  |  |  |  | RAN4 to decide |  | RAN4 to decide |  |  |  | RAN4 to decide |  |  |  |  |  | It is up to RAN4 to decide |  |  | RAN4 |  |  |  |  |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures |  |  | 1) RACH preamble format 2) SS block based RRM measurement [3) RMSI/broadcast OSI reception] [4] Paging] |  |  |  |  |  | [Yes, or No] |  |  |  |  |  | N.A. |  | No need |  |  |  | No need |  |  |  |  |  | RAN2 to check components 3 and 4 for SA and NSA applicability |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 1-1a | [On demand based system information] |  |  |  |  |  |  |  |  | [Yes, or No] |  |  |  |  |  |  |  | [Yes] |  |  |  | [No need] |  |  |  |  |  | RAN2 to check |  |  | RAN2 |  |  |  |  |
|  | 1-3 | SS block based SINR measurement (SS-SINR) |  |  | 1) SS-SINR measurement |  |  | 1-1 |  |  | Yes |  | Not support SS-SINR measurement |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 1-4 | SS block based RLM |  |  | 1) SS block based RLM |  |  | 1-1 |  |  | Yes |  | Not support SS block based RLM |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 1-5 | CSI-RS based RRM measurement with associated SS-block |  |  | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement |  |  | 1-1, CSI-RS |  |  | Yes |  | Not support CSI-RSRP and CSI-RSRQ measurement |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement |
|  | 1-5a | CSI-RS based RRM measurement without associated SS-block |  |  | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed |  |  | 1-1, CSI-RS |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling Note: This does not discourage RAN4 to complete their work Note: there is expectation that RAN4 will complete the corresponding RRM measurement |
|  | 1-6 | CSI-RS based RS-SINR measurement |  |  | 1) CSI-SINR measurement |  |  | 1-1 1-5 |  |  | Yes |  | Not support CSI-SINR measurement |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 1-7 | CSI-RS based RLM |  |  | 1) CSI-RS based RLM |  |  | 1-1, CSI-RS |  |  | Yes |  | Not support CSI-RS based RLM |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Mandatory with capability signaling |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP |  |  |  |  |  | 1-4 and 1-7 |  |  | Yes |  | UE does not support RLM based on a mix of SS block and CSI-RS signals |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | [Mandatory /optional with capability signaling] |  | Optional with capability signaling |
|  | 1-9 | CSI-RS based contention free RA for HO |  |  |  |  |  | 1-1 CSI-RS 1-5 or 1-5a |  |  | Yes |  | UE does not support CSI-RS based contention free RA for HO |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 1-10 | Support of SCell without SS/PBCH block |  |  | 1) Support SCell without SS/PBCH block |  |  | 1-1 |  |  | Yes |  | Not support SCell without SS/PBCH |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of SCell without SS/PBCH block |  |  | RAN1 |  | Mandatory with capability signaling for intra-band CA This feature is not supported for inter band CA |  | Mandatory with capability signaling for intra-band CA This feature is not supported for inter band CA |
|  | 1-11 | Support of CSI-RS RRM measurement for SCell without SS/PBCH block |  |  |  |  |  | 1-10 |  |  | Yes |  |  |  |  |  | Type 3 |  | N. A. |  |  |  | N.A. |  |  |  |  |  | RAN4 to check |  |  |  |  |  |  | Optional with capability signaling |
|  | 1-12 | E-UTRA RS-SINR measurement |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | [No need] |  |  |  |  |  | RAN2 to decide FR1/FR2 differentiation |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
| 2. MIMO | 2 -1 | Basic PDSCH reception |  |  | 1. Data RE mapping 2. Single layer transmission 3. Support one TCI state |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 2-2 | PDSCH beam switching |  |  | 1. Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. |  |  | 2-1 |  |  | Yes |  |  |  |  |  | Type-3 |  | No need |  |  |  | Applicable only for FR2 |  |  |  |  |  |  |  |  | RAN1 |  | Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |  |  |
|  | 2-3 | PDSCH MIMO layers |  |  | 1. Supported maximal number of MIMO layers |  |  | 2-1 |  |  | Yes |  | Only one layer is supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | Candidate values: {1,2,4,8} |  | For single CC standalone NR, it is mandatory with capability signaling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signaling. It is not expected that there is a signaling change (i.e. signaling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
|  | 2-4 | TCI states for PDSCH |  |  | 1. Support number of active TCI states per BWP per CC , including control and data 2. maximum number of configured TCI states per CC for PDSCH |  |  | 2-1 |  |  | Yes |  | Only one TCI state can be supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: UE is required to track only the active TCI states |  |  |  |  | Component-1: Candidate value set: {1, 2, 4, 8 } Component-2: candidate value set: {4, 8, 16, 32, 64, [128]} UE is mandated to signal 64 |  | Component 1: Only Value 1 is Mandatory; Others are optional |
|  | 2-4a (new FG) | Additional active TCI state for PDCCH |  |  | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH |  |  | 2-1, 2-4 only if Component-1 of 2-4 is set to 1 |  |  | Yes |  | No additional TCI state for control |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: Only applicable if Component-1 of 2-4 is set to 1 |  |  |  |  | Mandatory with capability signalling |  | Mandatory with capability signalling |
|  | 2-5 | Basic downlink DMRS for scheduling type A |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. |  |  | 2-1 |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | conditioned to whether PDSCH scheduling type A is supported |  |  |  |  | Mandatory without UE capability(condition to scheduling capability) |  | Mandatory without UE capability(condition to scheduling capability) |
|  | 2-6 | Basic downlink DMRS for scheduling type B |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | conditioned to whether PDSCH scheduling type B is supported |  |  |  |  | Mandatory without UE capability (condition to scheduling capability) |  | Mandatory without UE capability (condition to scheduling capability) |
|  | 2-6a | Support 1+2 DMRS (downlink) |  |  | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  |  | 2-6 |  |  | Yes |  | 1 FL + 2 additional DMRS for more than one port is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability signaling |  | Mandatory with capability signaling |
|  | 2-7 | Supported 2 symbols front-loaded DMRS(downlink) |  |  | 1. Support 2 symbols FL-DMRS |  |  | 2-5 |  |  | Yes |  | 2 FL DMRS is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS(downlink) |  |  | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  |  | 2-5 |  |  | Yes |  | 2 FL DMRS + one additional 2-symbols DMRS is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) |  |  | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  |  | 2-5 |  |  | Yes |  | 1 symbol FL DMRS and 3 additional DMRS symbols is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-10 | Support DMRS type (downlink) |  |  | Support DMRS {type 1, both type 1 and type 2} |  |  |  |  |  | Yes |  | Only the mandatory DMRS type(s) are supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | RAN1 will further discuss which Type will be mandatory or both type will be mandatory |  | Type 1 is mandatory with capability signaling. Type 2 is optional with capability signaling. |
|  | 2-11 | Downlink dynamic PRB bundling (downlink) |  |  | 1. Support dynamic PRB bundling indication via DCI Note: Support of semi-static PRB bundling--mandatory |  |  | 2-1 |  |  | Yes |  | No support of dynamic PRB bundling |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-12 | Basic PUSCH transmission |  |  | Data RE mapping Single layer (single Tx) transmission |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Mandatory without UE capability |  | Mandatory without UE capability |
|  | 2-13 | PUSCH codebook coherency subset |  |  | Proposal: 1. Supported codebook coherency subset type: Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |  |  | 2-12 |  |  | Yes |  | Only non-coherent codebook subset is supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Candidate values: {non-coherent, partial-non-coherent, full-coherent} |  | Only non-coherent is mandatory and others are optional |
|  | 2-14 | Codebook based PUSCH MIMO transmission |  |  | 1. Supported codebook based PUSCH MIMO with maximal number of supported layers 2. Supported max number of SRS resource per set (SRS set use is configured as for codebook). |  |  | 2-13 |  |  | Yes |  | Uplink codebook based MIMO (with >1 Tx port) transmission is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Component-1: Candidate value: {no-codebook based MIMO, 1, 2, 4} Component-2 Candidate value: {1, 2} |  |  |
|  | 2-15 | non-codebook based PUSCH transmission |  |  | 1. Maximal number of supported layers (non-codebook transmission scheme): |  |  | 2-12 |  |  | Yes |  | No support of non-codebook based PUSCH transmission |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Component-1 candidate values: {“No non-codebook based MIMO”, 1, 2, 4} |  | Optional with capability signaling |
|  | 2-15a | Association between CSI-RS and SRS |  |  | 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRS-AssocCSIRS” 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. |  |  | 2-15 |  |  | Yes |  | Association between CSI-RS and SRS is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} |  | Optional with capability signaling |
|  | 2-15b | CSI-RS processing framework for SRS |  |  | 1. Maximum number of periodic SRS resources associated with CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4. UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5. UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. |  |  | 2-15a |  |  | Yes |  | Association between CSI-RS and SRS is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. |  |  |  |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |  |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbols 3. Support 1 symbol FL DMRS and 2 additional DMRS symbols |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | conditioned to whether PUSCH scheduling type A is supported |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |
|  | 2-16a | Basic uplink DMRS for scheduling type B |  |  | 1. Support 1 symbol FL DMRS without additional symbol(s) 2. Support 1 symbol FL DMRS and 1 additional DMRS symbol |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | conditioned to whether PUSCH scheduling type B is supported |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |
|  | 2-16b | Support 1+2 DMRS (uplink) |  |  | 1. Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port |  |  | 2-16a and 2-16 |  |  | Yes |  | 1+2 DMRS for more than one port is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability signaling |  | Mandatory with capability signaling |
|  | 2-17 | Support DMRS type (uplink) |  |  | Support DMRS {type 1, both type 1 and type 2 } |  |  | 2-16 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | At least type-1 is mandatory. |  | Support both type 1 and type 2 are mandatory with capability signaling |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) |  |  | 1. Support 2 symbols FL-DMRS |  |  | 2-16 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) |  |  | 1. Support 2-symbol FL DMRS + one additional 2-symbols DMRS |  |  | 2-16 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) |  |  | 1. Support 1 symbol FL DMRS and 3 additional DMRS symbols |  |  | 2-16 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-20 | Beam correspondence |  |  | 1. Support Beam correspondence |  |  |  |  |  | Yes |  | Beam correspondence is not supported |  |  |  | Type 1 |  | No need |  |  |  | N.A. |  |  |  |  |  | Note: Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports |  |  |  |  | [Mandatory at least for FR2] |  | Decision to be deferred until RAN4 completes its work and responsible WG to be moved from RAN1 to RAN4. No support of beam correspondence in FR1 for Rel-15 |
|  | 2-20a | Configured spatial relations |  |  | Maximum number of configured spatial relations per CC for PUCCH and SRS |  |  |  |  |  | Yes |  | Only one configured spatial relation for UL signals can be supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: Only applicable for FR2 |  |  |  |  | candidate value set: {4, 8, 16, 32, 64, 96} |  |  |
|  | 2-20b | Active spatial relations |  |  | Maximum number of active spatial relations with regarding to PUCCH and SRS for PUSCH, per BWP per CC NOTE: ‘active spatial relation’ refers to the finally selected spatial relation for each PUCCH resource or SRS resource for PUSCH via MAC-CE or RRC. |  |  | 2-20a |  |  | Yes |  | Only one active spatial relation for UL control and data can be supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: Only applicable for FR2 FFS whether DL RS in the active TCI states and the active spatial relation info can be different if 2-20b is set to 1 and if different whether or not new UE behavior is needed |  |  |  |  | Candidate value set: {1, 2, 4, 8, 14} |  |  |
|  | 2-20c | Additional active spatial relation for PUCCH |  |  | Support one additional active spatial relation for PUCCH |  |  | 2-20b |  |  | Yes |  | No additional active spatial relation for control |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: Only applicable if 2-20b is set to 1 |  |  |  |  | Mandatory with capability signaling |  |  |
|  | 2-21 | Periodic beam report |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  |  |  |  | Yes |  | No support of periodic L1-RSRP report |  |  |  | Type 1 |  | No need |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability at least for FR2 FFS: for FR1 |  | Mandatory with UE capability at least for FR2 FR1: Optional with capability signaling |
|  | 2-22 | Aperiodic beam report |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | No support of aperiodic L1-RSRP report |  |  |  | Type 1 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability at least for FR2 |  | Mandatory with UE capability at least for FR2 |
|  | 2-23 | Semi-persistent beam report on PUCCH |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot(or piggybacked on a PUSCH) 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot(or piggybacked on a PUSCH) |  |  |  |  |  | Yes |  | No support of PUCCH based SP L1-RSRP report |  |  |  | Type 1 |  | N.A. |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-23a | Semi-persistent beam report on PUSCH |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | No support of PUSCH based SP L1-RSRP report |  |  |  | Type 1 |  | N.A. |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-24 | SSB/CSI-RS for beam measurement |  |  | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 1a. The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS |  |  | 2-21, 2-22 or 2-23, 2-23a |  |  | Yes |  | RSRP measurement is not supported |  |  |  | Type 1 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signaling. Component-1a, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signaling. |  |  |
|  | 2-25 | Beam reporting timing |  |  | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  |  | 2-24 |  |  | Yes |  | Beam reporting time capability is not known by gNB |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1/4 |  | Candidate value sets: X1 is {2, 4, 8} X2 is {4,8, 14} X3 is { 8,14, 28} X4 is{14,28, 56} |  |  |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" |  |  | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  |  |  |  |  | Yes |  | Rx beam switching is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Support Rx beam switching is mandatory for bands at least > 6GHz Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |  | Mandatory with UE capability at least for FR2 |
|  | 2-27 | Beam switching |  |  | 1. Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC BB_Total,. This number is defined as per SCS Note: it is assumed that spec enable the possibility to restrict the same beam across intra-band CCs Note: this FG is not applicable to FR1 |  |  | 2-24 |  |  | Yes. |  | No resetriction on the maximum number of Tx+Rx beam change for a slot |  |  |  | Type 1 |  | N.A. |  |  |  | N.A |  |  |  |  |  |  |  |  |  |  | Candidate value set: {4, 7, 14} |  |  |
|  | 2-28 | A-CSI-RS beam switching timing |  |  | 1. Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2 corresponding to 60,120 kHz SCS. |  |  | 2-27 |  |  | Yes |  | No recommendation on the desired beam switching timing |  |  |  | Type1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | [Note: any value larger than 56 is not supported in RRC configuration now. ] |  |  |  |  | Only applicable to FR2 Candidate values: {14, 28, 48, 224, 336} |  |  |
|  | 2-29 | Non-group based beam reporting |  |  | 1. Support of non-group based RSRP reporting with N_max RSRP values reported |  |  | 2-24 |  |  | Yes |  | Non-group based beam reporting is not supported |  |  |  | Type1 |  | N.A |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | candidate value set is {1, 2,4} |  | Mandatory with capability signaling |
|  | 2-29a | Group based beam reporting |  |  | 1. Support of beam group RSRP reporting for group of 2 beams |  |  |  |  |  | Yes |  | Group based beam reporting is not supported |  |  |  | Type1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-30 | Uplink beam management |  |  | 1 Support of SRS based beam management 2. Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3. Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  |  |  |  |  | Yes |  | Uplink beam management is not supported |  |  |  | Type1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |  |  |
|  | 2-31 | Beam failure recovery |  |  | 1. Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2. Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3. Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. 4. COREST recoveryControlResourceSetId if configured |  |  |  |  |  | Yes |  | Beam failure recovery is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Component 4) is in addition to component 1) of 3-1 |  |  |  |  | Component-1 candidate value set: {from 1 to 64} Component-2 candidate: {from 1 to 64} Component-3: Candidate value set is: {from 1 to 256} [UE is mandate to support at least 64.] |  | Optional with capability signaling (contents of the components need to be revised in RAN1#94bis to align with the current specifications) |
|  | 2-32 | Basic CSI feedback |  |  | 1. Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2. 2Tx codebook for FR1 and FR2 3. 4Tx codebook for FR1 4. 8Tx codebook for FR1 when configured as wideband CSI report 5. p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 6. p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) 7. a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |
|  | 2-32a | Semi-persistent CSI report on PUCCH |  |  | 1. Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot(or piggybacked on a PUSCH) s 2. Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  |  |  |  |  | Yes |  | SP-CSI on PUCCH is not supported |  |  |  | Type 4 |  | No |  |  |  | No |  |  |  |  |  |  |  |  | RAN1 |  | Optional |  | Optional with capability signaling |
|  | 2-32b | Semi-persistent CSI report on PUSCH |  |  | 1. Support report on PUSCH |  |  |  |  |  | Yes |  | SP-CSI on PUSCH is not supported |  |  |  | Type 4 |  | No |  |  |  | No |  |  |  |  |  |  |  |  | RAN1 |  | Optional |  | Optional with capability signaling |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback |  |  | 1. Supported max # of configured NZP-CSI-RS resources per CC, 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC 4. Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs |  |  | 2-32 |  |  | Yes |  | CSI acquisition is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values. |  |  |  |  | Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {8, 16, 24, …, 248, 256} |  |  |
|  | 2-33a | Supported PDSCH RE-mapping patterns Note: this FG will be moved to 5-x family |  |  | 1. Supported max # of RE mapping patterns, each pattern can be described as a RS resource (including NZP/ZP CSI-RS and CRS and bitmap configured in 5-26/27) Note: patterns are counted as per symbol per CC 2. Supported max # of RE mapping patterns, each pattern can be described as a RS resource (including NZP/ZP CSI-RS and CRS and bitmap configured in 5-26/27) Note: patterns are counted as per slot per CC |  |  |  |  |  | Yes |  | PDSCH RE mapping is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | candidate values: {10, 20} for FR1 {6, 20} for FR2 [Compponent-2 candidate values: {from 16: 16: 256 16} for FR1 {16: 16: 256} for FR2] |  |  |
|  | 2-33b (New FG) | SP CSI-RS |  |  | 1. Support SP CSI-RS |  |  | 2-1 |  |  | Yes |  |  |  |  |  | Type 4 |  | No |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Mandatory with capability signaling |
|  | 2-33c (New FG) | SP CSI-IM |  |  | 2. Support SP CSI-IM |  |  | 2-1 |  |  | Yes |  |  |  |  |  | Type 4 |  | No |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 2-34 | NZP-CSI-RS based interference measurement |  |  | 1. Support NZP-CSI-RS based interference measurement |  |  | 2-33 |  |  | Yes |  | NZP-CSI-RS based interference measurement is not supported |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | [Optional] |  | Optional with capability signaling |
|  | 2-35 | CSI report framework |  |  | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. Support of advanced CSI process table (the Minimum duration Z, Z’ for processing a CSI) [Conditioned to the completion of advanced CSI process] 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. |  |  | 2-32 |  |  | Yes |  | CSI report is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update Note: The CSI report setting in component 1-3 includes the beam report and CSI report Note: The CSI report in component 5 includes the beam report and CSI report |  |  |  |  | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4: signaling is a bitmap of size 8 Component-5: candidate values: {from 5 to 32} |  |  |
|  | 2-36 | Type I single panel codebook |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s) 3. Max # of CSI-RS resource in a resource set |  |  | 2-35 |  |  | Yes |  | No additional Type I codebook configurations other than the basic CSI feedback (2-32) is supported |  |  |  | Type 3 |  | No need |  |  |  |  |  |  |  |  |  | Note: simultaneously doesn’t mean in the same slot Note: for the purpose component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N resource settings |  |  |  |  | Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {“Mode-1 only”, “Mode-1 and Mode-2”}. Component-3 Candidate values set: {1:8} |  | Component-2 candidate values: Mode-1 as mandatory Mode-2 as optional. No significant difference in the performance but Mode-1 has less overhead than Mode-2. The performance results are provided in R1-1704729 |
|  | 2-37 | Support Semi-open loop CSI |  |  | 1. Support Semi-open loop CSI report |  |  | 2-35 |  |  | Yes |  | Semi-Open loop CSI report is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-38 | CSI report without PMI |  |  | Support CSI report without PMI |  |  | 2-35 |  |  | Yes |  | CSI report without PMI is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | FFS |  | Optional with capability signaling |
|  | 2-39 (This feature is removed) | CSI report with CRI |  |  | Support CSI report with CRI |  |  | 2-35 |  |  | Yes |  | CSI report with CRI is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | FFS |  |  |
|  | 2-39a | CSI report without CQI |  |  | Support CSI report without CQI |  |  | 2-35 |  |  | Yes |  | CSI report wihout CQI is not supported |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 2-40 | Type I multi-panel codebook |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s): 3. Supported number of panels, Ng 4. Max # of CSI-RS resource in a resource set |  |  | 2-35 |  |  | Yes |  | multi-panel Type I codebook is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: simultaneously doesn’t mean in the same slot Note: for the purpose component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N resource settings |  |  |  |  | Optional Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} Component-4: candidate value set is {1:8} |  | Optional with capability signaling Component-2 candidate values: Mode-1 as optional Mode-2 as optional. Supported codebook Mode should be signaled for each number of panels |
|  | 2-41 | Type II codebook |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Support amplitude subset restriction level 5. Max # of CSI-RS resource in a resource set |  |  | 2-35 |  |  | Yes |  | Type II codebook is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: simultaneously doesn’t mean in the same slot Note: for the purpose component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N resource settings |  |  |  |  | Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {“no amplitude subset restriction”, “support amplitude subset restriction”} Component-5: candidate value set is {1:8} |  |  |
|  | 2-42 | Support Type II SP-CSI feedback on long PUCCH |  |  | 1.Support type II SP-CSI feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot |  |  | 2-41 |  |  | Yes |  | Type II SP-CSI feedback on long PUCCH is not supported |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | [Optional] |  | Optional with capability signaling |
|  | 2-43 | Type II codebook with port selection |  |  | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3. Support amplitude scaling type 4. Max # of CSI-RS resource in a resource set |  |  |  |  |  | Yes |  | Type II codebook with port selection is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: simultaneously doesn’t mean in the same slot Note: for the purpose component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N resource settings |  |  |  |  | Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4: candidate value set is {1:8} |  |  |
|  | 2-44 | Basic DL PTRS |  |  | Support 1 port of PTRS |  |  |  |  |  | Yes |  | DL PTRS is not supported |  |  |  | Type 4 |  | N.A. |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability signaling for FR2 Optional for FR1 |  | Mandatory with UE capability signaling for FR2 Optional for FR1 |
|  | 2-45 | Downlink 2Tx PTRS |  |  | 1. Supported 2 ports of PTRS |  |  | 2-44 |  |  | Yes |  | 2 ports of PTRS is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-46 | Downlink PTRS density recommendation |  |  | 1. Preferred threshold sets, TSi for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  |  | 2-44 |  |  | Yes |  | Threshold recommendation is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density |  | Optional with capability signaling |
|  | 2-47 | Basic UL PTRS |  |  | Support 1 port of PTRS |  |  |  |  |  | Yes |  | Uplink 1 port PTRS is not supported |  |  |  | Type 4 |  | N.A. |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with UE capability signaling for FR2 [Optional for FR1] |  | Mandatory with UE capability signaling for FR2 |
|  | 2-48 | Uplink PTRS |  |  | 1. Supported 2 ports of PTRS |  |  | 2-47 |  |  | Yes |  | Uplink 2 ports PTRS is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional |  | Optional with capability signaling |
|  | 2-49 | Uplink PTRS density recommendation |  |  | 1. Preferred threshold sets, TSi , for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. |  |  | 2-47 |  |  | Yes |  | Threshold recommendation is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density, and five values each selected from {1..276} for sample density |  | Optional with capability signaling |
|  | 2-50 | Basic TRS |  |  | 1. Support of TRS (mandatory) 2. All the periodicity are supported. 3. Support TRS bandwidth configuration as both “BWP” and “min(52, BWP)” |  |  |  |  |  |  |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: TRS bandwidth configuration does not imply UE processing bandwidth |  |  |  |  | Mandatory without UE capability signaling |  | Mandatory without capability signaling |
|  | 2-51 | TRS (CSI-RS for tracking) |  |  | 1. Support TRS BW 2. TRS burst length (X), 3. Max # of TRS resource sets (per CC) UE is able to track simultaneously 4. Max # of TRS resource sets configured to UE per CC 5. Max # of TRS resource sets configured to UE across CCs |  |  | 2-50 |  |  | Yes |  | TRS is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Component-1: candidate values set: {BWP, min(52,BWP), both} Component-2: candidate values {1,2} Component-3: Candidate value set: {1 to 8} Component-4: Candidate value set: {1 to 64} Component-5: Candidate value set: {1 to 128} |  |  |
|  | 2-51a (new added FG) | Aperiodic TRS |  |  | DCI triggering Aperiodic TRS associated with periodic TRS |  |  | 2-50 |  |  | Yes |  | A-TRS is not supported |  |  |  | Type 1 |  | N.A. |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 2-52 | Basic SRS |  |  | 1. Support 1 port SRS transmission 2. Support periodic/aperiodic SRS transmission 3. Support SRS Frequency intra/inter-slot hopping within BWP 4. At least one SRS resource per CC for aperiodic and periodic separately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory without UE capability |  | Mandatory without capability signaling |
|  | 2-53 | SRS resources |  |  | 1. Maximum number of aperiodic SRS resources (configured to UE) per BWP 2. Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3. Maximum number of periodic SRS resources (configured to UE) per BWP 4. Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5. Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6. Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7. Maximum number of SRS port per resource |  |  | 2-52 |  |  | Yes |  | No more than one periodic and one aperiodic SRS resources per CC are supported and no SP-SRS is supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Component-1: candidate value: {from 1 , 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1 , 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1 , 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability |  |  |
|  | 2-54 | SRS transmission |  |  | 1. Minimum time interval, N in unit of symbols, between DCI triggering and A-SRS transmission, |  |  | 2-53 |  |  | Yes |  |  |  |  |  | Type 1 |  | No need |  |  |  | N.A. |  |  |  |  |  | Note: there is a minimal timing (42 symbols) between A-CSI-RS reception and updating of A-SRS precoding |  |  |  |  | candidate value range is the same as that of N2 plus 42. |  |  |
|  | 2-54a | Simultaneous SRS Tx |  |  | 1. Maximum number of simultaneous transmitted SRS resources per CC at one symbol for NCB PUSCH |  |  |  |  |  | Yes |  | Only one SRS resource can be transmitted at a given time |  |  |  | Type 1 |  | No need |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Candidate Value Set: {1, 2, 3, 4} |  |  |
|  | 2-55 | SRS Tx switch |  |  | 1. Support SRS Tx port switch, 2. Report whether the uplink TX switching impact to downlink receiving in a band, |  |  | 2-53 |  |  | Yes |  | SRS Tx Switch is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Component-2 is agreed with conditioned to RAN4’s decision.— |  |  | RAN1/4 |  | Component-1 is a list of TRx pairs, candidates are {“1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “T=R”} Component-2: Candidate value set:, {yes, no}, |  |  |
|  | 2-56 | SRS carrier switch |  |  | 1. Report inter-cell switching time capability |  |  | 2-53 |  |  | Yes |  | SRS carrier switch is not supported |  |  |  | Type 1 |  | No need |  |  |  | N.A. |  |  |  |  |  | RAN4 reply LS, R1-1805817, includes candidate value sets |  |  | RAN1/4 |  | candidate values set is up to RAN4 |  |  |
|  | 2-57 This FG is removed | Support low latency CSI feedback |  |  | Support low latency CSI feedback |  |  |  |  |  | Yes |  | Low latency CSI is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  | [Optional] |  |  |
| 3.DL control channel and procedure | 3-1 |  | Basic DL control channel |  |  | 1) One UE-specific configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - Single TCI state for a CORESET configuration 2) CSS and USS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, the monitoring occasion can be any OFDM symbol(s) of a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD |  |  |  |  | Yes |  |  |  |  |  | N.A. |  |  | N.A. |  |  | N.A |  |  |  |  |  |  |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 3-1’ | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot |  |  | - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Applicable only to FR1 |  |  |  |  |  |  |  |  |  |  | [Optional with capability signaling] |  | Optional with capability signaling |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  | Whether/How to differentiate FR1 and FR2 is up to RAN2 |  |  |  |  | Optional with capability signaling for FR1 Mandatory with capability signaling for FR2 |  | Optional with capability signaling for FR1 Mandatory with capability signaling for FR2 |
|  | 3-4 | More than one TCI state configurations per CORESET |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | UE is only required to track one active TCI state [per CORESET] |  |  |  |  | Optional with capability signaling |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 3-5 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 3-5a | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap |  |  | For type 1 CSS with dedicated RRC configuration, type 3 CSS and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation (including the cross-slot boundary case) between two DL unicast DCIs , between two UL unicast DCIs, or between a DL and an UL unicast DCI in different monitoring occasions for a same UE as - 2OFDM symbols for 15kHz - 4OFDM symbols for 30kHz - 7OFDM symbols for 60kHz with NCP - 14OFDM symbols for 120kHz In addition for TDD the minimum separation between the first two UL unicast DCIs in the first monitoring occasion within the first 3 OFDM symbols of a slot can be zero OFDM symbols. |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 3-5b | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap |  |  | For type 1 CSS with dedicated RRC configuration, type 3 CSS and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols in which PDCCH is configured to be monitored |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS. |  |  |  |  | Optional with capability signaling Candidate value set for (X, Y): {[(7, 3)], [(4, 3) and (7, 3)], [(2, 2) and (4, 3) and (7, 3)]} |  | Optional with capability signaling |
|  | 3-6 | Dynamic SFI monitoring |  |  | 1) Adjust periodic and semi-persistent signal reception and transmission in response to detected dynamic UL/DL configuration |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | [Optional with capability signaling] |  | Optional with capability signaling |
|  | 3-7 | Precoder-granularity of CORESET size |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
| 4.UL control channel and procedure | 4-1 | Basic UL control channel |  |  | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH 9) Semi-static beta-offset configuration for HARQ-ACK |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | RAN4 to check feasibility of frequency hopping for PUCCH formats for FR2 |  |  |  |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
|  | 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols |  |  | 1) 2 PUCCH format 0/2 in different symbols and once per slot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per slot for SR 3) 2 PUCCH format 2 in different symbols and once per slot for CSI over two consecutive OFDM symbols |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  | Note: RAN1 to check consistency with 2-32 |  |  | RAN1 |  | Mandatory with capability signaling which shall be set to ‘1’ |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  | Note: RAN1 to check consistency with 2-32 |  |  | RAN1 |  | Mandatory with capability signaling which shall be set to ‘1’ |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  | Note: RAN1 to check consistency with 2-32 |  |  | RAN1 |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | [Optional with capability signaling] |  | Mandatory with capability signaling |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | [Optional with capability signaling] |  | Mandatory with capability signaling |
|  | 4-10 | Dynamic HARQ-ACK codebook |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-11 | Semi-static HARQ-ACK codebook |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  | Applicable to UE supporting more than 4 layers |  |  | RAN1 |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 4-13 | More than one SR configurations per PUCCH group |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  | RAN2 to check |  |  | RAN1 and RAN2 |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol in a slot |  |  | Overlapping PUCCH resources have the same starting symbols in a slot while precluding the case of SR/HARQ-ACK by overlapping PUCCH resources with the same starting symbols in a slot |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling |  |  |
|  | 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot |  |  | Overlapping PUCCH resources have different starting symbols in a slot |  |  | 4-19 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  |  |
|  | 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot |  |  | Overlapping PUCCH resources have same or different starting symbols in a slot |  |  | 4-19c |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  |  |
|  | 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot |  |  | Overlapping PUCCH resources have different starting symbols in a slot |  |  | 4-19a |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Optional with capability signaling |  |  |
|  | 4-20 | UCI code-block segmentation |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot |  |  | 1) 1 PUCCH format 0 or 2and 1 PUCCH format 1, 3, and 4 in the same slot |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 4-23 | Repetitions for PUCCH format 1, 3,and 4 over multiple slots with K = 2, 4, 8 |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | [Mandatory with capability signaling] |  | Mandatory with capability signaling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory with capability signaling for FR2 Optional with capability signaling for FR1 |  | Optional or mandatory? No support in FR1 |
|  | 4-25 | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA |  |  | 1) Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA |  |  | 2-52, 4-1, 2-12, 6-6 |  |  | Yes |  | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA is not supported. |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This feature is supported only in inter-band CA |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 4-26 | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA |  |  | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA |  |  | 1-1, 2-52, 4-1, 2-12, 6-6 |  |  | Yes |  | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA is not supported |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This feature is supported only in inter-band CA |  |  | RAN1 |  |  |  | Optional with capability signaling |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation |  | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 before RRC connection 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 11) DL scheduling slot offset K0=1 for type 1 without dedicated RRC configuration and for type 0, 0A, and 2 12) UL scheduling slot offset K2<=12 For type 1 without dedicated RRC configuration and for type 0, 0A, and 2, interleaving for VRB-to-PRB mapping for PDSCH |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | Note: If UE is configured with more than 8 HARQ processes, RAN4 continue to discuss the impact of 16 HARQ processes |  |  | RAN1 |  |  | Mandatory without capability signaling | Mandatory without capability signaling |  |
|  | 5-1a | UE specific RRC configure UL/DL assignment |  |  | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | RAN1 needs to check component |  |  | RAN1 |  |  |  | Optional with capability signaling |
|  | 5-1b | More than one DL/UL switch point in a slot |  |  | In TDD support more than one switch points in a slot for actual DL/UL transmission(s) |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | N.A. TDD only |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-2 | RA Type 0 for PUSCH |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  |  |
|  | 5-6 | PDSCH mapping type A with less than 7 OFDM symbols |  |  | For type 1 CSS with dedicated RRC configuration and USS, PDSCH mapping type A with less than 7 OFDM symbols |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling [which shall be set to “1”] |  | Mandatory with capability signaling which shall be set to “1” |
|  | 5-6a | PDSCH mapping type B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  |  |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 before RRC connection |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  | Mandatory with capability signaling |
|  | 5-10 | Inter-slot frequency hopping for PUSCH |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-11 | Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 2 unicast PDSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-11a | Up to 7 unicast PDSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 7 unicast PDSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-11b | Up to 4 unicast PDSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 4 unicast PDSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-12 | Up to 2 PUSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 2 unicast PUSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-12a | Up to 7 PUSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 7 unicast PUSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-12b | Up to 4 PUSCHs per slot for different TBs for UE processing time Capability 1 |  |  | Up to 4 unicast PUSCHs per slot only in TDM is supported for Capability 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  |  | 5-19 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots |  |  | 1) K = 2, 4, 8 times repetitions with RV sequences |  |  | 5-20 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-17 | PUSCH repetitions over multiple slots |  |  | 1) K = 2, 4, 8 times repetitions |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 5-17a | PDSCH repetitions over multiple slots |  |  | 1) K = 2, 4, 8 times repetitions |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-18 | DL SPS |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-19 | Type 1 Configured UL grant |  |  | 1) K = 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-20 | Type 2 Configured UL grant |  |  | 1) K = 1 |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-21 | Pre-emption indication for DL |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL |  |  |  |  |  | 5-22 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL |  |  | - Bitmap 1/2/3 - controlResourceSet |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL |  |  | - Bitmap 1/2/3 |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-27a | Dynamic rate-matching control resource set for DL |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | No need |  |  |  |  |  |  |  |  |  |  | [Mandatory with capability signaling] |  |  |
|  | 5-28 | Rate-matching around LTE CRS |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 5-29 | LBRM for PUSCH |  |  | Limited buffer rate matching in UL |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 5-30 | DL scheduling slot offset greater than zero |  |  | Support of DL scheduling slot offset (K0) greater than zero |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  |  |
|  | 5-31 | UL scheduling slot offset greater than 12 |  |  | Support of UL scheduling slot offset (K2) greater than 12 |  |  |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  |  |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction |  |  | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell/PScell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW RAN4 may discuss other BW requirements. UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP |  |  | RAN4 |  | [Mandatory without capability signaling] |  |  |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) |  |  | 1) BW of UE-specific RRC configured BWP may not include BW of the initial DL BWP and SSB for PCell/PScell and BW of BWP may not include SSB for SCell |  |  | 6-1, 6-2, 6-3, or 6-4 |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. It is up to RAN2 how to create signaling for 6-1a associated with 6-1, 6-2, 6-3, and 6-4 |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-2 | Type A BWP adaptation with same numerology |  |  | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell/PScell and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |  | 6-1 |  |  | Yes |  | Type A BWP adaptation with same numerology is not possible |  |  |  | Type 1 |  | N.A |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-3 | Type B BWP adaptation with same numerology |  |  | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell/PScell and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |  | 6-1 |  |  | Yes |  | Type B BWP adaptation with same numerology is not possible |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-4 | BWP adaptation with different numerologies |  |  | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the initial DL BWP and SSB for Pcell/PScell and BW of the UE-specific RRC configured BWP includes SSB for Scell if there is SSB on Scell |  |  | 6-1 |  |  | Yes |  | BWP adaptation with different numerologies is not possible |  |  |  | Type 1 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 6-5 | Basic DL NR-NR CA operation |  |  | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  |  |  |  |  | Yes |  |  |  |  |  | N.A. |  | N. A. |  |  |  | N.A. |  |  |  |  |  | This is conditioned on the support of DL CA band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-5a | PDCCH blind detection capability for CA |  |  | - More than 4 DL CCs - Reporting value is one of integer from 4 to 16 |  |  | 6-5 |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} |
|  | 6-6 | Basic UL NR-NR CA operation |  |  | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel at a given time 3) One PUCCH group 4) Single TAG |  |  | [6-5] |  |  | Yes |  |  |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This is conditioned on the support of UL CA band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-7 | Two NR PUCCH group with same numerology |  |  | 1) For NR CA UE, same numerology across NR carriers for data/control channel at a given time 2) For EN-DC UE, same numerology across NR carriers for data/control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2 |  |  | 6-5, 6-6 |  |  |  |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-8 | Different numerology across NR PUCCH groups |  |  | 1) For both NR CA UE and EN-DC UE, different numerology between two NR PUCCH groups for data/control channel at a given time |  |  | 6-5, 6-7 |  |  |  |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-9 | Different numerologies across NR carriers within the same NR PUCCH group |  |  | 1) For both NR CA UE and EN-DC UE, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-2) For EN-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with smaller SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time |  |  | 6-5 |  |  |  |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-10 | Cross carrier scheduling for the same numerology |  |  | 1) Cross carrier scheduling for the same numerology with CIF |  |  | 6-5, 6-6 |  |  | Yes |  | Cross carrier scheduling is not possible |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-10a | Cross carrier scheduling for different numerologies |  |  | 1) Cross carrier scheduling for the different numerologies with CIF |  |  | 6-10 |  |  | Yes |  |  |  |  |  | Type 3, or SCS combination dependent |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-11 | Number of supported TAGs |  |  | Need of multiple capability question about the resolution here |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | {1, 2, 3, 4} |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases |  |  |  |  |  |  |  |  | Yes |  | 2 simultaneous UL transmissions are not supported for problematic cases |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | RAN2/4 to decide This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  | RAN2/4 |  |  |  | Optional with capability signaling |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC |  |  | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-Pcell 2) HARQ subframe offset |  |  |  |  |  | Yes |  |  |  |  |  | Type 2 |  | Yes |  |  |  | Yes |  |  |  |  |  | This is a UE feature for LTE for a LTE/NR dual connectivity UE |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 6-15 | 7.5kHz UL raster shift |  |  | 7.5kHz UL raster shift |  |  |  |  |  | Yes |  |  |  |  |  |  |  | [No need] |  |  |  | [No need] |  |  |  |  |  |  |  |  | RAN4 |  |  |  |  |
|  | 6-16 | Supplemental uplink |  |  | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers |  |  | 6-15 |  |  | Yes |  | UE will not be able to perform the RACH/PUSCH/PUCCH/SRS operation in a band combination including SUL |  |  |  | N.A. |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  |  |  |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers |  |  | Different numerologies between SUL and non SUL |  |  | 6-16 |  |  | Yes |  | The UE will not be able to access or operate on a SUL carrier |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  |  | Mandatory with capability signaling |
|  | 6-18 | Supplemental uplink with dynamic switch |  |  | DCI based selection of PUSCH carrier |  |  | 6-16 |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  | This is conditioned on the support of SUL band combination(s). The band combination definition is up to RAN4. |  |  |  |  |  |  |  |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS/PRACH on the other UL carrier in the same cell |  |  |  |  |  | 6-16 |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 6-21 | DL search space sharing for CA |  |  |  |  |  | 6-10 or 6-10a |  |  |  |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 6-22 | UL search space sharing for CA |  |  |  |  |  | 6-10 or 6-10a |  |  |  |  |  |  |  |  | Type 3 |  | N.A. |  |  |  | N.A. |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |  | Optional with capability signaling |
|  | 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  |  | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  |  |  |  |  | Yes |  |  |  |  |  | Type 3 (per band per band combination) |  | N.A. |  |  |  | N.A. |  |  |  |  |  | See LS (R1-1809992) |  |  | RAN1 /RAN4 |  | Optional with capability signaling |  |  |
| 7. Channel coding | 7-1 | Channel coding |  |  | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  |  |  |  |  | Yes |  | UE will not be able to transmit or receive data or control |  |  |  | N.A. |  | No need |  |  |  |  |  |  |  |  |  |  |  |  | RAN1 |  | Mandatory without capability signaling |  | Mandatory without capability signaling |
| 8. UL TPC | 8-1 |  | Dynamic power sharing for LTE-NR DC |  |  | When total transmission power exceeds Pcmax, UE scales NR transmission power. |  |  | EN-DC | Yes |  |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | RP-172833 |  |  | RAN |  |  |  |  | Mandatory with capability signaling |
|  | 8-1a |  | Operation A with single UL Tx case 1 |  |  |  |  |  | EN-DC | Yes |  |  |  |  |  | Type 2 |  | Yes |  |  | Yes |  |  |  |  |  | RP-172833 |  |  |  |  |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, optional for UEs supporting dynamic power sharing |  |  | Mandatory with capability signaling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |
|  | 8-2 |  | Basic power control operation |  |  | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 10) PHR |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | No need |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mandatory without capability signaling |
|  | 8-3 |  | TPC-PUSCH-RNTI |  |  | Specific group DCI message for TPC commands for PUSCH |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-4 |  | TPC-PUCCH-RNTI |  |  | Specific group DCI message for TPC commands for PUCCH |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-5 |  | TPC-SRS-RNTI |  |  | Specific group DCI message for TPC commands for SRS |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-6 |  | Absolute TPC command mode |  |  | Absolute TPC command mode |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 8-7 |  | UL power control with 2 PUSCH closed loops |  |  | Two different TPC loops |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  |  | Mandatory with capability signaling |
|  | 8-8 |  | UL power control with 2 PUCCH closed loops |  |  | Two different TPC loops |  |  |  | Yes |  |  |  |  |  | Type 4 |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Mandatory with capability signaling |  |  | Mandatory with capability signaling |
|  | 1 [5-5a]? |  | UE PDSCH processing capability #2 |  |  | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - UE supports Capability #2 processing time on all configured carriers if # configured carriers in a band <= X, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  |  |  | Yes |  |  |  |  |  | Type 3 |  |  |  |  | Applicable to FR1 only |  |  |  |  |  | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination |  |  | RAN1 |  |  | Optional with capability signaling Candidate values for Component 1: X in {1,..,16}, Fallback {‘SC’,’Cap1-only’} |  |  |  |
|  | 2 [5-5b]? |  | UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS |  |  | Capability #2 supported only if 1 carrier configured in the band (independent of #carriers configured in other bands) 2) Max PDSCH BW of 136 PRBs 3) N1 based on Table 5.3-2 of TS 38.214 for 30 kHz SCS |  |  |  | Yes |  |  |  |  |  | Type 3 |  |  |  |  | Applicable to FR1 only |  |  |  |  |  | This capability is applicable to 30kHz-SCS only |  |  | RAN1 |  |  | Optional with capability signaling |  |  |  |
|  | 3 [5-5c]? |  | UE PUSCH processing capability #2 |  |  | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - UE supports Capability #2 processing time on all configured carriers if # configured carriers in a band <= X, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  |  |  | Yes |  |  |  |  |  | Type 3 |  |  |  |  | Applicable to FR1 only |  |  |  |  |  | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination |  |  | RAN1 |  |  | Optional with capability signaling Candidate values for Component 1: X in {1,..,16}, Fallback {‘SC’,’Cap1-only’} |  |  |  |
|  | 4 [5-30a]? |  | New 64QAM MCS table for PDSCH |  |  | New 64QAM MCS table for PDSCH |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  | Optional with capability signaling |  |  |  |
|  | 5 [5-30b]? |  | New 64QAM MCS table for PUSCH |  |  | New 64QAM MCS tables for PUSCH with and without transform precoding respectively |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  | Optional with capability signaling |  |  |  |
|  | 6 [2-32c]? |  | New CQI table |  |  | CQI table with target BLER of 10^-5 |  |  |  | Yes |  |  |  |  |  | Type 4 |  | No need |  |  | Yes |  |  |  |  |  |  |  |  | RAN1 |  |  | Optional with capability signaling |  |  |  |
|  | 7 [5-30c]? |  | Dynamic indication of MCS table with new RNTI for PDSCH |  |  | 1) Dynamic indication of MCS table using new RNTI for PDSCH |  |  | 4 | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  |  |  |  | RAN1 |  |  | Optional with capability signaling |  |  |  |
|  | 8[5-30c] |  | Dynamic indication of MCS tables with new RNTI for PUSCH |  |  | 1) Dynamic indication of MCS tables using new RNTI for PUSCH |  |  | 5 | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 9 [5-13] |  | Up to 2 unicast PDSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 2 unicast PDSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 10 [5-13a] |  | Up to 7 unicast PDSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 7 unicast PDSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 11 [5-13c] |  | Up to 4 unicast PDSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 4 unicast PDSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 12 [5-13d] |  | Up to 2 PUSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 2 unicast PUSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 13 [5-13e] |  | Up to 7 PUSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 7 unicast PUSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |
|  | 14 [5-13f] |  | Up to 4 PUSCHs per slot for different TBs for UE processing time Capability 2 |  |  | Up to 4 unicast PUSCHs per slot only in TDM is supported for Capability 2 |  |  |  | Yes |  |  |  |  |  | Type 3 |  | N.A. |  |  | N.A. |  |  |  |  |  | This capability is necessary for each SCS |  |  |  |  |  | Optional with capability signaling |  |  |  |