5	Self evaluation of eMBB technical performance

[Editor’s note: The grouping of the technical performance requirements for eMBB usage scenario are based on Report ITU-R M.2410 “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. However, it does not imply that each of the following technical performance requirements is only related to eMBB. It is just the minimum requirement that shall be evaluated under eMBB. This should also be noted for Section 6 and 7.]

[Editor’s note: the evaluation results in Section 5 are preliminary. The results might be further updated before final approval of TR37.910 in June 2019. This also applies to the evaluation results in Section 6 and 7.]

5.3	5th percentile user spectral efficiency

As defined in Report ITU-R M.2410, the 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

As required by Report ITU-R M.2412, 5th percentile user spectral efficiency shall be assessed jointly with average spectral efficiency using the same simulation. Therefore, the evaluation results of the 5th percentile user spectral efficiency are provided together with average spectral efficiency in Section 5.4.

5.4	Average spectral efficiency

As defined in Report ITU-R M.2410, average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

As required by Report ITU-R M.2412, average spectral efficiency and 5th percentile user spectral efficiency are assessed jointly using the same simulation.

5.4.1	NR

Average spectral efficiency and 5th percentile user spectral efficiency are evaluated for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in Annex B.z.1.

The antenna configuration is indicated as (M, N, P, Mg,Ng; Mp, Np), where M and N are the number of vertical, horizontal antenna elements within a panel, P is number of polarizations, Mg is the number of panels in a column, Ng is the number of panels in row; and Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization.

5.4.1.1	Indoor Hotspot – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 30 GHz) with either 12TRxP or 36TRxP cases are applied for the evaluations of Indoor Hotspot– eMBB test environment for NR.

5.4.1.1.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A with 12TRxP are provided in Table 5.4.1.1.1-1.

It is noted that for NR, the component carrier bandwidth can be larger than the simulation bandwidth as defined in Report ITU-R M.2412. For those larger bandwidth cases, the guard band ratio can be further decreased; and the OFDM symbol occupation by PDCCH is expected to be reduced, even to less than one OFDM symbol due to the capability of PDCCH resource sharing with PDSCH.

Such capabilities are evaluated in Indoor Hotspot - eMBB test environment as well as in Dense Urban – eMBB test environment and Rural – eMBB test environment, for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

Table 5.4.1.1.1-1 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook; gNB Config. = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 10 | 10.95 | 12.38 | 13.24 | 5 | 11.20 | 12.67 | 13.54 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.36 | 0.41 | 0.43 |  | 0.38 | 0.42 | 0.45 |
| 32x4 MU-MIMO Type I Codebook; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | 11.50 | 13.05 | 13.97 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.35 | 0.38 |  | / | / | / |
| 32x4 MU-MIMO Ideal CSI feedback gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | / | / | / | / | 1 | [12.23] | [13.89] | ]14.87] |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | / | / |  | [0.49] | [0.55] | [0.59] |
| 32x8 MU-MIMO Type II Codebook; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | [9.80] | [10.99] | [11.70] | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | [0.35] | [0.39] | [0.42] |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.97 | 15.20 | 16.73 | 3 | [12.13] | [14.18] | [15.57] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.38 | 0.44 | 0.49 |  | [0.42] | [0.49] | [0.54] |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.77 | 14.65 | / | 1 | 12.77 | 14.64 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.45 | / |  | 0.40 | 0.46 | / |
| 32x4 MU-MIMO, Type II Codebook based; gNB Config = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 8.77 | 10.43 | / | 1 | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.33 | 0.39 | / |  | / | / | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 13.27 | 15.18 | / | 1 | 13.55 | 15.5 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.49 | 0.56 | / |  | 0.5 | 0.58 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 2 | 11.61 | 13.68 | 15.08 | 2 | 11.96 | 14.09 | 15.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.41 | 0.48 | 0.53 |  | 0.45 | 0.53 | 0.59 |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Configu = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 3 | 11.73 | 13.46 | / | 3 | 12.34 | 14.04 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.45 | / |  | 0.42 | 0.48 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config= (4,4,2,1,1;4,4) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 9 | 1 | 9.92 | 11.69 | 12.90 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.37 | 0.41 |  | / | / | / |

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A with 36TRxP are provided in Table 5.4.1.1.1-2.

It is observed that both NR FDD and NR TDD fulfill the DL spectral efficiency requirement for all the above configurations in evaluation configuration A.

Table 5.4.1.1.1-2 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook; gNB@ Config = (8,16,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 9 | 7 | 12.24 | 13.85 | / | 4 | 13.12 | 14.83 | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.36 | 0.40 | / |  | 0.35 | 0.39 | / |
| 32x4 MU-MIMO Type I codebook; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 9 | 1 | / | 10.67 | / | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | 0.31 | / |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,16,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.22 | 16.68 | 18.35 | 1 | 14.33 | 16.81 | 18.50 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.41 | 0.45 |  | 0.36 | 0.42 | 0.46 |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,16,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.56 | 16.71 | / | 1 | 14.67 | 16.84 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.39 | 0.44 | / |  | 0.39 | 0.45 | / |
| 32x4 MU-MIMO, Type II Codebook based; gNB Config = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 |  | 10.38 | 12.35 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 | 1 | 0.33 | 0.40 | / |  | / | / | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB@ (M,N,P,Mg,Ng; Mp,Np) = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 2 | 12.16 | 13.88 | / | 2 | 12.37 | 14.12 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.40 | / |  | 0.33 | 0.38 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,16,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 13.00 | 15.18 | 16.66 | 1 | 13.17 | 15.38 | 16.88 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.32 | 0.37 | 0.40 |  | 0.34 | 0.39 | 0.43 |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | / | / | / | / | 1 | [10.48] | [12.16] | [13.30] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | / | / |  | [0.31] | [0.36] | [0.40] |

The evaluation results of UL spectral efficiency for NR FDD and TDD for evaluation configuration A with 12TRxP are provided in Table 5.4.1.1.1-3.

Table 5.4.1.1.1-3 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x32 SU-MIMO, OFDMA; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 3 | 8.75 | 1 | 8.87 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.52 |  | 0.55 |
| 4x32 MU-MIMO, OFDMA; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 2 | [10.64] | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.49] |  | / |
| 4x32 SU-MIMO, OFDMA; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 3 | 8.88 | 4 | [8.17] |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.55 |  | [0.49] |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.95 | 2 | 7.70 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.39 |  | 0.44 |
| 2x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.17 | 1 | 7.26 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |  | 0.41 |
| 4x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.58 | 1 | 7.62 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |  | 0.43 |
| 4x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.87 | 1 | 7.95 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.44 |  | 0.45 |
| 4x32 MU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 12.91 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.55 |  | / |
| 4x32 SU-MIMO, Non-Codebook based; gNB Config = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 9.57 | 1 | 9.26 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.60 |  | 0.56 |
| 4x32 SU-MIMO, Non-Codebook based; gNB Config = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 9.25 | 1 | 8.99 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.58 |  | 0.52 |
| 4x32 SU-MIMO, Codebook based; gNB Configs = (4,4,2,1,1;4,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.99 | 1 | 8.65 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.63 |  | 0.54 |
| 4x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.58 | 1 | 8.12 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.57 |  | 0.48 |
| 2x32 SU-MIMO, Codebook based; gNB Config = (4,4,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 8.26 | 1 | 9.35 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.53 |  | 0.53 |

The evaluation results of UL spectral efficiency for NR FDD and TDD for evaluation configuration A with 36TRxP are provided in Table 5.4.1.1.1-4.

It is observed that both NR FDD and NR TDD fulfill the UL spectral efficiency requirement for all the above configurations in evaluation configuration A.

Table 5.4.1.1.1-4 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 4x32 MU-MIMO, OFDMA; gNB Config = (8,16,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 15.17 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.47 |  | / |
| 2x32 MU-MIMO, OFDMA; gNB Config = (4,4,2,1,1;4,4) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 2 | 7.17 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.27 |  | / |
| 4x32 SU-MIMO, OFDMA; gNB Config = (4,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | [7.20] | 1 | [7.34] |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.40] |  | [0.41] |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 4x32 MU-MIMO; gNB Config = (8,16,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 14.04 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |  | / |
| 4x32 SU-MIMO; gNB Config = (4,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | [6.98] | 1 | [7.59] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.40] |  | [0.47] |
| 4x32 SU-MIMO; gNB Config = (4,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | [7.44] | 1 | [7.48] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.42] |  | [0.43] |

5.4.1.1.2	Evaluation configuration B (CF = 30 GHz)

The evaluation results of DL spectral efficiency for NR for evaluation configuration B with 12TRxP are provided in Table 5.4.1.1.2-1.

It is noted that for NR, the component carrier bandwidth can be larger than the simulation bandwidth as defined in Report ITU-R M.2412. For those larger bandwidth cases, the guard band ratio can be further decreased; and the OFDM symbol occupation by PDCCH is expected to be reduced, even to less than one OFDM symbol due to the capability of PDCCH resource sharing with PDSCH.

Such capabilities are evaluated for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

Table 5.4.1.1.2-1 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Number of samples | BW=40 MHz |
| --- | --- | --- | --- | --- | --- |
| 32x32 SU-MIMO (2 panel@UE) gNB Config = (4,8,2,1,1;4,4); UE Config = (2,4,2,1,2; 2,4) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | [17.86] |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | [0.68] |
| 8x16 SU-MIMO (2 panel@UE) gNB Config = (4,8,2,1,1;2,2); UE Config = (2,4,2,1,2; 1,4) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | [9.40] |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | [0.47] |
| 8x4 MU-MIMO (2 panel@UE) gNB Config = (4,8,2,1,1;2,2) UE Config = (2,4,2,1,2; 1,1) | 120 | Average [bit/s/Hz/TRxP] | 9 | 1 | 8.50 |
|  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz / 100 MHz | BW= 200MHz | BW= 400MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 2 | 12.28 | 14.08 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.36 | / |
| 32x4 MU-MIMO, Type II Codebook based (2 panel@UE) gNB Config = (8,8,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | / | 9.80 | 10.46 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | 0.40 | 0.42 |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,8,2,1,1;2,8); UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.88 | 17.08 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.72 | 0.83 | / |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,8,2,1,1;2,8); UE Config = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.93 | 16.95 | 17.65 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.61 | 0.69 | 0.72 |
| 64x8 MU-MIMO, Reciprocity based ; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;2,16); UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 16.75 | 19.22 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 1.00 | 1.14 | / |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;2,16); UE Config = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 16.84 | 19.12 | 19.91 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 1.00 | 1.13 | 1.18 |
| 64x16 MU-MIMO, Reciprocity based ; 4T SRS (2 panel@UE) gNB Config = (4,32,2,1,1;1,32); UE Config = (1,4,2,1,2; 1,4) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 9 | 1 | 13.9 | 16.88 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.35 | 0.42 | / |
| 8x2 , MU-MIMO, Reciprocity based; 2T SRS (1 panel at UE) gNB Config = (16,8,2,1,1;2,2); UE Config = (4,4,2,1,1; 1,2) | 120 | DDDU | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.03 | 14.40 | 14.97 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.49 | 0.58 | 0.60 |

The evaluation results of DL spectral efficiency for NR for evaluation configuration B with 36TRxP are provided in Table 5.4.1.1.2-2.

It is observed that NR fulfills the DL spectral efficiency requirement for all the above configurations in evaluation configuration B.

Table 5.4.1.1.2-2 DL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz / 100MHz | BW= 200MHz | BW= 400MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x4 MU-MIMO Type II Codebook based (2 Panels @ UE) gNB Config = (8,8,2,1,1;4,4) UE Config = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | / | 10.04 | 10.72 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | / | 0.34 | 0.36 |
| 32x8 MU-MIMO, Reciprocity based; 4Tx SRS (2 Panels @ UE) gNB Config = (4,16,2,1,1;1,16) UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.72 | 14.60 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.38 | 0.43 | / |
| 32x8 MU-MIMO, Reciprocity based; 4Tx SRS (2 Panels @ UE) gNB Config = (4,16,2,1,1;1,16) UE Config = (2,4,2,1,2; 1,2) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 12.11 | 13.75 | 14.32 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.31 | 0.35 | 0.36 |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;2,16) UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.85 | 17.05 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.48 | 0.56 | / |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS (2 panel@UE) gNB Config = (8,16,2,1,1;2,16); UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 9 | 1 | 14.22 | 16.14 | 16.81 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.3 |  | 0.49 | 0.56 | 0.58 |

The evaluation results of UL spectral efficiency for NR for evaluation configuration B with 12TRxP are provided in Table 5.4.1.1.2-3.

Table 5.4.1.1.2-3 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Number of samples | BW=40MHz |
| --- | --- | --- | --- | --- | --- |
| 8x8 SU-MIMO, CB based, OFDMA (2 panel@UE) gNB Config= (4,8,2,1,1; 2,2) UE Config = (2,4,2,1,2; 1,2) | 120 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | [7.89] |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.48] |
| 4x8 MU-MIMO, CB based, OFDMA (2 panel@UE); gNB Config= (4,8,2,1,1; 2,2) UE Config = (2,4,2,1,2; 1,1) | 120 | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.66 |
|  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.39 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW= 80MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 8x32 SU-MIMO, CB based, OFDMA (2 panel@UE); gNB Config = (4,4,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,2) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.04 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.40 |
| 4x32 MU-MIMO, CB based, OFDMA (2 panel@UE); gNB Config = (8,8,2,1,1;4,4); UE Config = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 11.44 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.43 |
| 8x32 SU-MIMO,Non- CB based, OFDMA (2 panel@UE); gNB Config = (8,8,2,1,1;2,8); UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 7.44 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.37 |
| 8x64 MU-MIMO CB based (1 panel@UE); gNB Config = (4,32,2,1,1;1,32) UE Config = (1,4,2,1,1; 1,4) | 60 | DDDSU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 10.19 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.31 |
| 4x2, SU-MIMO, CB based, OFDMA (1 Panel @UE) gNB Config = (16,8,2,1,1;1,1) UE Config = (4,4,2,1,1; 1,2) | 120 | DDDU | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 6.90 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.30 |

The evaluation results of UL spectral efficiency for NR for evaluation configuration B with 36TRxP are provided in Table 5.4.1.1.2-4.

It is observed that NR fulfills the UL spectral efficiency requirement for all the above configurations in evaluation configuration B.

Table 5.4.1.1.2-4 UL spectral efficiency for NR in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Number of samples | BW=80MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 4x32 MU-MIMO OFDMA, gNB Config = (8,8,2,1,1;4,4) UE Config = (2,4,2,1,2; 1,1) | 120 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | 10.38 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | 0.31 |
| 8x32 SU-MIMO,Non- CB based, OFDMA (2 Panels @ UE), gNB Config = (8,16,2,1,1;2,8) UE Config = (2,4,2,1,2; 1,2) | 60 | DSUUD | Average [bit/s/Hz/TRxP] | 6.75 | 1 | [7.09] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.21 |  | [0.39] |

5.4.1.2	Dense Urban – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) is applied for the evaluations of Dense Urban – eMBB test environment for NR.

5.4.1.2.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.4.1.2.1-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the larger bandwidths are shown together with the evaluation results.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.2.1-1 DL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 8 | 11.48 | 12.98 | 13.88 | 4 | 11.84 | 13.37 | 14.28 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.38 | 0.43 | 0.46 |  | 0.39 | 0.44 | 0.47 |
| 32x4 MU-MIMO Type I Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 10.95 | 12.33 | 13.16 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.33 | 0.38 | 0.40 |  | / | / | / |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 14.81 | 16.84 | 18.04 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.54 | 0.61 | 0.65 |  | / | / | / |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 13.74 | 15.53 | 16.60 | 1 | 13.48 | 15.23 | 16.28 |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.48 | 0.54 | 0.57 |  | 0.5 | 0.56 | 0.6 |
| 32x4 MU-MIMO Ideal CSI feedback; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | / | / | / | / | 1 | [15.67] | [17.79] | [19.05] |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | / | / | / |  | [0.55] | [0.62] | [0.66] |
| 4x4 MU-MIMO Type II Codebook ; gNB Config = (8,8,2,1,1;2,1) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | [14.11] | [15.76] | [16.76] | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | [0.40] | [0.44] | [0.47] |  | / | / | / |
| 32x8 MU-MIMO Type II Codebook; gNB Config = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 9.20 | 10.31 | 10.99 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.33 | 0.37 | 0.39 |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 13.28 | 15.42 | 16.89 | 2 | [12.91] | [15.05] | [16.52] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.43 | 0.50 | 0.54 |  | [0.45] | [0.52] | [0.57] |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.95 | 14.78 | / | 1 | 12.74 | 14.54 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.39 | 0.44 | / |  | 0.4 | 0.46 | / |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.10 | 18.84 | 20.70 | 2 | 16.23 | 19.14 | 21.11 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.49 | 0.58 | 0.64 |  | 0.52 | 0.62 | 0.68 |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.71 | 18.02 | / | 1 | 15.46 | 17.73 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.48 | 0.56 | / |  | 0.49 | 0.56 | / |
| 32x4 MU-MIMO, Type II Codebook based; gNB Config = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.25 | 14.56 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.44 | 0.53 | / |  | / | / | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (16,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.87 | 18.16 | / | 1 | 15.47 | 17.70 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.54 | 0.62 | / |  | 0.54 | 0.62 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 12.54 | 14.39 | / | 1 | 14.95 | 17.02 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.36 | 0.41 | / |  | 0.42 | 0.47 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (16,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 15.36 | 17.93 | 19.68 | 1 | 14.94 | 17.44 | 19.14 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.51 | 0.60 | 0.66 |  | 0.52 | 0.61 | 0.67 |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.28 | 20.95 | / | 1 | 18.12 | 20.76 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.71 | 0.82 | / |  | 0.72 | 0.82 | / |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Configs = (16,8,2,1,1;4,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.93 | 21.69 | / | 1 | 18.91 | 21.67 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.76 | 0.87 | / |  | 0.81 | 0.93 | / |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 2 | 17.15 | 20.20 | 22.27 | 2 | 16.98 | 20.01 | 22.06 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.64 | 0.76 | 0.83 |  | 0.63 | 0.75 | 0.82 |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS ; gNB Config = (16,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 18.32 | 21.42 | 23.53 | 1 | 18.34 | 21.44 | 23.55 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.75 | 0.87 | 0.96 |  | 0.79 | 0.92 | 1.01 |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,32,2,1,1;1,32) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 12.2 | 14.82 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.30 | 0.36 | / |  | / | / | / |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 16.07 | 18.95 | 20.89 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.42 | 0.5 | 0.55 |  | / | / | / |
| 32x8 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (16,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 7.8 | 1 | 7.63 | 8.79 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | 0.23 | 0.27 | / |  | / | / | / |
| 128x4, MU-MIMO, Reciprocity based; 4T SRS; gNB Consfig = (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 7.8 | / | / | / | / | 1 | 10.78 | 12.56 | 13.78 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.225 |  | / | / | / |  | 0.36 | 0.42 | 0.46 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.4.1.2.1-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.2.1-2 UL spectral efficiency for NR in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x16 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 4 | 7.69 | 1 | 6.67 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.25 |  | 0.22 |
| 2x32 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 2 | 7.31 | 2 | 7.26 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.37 |  | 0.32 |
| 4x32 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 8.82 | 1 | 8.79 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.45 |  | 0.37 |
| 4x32 MU-MIMO, OFDMA; gNB Config = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 22.48 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.53 |  | / |
| 4x32 MU-MIMO, OFDMA; gNB Config = (16,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.73 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | / |
| 4x16 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.09 | 1 | 6.40 |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.33 |  | 0.35 |
| 16x16 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | [8.67] |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | [0.24] |
| 4x4 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,1) | 15 | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 9.57 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.19 |  | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x32 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.14 | 1 | 6.11 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | 0.25 |
| 2x32 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.57 | 1 | 6.48 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.28 |  | 0.25 |
| 2x64 SU-MIMO, Codebook based, OFDMA; gNB Config = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.03 | 2 | 7.21 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.36 |  | 0.41 |
| 2x64 SU-MIMO, Codebook based, OFDMA; gNB Config = (12,8,2,1,1;4,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.49 | 1 | 7.47 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.36 |  | 0.29 |
| 4x32 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.73 | 1 | 6.75 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.33 |  | 0.31 |
| 4x32 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.20 | 1 | 7.18 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.32 |  | 0.29 |
| 4x32 MU-MIMO, Codebook based, OFDMA; gNB Config = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 20.81 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.49 |  | / |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.56 | 1 | 6.90 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.27 |  | 0.32 |
| 4x16 SU-MIMO, Non-codebook based,OFDMA; gNB Config = (8,8,2,1,1;1,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.28 | 1 | 6.34 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.26 |  | 0.35 |
| 4x16 SU-MIMO, codebook based,OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 6.05 | 1 | 6.43 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.25 |  | 0.28 |
| 4x16 SU-MIMO, codebook based,OFDMA; gNB Config = (8,8,2,1,1;1,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 5.69 | 1 | 6.02 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.23 |  | 0.29 |
| 8x64 MU-MIMO, Codebook based, OFDMA; gNB Config = (4,32,2,1,1;1,32) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 7.28 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.16 |  | / |
| 2x32 SU-MIMO, Codebook based, OFDMA, gNB Config = (12,8,2,1,1;4,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | 8.16 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | 0.55 |
| 2x64 SU-MIMO, Codebook based, OFDMA; gNB Config = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 5.4 | 1 | 8.28 | 1 | 8.24 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | 0.60 |  | 0.58 |
| 4x128, MU-MIMO, Codebook based, OFDMA; gNB Config = (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 5.4 | / | / | 1 | 5.51 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.15 |  | / |  | 0.17 |

5.4.1.2.2	Evaluation configuration B (CF=30 GHz)

[Editor’s note: to be updated]

5.4.1.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for NR.

5.4.1.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.4.1.3.1-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.3.1-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 8x2 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 8 | 5.98 | 6.72 | 7.17 | 4 | 6.37 | 7.14 | 7.61 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | 0.18 | 0.20 | 0.21 |
| 16x2 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;2,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 7.48 | 8.39 | 8.94 | 1 | 7.46 | 8.37 | 8.92 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.15 | 0.17 | 0.18 |  | 0.16 | 0.18 | 0.19 |
| 8x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.42 | 10.66 | 11.40 | 1 | 9.08 | 10.27 | 10.99 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | 0.17 | 0.19 | 0.21 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 15.28 | 17.37 | 18.61 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.47 | 0.54 | 0.57 |  | / | / | / |
| 16x2 MU-MIMO Type II Codebook; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.58 | 10.81 | 11.55 | 1 | 10.02 | 11.23 | 11.96 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.25 | 0.28 | 0.30 |  | 0.27 | 0.30 | 0.32 |
| 8x2 MU-MIMO ideal CSI feedback; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | [9.37] | [10.6] | [11.33] |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | [0.26] | [0.3] | [0.32] |
| 8x2 MU-MIMO Type I codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 6.27 | 7.04 | 7.5 | 1 | [4.21] | [4.70] | [4.99] |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | [0.55] | [0.61] | [0.65] |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.79 | 8.96 | 9.76 | 3 | [6.50] | [7.51] | [8.21] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.17 | 0.20 | 0.21 |  | [0.18] | [0.21] | [0.23] |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 7.66 | 8.63 | / | 1 | 7.57 | 8.53 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | / |  | 0.16 | 0.18 | / |
| 16x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;2,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.21 | 10.64 | 11.61 | 1 | 9.22 | 10.65 | 11.62 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.21 | 0.25 | 0.27 |  | 0.22 | 0.25 | 0.28 |
| 16x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;2,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.9 | 10.08 | / |  | 8.85 | 10.14 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.18 | 0.2 | / |  | 0.18 | 0.21 | / |
| 16x4 MU-MIMO, Type II codebook based; gNB Config = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.61 | 14.99 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.39 | 0.46 | / |  | / | / | / |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 3 | 8.13 | 9.34 | / | 2 | 9.61 | 11.06 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.24 | 0.28 | / |  | 0.27 | 0.31 | / |
| 8x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 6.71 | 7.84 | 8.60 | 2 | 6.91 | 8.07 | 8.86 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.19 | 0.22 | 0.24 |  | 0.19 | 0.22 | 0.25 |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,8,2,1,1;1,8) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 6.61 | 7.7 | 8.45 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.13 | 0.15 | 0.17 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.4.1.3.1-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.3.1-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 4 | 4.35 | 2 | 3.61 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.14 |  | 0.13 |
| 4x16 MU-MIMO, OFDMA; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 15.55 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.63 |  | / |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 3 | 6.98 | 2 | 6.01 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.15 |  | 0.18 |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 2 | [4.19] |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | [0.15] |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.75 | 1 | 4.76 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.10 |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 5.08 | 1 | 5.05 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.09 |
| 4x16 MU-MIMO, Codebook based, OFDMA; gNB Config = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 14.40 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.58 |  | / |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.44 | 1 | 4.45 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.16 |
| 2x8 SU-MIMO, Non-codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 5.08 | 1 | 4.22 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.16 |  | 0.17 |
| 2x8 SU-MIMO, codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.71 | 1 | 4.74 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.20 |  | 0.16 |
| 2x8 SU-MIMO, codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.51 | 1 | 4.56 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.15 |
| 2x16 MU-MIMO, OFDMA; gNB Config = (4,8,2,1,1;1,8) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 4.19 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.22 |

5.4.1.3.2	Evaluation configuration B (CF = 4 GHz)

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration B are provided in Table 5.4.1.3.2-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.4.1.3.2-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 6 | 12.55 | 14.17 | 15.14 | 4 | 13.47 | 15.21 | 16.25 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.42 | 0.48 | 0.51 |  | 0.41 | 0.46 | 0.49 |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 17.32 | 19.64 | 21.01 | 1 | 16.63 | 18.79 | 20.09 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.49 | 0.56 | 0.59 |  | 0.52 | 0.58 | 0.62 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.58 | 14.23 | 15.22 | 1 | 11.96 | 13.53 | 14.47 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.12 | 0.14 | 0.15 |  | 0.13 | 0.14 | 0.15 |
| 32x4 MU-MIMO Type I codebook; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.76 | 12.07 | 12.86 | 1 | 5.30 | 5.96 | 6.35 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.22 | 0.25 | 0.26 |  | 1.76 | 1.98 | 2.11 |
| 16x4 MU-MIMO Type I codebook; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.21 | 11.43 | 12.16 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 | 0.18 | 0.19 |  | / | / | / |
| 16x4 MU-MIMO ideal CSI feedback; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | [14.23] | [16.11] | [17.23] |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | [0.38] | [0.43] | [0.45] |
| 32x8 MU-MIMO Type II Codebook; gNB Config = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.70 | 10.88 | 11.58 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.27 | 0.30 | 0.32 |  | / | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.35 | 17.83 | 19.52 | 1 | 15.23 | 17.77 | 19.50 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.37 | 0.43 | 0.48 |  | 0.38 | 0.44 | 0.48 |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 15.31 | 17.47 | / | 1 | 15.18 | 17.33 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.37 | 0.42 | / |  | 0.37 | 0.42 | / |
| 32x4 MU-MIMO, Type II Codebook based; gNB Config = (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 14.86 | 17.67 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.39 | 0.46 | / |  | / | / | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.04 | 17.46 | / | 1 | 15.17 | 17.84 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.58 | 0.68 | / |  | 0.49 | 0.58 | / |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 14.29 | 16.68 | 18.31 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.47 | 0.55 | 0.60 |  | / | / | / |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 30 | DDDDD DDSUU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 12.01 | 14.16 | 15.61 | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.29 | 0.35 | 0.38 |  | / | / | / |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;1,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | [12.67] | [14.74] | [16.16] |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | [0.57] | [0.66] | [0.72] |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 15.40 | 18.14 | 20.00 | 2 | 15.23 | 17.94 | 19.78 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.58 | 0.69 | 0.76 |  | 0.54 | 0.64 | 0.71 |
| 64x4 MU-MIMO, Reciprocity based; 4T SRS gNB Config = (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 14.52 | 16.90 | 18.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.49 | 0.57 | 0.63 |
| 128x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,32,2,1,1;2,32) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 10.36 | 12.08 | 13.25 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / | / | / |  | 0.13 | 0.15 | 0.16 |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration B are provided in Table 5.4.1.3.2-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.4.1.3.2-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x32 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 4 | 4.28 | 1 | 4.27 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.14 |  | 0.15 |
| 4x32 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 7.38 | 2 | [6.31] |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.29 |  | [0.15] |
| 4x32 MU-MIMO, OFDMA; gNB Config = (8,16,2,1,1;1,16) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 15.96 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.20 |  | / |
| 2x32 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 6.27 | 1 | 6.16 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.23 |  | 0.19 |
| 4x16 SU-MIMO, OFDMA; gNB Config = (8,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | / | 1 | [6.26] |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | [0.06] |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 1x32 SU-MIMO, Codebook based, OFDMA; gNB Config (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.18 | 1 | 3.12 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.11 |  | 0.09 |
| 1x32 SU-MIMO, Codebook based, OFDMA; gNB Config (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.50 | 1 | 3.43 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.12 |  | 0.10 |
| 4x32 SU-MIMO, Codebook based, OFDMA; gNB Config (8,8,2,1,1;2,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 5.73 | 1 | 5.76 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.13 |
| 4x32 SU-MIMO, Codebook based, OFDMA; gNB Config (8,8,2,1,1;2,8) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 6.24 | 1 | 5.98 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.18 |  | 0.15 |
| 4x32 MU-MIMO, Codebook based, OFDMA; gNB Config (8,16,2,1,1;1,16) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 21.30 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.26 |  | / |
| 8x64 SU-MIMO, Codebook based, OFDMA; gNB Config (4,32,2,1,1;1,32) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.73 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.07 |  | / |
| 2x64 SU-MIMO, Codebook based, OFDMA; gNB Config (12,8,2,1,1;4,8) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 7.83 | 1 | 7.66 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.31 |  | 0.32 |
| 2x64 SU-MIMO, Codebook based, OFDMA; gNB Config (12,8,2,1,1;4,8) | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 6.40 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.27 |
| 2x128, MU-MIMO, Codebook-based, OFDMA; gNB Config (8,16,2,1,1;4,16) | 30 | DDSU | Average [bit/s/Hz/TRxP] | 1.6 | / | / | 1 | 9.2 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | / |  | 0.11 |

5.4.1.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.4.1.3.3-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.1.3.3-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 8x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 7 | 7.93 | 8.93 | 9.53 | 5 | 8.15 | 9.18 | 9.8 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.22 | 0.24 |  | 0.21 | 0.24 | 0.25 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;2,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.13 | 9.13 | 9.73 | 1 | 8.14 | 9.13 | 9.73 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.24 |  | 0.2 | 0.23 | 0.24 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 9.02 | 10.22 | 10.93 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.3 | 0.34 | 0.36 |  | / | / | / |
| 8x4 MU-MIMO ideal CSI feedback; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | [11.26] | [12.73] | [13.61] |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | / | / | / |  | [0.27] | [0.30] | [0.32] |
| 8x4 MU-MIMO Type I codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | [9.29] | [10.42] | [11.11] | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | [0.29] | [0.33] | [0.35] |  | / | / | / |
| 8x4 MU-MIMO Type I codebook; gNB Config = (8,4,2,1,1;1,4) (Optimized for cell edge) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 3.9 | 4.37 | 4.65 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | / | / | / |  | 0.84 | 0.94 | 1.00 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.7 | 8.9 | 9.71 | 2 | 8.14 | 9.45 | 10.34 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.25 |  | 0.25 | 0.29 | 0.32 |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.01 | 9.02 | / | 1 | 8.02 | 9.04 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.22 | / |  | 0.2 | 0.22 | / |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;2,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.61 | 9.98 | 10.91 | 1 | 8.59 | 9.95 | 10.89 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.2 | 0.23 | 0.25 |  | 0.19 | 0.22 | 0.25 |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;2,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.83 | 9.98 | / | 1 | 8.82 | 9.96 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.22 | 0.25 | / |  | 0.21 | 0.24 | / |
| 16x4 MU-MIMO, Type II Codebook; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.44 | 12.41 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.35 | 0.41 | / |  | / | / | / |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,32,2,1,1;1,32) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 6.01 | 7.3 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | / | / | / |  | / | / | / |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.98 | 9.33 | / | 1 | 8.76 | 10.30 | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.25 | 0.29 | / |  | 0.23 | 0.27 | / |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.34 | 9.91 | 10.97 | 1 | 8.38 | 9.96 | 11.02 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.28 | 0.33 | 0.37 |  | 0.31 | 0.37 | 0.41 |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.24 | 6.04 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.16 | 0.18 | / |  | / | / | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.4.1.3.3-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.1.3.3-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 4.22 | 2 | 4.16 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.14 |  | 0.13 |
| 1x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | [4.54] | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | [0.09] |  | / |
| 4x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | [7.04] | 1 | [4.80] |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | [0.07] |  | [0.09] |
| 4x16 MU-MIMO, OFDMA; gNB Config = (4,8,2,1,1;1,8); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 8.10 | / | / |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.10 |  | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.33 | 1 | 3.31 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.06 |  | 0.05 |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.53 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.05 |  | / |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.84 | 2 | 3.53 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.07 |  | 0.07 |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.03 | 1 | 4.04 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.06 |  | 0.05 |
| 4x16 SU-MIMO, Codebook based, OFDMA; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.59 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.08 |  | / |
| 8x64 MU-MIMO, Codebook based, OFDMA; gNB Config = (4,32,2,1,1;1,32) | 15 | UUUUU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.99 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | / |  | / |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.41 | 1 | 3.56 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | / |  | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |  |

5.4.2	LTE

Average spectral efficiency and 5th percentile user spectral efficiency are evaluated for LTE. Both LTE FDD and TDD are evaluated. Detailed evaluation assumptions and results can be found in Annex B.z.1.

5.4.2.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for LTE.

5.4.2.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of DL spectral efficiency for LTE FDD and TDD for evaluation configuration A are provided in Table 5.4.2.3.1-1.

It is observed that both LTE FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.3.1-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 8x2 MU-MIMO Advanced CSI Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 4.66 | 1 | 4.63 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.12 |  | 0.12 |
| 16x2 MU-MIMO Advanced CSI Codebook; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 6.45 | 1 | 6.46 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.16 |  | 0.15 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 16x2 MU-MIMO, Reciprocity based; 2T SRS; gNB Config = (4,8,2,1,1;1,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 11.05 | 1 | 11.22 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.29 |  | 0.29 |

The evaluation results of UL spectral efficiency for LTE FDD and TDD for evaluation configuration A are provided in Table 5.4.2.3.1-2.

It is observed that both LTE FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration A.

Table 5.4.1.3.1-1 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 1x8 SU-MIMO, Codebook based, DFT-S-OFDM; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.59 | 1 | 3.59 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.10 |  | 0.10 |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.30 | 1 | 4.29 |
|  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.22 |  | 0.20 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.78 | 1 | 3.80 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.15 |  | 0.13 |

5.4.2.3.2	Evaluation configuration B (CF = 4 GHz)

The evaluation results of DL spectral efficiency for LTE FDD and TDD for evaluation configuration B are provided in Table 5.4.2.3.2-1.

It is observed that both LTE FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.4.2.3.2-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 32x4 MU-MIMO Type II Codebook; gNB Config = (8,8,2,1,1;2,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 9.65 | 1 | 9.63 |
|  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.28 |  | 0.28 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 32x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,8,2,1,1;2,8) | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | / | / | 1 | 12.25 |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | / |  | 0.43 |
| 64x8 MU-MIMO, Reciprocity based; 8T SRS; gNB Config = (4,32,2,1,1;1,32) | 15 | DSUDD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 14.75 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.12 |  | 0.36 |  | / |

The evaluation results of UL spectral efficiency for LTE for evaluation configuration B are provided in Table 5.4.2.3.2-2.

It is observed that both LTE fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration B.

Table 5.4.2.3.2-2 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz)

(a) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 8x64 SU-MIMO, Codebook based, OFDMA; gNB Config = (4,32,2,1,1;1,32) | 15 | DSUDD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.15 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | 0.045 |  | 0.07 |  | / |

5.4.2.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.4.2.3.3-1.

It is observed that both LTE fulfills the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.2.3.3-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 8x4 MU-MIMO, Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.96 | 1 | 5.97 |
|  |  | 5th-tile [bit/s/Hz] | 0.121 |  | 0.15 |  | 0.15 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.4.2.3.3-2.

It is observed that both LTE fulfills the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.2.3.3-2 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, Codebook based, DFT-S-OFDM; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.36 | 1 | 3.31 |
|  |  | 5th-tile [bit/s/Hz] | 0.0451 |  | 0.07 |  | 0.06 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement is not applicable to LMLC. The value shown here is for information only. |  |  |  |  |  |  |  |