3GPP TSG RAN meeting #81										RP-182070

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.7.2

| WI / SI Name | Additional enhancements for NB-IoT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | No |  | Testing part: | No |
| Acronym | NB_IOTenh3 |  |  |  |  |  |  |
| Unique ID | 800084 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181451 |  |  |  |  |  |  |
| Target Completion Date | Core part: 12/2019 |  |  |  | Performance part: 06/2020 |  |  |
| Is the WI/SI complete? | Core part: No |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | COZZO, Carmela |
|  | Company | Huawei |
|  | Email | carmela.cozzo@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table: Updated RAN3 TUs according to RAN#81 discussions.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#94

Contributions [1] - [64] were submitted to this meeting.

RAN1 discussed UE-group wake-up signal, with the following agreements

Agreements:

- UE-group WUSs are only multiplexed in the same carrier as associated PO

  - FFS TDM/CDM for UE-group NWUS multiplexing

- Rel-16 UE-group NWUS sequence should consider at least

  - Fallback to legacy UE behavior

  - Inter-cell interference randomization

  - UE group ID for different UE-group NWUS

  - Reuse of Rel-15 sequences is not precluded

  - Effect of sequence detection on UE complexity

- Study the RAN1 consequence of UE-grouping on the following basis:

  - UE ID

  - Coverage

  - DRX/eDRX

  - Gap configuration

  - Services

RAN1 discussed transmission in preconfigured UL resources, with the following agreements

Agreements:

- Idle mode based pre-configured UL resources is supported for UEs in possession of a valid TA

- FFS: Validation mechanism for TA

- FFS: How the pre-configured UL resources is acquired

- For transmission in preconfigured UL resources, the UE may use the latest TA of which its validity can be confirmed

- Study both shared and dedicated resource for preconfigured UL resources. If both shared and dedicated resources are supported, strive for commonality in design of both resource types.

- HARQ procedures for transmission in preconfigured UL resources should be studied and the following aspects should be considered:

  - Whether to support HARQ

    - If supported, details of HARQ design including the number of HARQ processes

  - Whether ACK/NACK is necessary

- Fallback mechanisms should be considered, e.g. fallback to legacy RACH/EDT procedures

RAN1 discussed scheduling of multiple DL/UL transport blocks, with the following agreements

Agreements:

- For unicast, scheduling multiple DL/UL transport blocks with single DCI is supported.

- One DCI to schedule multiple TBs for SC-MCCH is not supported

- For Unicast, the possibility of scheduling multiple DL/UL transport blocks is configured via RRC. Details TBD

- For unicast, the number of TBs scheduled should be dynamically indicated in the DCI, the maximum number of TBs is FFS

RAN1 discussed coexistence of NB-IoT with NR, with the following agreements

Agreements:

- Observation: From RAN1 perspective, no issues were identified that would prevent the coexistence of NR and NB-IoT

- RAN1 studies additional specification enhancement for improving the performance of coexistence of NB-IoT with NR.

RAN1 discussed the quality report in Msg3 for non-anchor access with the following agreements and working assumptions

Agreements:

- For channel quality report in Msg3 on non-anchor access, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%

- FFS: Whether the details on the hypothetical NPDCCH are specified or not

- Working Assumption: For channel quality report in Msg3 on non-anchor access, UE performs the channel quality measurement on the carrier it monitors to receive Msg2 (i.e. RAR)

- FFS: Whether the UE performs measurement on other carriers

- For non-anchor access, RAN1 further studies how UEs report the measured channel quality

RAN1 discussed the presence of NRS on a non-anchor carrier for paging with the following agreements

Agreements:

- The presence of NRS on subframes which will contain NRS even when no paging NPDCCH is transmitted is enabled by broadcast signalling.

  - Above applies only for non-anchor carriers

- For the objective on presence of NRS on a non-anchor carrier for paging, consider the following use case:

- Early termination of NPDCCH / WUS

- Above is not intended to restrict the study of other use cases in other WGs which may or may not have RAN1 specification impact

#### 2.1.2	Remaining Open issues

- Specify support for UE-group wake-up signal

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

  - Both shared resources and dedicated resources can be discussed.

  - Note: This is limited to orthogonal (multi) access schemes.

- Specify scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast.

- Conclude on additional potential specification enhancement for improving the performance of coexistence of NB-IoT with NR.

- Specify support of Msg3 quality reporting for non-anchor access.

- Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

## 2.2	RAN2

Not started.

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

Not started.

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

Not started.

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1808106 	Additional enhancements for NB-IoT workplan	Huawei, HiSilicon

- R1-1809532 	Feature summary of 6.2.2.1 UE-group wake-up signal	Ericsson

- R1-1808043	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1808107	On support for UE-group wake-up signal	Huawei, HiSilicon

- R1-1808216	On UE-group wake-up signal for NB-IoT	vivo

- R1-1808439	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1808474	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1808640	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1808660	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1808737	UE-group wake-up signal for eMTC	Samsung

- R1-1808959	UE-Group WUS in NB-IoT	MediaTek Inc.

- R1-1809031	UE-group wake-up signal	Qualcomm Incorporated

- R1-1809131	UE-group wake-up signal for Rel.16 IoT	NTT DOCOMO, INC.

- R1-1809483	UE-group wake-up signal for NB-IoT	Sony

- R1-1809571	Feature lead summary of Support for transmission in preconfigured UL resources Huawei, HiSi

- R1-1808044	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1808108	On support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1808358	Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R1-1808440	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1808475	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1808641	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1808661	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1808738	Discussion on transmission in preconfigured UL reosurces for NB-IOT	Samsung

- R1-1808960	Early Data Transmission on Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1809032	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1809132	UL transmission scheme in preconfigured resources	NTT DOCOMO, INC.

- R1-1809523	Feature lead summary of scheduling enhancement for NB-IoT	ZTE

- R1-1808045	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1808109	Discussion on scheduling multiple DL/UL transport blocks for SC-PTM and unicast	Huawei, HiSilicon

- R1-1808356	Multiple TB Grant Design for Unicast	Sierra Wireless, S.A.

- R1-1808441	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1808476	Discussion on multiple transmission blocks scheduling in NB-IoT	LG Electronics

- R1-1808559	Design of scheduling of multiple DL/UL transport blocks for Rel.16 NBIoT	Lenovo, Motorola Mobility

- R1-1808642	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1808739	Discission on scheduling of multiple TBs for NB-IoT	Samsung

- R1-1809033	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1809572	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSi

- R1-1808046	Coexistence of NB-IoT with NR	Ericsson

- R1-1808112	Discussion on NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1808442	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1808477	Discussion on coexistence of NB-IoT with NR	LG Electronics

- R1-1808643	Discussion on coexistence of NB-IoT with NR	ZTE

- R1-1808958	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1809034	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1809600	Feature lead summary on support of quality report in Msg3 for non-anchor access	Huawei, HiSi

- R1-1808047	Support of quality report in Msg3 for non-anchor access in NB-IoT	Ericsson

- R1-1808110	On support of Msg3 quality reporting for non-anchor access	Huawei, HiSilicon

- R1-1808443	Support of Quality report in Msg3 for non-anchor access	Nokia, Nokia Shanghai Bell

- R1-1808478   	Downlink channel quality report during random access procedure on a non-anchor carrier  LG Electronics

- R1-1808644	Support of Quality report in Msg3 for non-anchor access	ZTE

- R1-1808740	Discussion on quality report in Msg3 for NB-IoT	Samsung

- R1-1808961	Quality report in Msg3 on non-anchor carrier	MediaTek Inc.

- R1-1808962	NRS support on  non-anchor carrier	MediaTek Inc.

- R1-1809035	Support of quality report in msg3 for non-anchor	Qualcomm Incorporated

- R1-1809471	DL quality reporting in Msg3 for non-anchor access in NB-IoT	Intel Corporation

- R1-1809522	Feature lead summary of presence of NRS on a non-anchor carrier for paging	Qualcomm

- R1-1808048	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1808111	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1808444	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1808479	Discussion on NRS on an a non-anchor carrier	LG Electronics

- R1-1808645	NRS on non-anchor PRB	ZTE

- R1-1808741	Discussion on NRS presence on non-anchor carrier	Samsung

- R1-1809036	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R1-1808921	Early UL Transmission for NB-IoT and HD-FDD eMTC	Sequans Communications

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template