3GPP TSG RAN meeting #81										RP-182131

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.3

| WI / SI Name | Further NB-IoT enhancements |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | NB_IOTenh2 |  |  |  |  |  |  |
| Unique ID | 750066 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-172063 |  |  |  |  |  |  |
| Target Completion Date | Core part: 09/2018 |  |  |  | Performance part: 12/2018 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | WEBB, Matthew SUBRAMANI, Siva RICO-ALVARINO, Alberto SUI, Yutao |
|  | Company | Huawei Vodafone Qualcomm Ericsson |
|  | Email | matthew.webb@huawei.com siva.subramani@vodafone.com albertor@qti.qualcomm.com yutao.sui@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

None – core part completed in RAN#80.

#### 2.1.2	Remaining Open issues

None.

## 2.2	RAN2

#### 2.2.1	Agreements

CRs to TS 36.331 were agreed in R2-1813091 (for EDT) and R2-1813083 (for objectives other than EDT).

#### 2.2.2	Remaining Open issues

None.

## 2.3	RAN3

#### 2.3.1	Agreements

Papers [1] – [15] were submitted to this meeting.

RAN3 discussed UE differentiation based on RAN2 LS i.e.  subscription-based UE information IEs and/or a local RRM policy container. No agreement was reached.

#### 2.3.2	Remaining Open issues

RAN3 need to decide what to specify regarding the NB-IoT UE differentiation objective via UE subscription information and/or a local RRM policy container.

## 2.4	RAN4

#### 2.4.1	Agreements

Papers [16] – [56] were submitted to this meeting.

RRM core part

RAN4 further discussed on the NPBCH-based RRM measurement for FeNB-IoT. It is agreed in RAN4 that,

- it is feasible to use NPBCH for RRM measurement for serving cell

- it is up to UE implementation whether to use NPBCH in addition to NRS for RRM measurement, and RAN4 will not introduce any new requirement for NPBCH-based RRM measurement

An LS is sent to inform RAN2 about the above agreements in [47].

Regarding the WUS RRM requirements, a way forward in [53] is approved. It is captured in the way forward that,

- RAN4 agrees to work on the remaining part of WUS reception requirements for Rel-15 feNB-IoT under TEI15 agenda Q4-2019

- No test is defined to verify the minimum WUS reception requirements

One consolidated CR is agreed to finalize the remaining part of the WUS RRM relaxation requirements in TS36.133 in [56].

RAN4 also discussed in this meeting on the high quality threshold for NB-IoT and it is agreed that there is no need to lower the current high quality threshold used for PLMN selection by NB-IoT UE. An LS is sent to RAN2 to response in [30].

RRM performance

A way forward on enhanced PHR reporting for Rel-15 NB-IOT is approved in [55], in which we agreed that:

- existing PHR reporting requirements apply for Rel-15 NB-IoT UEs which are not capable of enhanced PHR reporting

- for Rel-15 NB-IoT UEs which are capable of enhanced PHR reporting, the new values are used to enhance the existing PHR reporting tables and the values of new tables are TBD and companies are encouraged to provide their views at RAN4#88bis meeting.

Test cases list is approved in [54] which also captured the work split among the interested companies of the listed test cases.

RAN4 further discussed on the NSSS-based RRM measurement accuracy enhancement for FeNB-IoT. It is agreed that,

- UE should meet the tighter measurement accuracy requirement derived based on NSSS-based RRM measurement when NSSS is always transmitted from the same antenna port across time

- UE should meet at least the legacy measurement accuracy requirement derived based on NRS-based RRM measurement when NSSS is transmitted using different precoders across time

- RAN4 also found that it is beneficial to inform UE whether the NSSS is always transmitted from the same antenna port or not either by adding “n1” as one of the enumerations for nsss-NumOccDiffPrecoders IE, or alternatively having the absence of the IE implicitly interpreted as “n1”

An LS is sent to inform RAN2 about the agreements above in [49]. A CR is agreed to capture the agreements in TS36.133 in [48].

TDD RF performance

E-UTRA TDD config 1 was confirmed to be used when testing with NB-IoT inband/guard band. To finalize the BS conformance testing, the following CR were agreed

- R4-1809689	CR to TS 36.104: Corrections on NB-IoT operating band unwanted emission and blocking requirements (6.6.3.3, 7.6.1.1)

- R4-1811575	CR to TS 36.141: Finalization of NB-IoT TDD test model and correction on NB-IoT operating band unwanted emission requirement (6.1.1, 6.1.3, 6.6.3.5.3)

- R4-1811576	CR to TS 36.141 - NB-IoT TDD introduction - TM updates

- R4-1811577	CR to TS 37.141 - NB-IoT TDD introduction

UE demodulation performance:

WF R4-1811370 is agreed.

RAN4 agreed to reuse the NPBCH TDD performance requirements for NPBCH TDD, with the RMC and test parameters updated for TDD.

RAN4 agreed to add a certain amount of margin on top of the existing NB-IoT FDD demodulation performance requirements for NB-IoT TDD as per the further simulation results.

BS demodulation performance:

WF R4-1811371 was approved.

Further down selection was made about the NRPACH TDD formats to preamble format 0 and 0-a for demodulation performance requirements definitions. If needed, performance requirements for other preamble formats can be added later

#### 2.4.2	Remaining Open issues

RRM core part

RAN4 agrees to work on the remaining part of WUS reception requirements for Rel-15 feNB-IoT under TEI15 agenda Q4-2019.

RRM performance part

The enhanced PHR reporting tables are to be updated.

Specific test cases requirements are to be defined in the spec for Rel-15 NB-IoT.

There are no RF open issues.

UE demodulation performance:

Whether TDD UL DL config#1 and special subframe config#4 that are using in the existing LTE demodulation performance requirements or TDD UL DL config#2 and special subframe config#7 that are using in the real network should be used for NB-IoT TDD demodulation performance requirements especially considering the in-band and guard-band scenario.

BS demodulation performance:

Similar open issue about TDD UL DL configuration 1 or 2 and special subframe configuration 4 or 7 as UE demodulation performance requirements.

## 2.5	RAN5

N/A

## 2.6	RAN6

N/A

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

N/A

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R3-184438	Introduction of UE differentiation	Huawei, China Telecom, CMCC, Ericsson, Vodafone, China Unicom

- R3-184439	Introduction of UE differentiation	Huawei, China Telecom, CMCC, Ericsson, Vodafone, China Unicom

- R3-184440	Consideration on UE differentiation	Huawei

- R3-184975	Introduction of the UE differentiation Information	Ericsson, Vodafone, Huawei

- R3-185100	Response to R3-184440	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

- R3-185110	NB IoT Session report	Vice chairman (Intel)

- R3-185130	Response to R3-184440	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Orange, Verizon Wireless

- R3-185195	Introduction of UE differentiation Information	Huawei, China Telecom, CMCC, Ericsson, Vodafone, China Unicom

- R3-185196	Introduction of UE differentiation Information	Huawei, China Telecom, CMCC, Ericsson, Vodafone, China Unicom

- R3-185197	Introduction of Subscription based UE differentiation	Huawei, China Telecom, CMCC, China Unicom

- R3-185198	Introduction of Subscription based UE differentiation	Huawei, China Telecom, CMCC, China Unicom

- R3-185199	Introduction of local RRM based UE differentiation information	Huawei

- R3-185200	Introduction of local RRM based UE differentiation information	Huawei

- R3-185201	Introduction of local RRM based UE differentiation information	Huawei

- R3-185290	SoD NB IoT	Huawei

- R4-1809689	CR to TS 36.104: Corrections on NB-IoT operating band unwanted emission and blocking requirements (6.6.3.3, 7.6.1.1)	Nokia, Nokia Shanghai Bell

- R4-1809690	CR to TS 36.141: Finalization of NB-IoT TDD test model and correction on NB-IoT operating band unwanted emission requirement (6.1.1, 6.1.3, 6.6.3.5.3)	Nokia, Nokia Shanghai Bell

- R4-1810075	CR to TS 36.141 - NB-IoT TDD introduction - TM updates	Ericsson

- R4-1810076	CR to TS 37.141 - NB-IoT TDD introduction	Ericsson

- R4-1810535	UE demodulation requirement for FeNB-IoT TDD	Qualcomm Incorporated

- R4-1810541	On the feasibility of NPBCH-based RRM measurement	Qualcomm Incorporated

- R4-1810543	LS response on NPBCH-based RRM measurement for FeNB-IoT UE	Qualcomm Incorporated

- R4-1810558	Serving cell RRM relaxation requirement for WUS-capable UE	Qualcomm Incorporated

- R4-1810632	CR on WUS RRM requirements	Huawei, HiSilicon

- R4-1810633	Test case list for R15 NB	Huawei, HiSilicon

- R4-1810634	Discussion on the enhanced PHR	Huawei, HiSilicon

- R4-1810635	CR on PHR enhancement	Huawei, HiSilicon

- R4-1810749	Discussions on high quality signal threshold for category M1/M2	Ericsson

- R4-1810760	Discussions on high quality signal threshold for NB-IoT	Ericsson

- R4-1810761	Response LS on high quality signal threshold in NB-IoT	Ericsson

- R4-1810762	Discussions on remaining work on serving cell measurement relaxation	Ericsson

- R4-1810763	Enhanced PHR reporting for Rel-15 NB-IOT UEs	Ericsson

- R4-1810764	Introduction of enhanced PHR for category NB1	Ericsson

- R4-1810765	Discussions on test configurations for Rel-15 NB-IoT	Ericsson

- R4-1810766	Discussions on test case scenarios for Rel-15 NB-IoT	Ericsson

- R4-1810767	List for RRM test cases for Rel-15 NB-IoT	Ericsson

- R4-1810768	Simulation results for WUS reception for NB-IoT	Ericsson

- R4-1810769	Remaining work on minimum WUS reception requirements for NB-IoT	Ericsson

- R4-1810939	Remaining work on serving cell measurement relaxation for category NB1	Ericsson

- R4-1810972	Addition of NSSS-based RRM measurement accuracy requirement	Qualcomm Incorporated

- R4-1811157	Discussion on FeNB-IoT enhancements UE demodulation performance requirements	Huawei, HiSilicon

- R4-1811158	Way forward for FeNB-IOT UE demodulation performance requirements	Huawei, HiSilicon

- R4-1811159	Discussion and simulation results on FeNB-IoT BS demodulation requirements	Huawei

- R4-1811160	Way forward for FeNB-IOT BS demodulation performance requirements	Huawei

- R4-1811370	Way forward for FeNB-IOT UE demodulation performance requirements	Huawei, HiSilicon

- R4-1811371	Way forward for FeNB-IOT BS demodulation performance requirements	Huawei

- R4-1811386	LS response on NPBCH-based RRM measurement for FeNB-IoT UE	Qualcomm Incorporated

- R4-1811389	Addition of NSSS-based RRM measurement accuracy requirement	Qualcomm Incorporated

- R4-1811417	LS on NSSS-based RRM measurement for FeNB-IoT UE	Qualcomm

- R4-1811575	CR to TS 36.141: Finalization of NB-IoT TDD test model and correction on NB-IoT operating band unwanted emission requirement (6.1.1, 6.1.3, 6.6.3.5.3)	Nokia, Nokia Shanghai Bell

- R4-1811576	CR to TS 36.141 - NB-IoT TDD introduction - TM updates	Ericsson

- R4-1811577	CR to TS 37.141 - NB-IoT TDD introduction	Ericsson

- R4-1811691	Way forward on WUS RRM requirements for Rel-15 NB-IOT	Ericsson, Sony

- R4-1811705	List for RRM test cases for Rel-15 NB-IoT	Ericsson

- R4-1811718	Way forward on enhanced PHR reporting for Rel-15 NB-IOT	Ericsson

- R4-1811731	CR on WUS RRM requirements	Huawei, HiSilicon

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template