3GPP TSG RAN meeting #82										RP-182334

Sorrento, Italy, December 10-13, 2018

## Status Report to TSG

Agenda item:			10.4.3

| WI / SI Name | SID on LTE-based 5G Terrestrial Broadcast |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_LTE_terr_bcast |  |  |  |  |
| Unique ID | 800091 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-181706 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2019 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 60 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Alberto Rico Alvarino |
|  | Company | Qualcomm Incorporated |
|  | Email | albertor@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#94b, the following agreements were reached:

Agreement:

“Gap analysis” can be based on all available techniques in Rel-14 and focuses on MBSFN and SC-PTM.

Decision: R1-1811661 is endorsed as TR36.776 v0.0.3

Agreement:

Spectral efficiency is one of performance metrics to be used for evaluation.

Agreement:

Pathloss model of ITU-R P.1546-5 is to be used for evaluation.

Agreement:

Full buffer traffic model is to be used for evaluation.

Agreement:

The first step in the evaluation of a requirement should be to determine whether the requirement is already met by LTE or not. If the requirement is partially met, the remaining gaps should be identified.

Agreement:

Only Req.7 and Req.8 involve simulation work. All the other requirements are to be verified based on analysis of current specifications.

Agreement:

CAS evaluation should be included in the simulations.

Agreement:

The following table provides the general parameters for system level simulations.

- Note: Potential enhancements, including evaluations, can be presented to RAN1#95

| Parameter | MPMT | HPHT-1 | HPHT-2 | LPLT |
| --- | --- | --- | --- | --- |
| ISD | 50km | 125km | 173.2km | 15km |
| BS Power | 60dBm | 70dBm | 70dBm | 46 dBm |
| BS effective antenna height | [100m] | 300m | 300m | 35m |
| CPs/Numerologies | CAS/SC-PTM: 4.7 us, 16.6us PMCH: 15kHz/7.5kHz/1.25kHz as baseline. |  |  |  |
| Carrier frequency | 700MHz |  |  |  |
| Channel BW | 10MHz as baseline |  |  |  |
| BS antenna gain | 10.5dBi | 13dBi | 13dBi | 15dBi |
| BS antenna pattern | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 1Tx: rooftop and non-rooftop 2Tx: non-rooftop (FFS for rooftop) | 3 sector antenna pattern Each sector has a horizontal and vertical pattern as defined in: 3GPP TR36.814, Table A.1.1: 3GPP Case 1 and Case 3 (Macro Cell) 2Tx for non-rooftop (FFS for rooftop) |
| Cellular Layout | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites, 1 sector per site | Hexagonal grid, 61 cell sites,3 sector per site |
| Tx EVM | [8%] |  |  | 8% |

Agreement:

The following table provides the channel characteristics for system level simulations.

| Parameter | Rooftop | Car-mounted | Indoor |
| --- | --- | --- | --- |
| Propagation model | ITU-R P.1546-5 50% / 1% (serving / interfering) | ITU-R P.1546-5 50% / 1% (serving / interfering) | ITU-R P.1546-5 50% / 1% (serving / interfering) |
| Fast fading channel type | FFS | FFS | FFS |
| Receiver velocity | 0 | Up to 250km/h | 3km/h |
| Receiving antenna height | 10m | 1.5m | 1.5m |
| Height Loss: The difference between the signal level at 10m and the actual receiving antenna height | 0dB | 16.5dB for rural 23.5 dB for urban | 16.5dB for rural 23.5 dB for urban |
| Building penetration loss | 0dB | 0dB | 20dB for urban 10dB for rural |
| Shadowing standard deviation | 5.5dB | 5.5dB | 8dB for rural 6dB for urban |
| Shadowing correlation | 1 for same site | 1 for same site | 1 for same site |

Agreement:

The following table provides the receiver characteristics for system level simulations.

| Parameter | Rooftop | Car-mounted | Indoor |
| --- | --- | --- | --- |
| Receiver noise figure | 6dB | 6dB | 9dB |
| Receiver noise bandwidth | 9MHz | 9MHz | 9MHz |
| Receiver antenna (gain & pattern) | 13.15 dBi Discrimination pattern according to ITU-R BT.419-3 band IV, V | 3.0 dBi Non-directional | -7.5dBi |
| Antenna Cable Loss | 4dB | 0dB | 0dB |
| 2-Rx Diversity | No (FFS for 2Tx) | Yes | Yes |
| Implementation Margin | 1dB |  |  |
| Body loss at receiver | 0 |  |  |
| Rx EVM | 4% |  |  |

Agreement:

- One of the following two options (to be declared by company) is used for rooftop antenna alignment:

  - Opt1: Strongest transmitter (based on pathloss and shadowing).

  - Opt2: Closest transmitter

- X% pre-processing SINR (to be mapped to X% tput by LLS) is used as a performance metric

  - Reporting points: 95% SINR. Other percentiles may also be reported

Agreement:

The following options may be used for the distribution of UEs in the system level simulations:

- Uniform in all cells as a baseline

- “Worst case” (i.e., drop all UEs in the geographical point with worse coverage) as an additional option

Agreement:

The synchronization method used (e.g., max SINR, max energy) should be reported with the evaluations

The following agreements were reached on email discussion after RAN1#94b:

Agreements:

- For fast fading generation,

  - For RAN1#95

    - Delay spread and path delay profile: adopt UMA/RMA model (according to TR 38.901 sc. 7.5 step 5 and 6 and ITU-R M.2412 sc. 4.3 step 5 and 6), all scenarios

    - Fading distribution: Rayleigh except for the first path of rooftop (Rice). For rooftop, used fixed K=10dB

      - NOTE: this intends to describe the following procedure:

        - Run up to step 6 to obtain the delay coefficients per cluster (e.g. for RMa LOS, 11 clusters)

        - Normalized as described

        - Instead of generating the rays per cluster, simply take each of the delay coefficients for each cluster and generate a Gaussian random variable with the variance according to the power of the cluster. This is equivalent to using step 10b in ITU-R M.2135 and assuming uncorrelated antennas.

    - These initial evaluations can be used for evaluating the baseline and for proposing potential enhancements, to be further verified in RAN1#96.

  - For RAN1#96

    - Discuss whether to keep RAN1#95 modelling or move to complete UMa/RMa 2D/3D with ITU-T P.1546 50/1 for all scenarios. FFS:

      - tx/rx horizontal/vertical antenna patterns

      - number of tx/rx antenna elements and antenna configuration

      - 3D vs. 2D for each scenario

      - To be discussed during RAN1#95

- LLS parameters are given by Table 1.

Table 1 LLS parameters

| Parameter | Values |
| --- | --- |
| System BW | 10 MHz |
| Carrier frequency | 700 MHz |
| CP/Numerology | CAS/SC-PTM: 4.7 us, 16.6us PMCH: 15kHz/7.5kHz/1.25kHz as baseline. |
| Doppler frequency | 1Hz (rooftop antenna), Up to 120km/h and 250km/h for car-mounted/in-car, up to 3km/h for Indoor |
| Reference signal pattern | For existing numerologies, use existing reference signal patterns as baseline. |
| Channel estimation | Realistic based on RS design |
| Channel model | To be determined from system level evaluations for different ISDs. TDL-i for NLOS and TDL-v for LOS with the scaled delay spread derived from system level simulations. |
| Number of Rx antennas at the UE | Rooftop: 1 (baseline) /2 Mobile/car receiver: 2 |
| MCS | Fixed MCS, selected by proponent |
| NOTE: Potential enhancements, including evaluations, can be presented to RAN1#95 |  |

- For in-car scenario, use μ = 9dB, and σ = 5dB for I2O loss, shadowing standard deviation of 8dB (rural) and 6dB (urban), and receiver antenna gain of -7.35dBi

- Prioritize the scenarios highlighted in Table 4. RAN1 should strive towards providing the complete set of simulation results

Table 4 Scenarios for prioritization

|  | MPMT | HPHT-1 | HPHT-2 | LPLT |
| --- | --- | --- | --- | --- |
| Rooftop | QC, BBC, ABS, HW | QC, BBC | ABS, HW | ABS, HW |
| Car-Mounted | QC, BBC, ABS, HW | BBC | HW | BBC, ABS, HW |
| In-Car Handset | BBC, ABS, HW |  | HW | QC, BBC, ABS, HW |
| Indoor | ABS, HW |  | ABS, HW | ABS, HW |

QC: Qualcomm; BBC: British Broadcasting Corp.; ABS: Academy of Broadcasting Science; HW: Huawei

Agreements:

- All proposals & TPs in R1-1811738 are agreed, except:

  - FFS proposal 5 & TP4

  - Replacing Rel-15 by Rel-14 in the proposals & TPs

R1-1811738	Email discussion on requirements not requiring simulations	Qualcomm Incorporated

R1-1811739	LS on 5G requirements for dedicated broadcast networks	RAN1, Qualcomm

Decision: LS to SA2 is approved.

In RAN1#95, the following agreements were reached

R1-1813054 is endorsed. Note that subclauses reflecting requirements that are not relevant for dedicated networks may be removed/modified depending on further discussions in RAN1.

R1-1813786	Updated TR for 36.776 v0.0.4	Qualcomm Incorporated

Agreement:

For SINR calculation, the formula in ITU-R BT.2243-3 page 47 is used. Positioning of the EI is up to receiver implementation and should be declared in the evaluation, including the length of the EI.

Agreement:

For CAS link-level evaluations:

- Link level simulations are required in order to confirm the SINR range [-10 dB to -7 dB] that the CAS would reliably operate in the transmission channels appropriate for fixed rooftop reception (1T1R)

- Link level simulations are required in order to confirm the SINR range [-6 dB to -4 dB] that the CAS would reliably operate in the transmission channels appropriate for car-mounted receiver (1T2R).

Agreement:

The following enhancements are to be evaluated for RAN1#96:

- High priority items:

- New numerologies for support of larger ISDs and mobility

- New MBSFN-RS patterns

- MUST for SC-PTM/PMCH

- Low priority items:

- Higher modulation schemes

- Time-based interleaving

- Lower guard BW for new numerologies

Agreement:

While 50/1 as the preferred assumption for all scenarios is already agreed, evaluations based on 50/50 for SC-PTM can be captured in the TR and would be clearly marked as such to allow for proper conclusions.

The text proposal in Section 4 of R1-1813056 is endorsed for inclusion in the TR.

Text proposals TP1 and TP2 in R1-1813785 are endorsed.

Agreement:

Proposal 1 in R1-1812248 is agreed in principle. FFS: Details and notation to capture this in the TR.

#### 2.1.2	Remaining Open issues

For requirements 2, 6, 7, 8 in TR 38.913:

- Identify if these requirements are relevant for dedicated terrestrial broadcast networks.

  - Capture the gap analysis and potential solutions (if needed) to meet the broadcast requirements in a TR.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

The following LS was sent to SA2 (CC CT3, SA4):

R1-1811739	LS on 5G requirements for dedicated broadcast networks	RAN1, Qualcomm

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

SA2 reply to the LS is related to objectives 2 and 6 in TR 38.913.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

R1-1811728	       Chairman's notes of AI 6.2.4 Study on LTE-based 5G Terrestrial Broadcast	Ad-hoc chair (Ericsson)

R1-1810170	Techniques for networks supporting TV services	Huawei, HiSilicon, Shanghai Jiao Tong University, ABS

R1-1810931	      Skeleton TR for 36.776	 Qualcomm Incorporated

R1-1811661	      Skeleton TR for 36.776	 Qualcomm Incorporated

R1-1810169	Relevant requirements for dedicated broadcast networks and evaluation methodology	Huawei, HiSilicon, Shanghai Jiao Tong University, ABS

R1-1810319	Public service broadcaster requirements and background information relevant to LTE-based 5G Terrestrial Broadcast	EBU, BBC. IRT

R1-1810933	      Requirements and evaluation methodology	Qualcomm Incorporated

R1-1811318	      5G Broadcast Requirements Analysis for LTE	Charter Communications

R1-1810171	Scenarios and simulation assumptions for "gap analysis"	Huawei, HiSilicon, Shanghai Jiao Tong University, ABS

R1-1810934	     Scenarios for evaluation and simulation assumptions	Qualcomm Incorporated

R1-1811588	     Scenarios and simulation assumptions for the LTE based terrestrial broadcast gap analysis	EBU, BBC, IRT

R1-1811731	      Summary of offline discussion on simulation assumptions	Qualcomm Incorporated

R1-1811738	      Email discussion on requirements not requiring simulations	Qualcomm Incorporated

R1-1811739	      LS on 5G requirements for dedicated broadcast networks	RAN1, Qualcomm

R1-1813773	Chairman's notes of AI 6.2.4 Study on LTE-based 5G Terrestrial Broadcast	Ad-hoc chair (Ericsson)

R1-1813054	      Updated TR for 36.776	Qualcomm Incorporated

R1-1813350	Spectral efficiency requirement for LTE-based 5G broadcast	ABS, Shanghai Jiao Tong University, Huawei, HiSilicon

R1-1813785	The conclusions of the offline discussions on LTE-based 5G terrestrial broadcast		Qualcomm Incorporated

R1-1813786	Updated TR for 36.776 v0.0.4	Qualcomm Incorporated

R1-1813711	      Preliminary evaluation results for LTE-based broadcast network	Huawei, HiSilicon

R1-1812430	      Evaluation Results for LTE-Based 5G Terrestrial Broadcast	EBU

R1-1813055	      Evaluation results and potential enhancements	Qualcomm Incorporated

R1-1813056	      Non-simulation requirements	Qualcomm Incorporated

R1-1813628	      Remaining requirements not involving simulations	Huawei, HiSilicon

R1-1812247	Potential enhancements for higher spectral efficiency	Huawei, HiSilicon, Shanghai Jiao Tong University, ABS

R1-1812248	Remaining issues on simulation assumptions and evaluation methodologies for LTE-based broadcast network	Huawei, HiSilicon

R1-1813710        Broadcast/Multicast and Unicast Superposition Transmission (BMUST) for 5G Physical Layer Radio Interfaces            BBC

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template