3GPP TSG RAN Meeting #82																			RP-182458

Sorrento, Italy, December 10-13, 2018

Source: 	Intel Corporation

Title:	Introduction of DL 1024QAM for NR in Rel-16	Agenda item:	9.8

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss motivations to introduce support of 1024QAM for DL NR in Rel-16.

# Motivation of 1024QAM for DL NR

For Rel-15 LTE, based on the conclusion from the study item phase [1], support of 1024QAM was introduced for DL. According to [2], 1024QAM, can be used in several scenarios such communication with stationary devices such as laptops, docket smartphone, outdoor-or-indoor customer premises equipment (CPE), integrated wireless backhaul to other cells in dense networks, to provide higher spectral efficiency over 256QM. In Rel-15, the highest modulation order supported in NR is still based on 256QAM and for Rel-16 there is no SI/WI scope to introduce 1024QAM modulation support.

This means that NR for the same types of the devices cannot provide higher spectral efficiency by using 1024QAM. Moreover, under same conditions between LTE and NR such as supported number of MIMO layers, overhead, etc., the theoretical peak spectral efficiency and data rates for LTE will be higher than that for NR. As an example, we will end up to show the better peak spectral efficiency with LTE than with NR towards IMT-2020 submission. Some examples are shown from TR37.910 demonstrating the better SE with LTE than NR only because LTE supports 1024QAM although quite lower overhead was assumed for NR [3].

- Peak spectral efficiency

  - FDD

    - NR (FR1 with SCS15 as max): 45.4 ~ 47.0 bit/s/Hz

    - LTE: 47.11 bit/s/Hz

  - TDD

    - NR (FR1 with SCS15 as max, DSUUD, S slot = 11DL:1GP:2UL, αDL=0.5643, with OH3): 45.2 bit/s/Hz

    - LTE (DSUUD, S slot = 11DL:1GP:2UL, αDL=0.5643): 46.57 bit/s/Hz

Another motivation for introduction of 1024QAM in NR, could be that it might give an impression that NR can support lower maximum modulation order than LTE or other wireless access technology operating in a similar frequency bands (e.g. IEEE 802.11ax). In our view, NR needs to be at least par with LTE or in general better than LTE and any other wireless technology operating in the same frequency bands.

Since IMT-2020 submission will be based on the releases up to 16, in order to properly include the results with 1024 QAM in DL NR, we propose to specify 1024 QAM for DL NR in Rel-16. On the other hand, considering feasibility study and impairment impact, we also suggest to specify 1024 QAM in FR1 in Rel-16.

The potential work objectives of the normative work for specification of 1024QAM can be as follows:

- Specify MCS table and CQI feedback to support 1024QAM [RAN1]

- Specify a related performance requirements for 1024QAM in FR1 [RAN4]

- Specify the relevant higher layer signaling and define the UE capability for 1024QAM [RAN1/RAN2/RAN4]

This work can be done in either NR TEI-16 or a short WI in Rel-16 and further discussion is needed which work item is more proper to handle this item, if approved.

# Conclusions

In this contribution, we discussed motivations to introduce 1024 QAM for DL NR in Rel-16. Based on the discussions, we propose:

Proposal:

- Specify 1024 QAM for DL NR in FR1 within Rel-16 time frame with the following objectives

  - Specify MCS table and CQI feedback to support 1024 QAM [RAN1]

  - Specify the related performance requirements for 1024QAM in FR1 [RAN4]

  - Specify the relevant higher layer signaling and define the UE capability for 1024QAM [RAN1/RAN2/RAN4]

- To decide in RAN#82 which WI (including TEI-16) will handle the proposal

# References

[1] TR 36.783, Introduction of 1024QAM in LTE downlink

[2] RP-171738	Revised WID: Enhancements for high capacity stationary wireless link and introduction of 1024 QAM for LTE DL	Huawei, HiSilicon, Qualcomm Incorporated, China Telecom

[3] TR37.910	Study on Self Evaluation towards IMT-2020 Submission