3GPP TSG RAN Meeting #83																			RP-190208

Shenzhen, China, March. 18 - 21, 2019

Source: 	ZTE

Title:	Review Draft TP to TR 38.807 - Operational requirements: spectrum availability and regulatory requirements

Agenda item:	9.3.17

Document for:	Discussion

- Introduction

In RAN#82, a RAN-level Study on NR beyond 52.6GHz was updated [1], and a draft text proposal for capturing spectrum availability and regulatory requirements for frequency beyond 52.6 GHz based on submitted contributions from RAN Plenary #82 [2] was postponed to allow for further review.

In this contribution we present our updated on spectrum allocation for three countries, including China, Australia and Singapore, from 52.6GHz to 114.25GHz. Moreover, spectrum availability and regulatory requirements is introduced in India.

- Text Proposal

The following is a draft text proposal for section 4 of TR38.807.

==================== Start of Text Proposal =====================

4.2.3	ITU Region 3

4.2.3.1	China

Table 4.2.3.1-1 shows the spectrum allocations for China for frequency between 52.6GHz and 114.25GHz based on [XX]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.3.1-1: China spectrum allocation between 52.6GHz and 114.25GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) RADIO ASTRONOMY | 5.340, 5.556 |
| 54.25-55.78 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE |
| 55.78-56.9 | EARTH EXPLORATION-SATELLITE (passive) Fixed INTER-SATELLITE Mobile SPACE RESEARCH (passive) | 5.557A for Fixed, 5.556A for INTER-SATELLITE, 5.5558 for Mobile , 5.547 |
| 56.9-57 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) Mobile Fixed | 5.558A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 57-58.2 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) Mobile Fixed | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 58.2-59 | EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) RADIO ASTRONOMY Mobile Fixed | 5.547, 5.556 |
| 59-59.3 | EARTH EXPLORATION-SATELLITE (passive) INTER-SATELLITE SPACE RESEARCH (passive) Mobile Fixed | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.559 |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558 for MOBILE, 5.559 for INTER-SATELLITE, 5.138 |
| 64-65 | FIXED INTER-SATELLITE RADIO ASTRONOMY | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE RADIOLOCATION | 5.553 for MOBILE, 5.558 for MOBILE, 5.554 |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) | 5.561 |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 77.5-78 | AMATEUR AMEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) | 5.149, 5.559B for RADIOLOCATION |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) | 5.149, 5.560 |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A for FIXED, 5.149, 5.561A |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.338A for FIXED, 5.149 |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.340 |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.338A for FIXED, 5.149 |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy | 5.562, 5.562A |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.149 |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.149, 5.554 |
| 100-102 | SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY | 5.340, 5.341 |
| 102-105 | FIXED MOBILE RADIO ASTRONOMY | 5.149, 5.341 |
| 105-109.5 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.149 for RADIO ASTRONMY, 5.341, 5.562B for SPACE RESEARCH (passive) |
| 109.5-111.8 | RADIO ASTRONOMY SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) | 5.340, 5.341 |
| 111.8-114.25 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.149 for RADIO ASTRONMY, 5.341, 5.562B for SPACE RESEARCH (passive) |

In China, MIIT (Ministry of Industry and Information Technology of the People's Republic China) publishes that short distant unlicensed applications can be applied between 59-64 GHz. While, 76-77GHz is considered for Vehicle Radar ranging application [15][16]. Details are summarized in Table 4.2.3.1-1.

Table 4.2.3.1-2: Regulatory requirements for available spectrum between 52.6 GHz and 114.25 GHz in China

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 59-64 | Output power of antenna port: ≤ 10 dBm Peak EIRP：47dBm Mean EIRP: ≤ 44 dBm | No requirement | Not specified | very low transmission power and short distant unlicensed applications | The unlicensed devices operating in 59-64 GHz shall cause interference to, nor claim protection from legal radio services, i.e. the space research service, earth exploration satellite service, radio astronomy service, etc. Carrier frequency tolerance: 500 x 10-6 Out of Band Emission limit (EIRP): -5dBm/MHz Spurious Emission limit (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz Spurious Emission limit in Idle/stand-by state (corresponding to frequency range outside 2.5*carrier bandwidth ): -47dBm/1MHz |
| 76-77 | Maximum EIRP：55dBm | No requirement | Not specified | Vehicle Range Radar | Spurious Emission limit (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz Spurious Emission limit in Idle/stand-by state (corresponding to frequency range outside 2.5*carrier bandwidth ): -47dBm/1MHz |

=================== End of Text Proposal ========================

….

==================== Start of Text Proposal =====================

4.2.3.4	India

The Government of India Ministry of Communications’ Telecommunications Department has recently released a national frequency allocation plan [26]. Table 4.2.3.4-1 is populated per this plan. The NFAP is released typically every 3 to 4 years and the next one is not expected until after 2020.

Table 4.2.3.4-1: India spectrum allocation between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 56.9 – 59.3 | Not defined | Not defined | Not defined | -ESS (Passive) -Space Research (Passive) - Fixed - Mobile |  |
|  | 59.3 – 71 | Not defined | Not defined | Not defined | -Fixed -Inter-satellite -Mobile -Radiolocation -Space Research |  |
|  | 71 – 78 | Not defined | Not defined | Not defined | -fixed satellite(space-to-earth) -Fixed -Broadcasting -Space Research -Radio astronomy |  |
|  | 78 – 86 | Not defined | Not defined | Not defined | -Radiolocation -Amateur Satellite -Radio astronomy -Space Research (Space-to-Earth) -Fixed -Mobile |  |
|  | 86-95 | Not defined | Not defined | Not defined | -Earth Exploration Satellite -Radio astronomy -Space Research |  |
|  | 95-111.8 | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio Atronomy -Radiolocation -Radionavigation -Radionavigation-satellite -Space research |  |
|  | 111.8-114.25 | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio astronomy -Space Research Inter satellite |  |

Table 4.2.3.2-2 below summarizes the survey on India regulatory requirements in the frequency band above 52.6GHz in India [YY].

Table 4.2.3.4-2: India – Regulatory Requirements for Available and Potential Spectrum between 52.6 GHz and 114.25 GHz

| Frequency band (GHz) |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 61-61.5 | 100 mW EIRP |  |  | Non-Specific Short Range Device, Note | EN 305 550 |

Note: non-specific short range device means radio device, regardless of the application or the purpose, which fulfill the technical conditions as specified for a given frequency band and used for telemetry, telecomm, alarms, data transmissions in general and other applications.

4.2.3.5	Taiwan

4.2.3.6	Singapore

Figure 4.2.3.6-1 shows the spectrum allocations for Singapore for frequency between 52.6GHz and 114.25GHz based on [XX]. More specifically, ISM means industrial, scientific and medical applications.

Figure 4.2.3.6-1: Singapore spectrum allocation between 52.6GHz and 114.25GHz

In Singapore, IMDA (Info-communications Media Development Authority) shows that 57-66 GHz is considered for Wireless LAN and broadband access application [27]. On the other hand, short range radar systems, such as automatic cruise control and collision warning systems for vehicle, are applied from76 to 77GHz. Moreover, details of their regulatory requirements can be found in Annex. Furthermore, it is notable that both spectrum band, 71 – 76 GHz paired with 81 – 86 GH, will be available for short-term commercial use in fixed-wireless backhaul and inter-building fixed-wireless network.

Table 4.2.3.6-1: Regulatory requirements for available spectrum between 52.6 GHz and 114.25 GHz in Singapore

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 76 – 77 Note1 | Maximum EIRP： 23.5 dBm when stationary Maximum EIRP： 37 dBm when vehicle is in motion | No requirement | Not specified | Short range radar systems such as automatic cruise control and collision warning systems for vehicle | A short range radar system is a movement and position detection device which is used to give a warning of collision by identifying the delay between a transmitted pulse and a return pulse. Standards for Measurement Methods FCC Part 95 Subpart M; or EN 301 091-1 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 57 – 66 Note1 | Maximum EIRP： 10 W Indoor use is restricted to maximum mean EIRP density of 13 dBm/MHz Outdoor use is restricted to max e.i.r.p. of 25 dBm and max e.i.r.p. power spectral density of -2 dBm/MHz | No requirement | Not specified | Wireless LAN and broadband access | Standards for Measurement Methods EN 302 567 EN 305 550 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 71-76, 81-86 |  |  |  | fixed wireless system | See Note 2 for specific information |
| Note1: The development of spectrum allocations for short range devices is an ongoing policy review process. IMDA will revise and update its frequency allocations for short range devices as new technologies emerge. |  |  |  |  |  |
| Note2: IMDA will make available 71 – 76 GHz paired with 81 – 86 GHz of the E-band for commercial wireless applications such as fixed-wireless backhaul and inter-building fixed-wireless network. The spectrum will be available for short-term commercial use of up to 12 months until 31 December 2019. IMDA will monitor market and technology developments of the E-band, and take into consideration the World Radiocommunication Conference 2019 recommendations before allocating the spectrum in this band beyond 2020. |  |  |  |  |  |

4.2.3.7	Australia

Table 4.2.3.1-1 shows the spectrum allocations for Australia for frequency between 52.6GHz and 114.25GHz based on [XX]. Primary services are listed in “capitals:”, e.g., MOBILE, and secondary services are listed in “normal characters;” e.g. Amateur.

Table 4.2.3.7-1: Australia spectrum allocation between 52.6GHz and 114.25GHz

| Frequency [GHz] | Allocations | Notes and ITU Radio Regulations Footnotes |
| --- | --- | --- |
| 52.6-54.25 | EARTH EXPLORATION–SATELLITE (passive) SPACE RESEARCH (passive) RADIO ASTRONOMY | 5.340 |
| 54.25-55.78 | EARTH EXPLORATION–SATELLITE (passive) INTER–SATELLITE SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE |
| 55.78-56.9 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.557A for Fixed, 5.556A for INTER-SATELLITE, 5.558 for Mobile |
| 56.9-57 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.558A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 57-58.2 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.547 |
| 58.2-59 | EARTH EXPLORATION–SATELLITE (passive) FIXED MOBILE SPACE RESEARCH (passive) RADIO ASTRONOMY | 5.547 |
| 59-59.3 | EARTH EXPLORATION–SATELLITE (passive) FIXED INTER–SATELLITE MOBILE RADIOLOCATION SPACE RESEARCH (passive) | 5.556A for INTER-SATELLITE, 5.558 for Mobile, 5.559 for RADIOLOCATION |
| 59.3-64 | FIXED MOBILE INTER-SATELLITE RADIOLOCATION | 5.558 for MOBILE, 5.559 for INTER-SATELLITE, |
| 64-65 | FIXED MOBILE except aeronautical mobile INTER-SATELLITE RADIO ASTRONOMY | 5.547, 5.556 |
| 65-66 | EARTH EXPLORATION-SATELLITE FIXED INTER-SATELLITE MOBILE except aeronautical mobile SPACE RESEARCH | 5.547 |
| 66-71 | INTER-SATELLITE MOBILE MOBILE-SATELLITE RADIONAVIGATION RADIONAVIGATION-SATELLITE | 5.553 for MOBILE, 5.558 for MOBILE |
| 71-74 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE MOBILE-SATELLITE (space-to-Earth) |  |
| 74-76 | FIXED FIXED-SATELLITE (space-to-Earth) MOBILE BROADCASTING BROADCASTING-SATELLITE Space research (space-to-Earth) |  |
| 76-77.5 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) | 5.149 |
| 77.5-78 | AMATEUR AMEUR-SATELLITE RADIOLOCATION Radio astronomy Space research (space-to-Earth) | 5.559B for RADIOLOCATION |
| 78-79 | RADIOLOCATION Amateur Amateur-satellite Radio astronomy Space research (space-to-Earth) |  |
| 79-81 | RADIO ASTRONOMY RADIOLOCATION Amateur Amateur-satellite Space research (space-to-Earth) |  |
| 81-84 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE MOBILE-SATELLITE (Earth-to-space) RADIO ASTRONOMY Space research (space-to-Earth) | 5.338A for FIXED |
| 84-86 | FIXED FIXED-SATELLITE (Earth-to-space) MOBILE RADIO ASTRONOMY | 5.338A for FIXED, 5.561B for FIXED-SATELLITE (Earth-to-space) |
| 86-92 | EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) |  |
| 92-94 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION | 5.338A for FIXED |
| 94-94.1 | EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) Radio astronomy |  |
| 94.1-95 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION |  |
| 95-100 | FIXED MOBILE RADIO ASTRONOMY RADIOLOCATION RADIONAVIGATION RADIONAVIGATION-SATELLITE |  |
| 100-102 | SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY |  |
| 102-105 | FIXED MOBILE RADIO ASTRONOMY |  |
| 105-109.5 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.562B for SPACE RESEARCH (passive) |
| 109.5-111.8 | RADIO ASTRONOMY SPACE RESEARCH (passive) EARTH EXPLORATION-SATELLITE (passive) |  |
| 111.8-114.25 | FIXED MOBILE RADIO ASTRONOMY SPACE RESEARCH (passive) | 5.562B for SPACE RESEARCH (passive) |

ACMA (Australian Communications and Media Authority) publishes sensors using radar which can be applied in some different bands from 52.6GHz to 85GHz in Australia [28]. Moreover, details of their regulatory requirements can be found in Table 4.2.3.7-1.

Table 4.2.3.7-2: Regulatory requirements for available spectrum between 52.6 GHz and 114.25 GHz in Australia

| Frequency band (GHz) | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 61-61.5 | Maximum EIRP：100 mW | No requirement | Not specified | Transmitters for non-specific applications |  |
| 59–63 | Maximum EIRP：150 W The maximum transmitter power must not exceed 20 mW. | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must not be operated on board an aircraft. The transmitter must only be used outdoors. The transmitter must not cause spurious emissions outside the band at or greater than –30 dBm/MHz. |
| 57–66 | Maximum EIRP：20 W | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must comply with FCC Rules Title 47 Part 15 Section 255. |
| 60-61 | Maximum EIRP：20 mW | No requirement | Not specified | Radio determination: sensors using radar for measurement |  |
| 75-85 | Maximum EIRP：75 nW | No requirement | Not specified | Same as 60-61GHz | The maximum EIRP applies outside the shielded room enclosure. The transmitter must meet the requirements of ETSI Standard EN 302 372. |
| 76-77 | Maximum EIRP：25 W | No requirement | Not specified | Same as 60-61GHz |  |
| 77-81 | See Note1 below | No requirement | Not specified | Same as 60-61GHz | The transmitter must meet the requirements of ETSI Standard EN 302 264. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| 57-64, 75-85 | See Note2 below | See Note3 below | Not specified | Same as 60-61GHz | The transmitter must be operated in a position such that emissions are directed towards: (i) the ground; or (ii) the floor or a wall of a building or similar structure. The transmitter must comply with ETSI Standard EN 302 729. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| Note1: Section 4.3.3.3 in ETSI EN 302 264 V2.1.1 The peak power for EUT with fixed beam or scanning antenna shall not be greater than 55 dBm. |  |  |  |  |  |
| Note2: Section 4.3.4.3 in ETSI EN 302 729 V2.1.1 57-64 GHz: Maximum peak power limit EIRP is 35dBm, measured in 50MHz within main beam. 75-85 GHz Maximum peak power limit EIRP is 34dBm, measured in 50MHz within main beam. |  |  |  |  |  |
| Note3: Section 4.7 in ETSI EN 302 729 V2.1.1 |  |  |  |  |  |

=================== End of Text Proposal ========================

Reference

- RP-182861, “Revised SID: Study on NR beyond 52.6GHz” Intel, RAN#82, Sorrento, Italy, December 10-13, 2018

- RP-182802, “Draft TP to TR 38.807- Operational requirements: spectrum availability and regulatory requirements” Intel, RAN#82, Sorrento, Italy, December 10-13, 2018

- China, “People’s republic of China Regulations on the Radio Frequency Allocation (English Version)”, The People's Posts and Telecommunications Press, 2010

- The Wireless Planning & Coordination (WPC) Wing of the Ministry of Communication & Information Technology, “License Exemption for SRD Device GSR 1047(E) dated 18/10/ 2018”, http://wpc.dot.gov.in/content/10_1_Regulations.aspx

- IMDA (Infocomm Media Development Authority), “Singapore Spectrum Allocation Chart”, https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/frameworks-and-policies/spectrum-management-and-coordination/spectrumchart.pdf?la=en

- ACMA, “Australian Radiofrequency Spectrum Plan 2017-including general information”, https://www.acma.gov.au/theacma/australian-radiofrequency-spectrum-plan-spectrum-planning-acma