3GPP TSG RAN meeting #83										RP-190225

Shenzhen, China, March 18-21, 2019

Status Report to TSG

Agenda item:			9.3.4

| WI / SI Name | Study on NR V2X |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_V2X |  |  |  |  |
| Unique ID | 800096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182491 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2019 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 100 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | SEO, Hanbyul |
|  | Company | LG Electronics |
|  | Email | hanbyul.seo@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

282 contributions were submitted to RAN1#AH1901. 287 contributions were submitted to RAN1#96.

2.1.1	Agreements

RAN1#AH1901

RAN1 endorsed the template for evaluation results in R1-1900021.

For sidelink design, RAN1 made the following agreements:

- Conclusion on CP length

  - No extended CP is supported for 30 kHz in FR1 in Rel-16

  - No extended CP is supported for 120 kHz in FR2 in Rel-16

- Agreements on sidelink BWP

  - Confirm the working assumption

    - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE

  - Configuration for SL BWP is separated from Uu BWP configuration signalling.

    - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time.

      - FFS the time scale

      - FFS relation to DL BWP including initial Uu BWP

      - FFS relation in terms of frequency location and bandwidth

- Agreements on resource pool

  - For time domain resources of a resource pool for PSSCH,

    - Support the case where the resource pool consists of non-contiguous time resources

      - FFS details including granularity

  - For frequency domain resources of a resource pool for PSSCH,

    - Down select following options:

      - Option 1: The resource pool always consists of contiguous PRBs

      - Option 2: The resource pool can consist of non-contiguous PRBs

- Agreements on DM-RS patterns

  - Multiple DMRS patterns in time domain are supported for PSSCH

    - FFS: Whether a DMRS pattern is selected based on the subcarrier spacing

    - FFS: Single or multiple DMRS pattern(s) per a resource pool

    - FFS: How TX UE and RX UE can be aligned in terms of the DMRS pattern used for PSSCH

    - FFS: RE mapping, sequence generation

  - Continue to study DMRS pattern in frequency domain for PSSCH

    - E.g. Whether multiple patterns are supported, whether PDSCH/PUSCH DMRS configuration 1 or 2 is reused.

- Agreements on PT-RS

  - Support PT-RS for PSSCH for FR2

- Conclusion on discovery

  - RAN1 to conclude on the need of physical channel for discovery in RAN1#96.

- Agreements on SCI contents

  - Layer-1 destination ID can be explicitly included in SCI

    - FFS how to determine Layer-1 destination ID

    - FFS size of Layer-1 destination ID

  - The following additional information can be included in SCI

    - Layer-1 source ID

    - FFS how to determine Layer-1 source ID

    - FFS size of Layer-1 source ID

    - HARQ process ID

    - NDI

    - RV

  - FFS whether some of the above information may not be present etc. in some operations (e.g., depending on whether they are used for unicast, groupcast, broadcast)

- Agreements on sidelink HARQ timeline

  - For determining the resource of PSFCH containing HARQ feedback, support that the time gap between PSSCH and the associated PSFCH is not signaled via PSCCH at least for modes 2(a)(c)(d) (if respectively supported)

    - FFS whether or not to additionally support other mechanism(s) for modes 2(a)(c)(d)

    - FFS for mode 1

- Working assumption on sideilnk HARQ in groupcast

  - When HARQ feedback is enabled for groupcast, support (options as identified in RAN1#95):

    - Option 1: Receiver UE transmits only HARQ NACK

    - Option 2: Receiver UE transmits HARQ ACK/NACK

  - FFS applicability of option 1 and option 2 – this part is particulary relevant to confirm (or not) the working assumption

- Agreements on sidelink HARQ in mode 1

  - It is supported that in mode 1 for unicast, the in-coverage UE sends an indication to gNB to indicate the need for retransmission

    - At least PUCCH is used to report the information

      - If feasible, RAN1 reuses PUCCH defined in Rel-15

    - The gNB can also schedule re-transmission resource

    - FFS transmitter UE and/or receiver UE

      - If receiver UE, the indication is in the form of HARQ ACK/NAK

      - If transmitter UE, FFS

- Agreements on sidelink HARQ

  - (Pre-)configuration indicates whether SL HARQ feedback is enabled or disabled in unicast and/or groupcast.

    - When (pre-)configuration enables SL HARQ feedback, FFS whether SL HARQ feedback is always used or there is additional condition of actually using SL HARQ feedback

- Agreements on sidelink power control

  - SL open-loop power control is supported.

    - For unicast, groupcast, broadcast, it is supported that the open-loop power control is based on the pathloss between TX UE and gNB (if TX UE is in-coverage).

      - This is at least to mitigate interference to UL reception at gNB.

      - Rel-14 LTE sidelink open-loop power control is the baseline.

      - gNB should be able to enable/disable this power control.

    - At least for unicast, it is supported that the open-loop power control is also based on the pathloss between TX UE and RX UE.

      - (Pre-)configuration should be able to enable/disable this power control.

      - FFS whether this is applicable to groupcast

      - FFS whether this requires information signaling in the sidelink.

    - Further study its potential impact, e.g., on resource allocation.

  - FFS whether closed-loop power control is additionally needed

- Agreements on sidelink measurement

  - Long-term measurement of sidelink signal is supported at least for unicast.

    - Long-term measurement here means a measurement with L3 filtering.

    - This measurement is used at least for the open-loop power control.

      - FFS for other purpose

    - FFS: measurement metric

    - FFS: which signal is used

    - FFS: whether feedback of this measurement is needed

    - FFS whether this is applicable to groupcast

- Agreements on sidelink S-SSB

  - For NR SLSS, as the baseline:

    - The sequence type for S-PSS is the same type as the M-sequence used for NR-PSS

    - The sequence type for S-SSS is the same type as the Gold sequence for NR-SSS

  - The frequency location for S-SSB is (pre-) configured

    - Note: it implies that there is no intended hypothses detection in frequency location of S-SSB performed by the UE for a carrier in a given band

    - Note: the potential frequency locations for the (pre-)configured frequency location may be restricted, up to RAN4

- Agreements on potential combinations for further evaluation

  - Combination 1:

    - Time domain: 2 symbol of length-127 S-PSS, 2 symbol of length-127 S-SSS

    - Frequency domain:11 or 12 RBs

    - BW containing S-SSB:

      - 2.5 MHz for 15 kHz SCS

      - 5 MHz for 30 kHz SCS

      - 10 MHz for 60 kHz SCS

      - 20 MHz for 120 kHz SCS

  - Combination 2:

    - Time domain: 2 symbol of length-127 S-PSS, 2 symbol of  length-127 S-SSS

    - Frequency domain:  20 RBs

    - BW containing S-SSB:

      - 5 MHz for 15 kHz SCS

      - 10 MHz for 30 kHz SCS

      - 20 MHz for 60 kHz SCS

      - 40 MHz for 120 kHz SCS

  - Combination 3:

    - Time domain: 1 symbol of length-127 S-PSS, 1 symbol of  length-127 S-SSS

    - Frequency domain:  20 RBs

    - BW containing S-SSB:

      - 5 MHz for 15 kHz SCS

      - 10 MHz for 30 kHz SCS

      - 20 MHz for 60 kHz SCS

      - 40 MHz for 120 kHz SCS

  - Combination 4:

    - Time domain: 1 symbol of length-255 S-PSS, 1 symbol of  length-255 S-SSS

    - Frequency domain:  24 RBs

    - BW containing S-SSB:

      - 5 MHz for 15 kHz SCS

      - 10 MHz for 30 kHz SCS

      - 20 MHz for 60 kHz SCS

      - 40 MHz for 120 kHz SCS

  - Other combinations are not precluded.

  - Note:  Company should specify the assumptions, such as total energy per SSB, when the performance results are compared between different combinations.

- Agreements on design target of S-PSS and S-SSS

  - Design Target for NR S-PSS/S-SSS:

    - At least for 15 kHz SCS NR S-PSS/S-SSS, same or better coverage to that of LTE under the same Tx/Rx configuration

      - Coverage:  measured in terms of coupling loss

        - MCL = P_Tx – (NF + N_floor + SINR)

          - N_floor = -174 +10 log (BW)

          - P_Tx = 23 dBm

          - NF = 9 dB

          - SINR = -6 dB for LTE as the reference

      - Companies to report detailed assumptions e.g. the detection method/probability/etc. for the target SINR

- Agreements on S-PSS and S-SSS sequence length

  - For the evaluation at next meeting, sequence length of  S-PSS/S-SSS for all evaluated SCS is assumed the same as that of S-PSS/S-SSS with 15 kHz SCS

    - Other sequence lengths are not precluded

- Agreements on synchronization procedure

  - At least for single-carrier operation:

    - For the SL synchronization procedure, each type of synchronization reference has a respective sync priority

      - FFS the priority between eNB and gNB (if necessary)

    - For the SL synchronization procedure, among the available references, a UE selects the synchronization reference with the highest priority as the reference to derive its transmission timing

      - FFS other potential usage

      - FFS how to handle the case when there are two or more references of a same priority to be selected as the highest priority

- Agreements on sensing and resource selection

  - Mode-2 supports the sensing and resource (re)-selection procedures according to the previously agreed definitions.

    - FFS resource granularity for sensing & resource (re)-selection, e.g., PRB(s), slots, resource patterns (when applicable), etc.

    - FFS detailed conditions when these procedures can apply

  - SCI decoding applied during sensing procedure provides at least information on sidelink resources indicated by the UE transmitting the SCI

- Agreements on resource pattern

  - For the purpose of performance evaluation for Mode-2(c), the following Mode-2(c) transmission pattern selection is used when a UE is configured with a pool of patterns:

    - Sensing based pattern selection (e.g. UE selects unused pattern based on sensing results)

      - Additional information to assist pattern selection is not precluded, e.g., by using UE geographical location information

- Agreements on sub-channel

  - Sub-channel based resource allocation is supported for PSSCH

    - FFS details for sub-channels

    - FFS other use cases for sub-channel (e.g., measurement, interaction with PSCCH, etc.)

- Agreements on resource pool configuration by another UE

  - At least for the purpose of evaluation, in Mode-2(d), at least for group operation, a member UE transmits on resources configured by another UE (S-UE) within the same group

    - High layer signaling is assumed between S-UE and a member UE

RAN1 endorsed text proposals in R1-1901463 to include mmWave channel measurement results in TR 38.885.

For evaluation of Uu for advanced V2X use cases, RAN1 made the following agreements:

- Conclusion on type 2 configured grant

  - For type 2 configured grant, discuss signalling details and whether single DCI can activate/release multiple configured grants during WI (if included in the WI).

- Agreements on the number of UL transmissions

  - A UE is not expected to transmit simultaneously according to more than one UL grant (e.g., dynamic or CG) in a given BWP.

For Uu-based sidelink resource allocation/configuration, RAN1 made the following agreements:

- Agreements on NR sidelink scheduling by NR Uu

  - When NR Uu schedules NR SL mode 1, both type 1 and type 2 configured grants are supported for NR SL

- Agreements on NR sidelink scheduling by LTE Uu

  - LTE Uu to schedule NR sidelink mode 1 is supported:

    - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

    - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

  - An LS was sent to RAN2 (R1-1901445)

For QoS management, RAN1 made the following agreements:

- Agreements on congestion control

  - Introduce at least one congestion metric for NR sidelink

    - FFS details – to be done in WI phase (if included)

  - Congestion control is supported at least for sidelink mode 2

    - Note: details of congestion control can be covered in the work item phase, not in this SI.

For coexistence, RAN1 made the following agreements:

- Agreements on TDM solutions

  - For TDM solutions for in-device coexistence between LTE and NR V2X:

    - Time Alignment

      - Subframe boundary alignment is required between LTE and NR V2X sidelinks

      - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers

- Agreements on long term time scale TDM solutions

  - For long term time scale TDM solutions for in-device coexistence between LTE and NR V2X:

    - For a UE with coexistence impact, non-overlapping (in time domain) resource pools are (pre-)configured for NR V2X and LTE V2X sidelinks

      - No information is exchanged between LTE and NR sidelinks within the UE

  - Long term time scale TDM solution is feasible from RAN1 point of view

    - Note: although feasible, it is expected that such a solution may have impact on latency, reliability and data rate requirements for some applications

    - No additional modifications to LTE specifications are needed

- Agreements on short term time scale TDM solutions

  - Assuming SPS scheduling (mode -3 or mode-4) for LTE V2X, for short time scale TDM solutions for in-device coexistence for V2X,

    - For each occurrence of Tx/Tx overlap, one RAT is prioritized over another

      - This requires some information exchange between LTE and NR sidelinks within the UE

      - FFS: whether the information exchange between LTE and NR sidelinks can support this requirement

      - FFS: if there is impact to RAN1 LTE specification with this agreement

      - FFS: whether this solution can be up to UE implementation

    - For each occurrence of Tx/Rx overlap, one RAT is prioritized over another

      - This requires some information exchange between LTE and NR sidelinks within the UE

      - FFS: if there is impact to RAN1 LTE specification with this agreement

      - FFS: whether this solution can be up to UE implementation

      - FFS: If determination of priority for Rx operation is feasible and whether the information exchange between LTE and NR sidelinks can support this requirement

- Agreements on FDM solutions

  - Inter-band FDM Solutions for coexistence

    - For static power assignment of Pc,max for each carrier

      - Synchronization is not assumed for inter-band coexistence of NR sidelink and LTE sidelink.

      - This FDM solution is feasible for resolution of Tx/Tx coexistence conflicts

      - If the band separation is large enough (based on RAN4 indication), then this FDM solution for coexistence is feasible for Tx/Rx coexistence

      - If the band separation is NOT large enough, then this FDM solution is not feasible for resolution of Tx/Rx coexistence conflicts

    - For dynamic power sharing between carriers,

      - FFS details of FDM solutions and whether they are feasible

RAN1#96

RAN1 endorsed TR 38.885 v1.0.1 (R1-1902864) and v1.0.2 (R1-1903593), respectively.

For sidelink design, RAN1 made the following agreements:

- Agreements on the waveform

  - Rel-16 NR sidelink supports CP-OFDM only.

- Agreements on CP length

  - For PSCCH/PSSCH in FR2, NR V2X supports normal CP for 60 kHz, 120 kHz, and extended CP for 60 kHz.

  - Note: it is understood that PSFCH follows the same CP as PSCCH/PSSCH

- Agreements on the slot structure

  - For the operation regarding PSSCH, a UE performs either transmission or reception in a slot on a carrier.

  - NR sidelink supports for a UE:

    - A case where all the symbols in a slot are available for sidelink.

    - Another case where only a subset of consecutive symbols in a slot is available for sidelink

      - Note: this case is not intended to be used for the ITS spectra, if there is no forward-compatibility issue. Finalize in the WI phase whether there is such an issue or not

      - The subset is NOT dynamically indicated to the UE

      - FFS the supported slot configuration(s)

      - FFS whether/how to operate it in partial coverage scenarios

- Agreements on PSFCH format

  - At least for sidelink HARQ feedback, NR sidelink supports at least a PSFCH format which uses last symbol(s) available for sidelink in a slot.

- Conclusion on discovery

  - RAN1 concludes that no additional physical channel needs to be defined for the purpose of discovery in Rel-16.

- Agreements on sidelink HARQ

  - (Pre-)configuration indicates the time gap between PSFCH and the associated PSSCH for Mode 1 and Mode 2.

  - In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE.

    - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc.

  - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

  - For sidelink groupcast, it is supported to use TX-RX distance and/or RSRP in deciding whether to send HARQ feedback.

    - Details to be discussed during WI phase, including whether the information on TX-RX distance is explicitly signaled or implicitly derived, whether/how this operation is related to resource allocation, accuracy of distance and/or RSRP, the aspects related to “and/or”, etc.

    - This feature can be disabled/enabled

- Agreements on sidelink power control

  - For unicast RX UEs, SL-RSRP is reported to TX UE

    - For sidelink open loop power control for unicast for the TX UE, TX UE derives pathloss estimation

    - Revisit during the WI phase w.r.t. whether or not there is a need regarding how to handle pathloss estimation for OLPC before SL-RSRP is available for a RX UE

  - TPC commands for SL PC are not supported

- Working assumption on sidelink CSI

  - For unicast, the following CSI reporting is supported based on non-subband-based aperiodic CSI reporting mechanism assuming no more than 4-port:

    - CQI

    - RI

    - PMI

  - CSI reporting can be enabled and disabled by configuration.

    - It is supported to configure a subset of the above metric for CSI reporting.

  - There is no standalone RS transmission dedicated to CSI reporting in Rel-16

  - NR sidelink CSI strives to reuse the CSI framework for NR Uu.

    - Discuss details during WI phase

- Agreements and conclusion on sidelnk beam management

  - RAN1 concludes the following regarding beam management:

    - Beam management is beneficial

    - RAN1 has conducted limited study on the beam management.

    - In FR1, it is feasible to support V2X use cases without beam management.

    - In FR2, it is feasible to support some V2X use cases without beam management in some scenarios.

      - Panel selection is necessary to improve the communication range in FR2.

  - There is no consensus in supporting beam management for normative work for NR V2X in Rel-16.

- Agreements on synchronization procedure

  - Whether GNSS-based synchronization or gNB/eNB-based synchronization is used is (pre)-configured.

    - The following table is a working assumption

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| - P0: GNSS - P1: the following UE has the same priority: - UE directly synchronized to GNSS - P2: the following UE has the same priority: - UE indirectly synchronized to GNSS - P3: the remaining UEs have the lowest priority. | - P0: gNB/eNB - P1’: UE directly synchronized to gNB/eNB - P2’: UE indirectly synchronized to gNB/eNB - P3’: GNSS - P4’: UE directly synchronized to GNSS - P5’: UE indirectly synchronized to GNSS - P6’: the remaining UEs have the lowest priority. |

- Agreements on using sidelink RS for synchronization

  - NR V2X supports using a sidelink RS for synchronization purpose

    - Applicable only on unlicensed (ITS) carrier with no network deployment on this carrier

    - This RS is not a standalone RS and not part of SLSS.

    - This RS will not appear in the synchronization procedure for the selection of sync sources.

    - RS used for the synchronization purpose would not impact any sidelink RS design

    - FFS:  Whether this RS is DM RS or other RS

    - FFS: Whether this could be achieved by UE implementation

    - FFS: Specification impact

- Agreements on retransmission in sidelink

  - Blind retransmissions of a TB are supported for SL by NR-V2X

    - Details are for the WI phase

- Agreements on sidelink resource allocation mode 2

  - NR V2X Mode-2 supports reservation of sidelink resources at least for blind retransmission of a TB

    - Whether reservation is supported for initial transmission of a TB is to be discussed in the WI phase

    - Whether reservation is supported for potential retransmissions based on HARQ feedback is for the WI phase

  - Mode-2 sensing procedure utilizes the following sidelink measurement

    - L1 SL-RSRP based on sidelink DMRS when the corresponding SCI is decoded

      - FFS whether/which measurement is used if the corresponding SCI is not decoded e.g. SL-RSRP after blind DMRS detection, SL-RSSI

  - In the context of Mode-2(d), NR V2X supports the following functionality:

    - A UE informs gNB about group members and gNB provides individual resource pool configuration and/or individual resource configuration through the same UE to each group member UE within the same group. It does not require connection between member UE and gNB

      - The UE cannot modify the configuration provided by gNB

      - Higher layer signaling is to be used to provide the configuration. No physical layer signaling is used

    - FFS if one or both options are supported (i.e. resource pool configuration(s) or resource configuration)

    - FFS which functionality defined as a part of Mode-2 is applicable for this feature

    - This functionality is up to UE capability(ies)

For evaluation of Uu for advanced V2X use cases, RAN1 made the following agreements:

- Conclusion on type 1 configured grant

  - For type 1 configured grant, discuss signalling details during WI (if included in the WI).

- Agreements on UE reporting to gNB

  - UE reports of assistance information to the gNB include, at least: traffic periodicity, timing offset, and message size for Uu V2X and sidelink V2X traffic (at least for periodic traffic).

- Conclusions on TR contents

  - Capture in TR 38.885 that NR Uu broadcast/multicast are beneficial at least in terms of resource utilization for V2X use cases in some scenarios.

  - To include for TR conclusion section:

    - In terms of supporting advanced V2X uses cases over the LTE Uu interface, some possible enhancements were studied but none are recommended.

  - Add a reference in TR 38.885 stating that the remote driving use case is handled in TR 38.824.

For Uu-based sidelink resource allocation/configuration, RAN1 made the following agreements:

- Agreements on LTE sidelink scheduling via NR Uu

  - Scheduling by gNB using RRC for LTE sidelink scheduled mode is supported from RAN1 perspective under the premise that there is sufficient time for coordination between the NR and LTE modules. No DCI to activate/release

    - RRC message delivers the SPS grant configuration and releases the SPS configuration.

    - Support of this scheduling mode is subject to UE capability (may or may not have capability for both LTE & NR)

    - Note: some specification LTE change is needed to support the reception of a grant through RRC

      - RRC message contains mode 3 grant content and timing

      - Up to the Editor to capture it as mode 3 or new LTE sidelink mode

    - No intention to have additional NR & LTE specification change (other than those described above) for this function in Rel-16

  - RAN1 studied the feasibility of SPS scheduling by gNB for LTE sidelink with DCI activation/release, but there is no consensus to support it

For QoS management, RAN1 made the following agreements:

- Conclusion on reporting sidelink congestion metric

  - It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB

    - Consequently, it is recommended to specify the corresponding details in the WI phase

For coexistence, RAN1 made the following agreements:

- Agreements TDM solutions

  - From RAN1 point of view, short term TDM solutions for NR and LTE V2X in-device coexistence is considered to be feasible for a UE when the load for the UE from LTE side and from NR side is at or below an acceptable level

  - For each occurrence of Tx/Tx overlap and of Tx/Rx  overlap, one RAT is prioritized over another

    - High-level principles of prioritization (e.g., BSM is deemed to have a higher priority, etc.) of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation

- Agreements on FDM solutions

  - From RAN1 point of view, for both intra-band and inter-band Tx/Tx FDM solutions for in-device coexistence are considered to be feasible, at least if the following conditions are met:

    - For the intra-band case for dynamic power sharing, NR and LTE transmissions are fully overlapped in the time domain, i.e., NR transmissions have to span the entire LTE TTI such that the total power across the transmissions is constant.

  - For intra-band and inter-band FDM dynamic power sharing solutions, the following additional conditions apply:

    - Subframe boundary alignment is required between LTE and NR V2X sidelinks

    - Both LTE and NR V2X sidelinks are aware of the time resource index (e.g., DFN for LTE) in both carriers

  - For purposes of dynamic power sharing between LTE and NR Tx,

    - High-level principles of prioritization (e.g., BSM is deemed to have a higher priority, etc.) of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation

- Agreements on simultaneous reception

  - Rx/Rx coexistence are feasible for intra- & inter-band from RAN1 point of view

    - High-level principles of Rx/Rx coexistence of LTE/NR can be discussed during the WI phase, while it is expected that detailed solutions may be left for implementation

- Agreements on the feasibility of in-device coexistence

  - Based on the study from physical layer specification perspective, in-device coexistence of LTE and NR sidelink is feasible for intra- & inter-band under the respective conditions & solutions for TX/TX, TX/RX, & RX/RX

    - In the TR, also provides a reference to the respective sections

RAN1 endorsed text proposals in R1-1903591 to update mmWave measurement results in TR 38.885.

RAN1 endorsed text proposals in R1-1903773 for LTE Uu evaluation and enhancement in TR 38.885

RAN1 endorsed text proposals in R1-1903791 for the conclusion of TR 38.885.

RAN1 endorsed TR 38.885 v2.0.0 via email discussion.

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#105

For sidelink design, RAN2 made the following agreements:

- Agreements on V2X unicast

  - PC5-RRC is used to exchange UE capability and AS-layer configuration at least.

  - PC5-RRC based UE capability transfer procedure is triggered during or after PC5-S signalling for direct link setup. Further details can be discussed in WI stage.

  - PC5-RRC based UE capability transfer can be done in either one-way or two-way manner. Further details can be discussed in WI stage.

  - Further details on which UE to send out its own capability information can be discussed in WI stage.

  - PC5-RRC based AS-layer configuration procedure is triggered during or after PC5-S signalling for direct link setup. Further details can be discussed in WI stage.

  - PC5-RRC based AS-layer configuration can be done in a two-way manner. Further details can be discussed in WI stage.

  - Further details on which UE to send out PC5-RRC based AS-layer configuration can be discussed in WI stage.

- Agreements on AS Level Link Management for unicast

  - SL RLM / RLF declaration based AS level link management is supported.

  - The definition and motivation of SL RRM based AS level link management need further discussion.

  - RAN2 asked to RAN1 for RLM RS design and if ok to follow Uu RLM model for SL RLM. RAN2 indicated RAN2 point of view, Uu RLM model is preferred as baseline for SL RLM with the description how Uu RLM works.

    - An LS was sent to RAN1 (R2-1902504)

  - The AS level link status (e.g., failure) should be informed to upper layer. The detailed information exchanged between layers should be decided together with SA2.

  - If SL RLC AM is supported for unicast, RLF declaration could be triggered by indication from RLC that the maximum number of retransmissions has been reached.

  - Note:

    - RRM

      - Noted: If SL RRM measurement is supported, the SL RRM measurement results are reported to gNB.

      - Noted: If SL RRM measurement results report from receiver UE to transmitter UE is supported, the SL RRM measurement results are transmitted to gNB by transmitter UE.

    - AS level recovery

      - Noted: As a baseline, after AS level link failure is declared, the SL AS level link is kept and recovered by AS layer.

- Agreements on groupcast

  - No need of 1:m PC5 RRC connection establishment and RLM/RLF declaration among group members for groupcast. Need of RRC signaling in groupcast manner is to be discussed in WI phase.

  - No any groupcast-specific RLM design which is different from the unicast-specific RLM procedures to be considered, from RAN2 point of view.

  - Any UEs configured to receive a group destination Layer 2 ID shall be allowed to receive the groupcast transmission, in regardless of whether it is within or out of the “minimum communication range”.

  - Handling of “minimum communication range” in AS layer control of QoS for unicast/groupcast (if needed) is to be discussed in WI phase.

  - RLC UM mode is used for groupcast. RLC AM mode for groupcast is not supported.

- RAN2 asked SA2 to take above information into account, and provide feedback on below questions:

  - Whether per-flow QoS model or per-packet QoS model is finally selected for NR SL unicast, groupcast and broadcast respectively;

  - Whether/How PC5 QoS parameters of V2X packets can be provided to the AS by the upper layers.

  - An LS was sent to SA2 (R2-1902494)

For Resource allocation/configuration, RAN2 made the following agreements:

- Agreements on NR Sidelink Mode 1

  - Separate SR resources and configurations are supported for UL and SL in NR Sidelink Mode 1.

  - Multiple SR resources and configurations are supported for different SL logical channels in NR Sidelink Mode 1.

  - At least Destination information, LCG information and Buffer Size are included in Sidelink BSR MAC Control Element for NR Sidelink Mode 1.

- Agreements on providing resource pool

  - The network can provide a pool of resources in which UE autonomously selects sidelink grant for ‘sidelink unicast/groupcast/broadcast’ via broadcast system information.

  - The network can provide a pool of resources in which UE autonomously selects sidelink grant for ‘sidelink unicast/groupcast/broadcast’ via dedicated signaling.

  - RAN2 supports mode-2 resource configuration for a given validity area where the UE(s) does not need to acquire a new mode-2 resource configuration while moving in the validity area, as least when this configuration is provided by SIB (e.g. to reuse valid area of NR SIB).

- General agreement

  - Confirm that UE may be configured to perform both network controlled sidelink transmission and UE autonomous sidelink transmission.

For RAT/Interface selection, RAN2 made the following agreements:

- Agreements on RAT/interface selection

  - The access stratum is not provided with a mapping between V2X services and related radio interfaces.

  - Irrespective of the UE coverage status and RRC status, the UE access stratum signals to UE upper layers the Uu/PC5 availability information, and UE upper layer selects the radio interface.

  - Agrees on the need of the criteria for UE access stratum to determine the availability/unavailability of Uu interface for V2X communication.

  - The UE in-coverage/out-of-coverage status is used as baseline to determine the availability/unavailability of the Uu radio interface. Need of others may be discussed in WI.

  - We will specify the criteria but we will not specify exactly when the Uu availability/unavailability is signaled from UE access stratum to UE upper layer.

  - Agrees there is no need to specify what UE access stratum should signal to UE upper layer related to Uu interface availability/unavailability.

  - The need to specify the criteria for UE access stratum to determine the availability/unavailability of PC5 interface may be discussed in WI.

  - RAN2 requests to SA2 to take the above agreements into account for upper layers design and provide feedback if there is any concern.

    - An LS was went to SA2 (R2-1902505)

  - RAN2 informed SA2 that the concurrent use of multiple PC5 RATs in the same area is feasible from RAN2 perspective. In addition, RAN2 indicated that RAN2 will further discuss whether/what assistance information needs to be provided to the upper layer for RAT selection.

    - An LS was sent to SA2 (R2-1902498)

For QoS management, RAN2 made the following agreements:

- Agreement on QoS metrics

  - From the AS perspective, data rate requirements need to be further supported for NR SL, besides QoS metrics (i.e. priority, latency and reliability) as well as minimum required communication range concluded by RAN1.

- Agreements on QoS parameters

  - From RAN2 perspective, PQI defined by SA2 for NR SL is feasible. Final decision on whether/how other QoS parameters are defined in addition to PQI is up to SA2.

  - For NR SL unicast, groupcast and broadcast, specific PC5 QoS parameters (e.g. PQI, etc) of V2X packets need to be instructed by the upper layers to the AS.

- Agreements on SLRB configuration and QoS model

  - From RAN2 perspective, per-flow QoS model is preferred for NR SL unicast.

  - The mapping between PC5 QoS flows and SLRBs is at least gNB/ng-eNB configured or pre-configured. RAN2 to further decide in which case(s) gNB/ng-eNB configuration and pre-configuration are applied respectively in WI.

  - Adopt the procedures in Option b and e (corresponding to Option 2 and 5 in Appendix respectively) for NR SL unicast.

    - Option b: per-flow QoS with NW-configured SLRB

    - Option e: per-flow QoS with pre-configured SLRB

  - For V2X transmission in SL gouprcast or SL broadcast, SLRB configurations are NW configured or pre-configured. The configuration of each SLRB may include only transmission related parameters which do not need to be known by the peer UEs.

  - RAN2 agrees that from RAN2 perspective, per-packet QoS model is preferred for NR SL broadcast. Also RAN2 prefers to apply per-packet QoS based model for SL groupcast.

  - For per-packet QoS model, the mapping between PC5 QoS profiles (i.e. specific PC5 QoS parameters) and SLRBs is gNB/ng-eNB configured or pre-configured.

    - Adopt the procedures in Option a, c and d (corresponding to Option 1, 3 and 4 in Appendix respectively) for NR SL broadcast. RAN2 to further decide in which case(s) gNB/ng-eNB configuration and pre-configuration are applied respectively in WI.

    - Option a: per-packet QoS with NW-configured SLRB

    - Option c: per-packet QoS with NW-configured SLRB

- Agreements on SLRB configuration

  - For V2X transmission in SL unicast, SLRB configurations are NW configured or pre-configured. The configuration of each SLRB may include transmission related parameters which do not need to be known by the peer UE, plus some parameters that are configured also need to be known by the peer UE.

  - For NR SL unicast, some SLRB configurations need to be informed by the one UE to the peer UE in SL, including at least SN length, RLC mode (related to also Q9) and PC5 QoS profile associated with each SLRB. Other SLRB related parameters are not excluded.

- Agreement on supporting SDAP layer

  - SDAP layer is needed at least for NR SL unicast, performing PC5 QoS flow to SLRB mapping. SDAP layer is not needed for per-packet QoS model, e.g. broadcast.

- Agreement on supporting RLC AM

  - RLC AM is supported for NR SL unicast.

- Agreement on admission control

  - Need of admission control in NR SL can be discussed in WI.

RAN2 informed RAN1 of RAN2 conclusions on NR V2X SI:

- From RAN2 perspective, it is feasible to introduce NR sidelink to support advanced V2X services and feasible to support advanced V2X services with necessary enhancements to NR Uu;

- RAN2 considers to move further details in NR V2X SI to potential NR V2X WI, pending RAN plenary final decision;

- All RAN2 related studies have been finished (by taking into account all completed RAN2 work), and from RAN2 perspective, NR V2X SI is considered complete.

- An LS was sent to RAN1 (R2-1902507)

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

RAN3#103

The following agreements were achieved for NR V2X Service Authorization:

- The V2X service authorization information is PC5 RAT specific.

- The cross-RAT PC5 control authorization information (implicit or explicit) is necessary in NG-RAN.

The following agreement was achieved for UE Sidelink AMBR in NG-RAN:

- The UE sidelink AMBR is PC5 RAT specific.

The following agreement was achieved on the impacts of F1 signalling:

- For NR V2X Mode 1 (dynamic case), gNB-DU is responsible for the scheduling of sidelink resource. gNB-DU transmits the V2X SL configuration information to the gNB-CU, which uses it to generate the RRC message containing mode 1 resource configuration and forwards it to the UE.

The following agreement was achieved on resource coordination:

- Resource pool coordination between NG-RAN nodes (e.g., ng-eNB and gNB) is beneficial.

RAN3 endorsed the text proposal in R3-191071 on network aspects for TR38.885.

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #90

The following agreements were achieved for NR V2X UE RF parameters and transient period to send reply LS to RAN 1.

AGC settling time

- AGC settling time includes the following components (AGC setting time =a+b+c+d)

  - (a) time offset from the start of the OFDM symbol to account for maximum signal propagation delay from different UEs

  - (b) time to receive the window of samples used for estimating the gain/power

  - (c) time for estimating and programming the desired gain setting

  - (d) time for the gain setting taking into effect

- AGC settling time will be derived under assumption of below two options of RBs size restrictions:

  - Option1

    - At least 10RBs for 15kHz

    - At least 5 RBs for 30kHz

    - At least 3 RBs for 60kHz

  - Option2

    - At least 10RBs for all SCSs

- AGC settling time considering the different waveform design for AGC symbols

    - At least, CP-OFDM will be considered

Enhanced IBE model for NR V2X UE

  - RAN4 will further discuss on the enhanced IBE model for NR V2X UE.

  - Interested companies are encouraged to provide the preferred IBE model for NR V2X UE at next RAN4 meeting.

  - Based on the analyzed results, RAN4 can send LS to inform the IBE model at FR1.

NR V2X UE transient period

  - Q1: Whether transient period is needed (if so, how much) if power spectral density is different between the last symbol containing PSCCH and the following symbol

  - A1: Yes, in case of RB configuration or total transmit power change, transient period is needed, which could result in phase discontinuity for PSSCH. Basically, RAN4 define transient period as shown in section 6.3.3 in TS38.101-1

    - between transmit OFF power and transmit ON power symbols (transmit ON/OFF)

    - between continuous ON-power transmissions with power change or RB hopping is applied.

  - Q2: Whether transient period is needed (if so, how much) if PSCCH and PSSCH use different frequency location and/or resource size in option 1B and 3

  - A2:

    - Option 1B could result in a different RB configuration (location and/or Size), power spectral density for control than that for data. This could result in a different MPR and AMPR for control and data. Hence, this option needs a transient period.

    - Option 3: This option will not need any transient period. In RAN4, MPR and AMPR should be derived for the entire slot and a single value of MPR/AMPR should be used for the entire slot.

  - Q3: Whether there is any impact on the above questions due to different assumptions of amplifiers (e.g., for MIMO purpose, etc.)

  - A3:

    - In the case where PSCCH is rank-1 and transmitted from one antenna only, while PSSCH is transmitted from both antennas, the transient period will be needed. It will result in phase discontinuity for PSSCH.

    - In the case where PSCCH and PSSCH are transmitted from same the antennas connectors, the answers to the above questions are not impacted.

  - Q4: Whether there are any other cases that require transient period (if so, how much)

  - A4: RAN4 can provide additional analysis result once sidelink physical design becomes clearer.

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA2

3.1.0	SA2 eV2XARC status – general

FS_eV2XARC (Study on architecture enhancements for 3GPP support of advanced V2X services) has been 95% complete. One key issue, Key Issue #15 (Enhancements to assist Application Adjustment) is expected to be concluded in SA2#132 (April 2019). TR 23.786 is presented for Approval to SA#83 (S2-1902909).

Since new WID on Architecture enhancements for 3GPP support of advanced V2X services (eV2XARC) was approved in SA#82 (SP-181121), SA2 progressed normative work in Q1 2019, mainly with a new TS 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services". eV2XARC is 30% complete.

3.1.1	Agreements with cross-TSG impacts

In Q1 2019, the following agreements were made as conclusions under FS_eV2XARC.

-	For Key Issue #3 (QoS Support for eV2X over Uu interface) on QoS Notification Control enhancements,

it is concluded that Solution #27 in clause 6.27 is taken as the baseline for normative work.

NOTE: As RAN WGs had been requested to evaluate the RAN impacts, Solution #27 may be updated and aligned with RAN WG feedbacks in the normative phase.

-	For Key Issue #12 (System migration and interworking for eV2X),

it is concluded to take Solution #20 in clause 6.20 also for normative work. PC5 interworking can be supported with proper UE configurations by taking into account e.g. regional regulations, deployments, and UE PC5 RAT capability.

-	For Key Issue #14 (Support of broadcast over NG-Uu),

it is concluded to stop further solution work on 5GS MBMS for Release 16. No normative work shall be performed for this key issue in Release 16.

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

Regarding some issue that has RAN dependency described above in clause 3.1.1, SA2 sent the following LS to RAN WGs.

S2-1902904: LS on QoS Support for eV2X over Uu interface (To: RAN2, RAN3 / Cc: RAN1) in order to get feedback on the following questions from RAN2 and RAN3:

-	Solution# 27 in clause 6.27 of TR 23.786v1.2.0 has been selected for normative work pending RAN WGs feedback, regarding Key Issue #3 (QoS Support for eV2X over Uu interface) on QoS monitoring, control and notification. SA2 would like to ask RAN2 and RAN3 to provide feedback on the feasibility and potential enhancements, especially steps 3 to 5 of clause 6.27.1.

※ It is considered that discussion in Plenary#83 is not needed for the above issue. However, it would be good to encourage RAN WGs to provide feedback to SA2 in time.

3.2	SA3

3.2.0	SA3 eV2X status – general

FS_eV2X_Sec (Study on Security Aspects of 3GPP support for Advanced V2X Services) was approved in SA3#94 (January 2019) - S3-190462. The study has been started in SA3#94-Ad hoc (March 2019).

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#AH1901

- R1-1900021	V2X evaluations template	Huawei, HiSilicon

- R1-1900022	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1900023	Sidelink physical layer procedures for NR V2X	Huawei, HiSilicon

- R1-1900024	Discussion on reference signal for sidelink control and data channel design	Huawei, HiSilicon

- R1-1900025	Sidelink synchronization mechanisms for NR V2X	Huawei, HiSilicon

- R1-1900026	Sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-1900027	NR Uu and LTE Uu support and enhancement for advanced V2X use cases	Huawei, HiSilicon

- R1-1900028	Discussion on Uu-based sidelink resource allocation/configuration	Huawei, HiSilicon

- R1-1900029	QoS management for NR V2X	Huawei, HiSilicon

- R1-1900030	Discussion on co-existence for NR V2X and LTE V2X	Huawei, HiSilicon

- R1-1900084	Reliable Groupcast HARQ feedback scheme for NR V2X	Kyocera Corporation

- R1-1900085	Support for Reservation in Mode 2a and 2c	Kyocera Corporation

- R1-1900117	Physical layer structure for NR sidelink	vivo

- R1-1900118	Physical layer procedure for NR sidelink	vivo

- R1-1900119	NR sidelink synchronization mechanism	vivo

- R1-1900120	Discussion on sidelink resource allocation mechanism	vivo

- R1-1900121	System level evaluations on sidelink for NR V2X	vivo

- R1-1900122	Enhancements of  Uu link to control  sidelink	vivo

- R1-1900123	On coexistence of NR and LTE sidelink for V2X	vivo

- R1-1900196	On sidelink physical layer structure	MediaTek Inc.

- R1-1900197	Discussion on physical layer procedure	MediaTek Inc.

- R1-1900198	Discussion on sidelink based synchronization mechanism	MediaTek Inc.

- R1-1900199	On sidelink resource allocation mechanism	MediaTek Inc.

- R1-1900200	NR Uu support for advanced V2X use cases	MediaTek Inc.

- R1-1900201	Uu-based SL resource allocation and configuration	MediaTek Inc.

- R1-1900202	Coexistence mechanisms for NR-V2X and LTE-V2X	MediaTek Inc.

- R1-1900247	Discussion on physical layer structure for NR sidelink	Fujitsu

- R1-1900248	Discussion on HARQ-ACK feedback for NR-V2X	Fujitsu

- R1-1900249	Geographic Information based Dynamic TFRP Resource Selection Procedure in NR-V2X	Fujitsu

- R1-1900250	Discussion on basic resource allocation methods  for NR-V2X sidelink communication	Fujitsu

- R1-1900251	Further considerations on NR-V2X resource selection procedure of mode 2(a) in mixture scenario	Fujitsu

- R1-1900252	Further considerations on Mode 1 resource allocation for NR-V2X	Fujitsu

- R1-1900253	Dynamic Sidelink Bi-mode Transmission in NR-V2X	Fujitsu

- R1-1900254	Discussion on QoS-related Fragmentation Metric in NR-V2X	Fujitsu

- R1-1900289	Discussion on mode 2 resource allocation in NR-V2X	OPPO

- R1-1900290	Discussion on NR Uu enhancements for NR-V2X	OPPO

- R1-1900291	Discussion on QoS management in NR-V2X	OPPO

- R1-1900292	Discussion on coexistence in NR-V2X	OPPO

- R1-1900293	Discussion on dual-connectivity in NR-V2X	OPPO

- R1-1900300	Physical layer structure for NR-V2X	OPPO

- R1-1900301	Physical layer procedure for NR-V2X	OPPO

- R1-1900302	Discussion of synchronization mechanism for NR-V2X	OPPO

- R1-1900303	Discussion of Uu-based sidelink resource allocation and configuration	OPPO

- R1-1900320	Discussion on physical layer structure in NR V2X	CATT

- R1-1900321	Discussion on physical layer procedures in NR V2X	CATT

- R1-1900322	Design and evaluation of S-SSB for NR V2X Sidelink	CATT

- R1-1900323	Discussion on resource allocation mechanism in NR V2X	CATT

- R1-1900324	Further considerations on sidelink unicast and groupcast in NR V2X	CATT

- R1-1900325	Further considerations on Uu enhancement in NR V2X	CATT

- R1-1900326	Discussion On Uu-based sidelink resource allocation/configuration in NR V2X	CATT

- R1-1900327	QoS management in NR V2X	CATT

- R1-1900328	Discussion on coexistence of LTE sidelink and NR sidelink in NR V2X	CATT

- R1-1900354	Analysis and Design of V2X Resource Entities	Fraunhofer HHI, Fraunhofer IIS

- R1-1900355	Physical Layer Procedures for Unicast and Groupcast	Fraunhofer HHI, Fraunhofer IIS

- R1-1900356	Designs for NR V2X Mode 2 Resource Allocation	Fraunhofer HHI, Fraunhofer IIS

- R1-1900357	NR V2X Uu-based Resource Allocation	Fraunhofer HHI, Fraunhofer IIS

- R1-1900358	QoS Management for NR V2X	Fraunhofer IIS, Fraunhofer HHI

- R1-1900368	Discussion on HARQ feedback for NR V2X communication	sony

- R1-1900369	Discussion on NR V2X resource allocation mechanism	sony

- R1-1900389	Physical layer structures in NR V2X	Lenovo, Motorola Mobility

- R1-1900390	Physical layer procedures in NR V2X	Lenovo, Motorola Mobility

- R1-1900391	Resource allocation mechanism in NR V2X	Lenovo, Motorola Mobility

- R1-1900392	Discussion on Uu-based sidelink resource allocation	Lenovo, Motorola Mobility

- R1-1900405	Discussion on HARQ feedback	CMCC

- R1-1900406	Discussion on resource allocation mechanism for NR V2X	CMCC

- R1-1900407	Discussion on Uu-based sidelink resource allocation/configuration	CMCC

- R1-1900442	Physical layer structure for NR sidelink design	AT&T

- R1-1900443	Physical layer procedures for NR sidelink design	AT&T

- R1-1900444	Synchronization mechanism for NR sidelink	AT&T

- R1-1900445	Resource allocation mechanism for NR sidelink	AT&T

- R1-1900446	Field trial results from 39GHz vehicle to vehicle communication	AT&T

- R1-1900447	Enhancements of NR Uu to control sidelink	AT&T

- R1-1900460	Physical procedures for Sidelink feedback transmission	ITRI

- R1-1900461	Discussion for Sidelink Synchronization Issues and Mechanisms  in NR V2X	ITRI

- R1-1900462	Discussion on Sidelink Resource Allocation for NR V2X	ITRI

- R1-1900463	Considerations on Design issues for NR V2X unicast, groupcast ,and broadcast	ITRI

- R1-1900464	NR Uu Enhancement for Sidelink Resource Management	ITRI

- R1-1900465	Considerations on resource sharing among different QoS levels	ITRI

- R1-1900480	Sidelink physical structure design for NR V2X communication	Intel Corporation

- R1-1900481	Sidelink physical layer procedures for NR V2X communication	Intel Corporation

- R1-1900482	Sidelink synchronization for NR V2X communication	Intel Corporation

- R1-1900483	Sidelink resource allocation schemes for NR V2X communication	Intel Corporation

- R1-1900484	Sidelink resource allocation aspects for NR V2X unicast and groupcast communication	Intel Corporation

- R1-1900485	Analysis of NR Uu interface to support advanced V2X use cases	Intel Corporation

- R1-1900486	Sidelink control by NR and LTE Uu interfaces for V2X use cases	Intel Corporation

- R1-1900487	Sidelink QoS management and congestion control for NR V2X communication	Intel Corporation

- R1-1900488	In-device coexistence mechanisms for NR V2X communication	Intel Corporation

- R1-1900538	On physical layer structures for NR V2X	Samsung

- R1-1900539	On physical layer procedures for NR V2X	Samsung

- R1-1900540	On synchronization mechanisms for NR V2X	Samsung

- R1-1900541	On resource allocation mechanisms for NR V2X	Samsung

- R1-1900542	On AGC settling issues with evaluation results for NR V2X	Samsung

- R1-1900543	Considerations on sidelink HARQ procedure	Samsung

- R1-1900544	Considerarions on Sidelink CSI	Samsung

- R1-1900545	On low-latency sidelink resouce allocation and scheduling	Samsung

- R1-1900546	Considerations on sidelink RLM	Samsung

- R1-1900547	On DMRS design for NR V2X	Samsung

- R1-1900548	System level evaluations for NR V2X sidelink resource allocation	Samsung

- R1-1900549	Considerations on channel structures in sensing slots for NR V2X sidelink	Samsung

- R1-1900550	On Uu for advanced V2X use cases	Samsung

- R1-1900551	On Uu-based sidelink resource allocation and configuration	Samsung

- R1-1900552	On QoS management for NR V2X	Samsung

- R1-1900553	On Coexistence between LTE and NR PC5	Samsung

- R1-1900643	Discussion on PSFCH channel design in NR V2X	Panasonic

- R1-1900644	Discussion on resource allocation mechanism in NR V2X	Panasonic

- R1-1900645	Discussion on sidelink resource allocation in mode 1 for NR V2X	Panasonic

- R1-1900646	Discussion on QoS management for NR V2X	Panasonic

- R1-1900647	Discussion on physical layer structure for NR V2X	LG Electronics

- R1-1900648	Discussion on physical layer procedure for NR V2X	LG Electronics

- R1-1900649	Discussion on synchronization mechanism for NR V2X	LG Electronics

- R1-1900650	Discussion on resource allocation mechanism for NR V2X	LG Electronics

- R1-1900651	Evaluation of NR Uu for advanced V2X use cases	LG Electronics

- R1-1900652	Uu-based sidelink resource allocation/configuration for NR V2X	LG Electronics

- R1-1900653	Discussion on QoS management for NR V2X	LG Electronics

- R1-1900654	Discussion on coexistence mechanism for NR V2X	LG Electronics

- R1-1900660	Views on resource allocation mechanism for NR sidelink	Hisense

- R1-1900667	Discussion on coexistence between NR and LTE sidelinks	NEC

- R1-1900668	Physical layer structure for NR V2X	NEC

- R1-1900669	Physical layer procedures for NR V2X	NEC

- R1-1900670	Synchronization mechanism for NR V2X	NEC

- R1-1900671	Resource allocation mechanism for NR V2X	NEC

- R1-1900684	Discussion on NR V2X Sidelink Design	ETRI

- R1-1900685	Discussion on NR V2X sidelink synchronization	ETRI

- R1-1900695	Considerations on sidelink synchronization for NR V2X	Sequans Communications

- R1-1900698	Discussion on PSCCH and PSSCH multiplexing	Panasonic Corporation

- R1-1900704	Considerations on sidelink QoS management	Spreadtrum Communications

- R1-1900705	Discussion on resource allocation for mode 2	Spreadtrum Communications

- R1-1900712	Discussion on synchronization mechanisms for NR V2X	Spreadtrum Communications

- R1-1900713	Discussion on NR sidelink physical layer structure	Spreadtrum Communications

- R1-1900714	Discussion on NR sidelink physical layer procedure	Spreadtrum Communications

- R1-1900715	Consideration on NR Uu-based sidelink resource allocation	Spreadtrum Communications

- R1-1900724	Discussion on physical layer structure for NR sidelink	Sony

- R1-1900738	Minimum set of KPI requirements for ProSe D2D Communications	FirstNet

- R1-1900742	Considerations on NR V2X physical layer procedure	Apple Inc.

- R1-1900743	Considerations on NR V2X Synchronization mechanism	Apple Inc.

- R1-1900744	Considerations on NR V2X Resource allocation mechanism	Apple Inc.

- R1-1900745	Considerations on NR V2X scheduling mechanism	Apple Inc.

- R1-1900769	Mode 2a and Mode 2d for NR V2X Resource Allocation	InterDigital, Inc.

- R1-1900770	Discussion on Range Requirement and CBR/CR in NR V2X	InterDigital, Inc.

- R1-1900774	Discussion on NR sidelink physical layer structure	ZTE, Sanechips

- R1-1900775	Discussion on PHY procedures for NR V2X	ZTE, Sanechips

- R1-1900776	Discussion on synchronization mechanism in NR V2X	ZTE, Sanechips

- R1-1900777	Analysis of Mode 2 resource schemes on sidelink	ZTE, Sanechips

- R1-1900778	Discussion on Uu based resource allocation/configuration for NR V2X	ZTE, Sanechips

- R1-1900779	Discussion on QoS management	ZTE, Sanechips

- R1-1900780	Coexistence between NR V2X and LTE V2X	ZTE, Sanechips

- R1-1900782	Sidelink resource allocation for NR V2X with mode 2(d) in mobile operator bands and bands over 6 GHz	TOYOTA Info Technology Center

- R1-1900793	Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1900794	Physical Layer Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1900795	On Synchronization Mechanism and Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1900796	Preliminary Link Level Evaluations on Reference Signal for NR V2X	InterDigital, Inc.

- R1-1900797	Link Level Evaluations on Waveform for NR V2X	InterDigital, Inc.

- R1-1900798	Preliminary System Level Evaluations on NR V2X Sidelink Unicast	InterDigital, Inc.

- R1-1900799	Preliminary Performance Evaluations on NR V2X Sidelink Synchronization	InterDigital, Inc.

- R1-1900800	Discussion on NR Uu for Advanced V2X Use Cases	InterDigital, Inc.

- R1-1900801	On Uu-based Sidelink Resource Allocation and Configuration	InterDigital, Inc.

- R1-1900802	Discussion on Coexistence of LTE and NR V2X Sidelink	InterDigital, Inc.

- R1-1900831	Discussion on HARQ-ACK and CSI procedure in NR V2X	Sharp

- R1-1900832	Considerations on synchronization design for NR V2X	Sharp

- R1-1900851	System level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1900852	Link level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1900853	LTE Uu support for advanced V2X use cases.	Huawei, HiSilicon

- R1-1900854	Discussion on NR Uu to control LTE sidelink	Huawei, HiSilicon

- R1-1900855	Sidelink discovery for NR V2X	Huawei, HiSilicon

- R1-1900856	Frame and slot structure for sidelink	Huawei, HiSilicon

- R1-1900857	Sidelink CSI	Huawei, HiSilicon

- R1-1900858	Design and contents of PSCCH and PSFCH	Huawei, HiSilicon

- R1-1900859	BWP operation for V2X sidelink	Huawei, HiSilicon

- R1-1900860	Discussion on LTE Uu to control NR sidelink	Huawei, HiSilicon, Vodafone

- R1-1900861	Power control and power sharing for V2X sidelink	Huawei, HiSilicon

- R1-1900862	Beamforming for V2X sidelink for FR1 and FR2	Huawei, HiSilicon

- R1-1900884	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1900885	Physical layer procedures for HARQ operation for groupcast and unicast transmissions	Qualcomm Incorporated

- R1-1900886	Synchronization design for NR V2X	Qualcomm Incorporated

- R1-1900887	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated

- R1-1900888	Uu for advanced V2X use cases	Qualcomm Incorporated

- R1-1900889	Enhancements of LTE Uu and NR Uu to control NR sidelink	Qualcomm Incorporated

- R1-1900890	Design aspects and requirements for QoS	Qualcomm Incorporated

- R1-1900891	Co-existence aspects for NR-V2X and LTE-V2X	Qualcomm Incorporated

- R1-1900962	Sidelink physical layer structure for NR V2X	NTT DOCOMO, INC.

- R1-1900963	Sidelink physical layer procedure for NR V2X	NTT DOCOMO, INC.

- R1-1900964	Sidelink synchronization mechanism for NR V2X	NTT DOCOMO, INC.

- R1-1900965	Sidelink resource allocation mechanism for NR V2X	NTT DOCOMO, INC.

- R1-1900966	Uu-based NR sidelink resource allocation/configuration for NR V2X	NTT DOCOMO, INC.

- R1-1900967	Sidelink QoS management for NR V2X	NTT DOCOMO, INC.

- R1-1900968	Views on LTE and NR sidelink coexistence	NTT DOCOMO, INC.

- R1-1900994	About Uu enhancements for advanced V2X use cases	ZTE, Sanechips

- R1-1901005	Consideration on Mode 2 NR-V2X sidelink communication	ASUSTEK COMPUTER (SHANGHAI)

- R1-1901014	Discussion on sidelink physical layer procedure on NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1901016	Discussion on physical layer structure for NR V2X	Xiaomi Communications

- R1-1901017	On Physical layer procedures for V2x communications	Xiaomi Communications

- R1-1901018	On Synchronization for NR V2X	Xiaomi Communications

- R1-1901019	On resource allocation of V2x communications	Xiaomi Communications

- R1-1901047	On physical layer structures for NR V2X	Samsung

- R1-1901048	On physical layer procedures for NR V2X	Samsung

- R1-1901049	On synchronization mechanisms for NR V2X	Samsung

- R1-1901050	On resource allocation mechanisms for NR V2X	Samsung

- R1-1901051	On AGC settling issues with evaluation results for NR V2X	Samsung

- R1-1901052	Considerations on sidelink HARQ procedure	Samsung

- R1-1901053	Considerarions on Sidelink CSI	Samsung

- R1-1901054	On low-latency sidelink resouce allocation and scheduling	Samsung

- R1-1901055	Considerations on sidelink RLM	Samsung

- R1-1901056	On DMRS design for NR V2X	Samsung

- R1-1901057	System level evaluations for NR V2X sidelink resource allocation	Samsung

- R1-1901058	Considerations on channel structures in sensing slots for NR V2X sidelink	Samsung

- R1-1901059	On Uu for advanced V2X use cases	Samsung

- R1-1901060	On Uu-based sidelink resource allocation and configuration	Samsung

- R1-1901061	On QoS management for NR V2X	Samsung

- R1-1901062	On Coexistence between LTE and NR PC5	Samsung

- R1-1901108	Views on resource allocation mechanism for NR sidelink	Hisense

- R1-1901112	Discussion on PSFCH in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1901127	Considerations on Pre-emption Issue in RA of NR V2X Sidelink Design	CAICT

- R1-1901128	Considerations on Scrambling Issue for NR V2X Unicast Sidelink	CAICT

- R1-1901143	Considerations on sidelink synchronization for NR V2X	ITL

- R1-1901144	Physical layer design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1901145	Physical layer structure for NR V2X	ITL

- R1-1901146	Discussion on NR V2X HARQ mechanism	ITL

- R1-1901147	Waveform evaluations for V2X sidelink	Mitsubishi Electric RCE

- R1-1901155	Discussions on NR V2X Sidelink Physical Layer Structures	Nokia, Nokia Shanghai Bell

- R1-1901156	On Sidelink physical layer procedures	Nokia, Nokia Shanghai Bell

- R1-1901157	Discussion on NR V2X Sidelink Synchronization	Nokia, Nokia Shanghai Bell

- R1-1901158	On Sidelink Resource Allocation	Nokia, Nokia Shanghai Bell

- R1-1901159	Discussions on DMRS for NR sidelink	Nokia, Nokia Shanghai Bell

- R1-1901160	On Uu for advanced V2X use cases	Nokia, Nokia Shanghai Bell

- R1-1901161	On Uu-based sidelink resource allocation/configuration	Nokia, Nokia Shanghai Bell

- R1-1901162	On QoS Management	Nokia, Nokia Shanghai Bell

- R1-1901163	Discussion on coexistence between NR sidelink and LTE sidelink	Nokia, Nokia Shanghai Bell

- R1-1901178	NR Sidelink physical layer structure for V2X	HEPTA 7291

- R1-1901179	NR Sidelink physical layer procedures for V2X	HEPTA 7291

- R1-1901180	NR Sidelink resource allocation schemes for V2X	HEPTA 7291

- R1-1901193	NR Sidelink synchronization procedures for NR V2X	HEPTA 7291

- R1-1901208	Design Considerations for NR V2X Synchronization	Convida Wireless

- R1-1901209	Discussion on Unicast, Groupcast and Broadcast for NR V2X	Convida Wireless

- R1-1901210	Discussion on Resource Allocation for NR V2X	Convida Wireless

- R1-1901211	Details on physical layer structure for SL V2X	Ericsson

- R1-1901212	On PHY procedures to support unicast and groupcast on NR sidelink	Ericsson

- R1-1901213	On Synchronization for NR Sidelink	Ericsson

- R1-1901214	Resource allocation procedures for Mode 2	Ericsson

- R1-1901215	NR Uu enhancements for advanced V2X use cases	Ericsson

- R1-1901216	Uu based resource allocation and configuration for SL	Ericsson

- R1-1901217	QoS related Layer 1 procedures for NR SL	Ericsson

- R1-1901218	In-device coexistence for SL V2X	Ericsson

- R1-1901219	On SCI and SFCI contents	Ericsson

- R1-1901220	Configuration and signalling principles for NR sidelink	Ericsson

- R1-1901221	Details on CSIT acquisition for SL unicast	Ericsson

- R1-1901222	On Interface Selection	Ericsson

- R1-1901223	Link level evaluations of NR PSCCH	Ericsson

- R1-1901224	On NR sidelink resources	Ericsson

- R1-1901225	On the NR sidelink discovery	Ericsson

- R1-1901226	System level evaluations of NR V2X sidelink	Ericsson

- R1-1901227	Transmission of sidelink HARQ feedback to gNB for mode 1 scheduling	Ericsson

- R1-1901228	Link level evaluations of NR PSSCH	Ericsson

- R1-1901229	Design of patterns for sidelink transmission	Ericsson

- R1-1901230	Clustering and Cluster Head Election Schemes for In-Coverage and Out-of-Coverage UEs	Ericsson

- R1-1901231	On Mode-1 Latency	Ericsson

- R1-1901232	On link adaptation for sidelink groupcast	Ericsson

- R1-1901271	Discussion on resource allocation mechanism for NR V2X	LG Electronics

- R1-1901290	Feature Lead Summary for 7.2.4.4 QoS Management	Nokia

- R1-1901302	Minimum set of KPI requirements for ProSe D2D Communications	FirstNet

- R1-1901304	Feature lead summary on Evaluation of Uu for advanced V2X use cases	Ericsson

- R1-1901307	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated

- R1-1901316	Feature lead summary for AI 7.2.4.3: Uu-based sidelink resource allocation/configuration	Huawei

- R1-1901322	Feature lead summary for agenda item 7.2.4.1.1 Physical layer structure	LG Electronics

- R1-1901323	Feature lead summary for agenda item 7.2.4.1.2 Physical layer procedures	LG Electronics

- R1-1901326	Summary of AI 7.2.4.1.3 NR V2X synchronization	CATT

- R1-1901335	Discussion on physical layer structure for NR V2X	LG Electronics

- R1-1901340	Summary for NR-V2X AI - 7.2.4.1.4 Resource allocation mechanism	Intel Corporation

- R1-1901375	Summary of Contributions and Initial Outcome of Offline Discussion for NR-V2X AI - 7.2.4.1.4 Resource Allocation Mechanism	Intel Corporation

- R1-1901377	Offline discussion summary of AI 7.2.4.1.3 V2X SL Synchronization mechanism	CATT

- R1-1901383	Feature lead summary#2 on Evaluation of Uu for advanced V2X use cases	Ericsson

- R1-1901392	Summary of Coexistence Aspects in NR-V2X Study	Qualcomm

- R1-1901393	Feature lead summary for AI 7.2.4.3: Uu-based sidelink resource allocation/configuration	Huawei

- R1-1901410	[DRAFT] LS on Uu-based sidelink resource allocation for NR V2X	Huawei

- R1-1901411	TP for TR38.885	AT&T, Huawei, HiSilicon

- R1-1901414	Offline summary of AI 7.2.4.1.3 V2X SL Synchronization mechanism	CATT

- R1-1901425	Updated summary of Coexistence Aspects in NR-V2X Study	Qualcomm

- R1-1901438	Feature lead summary for agenda item 7.2.4.1.1 Physical layer structure	LG Electronics

- R1-1901439	Feature lead updated summary for 7.2.4.1.2 Physical layer procedures	LG Electronics

- R1-1901445	LS on Uu-based sidelink resource allocation for NR V2X	RAN1, Huawei

- R1-1901448	Offline summary#2 of AI 7.2.4.1.3 V2X SL Synchronization mechanism	CATT

- R1-1901462	Outcome of Offline Discussion #2 for NR-V2X AI - 7.2.4.1.4 Resource Allocation Mechanism	Intel Corporation

- R1-1901463	TP for TR38.885	AT&T, Huawei, HiSilicon

RAN1#96

- R1-1901536	Sidelink physical layer structure for NR V2X	Huawei, HiSilicon

- R1-1901537	Sidelink physical layer procedures for NR V2X	Huawei, HiSilicon

- R1-1901538	Discussion on reference signal for sidelink control and data channel design	Huawei, HiSilicon

- R1-1901539	Sidelink synchronization mechanisms for NR V2X	Huawei, HiSilicon

- R1-1901540	Sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-1901541	System level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1901542	Link level evaluations on sidelink for NR V2X	Huawei, HiSilicon

- R1-1901543	NR Uu and LTE Uu support and enhancement for advanced V2X use cases	Huawei, HiSilicon

- R1-1901544	Discussion on Uu-based sidelink resource allocation/configuration	Huawei, HiSilicon

- R1-1901545	QoS management for NR V2X	Huawei, HiSilicon

- R1-1901546	Discussion on co-existence for NR V2X and LTE V2X	Huawei, HiSilicon

- R1-1901682	Physical layer structure for NR sidelink	vivo

- R1-1901683	Physical layer procedure for NR sidelink	vivo

- R1-1901684	NR sidelink synchronization mechanism	vivo

- R1-1901685	Discussion on sidelink resource allocation mechanism	vivo

- R1-1901686	System level evaluations on sidelink for NR V2X	vivo

- R1-1901687	Discussion on NR Uu enhancements for NR-V2X	vivo

- R1-1901688	Enhancements of  Uu link to control  sidelink	vivo

- R1-1901689	On coexistence of NR and LTE sidelink for V2X	vivo

- R1-1901752	Performance requirements for ProSe D2D communications within the scope of NR V2X sidelink	FirstNet, AT&T, SyncTechno, Bittium, VIRVE, The Police of Netherlands, Norwegian Communications Authority, French MOI, UIC (International union of railways)

- R1-1901809	On sidelink physical layer structure	MediaTek Inc.

- R1-1901810	Discussion on Physical layer procedure	MediaTek Inc.

- R1-1901811	Discussion on sidelink based synchronization mechanism	MediaTek Inc.

- R1-1901812	On sidelink resource allocation mechanism	MediaTek Inc.

- R1-1901813	Enhancements to NR Uu for advanced V2X use cases	MediaTek Inc.

- R1-1901814	Uu based SL configuration and resource allocation	MediaTek Inc.

- R1-1901815	Coexistence mechanisms between NR-V2X and LTE-V2X	MediaTek Inc.

- R1-1901829	Discussion on QoS management for NR V2X	vivo

- R1-1901830	Views on physical layer design for NR V2X sidelink	Mitsubishi Electric RCE

- R1-1901831	Waveform evaluations for V2X sidelink	Mitsubishi Electric RCE

- R1-1901842	Design and Analysis of NR V2X Physical Layer Structure	Fraunhofer HHI, Fraunhofer IIS

- R1-1901843	Physical Layer Procedures for Unicast and Groupcast	Fraunhofer HHI, Fraunhofer IIS

- R1-1901844	Evaluation of NR V2X Mode 2 Resource Allocation	Fraunhofer HHI, Fraunhofer IIS

- R1-1901845	NR V2X Uu-based Resource Allocation	Fraunhofer HHI, Fraunhofer IIS

- R1-1901877	Discussion on QoS management	ZTE, Sanechips

- R1-1901878	Coexistence between NR V2X and LTE V2X	ZTE, Sanechips

- R1-1901879	Discussion on Uu based resource allocation/configuration for NR V2X	ZTE, Sanechips

- R1-1901880	Analysis of Mode 2 resource schemes on sidelink	ZTE, Sanechips

- R1-1901895	Physical layer structure for NR sidelink design	AT&T

- R1-1901896	Physical layer procedures for NR sidelink design	AT&T

- R1-1901897	Synchronization mechanism for NR sidelink	AT&T

- R1-1901898	Resource allocation mechanism and performance analysis	AT&T

- R1-1901899	Additional field trial results from 39GHz vehicle to vehicle communication	AT&T

- R1-1901900	Enhancements of NR Uu to control sidelink	AT&T

- R1-1901901	On congestion level indication in resource pool configuration	AT&T

- R1-1901929	Text proposal for the conclusion of TR 38.885	LG Electronics

- R1-1901930	Discussion on physical layer structure for NR V2X	LG Electronics

- R1-1901931	Discussion on physical layer procedure for NR V2X	LG Electronics

- R1-1901932	Discussion on synchronization mechanism for NR V2X	LG Electronics

- R1-1901933	Discussion on resource allocation mechanism for NR V2X	LG Electronics

- R1-1901934	Discussion on NR Uu for advanced V2X use cases	LG Electronics

- R1-1901935	Discussion on Uu-based sidelink resource allocation/configuration	LG Electronics

- R1-1901936	Discussion on QoS management for NR V2X	LG Electronics

- R1-1901937	Discussion on coexistence mechanism for NR V2X	LG Electronics

- R1-1901943	Discussion on physical layer structure for NR sidelink	Fujitsu

- R1-1901944	Discussion on HARQ-ACK feedback for NR-V2X	Fujitsu

- R1-1901945	Geographic Information based Dynamic TFRP Resource Selection Procedure in NR-V2X	Fujitsu

- R1-1901946	Discussion on basic resource allocation methods  for NR V2X sidelink communication	Fujitsu

- R1-1901947	Further considerations on NR V2X resource selection procedure of mode 2(a) in mixture scenario	Fujitsu

- R1-1901948	Further considerations on Mode 1 resource allocation for NR-V2X	Fujitsu

- R1-1901949	Dynamic Sidelink Bi-mode Transmission in NR-V2X	Fujitsu

- R1-1901992	Discussion on physical layer structure in NR V2X	CATT

- R1-1901993	Discussion on physical layer procedures in NR V2X	CATT

- R1-1901994	Further design and evaluation of S-SSB for NR V2X Sidelink	CATT

- R1-1901995	Discussion on resource allocation mechanism in NR V2X	CATT

- R1-1901996	Further considerations on Uu enhancement in NR V2X	CATT

- R1-1901997	Discussion On Uu-based sidelink resource allocation/configuration in NR V2X	CATT

- R1-1901998	QoS management in NR V2X	CATT

- R1-1901999	Discussion on coexistence of NR V2X and LTE V2X	CATT

- R1-1902104	QoS Management for NR V2X	Fraunhofer IIS, Fraunhofer HHI

- R1-1902131	Support for Reservation in Mode 2	Kyocera Corporation

- R1-1902132	A Reliable Groupcast HARQ feedback scheme for NR V2X	Kyocera Corporation

- R1-1902137	Discussion on NR sidelink physical layer structure	ZTE, Sanechips

- R1-1902138	Discussion on PHY procedures for NR V2X	ZTE, Sanechips

- R1-1902139	Discussion on synchronization mechanism in NR V2X	ZTE, Sanechips

- R1-1902140	About Uu enhancements for advanced V2X use cases	ZTE, Sanechips

- R1-1902156	Physical layer structures in NR V2X	Lenovo, Motorola Mobility

- R1-1902157	Physical layer procedures in NR V2X	Lenovo, Motorola Mobility

- R1-1902158	Resource allocation mechanism in NR V2X	Lenovo, Motorola Mobility

- R1-1902159	Discussion on Uu-based sidelink resource allocation	Lenovo, Motorola Mobility

- R1-1902160	Discussion on co-existence for NR-V2X and LTE-V2X	Lenovo, Motorola Mobility

- R1-1902174	Discussion on HARQ feedback for NR V2X communication	Sony

- R1-1902175	Discussion on NR V2X resource allocation mechanism	Sony

- R1-1902201	Discussion on physical layer structure for NR V2X	Panasonic

- R1-1902202	Discussion on PSFCH channel design in NR V2X	Panasonic

- R1-1902203	Discussion on resource allocation mechanism in NR V2X	Panasonic

- R1-1902204	Discussion on sidelink resource allocation in mode 1 for NR V2X	Panasonic

- R1-1902205	Discussion on QoS management for NR V2X	Panasonic

- R1-1902206	Discussion on coexistence between LTE sidelink and NR sidelink for NR V2X	Panasonic

- R1-1902273	On physical layer structures for NR V2X	Samsung

- R1-1902274	On physical layer procedures for NR V2X	Samsung

- R1-1902275	On synchronization mechanisms for NR V2X	Samsung

- R1-1902276	On resource allocation mechanisms for NR V2X	Samsung

- R1-1902277	On AGC settling issues with evaluation results for NR V2X	Samsung

- R1-1902278	Considerations on sidelink HARQ procedure	Samsung

- R1-1902279	Considerarions on Sidelink CSI	Samsung

- R1-1902280	On low-latency sidelink resouce allocation and scheduling	Samsung

- R1-1902281	Considerations on sidelink RLM	Samsung

- R1-1902282	On DMRS design for NR V2X	Samsung

- R1-1902283	System level evaluations for NR V2X sidelink resource allocation	Samsung

- R1-1902284	System level evaluations considering resource reservation signal and LBT	Samsung

- R1-1902285	Considerations on channel structures in sensing slots for NR V2X sidelink	Samsung

- R1-1902286	Considerations on Sidelink power control	Samsung

- R1-1902287	On sidelink feedback channel format	Samsung

- R1-1902288	On Uu for advanced V2X use cases	Samsung

- R1-1902289	On Uu-based sidelink resource allocation and configuration	Samsung

- R1-1902290	On QoS management for NR V2X	Samsung

- R1-1902291	On Coexistence between LTE and NR PC5	Samsung

- R1-1902330	Discussion on HARQ feedback for NR V2X	CMCC

- R1-1902331	Discussion on Uu-based sidelink resource allocation/configuration	CMCC

- R1-1902385	Physical layer structure for NR-V2X	OPPO

- R1-1902386	Physical layer procedure for NR-V2X	OPPO

- R1-1902387	Discussion of synchronization mechanism for NR-V2X	OPPO

- R1-1902388	Discussion of Uu-based sidelink resource allocation and configuration	OPPO

- R1-1902389	Discussion on UE autonomous RA in NR-V2X	OPPO

- R1-1902390	Further enhancements on NR Uu for NR-V2X	OPPO

- R1-1902391	Further considerations on QoS management for NR-V2X	OPPO

- R1-1902392	Further considerations on NR and LTE V2X coexistence	OPPO

- R1-1902401	Sidelink Physical Procedures for NR V2X	ITRI

- R1-1902402	Discussion for NR V2X Sidelink Synchronization Issues and Mechanisms	ITRI

- R1-1902403	Discussion on Sidelink resource management techniques for NR V2X communication	ITRI

- R1-1902404	Considerations on Design issues for NR V2X unicast, groupcast, broadcast	ITRI

- R1-1902405	Considering on NR Uu for Resource Configuration on controlling NR sidelink	ITRI

- R1-1902406	Considerations on issue for resource sharing among different QoS levels	ITRI

- R1-1902439	Discussion on NR V2X Sidelink Design	ETRI

- R1-1902440	Discussion on NR V2X sidelink synchronization	ETRI

- R1-1902445	Discussion on sidelink physical layer procedure on NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1902446	Consideration on Mode 2 NR-V2X sidelink communication	ASUSTEK COMPUTER (SHANGHAI)

- R1-1902481	Sidelink physical structure for NR V2X communication	Intel Corporation

- R1-1902482	Physical layer procedures for NR V2X sidelink design	Intel Corporation

- R1-1902483	Synchronization for NR V2X sidelink communication	Intel Corporation

- R1-1902484	Resource allocation schemes for NR V2X sidelink communication	Intel Corporation

- R1-1902485	Analysis of NR Uu interface to support advanced V2X use cases	Intel Corporation

- R1-1902486	Uu-based sidelink resource allocation for V2X use cases	Intel Corporation

- R1-1902487	QoS and congestion control for NR V2X sidelink communication	Intel Corporation

- R1-1902488	In-device coexistence for NR V2X sidelink communication	Intel Corporation

- R1-1902541	Discussion on physical layer structure for NR sidelink	Sony

- R1-1902542	System level simulation results on NR sidelink communication	Sony

- R1-1902545	On Sidelink DMRS Design for NR V2X	TCL Communication Ltd.

- R1-1902554	On Coexistence Between LTE and NR Side-links	DISH Network

- R1-1902573	Discussions on NR V2X Sidelink Physical Layer Structures	Nokia, Nokia Shanghai Bell

- R1-1902574	On Sidelink physical layer procedures	Nokia, Nokia Shanghai Bell

- R1-1902575	Discussion on NR V2X Sidelink Synchronization	Nokia, Nokia Shanghai Bell

- R1-1902576	On Sidelink Resource Allocation	Nokia, Nokia Shanghai Bell

- R1-1902577	Discussions on DMRS for NR sidelink	Nokia, Nokia Shanghai Bell

- R1-1902578	On Uu for advanced V2X use cases	Nokia, Nokia Shanghai Bell

- R1-1902579	On Uu-based sidelink resource allocation/configuration	Nokia, Nokia Shanghai Bell

- R1-1902580	On QoS Management	Nokia, Nokia Shanghai Bell

- R1-1902581	Discussion on coexistence between NR sidelink and LTE sidelink	Nokia, Nokia Shanghai Bell

- R1-1902595	Discussion on Physical Layer Structure for NR V2X Sidelink	InterDigital, Inc.

- R1-1902596	Discussion on Physical Layer Procedures for NR V2X Sidelink	InterDigital, Inc.

- R1-1902597	On Synchronization Mechanism and Procedures for NR V2X	InterDigital, Inc.

- R1-1902598	Sidelink Resource Allocation Mechanism for NR V2X	InterDigital, Inc.

- R1-1902599	On Waveform for NR V2X Sidelink	InterDigital, Inc.

- R1-1902600	Performance Evaluation of NR V2X S-SSB Structures	InterDigital, Inc.

- R1-1902601	Performance Evaluation of DMRS for NR V2X	InterDigital, Inc.

- R1-1902602	On NR Uu enhancements for advanced V2X use cases	InterDigital, Inc.

- R1-1902603	Discussion on Uu-based Sidelink Resource Allocation and Configuration	InterDigital, Inc.

- R1-1902604	Discussion on QoS Management	InterDigital, Inc.

- R1-1902605	Discussion on Coexistence Between LTE and NR Sidelinks	InterDigital, Inc.

- R1-1902639	Discussion on sidelink resource allocation	Asia Pacific Telecom co. Ltd

- R1-1902662	Discussion on physical layer procedures in NR V2X	Sharp

- R1-1902679	Remaining issues on NR Sidelink PHY structure for V2X	HEPTA 7291

- R1-1902680	Remaining issues on NR Sidelink PHY layer procedures for V2X	HEPTA 7291

- R1-1902681	Remaining issues on NR Sidelink synchronization for V2X	HEPTA 7291

- R1-1902692	Physical layer structure for NR V2X	NEC

- R1-1902693	Physical layer procedures for NR V2X	NEC

- R1-1902694	Synchronization mechanism for NR V2X	NEC

- R1-1902695	Resource allocation mechanism for NR V2X	NEC

- R1-1902696	Discussion on the coexistence between NR and LTE sidelinks	NEC

- R1-1902697	Remaining issues on NR Sidelink resource allocation for V2X	HEPTA 7291

- R1-1902710	Discussion on physical layer structure for NR V2X	Xiaomi Communications

- R1-1902711	On HARQ operation for V2x communications	Xiaomi Communications

- R1-1902712	On Synchronization for NR V2X	Xiaomi Communications

- R1-1902713	On resource allocation of V2x communications	Xiaomi Communications

- R1-1902715	Considerations on sidelink QoS management	Spreadtrum Communications

- R1-1902722	Discussion on resource allocation for Mode-2	Spreadtrum Communications

- R1-1902723	Discussion on NR V2X physical layer structure	Spreadtrum Communications

- R1-1902724	Discussion on NR V2X physical layer procedure	Spreadtrum Communications

- R1-1902725	Consideration on NR Uu-based sidelink resource allocation	Spreadtrum Communications

- R1-1902726	Discussion on synchronization mechanism for NR V2X	Spreadtrum Communications

- R1-1902760	Considerations on NR V2X physical layer procedure	Apple Inc.

- R1-1902761	Considerations on NR V2X Synchronization mechanism	Apple Inc.

- R1-1902762	Considerations on NR V2X Resource allocation mechanism	Apple Inc.

- R1-1902763	Considerations on NR V2X scheduling mechanism	Apple Inc.

- R1-1902798	Sidelink physical layer structure for NR V2X	NTT DOCOMO, INC.

- R1-1902799	Sidelink physical layer procedure for NR V2X	NTT DOCOMO, INC.

- R1-1902800	Sidelink synchronization mechanism for NR V2X	NTT DOCOMO, INC.

- R1-1902801	Sidelink resource allocation mechanism for NR V2X	NTT DOCOMO, INC.

- R1-1902802	Uu-based NR sidelink resource allocation/configuration for NR V2X	NTT DOCOMO, INC.

- R1-1902803	Updated views on LTE and NR sidelink coexistence	NTT DOCOMO, INC.

- R1-1902831	Discussion on SL resource allocation	KDDI Corporation

- R1-1902864	TR 38.885 v1.0.1 Study on Vehicle-to-Everything	Huawei Tech.(UK) Co., Ltd

- R1-1902891	Physical layer structure for NR V2X	ITL

- R1-1902892	Physical layer procedure for NR V2X	ITL

- R1-1902896	Considerations on sidelink synchronization for NR V2X	Sequans Communications

- R1-1902900	Discussion on SL resource allocation	KDDI Corporation

- R1-1902910	Considerations on sidelink synchronization mechanism for NR V2X	ITL

- R1-1902911	Discussion on PSFCH in NR V2X	ASUSTEK COMPUTER (SHANGHAI)

- R1-1902916	Discussion on Sidelink groupcast HARQ	Nokia, Nokia Shanghai Bell

- R1-1902923	Considerations on Scrambling Issue for NR V2X Unicast Sidelink	CAICT

- R1-1902929	Views on the synchronization signal for NR V2X	KT Corp.

- R1-1902994	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated

- R1-1902995	Physical layer procedures for HARQ operation for groupcast and unicast transmissions	Qualcomm Incorporated

- R1-1902996	Synchronization design for NR V2X	Qualcomm Incorporated

- R1-1902997	Sidelink resource allocation mechanism for NR V2X	Qualcomm Incorporated

- R1-1902998	Uu for advanced V2X use cases	Qualcomm Incorporated

- R1-1902999	Enhancements of LTE Uu and NR Uu to control NR sidelink	Qualcomm Incorporated

- R1-1903000	Design aspects and requirements for QoS	Qualcomm Incorporated

- R1-1903001	Co-existence aspects for NR-V2X and LTE-V2X	Qualcomm Incorporated

- R1-1903066	LTE Uu support for advanced V2X use cases.	Huawei, HiSilicon

- R1-1903067	Discussion on NR Uu to control LTE sidelink	Huawei, HiSilicon

- R1-1903068	Sidelink discovery for NR V2X	Huawei, HiSilicon

- R1-1903069	Frame and slot structure for sidelink	Huawei, HiSilicon

- R1-1903070	Sidelink CSI	Huawei, HiSilicon

- R1-1903071	Design and contents of PSCCH and PSFCH	Huawei, HiSilicon

- R1-1903072	BWP operation for V2X sidelink	Huawei, HiSilicon

- R1-1903073	Discussion on LTE Uu to control NR sidelink	Huawei, HiSilicon

- R1-1903074	Power control and power sharing for V2X sidelink	Huawei, HiSilicon

- R1-1903075	Beamforming for V2X sidelink for FR1 and FR2	Huawei, HiSilicon

- R1-1903076	Sidelink measurements	Huawei, HiSilicon

- R1-1903130	V2X antenna model and pathloss correction	Fraunhofer HHI, Fraunhofer IIS

- R1-1903153	Discussion on Sidelink Procedures	Convida Wireless

- R1-1903155	Design Considerations for NR V2X Synchronization	Convida Wireless

- R1-1903156	On Sidelink Resource Allocation	Convida Wireless

- R1-1903163	Remaining details on physical structure for NR SL	Ericsson

- R1-1903164	Remaining details on PHY procedures for Rel. 16 NR V2X	Ericsson

- R1-1903165	On Synchronization for NR Sidelink	Ericsson

- R1-1903166	Resource allocation procedures for Mode 2	Ericsson

- R1-1903167	System level evaluations of NR V2X sidelink	Ericsson

- R1-1903168	Clustering and Cluster Head Election Schemes for In-Coverage and Out-of-Coverage UEs	Ericsson

- R1-1903169	NR Uu enhancements for advanced V2X use cases	Ericsson

- R1-1903170	Remaining details on Uu based resource allocation for SL	Ericsson

- R1-1903171	Remaining RAN1 issues for NR SL QoS	Ericsson

- R1-1903172	Remaining details on in-device coexistence issues for SL V2X	Ericsson

- R1-1903173	On SCI and SFCI formats	Ericsson

- R1-1903174	Configuration details for NR V2X scenarios	Ericsson

- R1-1903175	Details on CSIT acquisition for SL unicast	Ericsson

- R1-1903176	On L1 measurements for sidelink	Ericsson

- R1-1903177	On NR sidelink resources	Ericsson

- R1-1903178	On the NR sidelink discovery	Ericsson

- R1-1903179	On HARQ support for mode-1	Ericsson

- R1-1903180	Link level evaluations of NR PSCCH	Ericsson

- R1-1903181	Link level evaluations of NR PSSCH	Ericsson

- R1-1903182	Text Proposal for TR Conclusion	Ericsson

- R1-1903183	Discussion on NR V2X Work Item scope	Ericsson

- R1-1903184	On Interface Selection	Ericsson

- R1-1903185	On link adaptation for sidelink groupcast	Ericsson

- R1-1903188	TP for TR 38.885 on Uu multicast/broadcast	Huawei, HiSilicon, Samsung

- R1-1903189	TP for TR 38.885 Conclusions	Huawei, HiSilicon

- R1-1903213	TP for TR 38.885 Section 11: Evaluations	Huawei

- R1-1903330	Feature Lead Summary for 7.2.4.4 QoS Management	Nokia

- R1-1903336	NR sidelink synchronization mechanism	vivo

- R1-1903337	Discussion on sidelink resource allocation mechanism	vivo

- R1-1903345	Sidelink resource allocation mechanism for NR V2X	Qualcomm Incorporated

- R1-1903366	Feature lead summary for agenda item 7.2.4.1.1 Physical layer structure	LG Electronics

- R1-1903367	Feature lead summary for agenda item 7.2.4.1.2 Physical layer procedures	LG Electronics

- R1-1903375	Feature lead summary for AI 7.2.4.3: Uu-based sidelink resource allocation/configuration	Huawei

- R1-1903380	Feature lead summary on Evaluation of Uu for advanced V2X use cases	Ericsson

- R1-1903384	Summary of Coexistence Aspects in NR-V2X Study	Qualcomm

- R1-1903393	Synchronization for NR V2X sidelink communication	Intel Corporation

- R1-1903412	Summary of V2X Sidelink Synchronization	CATT

- R1-1903428	Text proposal on 2 Rx Antennas for NR V2X	General Motors, Volkswagen AG

- R1-1903450	Physical layer procedures for NR V2X sidelink design	Intel Corporation

- R1-1903484	Summary of V2X Sidelink Synchronization	CATT

- R1-1903489	TP to update mmWave results in TR 38.885	AT&T

- R1-1903490	Feature lead summary#2 on Evaluation of Uu for advanced V2X use cases	Ericsson

- R1-1903493	On synchronization mechanisms for NR V2X	Samsung

- R1-1903562	V2X antenna model and pathloss correction	Fraunhofer HHI, Fraunhofer IIS, Huawei, HiSilicon

- R1-1903568	Feature lead summary#2 for AI 7.2.4.3: Uu-based sidelink resource allocation/configuration	Huawei

- R1-1903572	Feature lead summary#2 for AI 7.2.4.3: Uu-based sidelink resource allocation/configuration	Huawei

- R1-1903579	TP for TR 38.885 on Uu multicast/broadcast	Huawei, HiSilicon, Samsung

- R1-1903591	TP to update mmWave results in TR 38.885	AT&T

- R1-1903593	TR 38.885 v1.0.2 Study on Vehicle-to-Everything	Huawei Tech.(UK) Co., Ltd

- R1-1903594	Updated Summary of Coexistence Aspects in NR-V2X Study	Qualcomm

- R1-1903596	Feature lead summary#2 for agenda item 7.2.4.1.1 Physical layer structure	LG Electronics

- R1-1903597	Feature lead summary#2 for agenda item 7.2.4.1.2 Physical layer procedures	LG Electronics

- R1-1903623	Feature Lead Summary for NR-V2X AI - 7.2.4.1.4 Resource Allocation Mechanism	Intel

- R1-1903649	Feature Lead Summary #2 for 7.2.4.4 QoS Management	Nokia

- R1-1903704	Text proposal on 2 Rx Antennas for NR V2X	General Motors, Volkswagen AG

- R1-1903736	Summary of V2X Sidelink Synchronization	CATT

- R1-1903760	Discussion on synchronization mechanism for NR V2X	LG Electronics

- R1-1903769	Feature lead summary#2 for agenda item 7.2.4.1.1 Physical layer structure	LG Electronics

- R1-1903770	Feature lead summary#2 for agenda item 7.2.4.1.2 Physical layer procedures	LG Electronics

- R1-1903773	TP for TR 38.885 on LTE Uu evaluation and enhancement	Huawei, HiSilicon

- R1-1903791	Combined TP for TR 38.885 Conclusions	Huawei

- R1-1903811	Summary of V2X Sidelink Synchronization	CATT

RAN2#105

- R2-1900024	LS on Uu-based sidelink resource allocation for NR V2X, Huawei

- R2-1900049	LS on conclusion of “Study on evaluation for 2 RX exception in Rel-15 vehicle mounted UE for NR, Samsung

- R2-1900064	Reply LS on LS to SA2 on unicast, groupcast and broadcast in NR sidelink, Qualcomm

- R2-1900070	Combinations of Uu QoS characteristics values for V2X services,  Huawei

- R2-1900081	Reply LS on RAT selection for SL, LGE

- R2-1900603	[DRAFT] Response LS on QoS characteristics for NR V2X over Uu	Nokia, Nokia Shanghai Bell

- R2-1901520	Prioritization of V2X Scenarios 3 and 4 	VODAFONE Group Plc

- R2-1902034	Clarification on the support of NR-NR DC scenario in NR V2X	Huawei, HiSilicon

- R2-1902222		Way forward for NR V2X SI	Huawei, HiSilicon

- R2-1900172	Discussion on PC5-RRC of unicast for NR-V2X	OPPO

- R2-1900176	Discussion on RRM and RLM of unicast for NR-V2X	OPPO

- R2-1900177	Discussion on left issues of group-cast for NR-V2X	OPPO

- R2-1900179	Left issues on UP aspects for unicast and groupcast	OPPO

- R2-1900180	Summary of [104#55] V2X Unicast (OPPO)	OPPO

- R2-1900181	TP of PC5-RRC based procedure for unicast in NR-V2X	OPPO

- R2-1900189	RLC AM Mode for V2X Service	CATT

- R2-1900190	SDAP sublayer for NR V2XSidelink	CATT

- R2-1900192	PDCP functions for NR V2XSidelink	CATT

- R2-1900193	RRC functions for NR V2XSidelink	CATT

- R2-1900195	Connection Setup Procedures for sidelink unicast	CATT

- R2-1900379	Consideration on sidelink measurement	ZTE, Sanechips

- R2-1900384	Considerations on groupcast	ZTE, Sanechipsn

- R2-1900814	Link monitoring for groupcast	MediaTek Inc.

- R2-1900831	NR V2X Support for Sidelink unicast and groupcast	, ITRI

- R2-1900883	SL V2X user plane aspects	Intel Corporation

- R2-1900914	Discussion on RLM and RLF for NR V2X	LG Electronics France

- R2-1900915	Discussion on measurement and report for NR V2X	LG Electronics France

- R2-1900999	Discussion on AS level connection for Unicast	Lenovo, Motorola Mobility

- R2-1901002	Radio link maintenance for NR V2X unicast	Lenovo, Motorola Mobility

- R2-1901033	Discussion on Request for SLRB configuration	ASUSTeK

- R2-1901034	Discussion on scenarios for applying exceptional pool	ASUSTeK

- R2-1901036	Discussion on UL/SL TX prioritization	ASUSTeK

- R2-1901063	Link management for unicast	MediaTek Inc.

- R2-1901064	Connection control for unicast	MediaTek Inc.

- R2-1901115	Sidelink groupcast in NR	vivo

- R2-1901116	Different destination service multiplexing in MAC	vivo

- R2-1901117	Signaling procedure details for sidelink unicast connection setup	vivo

- R2-1901519	NR V2X support for Groupcast Communication	Fraunhofer HHI, Fraunhofer IIS

- R2-1901571	On the need for PC5-RRC procedure based AS Connection Establishment	Convida Wireless

- R2-1901577	TP to TR 38.885 on MultiRAT CP Aspects in V2X	InterDigital

- R2-1901579	RLM/RLF and RRM for NR V2X	InterDigital

- R2-1901580	Further Details on Link Establishment Procedure	InterDigital

- R2-1901581	RAN2 Aspects of Groupcast Design	InterDigital

- R2-1901651	Mode-1 Implications for Supporting SL HARQ feedbacks	Ericsson

- R2-1901653	On L2 IDs	Ericsson

- R2-1901654	On Latency of mode-1 Sidelink Scheduling	Ericsson

- R2-1901655	On Sidelink UE Information	Ericsson

- R2-1901658	On the Support of HARQ feedbacks Over Sidelink	Ericsson

- R2-1901659	On the Support of Sidelink CSI feedbacks	Ericsson

- R2-1901661	Discussions on NR V2X in DC Scenarios	Ericsson

- R2-1901662	Miscellaneous MAC components	Ericsson

- R2-1901702	On the sidelink connection	Ericsson

- R2-1901707	On NR sidelink link management for unicast	Ericsson

- R2-1901708	On NR sidelink unicast bearer establishment	Ericsson

- R2-1901722	On the coordination of PC5-RRC and PC5-S		Qualcomm Incorporated

- R2-1901726	[DRAFT] Reply LS to SA2 on unicast, groupcast and broadcast in NR sidelink	Qualcomm

- R2-1901727	Discussion on Groupcast for NR V2X	Qualcomm Incorporated

- R2-1901730	Summary Report of [104#60][NR/V2X] Groupcast	Qualcomm Incorporated

- R2-1901732	TP to TR 38.885 on groupcast in NR-V2X	Qualcomm Incorporated

- R2-1901782	Group leader visibility to AS layer	Samsung Electronics

- R2-1901889	Discussion on unicast, groupcast and broadcast	Fujitsu

- R2-1901900	Summary of [104#56][NR/V2X] AS Level Link Management for Unicast, Apple

- R2-1901901	TP to TR38.885 on [104#56][NR/V2X] AS Level Link Management for Unicast	Apple

- R2-1901902	Draft LS on SL RLM/RLF in NR V2X	Apple	To:RAN1

- R2-1901903	Discussion on SL RLM and RRM	Apple

- R2-1901917	Discussion on unicast connection establishment	Beijing Xiaomi Software Tech

- R2-1901953	Discussion on sidelink unicast communication	CMCC

- R2-1902035	Discussion on channel mapping for NR SL	Huawei, HiSilicon

- R2-1902036	Analysis on connection establishment procedure for SL unicast	Huawei, HiSilicon

- R2-1902037	AS-related group communication for platooning	Huawei, HiSilicon

- R2-1902038	Considerations on RRM and RLM for NR V2X unicast	Huawei, HiSilicon

- R2-1902039	Consideration on miscellaneous MAC aspects for NR SL design	Huawei, HiSilicon

- R2-1902040	Discussion about sidelink LCP procedure	Huawei, HiSilicon

- R2-1902041	Consideration on use cases for groupcast in NR V2X	Huawei, HiSilicon

- R2-1902042	[Draft] LS on NR SL groupcast use case	Huawei, HiSilicon

- R2-1902074	Sidelink HARQ operation for NR V2X	ITL

- R2-1902079	PC5-RRC connection establishment for unicast

- R2-1902113	V2X UE behaviour upon releasing a unicast/groupcast session	ITL

- R2-1902121	PC5-RRC based AS-layer configuration procedure for Unicast in NR-V2X	Samsung Electronics

- R2-1902122	PC5-RRC based UE capability trasnfer procedure for Unicast in NR-V2X	Samsung Electronics

- R2-1902123	RLM based AS level link management for unicast in NR-V2X	Samsung Electronics

- R2-1902142	Analysis on connection establishment procedure for SL unicast	Huawei, HiSilicon

- R2-1902149	[Draft] LS on SL RLM and SL RRM	Huawei, HiSilicon

- R2-1902157	Support of multiple PC5-S connections over a PC5-RRC connection	LG Electronics Inc

- R2-1902158	Unicast-assisted groupcast transmissions	LG Electronics Inc

- R2-1902198	SL RRM report for unicast	Huawei, HiSilicon

- R2-1900146	Resource allocation mode control	CATT

- R2-1900147	Mode 2(d) resource allocation	CATT

- R2-1900148	Uu based sidelink scheduling	CATT

- R2-1900171	Discussion on Inter-RAT Control for NR-V2X	OPPO

- R2-1900175	Discussion on resource allocation mode for NR-V2X	OPPO

- R2-1900186	Carrier/resource (re-)selection function	CATT

- R2-1900217	Scenarios of Mode 1 for sidelink unicast/groupcast	CATT

- R2-1900218	HARQ Procedure for Mode 1	CATT

- R2-1900377	Discussion on NR V2X mode 2 resource allocation	ZTE, Sanechips

- R2-1900380	Discussion on mode 1 resource allocation	ZTE, Sanechips

- R2-1900381	Consideration on multiple  mode NR V2X resource allocation	ZTE, Sanechips

- R2-1900604	Validity area for NR Sidelink resource allocation in V2X communications	Nokia, Nokia Shanghai Bell

- R2-1900605	NR Sidelink resource allocation for V2X communications	Nokia, Nokia Shanghai Bell

- R2-1900813	Zone-based resource allocation for NR V2X	MediaTek Inc.

- R2-1900815	UE-assisted resource allocation for NR V2X	MediaTek Inc.

- R2-1900881	On mode 1 and mode 2 simultaneous operation and switching	Intel Corporation

- R2-1900884	Sidelink resource configuration considerations for NR V2X	Intel Corporation

- R2-1900934	Resource Allocation Consideration	Spreadtrum Communications

- R2-1901035	Resource pool selection for Mode 2	ASUSTeK

- R2-1901037	Discussion on association between sidelink data and resource allocation modes	ASUSTeK

- R2-1901112	Discussion on resource allocation for NR Sidelink mode 1	vivo

- R2-1901118	Discussion on support of simultaneous mode 1 and mode 2

- R2-1901234	SL and UL BWP Numerology Mismatch	Lenovo, Motorola Mobility

- R2-1901517	On NR V2X Resource Allocation	Fraunhofer HHI, Fraunhofer IIS

- R2-1901566	Buffer Status Reporting for NR V2X Transmission Mode 2	Convida Wireless

- R2-1901576	Text Proposal to 38.885 on Resource Pool Aspects for NR V2X	InterDigital

- R2-1901578	RAN2 Aspects for Simultaneous Support of Mode 1 and Mode 2	InterDigital

- R2-1901649	Bandwidth parts and Resource Pools for NR V2X	Ericsson

- R2-1901650	gNB-scheduled Resource Allocation for Sidelink	Ericsson

- R2-1901652	On Inter-RAT Network Control of Sidelink	Ericsson

- R2-1901656	Resource allocation for Sidelink Mobility	Ericsson

- R2-1901657	Configuration of Unicast/Groupcast/Broadcast Sidelink V2X    Communications	Ericsson

- R2-1901664	Layer-2 implications of Platoon header	Ericsson

- R2-1901665	Layer-2 Implications of Mode-2d Resource Allocation	Ericsson

- R2-1901666	HARQ feedbacks for Sidelink Groupcast	Ericsson

- R2-1901667	Coexistence of Mode-1 and Mode-2	Ericsson

- R2-1901759	Discussion on NR V2X resource allocation	KT Corp

- R2-1901783	RRC-dedicated signaling based resource pool configuration in MR-DC	Samsung Electronics

- R2-1901784	sidelink SPS for NR V2X	Samsung Electronics

- R2-1901908	Discussion on resource allocation	Beijing Xiaomi Software Tech

- R2-1902043	Considerations on resource management for NR V2X	Huawei, HiSilicon

- R2-1902044	Discussion about mode existence for NR sidelink	Huawei, HiSilicon

- R2-1902045	Mobility enhancements for mode-1 and mode-2	Huawei, HiSilicon

- R2-1902046	Potential RAN2 impacts on gNB scheduled resource allocation for NR V2X	Huawei, HiSilicon

- R2-1902124	On Supporting Inter-RAT Resource Allocation and Configuration for V2X Sidelink	Samsung Electronics

- R2-1902159	Report of [104#59][NR/V2X] Resource allocation	LG Electronics Inc

- R2-1902160	TP to 38.885 on [104#59][NR/V2X] Resource allocation	LG Electronics Inc

- R2-1900178	Left issues on target scenario for NR-V2X	OPPO

- R2-1900220	MAC PDU format in PC5	CATT

- R2-1900378	Discussion on RRC connection condition for NR V2X	ZTE, Sanechips

- R2-1900882	Enhancements to UE assistance and UE sidelink information	Intel Corporation

- R2-1901038	AS security on PC5-RRC messages	ASUSTeK

- R2-1901111	NR V2X scenario for PDCP duplication supporting	vivo

- R2-1901152	Considerations for sidelink resource pool design for NR V2X	Samsung

- R2-1901668	Discussion on NR V2X Work Item scope	Ericsson

- R2-1902047	Reliability enhancements for NR sidelink broadcast	Huawei, HiSilicon

- R2-1902048	Initial thinking on potential RAN2 impacts on NR SL synchronization	Huawei, HiSilicon

- R2-1902049	Discussion on HARQ support for NR sidelink	Huawei, HiSilicon

- R2-1902050	Discussion on reliability enhancements for NR sidelink	Huawei, HiSilicon

- R2-1902051	Discussion on the PHR for NR sidelink	Huawei, HiSilicon

- R2-1902111	Further consideration on Tx carrier (re)selection in NR V2X	ITL

- R2-1900161	New triggers for RRC connection establishment/resume	CATT

- R2-1900164	LS on path selection for V2X services	CATT

- R2-1900187	Cell (re-)selection function in NR V2X sidelink	CATT

- R2-1900188	System Information Design for V2X	CATT

- R2-1900219	Sidelink SR/BSR in Uu interface	CATT

- R2-1900382	Considerations on NR V2X over Uu	ZTE, Sanechips

- R2-1900832	  Uu enhancements for NR-V2X sidelink resource allocation	ITRI

- R2-1901075	Inter-RAT control of sidelink communication	MediaTek Inc.

- R2-1901113	Discussion on NR Uu to control LTE mode 3	vivo

- R2-1901660	Cell reselection for NR V2X	Ericsson

- R2-1901663	NR Uu Enhancements for Advanced V2X Use Cases	Ericsson

- R2-1902052	Cell reselection for NR SL communication	Huawei, HiSilicon

- R2-1902053	The enhancement of Uu to control inter-RAT V2X sidelink	Huawei, HiSilicon

- R2-1902054	SC-PTM support for NR Uu	Huawei, HiSilicon

- R2-1902055	Discussion on NR support for multiple active uplink configured grants	Huawei, HiSilicon

- R2-1900170	Daft LS reply on SL RAT selection	OPPO

- R2-1900174	Discussion on RAT and Interface selection for NR-V2X	OPPO

- R2-1900385	Consideration on RAT and Interface Selection in NR V2X	ZTE, Sanechips

- R2-1901022	Interface selection for NR V2X	Lenovo, Motorola Mobility

- R2-1901056	RAT and Interface Selection for NR-V2X	CATT

- R2-1901703	Interface and RAT selection	Ericsson

- R2-1901705	Report of email discussion 104#57 NR V2X interface selection_v4	Ericsson

- R2-1901706	TP for NR V2X interface selection_v0	Ericsson

- R2-1902056	Discussion on interface selection	Huawei, HiSilicon

- R2-1900173	Discussion on QoS for NR-V2X	OPPO

- R2-1900370	Summary of Email Discussion [104#58][NR V2X] - QoS support for NR V2X	Huawei

- R2-1900371	TP to TR 38.885 on QoS support for NR V2X	Huawei, HiSilicon

- R2-1900372	[Draft] LS response to SA2 on unicast, groupcast and broadcast in NR sidelink	Huawei, HiSilicon

- R2-1900383	Discussion on QoS management for NR V2X	ZTE, Sanechips

- R2-1900483	QoS Management for NR V2X	Fraunhofer IIS, Fraunhofer HHI

- R2-1900606	Mobility challenges for NR V2X platooning	Nokia, Nokia Shanghai Bell

- R2-1900833	QoS feedback based on  index measurement of NR V2X	ITRI

- R2-1900885	Further QoS considerations for NR V2X	Intel Corporation

- R2-1900933	Logical Channel Prioritization Consideration	Spreadtrum Communications

- R2-1901053	QoS management for NR V2X	Lenovo, Motorola Mobility

- R2-1901076	QoS handling in NR V2X	MediaTek Inc.

- R2-1901114	Congestion control in NR V2X	vivo

- R2-1901119	Discussion on QoS management for NR V2X	vivo

- R2-1901575	QoS Management for NR V2X	InterDigital

- R2-1901699	On the use of communication range at access stratum	Ericsson

- R2-1901700	QoS principles	Ericsson

- R2-1901701	On NR sidelink QoS flow and radio bearer	Ericsson

- R2-1901704	On NR sidelink admission control	Ericsson

- R2-1901729	Discussion on QoS design for NR PC5 communication	Qualcomm Incorporated

- R2-1901786	Unicast SLRB configuration for RRC-Idle or RRC-Inactive UEs	Samsung Electronics

- R2-1901787	SDAP layer for SL groupcast/broadcast	Samsung Electronics

- R2-1901788	Admission control in NR SL	Samsung Electronics

- R2-1902057	QoS monitoring and reporting for NR Sidelink V2X Communications	Huawei, HiSilicon

- R2-1902058	Consideration on the admission control in NR SL	Huawei, HiSilicon

- R2-1902059	Remaining issues for QoS support of NR V2X	Huawei, HiSilicon

- R2-1902077	Consideration on supporting dual mode for NR-V2X	ITL

- R2-1901153	Considerations for support mobility for NR V2X	Samsung

- R2-1901904	Support of Vulnerable Road User	Apple, Samsung Electronics

- R2-1902125	Discussion on Support of Vulnerable Road User	Samsung Electronics

- R2-1902498	LS on RAT selection for SL 	SA2	LG Electronics

- R2-1902504	LS on SL RLM / RLF in NR V2X for unicast	RAN1	  Apple

- R2-1902505	LS on interface selection, 	SA2	Ericsson

- R2-1902494	LS response to SA2 on unicast, groupcast and broadcast in NR sidelink, 	SA2	Huawei

- LS to RAN1 on all RAN2 agreed TPs and RAN2 conclusions for NR V2X SI		RAN1	Huawei

RAN3#103

- R3-190012	          LS on NR V2X agreements   RAN2

- R3-190818	          Discussion on NR V2X Work Item scope    Ericsson

- R3-190555	          Text proposal for the conclusion of TR 38.885 on RAN3   LG Electronics

- R3-190559            Discussion on authorization in NR V2X     ZTE Corporation

- R3-190236	          Service Authorization for NR V2X      CATT

- R3-190133            Text Proposal on Service Authorization      LG Electronics

- R3-190623	          NR V2X Service Authorization        Huawei

- R3-190237            UE Sidelink AMBR for NR V2X       CATT

- R3-190135	          Text Proposal on UE sidelink AMBR        LG Electronics

- R3-190624	          UE Sidelink AMBR in NG-RAN           Huawei

- R3-190819	          Signaling of UE sidelink AMBR in NG-RAN        Ericsson

- R3-190548	          Support of NR V2X in case of CU-DU split         ZTE Corporation

- R3-190238	          F1 issues for NR V2X                 CATT

- R3-190132	          Text Proposal on support of sidelink resource allocation          LG Electronics

- R3-190625	          Principles for resource pool assignment         Huawei

- R3-190820           NR V2X Sidelink Resource pool configuration in NG-RAN         Ericsson

- R3-190821           Signalling of cross-RAT PC5 and PC5 control parameters in NG-RAN     Ericsson

- R3-190822           Resource coordination for cross-RAT V2X sidelink communication       Ericsson

- R3-190016           LS on RAT selection for SL            RAN2

- R3-190113           Reply LS on RAT selection for SL           SA WG2

- R3-190036          Combinations of Uu QoS characteristics values for V2X services      SA WG2

- R3-190239          Resource Co-ordination              CATT

- R3-190554          Considerations on the issues for MR-DC based NR V2X       LG Electronics

- R3-191057          Merged TP for V2X           Huawei

- R3-191071          Merged TP for V2X           Huawei

- R3-191072          LS on Conclusion of NR V2X and Agreed TP for TR 38.885    LG Electronics

- R3-191104          LS on Conclusion of NR V2X and Agreed TP for TR 38.885    LG Electronics

- R3-191060          Summary of Offline Discussion: (WF) Potential agreements in RAN3#103  LG Electronics

RAN4#90

- R4-1900164, "Reply LS on transient period in sidelink channels," Intel Corporation

- R4-1900381, "Further discussion on NR V2X RRM characteristics," Intel Corporation

- R4-1900405, "On V2X transient periods in sidelinks – discussion for RAN1 LS reply," InterDigital, Inc.

- R4-1900666, "Discussion on transient period for sidelink," Huawei, HiSilicon

- R4-1900406, "[draft] LS reply on transient period in sidelink channels," InterDigital, Inc.

- R4-1900667, "LS reply on transient period in sidelink channels," Huawei, HiSilicon

- R4-1901670, "Reply LS on on transient period in sidelink channels," Ericsson

- R4-1901764, "Discussion on Transient Period and AGC Settling Time for NR V2X," Qualcomm Inc.

- R4-1900691, "On AGC settling time," MediaTek inc.

- R4-1900165, "On V2X IBE model," Intel Corporation

- R4-1901669, "Discussion of Reply LS for NR V2X," Ericsson

- R4-1900005, "LS on transient period in sidelink channels," RAN1, LG Electronics

- R4-1902513, "WF on NR V2X RF parameter and transient period," LG Electronics France

[816]  R4-1902514, "Reply LS on NR UE transient period in sidelink channels," LG Electronics France

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion level swith colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template