3GPP TSG RAN Meeting #83								RP-190236

Shenzhen, China, March 18th – 21st, 2019

Agenda Item:	9.4.1

Source:	Apple

Title:	View on Rel-16 MIMO enhancement WI multi-panel design scope

Document for:	Discussion/Decision

# Introduction

Rel-16 MIMO enhancement WI was approved with the following scope [1]. Among the scope, beam management enhancement is one of the main topics.

- Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support: - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2 - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul: - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission - Enhancements on multi-beam operation, primarily targeting FR2 operation: - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR - Perform study and make conclusion in the first RAN1 meeting after start of the WI, and if needed, specify CSI-RS and DMRS (both downlink and uplink) enhancement for PAPR reduction for one or multiple layers (no change on RE mapping specified in Rel-15) - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

With respect to enhancements on multi-beam operation, one important topic is multi-panel support for FR2. However, the understanding of the scope of multi-panel support/enhancement in Rel-16 framework is different among different companies, especially regarding whether simultaneous UL transmission from multiple panels is within the scope of Rel-16 MIMO enhancement WI. RAN1 has already spent four meetings discussing the scope of the multi-panel support/enhancement without consensus. As results, the following agreement was reached in the last RAN1 meeting RAN1#96 [2]

For purpose of further discussion on this topic for RAN1#96 and future meetings Following multi-panel UE (MPUE) categories can be used for discussions on possible enhancements over Rel-15, if needed. - MPUE-Assumption1: Multiple panels are implemented on a UE and only one panel can be activated at a time, with panel switching/activation delay of [X] ms - MPUE-Assumption2: Multiple panels are implemented on a UE and multiple panels can be activated at a time and one or more panels can be used for transmission - MPUE-Assumption3: Multiple panels are implemented on a UE and multiple panels can be activated at a time but only one panel can be used for transmission Note: Above does not imply the support of either one or both of the categories but is only for efficient discussions at least for this meeting, which may also be updated further. Whether to support either one or both categories will depend on subsequent discussions Note: There is no consensus among the companies in RAN1 whether MPUE-Assumption2 is in the work scope of Rel-16 WI Agreement If RAN1 cannot agree on the support of at least one of MPUE-Assumption1, MPUE-Assumption2, MPUE-Assumption3, enhancements on panel-specific beam selection for uplink will not be supported in Rel-16. - Deadline for decision: RAN1#96bis

In our view, there are two categories of UEs in terms of multi-panel support

- UE that only needs to support antenna panel switching, i.e. transmit from one antenna panel at any given time: MPUE-Assumption1 and MPUE-Assumption3 fit into this category

- UE that needs to support simultaneous UL transmission from multiple antenna panels: MPUE-Assumption2 fits into this category

In this contribution, we provide our view on the scope of Rel-16 MIMO enhancement WI multi-panel design.

# Antenna panel switching vs simultaneous UL transmission

When UE is equipped with multiple antenna panels in FR2, it is expected to have performance benefit at the expense of complexity, power consumption, etc. There are two main benefits from the multiple antenna panels:

- Diversity gain: Given a typical form factor and use case, a single antenna panel may not be able to provide equally good coverage in all the spherical directions. The signals transmitted from an antenna panel can be blocked by the device itself, hand holding the device or even the surroundings like human body, etc. Therefore, a second antenna panel can help alleviate the problem by providing complementary coverage. It is reasonable to expect that, for majority of the time and use case, at any given time, only one antenna panel has good coverage while the other antenna panel has much worse signal quality. Antenna panel switching can be effectively utilized to exploit the diversity provided by multiple antenna panels in order to improve communication reliability.

- Multiplexing gain: To exploit multiplexing gain, multiple data streams, i.e. TB, can be transmitted from multiple antenna panels at the same time. When SINR is high enough, and under the condition that the signal qualities of multiple antenna panels are not too much different, it is expected that this operation can improve user data rate.

It is important to note that coverage is a more important issue for FR2 compared to data rate, since FR2 can support up to 400MHz bandwidth in a single CC and even larger bandwidth can be supported by CA. Given the limited UE maximum transmit power, it is not expected to be a common case that UE can enjoy high enough SINR in order to apply spatial multiplexing from multiple panel to demonstrate data rate gain. At the same time, polarization-based MIMO from a single panel is already supported and more practical to be used in term of data rate improvement. Furthermore, as we discussed, in actual device design, the main use case for a second antenna panel is to provide better coverage by covering area complementary to the primary antenna panel.

We want to emphasize that, for UEs that have multiple physical antenna panels, it does not necessarily mean the UE has to support simultaneous UL transmission. Similar as 1T/4R. 2T/4R and 1T/2R design in Rel-15 NR for FR1, multiple antennas can be used for simultaneous reception, but only a subset of antennas can be used for simultaneous UL transmission. Even with multiple physical antenna panels, compared to antenna panel switching, it can increase the cost and complexity of UE significantly if UE is required to support simultaneous UL transmission from multiple panel, in addition, it may also cause thermal and RF issues, as well as increased power consumption etc.

With these considerations in mind, we believe Rel-16 MIMO enhancement design for multi-panel support/enhancement should be limited to antenna panel switching for UL transmission.

Proposal 1: Rel-16 MIMO enhancement design for multi-panel support/enhancement should be limited to antenna panel switching for UL transmission.

# Simultaneous UL transmission from multiple panels

Even within the scope of simultaneous UL transmission from multiple panels, there can be different operation modes with different level of complexity, similar to the coherent, partial coherent and non-coherent UL MIMO operation in Rel-15. We illustrated the 3 possible operation modes of simultaneous UL multi-panel transmission in Figure 1.

Figure 1 Modes for simultaneous UL transmission from multiple panels

- Mode 1: In this mode, a single TB is transmitted from both antenna panels at the same time. From function perspective, this can be viewed as single panel where two antenna panels are used to create a single virtual antenna panel. This is similar as open loop beam forming or small delay CDD which is considered for supporting full power transmission in Rel-16 as well.

- Mode 2: In this mode, two TBs are transmitted, one TB is transmitted from one antenna panels while the other TB is transmitted from the other antenna panels. This is similar as the non-coherent UL MIMO operation in Rel-15. In terms of UE complexity, mode 2 does not require the coherence between different antenna panels, but it relies on gNB receiver to process and suppress interference between different TB. It is important to note that Mode 2 can also be used to support multi-TRP operation where each TRP requires a different beam for reception as they are not collocated. In order to communicate with multiple TRP at the same time, mode 2 operation with multiple panels at UE side is required.

- Mode 3: In this mode, two TBs are transmitted, each TB can be transmitted from both antenna panels with different coefficient for precoding. This is similar as the coherent UL MIMO operation in Rel-15. In FR2 applications, it is beneficial to apply a MIMO precoder when the two Tx beams are correlated, such as the two panels transmit to the overlapped directions. That implies the two antenna panels have overlapped fields-of-view and a reduced coverage. It may not be an effective design regardless the type of devices (i.e. handheld, laptop, vehicle). In the worst case of such a design, it will be more effective to exploit polarization-based MIMO without using a MIMO precoder. Additionally, in terms of UE complexity, mode 3 is the most complicated for most of the cases.

In our view, for simultaneous UL transmission from multiple panels, Mode 2 operation is more beneficial in practice whole mode 3 operation has very limited use case and may only be feasible or practical for certain devices without form factor restriction, such as laptop etc. Therefore, even if it is decided to start the simultaneous UL transmission from multiple antenna panels in future release, we prefer to prioritize mode 2 operation over mode 3 operation.

Proposal 2: If it is decided to start the simultaneous UL transmission from multiple antenna panels in future release, we prefer to prioritize non-coherent MIMO operation over coherent MIMO operation.

# Conclusion

The actual scope of multi-beam operation in Rel-16 MIMO enhancement WI, e.g. multi-panel support for FR2, has been intensively discussed in RAN1 for the past four meetings without consensus. On one side, multiple panel support can bring in benefit such as increased reliability, more diversity and improved UL data rate. On the other side, it increases UE complexity, consumes more power, may introduce thermal and RF issue, etc. In this contribution, we present our view on Rel-16 MIMO enhancement WI multi-panel design scope, with the following proposals

Proposal 1: Rel-16 MIMO enhancement design for multi-panel support/enhancement should be limited to antenna panel switching for UL transmission.

Proposal 2: If it is decided to start the simultaneous UL transmission from multiple antenna panels in future release, we prefer to prioritize non-coherent MIMO operation over coherent MIMO operation.

# References

- RP-181453, "WI Proposal on NR MIMO Enhancements", 3GPP TSG RAN Meeting #80 La Jolla, USA, June 11-14, 2018

- Chairman Notes, 3GPP RAN1#96 Meeting, Athens, Greece, 25th February – 1st March, 2019