# Annex 2 – Modifications to TR37.910 (v.1.1.0) – Part1

## [Reason for changes] clarify note on applicability of IMT-2020 targets, and results, for user (5%-ile) SE in Rural-eMBB config c, for both NR and LTE

===================================  Start of changes ==================================

## 5.4	Average spectral efficiency

As defined in Report ITU-R M.2410, average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

As required by Report ITU-R M.2412, average spectral efficiency and 5th percentile user spectral efficiency are assessed jointly using the same simulation.

### 5.4.1	NR

### …

#### 5.4.1.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for NR.

##### 5.4.1.3.1	Evaluation configuration A (CF = 700 MHz)

…

##### 5.4.1.3.2	Evaluation configuration B (CF = 4 GHz)

…

##### 5.4.1.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.4.1.3.3-1.

Similar to Indoor Hotspot – eMBB test environment, the capability of NR in larger bandwidth are evaluated for DL spectral efficiency. The values of the assumed bandwidths are shown together with the evaluation results.

It is observed that both NR FDD and TDD fulfill the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.1.3.3-1 DL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement 1 |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz | Number of samples | BW= 10MHz | BW= 20MHz | BW= 40MHz |
| 8x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 7 | 7.93 | 8.93 | 9.53 | 5 | 8.15 | 9.18 | 9.8 |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.2 | 0.22 | 0.24 |  | 0.21 | 0.24 | 0.25 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (8,4,2,1,1;2,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.13 | 9.13 | 9.73 | 1 | 8.14 | 9.13 | 9.73 |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.2 | 0.23 | 0.24 |  | 0.2 | 0.23 | 0.24 |
| 16x4 MU-MIMO Type II Codebook; gNB Config = (4,8,2,1,1;1,8) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 9.02 | 10.22 | 10.93 | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.3 | 0.34 | 0.36 |  | / | / | / |
| 8x4 MU-MIMO ideal CSI feedback; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | [11.26] | [12.73] | [13.61] |
|  |  | 5th-tile [bit/s/Hz] | - |  | / | / | / |  | [0.27] | [0.30] | [0.32] |
| 8x4 MU-MIMO Type I codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 2 | [9.29] | [10.42] | [11.11] | / | / | / | / |
|  |  | 5th-tile [bit/s/Hz] | - |  | [0.29] | [0.33] | [0.35] |  | / | / | / |
| 8x4 MU-MIMO Type I codebook; gNB Config = (8,4,2,1,1;1,4) (Optimized for cell edge) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | / | / | / | / | 1 | 3.9 | 4.37 | 4.65 |
|  |  | 5th-tile [bit/s/Hz] | - |  | / | / | / |  | 0.84 | 0.94 | 1.00 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requirement defined for Rural – eMBB is not applicable to LMLC. The 5th-tile results shown here are for information only. |  |  |  |  |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement 1 |  | Channel model A |  |  |  | Channel model B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz | Number of samples | BW= 20MHz | BW= 40MHz | BW= 100MHz |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.7 | 8.9 | 9.71 | 2 | 8.14 | 9.45 | 10.34 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.2 | 0.23 | 0.25 |  | 0.25 | 0.29 | 0.32 |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.01 | 9.02 | / | 1 | 8.02 | 9.04 | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.2 | 0.22 | / |  | 0.2 | 0.22 | / |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;2,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.61 | 9.98 | 10.91 | 1 | 8.59 | 9.95 | 10.89 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.2 | 0.23 | 0.25 |  | 0.19 | 0.22 | 0.25 |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;2,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.83 | 9.98 | / | 1 | 8.82 | 9.96 | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.22 | 0.25 | / |  | 0.21 | 0.24 | / |
| 16x4 MU-MIMO, Type II Codebook; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 10.44 | 12.41 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.35 | 0.41 | / |  | / | / | / |
| 64x8 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,32,2,1,1;1,32) | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 6.01 | 7.3 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | / | / | / |  | / | / | / |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 2 | 7.98 | 9.33 | / | 1 | 8.76 | 10.30 | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.25 | 0.29 | / |  | 0.23 | 0.27 | / |
| 8x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (8,4,2,1,1;1,4) | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 8.34 | 9.91 | 10.97 | 1 | 8.38 | 9.96 | 11.02 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.28 | 0.33 | 0.37 |  | 0.31 | 0.37 | 0.41 |
| 16x4 MU-MIMO, Reciprocity based; 4T SRS; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.24 | 6.04 | / | / | / | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.16 | 0.18 | / |  | / | / | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requirement defined for Rural – eMBB is not applicable to LMLC. The 5th-tile results shown here are for information only. |  |  |  |  |  |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for NR FDD and NR TDD for evaluation configuration C are provided in Table 5.4.1.3.3-2.

It is observed that both NR FDD and TDD fulfill the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.1.3.3-2 UL spectral efficiency for NR in Rural – eMBB 
(Evaluation configuration C, LMLC)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement1 |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 4.22 | 2 | 4.16 |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.14 |  | 0.13 |
| 1x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | [4.54] | / | / |
|  |  | 5th-tile [bit/s/Hz] | - |  | [0.09] |  | / |
| 4x8 SU-MIMO, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | [7.04] | 1 | [4.80] |
|  |  | 5th-tile [bit/s/Hz] | - |  | [0.07] |  | [0.09] |
| 4x16 MU-MIMO, OFDMA; gNB Config = (4,8,2,1,1;1,8); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 2 | 8.10 | / | / |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.10 |  | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement defined for Rural – eMBB is not applicable to LMLC. The 5th-tile results shown here are for information only. |  |  |  |  |  |  |  |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement 1 |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Number of samples | BW=20MHz | Number of samples | BW=20MHz |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.33 | 1 | 3.31 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.06 |  | 0.05 |
| 2x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.53 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.05 |  | / |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.84 | 2 | 3.53 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.07 |  | 0.07 |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 15 | DDDSU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.03 | 1 | 4.04 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.06 |  | 0.05 |
| 4x16 SU-MIMO, Codebook based, OFDMA; gNB Config = (4,8,2,1,1;1,8); | 15 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 10.59 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | 0.08 |  | / |
| 8x64 MU-MIMO, Codebook based, OFDMA; gNB Config = (4,32,2,1,1;1,32) | 15 | UUUUU | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.99 | / | / |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | / |  | / |
| 4x8 SU-MIMO, Codebook based, OFDMA; gNB Config = (8,4,2,1,1;1,4); | 30 | DSUUD | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 4.41 | 1 | 3.56 |
|  |  |  | 5th-tile [bit/s/Hz] | - |  | / |  | / |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement defined for Rural – eMBB is not applicable to LMLC. 5th-tile results shown here are for information only. |  |  |  |  |  |  |  |  |

### 5.4.2	LTE

Average spectral efficiency and 5th percentile user spectral efficiency are evaluated for LTE. Both LTE FDD and TDD are evaluated. Detailed evaluation assumptions and results can be found in Annex B.4.1.

#### 5.4.2.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz), evaluation configuration B (carrier frequency = 4 GHz), and evaluation configuration C (LMLC) are applied for the evaluations of Rural – eMBB test environment for LTE.

##### 5.4.2.3.1	Evaluation configuration A (CF = 700 MHz)

< skip text>

##### 5.4.2.3.2	Evaluation configuration B (CF = 4 GHz)

< skip text>

##### 5.4.2.3.3	Evaluation configuration C (LMLC)

LMLC (Low mobility large cell) is characterized by the large inter-site distance (ISD=6000m) and the low mobility users in Rural – eMBB test environment.

The evaluation results of DL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.4.2.3.3-1.

It is observed that both LTE fulfills the DL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.2.3.3-1 DL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC)  (a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement 1 |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 8x4 MU-MIMO, Type II Codebook; gNB Config = (8,4,2,1,1;1,4) | 15 | Average [bit/s/Hz/TRxP] | 3.3 | 1 | 5.96 | 1 | 5.97 |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.15 |  | 0.15 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement defined for Rural – eMBB is not applicable to LMLC. The 5th-tile results shown here are for information only. |  |  |  |  |  |  |  |

The evaluation results of UL spectral efficiency for LTE for evaluation configuration C are provided in Table 5.4.2.3.3-2.

It is observed that both LTE fulfills the UL spectral efficiency requirement for these configurations in evaluation configuration C.

Table 5.4.2.3.3-2 UL spectral efficiency for LTE in Rural – eMBB 
(Evaluation configuration C, LMLC) (a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement 1 |  | Channel model A |  | Channel model B |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Number of samples | BW=10MHz | Number of samples | BW=10MHz |
| 2x8 SU-MIMO, Codebook based, DFT-S-OFDM; gNB Config = (8,4,2,1,1;1,4); | 15 | Average [bit/s/Hz/TRxP] | 1.6 | 1 | 3.36 | 1 | 3.31 |
|  |  | 5th-tile [bit/s/Hz] | - |  | 0.07 |  | 0.06 |
| Note 1: According to Report ITU-R M.2410, the 5th percentile user spectral efficiency requriement defined for Rural – eMBB is not applicable to LMLC. The 5th-tile results shown here are for information only.for information only. |  |  |  |  |  |  |  |

===================================  End of changes ==================================