# Annex 2 – Modifications to TR37.910 (v.1.1.0)

=====================================  First change =====================================

[Reason for changes] removing extra and partial/unclear options/assumptions (TDD+SUL)...beyond agreed/common duplex evaluations (FDD&TDD), and inconsistent with peak SE evaluation (no SUL).

## 5.2	Peak data rate

As defined in Report ITU-R M.2410, peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

For both NR and LTE, the DL/UL peak data rate for FDD and TDD over Q component carriers can be calculated as below

(5.2-1)

where Wj and SEpi (j = 1,…, Q) are the effective bandwidth and spectral efficiencies on component carrier j, respectively, (j) is the normalized scalar on component carrier j considering the downlink/uplink ratio on that component carrier; for FDD (j) =1 for DL and UL; and for TDD (j) for DL and UL is calculated based on the frame structure, and BW(j) is the carrier bandwidth of component j.

### 5.2.1	NR

#### 5.2.1.1	DL peak data rate

A range of configurations are considered in the evaluation of downlink peak data rate.

DL peak data rate for NR is evaluated based on the evaluation results of NR peak spectral efficiency provided in Section 5.1.1.1. Table 5.2.1.1-1 provides the evaluation results for the specific component carrier (CC) bandwidth. It is observed that NR fulfils the DL peak data rate requirement.

Table 5.2.1.1-1 NR DL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Aggregated peak data rate over 16 CCs (Gbit/s) | Required DL bandwidth to meet the requirement (MHz)1 | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 2.31~2.41 | 37.0~38.6 | 414~433 | 20 |
|  |  | 30 | 100 | 4.67~4.89 | 74.7~78.2 | 409~428 |  |
|  |  | 60 | 100 | 4.62~4.82 | 73.9~77.1 | 415~433 |  |
| TDD (DSUUD, S slot= 11DL:2GP:2UL) | FR1 | 15 | 50 | 1.32 | 21.1 | 757 |  |
|  |  | 30 | 100 | 2.69 | 43.0 | 745 |  |
|  |  | 60 | 100 | 2.64 | 42.3 | 757 |  |
|  | FR2 (Nlayer=6) | 60 | 200 | 3.86 | 61.8 | 1036 |  |
|  |  | 120 | 400 | 7.81 | 125.0 | 1024 |  |
| TDD (DSUUD, S slot= 6DL:2GP:6UL) | FR1 | 15 | 50 | 1.13 | 18.1 | 885 |  |
|  |  | 30 | 100 | 2.30 | 36.8 | 870 |  |
|  |  | 60 | 100 | 2.26 | 36.2 | 885 |  |
|  | FR2 (Nlayer=8) | 60 | 200 | 4.38 | 70.1 | 913 |  |
|  |  | 120 | 400 | 8.76 | 140.2 | 913 |  |
| NOTE 1: The value only indicates the required bandwidth to meet the DL peak data rate. It is not necessarily supported as NR Transmission bandwidth. |  |  |  |  |  |  |  |

#### [Note1 : option removed to be consistent with UL (see change below).
Note2: 140.2 is currently the max value (of the range) reported in the compliance template, consistent with the above change]

#### 5.2.1.2	UL peak data rate

A range of configurations are considered in the evaluation of uplink peak data rate.

UL peak data rate for NR is evaluated based on the evaluation results of NR peak spectral efficiency provided in Section 5.1.1.2. Table 5.2.1.2-1 provides the evaluation results for the specific component carrier (CC) bandwidth. It is observed that NR fulfils the UL peak data rate requirement.

Table 5.2.1.2-1 NR UL peak data rate

| Duplexing | SCS [kHz] |  | Per CC BW (MHz) | Peak data rate per CC (Gbit/s) | Aggregated peak data rate over 16 CCs (Gbit/s) | Required UL bandwidth to meet the requirement (MHz)1 | Req. (Gbit/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | FR1 | 15 | 50 | 1.12~1.18 | 17.9~18.9 | 424~446 | 10 |
|  |  | 30 | 100 | 2.28~2.39 | 36.5~38.2 | 418~439 |  |
|  |  | 60 | 100 | 2.27~2.38 | 36.3~38.1 | 420~441 |  |
| TDD (DSUUD, S slot =11DL:2GP:2UL) | FR1 | 30 | 100 | 1.06 | 17.0 | 943 |  |
|  |  | 60 | 100 | 1.05 | 16.8 | 952 |  |
|  | FR2 | 60 | 200 | 1.91 | 30.6 | 1047 |  |
|  |  | 120 | 400 | 3.85 | 61.6 | 1039 |  |
| TDD (DSUUD, S slot =6DL:2GP:6UL) | FR1 | 30 | 100 | 1.05 | 16.8 | 952 |  |
|  |  | 60 | 100 | 1.04 | 16.6 | 962 |  |
|  | FR2 | 60 | 200 | 2.02 | 32.3 | 990 |  |
|  |  | 120 | 400 | 4.04 | 64.6 | 990 |  |
| NOTE 1: The value only indicates the required bandwidth to meet the UL peak data rate. It is not necessarily supported as NR Transmission bandwidth. |  |  |  |  |  |  |  |

===================================  Next change ==================================

### [Reason for changes] removing confg C: Partial evaluation (UL only), and based on extra/unclear assumptions (TDD+SUL, offload)…beyond agreed/common options (FDD&TDD] and DL&UL evaluations.

## 5.5	User experienced data rate

As defined in Report ITU-R M.2410, user experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

### 5.5.1	NR

User experienced data rate for NR are evaluated under Dense Urban – eMBB test environment.

#### 5.5.1.1	Dense Urban – eMBB

For Dense Urban – eMBB test environment, single-band single-layer case using evaluation configuration A (carrier frequency = 4 GHz)as defined in Report ITU-R M.2412 is considered in evaluation. Detailed evaluation assumptions for configuration A are based on spectral efficiency evaluation, and can be found in Annex B.4.1 [Note: if changes proposed here are agreed, Annex B.4.3 should also be updated]

[ skip text ]

##### 5.5.1.1.2	Evaluation configuration B (CF = 30 GHz)

TBD

##### 5.5.1.1.3	Evaluation configuration C

TBD

===================================  Next change ==================================

### [Reason for changes] removing extra and unclear options/assumptions (TDD+SUL)..beyond agreed/common evaluations (FDD&TDD]

### 5.7.1	User plane latency

As defined in Report ITU-R M.2410, user plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms).

#### 5.7.1.1	NR

[ skip text ]

##### 5.7.1.1.1	Downlink

The downlink procedure is abstracted in Table 5.7.1.1.1-1, where the assumptions of each step for evaluation are given.

Table 5.7.1.1.1-1 DL user plane procedure for NR

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | DL data transfer | T1 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx |  |
| 1.1 | BS processing delay | tBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2, with d2,1= d2,2= d2,3=0. (Tproc,2 is defined in Section 6.4 of TS38.214) (NOTE1, NOTE2) |
| 1.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot. |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | UE processing delay | tUE,rx The time interval between the PDSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. (NOTE3) |
| 2 | HARQ retransmission | THARQ = T1 + T2 T2 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | UE processing delay | tUE,tx The time interval between the data is decoded, and ACK/NACK packet is generated. | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. (NOTE4) |
| 2.2 | UL frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot; Twait is the waiting time for next available UL slot if the current slot is not UL slot |
| 2.3 | TTI for ACK/NACK transmission | tUL_duration | 1 OFDM symbol |
| 2.4 | BS processing delay | tBS,rx The time interval between the ACK is received and the ACK is decoded. | Tproc, 2/2 with d2,1= d2,2= d2,3=0. (NOTE1, NOTE5) |
| 2.5 | Repeat DL data transfer from 1.1 to 1.4 | T1 |  |
| - | Total one way user plane latency for DL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. 3. For the above case, the UE is processing PDSCH reception on TDD band with 30kHz SCS, and it is assumed that the value of this step is Tproc,1 =30kHz)/2. 4. For the above case, the value of this step is Tproc,1( =15kHz) - Tproc,1( =30kHz)/2. 5. For the above case, the value of this step is Tproc,2( =15kHz)/2. |  |  |  |

Based on the DL user plane procedure and assumptions given in Table 5.7.1.1.1-1, a variety of configurations and UE capabilities are evaluated for NR.

For NR FDD, the evaluation results assuming an initial transmission error probability of p=0 and p=0.1 are provided in Table 5.7.1.1.1-2.

Table 5.7.1.1.1-2 DL user plane latency for NR FDD (ms)

| DL user plane latency – NR FDD |  |  | UE capability 1 |  |  |  |  | UE capability 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SCS |  |  |  |  | SCS |  |  |  |
|  |  |  | 15 kHz | 30 kHz | 60 kHz | 120 kHz | 15 kHz |  | 30 kHz | 60 kHz |
| Resource mapping Type A | M=4 (4OS non-slot) | p=0 | 1.37 | 0.76 | 0.54 | 0.34 | 1.00 |  | 0.55 | 0.36 |
|  |  | p=0.1 | 1.58 | 0.87 | 0.64 | 0.40 | 1.12 |  | 0.65 | 0.41 |
|  | M=7 (7OS non-slot) | p=0 | 1.49 | 0.82 | 0.57 | 0.36 | 1.12 |  | 0.61 | 0.39 |
|  |  | p=0.1 | 1.70 | 0.93 | 0.67 | 0.42 | 1.25 |  | 0.71 | 0.44 |
|  | M=14 (14OS slot) | p=0 | 2.13 | 1.14 | 0.72 | 0.44 | 1.80 |  | 0.94 | 0.56 |
|  |  | p=0.1 | 2.43 | 1.29 | 0.82 | 0.51 | 2.00 |  | 1.04 | 0.63 |
| Resource mapping Type B | M=2 (2OS non-slot) | p=0 | 0.98 | 0.56 | 0.44 | 0.29 | 0.49 |  | 0.29 | 0.23 |
|  |  | p=0.1 | 1.16 | 0.67 | 0.52 | 0.35 | 0.60 |  | 0.35 | 0.28 |
|  | M=4 (4OS non-slot) | p=0 | 1.11 | 0.63 | 0.47 | 0.31 | 0.66 |  | 0.37 | 0.27 |
|  |  | p=0.1 | 1.30 | 0.74 | 0.56 | 0.36 | 0.78 |  | 0.45 | 0.32 |
|  | M=7 (7OS non-slot) | p=0 | 1.30 | 0.72 | 0.52 | 0.33 | 0.93 |  | 0.51 | 0.34 |
|  |  | p=0.1 | 1.49 | 0.83 | 0.61 | 0.39 | 1.08 |  | 0.59 | 0.40 |

For NR TDD, the various DL/UL configurations are evaluated. The evaluation results of DDDSU (with S slot = 11DL:1GP:2UL), DSUUD (with S slot = 11DL:1GP:2UL), and DUDU (without GP) are provided in Table 5.7.1.1.1-3, Table 5.7.1.1.1-4, and Table 5.7.1.1.1-5, respectively.

[ skip text ]

It is observed that NR fulfils DL user plane latency requirement in a wide range of configurations.

##### 5.7.1.1.2	Uplink

The uplink procedure using a grant free transmission is abstracted in Table 5.7.1.1.2-1, where the assumptions of each step for evaluation are given.

Table 5.7.1.1.2-1 UL user plane procedure for NR

| Step | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | UL data transfer | T1 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx |  |
| 1.1 | UE processing delay | tUE,tx The time interval between the data is arrived, and packet is generated; | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 1.2 | UL Frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot, Twait is the waiting time for next available UL slot if the current slot is not UL slot. |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | BS processing delay | tBS,rx The time interval between the PUSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”; It is assumed that BS processing delay is equal to UE processing delay as for PDSCH (Note1) |
| 2 | HARQ retransmission | THARQ = T2 + T1 T2 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | BS processing delay | tBS,tx The time interval between the data is decoded, and PDCCH preparation | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. |
| 2.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot; |
| 2.3 | TTI for PDCCH transmission | tDL_duration | 1 OFDM symbols for PDCCH scheduling the retransmission. |
| 2.4 | UE processing delay | tUE,rx The time interval between the PDCCH is received and decoded. | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 2.5 | Repeat UL data transfer from 1.1 to 1.4 | T1 |  |
|  | Total one way user plane latency for UL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |
| Note: 1. The value is used for evaluation only; gNB processing delay may vary depending on implementation. Note: 2.The grant free transmission is assumed to use the following start symbols: - For 2-symbol PUSCH, the start symbol can be symbols {0,2,4,6,8,10,12} for PUSCH resource mapping type B - For 4-symbol PUSCH, the start symbol can be: - For PUSCH resource mapping type B: symbols {0,7} - For PUSCH resource mapping type A: symbol 0; - For 7-symbol PUSCH, the start symbol can be: - For PUSCH resource mapping type B: symbols {0, 7} - For PUSCH resource mapping type A: symbol 0; - For 14-symbol PUSCH, the start symbol can be at symbol #0 for PUSCH resource mapping type A and B. |  |  |  |

Based on the UL user plane procedure and assumptions given in Table 5.7.1.1.2-1, a variety of configurations and UE capabilities are evaluated for NR.

For NR FDD, the evaluation results assuming an initial transmission error probability of p=0 and p=0.1 are provided in Table 5.7.1.1.2-2.

[ skip text ]

It is observed that NR fulfils UL user plane latency requirement in a wide range of configurations.

===================================  next change ==================================

### [Reason for changes] removing extra and unclear options/assumptions (TDD+SUL)..beyond agreed/common duplex evaluations (FDD&TDD]

### 5.7.2	Control plane latency

As defined in Report ITU-R M.2410, control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

- 5.7.2.1	NR

For NR Rel-15, control plane latency is evaluated from RRC_INACTIVE state to RRC_CONNECTED state. Figure 5.7.2.1-1 provides an example control plane flow for NR Rel-15.

UE

gNB

1. Delay for RACH Scheduling Period

3. Processing delay in gNB

5. Processing delay in UE

7. Processing delay in gNB

9. Processing delay in UE

2. RACH Preamble

4. RA response

6. RRC Resume Request

8. RRC Resume

10. RRC Resume Complete

Control plane procedure

Figure 5.7.2.1-1 C-plane procedure (example for NR Rel-15)

The detailed assumption of each step as shown in Figure 5.7.2.1-1 is provided in Table 5.7.2.1-1. The evaluation is for UL data transfer. It is understood that the evaluation results for DL data transfer can be further reduced because UE processing delay in Step 9 for DL data transfer does not need to handle UL grant receiving, and therefore can be reduced compared to the case of UL data transfer.

NOTE: The delay values shown below do not include the waiting time for DL/UL subframe. It is only gNB or UE processing delay. The waiting time will be calculated and it depends on the detailed DL/UL configuration.

Table 5.7.2.1-1 Assumption of C-plane procedure for NR

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2 | Transmission of RACH Preamble | Length of the preamble according to the PRACH format as specified in [6 38.211] |
| 3 | Preamble detection and processing in gNB | Tproc,2 (assuming d2,1=0) |
| 4 | Transmission of RA response | Ts (the length of 1 slot / non-slot) NOTE: the length of 1 slot or 1 non-slot include PDCCH and PDSCH (the first OFDM symbol of PDSCH is frequency multiplexed with PDCCH). |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Resume Request) | NT,1+NT,2+0.5 ms |
| 6 | Transmission of RRC Resume Request | Ts (the length of 1 slot / non-slot) NOTE: the length of 1 slot or 1 non-slot is equal to PUSCH allocation length. |
| 7 | Processing delay in gNB (L2 and RRC) | 3 |
| 8 | Transmission of RRC Resume | Ts (the length of 1 slot / non-slot) |
| 9 | Processing delay in UE of RRC Resume including grant reception | 7 |
| 10 | Transmission of RRC Resume Complete and UP data | 0 |
| Notes: - For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. - For step 3, the value of Tproc,2 is used only for evaluation. gNB processing delay may vary depending on implementation. - For step 5, the latency of NT,1+NT,2+0.5ms is used according to Section 8.3 of TS 38.213. NT,1 is a time duration of N1 symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 when additional PDSCH DM-RS is configured; and NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1. The value of N1 and N2 are shown in Table 5.3-1 and Table 6.4-1 of TS38.214, respectively.. - For step 7, the processing delay in gNB (L2 and RRC) has been reduced to 3 ms. The delays due to inside-gNB or inter-gNB communication are not included in Step 7. Such delays may exist depending on deployment, but are not within the scope of this evaluation. - For step 9 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. - For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |

In addition, the following assumptions apply to the evaluation:

- The transmission duration of Step 2, 4, 6, and 8 cannot be crossing the boundary of a slot;

- The CP procedure can start from the OFDM symbols within the slot that PRACH preamble can be transmitted (assuming that the slot is UL slot; otherwise it will wait for the available UL slot).

Based on the control plane procedure and assumptions given in Table 5.7.2.1-1, a variety of configurations and UE capabilities are evaluated for NR for UL data transfer. For a specific configuration, the results are the average over the possible start timing of the control plane procedure.

For NR FDD, the evaluation results of different PRACH lengths are provided in Table 5.7.2.1-2. The evaluation is applied to various non-slot length and sub-carrier spacings. Resource mapping type A and B are considered. UE capability 1 and UE capability 2 are evaluated.

[ skip text ]

It is observed that NR fulfils the control plane latency requirement of 20ms in a wide range of configurations. If, in control plane procedure, the latency of step 7 and step 9 can be further reduced, the 10ms target as encouraged by ITU-R can be achieved in some cases.