3GPP TSG-RAN WG4 Meeting #90	R4-1901256

# Athens, Greece, 25 February - 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5381 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of CQI reporting test requirements for eFeMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Perf |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SNR test points are in the square bracktes. RC names are tentative. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of square brackets for SNR test points Put the correct RC number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-15 eFeMTC CQI reporting test requirement is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.8.3, 9.8.4, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.521 ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN

#### 9.8.3.1	FDD and half-duplex FDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-PDSCH-64QAM-r15. For the parameters specified in Table 9.8.3.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.31 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.3.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 11 | 12 |
|  |  | dB[mW/15kHz] | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| CQI delay |  | ms | 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| Note 1: Reference measurement channel RC.31 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD and two sided dynamic OCNG pattern OP.2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |

#### 9.8.3.2	TDD

The following requirements apply to UE supporting supporting ce-ModeA-r13 and ce-PDSCH-64QAM-r15. For the parameters specified in Table 9.8.3.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.31 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.3.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 11 | 12 |
|  |  | dB[mW/15kHz] | -87 | -86 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolLC) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| Note 1: Reference measurement channel RC.31 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and two sided dynamic OCNG pattern OP.2 TDD as described in Annex A.5.2.1 and A.5.2.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: DC subcarrier puncturing shall be considered. |  |  |  |  |

### 9.8.4	CQI reporting definition for UE supporting alternative table under AWGN

#### 9.8.4.1	FDD and half-duplex FDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-CQI-AlternativeTable-r15. For the parameters specified in Table 9.8.4.1-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.32 FDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.4.1-1: PUCCH 1-0 static test (FDD and half-duplex FDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 4) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 10 |  |
| CQI delay |  | ms | 10 |  |
| cqi-pmi-ConfigurationIndex |  |  | 12 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-FDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolBR) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1111111111 |  |
| Note 1: Reference measurement channel RC.32 FDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD and two sided dynamic OCNG pattern OP.2 FDD as described in Annex A.5.1.1 and A.5.1.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: DC subcarrier puncturing shall be considered. Note 4: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#1 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5. |  |  |  |  |

#### 9.8.4.2	TDD

The following requirements apply to UE supporting ce-ModeA-r13 and ce-CQI-AlternativeTable-r15. For the parameters specified in Table 9.8.4.2-1, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2, the reported CQI value according to RC.32 TDD in Table A.4-1 shall be in the range of ±1 of the reported median more than 90% of the time. If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

Table 9.8.4.2-1: PUCCH 1-0 static test (TDD)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| PDSCH transmission mode |  |  | 1 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  | δ | dB | 0 |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 1) |  |
| SNR (Note 2) |  | dB | 5 | 6 |
|  |  | dB[mW/15kHz] | -93 | -92 |
|  |  | dB[mW/15kHz] | -98 | -98 |
| Max number of HARQ transmissions |  |  | 1 |  |
| Physical channel for CQI reporting |  |  | PUSCH (Note 3) |  |
| PUCCH Report Type |  |  | 4 |  |
| Reporting periodicity |  | ms | Npd = 5 |  |
| cqi-pmi-ConfigurationIndex |  |  | 3 |  |
| Frequency hopping |  |  | Disabled |  |
| Frequency hopping inverval (interval-TDD) |  |  | N/A |  |
| Starting OFDM symbol (startSymbolLC) |  |  | 3 |  |
| PDSCH repetition level |  |  | 1 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| MPDCCH repetition level |  |  | 1 |  |
| Beamforming Precoder for MPDCCH |  |  | No precoding |  |
| Precoder update granularity for MPDCCH |  |  | N/A |  |
| BL/CE DL subframe comfiguration (fdd-DownlinkOrTddSubframeBitmapBR) |  |  | 1011110111 |  |
| Note 1: Reference measurement channel RC.32 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD and two sided dynamic OCNG pattern OP.2 TDD as described in Annex A.5.2.1 and A.5.2.2. Note 2: The minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. MPDCCH DCI format 6-0A shall be transmitted in downlink SF#3 and #8 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#7 and #2. Note 4: DC subcarrier puncturing shall be considered. |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

# A.4	CSI reference measurement channels

This section defines the DL signal applicable to the reporting of channel status information (Clause 9.2, 9.3 and 9.5).

In Table A.4-1 are specified the reference channels. Table A.4-13 specifies the mapping of CQI index to modulation coding scheme, which complies with the CQI definition specified in Section 7.2.3 of [6].

Table A.4-0: Void

Table A.4-1: CSI reference measurement channels

| RMC Name | Duplex |  | CH-BW |  | Alloc. RB-s |  | UL/DL Config |  | Alloc. SF-s |  |  | MCS Scheme |  | Nr. HARQ Proc. |  | Max. nr HARQ Trans. |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 CRS Port |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.1 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.1 |  | 8 |  | 1 |  |  |
| RC.1A FDD | FDD |  | 10 |  | 50 |  |  |  |  |  |  | MCS.1A |  | 8 |  | 1 |  |  |
| RC.1 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.1 |  | 10 |  | 1 |  |  |
| RC.1A TDD | TDD |  | 20 |  | 100 |  | Note 3 |  |  |  |  | MCS.1B |  | 10 |  | 1 |  |  |
| RC.3 FDD | FDD |  | 10 |  | 6 |  | - |  |  |  |  | MCS.10 |  | 8 |  | 1 |  |  |
| RC.3 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  |  |  |  | MCS.10 |  | 10 or 7 (Note 9) |  | 1 |  |  |
| RC.4 FDD | FDD |  | 10 |  | 15 |  | - |  |  |  |  | MCS.15 |  | 8 |  | 1 |  | Note 6 |
| RC.4 TDD | TDD |  | 10 |  | 15 |  | Note 3 |  |  |  |  | MCS.15 |  | 10 |  | 1 |  | Note 6 |
| RC.5 FDD | FDD |  | 10 |  | 3 |  | - |  |  |  |  | MCS.17 |  | 8 |  | 1 |  |  |
| RC.5 TDD | TDD |  | 10 |  | 3 |  | Note 3 |  |  |  |  | MCS.17 |  | 10 |  | 1 |  |  |
| RC.14 FDD | FDD |  | 5 |  | 25 |  | - |  |  |  |  | MCS.14 |  | 8 |  | 1 |  |  |
| RC.15 FDD | FDD |  | 5 |  | 15 |  | - |  |  |  |  | MCS.15 |  | 8 |  | 1 |  | Note 6 |
| RC.16 FDD | FDD/HD-FDD |  | 10 |  | 2 |  |  |  |  |  |  | MCS.20 |  | 8 |  | 1 |  | Note 8,10 |
| RC.16 TDD | TDD |  | 10 |  | 2 |  | Note 3 |  |  |  |  | MCS.20 |  | 10 |  | 1 |  | Note 8 |
| RC.23FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.23 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 10 |  | 1 |  | Note 12 |
| RC.25 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 8 |  | 1 |  | Note 12, 14 |
| RC.25 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.28 |  | 10 |  | 1 |  | Note 12, 15 |
| RC.31 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.40 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.31 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.40 |  | 10 |  | 1 |  | Note 12 |
| RC.32 FDD | FDD/HD-FDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.41 |  | 8 |  | 1 |  | Note 12, 13 |
| RC.32 TDD | TDD |  | 10 |  | 3 |  |  |  |  |  |  | MCS.41 |  | 10 |  | 1 |  | Note 12 |
| RC.X FDD | FDD |  | 10 |  | 50 |  |  |  |  |  |  | MCS.x1A |  | 8 |  | 1 |  |  |
| RC. X TDD | TDdD |  | 20 |  | 100 |  |  |  |  |  |  | MCS.x2A |  | 10 |  | 1 |  |  |
| 2 CRS Ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.2 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.2 |  | 8 |  | 1 |  |  |
| RC.2A FDD | FDD |  | 20 |  | 100 |  |  |  |  |  |  | MCS.2A |  | 8 |  | 1 |  |  |
| RC.2 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.2 |  | 10 or 7 (Note 9) |  | 1 |  |  |
| RC.4A FDD | FDD |  | 20 |  | 15 |  | - |  |  |  |  | MCS.16 |  | 8 |  | 1 |  | Note 6 |
| RC.6 FDD | FDD |  | 10 |  | 15 |  | - |  |  |  |  | MCS.16 |  | 8 |  | 1 |  | Note 6 |
| RC.6 TDD | TDD |  | 10 |  | 15 |  | Note 3 |  |  |  |  | MCS.16 |  | 7 |  | 1 |  | Note 6 |
| 4 CRS Ports |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.17 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.18 |  | 8 |  | 1 |  |  |
| RC.17 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.18 |  | 7 |  | 1 |  |  |
| RC.21 FDD | FDD |  | 10 |  | 50 |  | - |  |  |  |  | MCS.26 |  | 8 |  | 1 |  |  |
| RC.21 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  |  |  |  | MCS.26 |  | 7 |  | 1 |  |  |
| 1 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.8 FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.11 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12 |  |  |  |  |  |  |
| RC.8A FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.11A |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12A |  |  |  |  |  |  |
| RC.8 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS |  |  | MCS.11 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12 |  |  |  |  |  |  |
| RC.8A TDD | TDD |  | 20 |  | 8 |  | Note 3 |  | Non CSI-RS |  |  | MCS.11B |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.12B |  |  |  |  |  |  |
| RC.9 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.3 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4 |  |  |  |  |  |  |
| RC.9A FDD | FDD |  | 20 |  | 100 |  | - |  | Non CSI-RS |  |  | MCS.3A |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4A |  |  |  |  |  |  |
| RC.9B FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS, rank 1/2 |  |  | MCS.3 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | Non CSI-RS, rank 3/4 |  |  | MCS.30 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 1/2 |  |  | MCS.29 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 3/4 |  |  | MCS.31 |  |  |  |  |  |  |
| RC.9 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.3 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.4 |  |  |  |  |  |  |
| RC.9B TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS, rank 1/2 |  |  | MCS.3 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | Non CSI-RS, rank 3/4 |  |  | MCS.30 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 1/2 |  |  | MCS.29 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS, rank 3/4 |  |  | MCS.31 |  |  |  |  |  |  |
| 2 CRS Port + CSI-RS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.7 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.7 |  |  |  |  |  |  |
| RC.7 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 8 CSI-RS |  |  | MCS.8 |  |  |  |  |  |  |
| RC.11 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.6 |  |  |  |  |  |  |
| RC.11 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS |  |  | MCS.6 |  |  |  |  |  |  |
| RC.18 FDD | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS |  |  | MCS.13 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.19 |  |  |  |  |  |  |
| RC.18 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS |  |  | MCS.13 |  | 7 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.19 |  |  |  |  |  |  |
| RC.17 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | 4 ZP-CSI-RS |  |  | MCS.21 |  | 10 |  | 1 |  |  |
| RC.18 TDD | TDD |  | 10 |  | 6 |  | Note 3 |  | 4 ZP-CSI-RS |  |  | MCS.22 |  | 10 |  | 1 |  |  |
| RC.19 TDD | TDD |  | 10 |  | 41 |  | Note3 |  | 4 ZP-CSI-RS |  |  | MCS.23 |  | 10 |  | 1 |  | Note 11 |
| RC.20 TDD | TDD |  | 10 |  | 50 |  | Note3 |  | Non CSI-RS |  |  | MCS.24 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 2 CSI-RS, 4 ZP-CSI-RS |  |  | MCS.25 |  |  |  |  |  |  |
| RC.22 FDD | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS |  |  | MCS.5 |  | 8 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.27 |  |  |  |  |  |  |
| RC.22 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.5 |  | 10 |  | 1 |  |  |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.27 |  |  |  |  |  |  |
| RC.23 TDD | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS |  |  | MCS.9 |  | 10 |  | 1 |  | Rank 4 |
|  |  |  |  |  |  |  |  |  | 4 CSI-RS |  |  | MCS.32 |  |  |  |  |  |  |
| 1 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.13 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS/IM | MCS.3 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| RC.13 TDD |  | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS/IM | MCS.3 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| 2 CRS Port + CSI-RS + CSI-IM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.10 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.5 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 4 CSI-RS, 1 CSI process | MCS.8 |  |  |  |  |  |  |  |
| RC.10 TDD |  | TDD |  | 10 |  | 50 |  | Note 3 |  | Non CSI-RS | MCS.5 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | 8 CSI-RS, 1 CSI process | MCS.9 |  |  |  |  |  |  |  |
| RC.12 FDD |  | FDD |  | 10 |  | 6 |  | - |  | Non CSI-RS/IM | MCS.13 |  | 8 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| RC.12 TDD |  | TDD |  | 10 |  | 6 |  | Note 3 |  | Non CSI-RS/IM | MCS.13 |  | 10 |  | 1 |  |  |  |
|  |  |  |  |  |  |  |  |  |  | CSI-RS/IM | N/A |  |  |  |  |  |  |  |
| Short TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RC.26 FDD |  | FDD |  | 10 |  | 50 |  | - |  | - | MCS.32-1 MCS.32-2 |  | 8 |  | 1 |  | Note 2 |  |
| RC.27 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.33-1 MCS.33-2 |  | 8 |  | 1 |  | Note 2 |  |
|  |  | FDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.34-1 MCS.34-2 |  | 8 |  | 1 |  | Note 2 |  |
| RC.28 FDD |  | FDD |  | 10 |  | 50 |  | - |  | - | MCS.35-1 MCS.35-2 MCS.35-3 MCS.35-4 MCS.35-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 18 |  |
| RC.29 FDD |  | FDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.36-1 MCS.36-2 MCS.36-3 MCS.36-4 MCS.36-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 19 |  |
|  |  | FDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.37-1 MCS.37-2 MCS.37-3 MCS.37-4 MCS.37-5 |  | 16 |  | 1 |  | Note 2 Note 17 Note 19 |  |
| RC.26 TDD |  | TDD |  | 10 |  | 50 |  | - |  | - | MCS.32-1 MCS.32-2 |  | 10 |  | 1 |  | Note 5 |  |
| RC.27 TDD |  | TDD |  | 10 |  | 50 |  | - |  | Non CSI-RS | MCS.33-1 MCS.33-2 |  | 16 |  | 1 |  | Note 5 |  |
|  |  | TDD |  | 10 |  | 50 |  | - |  | 2 CSI-RS | MCS.34-1 MCS.34-2 |  | 16 |  | 1 |  | Note 5 |  |
| Note 1: 3 symbols allocated to PDCCH. Note 2: For FDD only subframes 1, 2, 3, 4, 6, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 3: TDD UL-DL configuration as specified in the individual tests. Note 4: For TDD when UL-DL configuration 1 is used only subframes 4 and 9 are allocated to avoide PBCH and synchronizaiton signal overhead. Note 5: For TDD when UL-DL configuration 2 is used only subframes 3, 4, 8, and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 6: Centered within the Transmission Bandwidth Configuration (Figure 5.6-1). Note 7: Only subframes 2, 3, 4, 7, 8 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 8: Allocate PDSCH on 5th and 6th PRBs within a subband. Note 9: The number of HARQ processes is 10 for TDD UL/DL configuration 2 and 7 for TDD UL/DL configuration 1. Note 10: The downlink subframes are scheduled at the 1st, 2nd, 8th, 9th, 16th, 17th, 18th, 24th, 26th, 32nd, 33rd, 34th subframes every 40ms. Information bit payload is available if downlink subframe is scheduled.(starting from 0th subframe) Note 11: 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in subframe 0 and 5 in RC.19 TDD. Note 12: Allocate PDSCH on 3th, 4th and 5th PRBs within a narrowband. Allocate MPDCCH on the 0th and 1st PRBs within a narrowband. Note 13: The PDSCH subframes are scheduled at the 0th and 1st subframes every 10ms. Information bit payload is available if downlink subframe is scheduled (starting from 0th subframe). MPDCCH subframes are scheduled at the 8th and 9th subframes every 10ms. Note 14: The downlink subframes are scheduled at the 0th to 4th subframes every 20ms. Information bit payload is scheduled at the 4th subframe (starting from 0th subframe). MPDCCH and Information bit payload are not scheduled in the radio frames where systemInformation1-BR is scheduled and = 4 with the set of frames and subframes for SIB1-BR defined in TS 36.211 [16] Table 6.4.1-2. Note 15: Information bit payload is scheduled at the 8th subframe every 20ms (starting from 0th subframe). Note 16: 2 symbols allocated for PDCCH. Note 17: No PDSCH is scheduled in subslot index 0. Note 18: Subslot-PDSCH is scheduled in subslots 2, 3, and 4. Note 19: Subslot-PDSCH is scheduled in subslots 1 and 5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table A.4-19: Mapping of CQI Index to Modulation coding scheme (4-bit CQI Table 5)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0391 | 0.0762 | 0.1172 | 0.1885 | 0.3008 | 0.4385 | 0.5879 | 0.3691 | 0.4785 | 0.6016 | 0.4551 | 0.5337 | 0.6504 | 0.7439 | 0.8525 |  |
| Modulation |  |  | OOR |  |  |  | QPSK |  |  |  | 16QAM |  |  |  | 64QAM |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.40 | 1 | 126 | DTX | 0 | 0 | 0 | 1 | 4 | 5 | 8 | 12 | 14 | 16 | 18 | 21 | 23 | 25 | 27 |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4-20: Mapping of CQI Index to Modulation coding scheme (4-bit CQI Table 6)

| CQI Index |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Target Coding Rate |  |  | OOR | 0.0547 | 0.2021 | 0.2598 | 0.1904 | 0.1387 | 0.2598 | 0.4424 | 0.6221 | 0.4131 | 0.5439 | 0.6797 | 0.8252 | 0.6357 | 0.7617 | 0.8672 |  |
| Modulation |  |  | OOR |  |  |  | QPSK |  |  |  | 16QAM |  |  |  | 64QAM |  |  |  |  |
| MCS Scheme | PRB | Available RE-s | Imcs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MCS.41 | 3 | 378 | DTX | 0 | 2 | 3 | 2 | 1 | 3 | 7 | 9 | 12 | 15 | 16 | 16 | 22 | 25 | 27 |  |
| Note 1: Mapping between Imcs and TBS according to Tables 7.1.7.1-1 and 7.1.7.2.1-1 in TS 36.213 [6]. Note 2: startSymbolBR = 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------