3GPP TSG-RAN WG4 Meeting #90 	R4-1902019

# Athens, Greece, 25th February – 1st March, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6319 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenace CR on WUS requirements for cat-M UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  | Date: |  |  | 2019-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | WUS repetition levels are TBD in current specification in section 4.7.2.3 in TS 36.133. This CR contains changes to replace TBDs with actual values based on our simulation results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: TBD values are replaced with actual repetition levels |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification as it is not known how many reptetitions are needed for WUS receptions for cat-M. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Start of Change 1 ---

#### 4.7.2.3	WUS receptions for UE category M1

This clause contains requirements on the UE regarding WUS reception provided that WUS has been configured in the serving cell.

The UE shall be capable of receiving the WUS signals of the serving cell provided that the minimum number of repetitions configured in the serving cell is according to Table 4.7.2.3-1 for normal coverage and Table 4.7.2.3-2 for enhanced coverage.

Table 4.7.2.3-1: Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 1.28 | 32 | 4 |
| > 1.28 | 64 | 4 |

Table 4.7.2.3-2: Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| ≤ 1.28 | 128 | 32 |
| > 1.28 | 256 | 64 |

--- End of Change 1 ---