3GPP TSG RAN WG1 Meeting #96	R1-1903307

# Athens, Greece, 25 February – 1 March 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1232 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the 1ms HARQ-ACK transmission for the collision between sPUSCH and PUCCH/PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_sTTIandPT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE behaviour under the sPUSCH and 1ms HARQ-ACK collision case for fixed codebook size configuration has not been captured in spec, as in following agreements in RAN1#92 meeting: Agreement: Update the previous agreement as per below: UE behaviour in case of sPUSCH transmission when simultaneous transmission of sPUCCH and sPUSCH is not configured and in case fixed codebook size is configured: - PDSCH assignment detected and sPDSCH assignment not detected, and it is the first transmitted UL sTTI in the subframe - no sHARQ-ACK bits on sPUSCH + all HARQ-ACK bits for 1ms TTI on sPUSCH - PDSCH assignment detected and sPDSCH assignement detected - include all HARQ-ACK bits for 1ms TTI + sHARQ-ACK bits on sPUSCH - PDSCH assignment not detected and sPDSCH assignement detected, and it is the first transmitted UL sTTI in the subframe - include all HARQ-ACK bits for 1ms TTI + sHARQ-ACK bits on sPUSCH - PDSCH assignment not detected and sPDSCH assignment not detected - no HARQ bits on sPUSCH. |
| Summary of change: |  | Capture the UE behaviour under the collision between sPUSCH and 1ms HARQ-ACK for fixed codebook size configuration according to the #92 meeting agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior under the sPUSCH and 1ms HARQ-ACK collision case for fixed codebook size configuration is not captured in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

7.3	UE procedure for reporting HARQ-ACK

< Unchanged parts are omitted >

If the UE is configured with higher layer parameter shortTTI, and the UE transmits slot/subslot-PUSCH on slot/subslot s of subframe n without associated PDCCH/SPDCCH or with associated PDCCH/SPDCCH with DCI format 7-0A/7-0B and if the UE is not configured with simultaneous PUSCH and PUCCH transmission,

-	the UE shall transmit the HARQ-ACK corresponding to subframe-PDSCH on the slot/subslot-PUSCH if the UE has received subframe-PDSCH on subframe of a serving cell and if the slot/subslot-PUSCH is the first occurrence of the slot/subslot-PUCCH/PUSCH in the subframe;

-	if the UE is configured with no more than five serving cells or if the UE is configured with higher layer parameter codebooksizeDetermination-r13 = cc, the UE shall transmit the HARQ-ACK corresponding to subframe-PDSCH for all serving cells on the slot/subslot-PUSCH if the UE has received slot/subslot-PDSCH without an associated PDCCH/SPDCCH or with an associated PDCCH/SPDCCH with DCI format 7-1A/7-1B/7-1C/7-1D/7-1E/7-1F/7-1G for which HARQ-ACK response shall be provided on slot/subslot s of subframe n, and

-	if the UE has received subframe-PDSCH without an associated PDCCH/EPDCCH or with an associated PDCCH/EPDCCH with DCI format 1/1A/1B/1D/2/2A/2B/2C/2D for at least one of the serving cell(s) on subframe , or

-	if the UE has not received subframe-PDSCH without an associated PDCCH/EPDCCH or with an associated PDCCH/EPDCCH with DCI format 1/1A/1B/1D/2/2A/2B/2C/2D for any of the serving cell(s) on subframe , and if the slot/subslot-PUSCH is the first occurrence of the slot/subslot-PUCCH/PUSCH in the subframe;

-	the corresponding HARQ-ACK bit(s) is NACK, if the UE has not received subframe-PDSCH without an associated PDCCH/SPDCCH or with an associated PDCCH/EPDCCH with DCI format 1/1A/1B/1D/2/2A/2B/2C/2D for a serving cell(s) on subframe ;

and

-	 for FDD, and  for TDD where  is defined in Table 10.1.3.1-1B if the UE is configured with higher layer parameter shortProcessingTime and the corresponding PDCCH with CRC scrambled by C-RNTI is in the UE-specific search space for subframe-PDSCH, and in Table 10.1.3.1-1 otherwise.

-	spatial bundling of the HARQ-ACK corresponding to subframe-PDSCH is applied if the HARQ-ACK is to be transmitted on

-	subslot s or

-	slot s and spatialBundlingPUCCH is set TRUE

-	the sequence of HARQ-ACK bit(s) corresponding to subframe-PDSCH  is determined in Subclause 5.2.2.6 of [4].

< Unchanged parts are omitted >