3GPP TSG-RAN WG1 Meeting #96 	R1-1903282

# Athens, Greece February 25th – 1st March 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0477 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DMRS sequence index for PUSCH sub-PRB allocation for LTE-MTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Core |  |  |  |  |  |  | Date: |  | 2019-02-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect index range of the DMRS base sequence for a 3 subcarrier allocation using a modulation scheme other than π/2-BPSK |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 5.5.2.1A.1-1 the range of n in was corrected to go from 0 to 2 instead of 0 to 3 which is out of range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DMRS base sequence for a 3 subcarrier allocation using a modulation scheme other than π/2-BPSK will be out of range. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.1A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.2.1A	Demodulation reference signal for PUSCH with sub-PRB allocations

##### 5.5.2.1A.1	Reference signal sequence using modulation schemes other than π/2-BPSK

The reference signal sequence  for  is defined by a cyclic shift  of a base sequence according to

,

where  is given by Tables 5.5.2.1A.1-1 and 5.5.2.1A.1-2 for and , respectively. The cyclic shift  is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 5.5.2.1A.1-3.

If group hopping is enabled, the base sequence index  is given by clause 5.5.2.1A.3.

If group hopping is not enabled, the base sequence index  is given by

-	 for

-	 for

Table 5.5.2.1A.1-1: Definition of  for

|  | ∅0, … ,∅2 |  |  |
| --- | --- | --- | --- |
| 0 | 1 | -3 | -3 |
| 1 | 1 | -3 | -1 |
| 2 | 1 | -3 | 3 |
| 3 | 1 | -1 | -1 |
| 4 | 1 | -1 | 1 |
| 5 | 1 | -1 | 3 |
| 6 | 1 | 1 | -3 |
| 7 | 1 | 1 | -1 |
| 8 | 1 | 1 | 3 |
| 9 | 1 | 3 | -1 |
| 10 | 1 | 3 | 1 |
| 11 | 1 | 3 | 3 |

Table 5.5.2.1A.1-2: Definition of  for

| 0 | 1 | 1 | 1 | 1 | 3 | -3 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 3 | 1 | -3 | 3 |
| 2 | 1 | -1 | -1 | -1 | 1 | -3 |
| 3 | 1 | -1 | 3 | -3 | -1 | -1 |
| 4 | 1 | 3 | 1 | -1 | -1 | 3 |
| 5 | 1 | -3 | -3 | 1 | 3 | 1 |
| 6 | -1 | -1 | 1 | -3 | -3 | -1 |
| 7 | -1 | -1 | -1 | 3 | -3 | -1 |
| 8 | 3 | -1 | 1 | -3 | -3 | 3 |
| 9 | 3 | -1 | 3 | -3 | -1 | 1 |
| 10 | 3 | -3 | 3 | -1 | 3 | 3 |
| 11 | -3 | 1 | 3 | 1 | -3 | -1 |
| 12 | -3 | 1 | -3 | 3 | -3 | -1 |
| 13 | -3 | 3 | -3 | 1 | 1 | -3 |

Table 5.5.2.1A.1-3: Definition of

| threeTone-CyclicShift |  | sixTone-CyclicShift |  |
| --- | --- | --- | --- |
| 0 |  | 0 |  |
| 1 |  | 1 |  |
| 2 |  | 2 |  |
| - | - | 3 |  |