3GPP TSG-TSG RAN WG1 Meeting # 96	 R1-1903263

# Athens, Greece, February 25th – March 1st, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1212 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Priority rules for SRS carrier switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SRS_switch-Core |  |  |  |  |  | Date: |  |  | 2019-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#96 meeting, we have following agreements for priority rules of SRS carrier switching: - If PUCCH/PUSCH containing a CSI report with PUCCH reporting type 2a on a serving cell in S(d) overlaps in the same symbol with the SRS transmission on serving cell d, then the UE shall not transmit SRS |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE with SRS carrier switching drops SRS if colliding with PUCCH/PUSCH containing a CSI report with PUCCH reporting type2a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The priority rule for PUCCH reporting type2a is inconsistant for SRS-PUSCH collision and PUSCH-PUSCH collision, resulting in different UE behavors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.2	UE sounding procedure

< Unchanged parts are omitted >

For a TDD serving cell d not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding serving cell whose UL transmissions may be interrupted as signalled by srs-SwitchFromServCellIndex. Define the set S(d)= {s0(d)… sN-1(d)} as the set of serving cells that meet the all the following conditions:

-	{s0(d)… sN-1(d)} are in the same band as s0(d).

-	{s0(d)… sN-1(d)} have the same CP as s0(d).

-	{s0(d)… sN-1(d)} are in the same TAG as s0(d).

The following prioritization rules shall be applied in case of collision between a transmission of SRS over serving cell d and transmission of a physical signal/channel over a serving cell in set S(d):

-	If PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/PTI/CRI/wideband PMI only (PUCCH reporting type 2a in Clause 7.2.2) and/or PRACH on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, then the UE shall not transmit SRS. Otherwise,

-	if PUSCH transmission carrying aperiodic CSI on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, and if the SRS transmission is a type 0 SRS transmission, then the UE shall not transmit the type 0 SRS. Otherwise,

-	if PUSCH transmission on a serving cell in set S(d) overlaps in more than one symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) in serving cell d, then the UE shall drop the PUSCH transmission. If PUCCH/SRS transmission on a serving cell in set S(d) overlaps in the same symbol with the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell d, the UE shall drop the PUCCH/SRS transmission.

In case an SRS transmission in subframe N on serving cell d is dropped due to a collision with a higher priority transmission (as defined above) in subframe N+1, and there is a lower priority transmission (as defined above) in subframe N that would have been dropped had the transmission in subframe N+1 not occurred, the UE is not required to transmit the lower priority transmission in subframe N.

< Unchanged parts are omitted >