3GPP TSG-TSG RAN WG1 Meeting # 96	 R1-1903266

# Athens, Greece, February 25th – March 1st, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1214 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of 6-symbol SRS in UpPTS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SRS_switch-Core |  |  |  |  |  | Date: |  |  | 2019-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN1#87 meeting, we have following agreements for SRS carrier switching: - Through RRC configuration, allow SRS transmissions on more than 2 UpPTS symbols in the same subframe for the same UE, for A-SRS and P-SRS - On the same CC or multiple CCs - Allow more than one SRS configurations for the subframe, for A-SRS and P-SRS - Reuse existing SRS configurations (1 or 2 UpPTS symbols per SRS configuration) - Clarification: - Up to 6 symbols can be used for SRS on the same CC in one UpPTS according to RRC configuration However, it is not specified that the multiple SRS symbols by using more than on SRS configuration are in one UpPTS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE is not expected to receive more than one SRS configuration indicating SRS symbols not fully in UpPTS for SRS carrier switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreement is not correctly captured and the UE behavior is not clear when more than one SRS configuration indicate the symbols partially in UpPTS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.2	UE sounding procedure

< Unchanged parts are omitted >

For a TDD serving cell,

-	If the serving cell not configured for PUSCH/PUCCH transmission, or if the UE supports ce-srsEnhancement-r14

-	For trigger type 0, the UE can be configured with more than one configuration of SoundingRS-UL-ConfigDedicatedUpPTsExt and/or SoundingRS-UL-ConfigDedicated, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	For trigger type 1, the UE can be configured with more than one configuration of SoundingRS-UL- ConfigDedicatedAperiodicUpPTsExt and/or SoundingRS-UL- ConfigDedicatedAperiodic, and the SRS parameters in each of the configurations shall be used. The UE is not expected to receive more than one SRS configuration indicating SRS transmission on multiple symbols in different subframes within a half frame.

-	Otherwise

-	For trigger type 0, if SoundingRS-UL-ConfigDedicatedUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used.

-	For trigger type 1, if SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicatedAperiodic shall be used.

< Unchanged parts are omitted >