3GPP TSG RAN WG1 Meeting #96	R1-1903785

# Athens, Greece, February 25 – March 1, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0008 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on physical resource mapping for PUSCH with configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The number of DM-RS symbols determination for configured grant when higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 'len2' is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “or configured grant configurations” for the determination of number of DM-RS symbols for PUSCH transmission with configured grant. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot determine the number of DM-RS symbols when higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 'len2' and the PUSCH is transmitted using configured grant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has isolated impact. This CR impacts PUSCH transmission with configured grant. Inter-operability analysis: - If the UE is implemented in accordance to this CR and the gNB is not, there would be an issue that gNB may determine the number of DM-RS for PUSCH tranmission different from a UE. - If the gNB is implemented in accordance to this CR and the UE is not, there would be an issue that the UE may not be able to determine the number of DM-RS for PUSCH tranmission with configured grant since there may not be an associated DCI. |  |  |  |  |  |  |  |  |

##### 6.4.1.1.3	Precoding and mapping to physical resources

The sequence  shall be mapped to the intermediate quantity ak,l(pj,μ) according to

-	if transform precoding is not enabled,

-	if transform precoding is enabled

where , , and Δ are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2 and the configuration type is given by the higher-layer parameter DMRS-UplinkConfig, and both k' and Δ correspond to p0, …, pν-1.  The intermediate quantity ak,l(po,μ)=0  if either k' or Δ does not correspond to p0, …, pν-1.

The intermediate quantity ak,l(pj,μ) shall be precoded, multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [6, TS 38.214], and mapped to physical resources according to

where

-	the precoding matrix  is given by clause 6.3.1.5,

-	the set of antenna ports  is given by clause 6.3.1.5, and

-	the set of antenna ports  is given by [6, TS 38.214];

and the following conditions are fulfilled:

-	the resource elements ak,l(pj,μ) are within the common resource blocks allocated for PUSCH transmission.

The reference point for  is

-	subcarrier 0 in common resource block 0 if transform precoding is not enabled, and

-	subcarrier 0 of the lowest-numbered resource block of the scheduled PUSCH allocation if transform precoding is enabled.

The reference point for  and the position  of the first DM-RS symbol depends on the mapping type:

-	for PUSCH mapping type A:

-	 is defined relative to the start of the slot if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-	 is given by the higher-layer parameter dmrs-TypeA-Position

-	for PUSCH mapping type B:

-	 is defined relative to the start of the scheduled PUSCH resources if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled

-

The position(s) of the DM-RS symbols is given by  and duration ld where

-	ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or

-	ld is the duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if intra-slot frequency hopping is not used, or

-	ld is the duration per hop according to Table 6.4.1.1.3-6 if intra-slot frequency hopping is used.

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is not configured, the tables shall be used according to single-symbol DM-RS

-	if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 'len2', the associated DCI or configured grant configuration determines whether single-symbol or double-symbol DM-RS shall be used

-	if the higher-layer parameter dmrs-AdditionalPosition is not set to 'pos0' and intra-slot frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212] and by higher layer, Tables 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is equal to 'pos1' for each hop.

For PUSCH mapping type A, the case dmrs-AdditionalPosition equal to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2'. For PUSCH mapping type A, ld=4 symbols in Table 6.4.1.1.3-4 is only applicable when dmrs-TypeA-Position is equal to 'pos2'.

The time-domain index  and the supported antenna ports  are given by Table 6.4.1.1.3-5.