3GPP TSG RAN meeting #84										RP-190845

Newport Beach, USA, June 3-6, 2019

## Status Report to TSG

Agenda item:			10.2.2

| WI / SI Name | Additional enhancements for NB-IoT |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NB_IOTenh3 |  |  |  |  |
| Unique ID | 800084 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190757 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 | Performance part: 09/2020 (changed) | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 50% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | COZZO, Carmela |
|  | Company | Futurewei |
|  | Email | ccozzo@futurewei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The completion of the performance part is changed from 06/2020 to 09/2020 to allow a period of 6 months between the completion date of core part and performance part.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

Contributions [1] – [77] were submitted to RAN1#96bis meeting. A summary of the RAN1 agreements was submitted after RAN1#96bis meeting in R1-1905771.

Contributions [78] – [144] were submitted to RAN1#97 meeting.

#### 2.1.1	Agreements

UE-group wake-up signal

RAN1#96bis

Agreement

Up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not.

Agreement

Group WUS location in relation to legacy WUS may be configured such that:

- If one group WUS resource is configured, that group WUS resource may be configured to coincide with the legacy WUS resource or to occur immediately before the legacy WUS resource, and,

- If two group WUS resources are configured, the first group WUS resource coincides with the legacy WUS resource and the second group WUS resource occurs immediately before the first group WUS resource.

Agreement

Group WUS sequence design is down selected between the following in RAN1#97

- Legacy WUS with shifted scrambling codes

- Legacy WUS with time-frequency short OCCs

- Phase shifted legacy WUS using a suitable subset of the phase increments.

- Legacy WUS with Gold codes

Companies are encouraged to provide evaluation results

Agreement

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

RAN1#97

Working Assumption

At least for the group WUS in the same WUS resource, legacy WUS with phase shifts is selected as group WUS sequence design according to wgroup(m’) = w(m’) exp(j2πgm/G)

- FFS: Details of g and G

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence.

Working Assumption

The UE may assume that the transmit power for Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

Working Assumption

The maximum WUS duration for the Rel-16 WUS sequence is the same as that of the Rel-15 WUS sequence.

Agreement

Each UE monitors up to X WUS sequences.

- Value of X will be selected between 2 and 3

  - X=3 can only be supported if a common WUS for a subset of UE groups for service-based grouping is supported

Support for transmission in preconfigured UL resources

RAN1#96bis

Agreement

In idle mode, a UE can be configured such that TA is always valid within a given cell.

- Up to RAN2 how to implement

  - e.g. PUR Time Alignment Timer or NRSRP Threshold = infinity

Agreement

The value(s) of NRSRP threshold(s) is UE specific

Agreement

The UE monitors the NPDCCH for at least a time period after a PUR transmission.

- FFS: Details of the time period

- FFS: UE behaviour if nothing is received in that time period.

- FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted

Agreement

Reuse existing field(s) of DCI format N0 to convey the dedicated PUR ACK

Agreement

After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH. Other behaviors are FFS.

Working Assumption#1

In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

- FFS: Which PUR configurations and PUR parameters will be signaled via L1

- FFS: Definition of PUR configurations and PUR parameters

The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

Working Assumption#2

For dedicated PUR

- During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

- FFS if the UE monitors any additional RNTI which may be shared with other UEs.

- Note: The same RNTI may be used over non-overlapping time and/or frequency resources

Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in R1-1905570)

Agreement

For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

RAN1#97

Agreement

For dedicated PUR in idle mode and for HD-FDD UEs, the start of the PUR SS Window is [x] subframes after the end PUR transmission

- FFS: Value of x, and if x is fixed or signaled

- FFS: Support for monitoring of PUR SS Window before PUR transmission

Note: The PUR SS Window is the time period where the UE monitors the NPDCCH for at least a time period after a PUR transmission

Agreement

NPDCCH candidates are determined by USS like search space

- FFS: Other details on the USS like search space

  - Type2-CSS can also be discussed as part of the FFS

Conclusion

CBS PUR is not supported in Rel-16

For further discussion

- Aspects related to notifying eNB of unused PUR resources.

- Potential enhancements of power control mechanisms for PUR. (The baseline is the existing NB-IoT open loop power control.)

Scheduling of multiple DL/UL transport blocks

RAN1#96bis

Agreement

1 bit for RV indication in UL transmission is used regardless of the number of TBs

- Common RV indication is mapped to both TBs

Working Assumption

3 bits are used to indicate scheduled TB number, HARQ process index, NDI for HARQ operation

- FFS: Details coding scheme of these 3 bits

Conclusion

Relationship 2 is not supported in Rel-16.

Agreement

In case 2 TBs are scheduled in the downlink, the timing of the ACK/NACKs for the scheduled TBs is with respect to the last TB scheduled by the DCI, detailed value FFS.

- For the case of 1 TB scheduling, legacy UE behavior is maintained

Agreement

For SC-MTCH multiple TBs scheduling, select one from the three options in RAN1#97

- Modify existing DCI to indicate the number of scheduled TBs (e.g. by adding new field)

- Reuse Rel-15 DCI and use SC-MCCH to indicate TB numbers.

- Support both a) and b)

RAN1#97

Agreement

Confirm the working assumption last meeting:

3 bits are used to indicate scheduled TB number, HARQ process index, NDI for HARQ operation

  - FFS: Details coding scheme of these 3 bits

Agreement

The coding scheme of the three bits (scheduled TB number, HARQ process index, NDI for HARQ operation) is as follows:

| Codepoint | Description |
| --- | --- |
| 000 | Single TB scheduling, HARQ ID=0, NDI=0 |
| 001 | Single TB scheduling, HARQ ID=0, NDI=1 |
| 010 | Single TB scheduling, HARQ ID=1, NDI=0 |
| 011 | Single TB scheduling, HARQ ID=1, NDI=1 |
| 100 | Multi-TB scheduling, NDI=00 |
| 101 | Multi-TB scheduling, NDI=01 |
| 110 | Multi-TB scheduling, NDI=10 |
| 111 | Multi-TB scheduling, NDI=11 |

Table 1: Joint coding scheme of scheduled TB number, HARQ process index, NDI for HARQ operation

- Only one additional bit is added in the DCI format to support the above indication of 8 states

Note: How to capture the above agreement is up to the editor

Agreement

In case of non-interleaved transmission, for individual feedback of 2 TBs case, continuous uplink feedback starts, after the end of  DL subframe for FDD, where n is the ending subframe of last scheduled TB and  k0' is down-selected from the following two choices:

- The same value as the one for legacy one TB case (i.e. reuse the existing specification without change)

- New values are introduced which depends on the length of last TB and ACK/NACK resources

  - Existing values can also be used

Agreement

HARQ-ACK resource field is common across the HARQ-ACK feedback for all the DL TBs scheduled.

- ACK/NACK subcarriers are the same across all the TBs

Coexistence of NB-IoT with NR

RAN1#96bis

For further study until RAN1#97:

- RAN1 continues to study finer-granularity NB-IoT resource reservation until RAN1#97.

  - Assume that the NB-IoT resource reservation should accommodate at least part of NR initial CORESET, NR CSI-RS and NR TRS.

  - The reference case is the existing DL/UL subframe-level NB-IoT resource reservation for TDD NB-IoT. For FDD NB-IoT, the reference case is the existing DL subframe-level NB-IoT resource reservation.

  - Assume that the resource reservation is semi-statically configured. It is FFS whether the eNB can furthermore dynamically override or indicate additional reserved resource via DCI.

    - Companies are encouraged to consider the impact on UE complexity for DCI based indication

  - Consider whether the time-domain granularity of the NB-IoT valid subframe configuration should be scaled dependent on the NR subcarrier spacing.

  - Consider both DL and UL aspects.

    - NPDSCH, NPUSCH, NPDCCH

  - It is FFS whether and when to drop or postpone NB-IoT transmissions that would fall into NB-IoT reserved resources (if introduced).

Conclusion

RAN1 concludes that overlap between NR SSB and NB-IoT non-anchor carriers can be avoided using the NB-IoT DL valid subframe bitmap.

For further study until RAN1#97:

RAN1 continues to study the aspects of coexistence of NB-IoT and NR using different SCSs.

RAN1#97

Conclusion

Based on the study of aspects of FDD/TDD NB-IoT coexistence with NR, it is RAN1 recommendation to introduce DL/UL resource reservation in NB-IoT for at least FDD/TDD NB-IoT non-anchor carriers to avoid time and frequency domain resource overlap between NR and NB-IoT when NB-IoT is deployed within an NR carrier.

Resource reservation in FDD/TDD NB-IoT considers the following aspects:

- sub-frame/slot/symbol level granularity

- dropped/postponed resources associated to a certain granularity

- semi-static configuration is recommended. FFS: dynamic DCI signaling

- UL valid/invalid subframe configurations for FDD NB-IoT

Agreement

Include above conclusion as part of LS to RAN

The LS is endorsed in R1-1907635

Support of Quality report in Msg3 and Connected Mode

RAN1#96bis

Agreement

For channel quality report in connected mode, the channel quality definition is denoted by the number of repetitions that the UE needs to decode hypothetical NPDCCH with BLER of 1%.

Agreement

For channel quality report in connected mode other than Msg3, UE performs measurement on the carrier it is assigned to monitor in USS for NPDCCH and the associated NPDSCH.

- FFS other carrier(s)

Agreement

For channel quality report in Msg3 for non-anchor access, RAN1 send LS to RAN2 requesting the following information:

- Number(s) of available bits for the quality reporting in Msg3

- Whether the quality report in Msg3 is transmitted in RRC or MAC CE

Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases

The LS is endorsed in R1-1905593

Agreement

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is no larger than 2.

- FFS detail of the maximum number (1 or 2)

Agreement

eNB can enable/disable channel quality report in Msg3 for non-anchor access via SIB

- FFS details

- FFS whether the field in Rel-14 for anchor access is reused

RAN1#97

Agreement

For channel quality measurement in Msg3 for non-anchor access, the maximum number of carriers reported in Msg3 is 1.

Agreement

For channel quality report in Msg3 on non-anchor access in idle mode, channel quality measurement on the carrier UE monitors to receive Msg2 (i.e. RAR) is supported.

- FFS: Support of network indication of which carrier to use to report Msg3 (e.g. explicit signaling in SI)

Agreement

For channel quality report in connected mode other than Msg3, RAN1 assumes the following aspects are decided by RAN2 and RAN4.

- number of candidate values for channel quality reports

- mapping value of reported channel quality

- report in RRC message or MAC CE

LS to RAN2 and RAN4 to convey this agreement is endorsed in R1-1907624.

For further discussion

For channel quality report in connected mode other than Msg3, RAN1 further discuss whether physical layer triggering mechanism is needed.

For further discussion

RAN1 further discuss the followings for channel quality report in Msg3 on non-anchor access in connected mode

- The support of channel quality report in Msg3 in connected mode

  - If supported, which carrier can be measured for the channel quality report in Msg3

    - RAR carrier

    - unicast carrier

  - If supported, how to trigger the report in Msg3

Presence of NRS on a non-anchor carrier for paging

RAN1#96bis

Conclusion

There is no consensus in RAN1 to support configuration of NRS for WUS early termination in Rel-16.

Agreement

The subset of POs that have subframes with NRS even when no NPDCCH is transmitted is determined based at least on nB-r13/nB

- FFS: defaultPagingCycle-r13/T

- Note: The paging frame and the number of UE groups can be derived from T and nB

Agreement

For the NRS pattern: Define detailed pattern of (M,N) depending on the value of nB:

- If nB >= X, specify the decimation pattern/M/N for each nB.

  - FFS if the value of M/N is different for different POs within a DRX cycle

- For nB < X, all POs have NRS. FFS value of M/N.

where X is a fixed value which will be determined in RAN1#97.

RAN1#97

For further discussion

Companies are encouraged to provide detailed decimation patterns to RAN1#98

Agreement

With regards to the RAN1#96bis agreement:

- If nB >= X, specify the decimation pattern/M/N for each nB.

  - FFS if the value of M/N is different for different POs within a DRX cycle

- For nB < X, all POs have NRS. FFS value of M/N.

It is agreed that the value of X is T/2.

Agreement

For nB < X, the value of M+N is 10

- FFS: Whether the values of M and N are configurable

Agreement

For nB>=X, the decimation/N/M pattern guarantees that there are [10] subframes with NRS nearby every PO (after decimation pattern, and from network perspective)

Agreement

For the decimation pattern for nB>=X, RAN1 to decide for each nB value:

- M/N

- Decimation pattern

  - FFS: Offset (if used)

#### 2.1.2	Remaining Open issues

- Specify further details to support UE-group wake-up signal.

- Specify further details to support transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

- Specify further details for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast.

- Specify further details to support Msg3 quality reporting for non-anchor access and in connected mode.

- Specify further details of the decimation pattern/M/N on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

## 2.2	RAN2

Contributions [145] – [276] were submitted to RAN2#105bis meeting. A summary of the RAN2 agreements was submitted after RAN2#105bis meeting in R2-1905262.

Contributions [277] – [414] were submitted to RAN2#106 meeting. A summary of the RAN2 agreements was submitted after RAN2#106 meeting in R2-1908188.

#### 2.2.1	Agreements

Mobile-terminated (MT) early data transmission (EDT)

RAN2 discussed Mobile-terminated (MT) early data transmission (EDT), with the following agreements:

RAN2#105bis agreements: For both UP and CP solutions, an MT-EDT indication is needed in the S1 paging message to eNB. It is up to RAN3 to decide how such indication is provided to the eNB. RAN2 assumes that MME initiates MT-EDT. It is up to eNB to use MT-EDT based on e.g., UE capability. RAN2 assumes that DL data information is needed from S-GW/SCEF to the MME to assist MME to initiate MT-EDT. For the UP solution, the DL data are ciphered and sent over DTCH. For both UP and CP solutions eNB sends MT EDT indication to the UE via paging. For Msg-2 based solution (if agreed): A CF RACH resource is provided in the paging message to page the UE for MT-EDT. FFS whether/how security related concerns are addressed and how number of repetitions required and RNTI are provided. For Msg-4 based solution (if agreed): For UP solution, RRCConnectionResume is used in Msg4 in case UL transmission is expected in response. R2-1905276 LS on Mobile-terminated Early Data Transmission RAN2 LS out Rel-16 To:RAN3, SA2, CT1, and CT4. RAN2#106 agreements: DL data scheduled, i.e. DL grant, in paging message is excluded (Opt C). DL data scheduled in paging occasion is excluded (Opt D). DL data transmitted in Msg4 option B is excluded (Msg4-B). For Msg2-based solution (if agreed): No need to cipher the dedicated RACH resource provided in the paging message. RAN2 intends to introduce a mechanism to acknowledge that Msg2 was received by the intended UE. At least one dedicated RACH resource is provided in the paging message for MT-EDT. FFS what other parameters, e.g., CE level, subcarrier index, are needed to be provided. FFS: whether RNTI is provided in the paging message. For Msg4-based solution (if agreed): Working assumption: UE should be able to indicate in Msg3 if it intends to send data in the UL.

RAN2 agreed to have the following email discussions until next meeting:

[106#64][R16 NB-IoT/eMTC]  UP MT-EDT (Ericsson): To identify the issues in Msg2 and Msg4 based solutions

[106#65][R16 NB-IoT/eMTC]  CP MT-EDT (Intel): To identify the issues in Msg2 and Msg4 based solutions

UE-group wake-up signal (WUS)

RAN2 discussed UE-group wake-up signal (WUS), with the following agreements:

RAN2#105bis agreements: Additional grouping based on DRX/eDRX is not supported. Coverage based grouping is not supported. Additional grouping based on gap is not supported. - FFS whether number of groups can depend on gap duration. RAN2#106 agreements: WUS grouping based on paging probability is beneficial for improving false wake-up probability for UEs which are not frequently paged, but may increase false wake-up probability for UEs which are frequently paged. Send an LS to SA2, cc RAN3, CT1, to ask about the feasibility and impact for the MME to provide information on UE paging probability. R2-1908260 LS on assistance indication for WUS grouping LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core R2-1905261 To:SA2 Cc: CT1, R3

Transmission in preconfigured resources

RAN2 discussed transmission in preconfigured resources, with the following agreements:

RAN2#105bis agreements: From RAN2 point of view it is possible to configure TBS for D-PUR for both NB-IoT and eMTC up to the maximum supported based on the UE category and TBS capability. For UP the UE may transmit D-PUR release request/(re)configuration request when transmitting using D-PUR. FFS For CP. The following are FFS: - Whether a UE can have more than one parallel D-PUR configurations - Ability of the UE to request (indicate) a time offset - The range of values and limits of preconfigured D-PUR allocations. - Whether mechanisms for a UE to reject a D-PUR configuration including by explicitly or implicitly releasing are needed. RAN2#106 agreements: RRC response message needs to be supported by the UE and could be used in all cases. For some cases L1 signalling is sufficient to acknowledge, i.e. RRC response message is not needed. RAN2 assumes the L1 signalling for acknowledgement is sent only after the eNB determines there is no pending downlink data or signalling. RAN2 assumes that the configuration for D-PUR provided by RRC signalling is not updated via L1 signalling, and should check with RAN1 what parameters are planned to be (re)configured using DCI. It is feasible to provide the UE with a UE-specific RNTI for D-PUR. Common or shared RNTI is also feasible. The RNTI used for D-PUR is signalled together with other D-PUR configuration. R2-1908196 Reply LS on PUR Working Assumptions for NB-IoT and LTE-M LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:RAN1.

RAN2 agreed to have the following email discussions until next meeting:

[106#59][R16 NB-IoT/eMTC] D-PUR Procedural steps (Huawei)

[106#60][R16 NB-IoT/eMTC] D-PUR TA validation criteria (Ericsson)

[106#61][R16 NB-IoT/eMTC] D-PUR Request, (re)configuration and release mechanism (Qualcomm)

Scheduling multiple DL/UL transport blocks

RAN2 discussed scheduling multiple DL/UL transport blocks, with the following agreements:

RAN2#105bis agreements: None. RAN2#106 agreements: Multi-TB scheduling for Msg4 during EDT (and MT-EDT) is not considered in Rel-16. For multi-TB scheduling, when HARQ ack bundling is configured, HARQ RTT timer is not started until the last TB has been received. FFS: For multi-TB scheduling for both LTE-M and NB-IoT, how to handle drx-InactivityTimer when HARQ Ack bundling is configured. UE capability for multiple TB is indicated separately for CE Mode A and CE mode B. UE capability for multiple TB is indicated separately for uplink and downlink. Wait for RAN1 before further discussion on multiple TB support for SC-PTM.

Network management tool enhancement

RAN2 discussed network management tool enhancement, with the following agreements:

RAN2#105bis agreements: - RACH report: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. At least RACH report for NB-IoT based on UE information request/response procedure is supported. It is up to the network to decide when to request for the RACH report. RAN2#106 agreements: - SON-ANR: ANR report does not trigger EDT. ANR report is not carried in EDT Msg3. anr-InfoAvailable indication can be reported in EDT Msg3. ANR procedure steps: - 1) UE receives “measurement” configuration in dedicated signalling. - 2) When the UE is released to idle - The UE performs the instructed measurements according to existing RAN4 cell reselection measurement performance requirements. - CGI reading is limited to cells being above a certain threshold. Threshold details FFS. - 3) Reporting takes place, upon network request, next time UE is connected mode again. - RACH report: Not support EDT RACH report in EDT Msg3.

RAN2 agreed to have the following email discussions until next meeting:

[106#62][R16 NB-IoT] ANR procedure, configuration and report details (Huawei)

Improved multi-carrier operation

RAN2 discussed improved multi-carrier operation, with the following agreements:

RAN2#105bis agreements: - Channel Quality Reporting: Existing Msg3 signalling is used for reporting downlink channel quality when UE makes access on non-anchor carrier. Existing SIB2 signalling is used to enable UE to report downlink channel quality on non-anchor carrier. The DL channel quality reported in connected mode corresponds to the carrier used for the unicast transmission (i.e. configured by MSG4 or by a subsequent reconfiguration procedure). eNB enables the reporting of the DL Channel quality in connected mode. - FFS whether dedicated or broadcast signaling is used for enabling. Periodic reporting or on-demand reporting are not supported. DL channel quality reported in connected mode is optional for the UE. Working assumption that the same code points as Msg3 reporting is used. FFS whether MAC or RRC is used for reporting. R2-1905263 LS on non-anchor carrier CQI reporting in MSG3 Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN1 Cc: RAN4 - Non-anchor carrier measurements for RRM: If RAN4 confirms feasibility, and NRS is present in non-anchor carrier, RAN2 can make use of that for RRM measurements in non-anchor carrier. R2-1905264 LS on non-anchor carrier idle mode measurements for RRM Qualcomm Incorporated LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc: RAN1 RAN2#106 agreements: - Channel Quality Reporting: The DL channel quality of the configured carrier in RRC connected mode is reported by MAC CE. Use the same LCID as agreed for eMTC. We need to wait for RAN4 before deciding when to report. FFS whether broadcast or dedicated signalling is used for NW to enable the reporting. R2-1908262 LS on DL channel quality report for configured carrier in RRC connected mode LS out Rel-16 NB_IOTenh3-Core To:RAN4 Cc:RAN1

Inter-RAT cell selection

RAN2 discussed Inter-RAT cell selection with no agreements.

RAN2 agreed to have the following email discussions until next meeting:

[106#63][R16 NB-IoT] Discussion of signalling aspects for inter-RAT cell selection assistance. (Nokia)

Coexistence with NR

RAN2 did not discuss coexistence with NR.

Connection to 5GC

RAN2 discussed connection to 5GC, with the following agreements:

RAN2#105bis agreements: Support of eDRX in CM-IDLE and EDT: Working assumption: Introduce a new IE “cp-EDT-5GC-r16” in SIB2-BR/SIB2-NB to indicate ng-eNB connected to 5GC supports CP-EDT optimization. Rel-15 eMTC/NB-IoT EDT PRACH Configuration parameters can be commonly used for both EPC and 5GC. Use critical extension for RRCEarlyDataRequest/RRCEarlyDataRequest-NB messages for CP-EDT eMTC/NB-IoT UEs connecting to 5GC. Introduce 48-bit 5G S-TMSI in critical extension of RRCEarlyDataRequest /RRCEarlyDataRequest-NB messages. For eMTC, in critical extension of RRCEarlyDataRequest message, add establishment cause of mo-Data. For NB-IoT, in critical extension of RRCEarlyDataRequest-NB message, add establishment cause of mo-Data and mo-ExceptionData. Working assumption: For eMTC/NB-IoT, delayTolerantAccess is not applicable for 5GC. Update stage 2 spec to capture that idle mode eDRX feature is supported for eMTC and NB-IoT connected to 5GC. For eMTC, hyperSFN in SIB1-BR is commonly used for both EPC and 5GC. For NB-IoT, hyperSFN-LSB in MIB-NB and hyperSFN-MSB in SIB1-NB are commonly used for both EPC and 5GC. For eMTC connected to 5GC, introduce a flag in SIB1-BR to indicate if idle mode eDRX is allowed in the cell. Working assumption: No indication of support for idle mode eDRX is needed for NB-IoT connected to 5GC. For eMTC and NB-IoT, systemInfoModification-eDRX is commonly used in paging messages for both EPC and 5GC. Use 5G S-TMSI as input for Hash ID to calculate PH and PTW_start. Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_CONNECTED Mode extended long DRX cycle, e.g. 5.12 and 10.24 sec, is supported for eMTC connected to 5GC. FFS whether new optional UE capability is required. RRC_CONNECTED Mode long DRX cycle, e.g., 5.12 and 10.24 sec, is supported for NB-IoT connected to 5GC. Mandatory to support without new UE capability. For eMTC, eDRX-Config-CycleStartOffset and drx-StartOffset are commonly used for both EPC and 5GC. For NB-IoT, DRX-Cycle and drx-StartOffset are commonly used for both EPC and 5GC. Indication of supported CIoT features and other common aspects For NB-IoT: SIB1-NB extended to include 5GC PLMN list, per PLMN indication of Cell Reserved for Operator Use, common 5GC Tracking Area Code and 5GC Cell Identity across all PLMNs, common Cell Barring for 5GC connectivity across all PLMNs. For NB-IoT/eMTC: SIB-NB/SIB1-BR extended to include per PLMN indication of the supported CIoT 5GS Optimisations. For NB-IoT: Update Paging-NB message to include 5G S-TMSI as UE Identity for core network paging. For NB-IoT: Adopt critical extension of RRCConnectionRequest-NB message for 5GC connectivity and include 48 bit 5G S-TMSI and random value as Initial UE identity along with specific RRC establishment cause. For NB-IoT: Update RRCConnectionSetupComplete-NB to include RegisteredAMF-r15, full 5G S-TMSI (48 bit long). For NB-IoT/eMTC: FFS :Whether s-NSSAI-list-r15 is applicable. For NB-IoT/eMTC: FFS: Applicability of NR PDCP for SRB1, DRBs For eMTC: UAC feature is supported. For NB-IoT: FFS how to support access control. For NB-IoT: FFS how to support slicing. For NB-IoT: FFS whether to adopt SDAP as user plane protocol, and whether AS reflective QoS is applicable. For eMTC: Adopt SDAP as user plane protocol, and AS reflective QoS is optional. For NB-IoT/eMTC: Working assumption that CN type is not used in RRCConnectionRelease/ RRCConnectionRelease-NB. For NB-IoT: How to support RRC Connection Re-establishment for the UP and CP optimization. For NB-IoT/eMTC: FFS whether for data transfer through DRBs, use AS security algorithms same as LTE AS security algorithms, and KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC: FFS whether AS security algorithms are identified by using LTE code points. For eMTC: mt-Access, mo-Signaling, mo-Data, emergency, highPriorityAccess and mo-VoiceCall are applicable establishment causes. For NB-IoT/eMTC: Release cause loadBalancingTAURequired at RRC connection release is not applicable. RAN2#106 agreements: Support of RRC_INACTIVE and eDRX in CM-CONNECTED: RRC_INACTIVE state in NB-IoT connected to 5GC is not supported. RRC_INACTIVE state with short eDRX cycles is optionally supported for eMTC connected to 5GC with capability signalling. FFS if EDT in RRC_INACTIVE state is not supported in Rel-16. UP optimization solution is supported for both eMTC and NB-IoT connected to 5GC with capability signalling. eLTE is assumed to be the baseline for RRC_INACTIVE state for eMTC UEs connected to 5G. Working assumption: Same cell selection/reselection criteria and measurement rules for BL UEs and non-BL UEs in CE in RRC_IDLE when attached to EPC are applied to BL UEs and non-BL UEs in CE in RRC_INACTIVE or RRC_IDLE when attached to 5GC. For transmission over DRB, NB-IoT/eMTC UE connected to 5GC uses the same AS security algorithms as eLTE regardless of PDCP type. KeNB root key is derived from Kamf as specified in TS 33.501. For NB-IoT/eMTC UE connected to 5GC, AS security algorithms are indicated by using LTE code points, e.g., EIA, EEA. R2-1908202 Reply LS on Conclusion on 5G CIoT user plane small data solution LS out Rel-16 FS_CIoT_5G To:SA2, RAN3 Cc:RAN, SA, CT Indication of supported CIoT features and other common aspects: RAN2 agrees to send a LS to SA2, CC: CT1,RAN3 - ask how many slices NB-IoT/eMTC UEs can support simultaneously and whether S-NSSAI will be used by the ng-eNB in addition to the indications of the supported 5G CIoT optimisations for AMF selection. - ask about applicability 5GC QoS frame work for NB-IoT/5GC - ask how QoS mapping is handled during mobility between NB-IoT and other RATs. NB-IoT UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. eMTC UE AS forwards all CIoT feature support indications broadcast in the cell to NAS as assistance info for CN type selection by NAS. For NB-IoT, CN Type is not used in RRCConnectionRelease-NB message as NAS based redirection between EPC and 5GC is used for CIoT. eMTC re-uses the existing CN type in RRCConnectionRelease message For eMTC intra RAT connected mode mobility (for intra 5GC case), adopt R15 LTE connected to 5GC introduced changes for RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> intra5GC-r15 IE. For eMTC intra RAT connected mode mobility (for inter system mobility between EPC and 5GC), adopt R15 LTE connected to 5GC introduced MobilityFromEUTRACommand carrying RRCConnectionReconfiguration (including Mobility Control Info) including SecurityConfigHO-v1530 -> fivegc-ToEPC-r15 and epc-To5GC-r15 IEs. For eMTC, re-use SIB25 and the procedure in subclause 5.3.16 of 36.331 as a baseline. Ask SA1: - whether UAC needs to be supported for NB-IoT. - whether a new UAC access category corresponding to mo-ExceptionData can be defined as one of the reserved standardised access categories in the range 9-31. - Whether up to 64 access categories need to be defined for support of UAC, given the currently supported establishment causes and necessity for operator defined categories and potential future extension - Whether access classes 0-9 (mapped to access identity 0) are applicable or not - Whether barring for access identities 1, 2, 11 to 15, is common for all access categories or independently controlled for different access categories. Use the same Enhanced Coverage authorisation mechanism of UEs connected to EPC to the NB-IoT/eMTC UEs connected to 5GC and no impacts on RAN2 specification are identified. R2-1908190 LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC LS out Rel-16 LTE_eMTC5-Core, NB_IOTenh3-Core To:SA2 Cc:CT1, RAN3. R2-1908263 LS on UAC for NB-IoT LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA1, CT1 Cc:SA2, RAN3 Other: RRC Connection re-establishment for the control plane for NB-IoT connected to 5GC is supported, assuming feasibility is confirmed. Send a LS to SA3, SA2 CC: RAN3, CT1, CT4 - Ask whether they can introduce RRC Connection re-establishment for the Control plane for NB-IoT connected to 5GC. - Ask the feasibility that a truncated 5G S TMSI (e.g. 40 bits) needs to be defined. R2-1908264 LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC LS out Rel-16 NB_IOTenh3-Core, 5G_CIoT To:SA3, SA2 Cc:CT1, RAN3, CT4

#### 2.2.2	Remaining Open issues

- Solution(s) for MT-EDT for UP solution and CP solution. Details of the agreed MT-EDT solution(s).

- Feasibility and support of paging probability based UE-grouping for WUS.  RAN2 aspects of UE-Group WUS.

- Support for transmission in preconfigured resources:

  - Whether shared resource is supported?

  - Dedicated resource (D-PUR)

    - Procedure aspects, e.g. Request /(re-)configuration/release procedures, triggering conditions, D-PUR transmission, downlink transmission after D-PUR transmission and fallback cases

    - Configuration aspects, e.g. TA validation criteria and resource configuration

    - CN impacts, if any

- Scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast:

  - Signalling for scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast,

  - MAC impact for scheduling multiple DL/UL transport blocks in unicast.

- Network management tools:

  - SON/ANR: stage 3 aspects of ANR procedure, configuration and reporting

  - RLF report: detailed contents of the RLF report.

- Improved multi-carrier operation

  - Support of Msg3 quality reporting in connected mode:  Timing  of the measurement reporting

  - NRS presence on non-anchor even when no paging NPDCCH is transmitted:

    - Signalling to indicate on a non-anchor carrier the set of subframes which will contain NRS even when no paging NPDCCH is transmitted.

    - Whether to enable RRM measurements on non-anchor carrier.

- Inter-RAT cell selection: Signalling aspect of the assistance information in the NB-IoT and LTE/eMTC RATs

- Signalling impact of coexistence of NB-IoT with NR, if any.

- Connection to 5GC.

  - Whether/ how to support slicing and 5GC QoS frame work

  - Whether/ how to support SDAP and NR PDCP

  - Whether/ how to support  UAC

  - Whether/ how to support RRC Connection Re-establishment for the Control Plane

  - Stage 3 aspects of support of  UP optimisation solution

  - Stage 3 aspects of support of AS RAI

## 2.3	RAN3

Contributions [415] – [448] were submitted to RAN3#103bis meeting.

Contributions [449] – [502] were submitted to RAN3#104 meeting.

#### 2.3.1	Agreements

RAN3#103bis

RAN3 discussed the support for Mobile-Terminated Early Data Transmission and the support for Functionality for Connection to 5GC jointly with eMTC. It was concluded to manage the Common part CRs by using both WI-codes, and individual part CRs by using separate CR with corresponding WI code.

- Mobile-Terminated Early Data Transmission: no agreement achieved.

- Functionality for Connection to 5GC:

  - The support of CP solution, UP solution, Common part, eDRX, and others were discussed.

  - RAN3 has evaluated the RAN3 impact of UP C-IoT optimization and RRC_INACTIVE connected to 5GC solutions, concluded that both solutions are feasible, reply LS approved in R3-192176.

- SON Support for Reporting and Others: not discussed due to limited of time.

RAN3#104

- Mobile-Terminated Early Data Transmission

  - Agreed to introduce the explicit EDT indicator in Paging message.

  - Baseline CR endorsed R3-193155.

  - Reply LS approved in R3-193156 to inform the progress and ask RAN2 to feedback the benefits on providing the data size in the S1 paging message.

- Functionality for Connection to 5GC:

  - Paging eDRX: Paging eDRX Information and NB-IoT Paging eDRX Information are needed in Paging message. MTC baseline in R3-193158, NB-IoT baseline in R3-193159.

  - Coverage Enhancement support: baseline CR endorsed R3-193160.

  - To share the work load, the task for next meeting were endorsed, refer to R3-193161 slide 6.

- SON Support for Reporting and Others: not discussed due to limited of time.

#### 2.3.2	Remaining Open issues

- Specify support for mobile-terminated (MT) early data transmission (EDT).

- Specify support for connection to 5GC.

- Specify support for SON Support for Reporting.

## 2.4	RAN4

Contributions [503] – [542] were submitted to RAN4#90bis meeting and [543] – [562] were submitted to RAN4#91 meeting.

#### 2.4.1	Agreements

RF

In RAN4#90bis, RAN4 discussed co-existence of NB-IoT with NR, with the following agreement (from agreed WF in R4-1905203):

- WF on system scenarios - To clarify the current definition of NB-IoT operation modes when NB-IoT is deployed within NR channel bandwidth, including when spectrum is refarmed from E-UTRA to NR - whether NB-IoT in-band or guard band operation without a hosting E-UTRA carrier would have backward compatibility impact on Rel’13/14/15 NB-IoT UE, which may expect the presence of a hosting E-UTRA carrier, especially for NB-IoT in-band operation - Agreement on NB-IoT standalone operating - There is no issue for NB-IoT standalone coexistence with NR and no further study is needed. - simulation results will be added in TR annex - WF on power boosting - Further investigate if currently specified power boosting could be supported, including when NB-IoT is located at NR edge positions for any channel bandwidth. - At least 3dB (currently specified RE power dynamic range) power boosting should be feasible. - Agreement on channel raster - For NR bands with 100 kHz channel raster and 15 kHz SCS, subcarrier and PRB alignment can be achieved for NB-IoT operating in NR in-band. - For NR bands with 15 kHz channel raster and 15 kHz SCS, subcarrier and PRB alignment can be achieved for NB-IoT operating in NR in-band. - Agreement on mixed numerology - No requirement is defined for mixed numerology between NR and NB-IoT similar as NR mixed numerology itself. - Agreement on bands - NB-IoT shall be supported on all NR bands refarmed from E-UTRA bands supporting NB-IoT.

And TR skeleton for coexistence of NB-IoT with NR was endorsed in R4-1905109. The TP on frequency band for NR and NB-IoT was endorsed for the TR in R4-1903848.

In RAN4#91, RAN4 discussed co-existence of NB-IoT with NR, with the following agreement (from agreed WF in R4-1907809):

- Agreements on NB-IoT operation in NR guard band - Considering legacy deployments, even if one NB-IoT 15 kHz sub-carrier would operate in NR guard band, still consider this case as NB-IoT operation in NR in-band. - NB-IoT operating in NR guard band is handled as implementation issue and RF requirements are not specified in Rel-15 (nor in Rel-16 unless new Rel-16 features make this essential). - Define in-band, guard band and standalone operation with NR. - WF on Power Boosting: - Option 1: - For 5 MHz channel bandwidth, specify the same +6 dB power boosting requirement for in-band 25 RB. - For 10 MHz channel bandwidth, specify the same +6 dB power boosting requirement for in-band 52 RB. - For 15 MHz channel bandwidth, specify the same +6 dB power boosting requirement for in-band center 77 RB. - For 20 MHz channel bandwidth, specify the same +6 dB power boosting requirement for in-band center 102 RB - (Single/multiple) declaration(s) on other RBs on the edge for the 15/20 MHz channel bandwidth. - FFS on other channel bandwidth>20 MHz - Option 2: - +6 dB power boosting shall be supported for any NR channel bandwidth and NB-IoT RB position. - Agreements on TDD - Conclusion: NB-IoT could operate in NR TDD by choosing suitable UL-DL configuration(s). - Agreements on operations and Testability - MSR BS supporting NR and LTE+NB-IoT (LTE in-band/guard band) should also meet current RF requirements when operating NR+(LTE)+NB-IoT (NR in-band) with following limitations: - Same NR channel bandwidth as LTE - NR uses 15 kHz SCS - NB-IoT carrier frequency kept when operating with LTE and NR, or shifted closer to NR carrier. - Therefore: - MSR BS could operate NB-IoT in NR in-band with above limitations. - No need to redo conformance testing in this case. - To remove any limitation: - Add support for NB-IoT operating in NR in-band in TS 38.104, TS 38.141-1 and TS 37.141 - Rel-15 NR BS can declare to support Rel-16 NB-IoT RF requirements for NB-IoT coexistence with NR.

And the following TPs were endorsed for the TR

  - R4-1906926	TP to TR38.xxx: Summary of system level simulation results for coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation

  - R4-1907810	TP for NB-IoT standalone operating

  - R4-1907811	NB-IoT - NR coexistence : TP to TR - channel raster

  - R4-1907812	TP to the new TR related to NB-IoT: mixed numerologies

RRM

In RAN4#90bis, RAN4 discussed UE-group wake-up signal, with the following agreement (from agreed WF in R4-1904807):

- RAN4 shall wait for further RAN1 progress on release 16 group WUS design to decide whether release 15 WUS requirements can be reused.

In RAN4#90bis, RAN4 discussed pre-configured uplink resources, with the following agreement (from agreed WF in R4-1904807):

- Serving cell NRSRP change based TA validation - The value(s) for the threshold(s) used for validating the TA are up to network implementation - Following options can be considered: - Option 1: Using relative NRSRP change together with TA change or DL/UL timing change - Option 2: applying PUR only in normal coverage considering decreased NRSRP accuracy in enhanced coverage. - Option 3: using only relative NRSRP change against signaled threshold(s) - RAN4 is to study how PUR transmissions are aligned with configured DRX/eDRX and its impact on the synchronization state to perform PUR transmission.

In RAN4#90bis, RAN4 discussed multi-carrier operation, with the following agreement (from agreed WF in R4-1904807):

- Legacy requirements defined for R14 msg3 based quality reporting apply for msg3 based quality reporting on non-anchor carriers including, - Hypothetical NPDCCH and - Report mapping and - Reporting accuracy - FFS T1 and T2 - Define non-anchor carrier serving cell RRM requirements based on NRS measurement on the non-anchor carrier in TS 36.133. - If configured by the network, serving cell non-anchor carrier measurements are feasible provided that - the relaxed cell monitoring criteria defined in TS 36.306 are met, and - transmit power difference of the signals/channels between anchor- and non-anchor carriers is known to the UE, and - UE is not configured with any positioning measurements and there are sufficient number of NRS subframes available for measurements as being discussed in RAN1 - FFS if the RSRP/RSRQ offset is used to compensate the measurement result towards non-anchor carriers.

In RAN4#91, RAN4 discussed UE-group wake-up signal, with the following agreement (from agreed WF in R4-1907734):

- In release 16, release 15 WUS reception requirements apply for release 15 UE. - RAN4 shall wait for further RAN1 progress related to WUS design, and shall disusss whether it is possible to reuse the Rel-15 WUS reception requirements at RAN4#92 meeting.

In RAN4#91, RAN4 discussed pre-configured uplink resources, with the following agreement (from agreed WF in R4-1907734):

- When TA validation based on serving cell NRSRP change method is configured: - eNodeB determines and configures K NRSRP threshold(s) to the UE, where K is 1 or 2. - It is FFS whether relaxed serving cell monitoring shall not be allowed when the signaled relaxation factor N is ≥ X, where X > 1. The value of X is FFS. - PUR transmission: - UE shall transmit in the PUR transmission opportunity using the obtained TA - If UE is synchronized towards the serving cell before PUR opportunity - Otherwise, UE may postpone the transmission to the next intended PUR transmission opportunity according to higher layer request of the UE or network configuration after UE has obtained the synchronization towards the serving cell within Tsearch, or it may drop the transmission. - Tsearch =the time required for NPSS/NSSS acquisition in eDRX. In DRX, Tsearch is the time required for performing time tracking and exact duration is FFS.

In RAN4#91, RAN4 discussed multi-carrier operation, with the following agreement (from agreed WF in R4-1907734):

- For NRSRQ measurement on non-anchor carriers - NRSRQ measurements on a non-anchor paging carrier cannot be translated to the equivalent NRSRQ measurements of the anchor carrier in the same way that NRSRP can be translated. - RAN4 is further investigating possible alternative methods for NRSRQ measurements translation between non-anchor paging and anchor carriers - It is FFS whether serving cell measurement relaxation introduced in Rel 15 for WUS can also be applied when the UE performs serving cell measurements on the non-anchor paging carrier - FFS whether T1 defined in R14 applies for MSG3 DL quality report in non-anchor carriers - RAN4 shall take into account further RAN1/RAN2 progress on reporting message type details.

#### 2.4.2	Remaining Open issues

- Specify support for UE-group wake-up signal

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance.

- Conclude on potential specification enhancement(s) for improving the performance of coexistence of NB-IoT with NR.

- Specify support for Msg3 quality reporting for non-anchor access.

- Identify scenarios under which RRM measurements based on NRS on non-anchor carriers are feasible.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-1905536	Feature lead summary of UE-group wake-up signal in NB-IoT	Ericsson

- R1-1905769	Draft LS on WUS gap configurability for NB-IoT	Ericsson

- R1-1905590	Draft LS on WUS weighting function for NB-IoT

- R1-1903891	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1903909	UE-group wake-up signal	Huawei, HiSilicon

- R1-1904055	UE-group wake-up signal for NB-IoT	vivo

- R1-1904160	UE-Group WUS in NB-IoT	MediaTek Inc.

- R1-1904176	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1904221	Group Wake-Up Signal for NB-IoT	Fraunhofer IIS, Fraunhofer HHI

- R1-1904229	UE group wake up signal for NB-IoT	Sony

- R1-1904270	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1904351	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1904381	UE-group wake-up signal for NB-IoT	Samsung

- R1-1904526	UE-group wake-up signal	Qualcomm Incorporated

- R1-1904568	UE-group wake-up signal for Rel-16 NB-IoT	Lenovo, Motorola Mobility

- R1-1904614	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1904865	Consideration on UE-group wake-up signal for Rel-16 NB-IoT	Sharp

- R1-1904942	Discussion on UE-group wake up signal in NB-IoT	NTT DOCOMO, INC.

- R1-1905538	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1905585	Updated feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1903892	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1903906	NB-IOT Pre-configured UL Resources Design Considerations 	Sierra Wireless, S.A.

- R1-1903910	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1904161	Early Transmission in Preconfigured UL resources in NB-IoT	MediaTek Inc.

- R1-1904177	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1904271	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1904352	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1904382	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1904527	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1904569	UL transmission in preconfigured resources for NBIoT	Lenovo, Motorola Mobility

- R1-1904615	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1904748	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

- R1-1904916	Discussion on NB-IoT PUR resource configuration in respect to radio  condition	ASUSTEK COMPUTER (SHANGHAI)

- R1-1904943	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1905119	Discussion on transmission in preconfigured UL resources	III

- R1-1905211	CBS PUR for NB-IoT	Fraunhofer IIS, Fraunhofer HHI

- R1-1905573	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1903893	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1903908	NB-IOT Multiple Transport Block Grant Design Considerations	Sierra Wireless, S.A.

- R1-1903911	Scheduling multiple DL/UL transport blocks	Huawei, HiSilicon

- R1-1904164	Scheduling of multiple DL/UL transport blocks in NB-IoT	MediaTek Inc.

- R1-1904178	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1904272	Scheduling of multiple DL/UL TBs for NB-IoT	Intel Corporation

- R1-1904353	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1904383	Scheduling of multiple transport blocks for NB-IoT	Samsung

- R1-1904528	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1904570	Design of scheduling of multiple DL/UL transport blocks for NBIoT	Lenovo, Motorola Mobility

- R1-1904616	Discussion on multiple transport blocks scheduling in NB-IoT	LG Electronics

- R1-1905539	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1905586	Updated feature lead summary of coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1903894	Coexistence of NB-IoT with NR	Ericsson

- R1-1903912	Performance enhancements for NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1904159	Coexistence of NB-IoT with NR	MediaTek Inc.

- R1-1904179	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1904354	Coexistence of NB-IoT with NR	ZTE

- R1-1904529	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1905540	Feature lead summary of quality report in Msg3 and connected mode	Huawei, HiSilicon

- R1-1905592	[DRAFT] Draft LS on channel quality report in Msg3 for NB-IoT and LTE-M	Huawei, HiSilicon

- R1-1905587	Updated Feature lead summary of Support of Quality report in Msg3 for non-anchor access and connected mode	Huawei, HiSilicon

- R1-1903895	Quality report in Msg3 and connected mode in NB-IoT	Ericsson

- R1-1903913	Support of quality report in Msg3 and Connected Mode	Huawei, HiSilicon

- R1-1904162	Quality report in Msg3 on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1904180	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1904273	Quality report in Msg3 for NB-IoT non-anchor access	Intel Corporation

- R1-1904355	Support of quality report in Msg3 and Connected Mode for NB-IoT	ZTE

- R1-1904384	Discussion on Msg3 report for non-anchor access	Samsung

- R1-1904530	Support of quality report in msg3 and connected mode	Qualcomm Incorporated

- R1-1904617	Downlink channel quality report during random access procedure on a non-anchor carrier LG Electronics

- R1-1905531	Presence of NRS on a non-anchor carrier for paging (feature lead summary)	Qualcomm

- R1-1903896	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1903914	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1904163	NRS presence on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1904181	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1904356	Presense of NRS on non-anchor PRB	ZTE

- R1-1904385	NRS presence on non-anchor carrier	Samsung

- R1-1904531	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R1-1904618	Discussion on NRS on a non-anchor carrier	LG Electronics

- R1-1907578	Feature lead summary of UE-group wake-up signal in NB-IoT	Ericsson

- R1-1905965	UE-group wake-up signal in NB-IoT	Ericsson

- R1-1905971	UE-group wake-up signal	Huawei, HiSilicon

- R1-1906189	Discussion on UE-group wake up signal in NB-IoT	NTT DOCOMO, INC.

- R1-1906505	Discussion on Wake-up signal for NB-IoT	ZTE

- R1-1906690	Discussion on UE-grouping wake up signal in NB-IoT	LG Electronics

- R1-1906710	UE group wake-up signal for NB-IoT	Nokia, Nokia Shanghai Bell

- R1-1906772	UE-group wake-up signal for NB-IoT	Intel Corporation

- R1-1907569	UE-group wake-up signal	Qualcomm Incorporated

- R1-1907192	UE group wake up signal for NB-IoT	Sony

- R1-1907207	Consideration on UE-group wake-up signal for Rel-16 NB-IoT	Sharp

- R1-1907560	UE-Group WUS in NB-IoT	MediaTek

- R1-1907573	Feature lead summary of Support for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1907627	Further topics for transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1907634	Reply LS on PUR Working Assumptions for NB-IoT and LTE-M	RAN2

- R1-1905966	Support for transmission in preconfigured UL resources in NB-IoT	Ericsson

- R1-1905972	Transmission in preconfigured UL resources	Huawei, HiSilicon

- R1-1906190	Discussion on preconfigured UL resources in NB-IoT	NTT DOCOMO, INC.

- R1-1906280	UL transmission in preconfigured resources for NBIoT	Lenovo, Motorola Mobility

- R1-1906461	NB-IOT Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

- R1-1906506	Support for transmission in preconfigured UL resources for NB-IoT	ZTE

- R1-1906691	Discussion on preconfigured UL resources in NB-IoT	LG Electronics

- R1-1906711	Preconfigured Grant for Uplink transmission	Nokia, Nokia Shanghai Bell

- R1-1906773	UL transmission in preconfigured resources for NB-IoT	Intel Corporation

- R1-1906897	Discussion on transmission in preconfigured UL reosurces for NB-IoT	Samsung

- R1-1907001	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

- R1-1907081	Consideration for preconfigured uplink resources (PUR)	Sequans Communications

- R1-1907358	Discussion on NB-IoT PUR resource configuration in respect to radio condition	ASUSTEK COMPUTER (SHANGHAI)

- R1-1907370	Discussion on transmission in preconfigured UL resources	III

- R1-1907567	Summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

- R1-1905967	Scheduling of multiple DL/UL transport blocks in NB-IoT	Ericsson

- R1-1905973	Scheduling multiple DL/UL transport blocks	Huawei, HiSilicon

- R1-1906281	Design of scheduling of multiple DL/UL transport blocks for NBIoT	Lenovo, Motorola Mobility

- R1-1906459	NB-IoT Multiple Transport Block Grant Design Considerations	Sierra Wireless, S.A.

- R1-1906507	Consideration on scheduling enhancement for NB-IoT	ZTE

- R1-1906692	Discussion on multiple transport blocks scheduling in NB-IoT	LG Electronics

- R1-1906712	Scheduling of multiple DL/UL transport blocks	Nokia, Nokia Shanghai Bell

- R1-1906774	Scheduling of multiple DL/UL TBs for NB-IoT	Intel Corporation

- R1-1906898	Scheduling of multiple transport blocks for NB-IoT	Samsung

- R1-1907002	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

- R1-1907574	Feature lead summary of Coexistence of NB-IoT with NR	Huawei, HiSilicon

- R1-1907623	[DRAFT] LS to RAN on Coexistence of NB-IoT with NR

- R1-1905968	Coexistence of NB-IoT with NR	Ericsson

- R1-1905974	Performance enhancements for NB-IoT coexistence with NR	Huawei, HiSilicon

- R1-1906508	Coexistence of NB-IoT with NR	ZTE

- R1-1906713	Coexistence of NB-IoT with NR	Nokia, Nokia Shanghai Bell

- R1-1907003	Coexistence of NB-IoT with NR	Qualcomm Incorporated

- R1-1907575	Feature lead summary of support of quality report in Msg3 and connected mode	Huawei, HiSilicon

- R1-1907633	Further topics for quality report in Msg3 and connected mode	Huawei, HiSilicon

- R1-1905969	Quality report in Msg3 and connected mode in NB-IoT	Ericsson

- R1-1905975	Support of channel quality report in Msg3 and connected mode	Huawei, HiSilicon

- R1-1906509	Support of quality report in Msg3 and Connected Mode for NB-IoT	ZTE

- R1-1906693	Downlink channel quality report during random access procedure on a non-anchor carrier LG Electronics

- R1-1906714	Support of Quality report in Msg3 on non-anchor carrier	Nokia, Nokia Shanghai Bell

- R1-1906775	Quality report in Msg3 for NB-IoT non-anchor access	Intel Corporation

- R1-1906899	Discussion on Msg3 report for non-anchor access	Samsung

- R1-1907004	Support of quality report in msg3 and connected mode	Qualcomm Incorporated

- R1-1907173	DL Quality Report on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1907571	Presence of NRS on a non-anchor carrier for paging (feature lead summary)	Qualcomm

- R1-1905970	Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R1-1905976	NRS presence on non-anchor carriers for paging	Huawei, HiSilicon

- R1-1906510	Presense of NRS on non-anchor PRB	ZTE

- R1-1906694	Discussion on NRS on a non-anchor carrier	LG Electronics

- R1-1906715	Presence of NRS on a non-anchor carrier for paging	Nokia, Nokia Shanghai Bell

- R1-1907005	Presence of NRS on a non-anchor carrier for paging	Qualcomm Incorporated

- R1-1907314	NRS presence on non-anchor carrier in NB-IoT	MediaTek Inc.

- R1-1906654	Support of 16QAM in NB-IoT	MediaTek Inc.

- R2-1903063	Pre-configured UL Resource Design Considerations	Sierra Wireless, S.A.

- R2-1903181	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)

- R2-1903182	DRX considerations for preconfigured resources	Sequans Communications

- R2-1903186	Remaining solution options for MT-initiated DL EDT	Qualcomm Incorporated

- R2-1903187	(Re)configuration and release of PUR	Qualcomm Incorporated

- R2-1903188	QoS for data transmission on PUR	Qualcomm Incorporated

- R2-1903189	Contention-free shared (CFS) PUR	Qualcomm Incorporated

- R2-1903190	RACH Report for NB-IoT: how to report	Qualcomm Incorporated

- R2-1903300	D-PUR configuration and release aspects	Gemalto N.V.

- R2-1903301	Idle mode ANR Measurements for NB-IoT	Gemalto N.V.

- R2-1903380	Analysis of Service based GWUS	Nokia, Nokia Shanghai Bell

- R2-1903381	Analysis of Interworking of GWUS and common WUS	Nokia, Nokia Shanghai Bell

- R2-1903382	Further Analysis of MT EDT options	Nokia, Nokia Shanghai Bell

- R2-1903383	Mapping of MT EDT Use cases to Solution Options	Nokia, Nokia Shanghai Bell

- R2-1903384	Further analysis on remaining signaling Aspects for D-PUR	Nokia, Nokia Shanghai Bell

- R2-1903385	Further Analysis on shared PUR	Nokia, Nokia Shanghai Bell

- R2-1903386	Idle mode ANR Measurements for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-1903387	RACH Report for NB-IoT SON	Nokia, Nokia Shanghai Bell

- R2-1903388	NB configuration for GWUS/WUS capable UE	Nokia, Nokia Shanghai Bell

- R2-1903389	Analysis of IRAT cell selection assistance information signaling	Nokia, Nokia Shanghai Bell

- R2-1903399	Support of User Plane Optimization for 5GC	Gemalto N.V.

- R2-1903405	DL channel quality reporting in MSG3 and in connected mode	Huawei, HiSilicon

- R2-1903406	RRM measurements on non-anchor carrier	Huawei, HiSilicon

- R2-1903407	DRAFT LS on enabling RRM measurements on the non-anchor paging carrier in Idle mode	Huawei

- R2-1903408	Left over from Report of [104#44][NB-IoT R16] SON ANR report	Huawei, HiSilicon

- R2-1903409	Reporting procedure for SON	Huawei, HiSilicon

- R2-1903410	Discussion on MT-EDT options	Huawei, HiSilicon

- R2-1903411	Overall procedure for DL data after preamble	Huawei, HiSilicon

- R2-1903412	General procedure of D-PUR	Huawei, HiSilicon

- R2-1903413	Discussion on D-PUR configuration	Huawei, HiSilicon

- R2-1903414	[DRAFT] LS on D-PUR	Huawei

- R2-1903415	Analysis on resource types in PUR	Huawei, HiSilicon

- R2-1903416	General aspects on group based WUS	Huawei, HiSilicon

- R2-1903417	Performance analysis on UE_ID based WUS grouping	Huawei, HiSilicon

- R2-1903418	Feasibility of MO/MT service type based WUS grouping	Huawei, HiSilicon

- R2-1903419	[DRAFT] LS on MO/MT service type indication for WUS grouping	Huawei

- R2-1903420	Applicability of multiple TB scheduling to EDT	Huawei, HiSilicon

- R2-1903421	Introducing Connectivity to 5GC for NB-IoT	Huawei, HiSilicon

- R2-1903422	System information to support CN selection to 5GC	Huawei, HiSilicon, Qualcomm Incorporated

- R2-1903423	Indications for connection to 5GC during connection establishment	Huawei, HiSilicon

- R2-1903424	CIoT support and evolution for 5GC	Huawei, HiSilicon

- R2-1903425	Discussion on support of UP optimisation and RRC_INACTIVE for NB-IoT and eMTC connected to 5GC	Huawei, HiSilicon, Telecom Italia

- R2-1903457	UE grouping on wake-up signal channel	Qualcomm Incorporated

- R2-1903458	Channel quality reporting for non-anchor carriers	Qualcomm Incorporated

- R2-1903459	Non-anchor carrier measurements for RRM	Qualcomm Incorporated

- R2-1903460	Comparison of UP CIoT and RRC-INACTIVE for 5GC	Qualcomm Incorporated, Telecom Italia S.p.A, Sierra Wireless S.A.

- R2-1903461	Scheduling of multiple DL/UL transport blocks - RAN2 impacts	Qualcomm Incorporated

- R2-1903485	Further consideration on UL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1903486	Further consideration on DL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1903487	Further consideration on non-anchor carrier measurement	ZTE Corporation, Sanechips

- R2-1903489	Consideration on coexistence of NB-IoT with NR	ZTE Corporation, Sanechips

- R2-1903491	Consideration on service based UE grouping for WUS	ZTE Corporation, Sanechips

- R2-1903492	Remaining issues of UE grouping for WUS	ZTE Corporation, Sanechips

- R2-1903493	Further discussion on options for MT EDT	ZTE Corporation, Sanechips

- R2-1903494	Consideration on user plane solution for connection to 5GC	ZTE Corporation, Sanechips

- R2-1903495	Consideration on common aspects for connection to 5GC	ZTE Corporation, Sanechips

- R2-1903496	Further consideration on quality report in Msg3 for non-anchor access	ZTE Corporation, Sanechips

- R2-1903662	Mobile Terminated EDT for CP and UP solutions	Intel Corporation

- R2-1903665	HARQ process for PUR transmission in idle mode	Intel Corporation

- R2-1903666	Support of connectivity to 5GCN in eMTC and NB-IoT	Intel Corporation

- R2-1903667	RRC INACTIVE state for eMTC and NB-IoT	Intel Corporation

- R2-1903708	Discussion on PUR resource configuration in respect to radio condition	ASUSTeK

- R2-1903709	Discussion on TA timer for D-PUR	ASUSTeK

- R2-1903766	Further consideration on RACH-report in NB-IoT	SHARP Corporation

- R2-1903784	Improvements to Inter-RAT cell selection for IDLE mode	Fujitsu

- R2-1903828	MT early data after preamble	Ericsson

- R2-1903829	MT early data in Msg4	Ericsson

- R2-1903830	PUR L1 or L23 ACK	Ericsson

- R2-1903831	PUR TA validation & release	Ericsson

- R2-1903832	PUR Configuration	Ericsson

- R2-1903833	PUR UE Multiplexing	Ericsson

- R2-1903834	Draft LS to RAN1 on PUR acknowledgement	Ericsson

- R2-1903835	Scheduling enhancements	Ericsson

- R2-1903840	UP solution when connecting LTE-M or NB-IoT to 5GC	Ericsson

- R2-1903841	Support of extended DRX in RRC_INACTIVE when connected to 5GC	Ericsson

- R2-1903868	Considerations to WUS Grouping	Gemalto N.V.

- R2-1904038	MT EDT single solution consideration	Gemalto N.V.

- R2-1904095	NB-IoT ANR Reporting	Sequans Communications

- R2-1904098	Inter-RAT Deployment scenarios	Sequans Communications

- R2-1904099	On UP C-IoT 5GS optimization support	TIM, Vodafone

- R2-1904126	Consideration on ANR measurement reporting in NB-IOT	Lenovo, Motorola Mobility

- R2-1904127	Procedure of RACH reporting in NB-IOT SON	Lenovo, Motorola Mobility

- R2-1904131	Consideration on service-based UE grouping	Lenovo, Motorola Mobility

- R2-1904370	On RAN2 impacts on introduction of MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1904371	On RAN2 impacts on introduction of NB-IoT connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1904373	On user plane solution for MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1904375	On user plane solution for NB-IoT connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1904389	Remaining issues for RLF report in NB-IoT	ZTE Corporation, Sanechips

- R2-1904393	Remaining issues for ANR report in NB-IoT	ZTE Corporation, Sanechips

- R2-1904396	Remaining issues for RACH report in NB-IoT and eMTC	ZTE Corporation, Sanechips

- R2-1904453	D-PUR related RRC Signaling Procedure for CP Solution	Nokia, Nokia Shanghai Bell

- R2-1904496	UE_ID based WUS grouping	Ericsson

- R2-1904498	Mobility WUS group and paging multiplexing	Ericsson

- R2-1904513	Discussion on ANR Measurement Reporting Response Time	Ericsson

- R2-1904514	Discussion on ANR Measurement using single set of measurements	Ericsson

- R2-1904515	Configuration and Reporting of RLF for NB-IoT	Ericsson

- R2-1904516	Introducing IRAT cell selection for NB-IoT	Ericsson

- R2-1904517	Introduction of Inter-RAT cell selection indication in 36.331	Ericsson

- R2-1904523	Draft LS on MT EDT	Ericsson

- R2-1904525	Support of extended DRX in RRC_IDLE when connected to 5GC	Ericsson

- R2-1904547	Msg3 Quality Report Format for NB-IoT	Ericsson

- R2-1904639	Supporting Presence of NRS on a non-anchor carrier for paging in NB-IoT	Ericsson

- R2-1904640	Configuration and Reporting of RACH for NB-IoT	Ericsson

- R2-1904641	Configuration and Reporting of RACH Failure measurement for eMTC	Ericsson

- R2-1904684	Access stratum changes to enable NB-IoT connectivity to 5GC	Qualcomm Incorporated

- R2-1904686	Access stratum changes to enable eMTC connectivity to 5GC	Qualcomm Incorporated

- R2-1904687	Access Stratum Release Assistance Indicator for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated

- R2-1904689	Access stratum changes to support CP-EDT optimization for eMTC & NB-IoT connected to 5GC	Qualcomm Incorporated

- R2-1904695	Access stratum changes to support Non-EDT control plane optimization for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated

- R2-1904719	Access stratum changes to support idle mode eDRX for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated

- R2-1904722	Access stratum changes to support connected mode eDRX for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated

- R2-1904759	On Inter-RAT assistance information for NB-IoT and LTE(-eMTC)	Sequans Communications

- R2-1904772	Discussion on UE operations using D-PUR	LG Electronics UK

- R2-1904773	Consideration of time alignment in PUR	LG Electronics UK

- R2-1904774	Discussion on Msg2 based MT EDT solution	LG Electronics UK

- R2-1904775	Discussion on Msg4 based MT EDT solution	LG Electronics UK

- R2-1904776	Discussion on specifying multiple MT EDT solutions	LG Electronics UK

- R2-1904777	Support for Shared-PUR	LG Electronics UK

- R2-1905110	Mobile-terminated Early Data Transmission	MediaTek Inc.

- R2-1905194	Comparison of UP CIoT and RRC-INACTIVE for 5GC	Qualcomm Incorporated, Telecom Italia S.p.A, Sierra Wireless S.A., Vodafone, BlackBerry

- R2-1905199	Discussion on ANR Measurement Reporting Response Time	Ericsson

- R2-1905204	Email discussion [104#43][eMTC & NB-IoT R16] D-PUR report	Sierra Wireless

- R2-1905205	Comparison of UP CIoT and RRC-INACTIVE for 5GC	Qualcomm Incorporated, Telecom Italia S.p.A, Sierra Wireless S.A., Vodafone, BlackBerry, DISH Network

- R2-1905214	UE power comparisons for D-PUR	Sierra Wireless, S.A.

- R2-1905259	[DRAFT] LS on non-anchor carrier CQI reporting in MSG3	Qualcomm

- R2-1905260	[DRAFT] LS on non-anchor carrier  measurements for RRM	Qualcomm Incorporated

- R2-1905261	[DRAFT] LS on MO/MT service type indication for WUS grouping	Huawei

- R2-1905263	LS on non-anchor carrier CQI reporting in MSG3	RAN2

- R2-1905264	LS on non-anchor carrier idle mode measurements for RRM	RAN2

- R2-1905271	[DRAFT] LS on Mobile-terminated Early Data Transmission	Ericsson

- R2-1905276	LS on Mobile-terminated Early Data Transmission	RAN2

- R2-1905361	LS on PUR Working Assumptions for NB-IoT and LTE-M (R1-1905570; contact: Sierra Wireless)

- R2-1905619	Measurement, configuration and procedure for ANR report in NB-IoT	ZTE Corporation, Sanechips

- R2-1905630	Detailed procedure for RACH report in NB-IoT	ZTE Corporation, Nokia, Nokia Shanghai Bell

- R2-1905632	Remaining issues for RLF report in NB-IoT	ZTE Corporation, Sanechips

- R2-1905642	Remaining issues for UL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1905643	Network based D-PUR configuration and release	ZTE Corporation, Sanechips

- R2-1905645	Further consideration on DL aspects of D-PUR in IDLE	ZTE Corporation, Sanechips

- R2-1905646	Quality report in connected mode for configured carrier in NB-IoT	ZTE Corporation, Sanechips

- R2-1905649	Further discussion on common aspects for connection to 5GC	ZTE Corporation, Sanechips

- R2-1905733	Idle mode ANR Measurements for NB-IoT	Gemalto N.V.

- R2-1905734	D-PUR configuration and release aspects	Gemalto N.V.

- R2-1905735	Further consideration on service based UE grouping for WUS	ZTE Corporation, Sanechips

- R2-1905736	Remaining issues of UE grouping for WUS	ZTE Corporation, Sanechips

- R2-1905737	[DRAFT] LS on D-PUR resource configuration and release	ZTE Corporation, Sanechips

- R2-1905739	Consideration on coexistence of NB-IoT with NR	ZTE Corporation, Sanechips

- R2-1905740	On UP C-IoT 5GS optimization support	Telecom Italia S.p.A.

- R2-1905756	Further discussion on Msg2-based solution for MT EDT	ZTE Corporation, Sanechips

- R2-1905757	[DRAFT] LS on DL channel quality report for configured carrier in RRC connected mode	ZTE Corporation, Sanechips

- R2-1905899	Further discussion on Msg4-based solution for MT EDT	ZTE Corporation, Sanechips

- R2-1906051	Access stratum changes to support Non-EDT control plane optimization for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated, Huawei, HISilicon

- R2-1906052	Access stratum security aspects for NB-IoT and eMTC connectivity to 5GC	Qualcomm Incorporated, Huawei, HISilicon

- R2-1906053	PDCP type to be used for NB-IoT and eMTC connected to 5GC	Qualcomm Incorporated, Huawei, HISilicon

- R2-1906054	Access Stratum Release Assistance Indicator for eMTC and NB-IoT connected to 5GC	Qualcomm Incorporated, Huawei, HISilicon

- R2-1906055	Draft LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC	Qualcomm India Pvt Ltd

- R2-1906056	Access stratum enhancements to support 5GC features for both NB-IoT and eMTC.	Qualcomm India Pvt Ltd, Huawei, HiSilicon

- R2-1906147	Discussion on ANR procedure for NB-IoT	Huawei, HiSilicon

- R2-1906152	Discussion on MT-EDT options	Huawei, HiSilicon

- R2-1906153	Overall procedure for Msg2 based option	Huawei, HiSilicon

- R2-1906154	Overall procedure for Msg4 based option	Huawei, HiSilicon

- R2-1906155	Discussion on D-PUR steps	Huawei, HiSilicon

- R2-1906156	[Draft] LS on downlink message after D-PUR transmission	Huawei

- R2-1906157	Discussion on RAN1 LS for D-PUR	Huawei, HiSilicon

- R2-1906158	[DRAFT] Reply LS on D-PUR working assumptions	Huawei

- R2-1906159	Other General aspects on PUR with dedicated resource	Huawei, HiSilicon

- R2-1906160	Discussion on D-PUR configuration	Huawei, HiSilicon

- R2-1906161	Performance analysis on UE_ID based WUS grouping	Huawei, HiSilicon

- R2-1906162	Feasibility of MO/MT service type based WUS grouping	Huawei, HiSilicon

- R2-1906163	[DRAFT] LS on assistance indication for WUS grouping	Huawei

- R2-1906164	Applicability of multiple TB scheduling to EDT	Huawei, HiSilicon

- R2-1906165	TP to capture the agreements at RAN2#105bis on SIB signalling for NB-IoT and eMTC connected to 5GC	Huawei, HiSilicon, Qualcomm Incorporated

- R2-1906166	Impact to support user plane optimisations for eMTC and NB-IoT connected to 5GC	Huawei, HiSilicon, Qualcomm Incorporated

- R2-1906167	Support of RRC connection Re-establishment for the Control plane for NB-IoT connected to 5GC	Huawei, HiSilicon, Qualcomm Incorporated

- R2-1906168	[DRAFT] LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC	Huawei

- R2-1906169	Unified access control for NB-IoT and eMTC connected to 5GC	Huawei, HiSilicon, Qualcomm Incorporated

- R2-1906170	[DRAFT] LS on Access Categories for NB-IoT	Huawei

- R2-1906171	Consideration on ANR measurement reporting in NB-IOT	Lenovo, Motorola Mobility

- R2-1906186	Procedure of RACH reporting in NB-IOT SON	Lenovo, Motorola Mobility

- R2-1906208	Consideration on service-based UE grouping	Lenovo, Motorola Mobility

- R2-1906236	Consideration on MT-EDT procedure	Lenovo, Motorola Mobility

- R2-1906244	NB-IoT ANR Reporting	Sequans Communications

- R2-1906245	Inter-RAT Deployment scenarios	Sequans Communications

- R2-1906246	Support for RRC-INACTIVE state with short eDRX for 5GC CIoT	Qualcomm Incorporated, Huawei, HiSilicon

- R2-1906247	[DRAFT] Reply LS on Conclusion on 5G CIoT user plane small data solution	Qualcomm Incorporated, Huawei, HiSilicon

- R2-1906248	Scheduling of multiple DL/UL transport blocks - RAN2 impacts	Qualcomm Incorporated

- R2-1906249	WUS grouping design overview and UE distribution	Qualcomm Incorporated

- R2-1906258	Improvements to Inter-RAT cell selection for IDLE mode	Fujitsu

- R2-1906272	Discussion on FFSes of D-PUR	Huawei, HiSilicon

- R2-1906434	Details on MT EDT for UP solution (Msg4-based)	Intel Corporation

- R2-1906435	Details on MT EDT for CP data (Msg2-based)	Intel Corporation

- R2-1906438	Discussion on the support of 5GCN connectivity	Intel Corporation

- R2-1906439	Support of RRC inactive state	Intel Corporation

- R2-1906440	Transmission in D-PUR in IDLE mode	Intel Corporation

- R2-1906444	Remaining solution options for MT-initiated DL EDT	Qualcomm Incorporated

- R2-1906445	(Re)configuration and release of PUR	Qualcomm Incorporated

- R2-1906446	Retransmission and fallback for data transmission on PUR	Qualcomm Incorporated

- R2-1906447	L2/L3 ACK for data transmission on PUR	Qualcomm Incorporated

- R2-1906549	Support of RRC_INACTIE state for eMTC/NB-IoT	ASUSTeK

- R2-1906550	Discussion on PUR resource configuration in respect to radio condition	ASUSTeK

- R2-1906551	Discussion on TA timer for D-PUR	ASUSTeK

- R2-1906620	Performance And Feasibility of Service based GWUS	Nokia, Nokia Shanghai Bell

- R2-1906621	Analysis of Interworking of GWUS and common WUS	Nokia, Nokia Shanghai Bell

- R2-1906622	Further Analysis of Msg2 based Option	Nokia, Nokia Shanghai Bell

- R2-1906623	Further Analysis of Paging based Option	Nokia, Nokia Shanghai Bell

- R2-1906624	Further analysis on D-PUR configuration and RRC signaling	Nokia, Nokia Shanghai Bell

- R2-1906625	Further Analysis on shared PUR	Nokia, Nokia Shanghai Bell

- R2-1906626	Idle mode ANR Measurements for NB-IoT	Nokia, Nokia Shanghai Bell

- R2-1906627	NB configuration for GWUS/WUS capable UE	Nokia, Nokia Shanghai Bell

- R2-1906628	Analysis of IRAT cell selection assistance information signaling	Nokia, Nokia Shanghai Bell

- R2-1906629	On the security aspects of D-PUR for CP Solution	Nokia, Nokia Shanghai Bell

- R2-1906721	Considerations to WUS Grouping	Gemalto N.V.

- R2-1906732	MT EDT single solution consideration	Gemalto N.V.

- R2-1906832	DRX considerations for preconfigured resources	Sequans Communications

- R2-1906898	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)

- R2-1906929	MT early data in Msg4	Ericsson

- R2-1906930	MT early data after preamble	Ericsson

- R2-1906931	Handling UE context for Msg2-based MT UP-EDT	Ericsson

- R2-1906932	Scheduling enhancements	Ericsson

- R2-1906937	Support of RRC_INACTIVE (with 'short eDRX') for LTE-M	Ericsson

- R2-1906938	PUR TA validation & release	Ericsson

- R2-1906939	PUR Configuration	Ericsson

- R2-1906940	PUR L1 or L2/L3 acknowledgement	Ericsson

- R2-1906941	[Draft] LS reply on PUR Working Assumptions for NB-IoT and LTE-M	Ericsson

- R2-1906942	PUR UE multiplexing	Ericsson

- R2-1906943	Use of CN type in RRCConnectionRelease/RRCConnectionRelease-NB	Ericsson

- R2-1906944	Addressing FFSs in 5GC for LTE-M and NB-IoT	Ericsson

- R2-1906960	Expected Behavior after PUR Transmission	Sequans Communications

- R2-1907053	On supporting UE-group wake-up signal for MTC	Sony

- R2-1907119	On Inter-RAT assistance information for NB-IoT and LTE(-eMTC)	Sequans Communications

- R2-1907244	D-PUR Configuration Details	Sierra Wireless, S.A.

- R2-1907259	D-PUR Message Acknowledgement	Sierra Wireless, S.A.

- R2-1907285	[Draft] Reply LS on PUR Working Assumptions for NB-IoT and LTE-M	Sierra Wireless, S.A.

- R2-1907290	On RRC_INACTIVE support for NB-IoT and MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R2-1907303	Reply LS on Conclusion on 5G CIoT user plane small data solution	Nokia, Nokia Shanghai Bell

- R2-1907465	Text proposal for adding new SIB in 36.300	Ericsson

- R2-1907466	Msg3 Quality Report Format for NB-IoT	Ericsson

- R2-1907467	Discussion on ANR Measurement using single set of measurements	Ericsson

- R2-1907468	Text Proposal on RLF Report NB-IoT	Ericsson

- R2-1907469	Text Proposal on RACH Report NB-IoT	Ericsson

- R2-1907470	Text Proposal on RACH Report NB-IoT	Ericsson

- R2-1907474	Discussion on ANR Measurement Reporting Response Time	Ericsson

- R2-1907475	Adding new system information on IRAT cell selection for NB-IoT	Ericsson

- R2-1907476	Text proposal for adding support for IRAT cell selection in 36.331	Ericsson

- R2-1907551	PUR with DCI scheduling	Ericsson

- R2-1907778	RRC release for User Plane EDT	III

- R2-1907872	Discussion on UE operations using D-PUR	LG Electronics UK

- R2-1907873	Consideration of time alignment in PUR	LG Electronics UK

- R2-1907874	Discussion on Msg2 based MT EDT solution	LG Electronics UK

- R2-1907875	Discussion on Msg4 based MT EDT solution	LG Electronics UK

- R2-1907876	Discussion on specifying multiple MT EDT solutions	LG Electronics UK

- R2-1907877	Support for Shared-PUR	LG Electronics UK

- R2-1907908	UE_ID based WUS grouping	Ericsson

- R2-1907912	Mobility WUS group and paging multiplexing	Ericsson

- R2-1907929	RRC release after EDT for User Plane CIoT EPS optimization	Ericsson

- R2-1908053	[Draft] LS on Traffic Profile in CIoT EPS optimisation	LG Electronics UK

- R2-1908188	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)

- R2-1908189	[Draft] Reply LS on PUR Working Assumptions for NB-IoT and LTE-M	Sierra Wireless, S.A.

- R2-1908190	LS on support for flow based QoS & slicing for NB-IoT & eMTC connected to 5GC	RAN2

- R2-1908191	[DRAFT] LS on Access Categories for NB-IoT	Huawei

- R2-1908192	[DRAFT] LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC	Huawei

- R2-1908193	Summary of email discussion scopes for D-PUR	Huawei

- R2-1908194	[DRAFT] LS on DL channel quality report for configured carrier in RRC connected mode	ZTE Corporation, Sanechips

- R2-1908195	[DRAFT] LS on assistance indication for WUS grouping	Huawei

- R2-1908196	Reply LS on PUR Working Assumptions for NB-IoT and LTE-M	RAN2

- R2-1908202	Reply LS on Conclusion on 5G CIoT user plane small data solution	RAN2

- R2-1908260	LS on assistance indication for WUS grouping	RAN2

- R2-1908262	LS on DL channel quality report for configured carrier in RRC connected mode	RAN2

- R2-1908263	LS on UAC for NB-IoT	RAN2

- R2-1908264	LS on RRC Connection Re-Establishment for CP for NB-IoT connected to 5GC	RAN2

- R2-1908373	[Draft] Reply LS on PUR Working Assumptions for NB-IoT and LTE-M	Sierra Wireless, S.A.

- R3-191188	Response LS on on eDRX cycles for CM-CONNECTED with RRC inactive	3GPP RAN2, Qualcomm

- R3-191189	Response LS on Completion of Study on Cellular IoT support and evolution for the 5G System	3GPP RAN2, Qualcomm

- R3-191190	Reply LS on EDT integrity protection	3GPP RAN2, Huawei

- R3-191256	Consideration on MT EDT supporting	Huawei

- R3-191257	Introduction of CIoT CP solution	Huawei

- R3-191258	Consideration on eDRX supporting	Huawei

- R3-191259	Support of MO EDT in CP and UP solutions	Huawei

- R3-191260	Support of Inter UE QoS	Huawei

- R3-191261	Support of Enhanced Coverage	Huawei

- R3-191262	Support of Expected UE Behavior information	Huawei

- R3-191263	SON Supporting for Reporting	Huawei

- R3-191281	Consideration on user plane solution for connection to 5GC	ZTE

- R3-191324	On User Plane Small Data Solution	Qualcomm Incorporated

- R3-191325	Introduction of CP-CIoT support in NGAP	Qualcomm Incorporated

- R3-191326	Potential impacts of idle mode eDRX in 5GS on RAN3 specifications	Qualcomm Incorporated

- R3-191327	Further discussion on impacts of MT EDT on RAN3 specifications	Qualcomm Incorporated

- R3-191329	Consideration on RAN3 impacts for MT EDT	ZTE

- R3-191351	Solutions for MT EDT use cases	Nokia, Nokia Shanghai Bell

- R3-191352	RAN3 functions for MTC and NB-IOT connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-191353	Introduction of NB-IOT support to 5GC	Nokia, Nokia Shanghai Bell

- R3-191683	MT UP EDT indication in S1-AP	Ericsson

- R3-191687	MT EDT initiation in Paging	Ericsson

- R3-191689	Selection of UP solution for NB-IOT and MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-191691	Data Notification	Ericsson

- R3-191746	WI work plan for Rel-16 LTE-MTC	Ericsson

- R3-191747	Further discussion on enhancement: the Intra eNB CP Relocation for NB-IoT	Ericsson

- R3-191748	MT early data transmission options	Ericsson

- R3-191749	Discussion on potential MT early data transmission indication in Msg4	Ericsson

- R3-191750	Support of eDRX when connected to 5GC	Ericsson

- R3-191751	Data notification for CIoT Ues	Ericsson

- R3-191786	Discussion on UP solution for connection to 5GC	LG Electronics

- R3-191807	UP solution for connecting LTE-M or NB-IoT to 5GC	Ericsson

- R3-192079	MTC&IoT Enh session report	Vice-chairman (ZTE)

- R3-192176	Reply LS on Completion of Study on Cellular IoT support and evolution for the 5G System	Nokia, Nokia Shanghai Bell

- R3-192300	Consideration on restriction of use of Enhanced Coverage for NB-IoT and eMTC connected to 5GC	ZTE

- R3-192301	pCR38413 for Restriction of use of Enhanced Coverage for NB-IoT/eMTC connected to 5GC	ZTE

- R3-192302	pCR38473 for Restriction of use of Enhanced Coverage for NB-IoT/eMTC connected to 5GC	ZTE

- R3-192303	Support of extended DRX in CM-IDLE for NB-IoT and eMTC connected to 5GC	ZTE

- R3-192305	pCR38473 for Support of extended DRX in CM-IDLE for NB-IoT/eMTC connected to 5GC	ZTE

- R3-192322	Consideration on NB-IoT only aspects when connecting to 5GC	Huawei, Qualcomm Incorporated

- R3-192323	Introduction of NB-IoT aspects when connecting to 5GC	Huawei, Qualcomm Incorporated

- R3-192324	Introduction of NB-IoT aspects when connecting to 5GC	Huawei, Qualcomm Incorporated

- R3-192325	NB-IoT only_Paging and eDRX aspects	Huawei, Qualcomm Incorporated

- R3-192327	New NGAP procedures on NB-IoT only aspects	Huawei, Qualcomm Incorporated

- R3-192328	Introduction of NB-IoT related NG-AP procedures	Huawei, Qualcomm Incorporated

- R3-192329	Introduction of NB-IoT related NG-AP procedures	Huawei, Qualcomm Incorporated

- R3-192330	Control Plane CIoT 5GS Optimisation for NB and MTC	Huawei, Qualcomm Incorporated

- R3-192331	Introduction of Control Plane CIoT 5GS Optimisation for NB and MTC	Huawei, Qualcomm Incorporated

- R3-192332	Introduction of Control Plane CIoT 5GS Optimisation for NB and MTC	Huawei, Qualcomm Incorporated

- R3-192333	SON Supporting for Reporting	Huawei

- R3-192334	Introduction of SON supporting for NB-IoT	Huawei

- R3-192335	Introduction of SON supporting for NB-IoT	Huawei

- R3-192458	Introduction of MT EDT in S1 Paging	Nokia, Nokia Shanghai Bell

- R3-192459	Introduction of MT EDT in S1 Paging	Nokia, Nokia Shanghai Bell

- R3-192460	RAN3 functions for MTC and NB-IOT connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-192461	Introduction of NBIOT connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-192462	Introduction of MTC connected to 5GC	Nokia, Nokia Shanghai Bell

- R3-192465	Impacts of MT EDT (“data after preamble” option) on RAN3 specifications	Qualcomm Incorporated, Huawei

- R3-192467	Introduction of UP CIoT Optimization in NG-RAN	Qualcomm Incorporated, Huawei

- R3-192468	Introduction of support for UP CIoT	Qualcomm Incorporated, Huawei

- R3-192469	Introduction of UP-CIoT support in NGAP	Qualcomm Incorporated, Huawei

- R3-192470	Introduction of UP-CIoT support in XnAP	Qualcomm Incorporated, Huawei

- R3-192471	Common aspects of CIoT	Qualcomm Incorporated, Huawei

- R3-192472	Introduction of support for CIoT functionality in NGAP (common aspects)	Qualcomm Incorporated, Huawei

- R3-192473	Introduction of support for CIoT functionality in XnAP (common aspects)	Qualcomm Incorporated, Huawei

- R3-192485	LS on Mobile-terminated Early Data Transmission	RAN2, Ericsson

- R3-192497	Conclusion on 5G CIoT user plane small data solution	SA2, Qualcomm

- R3-192868	MT EDT initiation in Paging	Ericsson

- R3-192869	MT UP EDT indication in S1-AP	Ericsson

- R3-192870	Data Notification	Ericsson

- R3-192908	Further discussion on MT-EDT	LG Electronics

- R3-192909	Support of UP solution for MTC and NB-IoT connected to 5GC	LG Electronics

- R3-193017	Further discussion on enhancement: the Intra eNB CP Relocation for NB-IoT	Ericsson

- R3-193018	MT EDT indication in paging	Ericsson

- R3-193019	Discussion on general aspects of MT EDT in Msg4	Ericsson

- R3-193021	Support of restriction of use of Enhanced Coverage	Ericsson

- R3-193023	Support of extended DRX in CM-IDLE	Ericsson

- R3-193024	Support of extended DRX in CM-IDLE	Ericsson

- R3-193025	RRC_INACTIVE support for LTE-M and NB-IoT connected to 5GC	Ericsson

- R3-193026	Data notification for CIoT UEs	Ericsson

- R3-193103	Response to R3-192467	Nokia, Nokia Shanghai Bell

- R3-193151	MTC and IoT Enhancements Session Report	Vice-chaiman (ZTE)

- R3-193155	Introduction of MT Early Data Transmission	Qualcomm Incorporated, Huawei

- R3-193156	Draft Reply LS on Mobile-terminated Early Data	Ericsson

- R3-193158	pCR38413 for Support of extended DRX in CM-IDLE for NB-IoT/eMTC connected to 5GC	ZTE

- R3-193159	Introduction of NB-IoT Paging and eDRX aspects	Huawei, Qualcomm Incorporated

- R3-193160	Support of Enhanced Coverage restriction in NGAP for LTE-M Ues	Ericsson

- R3-193161	CIoT Work Structure	Huawei

- R4-1904807 Way forward on RRM impact of the R16 enhancement for NB-IoT Huawei, HiSilicon, Ericsson, Qualcomm, Nokia

- R4-1904581 Agreements for Rel-16 NB-IoT RRM enhancement Ericsson

- R4-1904679	On RRM aspects for additional NB-IoT enhancements Nokia, Nokia Shanghai Bell

- R4-1903671	Discussion on the RRM impact from group WUS Huawei, HiSilicon

- R4-1904580	Discussions on release 16 NB-IoT WUS impact on RRM Ericsson

- R4-1904578	Discussions on RRM requirements for transmissions using PUR for NB-IoT Ericsson

- R4-1903290	On TA validation for NB-IoT PUR Qualcomm Incorporated

- R4-1903672	Discussion on the preconfigured UL resource Huawei, HiSilicon

- R4-1902962	NB-IOT PUR RSRP TA Validation Design Considerations Sierra Wireless, S.A.

- R4-1903673	Discussion on the multi carrier operation for NB R16 Huawei, HiSilicon

- R4-1904579	Discussion on non-anchor carrier RRM measurements for NB-IoT Ericsson

- R4-1903885	Discussion on channel quality reporting for NB-IoT Ericsson

- R4-1907734	Way forward on R16 enhancement for NB-IoT RRM Huawei, HiSilicon, Nokia, Ericsson, Qualcomm

- R4-1906798	Agreements for Rel-16 NB-IoT RRM enhancement Ericsson

- R4-1905500	NB-IOT PUR RSRP TA Validation Design Considerations Sierra Wireless, S.A.

- R4-1905607	Discussion on the preconfigured UL resource Huawei, HiSilicon

- R4-1906795	Discussions on RRM requirements for transmissions using PUR for NB-IoT Ericsson

- R4-1906963	On NB-IoT PUR Qualcomm Incorporated

- R4-1905608	Discussion on NRS presence on non-anchor carriers Huawei, HiSilicon

- R4-1906796	Remaining discussions on non-anchor carrier RRM measurements Ericsson

- R4-1906964	On NB-IoT RRM measurements in non-anchor carrier Qualcomm Incorporated

- R4-1905609	reply LS on non-anchor carrier idle mode measurements for RRM Huawei, HiSilicon

- R4-1907735	Draft reply to LS on non-anchor carrier measurements for RRM Qualcomm Incorporated

- R4-1906410	Discussion on channel quality reporting for NB-IoT Ericsson

- R4-1906960	On MSG3 quality reporting in non-anchor carrier Qualcomm Incorporated

- R4-1903112	System level simulation results for coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation

- R4-1903113	System level simulation results for coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation (with NR channel model)

- R4-1903114	Consideration on NB-IoT operation within NR channel bandwidth

- R4-1903842	Discussion on NR and NB-IoT coexistence

- R4-1903848	Frequency band for NR and NB-IoT

- R4-1903849	NB-IoT standalone operating

- R4-1903850	NB-IoT operating in NR guard band

- R4-1903851	System scenarios of NB-IoT coexistence with NR

- R4-1903852	Channel raster, subcarrier and PRB alignment

- R4-1903853	Power boosting for coexistence of NB-IoT with NR

- R4-1904017	NB-IoT - NR coexistence : SCS based channel raster

- R4-1904018	NB-IoT - NR coexistence : BS RF topics

- R4-1904019	TP to the new TR related to NB-IoT - NR coexistence – channel raster

- R4-1905109	TR skeleton for coexistence of NB-IoT with NR

- R4-1905203	WF on coexistence of NB-IoT with NR

- R4-1905565	Clarification on NB-IoT operation modes	Huawei, HiSilicon

- R4-1905567	NB-IoT operating in NR guard band	Huawei, HiSilicon

- R4-1905568	Discussion on power boosting	Huawei, HiSilicon

- R4-1905983	NB-IoT - NR coexistence : System scenarios considerations	Ericsson

- R4-1905984	NB-IoT - NR coexistence : TDD synchronization considerations	Ericsson

- R4-1905985	NB-IoT - NR coexistence : Testability considerations	Ericsson

- R4-1905986	NB-IoT - NR coexistence : Power boosting considerations	Ericsson

- R4-1906385	Considerations on NB-IoT/NR Co-existence	Dish Network

- R4-1906586	Further aspects on coexistence scenarios between 15kHz SCS NR and in-band NB-IoT	CHTTL

- R4-1906587	Discussion on the operating scenarios for NB-IoT with NR in-band/guard band	CHTTL

- R4-1906604	Discussion on NR and NB-IoT coexistence	ZTE Corporation

- R4-1906924	System level simulation results for uplink coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation	Nokia, Nokia Shanghai Bell

- R4-1906925	System level simulation results for uplink coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation (with NR channel model)	Nokia, Nokia Shanghai Bell

- R4-1906926	TP to TR38.xxx: Summary of system level simulation results for coexistence study between R15 NR and R13/R14/R15 NB-IoT standalone operation	Nokia, Nokia Shanghai Bell

- R4-1906927	Proposals on definitions of in-band, guard band and stand-alone operations when NB-IoT is located within NR channel bandwidth	Nokia, Nokia Shanghai Bell

- R4-1906928	Proposals on power boosting requirement when NB-IoT is located within NR channel bandwidth	Nokia, Nokia Shanghai Bell

- R4-1907809	WF on co-existence NB-IoT with NR	Huawei, HiSilicon, Ericsson, ZTE, DISH, T-Mobile USA, CHTTL, Nokia

- R4-1907810	TP for NB-IoT standalone operating	Huawei, Nokia, Nokia Shanghai Bell

- R4-1907811	NB-IoT - NR coexistence : TP to TR - channel raster	Ericsson, ZTE

- R4-1907812	TP to the new TR related to NB-IoT: mixed numerologies	ZTE Corporation

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion level swith colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template