3GPP TSG RAN meeting #84										RP-190917

Newport Beach, USA, June 3-6, 2019

## Status Report to TSG

Agenda item:			9.4.4

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: Yes/No |
| Acronym | NR_2step_RACH |  |  |  |  |
| Unique ID | 820068 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190711 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2020 | Performance part: 06/2020 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 50% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Li Tian |
|  | Company | ZTE Corporation |
|  | Email | tian.li150@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

The following agreements were made in RAN1#96:

Agreements:

- PUSCH occasion for 2-step RACH is defined as

  - the time-frequency resource for payload transmission

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP,  reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is single specification fixed value. Different preambles in different PRACH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: ​follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2:  same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

Agreements:

- For the relation of PRACH resources between 2-step and 4-step RACH, further study the following options (for possible down-selection or combination(s) of the options)

  - Option 1: Separate ROs are configured for 2-step and 4-step RACH

  - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

  - Option 3: Shared RO and shared preambles for 2-step and 4-step RACH

Agreements:

- The beam association rule between SSB and RACH occasion of 4-step RACH is to be used for 2-step RACH

  - FFS beam association for PUSCH

Agreements:

- At least open loop power control for PUSCH transmission in MsgA should be supported

  - FFS PC for preamble vs. PC for PUSCH

Agreements [Post RAN1#96 Email discussion]

- Adopt the link-level simulation assumptions in the following table for the initial evaluation of feasible payload size, and for the potential down-selection of schemes, e.g.

  - whether to have shared ROs and/or preambles between 2-step RACH and 4-step RACH, and if yes the percentage for partitioning

  - whether to have the guard time between PRACH and PUSCH and/or guard period within PUSCH, and if yes the length of GT/GP

  - the mapping scheme between preamble and PUSCH+DMRS, .e.g 1-to-1, multiple-to-1, or 1-to-multiple

  - appropriate power offset(s) between preamble and PUSCH, and whether to support repetition of MsgA PUSCH

  - whether to have UCI in msgA PUSCH, if a PUCCH transmission overlaps the PUSCH part of MsgA

  - whether to dynamically adapt the payload size and indicate by UCI in msgA PUSCH, and if yes the content and structure of UCI

  - FFS other schemes, e.g. whether guard band is included

- Additional system-level simulations or analytical evaluations can be considered for the following analysis:

  - Latency

  - Signalling overhead

  - Resource reservation overhead

  - PUSCH collision, with definition FFS, e.g. overlapped PUSCH occasion, with shared or separate DMRS port, and with same or different scrambling ID

  - Rx detection complexity

- Note 1: the supported/recommended payload size from RAN1 perspective may also need to take other factors into account, e.g. use cases, resource utilization.

- Note 2: the WID scope should be strictly followed when using the evaluation results for the comparison of schemes.

Table 2: Link-level evaluation assumptions

| Parameters | Values |
| --- | --- |
| Scenario | - For evaluation of schemes: 200m, UMi, 4 GHz. FFS: 500m, UMa, 4 GHz. - For evaluation of payload size: 200m, UMi, 4 GHz; or 500m, UMa, 4 GHz; or 1732m, RMa, 700 MHz; or 25km, RMa, 700 MHz. Other values can be reported if applicable. Note: this does not restrict preamble format selection. |
| Preamble format | Company report |
| Waveform (data part) | CP-OFDM, or DFT-s-OFDM |
| Subcarrier spacing for PUSCH | 15kHz at 700MHz, 30/60kHz at 4 GHz, 120kHz at 30GHz |
| TBS | - 56 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |
| MCS and Resource size | Company report the MCS, time/frequency resource size, and DMRS overhead. Strive to agree to some common values in RAN1#96bis. |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or interference from the adjacent PUSCH resource, including how to model relative SINR, timing, etc. |
| Traffic model | Full buffer |
| UE antenna configuration | 1Tx. FFS: 2 Tx |
| gNB antenna configuration | 2Rx or 4Rx, 8Rx as optional |
| Propagation channel & UE velocity | TDL/CDL-A 30ns, or TDL/CDL-C 300ns, 3km/h or 30km/h |
| Timing offset | Uniform [0, RTT]. |
| Frequency offset | 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE |
| Max number of HARQ transmission | 1 as starting point, other values are not precluded and company should report the details of HARQ |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | Realistic for both channel estimation and TO/FO estimation. Ideal can be considered for calibration, if needed. |
| Target BLER | [10%, 1%] for 1st transmission of msgA as starting points. |
| Performance metrics | 1) Missed detection probability vs. SNR for a given false alarm rate, e.g. 0.1%; 2) BLER vs. SNR; MCL can be reported using link budget calculations. 3) Optional: False alarm probability vs. SNR |

The following agreements were made in RAN1#96bis:

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt. 1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt. 2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period

Agreements:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported

Agreements:

- Adopt the following table for the link-level evaluation:

| Parameters | Values |
| --- | --- |
| The number of PUSCH symbols & PUSCH mapping type | 14, Type A; [6], Type B as optional |
| 1) Total Number of PRBs for msgA PUSCH Or 2) number of PRBs per PUSCH occasion Note: either of them should be aligned for scheme comparison | [6, 12] Or [1,2,3] |
| PUSCH DMRS overhead | [2 or 3] DMRS symbols |
| Frequency hopping for msgA PUSCH | Company report, enabled/disabled |
| Preamble format | Format 0/[A1]; [32, 64] preambles in each RO. Other preamble formats or number of preambles are not precluded Note: company report number of SSBs per RO |
| Number of UEs | 1 as a starting point; FFS: 2 or more for evaluation of shared PUSCH occasion or can be reported Power modelling for FFS (Note: before the FFS is resolved, companies to report the detailed modelling) FFS: interference from the adjacent PUSCH resource occasion, including how to model relative SINR, timing, etc. |
| TBS | - 56 72 bits as starting point for minimum payload size, other values are not precluded - Company report for the evaluation of payload size |

Agreements:

- For the relation of PRACH resources between 2-step and 4-step RACH, the network has the flexibility to configure the following options:

  - Option 1: Separate ROs are configured for 2-step and 4-step RACH

  - Option 2: Shared RO but separate preambles for 2-step and 4-step RACH

Agreements:

- Further study the granularity of the time advance command, if supported in MsgB:

  - E.g., Based on the subcarrier spacing of MsgA PUSCH using a 12-bit TA command, where the granularity of the TA command is determined according to the following table.

| Subcarrier Spacing (kHz) of the msgA PUSCH data part | Unit |
| --- | --- |
| 15 | 16*64 Tc |
| 30 | 8*64 Tc |
| 60 | 4*64 Tc |
| 120 | 2*64 Tc |

  - Other options/variations are not precluded

Agreements:

For 2-step RACH preamble power control parameter configuration, further study and down select from the following options:

- Option 1: Power control parameters can be separately configured for 2-step and 4-step RACH.

  - If a power control parameter is not configured for 2-step RACH, the corresponding power control parameter of 4-step RACH is used instead for 2-step.

- Option 2: The corresponding power control parameter of 2-step RACH preamble follows that of 4-step RACH preamble.

Agreements:

For the determination of the PUSCH Tx power, further study at least the following components including possible down selection:

- An offset relative to the preamble received target power

  - Option 1.1: Offset configured for 2-step RACH

  - Option 1.2: Offset is the release 15 delta_preamble_msg3

  - Option 1.3: Offset is the release 15 delta_preamble_msg3 + configurable delta

- An offset relative to the MsgA PRACH Tx power for the MsgA PUSCH Tx power configured for 2-step RACH.

- Transmission bandwidth of MsgA PUSCH

- MsgA PUSCH Transport format (ΔTF). Further study the following options for further down selection

  - Option 2.1: deltaMCS configured for 2-step separate from 4-step

  - Option 2.2: reuse deltaMCS of 4-step RACH

- Preamble received target power.

- Pathloss. Further study the following options for further down selection

  - Option 4.1: Full pathloss compensation (α = 1)

  - Option 4.2: Partial pathloss compensation alpha configured for 2-step separate from that of 4-step RACH.

  - Option 4.3: Partial pathloss compensation using msg3-alpha.

- RS resource index for pathloss estimation.

- Total power ramp-up requested by higher layers for MsgA PUSCH Tx:

  - Option 6.1: from the first to the current MsgA PUSCH transmission (∆Prampuprequested).

  - Option 6.2: from the first to the latest random access MsgA preamble transmission (∆Prampuprequested).

  - Note: Latest means most recent transmitted.

- Power reduction priority rule in CA/DC

Agreements:

- For MsgA Tx beam selection further study at least the following options:

  - Option 1: The MsgA PRACH and MsgA PUSCH use the same Tx spatial filter (beam).

  - Option 2: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) up to UE implementation.

    - No spec impact expected.

    - Note: in 4-step RACH it is up to UE implementation to decide the beams for Msg1 and Msg3.

  - Option 3: The MsgA PRACH and MsgA PUSCH use same or different Tx spatial filter (beam) under network control/assistance.

- MsgA retransmission, if supported, is defined as a retransmission of MsgA PRACH (with a re-selection of preamble) and MsgA PUSCH. Further study at the following options:

  - Option 1: Using the same payload for MsgA PUSCH.

  - Option 2: MsgA PUSCH payload can be different.

  - FFS: Conditions for MsgA retransmission and relation to fall back.

- FFS: retransmission of PUSCH only.

- FFS: retransmission of PRACH only.

The following agreements were made in RAN1#97:

Agreements:

- The following parameters are defined per msgA PUSCH configuration:

  - Common parameters for both option 1 (separate configuration) and option 2 (relative location), at least include:

    - MCS and/or TBS (to be further decided)

    - Number of FDMed POs

      - POs (including guard band or guard period, if exist) under the same msgA PUSCH configurations are consecutive in frequency domain

    - Number of PRBs per PO

    - Number of DMRS symbols/ports/sequences (if support) per PO

    - FFS whether or not support repetitions for msgA PUSCH

    - FFS bandwidth of PRB-level guard band or duration of guard time

    - FFS PUSCH mapping type

  - Parameters specific to option 1, at least include:

    - Periodicity (msgA PUSCH configuration period)

      - FFS value range

    - Offset(s) (e.g., symbol, slot, subframe, etc.)

    - Time domain resource allocation, details FFS, e.g., in a slot for msgA PUSCH: starting symbol, number of symbols per PO, number of time-domain POs, etc.

    - Frequency starting point

  - Parameters specific to option 2, at least include:

    - Single time offset (combination of slot-level and symbol-level indication) with respect to a reference point

      - FFS, e.g., each PRACH slot (e.g., start or end of the PRACH slot), etc.

    - Number of symbols per PO

      - FFS explicit or implicit indication

    - Single frequency offset with respect to FFS (the start of the first RO in frequency or the end of the last RO in frequency)

  - FFS: Number of TDMed POs

- Support multiple msgA PUSCH configurations for a UE

  - FFS the maximum number of configurations

  - FFS which parameters, if any, are common for all configurations

  - FFS indication of different msgA PUSCH configurations, e.g. by different ROs, by different preamble groups, or by UCI

  - FFS whether or not resources for different msgA PUSCHs can be overlapped in time-frequency, and if so, any spec impact

- FFS whether the frequency resource of msgA PUSCH should be limited to the bandwidth of PRACH

- FFS validation rule of msgA PUSCH

Agreements:

- The c_init for msgA PUSCH scrambling is at least derived based on a RNTI, preamble index, and/or n_ID (which can be  cell ID or configurable, to be FFS).

  - FFS details of the RNTI

  - FFS the inclusion of DMRS index.

Agreements:

- MsgA shall support all the preamble formats specified for NR release 15.

Agreements:

- From RAN1 perspective, when re-transmitting MsgA, and if the MsgA PRACH is on a different spatial filter (beam) than the latest MsgA PRACH transmission, layer 1 notifies higher layer to suspend the power ramping counter of MsgA PRACH,

  - FFS: How to determine the retransmitted MsgA PUSCH Tx power.

Conclusion:

In the reply LS to the RAN2 on power ramping,

- Include the agreements right above, and

- Mention that RAN1 discussed the suspension of power ramping counter when retransmitting MsgA, and if MsgA preamble is associated with a different SSB than the latest MsgA preamble transmission. The suspension of the power ramping counter for this scenario in case of 4-step RACH is described in the RAN2 specifications. It is up to RAN2 to agree on a similar behavior for 2-step RACH.

Agreements:

- The proposals in 5.2.6 of R1-1907900 is agreed

[Copied as below, also captured in the approved LS to RAN2 R1-1907948]

During MsgA PUSCH retransmissions, the MsgA PUSCH Tx power in transmission instance i is PPUSCHi, where

PPUSCHi=maxPCMAX,[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+10log102μMRBPUSCHi+αPL(i)+∆TFi+∆rampupi

- ∆MsgA_PUSCH is an offset relative to the preamble received target power that could be configured for 2-step RACH. If the offset parameter is absent, the parameter delta_preamble_msg3 of 4-step RACH is used.

- [Working Assumption] The power component from the transport format ∆TFi is determined based on the same mechanism and the same parameter deltaMCS of Rel-15 Msg3 for the current transmission instance.

- The power component from pathloss compensation, αPLi, is determined by an alpha parameter, which is UE specific that is configured for 2-step separate from that of 4-step RACH. If the 2-step RACH alpha parameter is absent, the parameter msg3-alpha of 4-step RACH is used.

  - FFS: cell-specific MsgA PUSCH alpha.

- For the downlink pathloss estimate for MsgA PUSCH power control, the UE uses the same RS resource index as that used for the corresponding MsgA PRACH

- The power ramping component is given by;

∆rampup(i)=minmax0,PCMAX-10log102μMRBPUSCHi+[MsgA]preambleReceivedTargetPower+∆MsgA_PUSCH+αPLi+∆TFi,∆rampuprequested

  - Where, ∆rampuprequested is the requested ramp up from higher layers

  - Further study and down select from the following alternatives:

    - Alt1: Same ramp up for MsgA PUSCH and MsgA PRACH

∆rampuprequested=PREAMPLE_POWER_RAMPING_COUNTER-1×[MsgA]powerRampingStep

      - FFS: same power ramping counters for 2-step RACH MsgA PRACH and 4-step RACH Msg1.

    - Alt 2: Separate ramp up for MsgA PUSCH and MsgA PRACH, with different counters

∆rampuprequested=MSGAPUSCH_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

    - Alt3: Separate ramp up for MsgA PUSCH and MsgA PRACH, with the same counter

∆rampuprequested=PREAMBLE_POWER_RAMPING_COUNTER-1×PUSCHpowerRampingStep

Agreements:

- RACH preamble power control parameters include: powerRampingStep and preambleReceivedTargetPower.

#### 2.1.2	Remaining Open issues

Note: Further identified open issues may be added later as we proceed with the work item.

- Channel structure for msgA

  - Detailed time/frequency domain resource allocation for msgA payload

  - Detailed configuration and value range for PUSCH occasions, e.g.

    - MCS/TBS

    - Guard time and PRB-level guard band

    - Indication of multiple configurations

    - Validation rule

  - Detailed mapping design between PRACH preamble and PUSCH resource unit

    - Definition of PUSCH resource unit

    - Ordering of preamble and PUSCH resource unit

    - Mapping ratio between preamble and PUSCH resource unit

    - Association period and handling of redundant

- MsgA payload

  - Content and payload size for msgA in RRC_CONNECTED state

  - Remaining issues on the waveform and numerology for msgA PUSCH

  - Details on the scrambling ID for msgA PUSCH

  - Whether and how to support repetition, frequency hopping

- RAN1 related procedures

  - Criterion of RACH type selection (specifically, the need for radio quality criterion and if needed, the details)

  - Details on the PRACH configuration for 2-step RACH

  - SSB to RO mapping in case of shared RO between 2-step RACH and 4-step RACH

  - Beam association for PUSCH part of msgA

  - Power control for msgA

    - Detailed open loop power control for msgA

    - Detailed power ramping for preamble and/or PUSCH

  - msgB structure and response window

- Collection of simulations results

- For unlicensed operation: after PRACH and PUSCH design enhancements are completed for NR-U in the Rel-16 NR-U WI, identify and specify the necessary modification of 2-step RACH design for its application in NR-U

## 2.2	RAN2

#### 2.2.1	Agreements

The following agreements were made in RAN2#105bis:

Agreements:

- Criteria on whether the UE uses 2-step RACH or 4-step RACH shall be clearly specified

- The start of the msgB reception window is after the PUSCH transmission opportunity of msgA.  Details are FFS for 2-step RACH and fallback.

- If CCCH SDU was included in MsgA, then the contention resolution will be based on the contention resolution ID included in MsgB.  FFS for other conditions.

Approved LS to RAN1

1. RAN2 has discussed the contents and content size of MsgA data part and concluded that the following could be included into MsgA depending on the Random Access trigger:

- RRC message for requesting RRC connection setup/re-establishment/resume for IDLE/INACTIVE/CONNECTED UEs;

- RRC message for requesting on-demand system information for IDLE/INACTIVE UEs;

- C-RNTI MAC CE + possibly other MAC CE(s) like BSR + UL data (CP or UP) for CONNECTED mode UEs.

Based on the above, it was concluded that minimum payload sizes the MsgA shall be able to support are 56 and 72 bits depending on whether short or long I-RNTI is requested by the NW from RRC INACTIVE UEs inside the RRCResumeRequest message, respectively.

On the other hand, no upper bound for the content size is defined by RAN2 but any limit based on the RAN1 design would be good to be known to be able to design the signaling options properly.

2. In addition, RAN2 would appreciate any early information for the power ramping for MsgA transmission.

ACTION: 	RAN2 respectfully asks RAN1 to take the minimum MsgA content sizes into account for their design as well as provide any information about the upper bound of the MsgA content size if/when available. Furthermore, RAN2 would respect any early information for the power ramping design for MsgA transmission.

The following agreements were made in RAN2#106:

Agreements:

- From RAN2 perspective, 2-step RACH selections can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states.  FFS if radio quality is used for 2-step RACH selection.

- From RAN2 perspective, for msgA retransmission (i.e. preamble and PUSCH) we assume that the UE retries on 2-step RACH

- FFS whether the UE can fallback to 4-step RACH after certain time.  Ask RAN1 whether the preamble transmission performance for 2-step RACH and 4-step RACH is the same.

- For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI)

- Contention resolution:

  - If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already.

  - If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly.

  - If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB.

  - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced

- For CCCH, MsgB can include the SRB RRC message.  The format should be designed for both with and without RRC message.

- For CCCH, for success or fallback RAR MsgB can multiplex messages for multiple UEs.  FFS if we can multiplex SRB RRC messages of multiple UEs.

- Network response to msgA (i.e. msgB/msg2) can include the following:

  - SuccessRAR

  - FallbackRAR

  - Backoff Indication

FFS: format of successRAR and whether successRAR is split into more than one message and format of fallbackRAR and whether legacy msg2 can be reused for fallbackRAR

- Proposal 10: The following fields can be included in the successRAR when CCCH message is included in msgA.

  - Contention resolution ID

  - C-RNTI

  - TA command

- Upon receiving the fallbackRAR, the UE shall proceed to msg3 step of 4-step RACH procedure

- FallbackRAR should contain the following fields

  - RAPID

  - UL grant (to retransmit the msgA payload).  FFS on restrictions on the grant and UE behavior if different grant and rebuilding

  - TC-RNTI

  - TA command

- From RAN2 perspective, no further offset is needed for the start of msgB monitoring window (i.e. no offset is needed to cover the RRC processing delay and/or F1 delay).

- The UE will monitor for response message using the single msgB agreed window

- MsgB containing the succcessRAR shall not be multiplexed with the legacy 4-step RACH RAR in the same MAC PDU

#### 2.2.2	Remaining Open issues

Note: Further identified open issues may be added later as we proceed with the work item.

- Further stage-2 aspects of overall procedure

  - Conclude on the need for fallback to 4-step RACH after certain time (waiting for feedback from RAN1 on preamble performance)

  - For CCCH case, decide whether multiplexing multiple UEs is allowed when SRB data is included in msgB

  - For CCCH case, whether successRAR is split into more than one message

  - Whether to use legacy msg2 or the msgB for fallbackRAR

  - Aspects related to ACK for msgB reception

  - How to distinguish msgB from legacy msg2 – if needed (new coreset/searchspace vs new RNTI) – input from RAN1 discussions is also likely needed for this

- Stage 3 aspects

  - MAC aspects

    - Design of SuccessRAR with and without SRB data

    - Design of fallbackRAR

    - Design of new MAC CE for 12 bit TA (if needed per above)

    - Overall procedure for 2-step RACH

  - RRC aspects

    - Signaling (SIB and Dedicated) for 2-step RACH configuration

- For unlicensed operation: after PRACH and PUSCH design enhancements are completed for NR-U in the Rel-16 NR-U WI, identify and specify the necessary modification of 2-step RACH design for its application in NR-U

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- 3GPP, RAN1, RAN1#96 Chairman Notes

- R1-1901625 	Work plan for 2-step RACH 	ZTE

- R1-1902027 	Discussion on Channel Structure for 2-Step RACH	CATT

- R1-1902821 	Use Cases and Scenarios for 2-Step RACH, Ericsson

- R1-1903147 	Considerations on two-step RACH for NTN	THALES

- R1-1901626	Considerations on the channel structure of msgA	ZTE

- R1-1902466	Channel structure for two-step RACH	Intel Corporation

- R1-1902977	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1901669	Discussion on channel structure for 2-step RACH	vivo

- R1-1901793	On channel structure for two-step RACH	MediaTek Inc.

- R1-1902133	Channel Structure for Two-Step RACH Considerations 	Sierra Wireless, S.A.

- R1-1902135	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1902165	On Channel Structure for Two-Step RACH	Sony

- R1-1902241	Channel Structure for Two-Step RACH	Samsung

- R1-1902326	Discussion on channel structure for two-step RACH	CMCC

- R1-1902393	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1902533	Discussion on 2step RACH Structure	LG Electronics

- R1-1902582	On Signal Structure for Two-Step RACH 	InterDigital, Inc.

- R1-1902747	On Channel Structure for 2-step RACH	OPPO

- R1-1902784	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1902822	Channel Structure for Two-step RACH	Ericsson

- R1-1902917	Considerations on Resource Pool Design for PUSCH in MsgA of 2-step RACH	CAICT

- R1-1902961	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1903056	Channel structure for 2-step RACH	Huawei, HiSilicon

- R1-1902136	On 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1902785	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1903057	Discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1901627	Considerations on 2-step RACH procedures	ZTE

- R1-1901670	Discussion on 2-step RACH procedure	vivo

- R1-1901794	On procedure for two-step RACH	MediaTek Inc.

- R1-1902134	Procedure for Two-Step RACH Considerations 	Sierra Wireless, S.A.

- R1-1902166	Discussion on Procedure for 2-step RACH	Sony

- R1-1902242	Procedure for Two-step RACH	Samsung

- R1-1902327	Discussion on procedure for two-step RACH	CMCC

- R1-1902394	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1902467	Procedure  for two-step RACH	Intel Corporation

- R1-1902534	Discussion on 2step RACH Procedure	LG Electronics

- R1-1902583	Perspectives on operation of Two-Step RACH 	InterDigital, Inc.

- R1-1902748	On Procedure for 2-step RACH	OPPO

- R1-1902823	Procedure for Two-step RACH	Ericsson

- R1-1902843	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1902918	A scheme of fallback from 2-step RACH to 4-step RACH	CAICT

- R1-1902964	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1903321	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1903186	Discussion on 2-step RACH for NR	Fraunhofer HHI, Fraunhofer IIS

- R1-1903878	On the channel structure of msgA	ZTE, Sanechips

- R1-1903923	Further discussion on channel structure for 2-step RACH	Huawei, HiSilicon

- R1-1904392	Channel Structure for Two-Step RACH	Samsung

- R1-1904059	Discussion on channel structure for 2-step RACH	vivo

- R1-1904196	Channel Structure for 2-Step RACH	LG Electronics

- R1-1904246	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1904280	Channel structure for 2-step RACH	Intel Corporation

- R1-1904357	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1904542	Discussion on channel structure for 2-step RACH	CATT

- R1-1904584	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1904715	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1904724	Discussion on channel structure for two-step RACH	CMCC

- R1-1904778	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1904819	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1904842	Channel Structure for Two-step RACH	Ericsson

- R1-1904848	On PUSCH Transmission in msgA 	InterDigital, Inc.

- R1-1904944	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1904992	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1905043	Discussion on channel structure for 2-Step RACH	ITRI, NTPU

- R1-1905051	On Channel Structure for 2-step RACH	OPPO

- R1-1905125	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1905205	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1904716	On 2-step RACH Procedure	Nokia, Nokia Shanghai Bell

- R1-1904843	Procedure for Two-step RACH	Ericsson

- R1-1904993	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1903879	On 2-step RACH procedures	ZTE, Sanechips

- R1-1903924	Further discussion on 2-step RACH procedure	Huawei, HiSilicon

- R1-1904060	Discussion on 2-step RACH procedure	vivo

- R1-1904197	Discussion on 2-Step RACH Procedure	LG Electronics

- R1-1904247	Discussion on Procedure for 2-step RACH	Sony

- R1-1904281	Procedure for 2-step RACH	Intel Corporation

- R1-1904358	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1904393	Procedure for Two-step RACH	Samsung

- R1-1904725	Discussion on procedure for two-step RACH	CMCC

- R1-1904779	Considerations on the random access procedure on 2-step RACH	Spreadtrum Communications

- R1-1904820	Procedure for Two-step RACH Considerations	Sierra Wireless, S.A.

- R1-1904850	Discussion on Procedures for Two-Step RACH 	InterDigital, Inc.

- R1-1904928	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1904945	Discussion on Procedure for Two-step RACH	NTT DOCOMO, INC.

- R1-1905044	Discussion on procedure for 2-Step RACH	ITRI, NTPU

- R1-1905052	On Procedure for 2-step RACH	OPPO

- R1-1905126	Considerations on Procedure for Two-step RACH	CAICT

- R1-1905207	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1903880	Analysis on the collision probability of msgA	ZTE, Sanechips

- R1-1903881	Preliminary evaluation results for 2-step RACH	ZTE, Sanechips

- R1-1904061	Evaluation for 2-step RACH	vivo

- R1-1904198	Discussion on preamble selection for 2-Step RACH	LG Electronics

- R1-1904199	Discussion on RAR detection in 2step RACH procedure	LG Electronics

- R1-1904684	Preliminary results for 2-step RACH evaluations	Huawei, HiSilicon

- R1-1904685	Other issues for 2-step RACH	Huawei, HiSilicon

- R1-1904717	2-step RACH latency and resource reservation	Nokia, Nokia Shanghai Bell

- R1-1904844	Evaluation methodology and metrics for Two-Step RACH	Ericsson

- R1-1905515	Initial results on coverage and msgA configurations	Ericsson

- R1-1904994	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R1-1907179	Use Cases and Scenarios for 2-Step RACH	Ericsson

- R1-1905989	Further discussions on the channel structure of msgA	ZTE

- R1-1907752 	Channel Structure for Two-step RACH	Ericsson

- R1-1907691	Channel Structure for Two-Step RACH	Qualcomm Incorporated

- R1-1907673	Feature lead summary for 2-step RACH channel structure	ZTE

- R1-1906050	Discussion on channel structure of 2-step RACH	Huawei, HiSilicon

- R1-1906124	Discussion on channel structure for 2-step RACH	vivo

- R1-1906192	Discussion on Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

- R1-1906366	Considerations on the channel structure on 2-step RACH	Spreadtrum Communications

- R1-1906429	Discussion on channel structure for Two-Step RACH	Fujitsu

- R1-1906511	Discussion on Channel Structure for Two-step RACH	CMCC

- R1-1906580	On Channel Structure for 2-step RACH	OPPO

- R1-1906717	Discussion on Channel Structure for 2-step RACH	LG Electronics

- R1-1906746	On 2-step RACH Channel Structure	Nokia, Nokia Shanghai Bell

- R1-1906779	Discussion on channel structure for 2-step RACH	Intel Corporation

- R1-1906848	Considerations on Channel Structure for Two-Step RACH	Sony

- R1-1906858	On preamble and PUSCH mapping	InterDigital, Inc.

- R1-1906863	Discussion on channel structure for 2-step RACH	Panasonic Corporation

- R1-1906905	Channel Structure for Two-Step RACH	Samsung

- R1-1907127	Channel Structure for Two-Step RACH Considerations	Sierra Wireless, S.A.

- R1-1907196	Considerations on Channel Structure for Two-step RACH	CAICT

- R1-1907446	Discussion on channel structure for two-step RACH	KDDI Corporation

- R1-1905990	Further discussions on 2-step RACH procedures	ZTE

- R1-1906125	Discussion on 2-step RACH procedure	vivo

- R1-1906367	Considerations on 2-step RACH procedure	Spreadtrum Communications

- R1-1906581	On Procedure for 2-step RACH	OPPO

- R1-1906582	On recognition of msgB or msg2	Fujitsu

- R1-1906718	Discussion on Procedure for 2-step RACH	LG Electronics

- R1-1906780	Discussion on procedure for 2-step RACH	Intel Corporation

- R1-1906831	Discussion on Procedure for 2-step RACH	Sony

- R1-1906859	On 2-step RACH procedure	InterDigital, Inc.

- R1-1906864	Discussion on 2-step RACH procedure	Panasonic Corporation

- R1-1906906	Procedure for Two-step RACH	Samsung

- R1-1907123	Views on 2-step RA Procedures	Charter Communications

- R1-1907128	Procedure for Two-step RACH Considerations	Sierra Wireless, S.A.

- R1-1907181	Procedure for Two-step RACH	Ericsson

- R1-1907197	Considerations on Procedure for Two-step RACH	CAICT

- R1-1907238	2-step RACH procedure	Motorola Mobility, Lenovo

- R1-1907256	Procedures for Two-step RACH	Qualcomm Incorporated

- R1-1907447	Discussion on procedure for two-step RACH	KDDI Corporation

- R1-1905991	Simulation results for 2-step RACH	ZTE

- R1-1906126	Evaluation for 2-step RACH	vivo

- R1-1906604	Further results for 2-step RACH payload size evaluations	Huawei, HiSilicon

- R1-1906605	Evaluation results for other issues for 2-step RACH	Huawei, HiSilicon

- R1-1906748	2-step RACH latency and resource reservation	Nokia, Nokia Shanghai Bell

- R1-1906749	2-step RACH evaluation results	Nokia, Nokia Shanghai Bell

- R1-1906781	Initial evaluation results for 2-step RACH	Intel Corporation

- R1-1907182	Evaluation methodology and metrics for Two-Step RACH	Ericsson

- R1-1907183	Power control of msgA PUSCH transmissions	Ericsson

- R1-1907187	Modeling UE power differences in link simulations	Ericsson

- R1-1907677	Initial results on DMRS antenna ports and scrambling	Ericsson

- R1-1907257	Other Considerations for Two-Step RACH	Qualcomm Incorporated

- R1-1907428	Discussion on RAR and  msgB PDCCH monitoring for 2-step RACH	LG Electronics

- R1-1907495	Latency analysis on 2-step and 4-step RACH	Huawei, HiSilicon

- R1-1907540	Considerations for simultaneous RACH operation	Huawei, HiSilicon

- R1-1907541	Other issues for 2-step RACH	Huawei, HiSilicon

- R2-1903534	Workplan for 2-step RACH	ZTE Corporation, Sanechips

- R2-1904033	Draft LS on MsgA content size	Nokia, Nokia Shanghai Bell

- R2-1904377	LS on message sizes for 2-step RACH	Ericsson

- R2-1903536	LS on msgA payload size for 2-step RACH	ZTE Corporation, Sanechips

- R2-1903428	MsgA and msgB size for 2-step RACH	Ericsson

- R2-1903535	MsgA payload size for 2-step RACH	ZTE Corporation, Sanechips

- R2-1904109	Discussion on MsgA payload content	Huawei, HiSilicon

- R2-1904979	Discussion on payload size of msgA for 2-step RACH	LG Electronics

- R2-1904439	Procedure for 2-Step RACH	Intel Corporation

- R2-1903112	2 Step RA_MsgA Aspects	Samsung Electronics Co., Ltd

- R2-1903077	Considerations on the 2-step RACH Procedure	vivo

- R2-1903113	2 Step RA_MsgB Aspects	Samsung Electronics Co., Ltd

- R2-1903152	Consideration on 2-step RACH	CATT

- R2-1903246	Procedures for 2-step RACH	ETRI

- R2-1903426	2-step RACH General Aspects	Ericsson

- R2-1903794	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R2-1903914	Discussion on 2-step RACH procedure	Panasonic Corporation

- R2-1904110	Discussion on the 2-step RACH procedure	Huawei, HiSilicon

- R2-1904426	RNTI and HARQ aspects of 2-step RACH	MediaTek Inc.

- R2-1904438	Consideration of MsgA contents and size	Intel Corporation

- R2-1903073	Discussion on the Trigger Events of 2-step RACH	vivo

- R2-1904108	Discussion on supported triggers and scenarios for 2-step RACH	Huawei, HiSilicon

- R2-1904031	Applicable triggers for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1904429	SI Request and BFR triggers for 2-step RACH	MediaTek Inc.

- R2-1904428	On the contents of msgA and msgB in 2-step RACH	MediaTek Inc.

- R2-1903292	Contents for msgA and msgB	OPPO

- R2-1904032	MsgA for 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1903548	General consideration on the content of MsgB	ZTE Corporation, Sanechips

- R2-1903608	Msg-A Content of 2-step RACH	Apple

- R2-1903716	msgB design	Nokia, Nokia Shanghai Bell

- R2-1903731	Contents in MAC layer of msgA and msgB	LG Electronics Inc.

- R2-1904969	two-step RACH msgA contents and UE ID for two-step RACH	Qualcomm

- R2-1904970	two-step RACH msgB contents and RNTI for msgB	Qualcomm

- R2-1905118	Initial consideration on 2 step RACH	NTT DOCOMO INC.

- R2-1903115	2 Step RA_MAC PDU Format for MsgB	Samsung Electronics Co., Ltd

- R2-1904113	Discussion on MsgB for 2-step RACH	Huawei, HiSilicon

- R2-1904325	Discussion of MsgA and MsgB identification	CMCC

- R2-1904971	two-step RACH fall back to four-step RACH	Qualcomm Incorporated

- R2-1904210	Consideration on fall back procedure from 2-step RACH to 4-step RACH	Sony

- R2-1903114	Fallback from 2 step to 4 step RACH	Samsung Electronics Co., Ltd

- R2-1903294	Fall back mechanisms for 2-steps RACH	OPPO

- R2-1903427	Fallback for 2-step RACH	Ericsson

- R2-1903715	Fall back from 2-step RACH to 4-step RACH	Nokia, Nokia Shanghai Bell

- R2-1904071	Fall back to 4 step RACH procedure	NEC

- R2-1904324	Discussion on RACH resources and fallback from 2-step to 4-step random access	CMCC

- R2-1903075	Differentiation between 2-step and 4-step RACH	vivo

- R2-1903782	On procedure selection among 2-step and 4-step RACH	Fujitsu

- R2-1903783	On recognition of msgB or msg2	Fujitsu	R2-1904111	Discussion on the MsgA resource configuration	Huawei, HiSilicon

- R2-1904112	Discussion on RACH type switching between 2-step, 4-step RACH and CFRA	Huawei, HiSilicon	R2-1904211	PUSCH Configurations for MsgA of 2-step RACH	Sony

- R2-1904947	Random Access Resource selection procedure for 2-step CBRA	LG Electronics

- R2-1903293	Contention resolution for 2-step RACH	OPPO

- R2-1903429	RNTIs and Contention Resolution for 2-step RACH	Ericsson

- R2-1903729	Contention-based 2-Step RACH procedure	LG Electronics Inc.

- R2-1903074	Clarification on the Cntention-based 2-step RACH	vivo

- R2-1903291	Clarification on contention based 2-step RACH	OPPO

- R2-1903717	Network control of 2-step RACH	Nokia, Nokia Shanghai Bell

- R2-1903732	RNTI for msgB in 2-step RACH	LG Electronics Inc.

- R2-1904034	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1903781	Commonality of MsgB for RAR	Fujitsu

- R2-1903076	Prioritized RA Parameters for 2-step RACH	vivo

- R2-1906308	email discussion report: Procedures and mgsB content [105bis#30][NR/2-step RACH]	ZTE Corporation (email disc Rapporteur)

- R2-1905596	Procedure of contention resolution for 2-step RACH	OPPO	discussion

- R2-1905597	Considering msgB reception window in NR and NR-U	OPPO	discussion

- R2-1905598	Draft LS on MsgB reception window	OPPO

- R2-1905650	The Starting Point of the MsgB Reception Window	vivo

- R2-1905686	Discussion on 2-step RACH procedure	Spreadtrum Communications

- R2-1905720	2 Step RA_MsgB Aspects	Samsung Electronics Co., Ltd

- R2-1905753	2-step RACH procedure	CATT

- R2-1906303	Consideration on the contention resolution in 2-step RACH	ZTE Corporation, Sanechips

- R2-1906304	Ack in UL for MsgB reception	ZTE Corporation, Sanechips

- R2-1906307	Transmission of MAC SDU for SRB/DRB in 2-step RACH	ZTE Corporation, Sanechips

- R2-1906555	General 2-Step RACH procedure	LG Electronics Inc.

- R2-1906572	Completion of 2-Step RACH procedure	LG Electronics Inc.

- R2-1906661	Need for different window between fallback and contention resolution	LG Electronics Inc.

- R2-1906746	Contention resolution and SRB/DRB delivery for 2-step RACH	ETRI	discussion

- R2-1906953	RNTIs and Contention Resolution for 2-step RA	Ericsson

- R2-1906955	Back-off for 2-step RA	Ericsson

- R2-1906956	RAR and msgB window in 2-step RA	Ericsson

- R2-1907070	Contention resolution for CONNECTED mode UE	Nokia, Nokia Shanghai Bell

- R2-1907071	RNTI design for MsgB reception	Nokia, Nokia Shanghai Bell

- R2-1907073	RNTI and HARQ aspects of 2-step RACH	MediaTek Inc.

- R2-1907179	Contention Resolution in 2-step RACH	Apple

- R2-1907639	TA handling in 2-step RACH	Samsung

- R2-1907727	Discussion on MsgB reception	Huawei, HiSilicon

- R2-1907728	Discussion on the contention resolution for 2-step RACH	Huawei, HiSilicon

- R2-1907732	Discussion on the MsgB window for 2-step RACH	Huawei, HiSilicon

- R2-1907899	Contention resolution for 2-step RACH	Qualcomm Incorporated

- R2-1907909	RAR window for msgB reception	Qualcomm Incorporated

- R2-1905599	Fall back mechanisms for 2-steps RACH	OPPO

- R2-1905651	Fallback Procedure from 2-step to 4-step RACH	vivo

- R2-1905721	Fallback from 2 step to 4 step RACH	Samsung Electronics Co., Ltd

- R2-1905923	Fallback from 2-step to 4-step RA	CMCC

- R2-1906279	Fallback procedure for 2 -step RACH	Intel Corporation

- R2-1906406	2-Step RA procedure and fallback to 4-step RA	InterDigital

- R2-1906570	Fall back procedure to 4-step RACH	LG Electronics Inc.

- R2-1906755	Details on fallback mechanisms	NEC

- R2-1906759	2-step RACH overload control	Nokia, Nokia Shanghai Bell

- R2-1906954	Fall-back for 2-step RA	Ericsson

- R2-1907046	Consideration on fall back procedure from 2-step RACH to 4-step RACH	Sony

- R2-1907733	Discussion on RACH type switching between 2-step, 4-step RACH and CFRA	Huawei, HiSilicon

- R2-1907910	2-step RACH fall back to 4-step RACH	Qualcomm Incorporated

- R2-1908021	Open aspects of the fallback operation	ZTE Corporation

- R2-1905600	msgB contents and formats in 2-step RACH	OPPO	discussion

- R2-1905652	A Single MsgB Format for both RRC CONNECTED and IDLE&INACTIVE UE	vivo

- R2-1905722	2 Step RA_MAC PDU Format for MsgB	Samsung Electronics Co., Ltd

- R2-1905754	Reception of Msg2 and msgB	CATT

- R2-1906280	Consideration of MsgB contents and design	Intel Corporation

- R2-1906306	MsgB content and format	ZTE Corporation, Sanechips

- R2-1906420	RA response for 2-step RACH	ETRI	discussion

- R2-1906577	Considerations on msgB design	LG Electronics Inc.

- R2-1906579	RNTI design for msgB	LG Electronics Inc.

- R2-1906952	Multiplexing and msgB in 2-step RA	Ericsson

- R2-1907731	Discussion on MsgB content for 2-step RACH	Huawei, HiSilicon

- R2-1907911	msgB content in 2-step RACH	Qualcomm Incorporated

- R2-1905601	RACH initialization and resource selection for 2-step RACH	OPPO

- R2-1905644	Criteria on RACH type selection	OPPO

- R2-1905653	Selection Between 2-step and 4-step RACH	vivo

- R2-1905654	Differentiation Between 2-step and 4-step RACH	vivo

- R2-1905655	Discussion on the MsgA Resource Selection	vivo

- R2-1905719	2 Step RA_MsgA Aspects	Samsung Electronics Co., Ltd

- R2-1905922	System information related aspects for 2-step RACH resources configuration	CMCC

- R2-1906256	On procedure selection among 2-step and 4-step RACH	Fujitsu

- R2-1906257	On recognition of msgB or msg2	Fujitsu

- R2-1906305	Consideration on the load balance between 2-step RACH and 4-step RACH	ZTE Corporation, Sanechips, CATT

- R2-1906580	Criteria for 2-step RACH selection	LG Electronics Inc.

- R2-1906957	Selection of 2-step or 4-step RA	Ericsson

- R2-1907047	PUSCH Configurations for MsgA of 2-step RACH	Sony

- R2-1907725	Discussion on the MsgA resource selection	Huawei, HiSilicon

- R2-1907730	Discussion on the MSGA resource configuration	Huawei, HiSilicon

- R2-1907913	Criteria for selecting 2-step RACH and 4-step RACH	Qualcomm Incorporated

- R2-1907914	2-step RACH for CFRA BFR	Qualcomm Incorporated

- R2-1907925	Random Access Resource selection procedure for 2-step RACH	LG Electronics

- R2-1905595	Clarification on contention free 2-step RACH	OPPO	discussion

- R2-1905656	Considerations on the MsgA Without PRACH Transmission	vivo	discussion

- R2-1905657	Prioritized RA Parameters for 2-step RACH	vivo	discussion

- R2-1905920	Discussion of MsgA content for different cases	CMCC

- R2-1905921	PUSCH segmentation and repetition	CMCC

- R2-1906574	Need for RAPID MAC CE in payload of msgA	LG Electronics Inc.

- R2-1907726	Discussion on MsgA transmisison	Huawei, HiSilicon

- R2-1907729	Discussion on power control for MsgA	Huawei, HiSilicon

- R2-1907734	Backoff for two-step random access	Huawei, HiSilicon

- R2-1907735	RACH differentiation for two-step random access	Huawei, HiSilicon

- R2-1907736	Discussion on supported triggers and scenarios for 2-step RACH	Huawei, HiSilicon

- R2-1907919	Discussion on scope of 2-step RACH WI	LG Electronics

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template