3GPP TSG RAN Meeting #84									  RP-191110

New Port Beach, US, June 3-6, 2019

Agenda Item:	9.4.9

Source:	Apple Inc.

Title:	On Mobility in FR2

Document for:	Discussion/Decision

# Introduction

The NR Mobility Enhancement Work Item [1] was approved in 3GPP RAN #80 meeting. An objective of [1] is “to study solution(s) to improve HO/SCG change reliability and robustness especially considering challenges in high/med frequency”. RAN2 made some progress on conditional HO to improve the mobility robustness in general; however, how the challenges specific to high/med frequency still have not been clearly addressed.

In this contribution, we further highlight the importance on FR2 (high/med frequency) mobility, and propose RAN2 to assign a dedicated sub-agenda item for FR2 scenario under NR mobility agenda item in the future RAN2 meetings.

# Discussion

NR is expected to work in the frequency range higher than 24 GHz known as FR2 [2]. UE beams are directional in FR2 and therefore, when a UE moves or rotates it may experience rapid signal degradation. Such rapid degradation will result in increased signalling, a high frequency of HO and associated failure rates, and finally, more ping-pong between cells. Therefore, HO is considerably more challenging in NR at FR2 than in LTE.

In RAN2#105, RAN2#105bis and RAN2#106, we provided simulation results showing the performance of conditional handover for NR UEs using directional antenna and operating in FR2 [3][4][5]. We particularly studied the impact of slow rotations of UE on the number of HO attempts, handover failure rate, and number of measurement reports, and observed that [6]:

Observation 1: UEs may send a large number of measurement reports due to fast changes in RSRP in FR2 stemming from signal directionality.

Observation 2: Number of attempted HO is large in FR2 due to rapid rate of change in RSRP on account of signal directionality.

Observation 3: The HoF rate due to RLF using NR release 15 mobility solutions is high in FR2.

We showed some advantages and also issues of deploying CHO in NR at FR2.

Observation 4: CHO decreases HoF rate in FR2 but that HoF rate is still significantly high.

In RAN2#105bis, some agreements on CHO were made including the followings as captured in meeting report, however no real progress was made on the aspect of high frequency mobility:

Agreements:

0:	CHO is introduced in NR to solve robustness/reliability issue.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

Agreements:

0:	CHO is introduced in NR to solve robustness/reliability issue.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

In RAN2#106, further agreements were made on CHO details, but there is still no progress on high frequency mobility, even though there were quite some contributions submitted to RAN2#106 in [6] - [11]. And RAN2 allocated an email discussion on beam specific aspects of CHO until next RAN2 meeting, hopefully some progress on high frequency mobility could be made then.

We believe in order to facilitate the progress on this specific issue of mobility enhancement in high/med frequency (FR2), adequate time allocation for RAN2 online discussion on this subject needs to be guaranteed. Therefore, we propose followings:

Proposal 1: RAN2 to assign a dedicated sub-agenda item under “Handover robustness improvements” for NR Mobility enhancement work item from next RAN2 meeting.

Proposal 2: To improve the performance of CHO, particularly in FR2, RAN2 should take into account but not limit its study to the following issues:

- Large number of measurement reports needed to prepare sufficient number of neighboring cells within a suitable period of time

- High handover failure rate due to RLF

- Large amount of RRC signaling due to a large number of HO attempts

# Conclusion

In this contribution, we further highlight the importance of FR2 (high/med frequency) mobility, and propose followings:

Proposal 1: RAN2 to assign a dedicated sub-agenda item under “Handover robustness improvements” for NR Mobility enhancement work item from next RAN2 meeting.

Proposal 2: To improve the performance of CHO, particularly in FR2, RAN2 should take into account but not limit its study to the following issues:

- Large number of measurement reports needed to prepare sufficient number of neighboring cells within a suitable period of time

- High handover failure rate due to RLF

- Large amount of RRC signaling due to a large number of HO attempts

# References

- RP-181433, “New WID: NR mobility enhancements,” Intel Corporation, 3GPP RAN #80, June 2018

- TS38.101-2 v15.2.0, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” 3GPP, June 2018

- R2-1901546, “On NR Mobility in FR2”, Apple Inc., 3GPP RAN2#105, February 2019

- R2-1904980, “Performance of Conditional HO in FR2”, Apple Inc., 3GPP RAN2 #105bis, April 2019

- R2-1904344, “Consideration of Beamforming for NR Conditional Handover”, CMCC, 3GPP RAN2#105bis, April 2019

- R2-1907216, “Enhancements for Conditional Handover”, Apple Inc., 3GPP RAN2#106, May 2019

- R2-1905949, “Consideration of Beamforming for NR Conditional Handover”, CMCC, 3GPP RAN2#106, May 2019

- R2-1906092, “Beam level trigger for conditional handover”, vivo, 3GPP RAN2#106, May 2019

- R2-1906218, “On FR2 impact on CHO”, Ericsson, 3GPP RAN2#106, May 2019

- R2-1908030, “Further consideration of beam quality on NR CHO execution”, Samsung, 3GPP RAN2#106, May 2019

- R2-1907266, “Beam selection and consolidation enhancements”, Nokia, Nokia Shanghai Bell, 3GPP RAN2#106, May 2019