3GPP TSG RAN meeting #84										RP-191139

Newport Beach, USA,  June 3rd – 6th , 2019

Status Report to TSG

Agenda item:		9.4.6

| WI / SI Name | Study on UE Power Saving in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UE_pow_sav_CORE |  |  |  |  |
| Unique ID | 830175 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190727 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2019 | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: | Core part: 60% | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Fang-Chen Cheng |
|  | Company | CATT |
|  | Email | fcc@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

    - Agreements

Agreements made in RAN1#96bis meeting (Xi’an, China, 08th – 12th  April 2019):

Agreements:

- The PDCCH-based power saving signal/channel is UE-specifically configured.

- The DCI format(s) contain information for (including potential down-selection, which may or may not depend on power saving techniques/scenarios):

  - Alt 1: triggering a single UE only

  - Alt 2: triggering UE(s) within a group

    - FFS whether to always trigger all UEs in a group or a subset of it

  - Alt 3: Alt 1 & Alt 2

Agreements:

The assumptions of the DCI design of the PDCCH-based power saving signal/channel include:

- No increase of DCI format size budget

  - FFS whether or not the same or different sets of DCI format sizes for Active time vs. out of Active time

- Working assumption: no increase of UE BD/non-overlapping CCE limit

Agreements:

Possible candidates of DCI format design for the PDCCH-based power saving signal/channel (including potential down-selection, which may or may not depend on power saving techniques/scenarios):

- New DCI format(s)

  - The size of new DCI format may or may not be the same size as the existing DCI size

- Enhancement of existing DCI. E.g.,:

  - Additional new field(s)

  - Using the existing DCI format for the power saving purpose

    - Re-purpose field(s) in the DCI

    - The detection of existing DCI format as the indication for the power saving technique

Agreements:

The DCI format(s) of the PDCCH-based power saving signal/channel is designed to provide:

- Indication of one or more power saving associated operations.

- Supporting configurability (e.g., 0, 1, 2, …bits, etc.), if needed, for one or more information fields in the DCI at least for one DCI format

Agreements:

Potential DCI contents in DCI format(s), to be further investigated:

- Power saving technique associated with C-DRX–

  - Essential for UE function for the C-DRX

    - Wakeup –

      - UE is indicated to transition from outside Active Time to Active Time

      - UE is indicated to stay at Active Time

    - Go to sleep–

      - UE is indicated to transition from Active Time to outside Active Time.

      - UE is indicated to stay outside Active Time

    - FFS: The time of receiving the wakeup and go-to-sleep indication inside or outside Active Time.

- Cross-slot scheduling

- Triggering RS transmission

- CSI report

- Single vs. multi-cell operation

- BWP /SCell

  - BWP & SCell together

  - BWP and SCell have separated fields

- MIMO layer adaptation/number of Anenna adaptation

  - May further depend on RAN4’s work

- Indication of CORESET/search space/candidate of subsequent PDCCH decoding

- PDCCH monitoring periodicity

- PDCCH skipping (skipping duration)-

  - PDCCH skipping – UE is indicated to skip number of the PDCCH monitoring occasions and stays in the Active Time

Note that

- For the bullets in italic, there are concerns that some of which may have dependence on the ongoing SI in RAN2.

- For the last two bullets, there are additional concerns that these are deemded by some companies to be not in the scope of the power saving WI approved so far

The following candidates may be discussed after RAN2’s SI is completed:

- Skipping number of DRX monitoring

- SPS activation

- DRX configuration

Agreements:

- The design of the DCI format(s) and size needs to account for one or more of the following aspects

  - Within or outside Active Time

  - DCI format size for the power saving signal/channel to fit the DCI format size budget

    - Including aspects whether or not it is necessary to align it with existing DCI format size

  - Other aspects are not precluded

Agreements:

- Support at least one CORESET with configured search space(s) for the power saving signal/channel

  - FFS separate vs. shared with a CORESET (and/or search space(s)) configured for other purposes (when applicable)

Agreements:

- For PDCCH-based power saving signal/channel,

  - The set of AL(s) is configured

  - The number of PDCCH candidate(s) for each AL is configured

Agreements:

- Regarding aperiodic CSI-RS triggering, at least if a UE is operated with cross-slot scheduling based power saving,

  - If all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states and the PDCCH SCS is equal to the CSI-RS SCS, specification allows the aperiodic CSI-RS triggering offset to be set to a non-zero value.

Agreements:

- For an active DL and an active UL BWP, a UE can be indicated via signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured) where the signalling type is to be down-selected from:

  - Alt 1: MAC-CE based

  - Alt 2: L1 based

- FFS: How to determine the minimum applicable value if explicit value is not provided.

Agreements:

Possible candidate indication methods to adapt the minimum applicable value of K0 (or K2) for an active DL (or UL) BWP, where the indication method is to be selected from:

- Alt 1: Indication of a subset of TDRA entries, e.g., bit-map based indication

- Alt 2: Indication of one active table from multiple configured TDRA tables

- Alt 3: Indication of the minimum applicable value

- Note: Other option is not precluded

Note: PDCCH monitoring case 1-1 is prioritized for the design.

FFS: Whether and how the minimum applicable K0 (or K2) value of the active DL (or UL) BWP is also applied to cross-BWP scheduling

Agreements:

Possible candidate indication methods to adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, where the indication method is to be selected from:

- Alt 1: Implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated

- Alt 2: Indication of the minimum applicable value

- Note: Other option is not precluded

Note: PDCCH monitoring case 1-1 is prioritized for the design.

Agreements:

- The adaptation on the minimum applicable value of K0 does not apply to at least the following cases:

| RNTI | PDCCH search space |
| --- | --- |
| SI-RNTI | Type0 common |
| SI-RNTI | Type0A common |
| RA-RNTI, TC-RNTI | Type1 common |
| P-RNTI | Type2 common |

Conclusion:

Companies are encouraged to further investigate whether to apply the adaptation for at least the following cases:

- K0 related:

| RNTI | PDCCH search space |
| --- | --- |
| C-RNTI, CS-RNTI |  |
| MCS-C-RNTI |  |

- K2 related:

| RNTI | PDCCH search space |  |
| --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in subclause 8.2 of TS 38.213 |  |  |
| C-RNTI, TC-RNTI, CS-RNTI |  |  |
| MCS-C-RNTI |  |  |

Agreements made in RAN1#97 meeting (Reno, US, 13th  – 17th  May 2019):

Agreements:

The monitoring occasion(s) of the power saving signal/channel outside the Active Time is “indicated” to the UE by the gNB with an offset before the DRX ON

- “Indicated” implies the explicit signalling by higher layer signalling or implicit through the CORESET/search space

- FFS: The value and the range of offset

Agreements:

For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format, with the power saving information for the UE in the DCI configurable by RRC

- FFS whether the DCI is in UESS or CSS or both

- FFS detailed configuration of the power saving information

- FFS the new DCI format

- Note: the same DCI may carry power saving information for one or more UEs

Agreements:

The indication of at least one power saving technique(s) is supported at least by the enhancement of existing scheduling DCI formats with additional field(s), if any, and/or repurposing the existing field(s), if identified, when UE is in the Active Time.

- It applies to UE-specific search space.

  - It is FFS for the common search space.

- The at least one power saving technique(s) includes at least “Cross-slot scheduling”

- FFS: Which existing DCI formats includes the power saving information

  - Whether power saving information is not included in the fallback DCI(s) (e.g., DCI format 0_0, DCI format 1_0)

  - Use of non-scheduling DCI formats.

- It is FFS which field(s) is used to be repurposed for the indication of the power saving technique if the repurpose of existing field(s) is used.

- FFS: New DCI format with size aligned with existing DCI format

Agreements:

A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured

- FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

Agreements:

Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time

- Following the principle of Rel-15 search space configuration

- FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time

- FFS whether UE can further monitor the search space set(s) inside Active Time

Agreements:

- The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring

  - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring)

  - FFS whether or not to increase the number of CORESETs relative to that in Rel-15

  - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel

R1-1907939 LS to RAN2 on RAN1 agreements on the PDCCH-based power saving signal/channel design  RAN1 (CATT)

Agreements:

- For outside Active Time, up to [3] CORESETs per BWP is supported for the power saving signal/channel outside Active Time with each CORESET associated with its TCI state and QCL assumption

  - FFS details

  - FFS whether any other additional handling is necessary for FR2 w.r.t. TCI state, and if so, how

Agreements:

Outside Active Time, the PDCCH-based power saving signal/channel is configured for triggering UE to or not to monitor the subsequent ON duration(s)

- FFS a single vs. multiple durations, particularlly checking consistency with RAN2 agreements

Further study in the new DCI how to potentially indicate at least the following techniques (subject to further WID update):

- Indicating UE to use the aperiodic RS

  - Aperiodic CSI-RS

  - Aperiodic SRS

  - TRS

- Triggering aperiodic CSI report

- Cross-slot scheduling

- Rel-15 DCI-based BWP switching

The power saving techniques can be explicitly included in the DCI contents or implicitly indicated by other techniques (e.g., BWP switching).

Agreements:

For an active DL and an active UL BWP, a UE can be indicated via L1-based signalling(s) from gNB to adapt the minimum applicable value(s) of K0, K2 and/or aperiodic CSI-RS triggering offset (with/without QCL_typeD configured).

Agreements:

To adapt the minimum applicable value of K0 (K2) for an active DL (UL) BWP, indication of the minimum applicable value is supported.

- FFS: Direct assignment of the minimum application value, indication of one value from one or multiple preconfigured or predetermined value(s), and/or implicit indication.

- FFS: How the indicated minimum applicable value is applied to the selection of a DL (UL) TDRA entry. Example directions include at least the following:

  - Excluding the invalid TDRA entries

  - Re-interpret the selected K0 (K2) value

Agreements:

When UE is indicated of the minimum applicable value of K0 (K2) for an active DL (UL) BWP, the application method to the selection of a DL (UL) TDRA entry is to be decided from:

- An entry in the active DL (UL) TDRA table with K0 (K2) value smaller than the indicated minimum is not expected by or not valid for the UE for the TDRA indication(s)

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to PUSCH scheduled by MAC RAR for at least contention-based RACH procedure.

- FFS: Exclude contention-free RACH

- FFS: Exclude PUSCH scheduled with TC-RNTI

Agreements:

- To adapt the minimum applicable value of the aperiodic CSI-RS triggering offset for an active DL BWP, implicit indication by defining the minimum applicable value the same as the minimum applicable K0 value when indicated is supported.

Agreements:

At least for the L1-based adaptation on the minimum applicable value of K2, it does not apply to:

- PUSCH scheduled by RAR UL grants for contention-free RACH procedure

- PUSCH scheduled with TC-RNTI

Conclusion:

Companies are encouraged to check the following proposal for the application delay:

For an active DL and an active UL BWP, when UE is indicated by L1-based signalling(s) in slot n to change the minimum applicable value(s) of K0 and/or K2, UE is not expected to apply the new indicated minimum applicable value(s) before slot n+X⋅2μPDSCH2μPDCCH for K0, or slot n+X⋅2μPUSCH2μPDCCH for K2, where

- X = max(Y, Z)

- Y is the minimum applicable K0 value prior to the indicated change

- Z = [1]

  - Z is the smallest feasible non-zero application delay that may depend on DL SCS

  - FFS: Z > 1 for 60kHz/120kHz SCS or multi-TRP

- FFS: Cross-carrier scheduling with different numerologies

- FFS: interruption time, if any

2.1.2	Remaining Open issues

- New DCI format size and the content for the PDCCH-based power saving signal/channel

- CORESET configuration of PDCCH-based power saving signal/channel

  - UESS or CSS

  - Multi-beam configuration

- Search space of the PDCCH-based power saving signal/channel

- General procedure of PDCCH-based power saving signal/channel triggering UE adaptation to the power saving techniques

- Detail parameter setting of dynamic switching of cross-slot scheduling

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

R4-1907615 WF on swiching and interruption time for MIMO layer adaptation for power saving

It was agreed to use BWP requirement structure as a starting point. The following open issues were identified for further discussion.

- Switching time for MIMO layer adaption

Final delay (y) to be specified

PDCCH symbols

MIMO layer switching CC

time

DCI parsing

New parameters calculating and loading

Applying the new parameters

Start of slot n+y for new MIMO layer configuration

Start of slot n carrying DCI switching command

Potential  interruption

MIMO layer switch delay

  - To ensure power saving gain of MIMO layer adaption, it is anticipated that UE shall have the ability to switch ON/OFF its transmitter/receiver and apply the new MIMO layer configurations within a specific time delay.

  - The MIMO layer adaption delay could be tentatively defined as [the time duration from end of the control symbol carrying switching command to the start of the new MIMO layer configurations in slot n+Y].

  - 3 time components are considered to be relevant for MIMO layer adaption, e.g.

    - The time required for PDCCH decoding time

    - New parameter calculating and loading

    - Applying the new parameters

- Interruption time

  - During the process of MIMO layer adaption, the ON/OFF switch of UE RF chain may cause potential interruption to the ongoing communications on active RF chains.

2.4.2	Remaining Open issues

- Detail of RF switching time

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTx

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#96bis

- R1-1905367	Work plan of UE power saving in NR	CATT

- R1-1904317	On PDCCH-based power saving signal	Intel Corporation

- R1-1904738	Discussion on power saving signal triggering UE adaptation	CMCC

- R1-1905368	PDCCH based power saving signal/channel design	CATT

- R1-1905384	PDCCH-based power saving signal/channel	Nokia, Nokia Shanghai Bell

- R1-1903988	PDCCH-based power saving signal/channel	Huawei, HiSilicon

- R1-1904103	PDCCH-based power saving signal/channel design	vivo

- R1-1904461	On PDCCH-based power saving signal/channel	Samsung

- R1-1904487	Design of PDCCH-based power saving signal/channel for NR	MediaTek Inc.

- R1-1904635	Discussion on PDCCH-based power saving signal/channel	LG Electronics

- R1-1904799	Discussion on PDCCH-based power saving signal	Spreadtrum Communications

- R1-1904902	PDCCH-based power saving signal/channel for UE adaptation	ASUSTEK COMPUTER

- R1-1904985	Design of PDCCH-based power saving signal and channel	Apple Inc.

- R1-1905031	PDCCH-based power saving channel design	Qualcomm Incorporated

- R1-1905045	Discussion on PDCCH-based power saving signal	ZTE Corporation

- R1-1905050	Discussion on PDCCH-based power saving signal/channel	OPPO

- R1-1905185	Discussion on PDCCH-based power saving signal/channel triggering adaptation	Panasonic

- R1-1905409	PDCCH-based Power Saving Signal Design Considerations 	InterDigital, Inc.

- R1-1905411	Considerations on PDCCH-based power saving signal/channel	NEC Corporation

- R1-1905417	PDCCH-based power saving channel	Motorola Mobility, Lenovo

- R1-1905428	Discussion on PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-1905466	Design of PDCCH-based power saving signal/channel	Ericsson

- R1-1905474	Triggering UE adaptation using a PDCCH-based power saving channel	Sony

- R1-1904488	Enabling cross-slot scheduling for NR	MediaTek Inc.

- R1-1904986	Cross-slot scheduling for UE power saving	Apple Inc.

- R1-1905032	Cross-slot scheduling power saving techniques	Qualcomm Incorporated

- R1-1903987	Procedure of cross-slot scheduling for UE power saving	Huawei, HiSilicon

- R1-1904104	Procedure of cross-slot scheduling power saving techniques	vivo

- R1-1904318	Support cross-slot scheduling for power saving	Intel Corporation

- R1-1904462	On corss-slot scheduling power saving techniques	Samsung

- R1-1904636	Discussion on cross-slot scheduling for power saving	LG Electronics

- R1-1904803	Discussion on cross-slot scheduling for UE power saving	Spreadtrum Communications

- R1-1904903	Cross-slot scheduling for UE power saving	ASUSTEK COMPUTER (SHANGHAI)

- R1-1905046	Procedure of cross-slot scheduling power saving technique	ZTE Corporation

- R1-1905049	Cross-slot Scheduling Procedure for Power Saving	OPPO

- R1-1905186	Discussion on procedure of cross-slot scheduling power saving techniques	Panasonic

- R1-1905369	Power saving scheme with cross-slot scheduling	CATT

- R1-1905385	Procedure of cross-slot scheduling power saving techniques	Nokia, Nokia Shanghai Bell

- R1-1905410	On Cross-slot Scheduling for UE Power Saving 	InterDigital, Inc.

- R1-1905429	Discussion on procedure of cross-slot scheduling power saving techniques	NTT DOCOMO

- R1-1905467	Procedure for cross-slot scheduling technique	Ericsson

- R1-1904105	Discussion on UE adaptation in antenna domain	vivo

- R1-1904106	Consideration on RAN1 recommendation for power saving study	vivo

- R1-1904120	UE assistant information	vivo

- R1-1904121	UE Power Consumption Reduction in RRM Measurement	vivo

- R1-1904463	On UE assistance information	Samsung

- R1-1904464	Evaluation results for power saving schemes	Samsung

- R1-1904489	Recommendations on UE adaptation	MediaTek Inc.

- R1-1904702	UE dynamic adaptation to the maximum number of receive antennas	Huawei, HiSilicon

- R1-1904739	Discussion on dynamic adaption of power saving states	CMCC

- R1-1904798	Consideration on NR RRM for UE power saving	Spreadtrum Communications

- R1-1905047	Discussion on potential techniques for UE power saving	ZTE Corporation

- R1-1905370	Power saving schemes for UE adaptation to the BWP/SCell operation	CATT

- R1-1905371	PDCCH skipping and switching of PDCCH monitoring periodicity	CATT

- R1-1905372	Power saving scheme with UE adaptation to maximum MIMO layer	CATT

- R1-1905373	UE assistance information	CATT

- R1-1905453	Reduction of PDCCH monitoring for UE power saving	Huawei, HiSilicon

- R1-1905454	On the reduction of SSB measurements	Huawei, HiSilicon

- R1-1905468	PS considerations from high-level modeling	Ericsson

- R1-1905469	RRM measurement aspects of NR UE power saving	Ericsson

- R1-1905470	PDCCH monitoring related aspects of power saving	Ericsson

- R1-1905471	Discussion on MIMO layers	Ericsson

- R1-1905472	Discussion on UE assistance	Ericsson

- R1-1905473	Cross-slot scheduling for group paging	Ericsson

RAN1#97

- R1-1906005	PDCCH-based power saving signal/channel	Huawei, HiSilicon

- R1-1906170	PDCCH-based power saving signal/channel	vivo

- R1-1906980	On PDCCH-based power saving signal/channel	Samsung

- R1-1907294	PDCCH-based power saving channel design	Qualcomm Incorporated

- R1-1905988	DCI format design for PDCCH-based Power Saving Signal/Channel	NEC

- R1-1906227	Discussion on PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-1906350	Power saving signal/channel  design and performance	CATT

- R1-1906373	Discussion on PDCCH-based power saving channel	Spreadtrum Communications

- R1-1906524	Discussion on PDCCH-based power saving signal/channel design	CMCC

- R1-1906548	Design of PDCCH-based power saving signal/channel for NR	MediaTek Inc.

- R1-1906579	Discussion on PDCCH-based power saving signal/channel	OPPO

- R1-1907688	Discussion on PDCCH-based power saving signal	ZTE

- R1-1906695	Discussion on PDCCH-based power saving signal/channel	LG Electronics

- R1-1906819	Considerations on PDCCH-based power saving signal	Intel Corporation

- R1-1906856	Issues related to PDCCH-based power saving channel	Sony

- R1-1906877	PDCCH-based power saving signal/channel for UE adaptation	ASUSTEK COMPUTER

- R1-1907104	PDCCH-based Power Saving Signal Design	InterDigital, Inc.

- R1-1907322	Design of PDCCH-based power saving signal/channel	Ericsson

- R1-1907345	Design of PDCCH-based power saving signal and channel	Apple Inc.

- R1-1907363	Discussion on PDCCH-based power saving signal/channel (PoSS)	Panasonic

- R1-1907375	PDCCH-based power saving signal/channel	Nokia, Nokia Shanghai Bell

- R1-1907467	Go-To-Sleep Signaling for Power Savings	Convida Wireless

- R1-1906578	Cross-slot Scheduling Procedure for Power Saving	OPPO

- R1-1906640	Procedure of cross-slot scheduling power saving techniques	ZTE

- R1-1906857	On cross-slot scheduling for UE power saving	Sony

- R1-1907376	Procedure of cross-slot scheduling power saving techniques	Nokia, Nokia Shanghai Bell

- R1-1906006	Procedure of cross-slot scheduling for UE power saving	Huawei, HiSilicon

- R1-1906171	Discussion on cross-slot scheduling power saving techniques	vivo

- R1-1906228	Discussion on procedure of cross-slot scheduling power saving techniques	NTT DOCOMO

- R1-1906351	Power saving scheme with cross-slot scheduling	CATT

- R1-1906372	Discussion on cross-slot scheduling for UE power saving	Spreadtrum Communications

- R1-1906549	Design of cross-slot scheduling based power saving for NR	MediaTek Inc.

- R1-1906696	Discussion on cross-slot scheduling for power saving	LG Electronics

- R1-1906820	On cross-slot scheduling operation for power saving	Intel Corporation

- R1-1906878	Adaptation of TDRA table	ASUSTEK COMPUTER (SHANGHAI)

- R1-1906981	On corss-slot scheduling power saving techniques	Samsung

- R1-1907105	On Cross-slot Scheduling for UE Power Saving	InterDigital, Inc.

- R1-1907249	Cross-slot scheduling for power saving	Motorola Mobility, Lenovo

- R1-1907295	Cross-slot scheduling power saving techniques	Qualcomm Incorporated

- R1-1907323	Procedure for cross-slot scheduling technique	Ericsson

- R1-1907346	Cross-slot scheduling technique for UE power saving	Apple Inc.

- R1-1907366	"Discussion on procedure of cross-slot scheduling power saving techniques"	Panasonic

- R1-1906172	Discussion on PDCCH monitoring skipping and PDCCH monitoring periodicity switch	vivo

- R1-1906173	Discussion on UE assistance information	vivo

- R1-1906174	UE Power Consumption Reduction in RRM Measurement	vivo

- R1-1906175	Evaluation on mobility impact of RRM measurement relaxation	vivo

- R1-1906176	Discussion on MIMO layer adaptation	vivo

- R1-1906352	Power saving schemes for UE adaptation to the BWP/SCell operation	CATT

- R1-1906353	PDCCH skipping and switching of PDCCH monitoring periodicity	CATT

- R1-1906354	Power saving scheme with UE adaptation to maximum MIMO layer	CATT

- R1-1906355	UE assistance information	CATT

- R1-1906376	Consideration on NR RRM for UE power saving	Spreadtrum Communications

- R1-1906525	Discussion on the impact of waking up signal on BWP switching operation	CMCC

- R1-1906550	Recommendations on UE adaptation	MediaTek Inc.

- R1-1906591	UE dynamic adaptation to the maximum number of receive antennas	Huawei, HiSilicon

- R1-1906592	Reduction of PDCCH monitoring for UE power saving	Huawei, HiSilicon

- R1-1906641	Discussion on potential techniques for UE power saving	ZTE

- R1-1906697	Discussion on MIMO layer/number of antenna adaptation	LG Electronics

- R1-1906698	Discussion on PDCCH monitoring adaptation	LG Electronics

- R1-1906982	On UE assistance information	Samsung

- R1-1906983	Evaluation results for power saving schemes	Samsung

- R1-1907311	Measurement and QCL assumption for UE power saving	ITRI

- R1-1907324	PS considerations from UE energy consumption profiling	Ericsson

- R1-1907325	RRM measurement aspects of NR UE power saving	Ericsson

- R1-1907326	PDCCH monitoring related aspects of power saving	Ericsson

- R1-1907327	Discussion on antenna adaptation	Ericsson

- R1-1907328	Cross-slot scheduling for UE power saving in paging	Ericsson

- R1-1907329	Discovery and mitigation of WUS miss detection	Ericsson

- R1-1907377	Detection Performance of PDCCH Power Saving Channel	Nokia, Nokia Shanghai Bell

- R1-1907478	On UE Assistance for NR UE Power Savings	Ericsson

- R1-1907518	BWP/SCell operation for UE power saving	Huawei, HiSilicon

- R1-1907519	On UE assistance information	Huawei, HiSilicon

- R1-1907520	Reducing RRM measurements for UE power saving	Huawei, HiSilicon

- R1-1907521	UE power saving for paging	Huawei, HiSilicon

- R1-1907522	CC group operation for UE power saving	Huawei, HiSilicon

RAN4#91

- R4-1905380	Work plan for power saving WI	CATT

- R4-1905381	Discussion on RF aspects related to dynamic adaption for power saving  CATT

- R4-1905530 	Evaluation of switching and interruption time for UE antenna switching     Source: vivo

- R4-1905879 	UE RF tuning time for MIMO layer switching       Ericsson

- R4-1906660 	Discussion on UE dynamic adaptation to the maximum number of MIMO layers          MediaTek

- R4-1907615 	WF on switching and interruption time for MIMO layer adaptation for power saving  CATT.

- R4-1905531 	[DRAFT]LS out on the switching and interruption time for UE dynamic adaptation to the maximum number of MIMO layers         vivo

- R4-1905880	 LS to RAN1 and RAN2 on UE RF tuning time for MIMO layer switching        Ericsson

- R4-1906661 	LS on switching and interruption time for UE dynamic adaptation to the maximum number of MIMO layers      MediaTek