3GPP TSG-RAN4 Meeting #90bis	R4-1903752

# Xi’An, China, 8th – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6397 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE measurement capability for NE-DC in 36.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE measurement capability for NE-DC has been defined in 36.133, but the per-RAT capability for E-UTRA measurement configured by NR PCell is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the per-RAT capability for E-UTRA measurement configured by NR PCell for NE-DC measurement capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE measurement capability for NE-DC is incomplete in 36.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1c |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)

The requirements in this section are applicable for UE capable of and configured with the NE-DC operation mode.

When monitoring of multiple inter-frequency E-UTRAN carriers as configured by PSCell, inter-RAT E-UTRAN carriers as configured by NR PCell, and inter-frequency NR carriers as configured by NR PCell using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (NR SS-RSRP, NR SS-RSRQ, NR SS-SINR, SFTD, RSRP, RSRQ, and RS-SINR measurements, etc.) of detected cells on all the layers.

For UE configured with the NE-DC operation, the effective total number of frequencies excluding the frequencies of the NR PCell, NR SCells, PSCell, and SCells being monitored is Nfreq, NE-DC, which is defined as:

Nfreq, NE-DC = Nfreq, NE-DC, NR + Nfreq, NE-DC, E-UTRA,

where

Nfreq, NE-DC, NR is the number of NR inter-frequency carriers being monitored as configured by NR PCell.

Nfreq, NE-DC, E-UTRA ≤ Nfreq, NE-DC, E-UTRA, inter-RAT + Nfreq, NE-DC, E-UTRA, inter-freq

where

Nfreq, NE-DC, E-UTRA, inter-RAT is the number of E-UTRA inter-RAT carriers (FDD and TDD) excluding E-UTRA serving carrier(s) being monitored as configured by PSCell [15] or via LPP [22],

Nfreq, NE-DC, E-UTRA, inter-freq is the number of E-UTRA inter-frequency carriers (FDD and TDD) being monitored as configured by NR PCell or via LPP [22].

8.1.2.1.1c.1	NE-DC: Maximum allowed layers for multiple monitoring

If a UE is configured with NE-DC operation, the UE shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, 7 NR inter-frequency carriers configured by NR PCell [50], and

-	Depending on UE capability, 6 E-UTRA TDD inter-RAT carriers excluding E-UTRA serving carriers configured by NR PCell [50], and

-	Depending on UE capability, 6 E-UTRA FDD inter-RAT carriers excluding E-UTRA serving carriers configured by NR PCell [50], and

-	Depending on UE capability, 6 E-UTRA TDD inter-frequency carriers configured by PSCell, and

-	Depending on UE capability, 6 E-UTRA FDD inter-frequency carriers configured by PSCell, and

-	Depending on UE capability, 1 E-UTRA FDD inter-frequency carrier for RSTD measurements configured via LPP [57], and

-	Depending on UE capability, 1 E-UTRA TDD inter-frequency carrier for RSTD measurements configured via LPP [57].

In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, and E-UTRA TDD layers. The UE shall be capable of monitoring a total of at least 6 effective E-UTRA carrier frequency layers, excluding E-UTRA serving carrier(s), comprising of any above defined combination of E-UTRA inter-RAT carriers excluding E-UTRA serving carrier(s) configured by NR PCell and E-UTRA inter-frequency carriers configured by PSCell.

<End of Change 1>