3GPP TSG-RAN4 Meeting #90bis	R4-1904055

# Xi'an, China, 8th Apr 2019 - 12th Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5430 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 4Rx Requirement for Band 30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | PCTEST Engineering Lab |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_4Rx_AP_DL_bands_R15-Core |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The minimum reference sensitivity requirement modification for UE(s) equipped with 4 antenna ports, ΔRIB,4R, is not specified for Band 30. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 7.3.1-1a to include Band 30. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum reference sensitivity requirement modification for UE(s) equipped with 4 antenna ports, ΔRIB,4R, would not be specified for Band 30 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 Isolated impact analysis: Updates 4Rx UE requirements for Band 30 and any associated CA combos. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged content omitted >>

### 7.3.1	Minimum requirements (QPSK)

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1 and Table 7.3.1-2

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| 10 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 11 |  |  | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  |  | -97 | -94 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 17 |  |  | -97 | -94 |  |  | FDD |
| 18 |  |  | -1007 | -977 | -95.27 |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 |  |  | -99 | -96 |  |  | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 34 |  |  | -100 | -97 | -95.2 |  | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 43 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 50 |  | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 51 |  | -102.2 | -100 |  |  |  | TDD |
| 52 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| ... |  |  |  |  |  |  |  |
| 65 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 68 |  |  | -98.5 | -95.5 | -93.7 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |
| 72 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 73 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 74 | -104.78 | -101.78 | -99.58 | -96.58 | -94.78 | -93.58 | FDD |
| 85 |  |  | -97 | -94 |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

For UE(s) equipped with 4 antenna ports, the minimum requirement for reference sensitivity in Table 7.3.1-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3.1-1a for the applicable E-UTRA bands.

Table 7.3.1-1a: ΔRIB,4R

| E-UTRA Band | ΔRIB,4R [dB] |
| --- | --- |
| 1, 2, 3, 4, 7, 20, 21, 25, 30, 34, 39, 40, 41, 66 | - 2.7 |
| 42, 43, 52 | - 2.2 |

## << End of changes >>