3GPP TSG-RAN WG1 Meeting #97	R1-1907577

# Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1255 | rev | 1 | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On set of candidates for different ALs for MPDCCH in USS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For MPDCCH UE-specific search space configured with 2+4 PRB pairs or mPDCCH-NumRepetition > 1, the ALs and set of candidates are not clearly defined for cases with different number of EREGs per ECCE. For example, for MPDCCH UE-specific search space configured with 2 PRB pairs and mPDCCH-NumRepetition > 1, Table 9.1.5-1a should be used, which includes MPDCCH candidate with L’=8, but AL ≥ 8 cannot be supported for cases with 8 EREGs per ECCE. Similar to the CSS, different sets of MPDCCH candidates with different supported ALs should be defined for 4 EREGs per ECCE and 8 EREGs per ECCE respectively, for MPDCCH UE-specific search space configured with 2+4 PRB pairs or mPDCCH-NumRepetition > 1. In addition, special subframe configurations #8 and #9 are not defined for extended cyclic prefix. Special subframe configuration #10 is also not defined for extended cyclic prefix. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) For MPDCCH UE-specific search space configured with 2+4 PRB pairs or mPDCCH-NumRepetition > 1, define the parameter L’ as the ALs for MPDCCH candidates with 4 EREGs per ECCE, and half the values of L’ as the ALs for MPDCCH candidates with 8 EREGs per ECCE. 2) Remove the special subframe configurations #8 and #9 for extended cyclic prefix. Sepcial subframe configuration #10 undefined for extended cyclic prefix is also removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For MPDCCH UE-specific search space configured with 2+4 PRB pairs or mPDCCH-NumRepetition > 1, some of the MPDCCH candidates defined in current spec cannot be supported for MPDCCH with 8 EREGs per ECCE. There may be confusions on supported special subframe configurations for extended cyclic prefix. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis This CR only affects the MPDCCH candidates to be monitored for MPDCCH UE-specific search space configured with 2+4 PRB pairs or mPDCCH-NumRepetition > 1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev.2: Remove special subframe configuration #10 for extended cyclic prefix |  |  |  |  |  |  |  |  |

### 9.1.5	MPDCCH assignment procedure

#### < Unchanged parts are omitted >

The aggregation and repetition levels defining the MPDCCH search spaces and the number of monitored MPDCCH candidates are given as follows:

For MPDCCH UE-specific search space

-	if the BL/CE UE is configured with =2 or =4 PRB-pairs, and mPDCCH-NumRepetition=1, and

-	if the MPDCCH-PRB-set is configured for distributed transmission, the aggregation levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.4-1a and Table 9.1.4-1b, where  is substituted with  for ≤24, and  is substituted with .

-	if the MPDCCH-PRB-set is configured for localized transmission, the aggregation levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.4-2a and Table 9.1.4-2b, where  is substituted with  and  is substituted with .

-	otherwise

-	if the UE is configured with CEModeA, and =2 or =4, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-1a

-	if the UE is configured with CEModeA, and =2+4 , the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-1b

-	if the UE is configured with CEModeB, and =2 or =4, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-2a

-	if the UE is configured with CEModeB, and =2+4 , the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are listed in Table 9.1.5-2b

is the number of PRB-pairs configured for MPDCCH UE-specific search space. When =2+4, it is given by the higher layer parameter numberPRB-Pairs-r13, and when =2 or =4, it is given by the higher layer parameter numberPRB-Pairs-r11.

, , , are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition.

The PRB-pairs within a Narrowband corresponding to an MPDCCH-PRB-set are indicated by higher layers and are determined using the description given in Subclause 9.1.4.4.

If higher layer configuration numberPRB-Pairs-r13 for MPDCCH-PRB-set is 6, =2+4, and the number of PRB-pairs in an MPDCCH-PRB-set = 2+4.

If Type2-MPDCCH common search space or Type2A-MPDCCH common search space,

-	PRB-pairs of the 2 PRB set in the 2+4 PRB set correspond to PRB-pairs with the largest two PRB indices in MPDCCH-PRB-set .

-	PRB-pairs of the4 PRB set in the 2+4 PRB set correspond to PRB-pairs with the smallest 4 PRB indices in MPDCCH-PRB-set .

-	PRB-pairs of the 2+4 PRB set in the 2+4 PRB set correspond to all PRB-pairs in MPDCCH-PRB-set

Table 9.1.5-1a: MPDCCH candidates monitored by a BL/CE UE 
(CEModeA, MPDCCH-PRB-set size – 2PRBs or 4PRBs)

|  | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 | r1 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r2 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r3 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |
| 2 | r4 | 2 | 1 | 1 | 0 | 0 |
| 4 |  | 1 | 1 | 1 | 1 | 0 |

Table 9.1.5-1b: MPDCCH candidates monitored by a BL/CE UE 
(CEModeA, MPDCCH-PRB-set size – 2+4PRBs)

| MPDCCH PRB set | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 PRB set in 2+4 PRB set | r1 | 1 | 1 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 2 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r2 | 0 | 1 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 2 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r3 | 0 | 0 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 1 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r4 | 0 | 0 | 0 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |

Table 9.1.5-2a: MPDCCH candidates monitored by a BL/CE UE 
(CEModeB, MPDCCH-PRB-set size – 2PRBs or 4PRBs)

|  | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 | r1 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r2 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r3 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |
| 2 | r4 | 0 | 0 | 1 | 0 | 0 |
| 4 |  | 0 | 0 | 1 | 1 | 0 |

Table 9.1.5-2b: MPDCCH candidates monitored by a BL/CE UE 
(CEModeB, MPDCCH-PRB-set size – 2+4PRBs)

| MPDCCH PRB set | R |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | L'=2 | L'=4 | L'=8 | L'=16 | L'=24 |
| 2 PRB set in 2+4 PRB set | r1 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r2 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r3 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |
| 2 PRB set in 2+4 PRB set | r4 | 0 | 0 | 1 | 0 | 0 |
| 4 PRB set in 2+4 PRB set |  | 0 | 0 | 0 | 1 | 0 |
| Both PRB sets in 2+4 PRB set |  | 0 | 0 | 0 | 0 | 1 |

Table 9.1.5-3: Determination of repetition levels

| 1 | 1 | - | - | - |
| --- | --- | --- | --- | --- |
| 2 | 1 | 2 | - | - |
| 4 | 1 | 2 | 4 | - |
| >=8 |  |  |  |  |

Table 9.1.5-4: Repetition levels for Type1/1A-MPDCCH common search space

| 256 | 2 | 16 | 64 | 256 |
| --- | --- | --- | --- | --- |
| 128 | 2 | 16 | 64 | 128 |
| 64 | 2 | 8 | 32 | 64 |
| 32 | 1 | 4 | 16 | 32 |
| 16 | 1 | 4 | 8 | 16 |
| 8 | 1 | 2 | 4 | 8 |
| 4 | 1 | 2 | 4 | - |
| 2 | 1 | 2 | - | - |
| 1 | 1 | - | - | - |

For Type0-MPDCCH common search space, the narrowband location and the MPDCCH-PRB-set  are the same as for MPDCCH UE-specific search space, and

-	if =2,

-	=1 for =8 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

-	if =4,

-	=1 for =16 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

-	if =2+4,

-	=1 for =24 and repetition levels ,, ,  given in Table 9.1.5.-3. For all other cases, =0

where , , ,  are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition.

For Type1-MPDCCH common search space and Type1A-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	=1 for =24 and repetition levels ,, ,  where the repetition levels are determined from Table 9.1.5-4 by substituting the value of

-	with higher layer parameter mPDCCH-NumRepetition-Paging for Type1-MPDCCH common search space, and

-	with higher layer parameter mpdcch-NumRepetitions-SC-MCCH for Type1A-MPDCCH common search space.

-	For all other cases, =0

For Type2-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	If the most recent coverage enhancement level used for PRACH is coverage enhancement level 0 and 1, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-1b, by assuming that the number of candidates for <8 as zero.

-	If the most recent coverage enhancement level used for PRACH is coverage enhancement level 2 and 3, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-2b.

where , , , are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mPDCCH-NumRepetition-RA.

For Type2A-MPDCCH common search space, the number of PRB-pairs in MPDCCH-PRB-set is 2+4 PRB-pairs, and

-	for CEModeA, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-1b, by assuming that the number of candidates for <8 as zero,

-	for CEModeB, the aggregation and repetition levels defining the search spaces and the number of monitored MPDCCH candidates are determined from Table 9.1.5-2b,

where , , , are determined from Table 9.1.5-3 by substituting the value of  with the value of higher layer parameter mpdcch-NumRepetitions-SC-MTCH.

In tables 9.1.5-1a, 9.1.5-1b, 9.1.5-2a, 9.1.5-2b, and for MPDCCH UE-specific search space when BL/CE UE is configured with =2+4 or mPDCCH-NumRepetition > 1, Type0, Type1, Type1A, Type2, Type2A MPDCCH common search space,  is applied for =4, and  is applied for =8 wherein  substituting the values of .

For Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space, distributed MPDCCH transmission is used.

For MPDCCH UE-specific search space, Type0-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space locations of starting subframe  are given by where is the th consecutive BL/CE DL subframe from subframe , and , and , and , where

-	subframe  is a subframe satisfying the condition , where

-	For MPDCCH UE-specific search space, and Type0-MPDCCH common search space,  is given by the higher layer parameter mPDCCH-startSF-UESS,

-	For Type1A-MPDCCH common search space,  is given by the higher layer parameter mpdcch-startSF-SC-MCCH

-	For Type2-MPDCCH common search space,  is given by the higher layer parameter mPDCCH-startSF-CSS-RA-r13

-	For Type2A-MPDCCH common search space,  is given by the higher layer parameter mpdcch-startSF-SC-MTCH

-	is given by higher layer parameter mpdcch-Offset-SC-MTCH for Type2A-MPDCCH common search space, and otherwise; and

-	is given by higher layer parameter mPDCCH-NumRepetition for MPDCCH UE-specific search space and Type0-MPDCCH common search space, and mPDCCH-NumRepetition-RA for Type2-MPDCCH common search space, and mpdcch-NumRepetitions-SC-MCCH for Type1A-MPDCCH common search space, and mpdcch-NumRepetitions-SC-MTCH for Type2A-MPDCCH common search space and

-	, , , are given in Table 9.1.5-3.

A BL/CE UE is not expected to be configured with values of and that result in non-integer values of .

For Type1-MPDCCH common search space, = and is determined from locations of paging opportunity subframes,

If SystemInformationBlockType1-BR or SI message is transmitted in one narrowband in subframe , a BL/CE UE shall assume MPDCCH in the same narrowband in the subframe  is dropped.

The BL/CE UE is not required to monitor an MPDCCH search space if any ECCEs corresponding to any of its MPDCCH candidates occur within a frame before and also occur within frame.

The BL/CE UE is not expected to be configured with overlapping MPDCCH search spaces.

A BL/CE UE configured to monitor MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI with the same payload size and with the same aggregation level in the Type0-MPDCCH common search space and the MPDCCH UE-specific search space shall assume that for the MPDCCH candidates with CRC scrambled by C-RNTI or SPS C-RNTI, only the MPDCCH in the UE specific search space is transmitted.

For MPDCCH UE-specific search space or for Type0-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition is set to 1; or for Type2-MPDCCH common search space if the higher layer parameter mPDCCH-NumRepetition-RA is set to 1; or for Type2A-MPDCCH common search space if the higher layer parameter mpdcch-NumRepetitions-SC-MTCH is set to 1;

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0 and 5 shown in Table 4.2-1 of [3]

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4 and 7 shown in Table 4.2-1 of [3];

otherwise

-	The BL/CE UE is not required to monitor MPDCCH

-	For TDD, in special subframes, if the BL/CE UE is configured with CEModeB

-	For TDD and normal downlink CP, in special subframes for the special subframe configurations 0, 1, 2, 5, 6, 7, 9, and 10 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA

-	For TDD and extended downlink CP, in special subframes for the special subframe configurations 0, 4, and 7 shown in Table 4.2-1 of [3], if the BL/CE UE is configured with CEModeA.

-	For TDD, in special subframes, for MPDCCH in Type1/1A-MPDCCH common search space.

The number of MPDCCH repetitions is indicated in the 'DCI subframe repetition number' field in the DCI according to the mapping in Table 9.1.5-5.

Table 9.1.5-5: Mapping for DCI subframe repetition number

| R | DCI subframe repetition number |
| --- | --- |
|  | 00 |
|  | 01 |
|  | 10 |
|  | 11 |

#### < Unchanged parts are omitted >