3GPP TSG-RAN#84	RP-191600

Newport Beach, CA, USA, 3 – 6 June 2019

# Source:	Ericsson, Nokia, Nokia Shanghai Bell

# Title:	Revised WID: Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements

# Document for:	Approval

Agenda Item:	9.4.10

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	WID on DC and CA enhancements

## Acronym: LTE_NR_DC_CA_enh-Core {Propose an acronym. The sign “-“ is a level separator between (Feature)-(Building Block)-(Work Task). The sign “_” can be freely used. Studies have to start by “FS_”. Each acronym level has to be simple and short, 8 characters max recommended.}

## Unique identifier: 800088 	{A number to be provided by MCC at the plenary}

NOTE:	For new Wis/Sis leave the Unique identifier empty but you may make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. Part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | x |
| --- | --- |
| This WID includes a Performance part | x |

or:

| This sID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No |  |  |  |  |  |
| Don’t know |  |  |  | x |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. “Feature / Building Block / Work Task” form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

{For a Feature: list here the children Building Blocks (optional) and Work Tasks (optional)}

{For a Building Block: list here the parent Feature (mandatory) and children Work Tasks (optional)}

{For a Work Task: list here the parent Building Block (mandatory)}

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {mandatory text: “parent WID” or “child WID”} |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. Part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is “parent WID”.

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

NOTE:	Classical examples: List a preceding SI or a preceding WI (e.g. if you further enhance a topic). Also related or dependent Wis in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header “Dependency on non-3GPP (draft) specification:” if no such dependency.}

## 3	Justification

In Release 15, 3GPP introduces EUTRA-NR Dual Connectivity, standalone NR operation, NR-EUTRA DC as well as  NR-NR DC (in late drop). E-UTRA-NR DC and NR-EUTRA DC are called Multi-Radio DC (MR-DC). However, asynchronous NR-NR Dual Connectivity is not introduced due to lack of time.

As compared to CA and synchronous DC, asynchronous NR-NR DC offers the following benefits:

- Avoid need to synchronise gNBs

- Allows non-co-located deployments

- Makes operator managed 5G unlicensed (5G LAA) easier to deploy

- Etc.

Thus, asynchronous NR-NR DC should be introduced in Rel-16 specifications.

In addition to DC, multiple carriers have been a key feature for boosting peak data rates in flexible manner for many different deployment scenarios. However, the CA framework is based on tight network coordination and availability of UE assistance information via UE measurement reporting. The operation relies on network configuring and deciding whether to keep the SCell activated (for maximum peak data rate) or deactivated (for minimized power consumption while maintaining CA).

It is known that traffic patterns are chatty in nature which may result short lived connections. It is also known that in the current LTE CA framework the setup delay for both PCell a and configured Scell can take from 30 ms to 1 s. Similar setup times can be assumed for DC scenarios.  This delay limits utilization of spectrum as the UE has may left connected mode already when CA/DC is setup.

For NR, cell access has been based on similar principles as LTE, but the LTE additions in Rel-15 are not yet part of NR procedures. One new key element NR is bringing is enabling the possibility of utilizing much larger spectrum bandwidths, ultimately leading to ~10 times higher peak data rates than LTE. Delays in accessing the high capacity (small) cells are detrimental to the performance and in NR this is can be even more pronounced (e.g. especially if macro is still LTE). Thus, to fully take advantage on the large BW possibilities, efficient and fast utilization of the large BW should be ensured in all cases, including initial connection establishment, reconfiguration of CA/DC and addition of secondary cells.

For signalling robustness, 3GPP introduced in Rel-15 split signalling radio bearer (SRB) which allows to send RRC messages over LTE and/or NR. Main benefit of split SRB is to add additional robustness. However, in Rel-15, the radio link failure and re-establishment is triggered even secondary link is still running. This should be enhanced.

As summary, asynch NR-NR DC, fast access to NR and Multi-Radio-DC (MR-DC) as well as robust operation in MR-DC should be designed in Rel-15.

## 4	Objective

### 4.1	Objective of WI

The objective of this work item is to investigate enhancements to DC and CA. At least the following topics should be considered in the work:

- Support of asynchronous and synchronous NR-NR Dual Connectivity [RAN1, RAN2, RAN4]

  - UE power control [RAN1]

  - RRC signalling to support of enhanced NR-NR DC [RAN2]

  - Core requirements to support enhanced NR-NR DC [RAN4]

Note: Synchronous DC enhancements in this WID considers only cases not covered in Rel-15 exception sheet for NR WI NR_newRAT-Core.

- Early Measurement reporting: Early and fast reporting of measurements information availability from neighbor and serving cells to reduce delay setting up MR-DC and/or CA. [RAN2, RAN4]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider measurements in IDLE, INACTIVE mode and CONNECTED mode

  - The impacts on UE power consumption should be minimized

  - The LTE Rel-15 euCA work should be utilized, when applicable

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

- Fast recovery: Support fast recovery of MCG link e.g. by utilizing the SCG link and split SRBs for recovery during MCG failure while operating under MR-DC. [RAN2, RAN3]

  - This objective applies to MR-DC and NR-NR DC

- Cross-carrier scheduling with different numerologies on the scheduling and scheduled carriers [RAN1, RAN2, RAN4]

  - This objective applies to CA only.

  - Target completion by RAN#84.

- Study and, if found beneficial over the existing single Tx switched uplink solution, specify enhancements to single Tx switched uplink solution for EN-DC, such as allowing all DL and UL subframes for data transmission for both NR and LTE. [RAN1].

Note: UE in single Tx switched uplink mode is not expected to be scheduled simultaneous transmission on MCG and SCG

- Enable the Release 15 behaviour of “DL HARQ timing for FDD Scell for LTE TDD-FDD CA with TDD Pcell, applied to FDD Pcell” to apply to dual uplink EN-DC, possibly including any conclusions on the previous objective (6) for LTE FDD Pcells [RAN1].

- Specify the interruption requirements for EN-DC and NE-DC related to features added during the Rel-15 LTE euCA WI [RAN4]

The work on objective 2 and 3 should consider UEs in all states (i.e. IDLE, INACTIVE and CONNECTED).

The work excludes combination of EN-DC and NR-NR DC configured for a UE.

At least scenarios with small cells should be considered, along with scenarios with large amounts of available cell bandwidth. Both FR1 and FR2 frequencies should be considered. The mechanisms already specified for LTE should be used as baseline for the work.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify necessary performance requirements related to the above-mentioned core requirements

- UE performance requirements

- UE RRM performance requirements

### 4.3	RAN time budget request (not applicable to RAN5 Wis/Sis)

NOTE:	For all RAN related Wis/Sis which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID’s zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each of them in the attached Excel table.

Additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

New specifications {One line per specification. Create/delete lines as needed}

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.331 | NR RRC signalling changes for MR-DC enhancements | 87 | Core parts |
| 38.306 | Necessary UE capabilities for NR and MR-DC | 87 | Core parts |
| 38.133 | Necessary RAN4 requirements | 87 | Core parts |
| 38.101-1 | Necessary RAN4 requirements | 87 | Core parts |
| 38.101-2 | Necessary RAN4 requirements | 87 | Core parts |
| 38.101-3 | Necessary RAN4 requirements | 87 | Core parts |
| 36.331 | LTE RRC signalling changes for MR-DC enhancements | 87 | Core parts |
| 36.306 | Necessary UE capabilities for LTE | 87 | Core parts |
| 38.321 | Possible changes to MAC specification | 87 | Core parts |
| 37.340 | Possible changes to Multi-connectivity Stage-2 | 87 | Core parts |
| 38.300 | Possible changes to NR Stage-3 | 87 | Core parts |
| 38.413 | Possible changes to Xn-AP | 87 | Core parts |
| 38.473 | Possible changes to F1AP | 87 | Core parts |
| 38.213 | Possible changes to physical layer procedures | 86 | Core parts |
| 38.101-4 | Necessary RAN4 requirements | 89 | Performance parts |
| 38.133 | Necessary RAN4 requirements | 89 | Performance parts |
| 38.133 | Necessary RAN4 requirements | 87 | Core parts |
| 36.133 | Necessary RAN4 requirements | 87 | Core parts |

NOTE:	If this is a RAN WID including Core and Perf. Part, then all new Core part specs have to be listed first and then all new Perf. Part specs. Indicate “Core part” or “Perf. Part” under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. Part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

RAN/RAN2:

Oumer Teyeb

Company:	Ericsson

Email: oumer.teyeb@ericsson.com

RAN1:

Karri Ranta-aho

Company: Nokia

Email: karri.ranta-aho@nokia.com

## 7	Work item leadership

Primary responsible WG: RAN WG2

Secondary responsible WG: RAN WG1, RAN WG3, RAN WG4

## 8	Aspects that involve other WGs

{Free text. If applicable, specify the other WG(s) and their task, e.g.: “SA1 for the Service Aspects”, “SA5 for the Charging Aspects”, “SA3 for the Security aspects”, etc. If not applicable, indicate “None” or “None identified yet”.}

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Ericsson

Nokia

Nokia Shanghai Bell

Huawei

HiSilicon

CATT

InterDigital

NTT Docomo

Intel

Sharp

MTI

APT

Motorola Mobility

Lenovo

Sierra Wireless

Mediatek

ITRI

vivo

AT&T

KDDI

KT Corp.

SK Telecom

Samsung

Verizon

Orange

Qualcomm Inc.

CHTTL

NEC

ZTE