3GPP TSG RAN Meeting #85										RP-191830

Newport Beach, USA, 16-20 September, 2019

Agenda Item:	8.1.4

Source:	Huawei, HiSilicon

Title:	SA and CT impact of RAN work area on Rel-17 NB-IoT and LTE-MTC

Document for:	Discussion

# Introduction

RP-191829 contains the moderator’s summary and proposals resulting from Phase 1 of the RAN_DRAFTS reflector email discussion on Rel-17 enhancements to NB-IoT and LTE-MTC ([NB-IoT_eMTC_enh]).

This document tentatively identifies where a proposal is expected to have impact to TSG SA or TSG CT, were it approved for Rel-17 work in RAN.

# NB-IoT impacts to SA and CT

| Area | Develop into a Rel-17 WI objective until RAN#86 | Do not consider further for Rel-17 | More demonstration of technical merits needed during Phase 2 |
| --- | --- | --- | --- |
| Scheduling and latency enhancements | Reduced paging cycle - CT impact: Signaling of DRX parameters wanted by the UE - SA impact: MME/NAS and paging strategy. | Additional NPDCCH period offsets and k0 values for NPDSCH Dynamic indication of ACK/NACK repetition number Multi-TB scheduling for 1 HARQ process Enhancement to invalid subframe configuration (but check Rel-16 outcome after November WG meetings) SI change notification and acquisition in connected mode | (None yet) |
| Peak data rate enhancements | Introduce 16-QAM UL/DL Support of wider bandwidth/multiple NB-IoT carriers | Introduce 64-QAM UL/DL Introduction of 4 HARQ processes Enhancement for high code rate repetition | (None yet) |
| Interference and load management | Introduce Transmit Power Control command in DCI | Improve NPRACH opportunity distribution in time domain Random access carrier selection improvement Paging carrier optimizations | Prevent NPRACH coverage level ramping in high NRSRP DL power allocation for NPDCCH, NPDSCH Introduce PHR in connected mode |
| Power consumption reduction (*): Proposed after many companies already responded. | Very low UE power class, e.g. 0 dBm | Differentiate between escalated and non-escalated paging - SA and CT impact: MME/NAS. Paging strategy and differentiation awareness in CN Relaxed NWUS monitoring requirements - SA impact: NWUS awareness in CN for the assistance information provision. Power consumption reduction for multicast - SA impact: Related with the architecture level solution for multicast | Relaxed paging monitoring(*) - CT impact: Signalling capability - SA impact: Paging strategy Additional NRS, at least for in-band operation mode(*) Early UL termination for HD-FDD(*) |
| Relaying (and common with LTE-MTC) | Develop IoT relaying into Rel-17 WI or SI objective(s) until RAN#86. - CT impact: TS 24.501 does not contain relaying for 5GS. - SA impact: Requirements; Architecture support. | N/A | Discuss until RAN#86 if selection(s) among the types of relays can be made. |
| Mobility enhancements | (None yet) | Enhancements to intra-RAT mobility triggers and measurements Inter-RAT mobility NB-IoT to/from NR - CT impact: Possibly need NAS protocol changes due to inter-RAT mobility between IoT radio enablers. - SA impact: Coordination needed with SA2. | Reduced reading of neighbor cells’ SI |
| Further improved multi-carrier operation | Frequency hopping between NB-IoT carriers Coverage level or service association to NB-IoT carrier - SA and CT impact: MME may need to be aware of this. | Non-anchor carrier with configurable NRS pattern Support no gap between anchor and non-anchor carrier in standalone operation mode Synchronization/System Information on non-anchor carriers Cross-carrier scheduling | (None yet) |
| NB-IoT working with NR/5GC | Multicast/broadcast with connection to 5GC - CT impact: NAS impacts possible depending on whether SA2 work has NAS impacts. - SA impact: SA2 work for 5GC to support MC/BC. SA2 SID ongoing. | (None yet) | UE switching between NB-IoT and NR procedures (proposal needs clarification) - SA impact: Possibly SA2 mobility-related procedures. |
| Others | NB-IoT via satellite (also discuss if this is WI, SI, or NTN WI objective) - CT impact: Possible introduction of new radio bearer enabler for NAS protocols and related details. - SA impact. NTN SID ongoing, architecture level support needed. Finer-grained channel quality reporting in normal coverage | Small message transmission to groups of UEs - CT impact: CN likely to be impacted. - SA impact: Architecture impact. Application layer response enhancement - CT impact: CN likely to be impacted. - SA impact: Architecture impact. | NB-IoT within smartphones Enhancements to asynchronous shared PUR (pre-configured uplink resources) – but wait until Rel-16 WI has progressed further Support of private networks |

# 3 LTE-MTC impacts to SA and CT

| Area | Develop into a Rel-17 WI objective until RAN#86 | Do not consider further for Rel-17 | More demonstration of technical merits needed during Phase 2 |
| --- | --- | --- | --- |
| Scheduling and latency enhancements | (None yet) | Additional MPDCCH period offsets Higher PDSCH code rates in 1-6 PRBs Same-subframe scheduling in bundling SI change notification and acquisition in connected mode Further enhancement of Multiple-TB scheduling (but check Rel-16 outcome after November WG meetings) Smaller Idle DRX values | (None yet) |
| Interference and load management | (None yet) | Prevent PRACH coverage level ramping in high RSRP Improve PRACH opportunity distribution in time domain | (None yet) |
| Power consumption reduction See section 3.3 of RP- 191829 for a number of late proposals after many companies already commented | (None yet) | Relaxed MWUS monitoring requirements - SA impact: NWUS awareness in CN for the assistance information provision. Power consumption reduction for multicast - SA impact: related with the architecture level solution for multicast | Differentiate between escalated and non-escalated paging - SA impact: Paging strategy and differentiation awareness in CN Power consumption reduction for positioning |
| Relaying | In common with NB-IoT |  |  |
| Mobility enhancements | (None yet) | (None yet) | Inter-RAT mobility LTE-MTC to/from NR - CT impact: Possibly need NAS protocol changes due to inter-RAT mobility between IoT radio enablers. - SA impact: Coordination needed with SA2. |
| LTE-MTC working with NR/5GC | Multicast/broadcast with connection to 5GC - CT impact: NAS impacts possible depending on whether SA2 work has NAS impacts. - SA impact: SA2 work for 5GC to support MC/BC. SA2 SID ongoing. | LTE-MTC – NR coexistence in a PRB (but check Rel-16 outcome after Rel-16 WG meetings) | (None yet) |
| Others | Enhancements to asynchronous shared PUR (pre-configured uplink resources) – but wait until Rel-16 WI has progressed further Transmit power boosting above power class maximum LTE-MTC via satellite (also discuss if this is WI, SI, or NTN WI objective) - CT impact: Possible introduction of new radio bearer enabler for NAS protocols and related details. - SA impact. NTN SID ongoing, architecture level support needed. | LTE-MTC within smartphones (already supported) Application-layer response enhancement - CT impact: CN likely to be impacted. - SA impact: Architecture impact. | (None yet) |

# Conclusions

TSG SA and TSG CT are tentatively alerted to potential impacts in their domain from work under consideration in TSG RAN for inclusion in Rel-17 NB-IoT and LTE-MTC enhancements.