3GPP TSG RAN Meeting #85	RP-191933

Newport Beach, USA, September 16 - 20, 2019

Agenda item:		9.4.5

Source:	OPPO

Title:	On scope of NR URLLC enhancement in Rel-16

Document for:		Discussion and Decision

1	Introduction

NR URLLC enhancement is one topic in Rel-16, the following features are recommoned to discuss[1]:

For PDCCH enhancement, it is recommended to support the following in Rel-16: -DCI format(s) with configurable sizes for some fields and potential reduction of the number of bits for some field(s) compared to Rel-15 DCI, while enabling the minimum DCI size target a reduction of 10~16 bits less than the DCI format size of Rel-15 fallback DCI and the maximum DCI size can be larger than Rel-15 fallback DCI, and provide the possibility to align with the size of the Rel-15 fallback DCI (including possible zero padding if any). -Increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for Rel-16 NR URLLC for at least one SCS subject to some restrictions, including at least explicit limitation on the maximum number of BDs/non-overlapping CCEs per monitoring occasion and/or per monitoring span, and the set of applicable SCS(s). For UCI enhancement, it is recommended to support the followings in Rel-16: -More than one PUCCH for HARQ-ACK transmission within a slot. -At least two HARQ-ACK codebooks can be simultaneously constructed for a Rel-16 UE, intended for supporting different service types for a UE. For PUSCH enhancements, it is recommended to support enhancements for both grant-based PUSCH and configured grant based PUSCH in Rel-16, to enable one UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. For enhancements to scheduling/HARQ, it is recommended to support the following in Rel-16: -Out-of-order HARQ-ACK: HARQ-ACK associated with the second PDSCH with HARQ process ID x received after the first PDSCH with HARQ process ID y (x != y) can be sent before the HARQ-ACK of the first PDSCH on the active BWP of a given serving cell -Out-of-order PUSCH scheduling: A UE can be scheduled with a second PUSCH associated with HARQ process x starting earlier than the ending symbol of the first PUSCH associated with HARQ process y (x != y) with a PDCCH that does not end earlier than the ending symbol of first scheduling PDCCH on the active BWP of a given serving cell For inter UE Tx prioritization/multiplexing, it is recommended to support UL cancelation scheme and enhanced UL power control scheme in Rel-16. For enhanced UL configured grant transmission, it is recommended to support multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell in Rel-16.

In this contribution, the above features’s status and process are discussed.

2		Discussion

Till now, 2/3 Timeline for URLLC has been passed and most features were fully discussed and proceed normally. However, unexpected details and work load from RAN2 were proposed during the discussion, which leads overload for limited TU. The details on discussion status and process are analyized firstly:

2.1 Topics captured in TR 38.824

For PDCCH enhancement,

-DCI format(s) with configurable sizes provides DCI reliability and schedule flexibility, which is important for URLLC. For DCI format for downlink, half of fields were discussed and achieved agreement. For uplink, only a few fields were discussed, but some of undiscussed fields can refer to the same field in downlink. In general, discussion on DCI format proceed normally.

-Increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for Rel-16 NR URLLC is benefit for URLLC latency. Currently, framework for increased PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation has been agreed. The maximum number of non-overlapped CCEs per PDCCH monitoring span is introduced and PDCCH monitoring span follows UE feature 3-5b as a starting point. The leftover is detail value for the maximum number of non-overlapped CCEs for any combination and inter-action between PDCCH configuration and capability,especially for multiple capability  are reported and uneven search space are configured. In general, discussion on the maximum number of non-overlapped CCEs per slot for channel estimation for Rel-16 NR URLLC proceed normally.

However, other topics related with PDCCH monitoring capability, e.g. PDCCH blind decoding number and DCI size alignment are suggested to be loaded in Rel-17 due to time limit.

For UCI enhancement, more than one PUCCH for HARQ-ACK transmission within one slot and at least two HARQ-ACK codebook simultaneous transmission plays a important role in latency reduction. And in general, discussion on UCI enhancement proceed normally.

The framework to support more than one PUCCH for HARQ-ACK transmission within a slot has been agreed and sub-slot based HARQ-ACK was defined and most of design follow Rel-15 mechanism. The leftover is detail configuration, e.g. PUCCH configuration per sub-slot.

- PUCCH configuration for at least two HARQ-ACK codebooks have also been fully discussed. And physical identification of HARQ-ACK codebook is being discussed. Several solutions on physical identification of HARQ-ACK codebook were summarized. Downselect and detail issues need to be discussed in futher meeting.

For PUSCH enhancements, framework to enable one UL grant scheduling two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots has been agreed. However, there are lots of details and enhancements to be discussed. Such as, time domain resource allocation,inter-action with SFI, Frequency hopping, DMRS, RV and TBS were proposed to discuss. However, due to time limit, it is suggested that basic and typical issues are focused firslty and simple solution is first consideration.

For enhancements to scheduling/HARQ, Out-of-order HARQ-ACK and PUSCH scheduling were recommended to support to reduce latency in TR38.824[1]. However, PDSCH overlapping issue and multiple processing time on the same carrier were proposed during WI discussion and these issue also impact latency significantly. But till now, solution to solve the above issues are not clear. So it is better to find common solution or downselect some scenarios.

For inter UE Tx prioritization/multiplexing, it was recommended to support UL cancelation scheme and enhanced UL power control scheme. Prioritized UL cancelation scheme, e.g. Group common DCI, has been defined from several UL cancelation schemes and details, e.g. methods to reduce the UE monitoring for UL CI, stop with /without resuming, signaling/channels that can be cancelled, processing time have started discussion. For UL power control scheme, open-loop parameter based solution for DG-PUSCH has been defined and details will be discussed further. For CG-PUSCH, there is no clear solution. However, CG-PUSCH is more typical for URLLC, so solution for CG-PUSCH should be high-prioritized in further discussion. Considering that the framework for UL cancelation and power control for DG-PUSCH have been defined and leftover details were recommended to continue.

For enhanced UL configured grant transmission, framework to support multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell was defined and details, e.g. DCI activation and release design, could continue to discuss.

In summary, although workload is very high and TU is almost exhausted, most topics captured in TR38.824 go well according to plan. However, some unassigned scenario and unexpected issues proposed in the above topics increase workload further, so it is suggested that simple solution for typical scenarios should be first consideration to avoid overload.

Proposal1: Most topics captured in TR38.824 go well according to plan. However, discussion still should be focused to avoid overload,e.g. simple solution and typical scenario should be firt consideration.

2.2 Topics from IIoT

Due to the AI merging of IIoT and eURLLC, RAN1 TU is further occupied by objectives from IIoT WID. Specifically, intra-UE multiplexing/prioritization and SPS enhancement from IIoT needs RAN1 effort.

For intra-UE multiplexing/prioritization, in general, three scenarios, i.e., collision between UCI and UCI, UCI and data, data and data, were to be considered. Some scenarios ,e.g. collision between SRs, collision among configured grant, can be handled by RAN2 completely, there may not be impact on RAN1. So for these scenarios, it depends on RAN2 conclusion.

Proposal 2: Some scenarios for intra-UE multiplexing/prioritization can be handled by RAN2 completely and it depends on RAN2 conculsion.

However, some scenarios have impact in RAN1 and [2] analyzed UCI related scenarios, considering multiple types of UCI, 16 sub-scenarios were defined. Within the 16 sub-scenarios, maybe some sub-sceanarios could follow same design principle and solution, However, there are still around 10 sub-scenario to be to be handled separately, i.e., using different solutions. it is more troublesome that solutions from different companies are very different and no common design principle has been achieved .

On top of the collision two channels, in addition, one has to consider the collision between more than two signaling/channel. Therefore,the framework for collision between more than two channels also needs to be discussed.However, till now there is no common solution on framework.

The last but not the least, physical identification for prioritiy is essential parameter to support intra UE multiplexing/multiplexing. However, solution on physical identification for priority are diverse and need to be discussed further.

Considering the above aspects, RAN1 workload due to intra-UE multiplexing/prioritization is very huge and it is difficult to complete within planned TU. However, intra-UE multiplexing/prioritization plays a important role in reduce latency and deliberate solutions are neccesarry. So for intra-UE topic with impact on RAN1, it is suggested to deprioritize or down scope part of work at least, the leftover work could be loaded in Rel-17. For example, physical identification for priority could be confirmed in Rel-16 firslty and some specific scenarios or framework could be considered further. The remaining work could be loaded in Rel-17.

For SPS enhancement, due to multiple SPS configuration and/or shorter than 10ms SPS period are introduced, more than 1 bit of HARQ-ACK feedback for SPS PDSCH needs to be considered in physical layer. However, till now, RAN1 only identified the impact, but failed to discuss the detailed solution and workload can not be estimated. So considering the limited TU left in Rel-16, HARQ-ACK enhancement for SPS PDSCH can be deprioritized, or can be handled as left-over later in Rel-17.

In summar, topics from RAN2 leads huge and unestimated workload, and considering topics from TR38.824 have exhausted all TU and there has been risk of overload, it is suggested to deprioritize or down scope part work and leftover could be loaded in Rel-17.

Proposal 3: Deprioritize or down scope part of work in IIoT and leftovers could be handled later in Rel-17.

- Conclusion

This contribution analyzes discussion statues of NR URLLC enhancement in Rel-16. Due to time limit, some of features may need to be deprioritized.

Proposal1: Most topics captured in TR38.824 go well according to plan. However, discussion still should be focused to avoid overload,e.g. simple solution and typical scenario should be firt consideration.

Proposal 2: Some scenarios for intra-UE multiplexing/prioritization can be handled by RAN2 completely and it depends on RAN2 conculsion.

Proposal 3: Deprioritize or down scope part of work in IIoT and leftovers could be handled later in Rel-17.

- Reference

[1] TR 38.824 Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)

[2] R1-1909645 Offline summary on UCI enhancements for URLLC