3GPP TSG-RAN WG1 Meeting #98	R1-1909723

# Prague, Czech Republic, August 26th – 30th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1269 | rev |  | Current version: | 14.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE procedure for transmitting PSCCH in mode 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_V2X-Core |  |  |  |  |  | Date: |  |  | 2019-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In subclause 14.2.1 of TS36.213, the value of is not specified if the field “Lowest index of the sub-channel allocation to the initial transmission” in DCI format 5A is absent (i.e. when NsubchannelSL=1, and accordingly log2NsubchannelSL=0). - In subclause 14.2.1 of TS36.213, kinit is referenced without any prior definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the description for LInit=0 when the field “Lowest index of the sub-channel allocation to the initial transmission” in DCI format 5A is absent. - Remove the sentence “subframe tqSL corresponds to the subframe n+kinit”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE is not able to determine the resource index of PSCCH transmission if the field “Lowest index of the sub-channel allocation to the initial transmission” in DCI format 5A is absent. - Ambiguous definition of subframe tqSL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 14.2.1	UE procedure for transmitting the PSCCH

### < Unchanged parts are omitted >

For sidelink transmission mode 3,

-	The UE shall determine the subframes and resource blocks for transmitting SCI format 1 as follows:

-	SCI format 1 is transmitted in two physical resource blocks per slot in each subframe where the corresponding PSSCH is transmitted.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-V-RNTI, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe tqSL that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3.  is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]) if the field “Lowest index of the sub-channel allocation to the initial transmission” in the corresponding DCI format 5A is present and LInit=0 otherwise,  is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3].

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe , where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

-	If the UE receives in subframe n DCI format 5A with the CRC scrambled by the SL-SPS-V-RNTI , the UE shall consider the received DCI information as a valid sidelink semi-persistent activation or release only for the SPS configuration indicated by the SL SPS configuration index field. If the received DCI activates an SL SPS configuration, one transmission of PSCCH is in the PSCCH resource  (described in Subclause 14.2.4) in the first subframe tqSL that is included in  and that starts not earlier than TDL-NTA2×TS+4+m×10-3.  is the value indicated by "Lowest index of the sub-channel allocation to the initial transmission" associated with the configured sidelink grant (described in [8]) if the field “Lowest index of the sub-channel allocation to the initial transmission” in the corresponding DCI format 5A is present and LInit=0 otherwise,  is determined by Subclause 14.1.5, the value m is indicated by 'SL index' field in the corresponding DCI format 5A according to Table 14.2.1-1 if this field is present and m=0 otherwise, TDL is the start of the downlink subframe carrying the DCI, and NTA and TS are described in [3]..

-	If "Time gap between initial transmission and retransmission" in the configured sidelink grant (described in [8]) is not equal to zero, another transmission of PSCCH is in the PSCCH resource  in subframe , where  is the value indicated by "Time gap between initial transmission and retransmission" field in the configured sidelink grant.  corresponds to the value  determined by the procedure in Subclause 14.1.1.4C with the RIV set to the value indicated by "Frequency resource location of the initial transmission and retransmission" field in the configured sidelink grant.

### < Unchanged parts are omitted >