3GPP TSG RAN Meeting #85								RP-192003

Newport Beach, USA, September 16-20, 2019

Agenda Item:		8.1.12

Source:			Qualcomm Incorporated

Title:				Scope of RAN data collection enhancements

Document for:		Discussion

### Introduction

Release-17 enhancements for RAN data collection can consider the following potential items in its scope:

- leftovers of R16 RAN centric data collection and utilization SID/WID,

- data collection for new R16 features,

- AI/ML

- other aspects.

This paper summarizes the candidate items and aims to provide a baseline for the upcoming email discussion on the scope of release-17 study item.

### Leftovers of release-16 RAN centric data collection and utilization

Release-16 RAN centric data collection and utilization SI has discussed the following items. However, these items were not included in the scope of the follow up WI.

| Features | Function description | Responsible WGs |
| --- | --- | --- |
| CCO, AAS and Massive-MIMO | Network dynamically tunes the cell coverage by AAS based on UE measurement and cell load information. | RAN3 |
| Per UE local RRM policy container | RRM container generated and used by RAN for RRM optimization; and cached in CN. | RAN3 |
| Service aware RAN | Service-aware by RAN for cross-layer optimization. [1][2] | RAN2 |
| Area optimization | RAN Notification Area and System Information Area optimization based on UE signalling and log | RAN2 |
| Potential leftover | WI topics not completed | RAN2, RAN3 |

### Data collection for new release-16 features

The following new features are to be added in release-16 of RAN specifications. These are natural candidates for optimization via RAN data collection.

| Features | Function description | Responsible WGs |
| --- | --- | --- |
| 2-step RACH | RACH report for 2-step RACH | RAN2 |
| Mobility enhancement | Performance measurement: interruption, failure, latency; Performance optimization for e.g. CHO, MBB HO. | RAN2, RAN3 |
| NR-U | Performance measurement and optimization. | RAN2 |
| IIOT and URLLC | QoS verification | FFS |
| 5G Voice | QoS verification for SRVCC, VoNR, EPS/RAT fallback. Inherit LTE MDT for MMTel. | RAN2 |
| DCCA enhancement | Performance measurement and optimization. | FFS |
| UE power saving | Power consumption estimation and reporting by RAN | FFS |
| IAB | Performance measurement and optimization | RAN3 |
| V2X and Sidelink | Performance measurement and optimization | RAN2 |
| MIMO enhancement | Performance measurement and optimization | RAN3 |
| Positioning | Performance measurement and optimization | FFS |
| UE radio capability signalling optimization | Not applicable | Not applicable |
| NTN | Reuse R16 MDT/SON | RAN2, RAN3 |
| High speed train | Reuse R16 MDT/SON | RAN2, RAN3 |
| CLI/RIM | Performance measurement and optimization | RAN3 |
| Architecture evolution for E-UTRAN and NG-RAN | Inherit R16 F1 impact of MDT/SON | RAN3 |
| Private network | Reuse R16 MDT/SON; FFS: ANR, network name, PLMN, security… | RAN2, RAN3 |

### AI/ML

Several companies proposed data collection for AI/ML in last plenary. However, the use case and 3GPP impact are not clear. We therefore propose to study the use case in release-17 for:

- Network learning (with and without UE assistance): network collects data from UE and RAN for network learning;

- UE learning with network assistance: network provides assistance data for UE learning.

### Other

The following proposals were submitted in the last RAN plenary meeting.

| Features | Function description | UE impact |
| --- | --- | --- |
| PCI collision detection | RAN to detect PCI collision | RAN3 |
| QoE measurement | Inherit LTE/UMTS QoE measurement mechanism. | RAN2 |
| Energy saving enhancement [3] | TRP on/off; Traffic pattern assisted on/off | RAN3 |
| Inter-RAT [3] | Enhancements for MR-DC MDT; Inter-RAT configuration exchange | RAN3 |

### Conclusion

In this document, we looked at candidate items for study in release-17 RAN data collection enhancements. We observe that the number of candidate items, hence the amount of necessary work to study them, is larger than the capacity of a study item within a single release. We think it is important to perform proper prioritization during the email discussion.

Proposal 1:	Initiate RAN email discussion for release-17 RAN data collection enhancements using the candidate items (in section 2-5).

Proposal 2:	Prioritization of items for study is part of RAN email discussion.

Proposal 3:	RAN email discussion should aim to keep the SI workload truly manageable level.

### References

[1] RP-191215  Further Study on RAN-Centric Data Collection and Utilization	CMCC

[2] RP-191216 Motivation for Further Study on RAN-Centric Data Collection and Utilization	CMCC

[3] RP-191007   Overview of Rel-17 work areas for NR and LTE	Huawei, HiSilicon

[4] RP-191370 SON Enhancements in Rel-17	Ericsson

[5] RP-191077 Draft new SID on AI and its applications in NR	ZTE, Sanechips

[6] RP-190969 CATT's view on NR evolution in Rel-17	CATT