3GPP TSG RAN Meeting #85								RP-192174

Newport Beach, USA, September 16-20, 2019

Agenda Item:	8.2.3

Source:	Apple

Title:	R17 MIMO enhancements

Document for:	Discussion/Decision

# Introduction

Rel-16 MIMO enhancement WI was approved with the following scope [1].

- Extend specification support in the following areas [RAN1] - Enhancements on MU-MIMO support: - Specify overhead reduction, based on Type II CSI feedback, taking into account the tradeoff between performance and overhead - Perform study and, if needed, specify extension of Type II CSI feedback to rank >2 - Enhancements on multi-TRP/panel transmission including improved reliability and robustness with both ideal and non-ideal backhaul: - Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission - Perform study and, if needed, specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission - Enhancements on multi-beam operation, primarily targeting FR2 operation: - Perform study and, if needed, specify enhancement(s) on UL and/or DL transmit beam selection specified in Rel-15 to reduce latency and overhead - Specify UL transmit beam selection for multi-panel operation that facilitates panel-specific beam selection - Specify a beam failure recovery for SCell based on the beam failure recovery specified in Rel-15 - Specify measurement and reporting of either L1-RSRQ or L1-SINR - Perform study and make conclusion in the first RAN1 meeting after start of the WI, and if needed, specify CSI-RS and DMRS (both downlink and uplink) enhancement for PAPR reduction for one or multiple layers (no change on RE mapping specified in Rel-15) - Specify enhancement to allow full power transmission in case of uplink transmission with multiple power amplifiers (assume no change on UE power class)

With the current progress of Rel-16 MIMO enhancement WI, particularly in WG RAN1, it is expected that down scoping of Rel-16 MIMO enhancement will happen especially with respect to the FR2 enhancement such as MPE (Maximum Permissible Emission) solution and multi panel enhancement.

We believe MIMO still remains as a very important topic that should be further enhanced in Rel-17. MIMO covers a wide variety of the design topics including the conventional multiple antenna design in many different scenarios such as multi-TRP, URLLC, etc., more importantly, MIMO also covers a new design for mmW band, i.e. FR2, which faces a completely new design requirement due to analog beam forming. Rel-15 provided very basic solutions for the beam management in FR2, while Rel-16 made very minor enhancement due to the slow progress. We believe FR2 commercial deployment will face new challenges, therefore, further enhancement is needed especially in the area of device power consumption reduction, NW resource efficiency improvement, as well as latency and robustness enhancement, etc..

In this contribution, we provide our view on the Rel-17 MIMO enhancement including the following 3 areas that are very important for us.

- MPE solutions, including enhancement on the UL beam management

- Further beam management latency/overhead/power consumption reduction and robustness improvement, focusing on the event based reporting and UE assistance request

- Beam restriction enhancement for simultaneous Rx/Tx in FR2 CA

# Beam management enhancement for MPE solution

One of the most important topics associated with the UE operation in FR2 frequency region is MPE (Maximum Permissible Exposure). FCC and other regulatory institutes have defined the maximum permissible exposure as the power density in terms of W/m2. Thus, a UE must reduce its transmit power when the human body is in proximity to be compliant with the requirement. With the definition of P-MPR in the power control requirements in TS 38.101-2 and the introduction of the corresponding UE capability maxUplinkDutyCycle, the Rel-15 specification supports basic mechanisms to comply with RF exposure regulations. While maximum UL duty cycle allows a UE to indicate its preferred UL scheduling cycle, the P-MPR is the UE based mechanism that can be always used to reduce transmission power. However, as already discussed in Rel-15, the aforementioned mechanisms lack full flexibility and may result either in shrunk UL coverage (in case of P-MPR) or unutilized UL capacity (in case of fixed conservative UL duty cycle).

In the scope of the Rel-16 NR specification development, RAN1 took an effort to enhance beam management procedures as part of the NR MIMO enhancement Work Item. Unfortunately, in RAN1#98, mainly due to the lack of time, there is no consensus to introduce specification support for MPE solution and, therefore, no further discussion will be allocated in RAN1 [2].

Conclusion On the issue of beam management enhancements to address MPE, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.

However, RAN4 sent a LS in this plenary meeting urging RAN1/RAN2 to provide support for MPE solutions, which reflects the urgency of the MPE issues that are also acknowledged by many companies, especially the device and modem companies, in RAN1. The latest agreement in RAN WG4 is to introduce both P-PMR and maxUplinkDutyCycle into the FR2 RF specification to trade off power reduction and duty cycle to achieve optimal operating points while maintaining compliance. Despite the intensive discussion, this capability is a static indication and remains unchanged over the duration of the RRC connection. Thus, both P-MPR and maxUplinkDutyCycle are to some extent partial solutions and do not provide a way to react to the dynamic environment.

One example of the dynamic environment is when a particular UE implementation may be able to utilize sensor information to minimize the requirement for either actual MPR or uplink duty cycle while still complying to the regulatory requirement. In this case, either fixed maximum UL duty cycle or P-MPR will result in sub-optimal system performance. Another potential scenario is where the configured beam (e.g. with LOS direction) is blocked, but there is another beam direction free of blockage. The DL RSRP of the configured beam along the LOS direction may be still larger than that of the other beams even with the blockage loss; however, the UL RSRP of the configured beam along the LOS direction can be significantly less due to MPE related power back-off in this direction. Since the Rel-15 beam report only includes DL RSRP, the network would still select the beam for the UL assuming beam correspondence.

From the perspective of accelerating the adoption of mmWave technologies, a solution in the NR specification to effectively manage the UE’s Tx beam such that the beam selection is informed by RF exposure considerations is a worthwhile investment of 3GPP resources and can be an essential enhancement of the baseline solution. With the above considerations, in our view, MPE solution is a crucial part of Rel-17 MIMO enhancement

Proposal 1: In Rel-17 MIMO enhancement, solutions to handle MPE should be provided focusing on the beam management enhancement

# Event based beam measurement reporting and UE assistance request

The main purpose of beam measurement and reporting is to track the beam change due to various reasons, such as UE movement, UE rotation, obstacle passing by, etc. Therefore, beam measurement and reporting have the following requirement

- Minimize the overhead for both DL/UL reference signals and corresponding UL/DL measurement report

- Fast response and robust to various mobility situations

Solutions provided by the Rel-15/16 can fulfill basic beam management need but further enhancement is highly desirable

## Event based beam measurement

When periodic or semi-persistent reference signals (CSI-RS, or SSB) are configured, the simplest solution is for UE to make measurement report periodically. However, this approach has the following limitation,

- It is hard for the gNB to optimally configure the periodicity of reference signals and report as gNB has less knowledge about the actual mobility situation compared to UE.

- To ensure robust beam management, frequent reference signals and reports are needed which can cause unnecessary overhead.

- Frequent periodic measurement reporting can also negatively impact the UE power consumption

On the other side, event based measurement report mechanism has been proved to effectively maintain good mobility service for LTE/UMTS etc. and thus deserved to be visited for beam management enhancement. While previous RATs, such as UMTS and LTE, focuses on the cell level mobility management, beam management can be viewed as the beam level mobility where similar concept can be applied in order to reduce the overhead while ensure robust mobility performance.

For event based aperiodic beam measurement reporting, UE only triggers mobility event report when certain predefined condition is met. Below are a few examples of the mobility even used in LTE and UMTS

LTE

- Event A1: Serving cell becomes better than threshold

- Event A2: Serving cell becomes worse than threshold

- Event A3: Neighbor cell becomes offset better than PCell

- Event A4: Neighbor cell becomes better than threshold

UMTS

- Event 1a: a cell enters the reporting range, i.e. add a cell to active set.

- Event 1b: a cell leaves the reporting range, i.e. removed a cell from active set.

- Event 1d: Neighbor cell becomes offset better than PCell.

Figure 1 UE Event Based Aperiodic Beam Measurement Reporting

Figure 1 illustrates the event based measurement reporting compared to periodic report. Assuming NW configures periodic measurement resources (SSB or CSI-RS). The baseline operation is for UE to make the periodic measurement report in order to track the beam change. Note that, due to the un-predictableness of the beam change, a frequent beam reporting is needed in order to ensure the robustness of the beam management as shown by red arrows in Figure 1. Assuming UE desirable receive beam changes due to, e.g., device rotation, UE can detect the change via periodic measurement. Only when the beam change is detected, or certain event is detected, UE is required to report the measurement as shown by green arrows. Clearly, event based measurement can reduce the reporting overhead while maintaining the same robustness of the beam management.

## UE assistance feedback

Based on the current design, the triggering and configuration of the beam management procedure, i.e. P1/P2/P3 or U1/U2/U3, is a gNB decision without much UE direct input on which procedure is preferred. Beam changes typically from UE movement, rotation, blockage (e.g., hand, body or surrounding). UE can utilize various reference signals, measurements or even sensors to detect beam changes, and may figure out causes and preferable actions of such changes. Therefore, it is beneficial to allow UE assisting gNB to configure beam management procedure via, e.g., beam measurement report or other events.

In addition, multi panel support was also heavily discussed in Rel-16 without much progress at all. With the presence of periodic reference signals for beam management, panel management can be done in a rather transparent way by the UE implementation. However, for the prompter reaction to beam/panel quality change, it is beneficial for UE to request aperiodic CSI-RS in order to quickly evaluate the panel condition without wasting much NW resource. Furthermore, beam switching between panels may require longer switching delay, comparing to intra-panel beam switch, especially if the other panel is inactive for power saving purpose. Therefore, it is also desirable for UE to indicate the desired beam switch timing to make the system operate efficiently.

Moreover, for both P1 and P3 procedure, UE needs to perform Rx beam sweep. The number of Rx beams that UE need to sweep may not be known at the gNB or may change dynamically. There can be multiple reasons for UE to prefer sweeping different number of Rx beam, for example, some of the antenna panel could be blocked by the hand, etc. Similarly, for UL beam management, UE Tx beam sweep is required for U1/U3 procedure. It is beneficial for UE to request the number of resource for P3/U3 (P1/U1) beam management procedure.

We listed a few areas that UE assistance request could help both the NW and device. Therefore, we propose

Proposal 2: In Rel-17 MIMO enhancement, for beam management, latency/overhead/power consumption should be further reduced and robustness should be further enhanced, at least in the area of event based measurement report and UE assistance request

# Beam restriction enhancement for simultaneous Rx/Tx in FR2 CA

Carrier Aggregation in FR2 is supported in RAN1, while the current RAN4 spec, 38.101-2, only specify the intra-band CA case. In terms of UE hardware implementation, it is not expected that UE will budget for multiple analog beam steering components for multiple CCs especially for intra-band CA. Figure 2 illustrates a general case where UE uses a common RF analogy beam steering to support FR2 CA.

Figure 2 Common RF analog beam steering component used for FR2 CA

With the limitation on RF analog beam steering component, for FR2 CA, UE is not able to use different beam for simultaneous transmission or receptions of different physical channels, within or across CCs. RAN1 and RAN4 have discussed this issue intensely in Rel-15 [1][2] and provided many specification support to restrict the beam used for intra-band CA and define the UE default behavior. However, the solutions provided in Rel-15 is not complete yet, due to the fact that there are many possible physical channel combinations especially considering the beam switching delay. This issue was brought up in RAN1 TEI again, however, RAN1 TEI only has 3 meetings with only 2 meeting left and there are a lot of proposed issues in TEI agenda. It is possible that this issue won’t be addressed in Rel-16 timeframe which, in that case, should definitely be addressed in Rel-17.

Furthermore, there are other forward compatible issues as following

- Even though current RAN4 spec only specifies the intra-band CA for FR2. It is possible that more band configurations may be introduced in RAN4 for FR2 CA, with the possibility of inter-band CA as well.

- We also believe, handling the simultaneous transmission and reception in FR2 with different beam should be a UE capability issue. With more and more band combination and CA deployment case, a generic solution is preferable in 3GPP to accommodate UE with different capability.

With the above consideration, we have the following proposal

Proposal 3: In Rel-17 MIMO enhancement, beam restriction enhancement for simultaneous Rx/Tx in FR2 CA should be, focusing on intra-band case and potentially extend to inter-band case as well

# Conclusion

We believe MIMO remains as a very important topic that should be further enhanced in Rel-17, especially in the area of beam management enhancement for FR2 deployment. Rel-15 provided very basic solutions for the beam management in FR2, while Rel-16 made very minor enhancement due to the slow progress. We believe FR2 commercial deployment will face new challenges, therefore, further enhancement is needed especially in the area of device power consumption reduction, NW resource efficiency improvement, as well as latency and robustness enhancement, etc..

In this contribution, we provide our view on the Rel-17 MIMO enhancement with the following 3 proposed areas that are very important to us.

Proposal 1: In Rel-17 MIMO enhancement, solutions to handle MPE should be provided focusing on the beam management enhancement

Proposal 2: In Rel-17 MIMO enhancement, for beam management, latency/overhead/power consumption should be further reduced and robustness should be further enhanced, at least in the area of event based measurement report and UE assistance request

Proposal 3: In Rel-17 MIMO enhancement, beam restriction enhancement for simultaneous Rx/Tx in FR2 CA should be, focusing on intra-band case and potentially extend to inter-band case as well

# References

- RP-181453, "WI Proposal on NR MIMO Enhancements", 3GPP TSG RAN Meeting #80 La Jolla, USA, June 11-14, 2018

- Chairman notes, 3GPP TSG RAN WG1 #98 Prague, CZ, August 26th – 30th, 2019

- RP-191660, "LS on enhanced FR2 MPE mitigation solutions for avoiding radio link failures and connection releases in Rel-16", 3GPP TSG RAN Meeting #85 Newport Beach, USA, Sep. 16-20, 2019

- R1-1809881, “LS on UE behaviour on reception of channels or RS in the same OFDM symbol”, Intel, TSG RAN1 Meeting #94, Gothenburg, August 2018.

- R1-1809881, “Reply LS on UE behaviour on reception of channels or RS in the same OFDM symbol”, LGE, TSG RAN4 meeting #97, Reno, May 2019