3GPP TSG-RAN2 Meeting #107 	R2-1911832

Prague, Czech Republic, 26th – 30th August 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0144 | rev | 2 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of handover terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE Corporation (Rapporteur), Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-08-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#106, the Editor’s note regarding the usage of the term “handover” in 8.3 was replaced with the following note: NOTE 1: In this subclause the term "handover" refers to a synchronous reconfiguration not necessarily implying a PCell and/or PSCell change. The text in 8.3 was originally written for EN-DC but for MR-DC, it is not clear what the term “handover” refers to. Even with the new note, it could mean synchronous reconfiguration with or without security key change while the description is aligned with annex A in the case where security key is not changed. Furthermore, the term “reconfiguration with sync” is only defined for NR, so it is unclear what "synchronous reconfiguration" refers to in case of E-UTRA cell groups. In order to work for all MR-DC options, the text should preferably be RAT agnostic by avoiding those terms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 8.3 is updated as follow: - Removing the term “handover” from the second and third bullet. Instead, the “MN security key change” is used, which works for both NR and E-UTRA cell groups, as the protocol actions are dependent on whether the security key is changed or not; - In NOTE 1 is clarified that “handover” refers to an E-UTRA handover or to an NR a synchronous reconfiguration not necessarily implying a PCell and/or PSCell change with or without security key change. - In the fourth bullet is clarified that if a bearer type change happens through SN change procedure, then SN terminated PDCP /and SCG RLC are re-established and SCG MAC is reset. MCG RLC/MAC behavior depends on the solution selected by the network, see Annex A; Impact analysis Impacted functionality: Bearer type change. Impacted 5G architecture options: EN-DC, NGEN-DC, NR-DC, NE-DC Inter-operability: The CR is provide just for clarification and does not require any new UE/NW implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be unclear since the term handover could mean synchronous reconfiguration with or without security key change. Furthermore, the term “reconfiguration with sync” is only defined for NR, so it will be unclear what it refers to in case of E-UTRA cell groups. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 8.3	Bearer type change

In MR-DC, all the possible bearer type change options are supported:

-	MCG bearer to/from split bearer;

-	MCG bearer to/from SCG bearer;

-	SCG bearer to/from split bearer.

Bearer termination point change is supported for all bearer types, and can be performed with or without bearer type change:

-	MN terminated bearer to/from SN terminated bearer.

For MR-DC:

-	when the security key is changed for a bearer, the associated PDCP and RLC entities are re-established, while MAC behavior might depend on the solution selected by the network, e.g. MAC reset, change of LCID, etc. (see Annex A);

-	for MCG bearer, split bearer and SCG bearer, during MN security key change the MCG/SCG PDCP and RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens together with MN security key change then for MCG bearer, split bearer and SCG bearer, the MCG/SCG PDCP and RLC are re-established and MCG/SCG MAC is reset;

-	if a bearer type change happens through SN change procedure, then SN terminated PDCP and SCG RLC are re-established and SCG MAC is reset. MCG RLC/MAC behavior depends on the solution selected by the network, see Annex A;

-	one step (direct) bearer type change between MN terminated bearer types without using the handover procedure is supported;

-	one step (direct) bearer type change between SN terminated bearer types without using the handover or SN change procedure is supported;

-	one step (direct) bearer type change from/to MN terminated bearer to/from SN terminated bearer without using the handover procedure is supported;

-	PDCP version change for DRB (only applicable for EN-DC) or PDCP SN length change for an AM DRB or RLC mode change for DRB is performed using a release and add of the DRBs (in a single message) or full configuration;

-	One step (direct) bearer type change with PDCP version change (only applicable for EN-DC) is supported.

NOTE 1: 	In this subclause the term "handover" refers to an E-UTRA handover or to an NR synchronous reconfiguration not necessarily implying a P(S)Cell change with or without security key change.

NOTE 2:	L2 handling for bearer type change in MR-DC is also summarized in Annex A (the table does not consider the cases that PDCP SN length is changed and avoiding reuse of COUNT).

END OF CHANGES