3GPP TSG-RAN2 Meeting #107 	R2-1911844

# Prague, Czech Republic,  26 – 30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0155 | rev | 2 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In a UE implementation, a UE supports NR and IMS voice over LTE should be allowed to not support IMS voice over NR. However, the current text does not allow the UE implementation. - In TS 23.501, it is specified that “a UE supporting IMS voice over 3GPP access connected to 5GC and that is EPS capable shall also support IMS voice over E-UTRA connected to EPC”. - Remove TBDs for voiceOverSCG-BearerEUTRA-5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in the field description of voiceOverNR so that the UE is allowed to support NR and IMS voice over E-UTRA/EPC but does not support IMS voice over NR. - Clarify in the field description of voiceOverEUTRA-5GC that if the UE supports IMS voice over E-UTRA/5GC and is capable of E-UTRA/EPC, the UE is mandated to support IMS voice over E-UTAR/EPC. - Clarify in the field description of voiceOverNR that if the UE supports IMS voice over NR and is capable of E-UTRA/EPC, the UE is mandated to support IMS voice over E-UTRA/EPC. - Remove TBDs for voiceOverSCG-BearerEUTRA-5GC. Impact analysis Impacted 5G architecture options: E-UTRA connected to 5GC, NR SA, NE-DC Impacted functionality: IMS over E-UTRA connected to 5GC IMS voice over NR Inter-RAT handover from NR or E-UTRA/5GC to LTE for IMS voice call IMS over SCG bearer of NE-DC Inter-operability: If one of the UE and the network is implemented according to the CR but the other is not, there is no inter-operability issue since the changes are the intended behaviours. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It may be misunderstood that the UE supports NR and IMS voice over LTE is required to support IMS voice over NR. - The UE requirements are not clear in the stage-3 specification. - TBDs remains in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revisoin of R2-1910548 |  |  |  |  |  |  |  |  |

### 4.2.13	IMS Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| voiceOverEUTRA-5GC Indicates whether the UE supports IMS voice over E-UTRA via 5GC. It is mandated to the UE if the UE is capable of IMS voice over E-UTRA via 5GC. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. | UE | No | No | No |
| voiceOverNR Indicates whether the UE supports IMS voice over NR. It is mandated to the UE if the UE is capable of IMS voice over NR. Otherwise, the UE does not include this field. If this field is included and the UE is capable of E-UTRA with EPC, the UE shall support IMS voice over E-UTRA via EPC. | UE | No | No | Yes |
| voiceOverSCG-BearerEUTRA-5GC Indicates whether the UE supports IMS voice over SCG bearer of NE-DC. | UE | No | No | N/A |