3GPP TR 21.916 V0.1.0 (2019-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 16 Description;

Summary of Rel-16 Work Items

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 16

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	 Abbreviations	5

4	Rel-16 Executive Summary	6

5	Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC)	7

6	5GS Enhanced support of Vertical and LAN Services	7

7	Cellular IoT support and evolution	7

8	Advanced V2X support	7

9	5G Location and Positionning Services	7

10	UE radio capability signalling optimization	7

11	Satellite Access in 5G	7

12	Enablers for Network Automation Architecture for 5G	7

13	Wireless and Wireline Convergence Enhancement	7

14	Mission Critical, Public Warning, Railways and Maritime	7

15	Streaming and TV	7

16	User Identities, Authentication, multi-device	7

17	Slicing	7

18	Other cross-TSG Release 16 Features	7

19	NR-related Release 16 Features	7

20	Release 16 Features impacting both LTE and NR	7

21	LTE-related Release 16 Features	8

22	All other Release 16 Features	8

Annex A:	List of expected contributions	9

Annex A1:	Template for contributing	17

Annex B:	Process to get further information	19

B.1	General	19

B.2	Direct method	20

B.3	Step by step method	20

Annex C:	Change history	23

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 15 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, slightly rewritten by the TR Rapporteur to ensure overall consistency. The original Work Item Rapporteur inputs can be retrieved as temporary document (tdoc), as stated in the first sentence of each clause.

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided at the beginning of each section. 
It also appears in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

# 1	Scope

The present document provides a summary of each Release 15 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 15, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

KPI	Key Performance Indicator

Rel	3GPP Release

# 4	Rel-16 Executive Summary

Editor's note: To be provided later.

# 5	Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC)

# 6	5GS Enhanced support of Vertical and LAN Services

# 7	Cellular IoT support and evolution

# 8	Advanced V2X support

# 9	5G Location and Positionning Services

# 10	UE radio capability signalling optimization

# 11	Satellite Access in 5G

# 12	Enablers for Network Automation Architecture for 5G

# 13	Wireless and Wireline Convergence Enhancement

# 14	Mission Critical, Public Warning, Railways and Maritime

# 15	Streaming and TV

# 16	User Identities, Authentication, multi-device

# 17	Slicing

# 18	Other cross-TSG Release 16 Features

# 19	NR-related Release 16 Features

# 20	Release 16 Features impacting both LTE and NR

# 21	LTE-related Release 16 Features

# 22	All other Release 16 Features

| Unique_ID | Name | Acronym | WG | WID | WI Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |

Summary based on the input provided by Ericsson in SP-181187.

The CAPIF4xMB work item resulted in the creation of the xMB specification in TS.26.348 [1]. This TS contains the reference point specification for the external interface towards content providers or other 3GPP defined API invokers.

3GPP SA4 aligned the xMB reference point to the Common API Framework (CAPIF) within the CAPIF4xMB work item targeting Release 16. The xMB reference point exists between Content Providers (as API invoker) and the BM-SC. An API invoker can be a 3rd party provider or other 3GPP defined services.

xMB offers a simple interface to control the MBMS delivery procedure and to ingest the data to be delivered via MBMS. The MBMS usage complexity, e.g. handling of packet losses, etc is handled by the BM-SC.

xMB offers control procedures for different session types, namely generic file delivery, application (timed file streaming like DASH over MBMS), streaming and transport-only (access to a plain IP multicast transport). The xMB reference point offers an extended set of control features for MC Services.

As part of the CAPIF4xMB work item, the xMB Stage 2 architecture and procedures are moved from TS 26.346 to a new TS 26.348, containing the xMB Stage 2 and necessary extensions for CAPIF. The new TS also contains more elaborative guidelines around the usage of the different xMB API features.

References

[1]	TS 26.348, "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point"

Annex A:
List of expected contributions

Summary needed for highlighted items. Summary to be produced by the Rapporteur as highlighted in the table below.

Green: summary provided.

No need for summary for items shown with strike through font.

| UID | Name | Acronym | WG | WID | Rapporteur |
| --- | --- | --- | --- | --- | --- |
| 0 | Release 16 Features and related studies | - |  | - | - |
| 0 | Enhancement of Ultra-Reliable (UR) Low Latency Communications (URLLC) | - |  | - | - |
| 830098 | Enhancement of URLLC support in the 5G Core network | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 790008 | Study on enhancement of URLLC supporting in 5GC | FS_5G_URLLC | S2 | SP-180118 | Hui Ni, Huawei |
| 810036 | Study on the security of URLLC for 5GS | FS_5G_URLLC_SEC | S3 | SP-180910 | Rong Wu, Huawei Technologies |
| 820019 | Stage 2 of 5G_URLLC | 5G_URLLC | S2 | SP-181122 | Hui Ni, Huawei |
| 840026 | Security of URLLC for 5GS | 5G_URLLC_SEC | S3 | SP-190351 | Rong Wu, Huawei Technologies |
| 840005 | CT Aspects of 5G_URLLC | 5G_URLLC | ct | CP-191063 | Qi Caixia, Huawei |
| 840060 | CT3 Aspects of 5G_URLLC | 5G_URLLC | C3 | CP-191063 | Qi Caixia, Huawei |
| 840061 | CT4 Aspects of 5G_URLLC | 5G_URLLC | C4 | CP-191063 | Qi Caixia, Huawei |
| 780030 | Stopped - Study on EPC support for Mobility with Low Latency Communication | FS_LLC_Mob | S2 | SP-171069 | Chris PUDNEY |
| 790009 | Stopped - Study on enht of systems using EPS for UR and Availability using commodity equipment | FS_EPS_URACE | S2 | SP-180119 | Chris Pudney |
| 830074 | Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC | R1 | RP-191584 | Huawei |
| 800095 | Study on physical layer enhancements for NR UR Low Latency Cases | FS_NR_L1enh_URLLC | R1 | RP-181477 | Huawei |
| 830174 | Core part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Core | R1 | RP-190726 | Huawei |
| 830274 | Perf. part: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC) | NR_L1enh_URLLC-Perf | R4 | RP-190726 | Huawei |
| 830080 | Support of NR Industrial Internet of Things (IoT) | NR_IIOT | R2 | RP-191561 | Nokia |
| 810060 | Study on channel modeling for indoor industrial scenarios | FS_IIIOT_CM | R1 | RP-182138 | Ericsson |
| 800098 | Study on NR Industrial Internet of Things (IoT) | FS_NR_IIOT | R2 | RP-181479 | Nokia |
| 830180 | Core part: Support of NR Industrial Internet of Things (IoT) | NR_IIOT-Core | R2 | RP-190728 | Nokia |
| 820999 | 5GS Enhanced support of Vertical and LAN Services | - |  | - | - |
| 750004 | Study on Communication for Automation in Vertical Domains | FS_CAV | S1 | SP-170169 | Walewski, Joachim, Siemens AG |
| 800007 | Service requirements for cyber-physical control applications in vertical domains | cyberCAV | S1 | SP-180321 | Joachim W. Walewski (Siemens) |
| 830042 | 5GS Enhanced support of Vertical and LAN Services | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 800028 | Study on 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN | S2 | SP-180507 | Devaki Chandramouli, Nokia |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN | S2 | SP-181120 | Devaki Chandramouli, Nokia |
| 810038 | Study on Security for 5GS Enhanced support of Vertical and LAN Services | FS_Vertical_LAN_SEC | S3 | SP-180697 | Jerichow, Anja (Nokia) |
| 840027 | Security for Vertical_LAN | Vertical_LAN_SEC | S3 | SP-190352 | Jerichow, Anja, Nokia |
| 830010 | CT aspects of Vertical_LAN | Vertical_LAN | ct | CP-191157 | WON, Sung Hwan, Nokia |
| 830035 | CT1 aspects of Vertical_LAN | Vertical_LAN | C1 | CP-191157 | WON, Sung Hwan, Nokia |
| 830036 | CT3 aspects of Vertical_LAN | Vertical_LAN | C3 | CP-191157 | WON, Sung Hwan, Nokia |
| 830037 | CT4 aspects of Vertical_LAN | Vertical_LAN | C4 | CP-191157 | WON, Sung Hwan, Nokia |
| 820027 | Service Enabler Architecture Layer for Verticals | SEAL | S6 | SP-181141 | Basavaraj (Basu) Pattan, Samsung |
| 820067 | NR-based access to unlicensed spectrum | NR_unlic | R1 | RP-191575 | Qualcomm |
| 750045 | Study on NR-based access to unlicensed spectrum | FS_NR_unlic | R1 | RP-181339 | Qualcomm |
| 820167 | Core part: NR-based access to unlicensed spectrum | NR_unlic-Core | R1 | RP-190706 | Qualcomm |
| 820267 | Perf. part: NR-based access to unlicensed spectrum | NR_unlic-Perf | R4 | RP-190706 | Qualcomm |
| 800006 | LAN support in 5G | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 760007 | Study on LAN Support in 5G | FS_5GLAN | S1 | SP-170456 | Nasielski, Jack, Qualcomm |
| 800047 | Stage 1 of 5GLAN | 5GLAN | S1 | SP-180593 | Jose Almodovar, KPN |
| 830024 | Study on non-public networks management | FS_OAM_NPN | S5 | SP-190137 | ZHANG, Kai, Huawei |
| 0 | Cellular IoT support and evolution | - |  | - | - |
| 830043 | Cellular IoT support and evolution for the 5G System | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 770038 | Study on 5G_CIoT | FS_CIoT_5G | S2 | SP-180614 | Sebastian Speicher |
| 820015 | Stage 2 of 5G_CIoT | 5G_CIoT | S2 | SP-181118 | Sebastian Speicher |
| 830013 | CT aspects of 5G_CIoT | 5G_CIoT | ct | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830038 | CT1 aspects of 5G_CIoT | 5G_CIoT | C1 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830039 | CT3 aspects of 5G_CIoT | 5G_CIoT | C3 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830040 | CT4 aspects of 5G_CIoT | 5G_CIoT | C4 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 830041 | CT6 aspects of 5G_CIoT | 5G_CIoT | C6 | CP-111237 | Mahmoud Watfa; Qualcomm |
| 800033 | Study on evolution of Cellular IoT security for the 5G System | FS_CIoT_sec_5G | S3 | SP-180440 | Noamen Ben Henda |
| 760040 | Study on MBMS User Services for IoT | FS_MBMS_IoT | S4 | SP-170592 | Cedric thienot, Expway |
| 800083 | Additional MTC enhancements for LTE | LTE_eMTC5 | R1 | RP-191356 | Ericsson |
| 800183 | Core part: Additional MTC enhancements for LTE | LTE_eMTC5-Core | R1 | RP-190770 | Ericsson |
| 800283 | Perf. part: Additional MTC enhancements for LTE | LTE_eMTC5-Perf | R4 | RP-190770 | Ericsson |
| 800084 | Additional enhancements for NB-IoT | NB_IOTenh3 | R1 | RP-191576 | Huawei |
| 800184 | Core part: Additional enhancements for NB-IoT | NB_IOTenh3-Core | R1 | RP-190757 | Huawei |
| 800284 | Perf. part: Additional enhancements for NB-IoT | NB_IOTenh3-Perf | R4 | RP-190757 | Huawei |
| 810050 | 5G message service | 5GMSG | S1 | SP-180772 | Zheng, Jianping, China Mobile |
| 770004 | Study on 5G message service for MIoT | FS_5GMSG | S1 | SP-170704 | Zheng, Jianping, China Mobile |
| 810014 | Message Service Within the 5G System requirements | MSGin5G | S1 | SP-180920 | Zheng, Jianping, China Mobile |
| 0 | Advanced V2X support | - |  | - | - |
| 790002 | Study on Improvement of V2X Service Handling | FS_V2XIMP | S1 | SP-180247 | SungDuck Chun; LG Electronics |
| 820024 | Improvement of V2X service Handling | V2XIMP | S1 | SP-181013 | SungDuck Chun; LG Electronics |
| 840074 | Application layer support for V2X services | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 780025 | Study on V2XAPP | FS_V2XAPP | S6 | SP-171071 | Niranth Amogh, Huawei Tel.India |
| 810020 | Stage 2 of V2XAPP | V2XAPP | S6 | SP-180898 | Niranth Amogh, Huawei Tel.India |
| 840010 | CT aspects of V2XAPP | V2XAPP | ct | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840075 | CT1 aspects of V2XAPP | V2XAPP | C1 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840076 | CT3 aspects of V2XAPP | V2XAPP | C3 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840077 | CT4 aspects of V2XAPP | V2XAPP | C4 | CP-191153 | Herrero Veron, Christian (Huawei) |
| 840078 | Architecture enhancements for 3GPP support of advanced V2X services | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 760043 | Study on eV2XARC | FS_eV2XARC | S2 | SP-180733 | LaeYoung Kim, LG Electronics |
| 820018 | Stage 2 of eV2XARC | eV2XARC | S2 | SP-181121 | LaeYoung Kim, LG Electronics |
| 840011 | CT aspects of eV2XARC | eV2XARC | ct | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840079 | CT1 aspects of eV2XARC | eV2XARC | C1 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840080 | CT3 aspects of eV2XARC | eV2XARC | C3 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840081 | CT4 aspects of eV2XARC | eV2XARC | C4 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 840082 | CT6 aspects of eV2XARC | eV2XARC | C6 | CP-191154 | Herrero Veron, Christian (Huawei) |
| 800096 | Study on NR Vehicle-to-Everything (V2X) | FS_NR_V2X | R1 | RP-190224 | LG Electronics |
| 770029 | Study on V2X Media Handling and Interaction | FS_mV2X | S4 | SP-170799 | Jung, Kyunghun, Samsung Electronics Co., Ltd |
| 830023 | Study on Security Aspects of 3GPP support for Advanced V2X Services | FS_eV2X_Sec | S3 | SP-190108 | Joonwoong Kim, LG Electronics |
| 830078 | 5G V2X with NR sidelink | 5G_V2X_NRSL | R1 | RP-190984 | LG Electronics |
| 830178 | Core part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Core | R1 | RP-190766 | LG Electronics |
| 830278 | Perf. part: 5G V2X with NR sidelink | 5G_V2X_NRSL-Perf | R4 | RP-190766 | LG Electronics |
| 0 | 5G Location and Positionning Services | - |  | - | - |
| 800013 | 5G positioning services | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 760006 | Study on positioning use cases | FS_5G_HYPOS | S1 | SP-170589 | Ries, Lionel, ESA |
| 800049 | Stage 1 of 5G_HYPOS | 5G_HYPOS | S1 | SP-180329 | Ries, Lionel, ESA |
| 830102 | Enhancement to the 5GC LoCation Services | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 780028 | Study on Enhancement to the 5GC Location Services | FS_eLCS | S2 | SP-180734 | Ming Ai, CATT |
| 810035 | Study on Security of the enhancement to the 5GC location services | FS_eLCS_Sec | S3 | SP-180694 | Wei Zhou, CATT |
| 820016 | Stage 2 of 5G_eLCS | 5G_eLCS | S2 | SP-181119 | Ming Ai, CATT |
| 830003 | CT aspects of 5G_eLCS | 5G_eLCS | ct | CP-190014 | Tingfang Tang, CATT |
| 830044 | CT1 aspects of 5G_eLCS | 5G_eLCS | C1 | CP-191064 | Tingfang Tang, CATT |
| 830045 | CT3 aspects of 5G_eLCS | 5G_eLCS | C3 | CP-191064 | Tingfang Tang, CATT |
| 830046 | CT4 aspects of 5G_eLCS | 5G_eLCS | C4 | CP-191064 | Tingfang Tang, CATT |
| 830077 | NR positioning support | NR_pos | R1 | RP-191156 | Intel |
| 800092 | Study on NR positioning support | FS_NR_pos | R1 | RP-181399 | Intel |
| 830073 | Study on local NR positioning in NG-RAN | FS_NR_local_pos | R3 | RP-190754 | CMCC |
| 830177 | Core part: NR positioning support | NR_pos-Core | R1 | RP-190752 | Intel |
| 830277 | Perf. part: NR positioning support | NR_pos-Perf | R4 | RP-190752 | Intel |
| 0 | UE radio capability signalling optimization | - |  | - | - |
| 840054 | Optimisations on UE radio capability signalling | RACS | S2 | RP-191460 | Haris Zisimopoulos |
| 800025 | Study on RACS | FS_RACS | S2 | SP-180599 | Haris Zisimopoulos, Qualcomm |
| 830027 | Stage 2 of RACS | RACS | S2 | SP-190180 | Haris Zisimopoulos |
| 840003 | CT aspects of RACS | RACS | ct | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840055 | CT1 aspects of RACS | RACS | C1 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840056 | CT3 aspects of RACS | RACS | C3 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 840057 | CT4 aspects of RACS | RACS | C4 | CP-191061 | Chaponniere, Lena, Qualcomm |
| 830079 | Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN | R2 | RP-190657 | MediaTek |
| 800097 | Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects | FS_RACS_RAN | R2 | RP-181459 | MediaTek |
| 830179 | Core part: Optimisations on UE radio capability signalling – NR/E-UTRA Aspects | RACS-RAN-Core | R2 | RP-191088 | MediaTek |
| 0 | Satellite Access in 5G | - |  | - | - |
| 800010 | Integration of Satellite Access in 5G | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 770002 | Study on using Satellite Access in 5G | FS_5GSAT | S1 | SP-170702 | MICHEL, Cyril |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 800099 | Study on solutions for NR to support non-terrestrial networks (NTN) | FS_NR_NTN_solutions | R3 | RP-190710 | Thales |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | MICHEL, Cyril, THALES |
| 0 | Enablers for Network Automation Architecture for 5G | - |  | - | - |
| 830047 | Enablers for Network Automation for 5G | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 760047 | Study of enablers for Network Automation for 5G | FS_eNA | S2 | SP-180792 | Xiaobo Wu, Huawei Technologies |
| 820020 | Stage 2 of eNA | eNA | S2 | SP-181123 | Xiaobo Wu, Huawei Technologies |
| 830009 | CT aspects of eNA | eNA | ct | CP-191111 | Yali Yan, Huawei |
| 830048 | CT3 aspects of eNA | eNA | C3 | CP-191111 | Yali Yan, Huawei |
| 830049 | CT4 aspects of eNA | eNA | C4 | CP-191111 | Yali Yan, Huawei |
| 0 | Wireless and Wireline Convergence Enhancement | - |  | - | - |
| 760044 | Study on the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC | S2 | SP-170380 | Marco Spini, Huawei Technologies |
| 800031 | Study on the security of the Wireless and Wireline Convergence for the 5G system architecture | FS_5WWC_SEC | S3 | SP-180438 | He Li, Huawei Technologies |
| 830050 | Wireless and Wireline Convergence for the 5G system architecture | 5WWC |  | SP-181117 | Marco Spini, Huawei Technologies |
| 820014 | Stage 2 of 5WWC | 5WWC | S2 | SP-181117 | Marco Spini, Huawei Technologies |
| 829910 | CT aspects of 5WWC | 5WWC | ct | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829911 | CT1 aspects of 5WWC | 5WWC | C1 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829912 | CT3 aspects of 5WWC | 5WWC | C3 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 829914 | CT4 aspects of 5WWC | 5WWC | C4 | CP-191156 | Herrero Veron, Christian (Huawei) |
| 830082 | NG interface usage for WWC | 5WWC-NG_interface | R3 | RP-190772 | Huawei |
| 830182 | Core part: NG interface usage for WWC | 5WWC-NG_interface-Core | R3 | RP-190999 | Huawei |
| 0 | Mission Critical, Public Warning, Railways and Maritime | - |  | - | - |
| 790024 | Study on MC services access aspects | FS_MCSAA | S6 | SP-181135 | Joakim Åkesson, Ericsson |
| 800024 | Study into discreet listening and logging for mission critical services | FS_MCLOG | S6 | SP-181138 | Chater-Lea, David, Motorola Solutions |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-180921 | Dom Lazara, Motorola Solutions |
| 800022 | Enhanced Mission Critical Push-to-talk architecture phase 2 | enh2MCPTT | S6 | SP-190068 | Dom Lazara; Motorola Solutions |
| 830051 | Enhancements to Functional architecture and information flows for Mission Critical Data | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 800018 | Stage 2 of eMCData2 | eMCData2 | S6 | SP-180378 | Shih, Jerry, AT&T |
| 830014 | CT aspects of eMCData2 | eMCData2 | C1 | CP-190199 | Val Oprescu (AT&T) |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-181136 | Verweij, Kees, The Police of the Netherlands |
| 810004 | MCData File Distribution support over xMB | MC_XMB | S4 | SP-180665 | Thiénot Cédric, Expway |
| 810011 | Protocol enhancements for Mission Critical Services | MCProtoc16 | C1 | CP-182110 | AXELL, Jörgen |
| 800053 | MBMS APIs for Mission Critical Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 760051 | Study on MBMS APIs for MC Services | FS_MBMSAPI_MC | S6 | SP-180237 | Ling Zhang, TD Tech Ltd |
| 800020 | Stage 2 of MBMS APIs for MC Services | MBMSAPI_MCS | S6 | SP-180380 | Ling Zhang, TD Tech Ltd. |
| 840013 | Enhancement of 3GPP Northbound APIs | eNAPIs | C3 | CP-191233 | Yali Yan, Huawei |
| 800019 | Enhanced Mission Critical System Migration and Interconnection | eMCSMI | S6 | SP-180379 | Chater-Lea, David; Motorola Solutions |
| 810054 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI |  |  | Monnes, Peter, Harris Corporation |
| 800021 | Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI | S6 | SP-180680 | Monnes, Peter, Harris Corporation |
| 800016 | Stage 3 for MC Communication Interworking with Land Mobile Radio Systems | MCCI_CT | C1 | CP-190203 | Monnes, Peter, Harris Corporation |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 800032 | Mission Critical Services Security Enhancements | MCXSec | S3 | SP-180596 | Woodward, Tim, Motorola Solutions, Inc |
| 780003 | Enhancements of Public Warning System | ePWS |  | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 730005 | Study on enhancements of Public Warning System | FS_ePWS | S1 | SP-160733 | SyncTechno Inc., Hyounhee Koo |
| 800052 | Stage 1 of ePWS | ePWS | S1 | SP-170998 | Hyounhee Koo, SyncTechno Inc. |
| 810012 | CT aspects of enhancements of Public Warning System | ePWS | ct | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810053 | Study on stages 2 and 3 of enhancements of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810047 | CT1 aspects of ePWS | ePWS | C1 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 810048 | CT4 aspects of ePWS (Possible impacts) | ePWS | C4 | CP-191155 | TaeHyeon Kim, SyncTechno Inc. |
| 760054 | Mobile Communication System for Railways 2 | MONASTERY2 |  | SP-170451 | Merkel, Juergen, Nokia |
| 760005 | Study on MONASTERY2 | FS_FRMCS2 | S1 | SP-170588 | Merkel, Juergen, Nokia |
| 790023 | Study on application architecture for the Future Railway Mobile Communication System (FRMCS) Phase 2 | FS_FRMCS2_SA6 | S6 | SP-181133 | Oettl, Martin, Nokia |
| 760004 | Stage 1 of MONASTERY2 | MONASTERY2 | S1 | SP-170451 | Merkel, Juergen, Nokia |
| 830007 | Application Architecture for MONASTERY2 | MONASTERY2 | S6 | SP-190064 | Oettl, Martin, Nokia |
| 840009 | CT aspects of MONASTERY2 | MONASTERY2 | ct | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840072 | CT1 aspects of MONASTERY2 | MONASTERY2 | C1 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840073 | CT3 aspects of MONASTERY2 | MONASTERY2 | C3 | CP-191152 | Gkatzikis, Lazaros (Nokia) |
| 840092 | NR support for high speed train scenario | NR_HST | R4 | RP-191512 | CMCC |
| 840192 | Core part: NR support for high speed train scenario | NR_HST-Core | R4 | RP-191512 | CMCC |
| 840292 | Perf. part: NR support for high speed train scenario | NR_HST-Perf | R4 | RP-191512 | CMCC |
| 800011 | Maritime Communication Services over 3GPP System | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 730007 | Study on Maritime Communication Services over 3GPP system | FS_MARCOM | S1 | SP-170453 | SyncTechno Inc., Hyounhee Koo |
| 800051 | Stage 1 of MARCOM | MARCOM | S1 | SP-180594 | Hyounhee Koo, SyncTechno Inc |
| 0 | Streaming and TV | - |  | - | - |
| 800008 | Enhancement of LTE for Efficient delivery of Streaming Service | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 740001 | Study on eLSTR | FS_eLESTR | S1 | SP-160960 | China Telecommunications, XIA Xu |
| 800054 | Stage 1 of eLSTR | eLSTR | S1 | SP-180322 | Xia, Xu, China Telecom |
| 800001 | Enhancements to Framework for Live Uplink Streaming | E_FLUS | S4 | SP-180285 | Lo, Charles, Qualcomm |
| 810003 | Coverage and Handoff Enhancements for Multimedia | CHEM | S4 | SP-180664 | Nikolai Leung |
| 770027 | Study on QoE metrics for VR | FS_QoE_VR | S4 | SP-170614 | Ericsson (Gunnar Heikkilä |
| 760039 | Study on 5G enhanced Mobile Broadband Media Distribution | FS_5GMedia_Distribution | S4 | SP-170334 | Bouazizi, Imed, Samsung Electronics Co., Ltd., |
| 820002 | Media streaming architecture | 5GMSA | S4 | SP-180984 | Frédéric GABIN |
| 810040 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 760041 | Study on Media Handling Aspects of Conversational Services in 5G Systems | FS_5G_MEDIA_MTSI | S4 | SP-170336 | Oyman, Ozgur, Company: Intel |
| 810002 | Media Handling Extensions for 5G Conversational Services | 5G_MEDIA_MTSI_ext | S4 | SP-180663 | Oyman, Ozgur, Company: Intel |
| 770031 | Study on EVS Float Conformance Non Bit-Exact | FS_EVS_FCNBE | S4 | SP-170618 | Fabrice Plante, Intel |
| 820001 | EVS Floating-point Conformance for Non Bit-Exact | EVS_FCNBE | S4 | SP-180983 | Fabrice, Plante, Intel |
| 800003 | Addition of HLG HDR to TV Video Profiles | HLG_HDR | S4 | SP-180287 | Paul Szucs, Sony Europe Limited |
| 0 | User Identities, Authentication, multi-device | - |  | - | - |
| 800012 | User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 780004 | Study on a Layer for User Centric Identifiers and Authentication | FS_LUCIA | S1 | SP-170995 | Bischinger, Kurt; Deutsche Telekom |
| 800050 | Stage 1 of User Identities and Authentication | UIA | S1 | SP-180328 | Bischinger, Kurt; Deutsche Telekom |
| 810044 | Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 800004 | Stage 1 of Multi-device and multi-identity | MuD | S1 | SP-180315 | Peter Bleckert, Ericsson |
| 810010 | CT aspects of Multi-device and multi-identity | MuD | ct | CP-182227 | Axell, Jörgen, Ericsson |
| 810045 | CT1 aspects of Multi-device and multi-identity | MuD | C1 | CP-182227 | Axell, Jörgen, Ericsson |
| 810046 | CT3 aspects of Multi-device and multi-identity | MuD | C3 | CP-182227 | Axell, Jörgen, Ericsson |
| 0 | Slicing | - |  | - | - |
| 830103 | Enhancement of Network Slicing | eNS |  | SP-181232 | ZTE, Tricci So |
| 790011 | Study on Enhancement of Network Slicing | FS_eNS | S2 | SP-180503 | ZTE, Tricci So |
| 820023 | Stage 2 of eNS | eNS | S2 | SP-181232 | ZTE, Tricci So |
| 830011 | CT aspects of eNS | eNS | ct | CP-190196 | Fei Lu, ZTE |
| 830052 | CT1 aspects of eNS | eNS | C1 | CP-190196 | Fei Lu, ZTE |
| 830053 | CT3 aspects of eNS | eNS | C3 | CP-190196 | Fei Lu, ZTE |
| 830054 | CT4 aspects of eNS | eNS | C4 | CP-190196 | Fei Lu, ZTE |
| 810051 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 770005 | Study on Business Role Models for Network Slicing | FS_BMNS | S1 | SP-180169 | Covell, Betsy, Nokia |
| 810015 | Business Role Models for Network Slicing | BRMNS | S1 | SP-180773 | Covell, Betsy, Nokia |
| 810034 | Study on Security aspects of Enhancement of Network Slicing | FS_eNS_SEC | S3 | SP-180692 |  |
| 810022 | Study on tenancy concept in 5G networks and network slicing management | FS_TENANCYC | S5 | SP-180815 | ZHU, Lei (Huawei |
| 810031 | Enhancement of performance assurance for 5G networks including network slicing | 5G_SLICE_ePA | S5 | SP-190247 | Xiaowei Sun (China Mobile) |
| 820029 | Study on Charging Aspects of Network Slicing | FS_NETSLICE_CH | S5 | SP-181065 | Chen Shan, Huawei |
| 0 | Other cross-TSG Release 16 Features | - |  | - | - |
| 760003 | Provision of Access to Restricted Local Operator Services by Unauthenticated UEs | PARLOS |  | SP-170449 | Covell, Betsy Nokia |
| 740002 | Study on Stage 1 of PARLOS | FS_PARLOS | S1 | SP-160904 | Covell, Betsy |
| 760046 | Study on Stage 2 for PARLOS | FS_PARLOS_SA2 | S2 | SP-180501 | Nokia (Nicolas Drevon |
| 800035 | Study on Security Aspects of PARLOS | FS_PARLOS_Sec | S3 | SP-180442 | Greg Schumacher, Sprint |
| 760071 | Stage 1 of PARLOS | PARLOS | S1 | SP-170449 | Covell, Betsy Nokia |
| 810008 | Stage 2 of PARLOS | PARLOS | S2 | SP-180738 | Nokia (Nicolas Drevon |
| 830012 | CT aspects of PARLOS | PARLOS | ct | CP-190197 | Liu, Jennifer; Nokia |
| 830062 | CT1 aspects of PARLOS | PARLOS | C1 | CP-190197 | Liu, Jennifer; Nokia |
| 830063 | CT3 aspects of PARLOS | PARLOS | C3 | CP-190197 | Liu, Jennifer; Nokia |
| 830064 | CT4 aspects of PARLOS | PARLOS | C4 | CP-190197 | Liu, Jennifer; Nokia |
| 830065 | CT6 aspects of PARLOS | PARLOS | C6 | CP-190197 | Liu, Jennifer; Nokia |
| 820043 | Enhancing Topology of SMF and UPF in 5G Networks | ETSUN |  | SP-181116 | Laurent Thiebaut (Nokia) |
| 770039 | Study on ETSUN | FS_ETSUN | S2 | SP-180731 | Laurent Thiebaut (Nokia) |
| 820013 | Stage 2 of ETSUN | ETSUN | S2 | SP-181116 |  |
| 830002 | CT aspects of ETSUN | ETSUN | ct | CP-190192 | Landais, Bruno, Nokia |
| 830055 | CT3 aspects of ETSUN | ETSUN | C3 | CP-190192 | Landais, Bruno, Nokia |
| 830056 | CT4 aspects of ETSUN | ETSUN | C4 | CP-190192 | Landais, Bruno, Nokia |
| 820044 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture | ATSSS |  | SP-181124 | So, Tricci, ZTE |
| 760052 | Study on ATSSS | FS_ATSSS | S2 | SP-180732 | So, Tricci, ZTE |
| 820021 | Stage 2 of ATSSS | ATSSS | S2 | SP-181124 | So, Tricci, ZTE |
| 830016 | CT aspects of ATSSS | ATSSS | ct | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830057 | CT1 aspects of ATSSS | ATSSS | C1 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830058 | CT3 aspects of ATSSS | ATSSS | C3 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 830059 | CT4 aspects of ATSSS | ATSSS | C4 | CP-190201 | ZHOU, Xingyue (Joy), ZTE |
| 840999 | (IETF) TCP Extensions for Multipath Operation with Multiple Addresses (draft-ietf-mptcp-rfc6824bis) | ATSSS | S2-IETF |  |  |
| 840998 | (IETF) 0-RTT TCP Convert Protocol (draft-ietf-tcpm-converters) | ATSSS | S2-IETF |  |  |
| 810041 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC |  | SP-180737 | Chi Ren, China Unicom |
| 790010 | Study for single radio voice continuity from 5GS to 3G | FS_5G-SRVCC | S2 | SP-180239 | Chi Ren, China Unicom |
| 810007 | Single radio voice continuity from 5GS to 3G | 5G_SRVCC | S2 | SP-180897 | Chi Ren, China Unicom |
| 820008 | Security aspects of single radio voice continuity from 5GS to UTRAN | 5GS_UTRAN_SEC | S3 | SP-181037 | Feng Gao, China Unicom |
| 820069 | RAN aspects of 5G_SRVCC | SRVCC_NR_to_UMTS | R2 | RP-190713 | China Unicom |
| 820169 | Core part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Core | R2 | RP-190713 | China Unicom |
| 830270 | Perf. part: Single Radio Voice Call Continuity from 5G to 3G | SRVCC_NR_to_UMTS-Perf | R4 | RP-190713 | China Unicom |
| 840004 | CT aspect of 5G_SRVCC | 5G_SRVCC | ct | CP-191062 | Chi Ren, China Unicom |
| 840058 | CT1 aspect of 5G_SRVCC | 5G_SRVCC | C1 | CP-191062 | Chi Ren, China Unicom |
| 840059 | CT4 aspect of 5G_SRVCC | 5G_SRVCC | C4 | CP-191062 | Chi Ren, China Unicom |
| 810049 | Remote Identification of Unmanned Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 790005 | Study on Remote Identification of Unmanned Aerial Systems | FS_ID_UAS | S1 | SP-180172 | Eddy Hall; Qualcomm |
| 810013 | Stage 1 of Remote Identification of Unmanned Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 800034 | Study on Security Aspects of the 5G Service Based Architecture | FS_SBA_Sec | S3 | SP-180598 | Hans Christian Rudolph, Deutsche Telekom AG |
| 820045 | Enhancements to the Service-Based 5G System Architecture | 5G_eSBA | S2 | SP-181125 |  |
| 790007 | Study on Enhancements to the Service-Based 5G System Architecture | FS_eSBA | S2 | SP-180231 | Tao Sun (China Mobile) |
| 820022 | Stage 2 of 5G_eSBA | 5G_eSBA | S2 | SP-181125 | Tao Sun (China Mobile) |
| 830001 | CT aspects of 5G_eSBA | 5G_eSBA | ct | CP-190191 | Song Yue (China Mobile) |
| 830060 | CT3 aspects of 5G_eSBA | 5G_eSBA | C3 | CP-190191 | Song Yue (China Mobile) |
| 830061 | CT4 aspects of 5G_eSBA | 5G_eSBA | C4 | CP-190191 | Song Yue (China Mobile) |
| 780029 | Study on Enhanced IMS to 5GC Integration | FS_eIMS5G | S2 | SP-180736 | Joul, Chris, T-Mobile USA |
| 840062 | SBA aspects of enhanced IMS to 5GC integration | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 830028 | Stage 2 of eIMS5G_SBA | eIMS5G_SBA | S2 | SP-190181 | T-Mobile USA, Christopher Joul |
| 840006 | CT aspects of eIMS5G_SBA | eIMS5G_SBA | ct | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840063 | CT3 aspects of eIMS5G_SBA | eIMS5G_SBA | C3 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 840064 | CT4 aspects of eIMS5G_SBA | eIMS5G_SBA | C4 | CP-191065 | de Gregorio, Jesus. Ericsson |
| 830081 | Private Network Support for NG-RAN | NG_RAN_PRN | R3 | RP-190729 | China Telecom |
| 830181 | Core part: Private Network Support for NG-RAN | NG_RAN_PRN-Core | R3 | RP-191563 | China Telecom |
| 810039 | Media Handling Aspects of RAN Delay Budget Reporting in MTSI | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 780022 | Study on E2E_DELAY | FS_E2E_DELAY | S4 | SP-170837 | Oyman, Ozgur, Company: Intel, |
| 810001 | Stage 2 of E2E_DELAY | E2E_DELAY | S4 | SP-180662 | Oyman, Ozgur, Company: Intel |
| 830004 | CT Aspects of E2E_DELAY | E2E_DELAY | ct | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830066 | CT1 Aspects of E2E_DELAY | E2E_DELAY | C1 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830067 | CT3 Aspects of E2E_DELAY | E2E_DELAY | C3 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830068 | CT4 Aspects of E2E_DELAY | E2E_DELAY | C4 | CP-190193 | Luetzenkirchen, Thomas, Intel |
| 830069 | Enhancements for Common API Framework for 3GPP Northbound APIs | eCAPIF |  | SP-181137 | Basavaraj Pattan, Samsung |
| 790022 | Stage 2 for eCAPIF | eCAPIF | S6 | SP-181137 | Basavaraj Pattan, Samsung |
| 830022 | Security aspects of eCAPIF | eCAPIF | S3 | SP-1901240 | Rajavelsamy Rajadurai, Samsung |
| 790019 | Usage of CAPIF for xMB API | CAPIF4xMB | S4 | SP-180031 | Thorsten Lohmar, Ericsson LM |
| 840065 | User data interworking, Coexistence and Migration | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 800055 | Study on UDICOM | FS_UDICoM | S2 | SP-190148 | Susana Sabater (Vodafone |
| 830029 | Stage 2 of UDICOM | UDICOM | S2 | SP-190182 | Alessio Casati, Nokia |
| 840007 | CT aspects of UDICOM | UDICOM | C4 | CP-191066 | Wiehe, Ulrich, Nokia |
| 0 | NR-related Release 16 Features | - |  | - | - |
| 830075 | UE Power Saving in NR | NR_UE_pow_sav | R1 | RP-191607 | CATT |
| 800094 | Study on UE power saving in NR | FS_NR_UE_pow_sav | R1 | RP-181463 | CATT |
| 830175 | Core part: UE Power Saving in NR | NR_UE_pow_sav-Core | R1 | RP-190727 | CATT |
| 830275 | Perf. part: UE Power Saving in NR | NR_UE_pow_sav-Perf | R4 | RP-190727 | CATT |
| 820070 | Integrated access and backhaul for NR | NR_IAB |  | RP-191558 | Qualcomm |
| 750047 | Study onNR_IAB | FS_NR_IAB | R2 | RP-181349 | Qualcomm |
| 820170 | Core part: NR_IAB | NR_IAB-Core | R2 | RP-190712 | Qualcomm |
| 820270 | Perf. Part: NR_IAB | NR_IAB-Perf | R4 | RP-190712 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-190106 | Rajavelsamy Rajadurai, Samsung, |
| 800072 | Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16 | R4 | RP-191126 | Huawei |
| 800172 | Core part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Core | R4 | RP-190714 | Huawei |
| 800272 | Perf. part: Band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R16-Perf | R4 | RP-190714 | Huawei |
| 800057 | NR intra-band Carrier Aggregation |  |  |  |  |
| 800073 | Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra | R4 | RP-191194 | Ericsson |
| 800173 | Core part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Core | R4 | RP-190689 | Ericsson |
| 800273 | Perf. part: Rel-16 NR intra band CA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R16_intra-Perf | R4 | RP-190689 | Ericsson |
| 800074 | Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL | R4 | RP-191565 | ZTE |
| 800174 | Core part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Core | R4 | RP-190494 | ZTE |
| 800274 | Perf. part: Rel-16 NR inter-band CA/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2) | NR_CADC_R16_2BDL_xBUL-Perf | R4 | RP-190494 | ZTE |
| 830099 | NR inter-band Carrier Aggregation |  |  |  |  |
| 830083 | UE Conformance Test Aspects - Rel-16 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R16-UEConTest | R5 | RP-190321 | CMCC |
| 830095 | Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL | R4 | RP-191153 | CATT |
| 830195 | Core part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Core | R4 | RP-190692 | CATT |
| 830295 | Perf. part: Rel16 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL | NR_CA_R16_3BDL_1BUL-Perf | R4 | RP-190692 | CATT |
| 830096 | Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL | R4 | RP-191566 | ZTE |
| 830196 | Core part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Core | R4 | RP-190693 | ZTE |
| 830296 | Perf. part: Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R16_3BDL_2BUL-Perf | R4 | RP-190693 | ZTE |
| 831000 | New bands for New Radio |  | R4 |  | Dish Network |
| 800077 | Band 65 for New Radio | NR_band_n65 | R4 | RP-190360 | Dish Network |
| 800177 | Core part: Band 65 for New Radio | NR_band_n65-Core | R4 | RP-190360 | Dish Network |
| 800277 | Perf. part: Band 65 for New Radio | NR_band_n65-Perf | R4 | RP-190360 | Dish Network |
| 820073 | Introduction of NR band n18 | NR_n18 | R4 | RP-191556 | KDDI |
| 820173 | Core part: Introduction of NR band n18 | NR_n18-Core | R4 | RP-190173 | KDDI |
| 820273 | Perf. part: Introduction of NR band n18 | NR_n18-Perf | R4 | RP-190173 | KDDI |
| 820077 | Introduction of NR band n48 | NR_n48 | R4 | RP-190908 | US Cellular |
| 820177 | Core part: Introduction of NR band n48 | NR_n48-Core | R4 | RP-182865 | US Cellular |
| 830271 | Perf. part: Introduction of NR band n48 | NR_n48-Perf | R4 | RP-190140 | Nokia |
| 830090 | Introduction of NR band n14 | NR_n14 | R4 | RP-190965 | AT&T |
| 830190 | Core part: Introduction of NR band n14 | NR_n14-Core | R4 | RP-190165 | AT&T |
| 830290 | Perf. part: Introduction of NR band n14 | NR_n14-Perf | R4 | RP-190165 | AT&T |
| 830091 | Introduction of NR band n30 | NR_n30 | R4 | RP-190967 | AT&T |
| 830191 | Core part: Introduction of NR band n30 | NR_n30-Core | R4 | RP-190166 | AT&T |
| 830291 | Perf. part: Introduction of NR band n30 | NR_n30-Perf | R4 | RP-190166 | AT&T |
| 830097 | Introduction of NR band n259 | NR_n259 | R4 | RP-191416 | Ericsson |
| 830197 | Core part: Introduction of NR band n259 | NR_n259-Core | R4 | RP-190765 | Ericsson |
| 830297 | Perf. part: Introduction of NR band n259 | NR_n259-Perf | R4 | RP-190765 | Ericsson |
| 840097 | Introduction of NR supplementary DL band n29 | NR_n29 | R4 | RP-191474 | Dish Network |
| 840197 | Core part: Introduction of NR supplementary DL band n29 | NR_n29-Core | R4 | RP-191474 | Dish Network |
| 840297 | Perf. part: Introduction of NR supplementary DL band n29 | NR_n29-Perf | R4 | RP-191474 | Dish Network |
| 831001 | New Radio Band-related topics |  | R4 |  | Nokia |
| 830087 | RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1 | R4 | RP-191114 | Huawei |
| 830187 | Core part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Core | R4 | RP-191114 | Huawei |
| 830287 | Perf. part: RF requirements for NR frequency range 1 (FR1) | NR_RF_FR1-Perf | R4 | RP-191114 | Huawei |
| 830088 | Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2 | R4 | RP-190925 | China Telecom |
| 830188 | Core part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Core | R4 | RP-190760 | China Telecom |
| 830288 | Perf. part: Add support of NR DL 256QAM for frequency range 2 (FR2) | NR_DL256QAM_FR2-Perf | R4 | RP-190760 | China Telecom |
| 830089 | NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh | R4 | RP-191290 | Nokia |
| 830189 | Core part: NR RF requirement enhancements for frequency range 2 (FR2) | NR_RF_FR2_req_enh-Core | R4 | RP-190761 | Nokia |
| 830092 | Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5 | R4 | RP-191116 | Huawei |
| 830192 | Core part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Core | R4 | RP-191116 | Huawei |
| 830292 | Perf. part: Introduction of an NR SUL (supplemental uplink) band with same uplink frequency range as NR band n5 | NR_SUL_UL_n5-Perf | R4 | RP-191116 | Huawei |
| 820074 | Adding wider channel bandwidths in NR band n7 | NR_n7_BW | R4 | RP-190137 | BT |
| 820174 | Core part: Adding wider channel bandwidths in NR band n7 | NR_n7_BW-Core | R4 | RP-190137 | BT |
| 830093 | Addition of wider channel bandwidth in NR band n38 | NR_n38_BW | R4 | RP-191472 | Huawei |
| 830193 | Core part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Core | R4 | RP-190392 | Huawei |
| 830293 | Perf. part: Addition of wider channel bandwidth in NR band n38 | NR_n38_BW-Perf | R4 | RP-190392 | Huawei |
| 820075 | Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW | R4 | RP-191569 | KDDI |
| 820175 | Core part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Core | R4 | RP-190174 | KDDI |
| 820275 | Perf. part: Addition of 30MHz channel bandwidth for NR band n41 | NR_n41_BW-Perf | R4 | RP-190174 | KDDI |
| 820076 | Adding wider channel bandwidths in NR band n50 | NR_n50_BW | R4 | RP-182829 | Huawei |
| 820176 | Core part: Adding wider channel bandwidths in NR band n50 | NR_n50_BW-Core | R4 | RP-182829 | Huawei |
| 800085 | Enhancements on MIMO for NR | NR_eMIMO | R1 | RP-182863 | Samsung |
| 800185 | Core part: Enhancements on MIMO for NR | NR_eMIMO-Core | R1 | RP-182863 | Samsung |
| 800285 | Perf. part: Enhancements on MIMO for NR | NR_eMIMO-Perf | R4 | RP-182863 | Samsung |
| 800087 | NR mobility enhancements | NR_Mob_enh | R2 | RP-190489 | Intel |
| 800187 | Core part: NR mobility enhancements | NR_Mob_enh-Core | R2 | RP-190489 | Intel |
| 800287 | Perf. part: NR mobility enhancements | NR_Mob_enh-Perf | R4 | RP-190489 | Intel |
| 820068 | 2-step RACH for NR | NR_2step_RACH | R1 | RP-190711 | ZTE |
| 820168 | Core part: 2-step RACH for NR | NR_2step_RACH-Core | R1 | RP-190711 | ZTE |
| 820268 | Perf. part: 2-step RACH for NR | NR_2step_RACH-Perf | R4 | RP-190711 | ZTE |
| 840091 | SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT | R3 | RP-191594 | CMCC |
| 840191 | Core part: SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-Core | R3 | RP-191594 | CMCC |
| 840093 | RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas | R4 | RP-191580 | CATT |
| 840193 | Core part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Core | R4 | RP-191580 | CATT |
| 840293 | Perf. part: RRM requirement for CSI-RS based L3 measurement in NR | NR_CSIRS_L3meas-Perf | R4 | RP-191580 | CATT |
| 840094 | NR performance requirement enhancement | NR_perf_enh | R4 | RP-191587 | China Telecom |
| 840294 | Perf. part: NR performance requirement enhancement | NR_perf_enh-Perf | R4 | RP-191587 | China Telecom |
| 840095 | NR RRM enhancement | NR_RRM_enh | R4 | RP-191601 | Intel |
| 840195 | Core part: NR RRM enhancement | NR_RRM_enh-Core | R4 | RP-191601 | Intel |
| 840295 | Perf. part: NR RRM enhancement | NR_RRM_enh-Perf | R4 | RP-191601 | Intel |
| 840096 | Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL | R4 | RP-191196 | Ericsson |
| 840196 | Core part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Core | R4 | RP-191196 | Ericsson |
| 840296 | Perf. part: Rel-16 NR inter-band Carrier Aggregation for 4 bands DL with 1 band UL | NR_CA_R16_4BDL_1BUL-Perf | R4 | RP-191196 | Ericsson |
| 840098 | Addition of wider channel bandwidth in NR band n1 | NR_n1_BW | R4 | RP-191595 | Ericsson |
| 840198 | Core part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Core | R4 | RP-191595 | Ericsson |
| 840298 | Perf. part: Addition of wider channel bandwidth in NR band n1 | NR_n1_BW-Perf | R4 | RP-191595 | Ericsson |
| 840099 | Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW | R4 | RP-191606 | Huawei |
| 840199 | Core part: Addition of channel bandwidths in Band n77 and n78 | NR_n77_n78_BW-Core | R4 | RP-191606 | Huawei |
| 841000 | Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL | R4 | RP-191598 | ZTE |
| 841100 | Core part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Core | R4 | RP-191598 | ZTE |
| 841200 | Perf. part: Dual Connectivity (EN-DC) with 3 bands DL and 3 bands UL | DC_R16_LTE_NR_3DL3UL-Perf | R4 | RP-191598 | ZTE |
| 841001 | Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex | R4 | RP-191567 | Huawei |
| 841101 | Core part: Introduction of NR FDD bands with variable duplex and corresponding framework | NR_FDD_bands_varduplex-Core | R4 | RP-191567 | Huawei |
| 841002 | Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC | R4 | RP-191604 | Huawei |
| 841102 | Core part: Common RF requirement configured output power for EN-DC with 3 uplink CC and 2 different bands (2CC LTE, 1CC NR FR1) | DC_Pcmax_3UL_CC-Core | R4 | RP-191604 | Huawei |
| 0 | Release 16 Features impacting both LTE and NR | - |  | - | - |
| 800088 | LTE-NR & NR-NR Dual Connectivity and NR Carrier Aggregation enhancements | LTE_NR_DC_CA_enh | R2 | RP-191600 | Ericsson |
| 800188 | Core part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Core | R2 | RP-190452 | Ericsson |
| 800288 | Perf. part: LTE-NR & NR-NR Dual Connectivity and NR CA enhancements | LTE_NR_DC_CA_enh-Perf | R4 | RP-190452 | Ericsson |
| 820079 | High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD | R4 | RP-190315 | CMCC |
| 820061 | Study on high power UE (power class 2) for EN-DC (1 LTE FDD band + 1 NR TDD band) | FS_ENDC_UE_PC2_FDD_TDD | R4 | RP-190167 | China Unicom |
| 820179 | Core part: High power UE (power class 2) for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Core | R4 | RP-190315 | CMCC |
| 820279 | Perf. part: Power class 2 UE for EN-DC (1 LTE TDD band + 1 NR TDD band) | ENDC_UE_PC2_TDD_TDD-Perf | R4 | RP-190315 | CMCC |
| 790054 | eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo | R3 | RP-180531 | China Unicom |
| 790154 | Core part: eNB(s) Architecture Evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo-Core | R3 | RP-181375 | China Unicom |
| 820080 | LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex | R4 | RP-191588 | KDDI |
| 820180 | Core part: LTE/NR spectrum sharing in Band 41/n41 frequency range | NR_n41_LTE_41_coex-Core | R4 | RP-190175 | KDDI |
| 800078 | 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm | R4 | RP-190502 | Sprint |
| 800178 | Core part: 29 dBm UE Power Class for LTE band 41 and NR Band n41 | LTE_NR_B41_Bn41_PC29dBm-Core | R4 | RP-190502 | Sprint |
| 800058 | Dual Connectivity (EN-DC) |  | R4 |  | Ericsson |
| 800066 | Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL | R4 | RP-191568 | NTT DOCOMO |
| 800166 | Core part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Core | R4 | RP-190705 | NTT DOCOMO |
| 800266 | Perf. part: Dual Connectivity (EN-DC) of 1 LTE band (1UL/1DL) and 1 NR band (1DL/1UL) | DC_R16_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-190705 | NTT DOCOMO |
| 800067 | Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL | R4 | RP-191124 | Huawei |
| 800167 | Core part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Core | R4 | RP-190379 | Huawei |
| 800267 | Perf. part: Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-190379 | Huawei |
| 800068 | Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL | R4 | RP-191195 | Ericsson |
| 800168 | Core part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Core | R4 | RP-190159 | Ericsson |
| 800268 | Perf. part: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-190159 | Ericsson |
| 800069 | Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL | R4 | RP-191205 | Nokia |
| 800169 | Core part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Core | R4 | RP-190294 | Nokia |
| 800269 | Perf. part: Dual Connectivity (EN-DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-190294 | Nokia |
| 800070 | Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL | R4 | RP-180997 | Samsung |
| 800170 | Core part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Core | R4 | RP-180997 | Samsung |
| 800270 | Perf. part: Dual Connectivity (EN-DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R16_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-180997 | Samsung |
| 800071 | Dual Connectivity (EN-DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL | R4 | RP-190949 | LG Electronics |
| 800171 | Core part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Core | R4 | RP-190151 | LG Electronics |
| 800271 | Perf. part: Dual Connectivity (EN-DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R16_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-190151 | LG Electronics |
| 0 | LTE-related Release 16 Features | - |  | - | - |
| 790057 | E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US | R4 | RP-182405 | Globalstar |
| 790157 | Core part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Core | R4 | RP-182405 | Globalstar |
| 790257 | Perf. part: E-UTRA 2.4 GHz TDD Band for US | LTE_TDD_2400_US-Perf | R4 | RP-182405 | Globalstar |
| 820063 | UE Conformance Test Aspects - E-UTRA 2.4 GHz TDD Band for US (Band 53) | LTE_TDD_2400_US-UEConTest | R5 | RP-182406 | Globalstar |
| 800059 | LTE intra-band Carrier Aggregation (CA) |  | R4 |  | Qualcomm |
| 800060 | Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra | R4 | RP-191193 | Ericsson |
| 800160 | Core part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Core | R4 | RP-190157 | Ericsson |
| 800260 | Perf. part: Rel-16 LTE intra-band CA for x CC DL/y CC UL including contiguous and non-contiguous spectrum (x>=y) | LTE_CA_R16_intra-Perf | R4 | RP-190157 | Ericsson |
| 800061 | Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL | R4 | RP-191085 | Qualcomm |
| 800161 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Core | R4 | RP-191085 | Qualcomm |
| 800261 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 1 band UL | LTE_CA_R16_2BDL_1BUL-Perf | R4 | RP-191085 | Qualcomm |
| 800062 | Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL | R4 | RP-191120 | Huawei |
| 800162 | Core part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Core | R4 | RP-191120 | Huawei |
| 800262 | Perf. part: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL | LTE_CA_R16_3BDL_1BUL-Perf | R4 | RP-191120 | Huawei |
| 800063 | Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL | R4 | RP-190906 | Nokia |
| 800163 | Core part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Core | R4 | RP-190138 | Nokia |
| 800263 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL | LTE_CA_R16_xBDL_1BUL-Perf | R4 | RP-190138 | Nokia |
| 800064 | Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL | R4 | RP-190377 | Huawei |
| 800164 | Core part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Core | R4 | RP-190377 | Huawei |
| 800264 | Perf. part: Rel-16 LTE inter-band CA for 2 bands DL with 2 band UL | LTE_CA_R16_2BDL_2BUL-Perf | R4 | RP-190377 | Huawei |
| 800065 | Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL | R4 | RP-190946 | LG Electronics |
| 800165 | Core part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Core | R4 | RP-190150 | LG Electronics |
| 800265 | Perf. part: Rel-16 LTE inter-band CA for x bands DL (x= 3, 4, 5) with 2 band UL | LTE_CA_R16_xBDL_2BUL-Perf | R4 | RP-190150 | LG Electronics |
| 810061 | UE Conformance Test Aspects – Rel-16 LTE CA configurations | LTE_CA_R16-UEConTest | R5 | RP-182648 | Ericsson |
| 830076 | LTE-based 5G terrestrial broadcast | LTE_terr_bcast | R1 | RP-190732 | Qualcomm |
| 800091 | Study on LTE-based 5G terrestrial broadcast | FS_LTE_terr_bcast | R1 | RP-181342 | Qualcomm |
| 830176 | Core part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Core | R1 | RP-190732 | Qualcomm |
| 830276 | Perf. part: LTE-based 5G terrestrial broadcast | LTE_terr_bcast-Perf | R4 | RP-190732 | Qualcomm |
| 830094 | Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72 | R4 | RP-191285 | Nokia |
| 830194 | Core part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Core | R4 | RP-190665 | Nokia |
| 830294 | Perf. part: Power class 2 UE for LTE bands 31 and 72 | LTE_PC2_B31_B72-Perf | R4 | RP-190665 | Nokia |
| 800056 | Additional LTE bands |  | R4 |  | Ericsson |
| 800075 | Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1 | R4 | RP-190332 | Ericsson |
| 800175 | Core part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Core | R4 | RP-190332 | Ericsson |
| 800275 | Perf. part: Additional LTE bands for UE category M1 and/or NB1 in Rel-16 | LTE_bands_R16_M1_NB1-Perf | R4 | RP-190332 | Ericsson |
| 800076 | Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2 | R4 | RP-190333 | Ericsson |
| 800176 | Core part: Additional LTE bands for UE category M2 and/or NB2 in Rel-16 | LTE_bands_R16_M2_NB2-Core | R4 | RP-190333 | Ericsson |
| 800079 | Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2 | R4 | RP-181482 | NTT DOCOMO |
| 800179 | Core part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Core | R4 | RP-181482 | NTT DOCOMO |
| 800279 | Perf. part: Further performance enhancement for LTE in high speed scenario | LTE_high_speed_enh2-Perf | R4 | RP-181482 | NTT DOCOMO |
| 800082 | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM | R1 | RP-191546 | LG Electronics |
| 800182 | Core part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Core | R1 | RP-190700 | LG Electronics |
| 800282 | Perf. part: Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | NR_CLI_RIM-Perf | R4 | RP-190700 | LG Electronics |
| 800086 | DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE | R1 | RP-182901 | Huawei |
| 800186 | Core part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Core | R1 | RP-182901 | Huawei |
| 800286 | Perf. part: DL MIMO efficiency enhancements for LTE | LTE_DL_MIMO_EE-Perf | R4 | RP-182901 | Huawei |
| 820071 | Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer | R3 | RP-190160 | Huawei |
| 820171 | Core part: Transfer of Iuant interface specifications from 25-series to 37-series | Iuant_transfer-Core | R3 | RP-190160 | Huawei |
| 820072 | Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR | R3 | RP-182886 | Ericsson |
| 820172 | Core part: Direct data forwarding between NG-RAN and E-UTRAN nodes for inter-system mobility | Direct_data_fw_NR-Core | R3 | RP-182886 | Ericsson |
| 800089 | Even further mobility enhancement in E-UTRAN | LTE_feMob | R2 | RP-190921 | China Telecom |
| 800189 | Core part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Core | R2 | RP-190272 | China Telecom |
| 800289 | Perf. part: Even further mobility enhancement in E-UTRAN | LTE_feMob-Perf | R4 | RP-190272 | China Telecom |
| 820078 | 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR | R4 | RP-191287 | Nokia |
| 820178 | Core part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Core | R4 | RP-182898 | Nokia |
| 820278 | Perf. part: 410 - 430 MHz E-UTRA FDD Band(s) for LTE PPDR and PMR/PAMR in Europe | LTE410_Europe_PPDR-Perf | R4 | RP-182898 | Nokia |
| 0 | All other Release 16 Features | - |  | - | - |
| 830086 | Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR | R4 | RP-190642 | Ericsson |
| 830186 | Core part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Core | R4 | RP-190642 | Ericsson |
| 830286 | Perf. part: Introduction of GSM, UTRA, E-UTRA and NR capability set(s) (CS(s)) to the multi-standard radio (MSR) specifications | MSR_GSM_UTRA_LTE_NR-Perf | R4 | RP-190642 | Ericsson |
| 840066 | Volume Based Charging Aspects for VoLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 810021 | Stage 2 of VBCLTE | VBCLTE | S5 | SP-180813 | Chen, Ai, China Mobile |
| 840012 | CT aspects of VBCLTE | VBCLTE | ct | CP-191110 | Huang Zhenning (China Mobile) |
| 840067 | CT1 aspects of VBCLTE | VBCLTE | C1 | CP-191110 | Huang Zhenning (China Mobile) |
| 840068 | CT3 aspects of VBCLTE | VBCLTE | C3 | CP-191110 | Huang Zhenning (China Mobile) |
| 790003 | Enhancements to IMS for new real time communication services | enIMS | S1 | SP-180137 | China Mobile, Hui Cai |
| 820028 | Enhancement of 5G PCC related services | en5GPccSer | C3 | CP-183246 | Xiaoyun Zhou, Huawei |
| 810023 | Energy efficiency of 5G | EE_5G | S5 | SP-180819 | CORNILY, Jean-Michel, ORANGE |
| 820039 | Signalling Improvements for Network Efficiency in 5GS | SINE_5G | C1 | CP-190142 | Shu, Lin |
| 790001 | New Services and Markets Technology Enablers – Phase 2 | SMARTER_Ph2 | S1 | SP-180589 | Li, Alice, Vodafone |
| 780055 | 5G Voice Service Continuity | 5GVSC | S1 | SP-171077 | Qun Wei, China Unicom |
| 790015 | Security Assurance Specification for 5G | SCAS_5G | S3 | SP-180689 | Marcus Wong, Huawei Technologies |
| 840069 | 5GS Transfer of Policies for Background Data Transfer | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 800030 | Stage 2 of xBDT | xBDT | S2 | SP-180509 | Takeshi Usui KDDI |
| 840014 | CT aspects of xBDT | xBDT | ct | CP-191113 | Yan Xiaojian, ZTE |
| 840070 | CT1 aspects of xBDT | xBDT | C1 | CP-191113 | Yan Xiaojian, ZTE |
| 840071 | CT3 aspects of xBDT | xBDT | C3 | CP-191113 | Yan Xiaojian, ZTE |
| 770020 | Service Interactivity | SerInter | S4 | SP-170796 | Lo, Charles, Qualcomm |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Gibbs, Jon, Huawei Technologies Co Ltd |
| 800002 | Alternative EVS implementation using updated fixed-point basic operators | Alt_FX_EVS | S4 | SP-180286 | Pawate, Raj, Cadence Design Systems Inc |
| 800046 | Shared Data Handling on Nudm and Nudr | Shared_Data | C4 | CP-181136 | Wiehe, Ulrich, Nokia |
| 800009 | Policy delivery to UE for background data transfer | PDBDT | S1 | SP-180324 | Takatsugu Kito, KDDI |
| 790004 | QoS Monitoring | QoS_MON | S1 | SP-180141 | Amar Deol |
| 800005 | Inter-RAT Mobility requirements for real time service | MOBRT | S1 | SP-180592 | Yang Xu, Oppo |
| 760058 | Management of QoE measurement collection | QOED | S5 | SP-181069 | Petersen, Robert, Ericsson |
| 760059 | Study on network policy management for mobile networks based on NFV scenarios | FS_NETPOL | S5 | SP-170593 | China Mobile Jinglei Liu |
| 810025 | Network policy management for mobile networks based on NFV scenarios | NETPOL | S5 | SP-180821 | China Mobile Hao Zhang |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 810024 | OAM aspects of LTE and WLAN integration | OAM_LTE_WLAN | S5 | SP-180820 | Yizhi Yao, Intel Corporation |
| 810026 | Methodology for 5G management specifications | METHOGY | S5 | SP-180822 | Groenendijk, Jan, Ericsson |
| 820030 | Nchf Online and Offline charging services | OFSBI_CH | S5 | SP-181066 | Zhu Lei, Huawei |
| 820031 | Charging Enhancement of 5GC interworking with EPC | 5GIEPC_CH | S5 | SP-181067 | Chen Shan, Huawei |
| 820032 | NRM enhancements | eNRM | S5 | SP-190140 | Jing Ping, Nokia |
| 820033 | Network Exposure Charging in 5G System Architecture | 5GS_Ph1_NEFCH | S5 | SP-181070 | Tornkvist, Robert, Ericsson |
| 820034 | Charging AMF in 5G System Architecture Phase 1 | 5GS_Ph1_AMFCH | S5 | SP-181071 | Gardella, Maryse, Nokia |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-181072 | Attila Horvat, Huawei Technologies Sweden AB |
| 820036 | Trace Management in the context of Services Based Management Architecture | TM_SBMA | S5 | SP-181073 | Andrianov, Anatoly, Nokia |
| 820005 | Lawful Interception Rel-16 | LI16 | S3LI | SP-181210 | Alex Leadbeater, BT |
| 820038 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc16 | C1 | CP-183084 | Leis, Peter, Nokia |
| 820041 | Stage-3 5GS NAS protocol development | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820049 | Stage-3 5GS NAS protocol development general aspects | 5GProtoc16 | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820050 | Stage-3 5GS NAS protocol development for support for non-3GPP accesses | 5GProtoc16-non3GPP | C1 | CP-183087 | Sedlacek, Ivo, Ericsson |
| 820042 | Stage-3 SAE Protocol Development for Rel16 | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820046 | Stage-3 SAE Protocol Development for Rel16 general aspects | SAES16 | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820047 | Stage-3 SAE Protocol Development for Rel16 for CS Fallback in EPS | SAES16-CSFB | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 820048 | Stage-3 SAE Protocol Development Rel16 for support for non-3GPP accesses | SAES16-non3GPP | C1 | CP-183088 | Aghili, Behrouz, InterDigital Communications |
| 840017 | Reliable Data Service Serialization Indication | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840018 | S6b Optional for ePDG connected to 5GS | 5GS_S6b_Optional | S2 | SP-190447 | Irfan Ali, Cisco |
| 840002 | Service Based Interface Protocol Improvements | SBIProtoc16 | ct | CP-191060 | Yue Song, China Mobile |
| 840052 | CT3 aspects of SBIProtoc16 | SBIProtoc16 | C3 | CP-191060 | Yue Song, China Mobile |
| 840053 | CT4 aspects of SBIProtoc16 | SBIProtoc16 | C4 | CP-191060 | Yue Song, China Mobile |
| 840008 | Withdrawal of TS 24.323 from Rel-11, Rel-12, Rel-13 | ISAT-MO-WITHDRAW | C1 | CP-191151 | Dawes, Peter, Vodafone |
| 840051 | Lawful Interception Report Rel-16 | LIR16 | S3LI | SP-190342 | Nag Rao, Nokia |
| 770050 | (Small) Technical Enhancements and Improvements for Rel-16 | TEI16 |  |  |  |
| 800100 | (IETF) The Transport Layer Security (TLS) Protocol Version 1.3 (RFC 8446) | TEI16 | S3-IETF |  |  |

Annex A1:
Template for contributing

Editor's note: To contribute to this document, please copy-paste the following template into a document to be submitted to the TSG plenary as "Work Item Summary". This will be further endorsed by the plenary.
Each summary is expected to be one page long maximum. 
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions.

---

Summary based on the input provided by [company name] in [tdoc numbed].

| Unique_ID | Name | Acronym | WG | WID | WI_rapporteur_name |
| --- | --- | --- | --- | --- | --- |
| AABBCC | Title here | XXX | e.g. S4 | xP-19XXXX | Firstname Familyname, Company |

1	Introduction

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- remember that only what ended up in the specs is of interest;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

2	Description

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- remember that only what ended up in the specs shall be covered here;

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illlustrate and  facilitate the understanding;

- remember that you do not write the WI summary for people who contributed to this WI so make sure that you use terminology and abbreviations that can be understood by any engineer or make sure you explain them;

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- make the following test: give this summary to a colleague who was not at all involved in this topic and let this colleague explain in her/his own words how your WI works after reading the summary>

3	References

<guidance: It is recommended to add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last status report (if any)

- main CRs

NOTE: Please provide references that can be found easily, e.g. indicating Tdoc numbers.>

[X] TS XX.XXX, Spec title

Annex B:
Process to get further information

# B.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# B.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# B.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure B.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure B.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure B.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure B.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex C:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | TSG#85 |  |  |  |  | Initital draft, call for contributions | 0.1.0 |