3GPP TSG RAN Meeting #85 	RP-192237

Newport Beach, USA, Sep 16-20, 2019

Title:	OFDM operation over satellite

Source: 	Inmarsat, Intelsat, HNS, Thales, ESA, Fraunhofer IIS, Fraunhofer HHI

Type:	Discussion

Document for:	Discussion

Agenda Item:	8.1.11

# Introduction

This document is revision of RP-192203 and such supersedes it.

This document discusses some of the implications of using CP-OFDM in the different satellite scenarios on Output Back-Off depending on spectral efficiency.

# Discussion

The distortion caused by non-linear amplifier characteristics is a function and proportional to the PAPR of the signal, modulation scheme, BER and EVM requirement. This may impact the Output back-off and hence the high power amplifier efficiency which directly reduces link capacity.

The operation of OFDM in satellite depends on the type of scenarios considered

- Satellite system operating with spectral efficiency lower than typically 2.0 b/s/Hz.

- Satellite system operating with spectral efficiency higher than typically 2.0 b/s/Hz), in which satellite payload can operate either single or multi carrier per high power amplifier or HPA (e.g. TWTA or SSPA). Typically most current in orbit satellites operate on the basis of a single carrier per HPA while it is expected that future satellite will increasingly support multi carrier per HPA

Assuming the usage of CP-OFDM in the downlink, the distortion can theoretically be reduced via the following methods:

- Use of Digital Pre-distortion

- Relaxing BLER requirements by using HARQ which is possible over LEO system but more of a challenge over geostationary satellite

Here under some further considerations on OBO with single/multi carrier per HPA operation, and impact of BLER and the order of the modulation scheme.

- Output back-off of the amplifier operating point (OBO):

Current satellite systems based on a single-carrier-per-beam access scheme typically operate with < 1 dB OBO with BLER < 1E-03 and anything more needs carefully considerations as any reductions in the order of multiple dB are not practical from capacity perspective. This create an issue since as reported in [2], [3], [4] as part of 5G NR evaluation for NTN. As example link level simulations reported in [3] for BLER of 1E-02:

- TWTA amplifier: OBO for QPSK is below 3 dB and 16 QAM is below 4 dB.

- SSPA amplifier: An improvement is observed here, where also 16 QAM shows OBO values below 3 dB

For future satellite payload architecture operating with multi carrier per HPA, the operating point of the OBO will be relaxed regardless of the waveform.

- BLER requirements

A relaxed BLER requirements will translate to a reduction in the output back-off requirements. This could be compensated by HARQ mechanisms which are possible for Non-GEO system but more of a challenge over geostationary satellite.

We also recognize that there could be other verticals (e.g. IoT) that can still operate with a BLER of 1E-02 for delay tolerant applications such as store and forward IoT. On the other hand, for GEO system, the system has to operate with BLER typically <1E-03.

- Lower order modulation schemes

Low order modulation schemes (e.g. QPSK and to some extent 16QAM) benefits from smaller OBO. This particularly is viable for services which operate with lower spectral efficiency.

For high data rate services, which are expected to operate with higher order modulation schemes (some of 16-QAM MCS and above) higher OBO will be required, which will reduce the power efficiency.

# Conclusion

Observation 1: Operating a radio interface via satellite should target a required Output Back-Off of less than typically 3 dB for BLER <1E-03, so that the system efficiency can be considered as acceptable for Rel 17 baseline.

Observation 2: Digital Pre-Distortion and/or relaxed BLER requirement enable to reduce the required OBO.

Observation 3: For future satellite payload architecture operating with multi carrier per HPA, the OBO requirements are comparable whatever waveform type.

Observation 4: For lower spectral efficiency – class A (e.g. scenarios operating in FR1 bands with spectral efficiency lower than typically 2.0 b/s/Hz), the required Output Back-Off with CP-OFDM (operated with QPSK and to some extent with 16 QAM modulation) lies in the range of 3-4 dB for BLER <1E-02 (see R1-1908996 [3]). Digital Pre-Distortion and/or HARQ enable to reduce the required OBO. The OBO for BLER <1E-03 with CP-OFDM (QPSK modulation and possible 16 QAM) without DPD is to be assessed.

Observation 5: For higher spectral efficiency – class B (e.g. with spectral efficiency higher than typically 2.0 b/s/Hz), one should distinguish between

- Satellite payload operating single carrier per high power amplifier (e.g. TWTA or SSPA): the CP-OFDM may create the need to increase the OBO as compared to legacy TDM type waveform. This issue needs to be further studied.

- Satellite payload operating multi carrier per high power amplifier (e.g. TWTA or SSPA): the OBO requirements are comparable between TDM and OFDM waveform type and hence, such scenarios can be considered for standardization as part of Rel-17.

Proposal 1: The spectrum efficiency scenarios-class A (e.g., with spectral efficiency less than typically 2.0 b/s/Hz) should be considered for standardization as part of the planned Rel-17 “NR supports NTN” New Work Item.

Proposal 2: The spectrum efficiency scenarios-class B (e.g. with spectral efficiency higher than typically 2.0 b/s/Hz)

- Proposal 2a: based on satellite payload operating multi carrier per high power amplifier (e.g. TWTA or SSPA) should be considered for standardization as part of the planned Rel-17 “NR supports NTN” New Work Item.

- Proposal 2b: based on satellite payload operating single carrier per high power amplifier (e.g. TWTA or SSPA) needs to be further studied as part of a study item on “enhanced NR supports NTN” during Rel-17, possibly leveraging the potential activity on “waveform beyond 52.6 GHz”.

# References

[1]		3GPP TR 38.811 - V15.1.0 (2019-06) – “Study on New Radio (NR) to support non-terrestrial networks (Release 15)”

[2]		R1-1907369, „On DL Performance in NTN“, Fraunhofer IIS, HHI

[3]		R1-198996, „DL performance evaluation in NTN“, Fraunhofer IIS, HHI

[4]		R1-1905216, "Key satellite parameters and simulation assumptions for NTN", ESA, RAN #96, Xi’an, China, April, 2019