3GPP TSG RAN Meeting #85	RP-192157

Newport Beach, USA, September 16-20, 2019	(revision of RP-191601)

# Source:	Intel Corporation

# Title:	Revised WID: NR RRM enhancement in R16

# Document for:	Approval

Agenda Item:	9.4.26

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

# Title: NR RRM Enhancement

## Acronym: NR_RRM_Enh

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

{For a Feature: list here the children Building Blocks (optional) and Work Tasks (optional)}

{For a Building Block: list here the parent Feature (mandatory) and children Work Tasks (optional)}

{For a Work Task: list here the parent Building Block (mandatory)}

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {mandatory text: "parent WID" or "child WID"} |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | New WID: RF requirements for FR2 | Some study phase items pend on the outcome of Phase 1 of non-simultaneous UL carrier operation in FR2 |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

The RRM technical scope for R16 have been discussed from RAN4 #90 meeting and way forward (R4-1902032) on NR RRM scope was agreed.

- Feature/requirements which were confirmed to be specified in R16

  - The requirements for LTE SRS carrier switching and NR SRS carrier switching will be specified in Rel-16.

  - The requirements for multiple SCell activation/de-activation will be specified in Rel-16

  - The RRM requirements for CGI reading with autonomous gap will be specified in Rel-16.

- Feature/requirements which were confirmed to be FFS in R16

  - Whether to specify RRM requirement related with inter-band FR2 scenario or independent beamforming on different FR2 CCs will be discussed in Rel-16

  - Whether to specify the requirements for NR SRS antenna port switching will be discussed in Rel-16.

In RANP #83, the scope of Rel-16 RRM and demodulation has been extensively discussed ( RP-190659), and in RAN4 #91 meeting, more leftover issues have been identified,

  - RRM requirements of BWP switching on multiple CCs

  - Inter-frequency measurement requirement without MG when the SSB is completely contained in the active BWP of the UE

  - Additional mandatory MG patterns for both FR1 and FR2

  - UE specific channel BW change in FR1 and FR2

In this RAN4 NR RRM enhancement WI, the identified issues for core requirement would be addressed, and corresponding performance requirements and test cases would be covered.

## 4	Objective

### 4.1	Objective of Core part WI

The objective of this work item is to specify NR RRM enhancement requirements for R16, and therefore the core part of this WI consists of study phase (target to study the necessity and feasibility of the corresponding RRM enhancement) and work phase (target to specify the necessary RRM enhancement):

Work phase:

To specify the following requirements:

- RRM requirement of SRS carrier switching requirements for NR SA, NR-DC, EN-DC and NE-DC including

  - Delay and interruption requirements of the following cases

    - LTE SRS carrier switching impacting NR CC

    - NR SRS carrier switching  impacting LTE CC

    - NR SRS carrier switching impacting NR CC

- RRM requirements of multiple Scell activation/deactivation in both FR1 and FR2

  - Scell activation and deactivation delay and interruption requirements for multiple SCell activation/deactivation in FR1, and FR2, and FR1+FR2

    - the corresponding delay requirements for both FR1 and FR2 should be power class agnostic

- CGI reading requirements with autonomous gap for NR capable UE

  - CGI reading for NR cell (NR capable UE in LTE SA, NR SA UE, EN-DC UE, NE-DC UE, and NR-DC)

  - CGI reading for LTE cell (NR SA UE, EN-DC UE, NE-DC UE, and NR-DC UE)

- RRM requirements of BWP switching

  - The Interruption requirement and BWP switching delay requirement when UE is configured or indicated to change BWP on multiple CCs

- Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam and/or common beam

  - Cell detection/measurement requirement,

  - Beam management requirements,

  - Scell activation requirement,

  - Interruption requirement,

  - Scheduling restriction requirement

Note: RAN4 RRM work should be held until RF session concludes the feasibility study on beamforming with independent Rx beam and/or common beam for inter-band CA in Rel-16 FR2 RF Work Item (RP-190761)

- Inter-frequency measurement requirement without MG

  - RRM requirements for inter-frequency measurement without MG when the SSB is completely contained in the active BWP of the UE

- Mandating more measurement gap patterns for R16 RRM

  - Discuss on the possible benefits and network/UE complexity aspects of mandating additional measurement gap pattern(s) for R16 RRM based on R15 measurement gap applicability requirements.

  - If the conclusion is to mandate additional gap pattern(s), inform RAN2 by LS.

Study phase:

To study the feasibility or necessity to introduce the requirements for the following aspects:

- RRM requirements for UE specific channel BW change in FR1 and FR2

  - Study to introduce RRM requirements for UE specific channel BW change

Note: If requirements are introduced, they should be power class agnostic for both FR1 and FR2

- Non-simultaneous UL carrier operation in FR2

  - RRM requirements for non-simultaneous transmission on aggregated inter-band FR2 UL carriers

    - Phase 1: Based on the outcome of Phase 1 of the corresponding objective in the Rel-16 FR2 RF Work Item (RP-190761), study switching delays associated with UL BWP configuration and activation across non-simultaneous UL carriers and study potential impacts on RAN1 and/or RAN2 specifications

      - NOTE: This work to start after Phase 1 of the RF Work Item concludes that it is feasible

    - Phase 2: Define the corresponding delay and interruption requirements if identified

  - RRM requirements for non-simultaneous transmission on aggregated intra-band non-contiguous FR2 UL carriers

    - Phase 1: Based on the outcome of Phase 1 of the corresponding objective in the Rel-16 FR2 RF Work Item (RP-190761), study switching delays associated with UL BWP configuration and activation across non-simultaneous UL carriers and study potential impacts on RAN1 and/or RAN2 specifications

      - NOTE: This work to start after Phase 1 of the RF Work Item concludes that it is feasible

    - Phase 2: Define the corresponding delay and interruption requirements if identified

### 4.2	Objective of Performance part WI

Specify the RRM performance requirements and test cases related to new core requirements

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {E.g. "22.XXX"} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g.: rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS38.133 | Core requirement for RRM enhancement | RANP #87 | Core part |
| TS38.133 | Performance requirement and test cases for RRM enhancement | RANP #88 | Performance part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Jie Cui

Company: Intel Corporation

Email: jie.cui@intel.com

Qian Yang

Company: ZTE Corporation

Email: yang.qian35@zte.com.cn

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if necessary, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIDs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

Intel

ZTE

Ericsson

Huawei

HiSilicon

CATT

China Telecom

CMCC

OPPO

CHTTL

LGE

NTT DCM

vivo

Samsung

Nokia

Mediatek