3GPP TSG RAN meeting #85										RP-192270

Newport Beach, USA, September 16-20, 2019

## Status Report to TSG

Agenda item:			9.4.1

| WI / SI Name | Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_eMIMO |  |  |  |  |
| Unique ID | 800085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-182863 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2020 | Performance part: 09/2020 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 75% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Adding TU for RAN4 core requirements for both RF and RD based on revised WID

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

The following agreements and conclusions were made in RAN1 98:

- CSI enhancement for MU-MIMO support

Agreement

On the remaining details on UCI parameters

- Fix α=2

- The range of values for is unrestricted, i.e.

Conclusion:

In RAN1#98, there is no consensus on the following issues:

- Refining the agreement on bitmap size of 2LMi for all RI values to 2LMi-1 only for RI>1

- Refining the agreement on KNZ,TOT definition/encoding to allow joint encoding with M’

- Specifying restriction on at least 1 NZC per polarization per layer

Agreement

On the supported parameter combinations

- The following parameter combinations are supported:

| L | p = y0 (RI= 1-2) | p = v0 (RI= 3-4) | β | Restriction (if any) |
| --- | --- | --- | --- | --- |
| 2 | ¼ | 1/8 | ¼ |  |
| 2 | ¼ | 1/8 | ½ |  |
| 4 | ¼ | 1/8 | ¼ |  |
| 4 | ¼ | 1/8 | ½ |  |
| 4 | ½ | ¼ | ½ |  |
| 6 | ¼ | - | ½ | RI=1-2, 32 ports |
| 4 | ¼ | ¼ | ¾ |  |
| 6 | ¼ | - | ¾ | RI=1-2, 32 ports |

- Support only 16PSK co-phasing, i.e. 8PSK co-phasing is not supported

Agreement

On Rel.16 extension for Type II port selection codebook:

- For rank 1-2, reuse Rel.15 Type II W1 port selection matrix for Rel.16 Type II port selection codebook

  - Only L=2 and 4 are supported

- FFS: support for rank 3-4

Agreement

On CBSR for Rel.16 Type II codebook:

- Support SD-only subset restriction (without FD)

- In RAN1#98bis, select one of the following criteria for SD subset restriction:

  - Alt1. Analogous to Rel.15 Type I

  - Alt2. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient amplitude restriction)

  - Alt3. Rel. 15 Type II SD beam group restriction + sum power per SD beam restriction

- Support RI restriction

Agreement

On the value of N3 for (NSB×R) > 13: Support Alt0 (N3=NSB×R)

Agreement:

The selected UCI omission scheme should meet the following criteria when CSI omission occurs:

- CSI calculation is identical to that for without omission – otherwise the UE may end up recalculating the CSI if UCI omission occurs.

  - When UCI omission occurs, the associated CQI may not be calculated conditioned on the PMI after omission

- The occurrence of UCI omission can be inferred from the associated CSI report without any extra signaling.

- The resulting UCI payload after omission should not be ambiguous (payload ambiguity would require the gNB to perform blind decoding of UCI Part 2).

- When CSI omission occurs, dropping all NZCs associated with any particular layer should not be done.

Note: CSI omission occurs when the allocated UL resource for UCI is not sufficient for full CSI reporting.

Agreement

Denote the non-zero LC coefficient (NZC) associated with layer , beam , and FD-basis  as . The associated bitmap component (including zero(s)) is.

For the purpose of UCI omission, the parameters in UCI Part 2 is divided into 3 groups where Group n is of a higher priority than Group (n+1), n=0, 1.

Agreement

On CBSR for Rel.16 Type II codebook, the three agreed alternatives for down selection are further clarified as follows. No other alternatives or sub-alternatives will be considered for down selection.

- Alt1. Analogous to Rel.15 Type I

  - Hard restriction (0 or 1) can be applied to any of the spatial beams (the restriction is applied for both polarizations of the beam) and is higher-layer configured with one size-N1N2O1O2 bitmap B

- Alt2. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient amplitude restriction)

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each spatial beam in each of the four beam groups, soft restriction (maximum amplitude of 0, ½, , or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps

- Alt3. Rel. 15 Type II SD beam group restriction + joint per SD beam restriction

  - Four beam groups are selected via higher-layer configured bitmap B1

  - Amplitude restriction:

    - Alt 3A (Sum power ratio): For each beam  in each of the four beam groups, power ratio threshold  (definition and values FFS) is configured, the following criterion should be satisfied:  λ=0RI-1m=0M-1k=01pres,l02λ,m,k≤γl0

    - Alt 3B (Restriction on W2=W2WfH): For each beam l0 in each of the four beam groups and FD index k0k0, 0≤k00≤k0<N3<N3, wideband gain threshold Γl0 (maximum threshold of 0, ,, or 1) is configured, the following criterion should be satisfied:  w2(l0,k0)maxl,kw2(l,k)≤Γl0

      - i.e. the “wideband gain” in the frequency domain of the precoder is restricted similarly to Rel. 15

  - This maximum amplitude restriction is higher-layer configured with four bitmaps

Agreement

On the support of Rel.16 Type II port selection codebook for RI=3-4, evaluate the need for supporting the following scheme in RAN1#98bis:

- Reuse the Rel-15 Type II W1 matrix and the Rel-16 Type II  and Wf

- Note: if there is no consensus on the need for this feature, such extension to RI=3-4 is not supported in Rel.16

Agreement

When the UE is configured to report NRep CSI reports,

- Group 0 includes at least: SD rotation factors, SD indicator, and SCI(s) for all the NRep reports,

- For each of the NRep reports, Group 1 includes at least: reference amplitude(s) for weaker polarization, , FD indicator

- For each of the NRep reports, Group 2 includes at least:

- Note: G1 and G2 exclude the indices associated with the strongest coefficient(s)

In RAN1#98bis, decide the following aspects. If there is no consensus in RAN1#98bis, UCI omission for Rel.16 Type II codebook is not supported in Rel.16 (i.e. UCI omission can be performed via UE implementation).

- Priority rule for determining G1 and G2: down select from the following:

- Alt 1.1: LC coefficients are prioritized from high to low priority according to (λ,l,m) (index triplet, the   highest priority coefficients belong to G1 and the  lowest priority coefficients belong to G2. Priority level is calculated as Prio(λ,l,m)=2L.RI. Perm1(m)+RI. Perm2(l)+λ

  - FFS: Exact structure of index permutation function Perm1(.) and Perm2(.), including no permutation

- Alt 1.2: The NZ coefficients  are sorted sequentially 0 to KNZ– 1 in the following order, based on λlm indexing (layer  SD  FD), or based on l λ m indexing (SD  layer  FD). The group G1 comprises at least firstsorted coefficients, and group G2 comprises the remaining second sorted coefficients.

- Alt 1.3: LC coefficients are prioritized from high to low priority according to (λ,l,m) index triplet, the  highest priority coefficients belong to G1 and the   lowest priority coefficients belong to G2. Priority level is calculated as Prio(λ,l,m)=2L.RI. Perm1(m)+RI. Perm2(l)+ λ

  - FFS: Exact structure of index permutation function Perm1(.) and Perm2(.), including no permutation

- Which group(s)  belong to: down select from the following

- Alt 2.1: (only coupled with Alt 1.1) First bits according to Prio(λ,l,m) value belong in Group 1, last according to Prio(λ,l,m) value belong in Group 2

- Alt 2.2: (only coupled with Alt 1.2) Bitmap and coefficients are segmented together into M segments (M = number of FD basis indices). Group 1 contains M1 segments and Group 2 contains M2 segments, where M = M1+M2. Each segment contains the bitmap (sub-bitmap) associated with all RI layers, all SD components and a single FD component and the corresponding combining coefficients. The payload size of Group 1 is given by  (N= number of bits for amplitude and phase). The payload size of Group 2 is .

  - FFS: Segmentation of sub-bitmap and coefficients per segment

- Alt 2.3: (only coupled with Alt 1.3) First bits according to Prio(λ,l,m) value belong in Group 1, last  according to Prio(λ,l,m) value belong in Group 2

- Alt 2.4 (only coupled with Alt 1.1) First RI.LM bits according to Prio(λ,l,m) value belong in Group 1, last RI.LM  according to Prio(λ,l,m) value belong in Group 2

- Alt2.5: (applicable to any Alt1.x) Bitmap  is included in Group 0

- Alt2.6: (applicable to any Alt1.x) Bitmap  is included in Group 1

- Enhancements on multi-TRP/panel transmission

Agreement

In case higher layer index per CORESET is configured, for joint semi-static HARQ-ACK codebook among M-TRP,

- HARQ-ACK information bits are concatenated by the increasing order of

  - PDSCH reception occasion index at first

  - and then serving cell index

  - and TRP (i.e. higher layer index configured per CORESET (if configured))

- FFS: Whether and how to specify UE behaviour in case the higher layer index per CORESET is not configured.

Agreement

In case higher layer index per CORESET is configured,

- For multi-PDCCH based multi-TRP operation, when multiple dataScramblingIdentityPDSCH parameters are configured, each dataScramblingIdentityPDSCH is associated with a higher layer signalling index per CORESET (if configured) and is applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index.

- FFS: Whether and how to specify UE behaviour in case the higher layer index per CORESET is not configured.

Conclusion

For multi-DCI based multi-TRP/Panel transmission, at least for eMBB, there is no consensus to increase the maximum number of HARQ process in Release 16

Agreement

For single-DCI based NJCT transmission, at least for eMBB, with regarding to following design principles for DMRS entries:

- Principle 1: No consensus to support 1+3 and/or 3+1 layer combinations from two TRPs indicated by antenna port field.

- Principle 2: No consensus to have additional specification support for MU cases

- Principle 3: No consensus to have additional specification support for two CWs

Agreement

For single-DCI based M-TRP URLLC scheme 2a and 2b:

- The number of TCI states is 2

- Support up to 2 transmission layers for scheme 2a

Agreement

For single-DCI based M-TRP URLLC scheme 3 & 4

- The maximum number of TCI states is 2

- Resource allocation in time domain:

  - Support same number of consecutive symbols scheduled for transmission occasion

- For scheme 3

  - All transmission occasions are in a single slot by NW implementation without dropping.

  - FFS for DL/UL switching within the slot

Agreement

For single-DCI based M-TRP URLLC scheme 2a and 2b support following design:

- Comb-like frequency resource allocation between/among TRPs. For wideband PRG, first ⌈N_RB/2⌉ RBs are assigned to TCI state 1 and the remaining ⌊N_RB/2⌋ RBs are assigned to TCI state 2. For PRG size=2 or 4, even PRGs within the allocated FDRA are assigned to TCI state 1 and odd PRGs within the allocated FDRA are assigned to TCI state 2.

Agreement

For schemes 3 and 4, the maximum number of transmission layers per TRP is up to 2

- The supported maximum TBS size is dependent on UE capability

Agreement

PDSCH repetition indication mechanism:

- For indication on the number of repetition occasions for scheme 3, select one of the following dynamic indication methods in RAN1#98bis

  - Option 1: It is dynamically indicated e.g. by reusing the proposed indication mechanism for PUSCH repetition in eURLLC

  - Option 2: It is implicitly determined by the number of TCI states indicated by a code point whereas one TCI state means one repetition and two states means two repetitions

- For indication on the number of repetition occasions for scheme 4, select one of the following in RAN1#98bis

  - Option 1: It is dynamically indicated

  - Option 2: By high-layer signaling following Rel-15 mechanism

Email discussion on the details of PDSCH repetition indication mechanism by 6th of Sep Min (Huawei)

Agreement

For joint dynamic HARQ-ACK codebook among M-TRP, select one from following alternatives in RAN1#98bis

- Alt 1: counter DAI is jointly counted across two TRPs (i.e. different higher layer index configured per CORESET (if configured)), and total DAI should count total number of DCIs in a PDCCH monitoring occasion across CCs and TRPs.

- Alt 2: counter DAI is counted per TRP, and and total DAI should count total number of DCIs in a PDCCH monitoring occasion across CCs for each TRP. HARQ-ACK information bits are then concatenated by the increasing order of TRPs (i.e. different higher layer index configured per CORESET (if configured)).

Agreement

In order to switch between joint and separated ACK/NACK feedback within a slot,

- RRC signaling is used to switch between joint feedback and separate feedback

  - Note that UE can use separate HARQ-ACK codebooks when the indicated PUCCH resources for two TRPs are different slots/[sub-slot].

Agreement

With regarding to PUCCH resource group for M-DCI NCJT transmission, select one of following options in RAN1#98bis

- Option 1: Support configuring explicit PUCCH resource grouping over resource or resource sets

- Option 2: Support implicit PUCCH resource grouping up to NW implementation whereas PUCCH may or may not be overlapped.

Agreement

For multi-PDCCH based multi-TRP operation, the maximum number of CORESETs that can be configured with the same TRP (i.e. same higher layer index configured per CORESET (if configured) per “PDCCH-Config”) is up to UE capability, including at least a candidate value of 3.

Email discussion on the following description (RED) of BD/CCE restriction for multi-DCI based multi-TRP transmission by 20th of September, Min (Huawei)

For the UE supporting multi-DCI based multi-TRP transmission,

- For CORESETs configured for the same TRP (i.e. same higher layer index configured per CORESET (if configured) per “PDCCH-Config”), the maximum number of monitored PDCCH candidate and non-overlapped CCEs per slot for a DL BWP is no greater than the Rel.15 limit.

- Total limits for BD/CCE numbers across configured CCs are calculated the same as that in Rel. 15 based on   Ncellscap= pdcch-BlindDetectionCA as reported by the UE

- (Bound derived from pdcch-BlindDetectionCA) The maximum numbers of monitored PDCCH candidate and non-overlapped CCEs per slot for total limits,   defined in 38.213,  for a serving cell configured with multi-DCI based multi-TRP transmission are increased as r times the numbers for a serving cell configured with single-DCI based transmission

- (Bound independent of pdcch-BlindDetectionCA) The maximum BD/CCE numbers,  and , are increased as r times the Rel-15 values defined in Table 10.1-2 and Table 10.1-3 in 38.213 for a serving cell configured with multi-DCI based multi-TRP

Agreement

At least for multi-DCI based multi-TRP/panel transmission, the UE shall rate match around: (down-select one option from following in RAN1#98bis):

- Alt1: configured CRS patterns for all PDSCHs transmitted from multiple TRPs

- Alt2: configured CRS patterns which are associated with a higher layer signalling index per CORESET (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same higher layer index.

Agreement

For multi-DCI based multi-TRP transmission with separate ACK/NACK feedback

- UE is allowed to transmit two TDMed long PUCCHs within a slot

- UE is allowed to transmit TDMed short PUCCH and long PUCCH within a slot

- UE is allowed to transmit TDMed short PUCCH and short PUCCH within a slot

FFS whether/how to use PRI indication with the granularity of sub-slot for eMBB with M-TRP

Agreement

When 2 TCI states are indicated by a TCI code point, at least for DMRS type 1 and type 2 for eMBB, if indicated DMRS ports are from two CDM groups,

- the first TCI state is applied to the first indicated CDM group

- the second TCI state is applied to the second indicated CDM group

FFS: the definition of the first or second indicated CDM group

FFS: Whether above applies for only Rel-15 DMRS or for both Rel-15 and Rel-16 DMRS

Agreement

For single-DCI based M-TRP URLLC schemes 2a/2b/3/4, indicated DMRS ports are from one CDM group.

- Enhancements on multi-beam operation

Conclusion

- From RAN1 point of view, a “UE panel” would be a logical entity and how to map physical UE antennas to the logical entity is up to UE implementation.

- (Informative) For certain condition(s), gNB can assume the mapping between UE’s physical antennas to the logical entity “UE panel” activated for transmission will not be changed

  - FFS: Whether “UE panel” is transparent to gNB

  - FFS: UE capability includes at least the number of “UE panels”.

  - FFS: Whether/how to define the certain condition(s)

    - For example: The duration of time over which the gNB assumes there will be no change

    - For example: Until next update or report from UE

- (Informative) Depending on UE’s own implementation, a “UE panel” can have at least the following functionality as an operational role of

  - Unit of antenna group to control its Tx beam independently

Agreement

The following working assumption is confirmed with modification (in red):

The supported feature of MAC CE based spatial relation update for aperiodic SRS is applicable to the usage of antenna switching per SRS resource level.

- In case of antenna switching, UE does not expect to be configured with different spatial relation within the same set

Agreement

At least for UEs supporting beam correspondance, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement', is not configured in FR2, a default spatial relation for dedicated-PUCCH/SRS is applied

- FFS: Detail on the default spatial relation

Agreement

When gNB configures UE to report SSBRI/CRI and corresponding L1-SINR, the following report format is supported.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1, if reported |
|  | CRI or SSBRI #2, if reported |
|  | CRI or SSBRI #3, if reported |
|  | CRI or SSBRI #4, if reported |
|  | SINR #1, if reported |
|  | Differential SINR #2, if reported |
|  | Differential SINR #3, if reported |
|  | Differential SINR #4, if reported |

- FFS: range and step size of differential SINR

- Differential SINR #N is determined based on the difference between measured SINR corresponding to the CRI/SSBRI #N and the measured SINR corresponding to CRI/SSBRI #1

- The SINR #1 is the largest SINR among reported SINRs

- The range of SINR is [-23, 40] dB

  - The SINR is quantized based on what is specified in 38.133

Agreement

For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on ZP-IMR only:

- CMR and IMR are 1-to-1 mapped from signaling perspective

Email discussion by September 20th Yushu (Apple) on the details (such as how measurement of interference is done, the mapping relation between CMR/IMR, QCL, etc) of L1-SINR based beam report using the following as a starting point:

For L1-SINR based beam report, in a CSI-reportConfig, if IMR is configured to be based on NZP-IMR only, down-select at least one of the following resource configuration schemes:

- Option 1: CMR and IMR are 1-to-1 mapped

- Option 2: 1 CMR can be mapped to 1 or more than 1 IMRs

- Option 3: 1 IMR can be mapped to 1 or more than 1 CMRs

- Option 4: More than 1 IMRs can be mapped to more than 1 CMRs.

Agreement

Support gNB to configure L1-SINR based beam report for both non-group based and group based beam reporting.

Agreement

Support that the BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold.

Agreement

- For SCell BFR, support maximum number of RS for new beam identification per BWP to be 64.

- The range of threshold for SCell new beam identification is based on range specified in RSRP-Range.

Agreement

For SCell with both UL and DL, at least reuse the same BFRQ procedure as SCell with DL only.

- Note: Whether to support CBRA/CFRA based BFRQ for both scenarios is a separate issue.

- Note: At least from RAN1 perspective, there is no need for introducing restrictions on MAC CE transmission for BFR in Rel-16

- FFS: Whether PUCCH-BFR can be configured on SCells

Agreement

Support PUCCH-BFR to be configured by either one of PUCCH format 0 and PUCCH format 1

- FFS: details when PUCCH-BFR transmission is to be made in the same slot with other uplink signal(s).

Conclusion

- The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2

- RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs

Agreement

Include the above conclusion as part of LS to RAN2.

Agreement

For latency/overhead reduction across multiple CCs/BWPs, support single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs

- Example 1: Reuse Rel-15 MAC-CE to activate same set of TCI state IDs for all active BWPs in same band or cell group(s) on FR2

  - Support of this mode can be indicated by UE capability

  - To operate in this mode, UE may expect the same QCL-TypeD RS is configured for same TCI state ID for all BWPs in each band or cell group(s)

  - For activation MAC-CE received on any active BWP in a band or cell group(s), indicated activated TCI state IDs will be applied to every active BWP in that band or cell group(s)

- Example 2: Reuse Rel-15 MAC-CE to activate one set of TCI state IDs (including both QCL Type-A and Type-D RSs) for an active BWP of the CC indicated by the MAC-CE to be applied to all active BWPs in same band or cell group(s) on FR2

  - Note: The QCL Type A RS(s) applied to each CC/BWP is that corresponding to the same resource ID(s) indicated by the TCI state IDs

- FFS: operation/signaling details including the possibility to activate different sets of PDSCH TCI state IDs for multiple CCs/BWPs

- Note: QCL type-A comes from the BWP where the TCI state is applied

From RAN1 perspective, the response after UE transmits PUCCH-BFR can be a normal uplink grant with C-RNTI/MCS-C-RNTI

- No RAN1 spec impact

Agreement

Send an LS to RAN2 suggesting higher priority for SCell BFR MAC CE than at least UL data, and to have a higher priority for SCell step 1 PUCCH than normal SR

- To be included as part of LS to RAN2

The draft LS for RAN2 is modified as follows and endorsed in R1-1909833.

Q1: Can the UE transmit BFR MAC CE using UL grant of any serving cell or should there be a restriction not to send it on failed serving cell(s)?

R1: At least from RAN1 perspective, there is no need for introducing such restrictions on MAC CE transmission for BFR in Rel-16.

Q2: If the UE already has the UL grant on serving cell(s) on which BFR MAC CE can be transmitted based on the answer to question 1, is the UE still required to transmit SR-like indication for BFR?

R2: In this case, UE is not required to transmit SR-like indication for SCell BFR.

Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell).

R3: RAN1 did not discuss this case and will leave it to RAN2. RAN1 plans to conclude on this by RAN1#98bis.

Q4: Is the SR-like dedicated PUCCH resource for SCell BFR configured for each SCell separately or is it common for all SCell(s) of the same cell group (i.e. MCG/SCG)?

R4: The SR-like dedicated PUCCH resource for SCell BFR is not configured separately for each SCell.

Q5: What conditions are used for the (successful) completion of the SCell BFR?

R5: When UE receives beam failure recovery response (BFRR) to step 2, UE can consider BFR procedure is finished, where the BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE, which is the same as normal “ACK” for PUSCH.

Additional information

RAN1 would like to provide the following additional information on SCell BFR to RAN2.

- RAN1 suggests RAN2 to give higher priority for SCell BFR MAC CE than at least UL data, and also higher priority for SCell BFR PUCCH than normal SR

- The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2

- RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs

Agreement

RAN1 will conclude on the following issue in RAN1#98bis

Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell).

Agreement

The BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE

- The procedure is the same as normal “ACK” for PUSCH

- When UE receives BFRR to step 2, UE can consider BFR procedure is finished

- No RAN1 spec impact

- Included as part of LS to RAN2

Agreement

Down-select one of the following alternatives on UE behavior when no new beam RS is configured in RAN1#98bis

- Alt 1: UE shall expect gNB to configure at least one new beam RS if BFR for corresponding SCell is configured

- Alt 3: If new beam RS is not configured, all SSBs are considered as new beam RS candidates

Agreement

Continue discussion on the support of updating pathloss RSs for power control for PUSCH and SRS via MAC-CE, including the following candidates until RAN1#98bis:

- Option 1: For codebook based PUSCH transmission, the pathloss RS follows DL RS in spatial relation associated with SRI indicated in scheduling DCI, if the pathloss RS is not configured and periodic DL RS is configured in the spatial relation.

  - FFS: the cases of non-codebook based PUSCH, SRS

- Option 2: Pathloss RS is associated/configured for downlink RS in spatial relation info.

  - gNB can configure more than 4 pathloss RSs.

- Option 3: At least the pathloss RSs for SRS or PUSCH can be explicitly activated/updated by the MAC-CE

  - FFS: The other power control parameters including P0, alpha, and a closed loop process index are also activated by the MAC-CE

  - FFS on whether to support the number of configured pathloss RSs are more than four.

  - Note: The MAC-CE is the activation MAC CE for ap-SRS/sp-SRS.

- Option 4: Support updating TCI state for periodic CSI-RS by MAC CE.

  - Note: The periodic CSI-RS is used for pathloss reference.

- Option 5: Support semi-persistent CSI-RS for pathloss reference RS.

- Note: Baseline is that the same transmission power is applied within SRS resource set (same as Rel-15).

Agreement

At least for UEs supporting beam correspondence, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement', is not configured in FR2, the applied default spatial relation for the dedicated-PUCCH/SRS is down-selected from the followings in RAN1#98bis

- Alt.1: default TCI state or QCL assumption of PDSCH (e.g. the most recent slot and the lowest CORESET ID)

- Alt.2: one of an active TCI state of CORESET

  - FFS: details of which TCI state

- Alt.3: TCI state of scheduling PDCCH for A-SRS/PUCCH, and default TCI state or QCL assumption of PDSCH for other than A-SRS/PUCCH

- Alt.4: CORESET#0 QCL assumption

- Alt.5: pathloss reference RS

  - FFS: details of which pathloss reference RS

- FFS: whether to apply the above for UEs not supporting beam correspondence

Conclusion

On the issue of beam management enhancements to address MPE, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.

Conclusion

On the issue of uplink transmit beam selection that facilitates UE panel specific uplink beam selection, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.

- Full TX power UL transmission

Agreement

For mode 1, 2Tx non-coherent UE, the new codebook subset at least includes rank=1 TPMI=2 defined in Rel-15 which can be used for UL full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset at least includes, rank 1 TPMI= 13 defined in Rel-15 which can be used for UL full power transmission

- FFS for the case that part of ports can deliver full power transmission

Agreement

For mode 1, 4Tx non-coherent UE, the new codebook subset

- at least includes, rank 2 TPMI=6 defined in Rel-15

- at least includes, rank 3 TPMI=1 defined in Rel-15

Agreement

For mode 2, in case of non-coherent with 2 ports, support following TPMI indication for rank 1 which support UL full power transmission:

- Rank 1: support {TPMI=0} and {TPMI=1}

- FFS: Details on UE capability signalling

Conclusion

For mode 2, no additional rule for spatial filter update for SRS resources with different number ports

Agreement

For a capability 1 UE working with full power operations, for PUSCH power control, power scaling factor is fixed to 1

Agreement

For a UE working with Mode1 operation, for PUSCH power control, down-select or merge from the following alternatives in RAN1#98bis

- Alt1: reuse Rel-15 power scaling mechanism.

- Alt2: power scaling factor is configured.

- Alt3: power scaling factor is determinded by #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI.

- Alt4: A UE can scale its transmit power by  to reach full power, where

  - Kρ,m,ν>1 is a scale factor associated with  ρ∈{1,2,4}SRS ports corresponding to the PUSCH transmission and an optional mth TPMI with rank v.

    - If a TPMI is not associated with Kρ,m,ν, then  is determined without regard to m and v.

    - If Kρ,m,ν is not configured by higher layers, a set of fixed values are defined for K(ρ).

  - ρ0is the number of non-zero PUSCH ports being transmitted

- Alt5: For the precoders in the new codebook subset for full power transmission, the power scaling factor is 1.

Agreement

For a UE working with Mode2 operation, for PUSCH power control, down-select or merge from the following alternatives in RAN1#98bis

- Alt1: power scaling factor is determinded by the reported TPMI precoders.

- Alt2: power scaling factor is configured.

- Alt3: power scaling factor is determinded by #non-zero-PUSCH-port divided by #SRS-ports in the SRS resource indicated by SRI.

- Alt4: A UE can scale its transmit power by min⁡(1,K(ρ,m,ν)ρ0/ρ) to reach full power, where

  - Kρ,m,ν>1 is a scale factor associated with ρ∈{1,2,4} SRS ports corresponding to the PUSCH transmission and an optional mth TPMI with rank v.

    - If a TPMI is not associated with Kρ,m,ν, then Kρ,m,ν is determined without regard to m and v.

    - If Kρ,m,ν is not configured by higher layers, a set of fixed values are defined for K(ρ).

  - is the number of non-zero PUSCH ports being transmitted

Agreement

- For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’

Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’

- Low PAPR RS

(None, completed)

#### 2.1.2	Remaining Open issues

- CSI enhancement for MU-MIMO support

  - Remaining issues on UCI omission, CBSR, and support for rank 3-4 Type II port selection extension

  - UE capability related issues

- Enhancements on multi-TRP/panel transmission

  - Remaining issues on multi-PDCCH for supporting NCJT:

    - for DL, detailed TRP-specific design for CCE/BD, down-selection of rate matching solutions and possible PDSCHs scheduling restrictions

    - for UL, remaining issues on joint codebook for multi-TRP and PUCCH transmission

  - Remaining issues on single-PDCCH for supporting NCJT for detailed DMRS tables/entries

  - Remaining issues on URLLC-related enhancement for signaling indication mechanism/restrictions for schemes SDM/FDM/TDM and possible M-DCI based URLLC

- Enhancements on multi-beam operation

  - Remaining issues on L1-SINR: detailed association between CMR(s) and IMR(s) in resource configuration for NZP-IMR based SINR measurement, and remaining issues for the working assumption on SINR measurement from both NZP and ZP based IMR, range and step size of differential L1-SINR

  - Remaining issues on SCell BFR: beam failure detection procedure, UE behavior when no new beam RS is configured, and remaining issues of PUCCH-BFR configuration including UE behavior when PUCCH-BFR is collided with other uplink signals, , whether PUCCH-BFR is not configured and UE behavior when PUCCH-BFR is not configured

  - Remaining issues on DL/UL beam indication with reduced overhead/latency: details on updating path-loss reference RS for PUSCH/SRS via MAC CE, simultaneous spatial relation update for multiple PUCCH resources, default spatial relation for PUCCH/SRS, and activation of TCI states for multiple CCs/BWPs

- Full TX power UL transmission

  - Power scaling schemes for Mode 1 and Mode 2

  - Potential additional TPMIs for Mode 1

  - UE capability signaling details

- Low PAPR RS

  - None (completed)

## 2.2	RAN2

#### 2.2.1	Agreements

MultiPDCCH MTRP:

[107#xx][NR] Multi PDCCH multi TRP impact to RAN2 (Ericsson)

Should aim to identify all RAN2 impacts both UP and CP.

Start to progress the solutions in RAN2 with priority on user plane

Include identification of information that we need to know from RAN1

Intended outcome: Report to next meeting

Deadline:  Thursday 2019-10-03

SinglePDCCH MTRP:

=>	Offline discussion to identify the questions we may need to ask RAN1 in order to progress the MAC CE design. (Offline discussion 67, Samsung)

BFR on SCell:

=>	Draft LS to be provided in R2-1911746

=>	Response to RAN1 that the WA to introduce a MAC is feasible from the RAN2 point of view

=>	Offline discussion to conclude questions that need to be asked to RAN1 e.g. ask whether the MAC CE should be sent on an UL grant of a specific cell, ask whether the SR like signal is still needed if the UE already has another grant available, ask if there is a case where the new SR like resource is not configured

=>	Draft LS in R2-1911747 (Offline discussion 68, Samsung). LS can be treated before Friday if available.

#### 2.2.2	Remaining Open issues

Pertinent design on MAC (e.g. MAC CE) and RRC

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not yet started

#### 2.4.2	Remaining Open issues

Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1] 3GPP, RAN1, RAN1#94bis Chairman Notes

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template