3GPP TSG RAN Meeting #85	RP-192286

Newport Beach, US, September 16th – 19th, 2019

Agenda Item:	9.4.8

Source:	Qualcomm Incorporated

Title:	Public Safety support over Rel-16 V2X Sidelink

Document for:	Information

# Introduction

Rel-16, 5G V2X with NR sidelink WI [1] is under progress. This WI is targeting V2X use cases and requirements as captured in [2]. However, the WID mentions, in addition, the following with respect to the usage of Rel-16 V2X NR sidelink for Public Safety:

Meanwhile, it was noted that solutions defined for V2X can also be used for public safety when the service requirement can be met.

Public Safety community in [3] and [4] has provided minimum set of requirements that ought to be fulfilled by Rel-16 V2X for the purpose of supporting Proximiy Services (ProSe) Direct communications. In this document we analyse our view with respect to the minimum set of requirements from Public Safety community.

# Discussion

In [3] and [4] following minimum set of requirements from public safety community are provided:

- Direct 1:1 communications in coverage - Direct 1:1 communications out of coverage - Direct 1:Many communications in coverage - Direct 1:Many communications out of coverage - Speech packet latency for both Direct and Group call: 200-250 msec - Call set-up time for both Direct and Group call: less than 300 msec - Average MOS audio quality for both Direct and Group calls: better than 3.0 for 96% and >= 2.5 for 100% - Maximum number of UEs in a Group call: 75

Rel-16 V2X is designed for in-coverage, partial coverage and out of coverage operation for unicast (Direct 1: 1 Communications), groupcast (Direct 1:1 communications) and broadcast operation. Unicast and groupcast have added feature of HARQ feedback to achieve higher reliability and higher efficiency from the system point of view. With such unicast/groupcast/broadcast capability to operate in coverage, partial coverage and out of coverage following minimum requirements are already supported by Rel-16 V2X:

- Direct 1:1 communications in coverage - Direct 1:1 communications out of coverage - Direct 1: Many communications in coverage - Direct 1: Many communications out of coverage

With respect to end to end latency, Rel-16 V2X is getting designed to meet the “Packet Delay Budget” (PDB) provided by upper layers as part of QoS framework. The PDB requirements are in the order of few ms to 10s of ms. With guaranteed PDB based delivery mechanism following minimum requirement from public safety communications can easily be met:

- Speech packet latency for both Direct and Group call: 200-250 msec

The low latency communications (in the order of 10ms) provided by Rel-16 V2X PHY and MAC provides a high speed pipeline for upper layers to setup calls, e.g. at application layer for group communications, or at RRC level for unicast. Thus, the following requirement is also within reach of Rel-16 V2X:

- Call set-up time for both Direct and Group call: less than 300 msec

Rel-16 V2X is designed to allow communications between large number of UEs and to achieve high reliability levels, so following requirements can be within reach of Rel-16.

- Average MOS audio quality for both Direct and Group calls: better than 3.0 for 96% and >= 2.5 for 100% - Maximum number of UEs in a Group call: 75

However Public Safety requirement specific deployment scenario, channel model, traffic specific simulations are required to confirm that the above mentioned average MOS audio quality level for different deployment scenarios are achievable. As part of Rel-17 WI this analysis can be performed based on the deployment/traffic/channel model for Public Safety UEs.

Observation 1: Ongoing design and specification of Rel-16 V2X appears to satisfy all the “Minimum set of ProSe D2D service requirements” [3].

Observation 2: Simulations can be performed as part of Rel-17 WI based on the deployment/traffic/channel model for Public Safety UEs to confirm the fulfilment of all the “Minimum set of ProSe D2D service requirements” [3] by the specified NR Sidelink for V2X feature in Rel-16.

Rel-12 D2D communications for Public Safety was designed based on the assumption that Public safety UEs can have larger battery or access to Public Safety Vehicle for charging the batteries. However, Rel-17 enhancements with respect to DRX for V2P can also help Public Safety UE to reduce power consumption and UE-to-NW relay will allow to reach to NW in difficult radio conditions.

Observation 3: Features being considered for Rel-17 Sidelink enhancements with respect to DRX for V2P and UE-to-NW relay would allow Public Safety UEs to reduce power consumption and to reach the NW in difficult radio conditions.

# Conclusion

Based on the discussion in the previous sections we make following observations:

Observation 1: Ongoing design and specification of Rel-16 V2X appears to satisfy all the “Minimum set of ProSe D2D service requirements” [3].

Observation 2: Simulations can be performed as part of Rel-17 Sidelink WI based on the deployment/traffic/channel model for Public Safety UEs to confirm the fulfilment of all the “Minimum set of ProSe D2D service requirements” [3] by the specified NR Sidelink for V2X feature in Rel-16.

Observation 3: Features being considered for Rel-17 Sidelink enhancements with respect to DRX for V2P and UE-to-NW relay would allow Public Safety UEs to reduce power consumption and to reach the NW in difficult radio conditions.

# References

[1] R1-1907668, “Revised WID on 5G V2X with NR sidelink”, LG Electronics

[2] TS 22.186, “Service requirements for enhanced V2X scenarios”

[3] R1-1907668, “Minimum set of ProSe D2D services in Release-16”, FirstNet, AT&T, The UK Home Office, SyncTechno Inc., Softil, Suomen virveverkko Oy, French MoI, UIC, The Police of Netherlands, BMWi

[4] R1-1907175, “Minimal set of ProSe D2D services in Release-16”, FirstNet, AT&T, The UK Home Office, SyncTechno Inc., Softil