3GPP TSG RAN TSG Meeting #86 	RP-192500

Sitges/Spain, 9-12th September 2019

Title:	Report on email discussion Rel-17 NWI Non Terrestrial Networks scoping

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	9.1 - New WI/SI proposals for NR

# Introduction

The email discussion on the scoping of Non-Terrestrial Networks (NTN) normative work in Release 17 contains two consecutive phases.

Phase 1: starting on Sept 20, 2019

This phase will identify candidate NTN scenarios characterized by a set of options and assess the impacts on RAN features (RAN1, RAN2, RAN3) associated to the proposed NTN scenarios

Deadline for Phase 1: Oct 18, 2019

Phase 2: starting on Oct 25, 2019

This phase will consolidate the ranking (taking into account and downselect the candidate NTN scenarios for Rel-17 NTN WI based on the assessment in Phase 1 and establish the corresponding WID objectives.

Deadline for Phase 2: Nov 22, 2019

Note this email discussion has been defined using as input documents:

- RP-192248 providing necessary background information for common understanding of NTN concepts based on the on-going Rel-16 study item FS_NR_NTN_solutions

- RP-192291 providing results of the NB_IoT_eMTC_enh email discussion on NB-IoT/LTE-M via satellite

# Phase 1 Candidate NTN scenarios and RAN impacts

## Description of candidate NTN scenarios

Candidate NTN scenario can be described by the following options:

a) HAPS based access, LEO based access and/or GEO based access scenarios

b) Max cell size (edge to edge). a cell may encompass several beams.

c) Transparent or regenerative payload options. Regenerative payload in the context of 3GPP refer to 5G system functions (e.g. gNB, gNB-DU, eNB etc) on board satellite/HAPS

d) Earth fixed or mobile beams for HAPS and LEO based access scenarios

e) UE with location determination capability (e.g. GNSS) or not especially for LEO and GEO based access scenarios

f) Targeted usage scenarios (See table B.2-1: Non-Terrestrial network target performances per usage scenarios in TR 38.821).

g) UE type (3GPP class 3 or other). Other refer to Directional antenna devices may refer to building or vehicle mounted devices with directional antenna such as parabolic or phased array antenna but also handset with high gain antenna (e.g. protuberating)

## Identification of most relevant NR based NTN scenarios and features

### Question Ph1.1: Which NR based NTN scenarios are to be considered in the Rel-17 NTN activity and with which priority ?

| Q- Ph1.1: Organization |  | Recommended Scenarios defined in section 2.1 of this TDOC |  | Comments |
| --- | --- | --- | --- | --- |
| Thales |  | Th.scen.nr.1 / 1st priority - Transparent based LEO with Earth fixed beam providing eMBB services directly to mass market devices : LEO at 600 km addressing at least 3GPP class 3 UE, transparent payload, 200 km max cell size (edge to edge), Earth fixed beams. Target usage scenarios include Pedestrian, vehicular connectivity, public safety. At least UEs without position determination capability (e.g. GNSS) shall be supported. |  | The normative phase should consider as a first priority, LEO based scenario providing direct access to 3GPP class 3 UE devices over worldwide/global or multi country coverage to address the challenge of coverage in extreme rural areas . Multiple narrow beam payload with high performances will only be able to close the link budget with 3GPP class 3 devices and provide eMBB services and hence the max beam size shall be limited. Transparent payload should be addressed in priority during Rel-17 to minimise impact on Rel-15/16 NG-RAN standard. Regenerative payload (e.g. supporting IAB) that would allow to provide coverages where gateways are not deployed (e.g. Maritime), can be considered as a further topic. Cell fixed on Earth is essential to prevent complex tracking area management and to comply with country specific regulations. The on board antenna system can steer the beams to ensure also fixed beam footprint pattern on Earth therefore Earth fixed beams is possible and would simplify the association between beams and cells. Note that such LEO access can serve not only mass market devices but also specific devices. |
| Thales |  | Th.scen.nr.2 / 2nd priority – Transparent based GEO providing eMBB services to specific devices : GEO addressing other than 3GPP class 3 UE, transparent payload, 1000 km max cell size (edge to edge), Earth fixed beams. Target usage scenarios include Stationary, airplane connectivity and possibly vehicular connectivity. UEs with position determination capability (e.g. GNSS) shall be supported. |  | The normative phase should consider in second priority GEO based scenario providing connectivity to cells on board moving platforms (e.g. Aircrafts, vessels or trains) or hot spots in unserved/underserved areas. Cell fixed on Earth associated with UE position determination is essential to prevent complex tracking area management and to comply with country specific regulations. Specific devices refers for example to smart phone equiped with protubating antenna or moving/fixed platform mounted devices equiped with directional antenna (dish or phased array). |
| Thales |  | Th.scen.nr.3 / 3rd priority – Transparent based HAPS with Earth fixed beam providing eMBB services directly to mass market devices : HAPS addressing 3GPP class 3 UE, transparent payload, 100 km max cell size (edge to edge), Earth fixed beams. Target usage scenarios include Pedestrian, vehicular connectivity, public safety. At least UEs without position shall be supported. |  | The normative phase should consider in third priority HAPS based scenario providing direct access over restricted area to address the challenge of resilience. Transparent payload (e.g. supporting IAB) can be addressed in priority during Rel-17 to minimise impact on Rel-15/16 NG-RAN standard. The on board antenna system can steer the beams to ensure also fixed beam foot print pattern on Earth. |
| Nokia |  | - 1) LEO highest priority 2) GEO second priority - 1000 km diameter - Both transparent and regenerative should be considered. - Mobile beams only - System should work for UEs without GNSS - Highest priority: IoT connectivity, eMBB for wireless backhaul, airplanes; Medium priority: pedestrian; rest low priority. - Connected to f: IoT connectivity: normal handset, eMBB dor wireless backhauls, airplanes: directional antennas. |  | - We believe challenges with HAPS can be tackled through LEO solutions; hence no explicit work is needed to support HAPS. - – - – - We see the peak in processing when switching cells in the fixed beams option as a bigger challenge for deployment than continuous handovers in the mobile beams option. |
| Pivotal Commware |  | Piv.scen.nr.1 = Th.scen.nr.3 / 1st priority—Transparent based HAPS with Earth fixed beam providing eMBB services directly to mass market devices |  | The normative phase should consider HAPS based scenario as an included priority. The scenario should provide direct access over a restricted area to address first the challenge of resilience in the face of impaired-mode RAN operations, as well as during normal-mode operations to meet the challenge of rapid fielding prior to new terrestrial infrastructure buildout and rapid/temporary fielding for temporary event support (eg: music festivals). The onboard antenna system can steer the beams to ensure also fixed beam foot print pattern on Earth. |
| Pivotal Commware |  | Piv.scen.nr.2 / 2nd priority—Transparent based HAPS with Earth moving beam(s) providing eMBB services directly to mass market devices |  | The normative phase should consider HAPS based scenario as an included priority. The scenario should provide direct access with Earth moving beams to moving terrestrial UE’s/devices (eg: cars) over a restricted area to address first the challenge of resilience in the face of impaired-mode RAN operations, as well as during normal-mode operations to meet the challenge of rapid/temporary fielding for initial temporary support of vehicles in, for example, travel corridors. The onboard antenna system can steer the beams to (better) address moving terrestrial vehicles as well as supporting higher SNR dedicated beams to geographically separated devices/vehicles on Earth. |
| Pivotal Commware |  | Piv.scen.nr3 = Th.scen.nr.1 / 5th priority: Transparent based LEO with Earth fixed beam providing eMBB services directly to mass market devices |  | Pivotal supports Thales’ justification for Th.scen.nr.1, specifically reinforcing the Earth fixed beam as priority, extension to regenerative payload as further topic as gateways will not be available in parts of the globe, and utilization of steerable beams to ensure fixed footprint. |
| Samsung |  | Scenario 1 (high priority): LEO, max cell size up to 500km, both transparent and regenerative, mobile beams, UE with location determination capability as baseline, Targeted usage scenarios: pedestrian, UE type: 3GPP class 3 Scenario 2(high priority): GEO, max cell size up to 1000km, both transparent and regenerative, fixed beams, UE with location determination capability as baseline, targeted use scenarios: stationary, UE type: other |  | HAPS can be handled by the current NR design and therefore can be deprioritized. |
| Vodafone |  | - LEO Satellites highest , next HAPs and lowest priority GEO - Cell sizes not more than 100km in diameter. Cell sizes to be optimised to cope with the DL and UL RF link budget, UE densities and traffic load - Both regenerative and transparent case to be considered however, regenerative could provide lower round trip latencies due to on-board modem architecture - Stationery cells (Earth Fixed) types of cells are preferred as the signalling load due to cell selection/re-selection etc. are minimised and the NTN Network design, hand-in and hand-out are simpler. - The system should be able to work with UEs with and without GNSS capabilities such as low end UEs etc. - Voice, SMS, and MBB applications, Coverage enhancement of Cellular Networks into rural and sparsely populated areas. Coverage over water/sea oceans. IoT type applications, remote monitoring. Emergency services (ambulance, police, fire, mountain rescue, sea rescue etc.) - UE Class 3 type of terminals should be the baseline. However for higher data rates and to improve the RF link budget, UEs with higher gain antenna or some kind of beam forming should also be considered. |  |  |
| SoftBank |  | 1st priority: a) HAPS based access b) 200 km diameter c) Both transparent and regenerative d) Earth fixed beam and mobile beams (for which the same solution as LEO can be used) e) N/A f) no specific restriction (i.e. support the same use cases as terrestrial network) g) Normal handset UEs |  |  |
| Asia Pacific Telecom (APT) |  | a) LEO-600km based access b) Max cell size (edge to edge) c) Transparent d) Moving beams e) 1) UE with location determination capability (e.g. GNSS) for eMBB; 2) UE without location information for eMTC f) Targeted usage scenarios: 1) eMBB; 2) eMTC g) UE type: 3GPP class 3 |  | a) to enhance NR coverage for eMBB service, LEO based access may provide acceptable link budget and latency requirement. b) without defining the max cell size, the cell size may increase when multiple beams are deployed per cell. c) we prefer to follow an agreement from working groups that transparent GEO (A) and LEO with moving beam (C2, D2) architectures are studied in priority. d) we prefer to follow the agreement that transparent GEO (A) and LEO with moving beam (C2, D2) architectures are studied in priority. e) considering satellite ephemeris may be carried by uSIM or sent by NW, if UE knows location, e.g., GNSS, or fixed location or fixed ship route or fixed air route, the spec impacts would be less. f) it may be beneficial for telecom operators to have low-cost alternatives for 5G deployment, meanwhile, it may be beneficial to bring mass eMBB device/users to satellite companies. g) for other UE types, e.g., VSAT or vehicle mounted devices, we believe that some solutions/products are available on the current market. However, support of handheld devices directly based on LEO access may provide more values. |
| TNO |  | a) 1st priority transparent LEO, 2nd priority transparent GEO. b) LEO 200 km cell size, GEO 1000 km cell size (edge to edge) c) Transparent, i.e. internal interfaces within constellation itself do not need to be standardized. InterSatellite Links may be used. d) fixed earth beams (for easier integration with terrestrial networks) for both LEO and GEO e) at least UEs with GNSS capabilities shall be supported f) With LEO, ordinary types of devices: smart phones, IoT devices (e.g. for logistics), cars for eMBB and NB-IoT, MTC-e type usage. With GEO relays/gateways type devices in planes, trains, ships, buses, etc. eMBB and NB-IoT, MTC-e type usage. g) UE class 3 for LEO. Directional antennas (mostly phased arrays) for GEO on planes, trains, ships, buses, etc. relaying for normal UEs inside. |  |  |
| Loon |  | 1st priority: Regenerative HAPs based communication with earth fixed beams (or slowly moving earth fixed beams) providing eMBB service directly to mass market devices. HAPS at 20 km addressing at least 3GPP class 3 UE. |  | HAPs platform should be configurable with the following functionality. - where HAPs platform acts as a gNB (using CU+DU functionality) - where HAPs platform acts as IAB node. HAPs platforms have situations where the cell footprint on the ground is generally earth fixed beams, but we should also consider the case where they are slowly moving. HAPS platforms have beams that cover a large area (100 km radius) which may have spots of terrestrial coverage using the same frequency channel. Coexistence with terrestrial coverage is of importance. Our primary focus is on 3GPP class 3 devices. Since the study item has considered regenerative payloads, we should include this concept. |
| Eutelsat |  | - LEO highest priority - max Cell size edge to edge : 1000km - Transparent payload - Earth fixed beams - System should work for both: with or without GNSS to reach low power consumption - High priority on IoT services (NB-IoT and LTE-M) |  | The normative phase should consider as a highest priority, LEO based scenario providing direct access to 3GPP IoT devices over a global coverage to address the challenge of IoT coverage in extreme rural areas. Requirements for IoT via satellite already present in SA1 specification. |
| Fraunhofer |  | LEO transparent payload with moving beams on earth (1st prio) Scenario as described by Thales UEs with and without position determination capability (e.g. GNSS) shall be supported |  |  |
| Fraunhofer |  | LEO processed payload with moving beams on earth (2nd prio) providing eMBB and MTC type of services to UEs with power class 3. Usage scenarios include pedestrian, vehicular connectivity and public safety. UEs with and without position determination capability (e.g. GNSS) shall be supported. |  |  |
| Fraunhofer |  | GEO transparent payload (1st prio) Scenario as described by Thales. |  |  |
| Sony |  | Transparent GEOs with earth-fixed beams |  | Leverage existing transponders. Mostly ‘Other’ terminals because of link budget. |
| Sony |  | Regenerative HAPS with both earth-fixed and moving beams |  | New designs – HAPS ranging zones are national and so minimize regulatory/coordination issues. Class 3 and other terminals. |
| Sony |  | Regenerative LEOs with both earth-fixed and moving beams |  | New constellations – Class 3 and other terminals. |
| Hughes Network Systems (HNS) |  | HNS.scen.nr / 1st Priority – GEO Transparent The normative phase should consider in highest priority GEO transparent based scenario providing connectivity to support specific handheld devices and user terminal on board moving platforms (e.g. Aircrafts, vessels or trains) or hot spots in unserved/underserved areas. GEO operating in FR1 using very large antenna with space-deployable reflectors will result in higher G/T and e.i.r.p., higher throughput capacity and efficiency, hence will be able to support specific devices including 3GPP class 3 UE and vehicular connectivity. GEO operating in FR2 can support direct connectivity via VSAT alternatives. With earth fixed beams having negligible doppler effect, GEO is preferred scenarios, easier to implement especially to support mobility management and service continuity with terrestrials. |  | With increase needs for connectivity, it is obvious, there is a strong market demand for global coverage where GEO scenario fits very well, what more with HTS GEO that can support higher capacity. GEO is a less complex deployment to serve continental, regional or local service. NTN cell fixed on Earth associated with UE position determination is essential to prevent complex tracking area management and to comply with country specific regulations. Specific devices refer for example to smart phone equipped with protuberating antenna or moving/fixed platform mounted devices equipped with directional antenna (dish or phased array). |
| Hughes Network Systems (HNS) |  | HNS.scen.nr / 2nd Priority – NGSO Transparent (to include LEO and MEO). The normative phase should also consider NGSO transparent based scenario providing direct access to 3GPP class 3 UE devices over worldwide/global or multi country coverage to address the challenge of coverage in rural areas. |  | Transparent based NGSO with Earth fixed beam providing eMBB services directly to mass market devices. Target usage scenarios include Pedestrian, vehicular connectivity, public safety and many more |
| Hughes Network Systems (HNS) |  | HAPS scenarios can be adapted through NGSO solutions; hence no explicit work may be needed to support HAPS. |  | We believe challenges with HAPS can be tackled through NGSO solutions; hence no explicit work may be needed to support HAPS. |
| Sequans |  | Generic scenario with the following assumption: a) GEO and LEO first, HAPS later b) No preference for Max cell size c) Transparent d) Earth fixed beams e) without GNSS capability f). All cases from table B2.1 of 38.821 while IoT take priority. g) UE type 3 without any specific antenna subsystem. |  | ideal specification of Cell size and structure (number of beams, different size of beams if relevant, logical association between cells ID and beam) is preferred. |
| MediaTek |  | We want to priotize LEO at 600 km, 200 km max cell size (edge to edge) , transparent payload.Target use case is pedestrian. |  | Fixed beams and moving beams could be considered in Rel-17. The impact on specifications is not likely to be very significant for RAN1 or RAN2. Fixed beam will allow faster re-synchronization algorithms and beam switching |
| ESA |  | Both GEO and LEO have the same priority with transparent payload. |  | Direct access to 3GPP class3 below 6GHz, whereas VSAT (or equivalent phase-array terminal) for mmWave. |
| Sierra Wireless |  | LEO at 600 km, 200 km max cell size (edge to edge) highest priority Pedestrian highest priority but also higher mobility as well |  |  |
| CATT |  | The prioritized scenarios in R17 are LEO based access and GEO based access with transparent payload. Earth fixed beam or moving beam are considered both. UE with location determination capability (e.g. GNSS) should be taken as the baseline. UE type may include the VSAT and handheld type. But the VSAT UE is the first priority. Regarding the service type, in R17, only EMBB is considered. |  | We think the transparent payload assumption would simply the network architecture and deployment. For UE with location determination capability, it is very important to achieve robust performance through the UE location based time/Doppler shift pre-compensation, UE location assisted handover etc. In case of without UE location determination capability, the specification change might be significant and actually sufficient evaluation has not been conducted at this stage, e.g PRACH performance. |
| Intel Corporation |  | LEO a) LEO based access b) Up to 500 km c) Transparent payload d) Earth fixed beams e) UE with and without GNSS f) Pedestrian, Vehicular connectivity, Stationary, Airplanes connectivity g) 3GPP class 3 and other |  | LEO based access providing ubiquitous coverage for eMBB services. FDD only should be considered. Two frequency bands should be considered: S-band for 3GPP class 3 UEs (pedestrian) and Ka-band for other UE type (Vehicular connectivity, Stationary, Airplanes connectivity). |
| ORANGE |  | - LEO as the first priority, HAPS as a second priority, GEO as a third priority - 100 / 200 km max cell size for LEO - transparent as a first priority, regenerative as a second priority - fixed beams preferred vs mobile beams - location determination should be supported (also in case no GNSS is available) to comply with country regulations (e.g. emergency calls) - eMBB for extreme rural coverage, eMBB for wireless backhaul, IoT, emergency services Class 3 as a first priority, Class 2 as a second priority |  |  |
| Panasonic |  | 1st priority – Transparent based GEO - Directional antenna device with location determination capability - Main target usage scenario is connectivity for moving platforms, e.g. airplanes, vessels, trains (up to 1200km/h) - Max cell size is 1000km |  | Transparent based GEO provides broadband connectivity in world wide area for moving platform, e.g. airplane, vessel, train, as in the current satellite communications. |
| Panasonic |  | 2nd priority – Regenerative based HAPS - Earth fixed beams - 3GPP class 3 terminal, up to 500km/h, without location determination capability |  | Regenerative HAPS provides direct access for pedestrian to high speed mobile with 3GPP class 3 terminal in wide area. This scenario can be realized mostly by the existing specification. |
| Panasonic |  | 3rd priority – Transparent LEO - Earth fixed beams - Directional antenna device and 3GPP class 3 terminal with/without location determination capability |  | Transparent LEO provides a low latency broadband connectivity (compared to GEO) and/or direct access for IoT devices. Because LEO are technically more difficult than GEO and HAPS, LEO is the 3rd priority. |
| Huawei, HiSilicon | For a) and c), recommended scenario for Rel-17: LEO transparent payload and GEO transparent payload |  |  |  |
| Huawei, HiSilicon | For e), we recommend focusing on the support of one UE type in Rel-17 (either with GNSS capability or without GNSS capability) |  |  |  |
| Huawei, HiSilicon | For f), we recommend that the targeted usage scenario of NTN is primarily to provide coverage where it is difficult for terrestrial network. |  |  |  |
| Huawei, HiSilicon | For g), we recommend focusing on the support of handheld power class 3 device. |  |  |  |
| NOVAMINT |  | - LEO based access highest priority, GEO 2nd priority - Up to 1000 km - Transparent payload - Earth fixed beams preferred for easier integration - Should work for UE with or without location determination capability – For IoT, some UE may not support GNSS for power consumption reduction. - ÌoT the highest priority - UE type: 3GPP class 3 (IoT driven) |  | For IoT, LEO seems the most adapted solution and is higher priority |
| ZTE | - The LEO based access is 1st priority and GEO is the 2nd - The satellite parameters (set-2, in beam level) defined in the SI should be reused for WI; - Transparent load is preferred; - Moving beam; - Both cases, e.g., with/without capability for location determination - Prioritized usage scenarios in Rel-17 should be eMBB for UEs, e.g., mobile UE, airplane, car or ship. - The usage for backhaul and IoT can be deprioritized; - All UE types for eMBB should be supported. |  | - The LEO based access is considered as promising solution to support the eMBB service in NTN system. In addition, most of discussion in SI is targeted for this case. - W.r.t HAPS, from phase-1 study item, no additional enhancements is required to support it. For extreme case of HAPS, it can be also supported based on the enhancement for LEO. - Instead of max cell size, the design target based on the typical beam parameters, e.g., HPBW, is preferred with consideration of different implementation of cell shape and mapping among beam and cells. - Regarding the value, the parameters (set-2) used in the study item (set-2) can be used as the baseline, and the corresponding evaluation results in SI can be considered as good reference for normative work. - With transparent load, impact on Rel-15/16 NG-RAN standard is limited and from implementation perspective, the transparent load is friendly to operation and management. - During the study item, the moving beam is prioritized, and we need follow the same principle for normative work. - Both should be supported considering the diversity of UE type. - eMBB is more attractive scenarios. - -- |  |
| Mitsubishi Electric | a) 1st priority : LEO, 2nd priority : GEO, 3rd priority : HAPS c) both transparent and regenerative are important scenarios d) 1st priority : mobile beams, 2nd priority : Earth fixed f) 1st priority : Airplanes, 2nd priority : Vehicular, 3rd priority : IoT, 4th priority : Stationary, 5th priority : Pedestrian g) directional antenna |  | NTN support for terrestrial mobility should also be considered (LEO with moving cells) |  |
| Inmarsat | Isat_scn_1 / 1st priority - Transparent based GEO with Earth fixed cells providing eMBB services specific devices: GEO at 35,786 km addressing special and 3GPP classes of devices, transparent payload, 3400 km max cell size (edge to edge, considered at the edge of coverage for low elevation angles i.e. 10 deg) driven by initial acquisition requirements, Earth fixed cells. Target usage scenarios include mobility use cases like vehicular connectivity, aircraft connectivity, maritime safety, etc and specific classes of handheld. UE assumed to have GNSS capability – would be nice to support UEs without GNSS capability as well |  | The normative phase should consider GEO as a first priority as most of the current satellite operators providing global coverage currently operate GEO payloads. Furthermore, the impact of GEO on NR protocol changes is limited as compared to LEO as: - Satellite Doppler shift is negligible - cells are fixed from UE perspective - UE can be considered as fixed from satellite perspective. Transparent payload should be addressed in priority during Rel-17 to minimize impact on normative work. Note 1: 3400 Km as the max GEO cell size was agreed as a scenario parameter during Prague RAN1 #98 meeting |  |
| Inmarsat | Isat_scn_2 / 2nd priority – Transparent or regenerative HAPS with Earth-fixed cells providing eMBB services directly to mass market devices: HAP addressing at least 3GPP class 3 UE, transparent (bent pipe) or regenerative payload, 100 km max cell size (edge to edge), Earth fixed cells. Target usage scenarios include Pedestrian, vehicular connectivity, public safety. UE without GNSS capability shall be supported. |  | The normative phase should consider in second priority HAPS-based scenario providing direct access to devices as gap fillers or regional coverage in rural areas. As compared to LEO, HAP would not require global deployment but targeted gap filler capability direct to Class 3 UE, which results in substantially lower cost of deployment. Additionally, it would have substantially less impact on NR specification, due to much lower Doppler shift. |  |
| DLR | GEO and LEO access supporting for eMBB services |  |  |  |
| Intelsat | Scenario 1 / 1st priority – Transparent based GEO providing eMBB services: - GEO addressing other than class 3 UE in 3GPP, - Transparent payload, - 1000 km max cell size (edge to edge), - Earth fixed beams. - Target usage scenarios include: Stationary, airplane connectivity and possibly vehicular connectivity. - UEs with position determination capability (e.g. GNSS) shall be supported. |  | First priority GEO based scenario providing connectivity to mobile platforms (e.g. Aircrafts, vessels or trains) or hot spots in unserved/underserved areas. smart phone equipped with protubating antenna or moving/fixed platform terminal with directional antenna (parabolic dish or Flat panel phased array). |  |
| Intelsat | Scenario 2 / 2nd priority – HAPS providing eMBB services directly to mass market devices: - HAPS addressing 3GPP class 3 UE, - Transparent/regenerative payload, - 100 km max cell size (edge to edge), - Earth fixed beams. - Target usage scenarios include Pedestrian, vehicular connectivity, stationary, public safety. - UEs with position determination capability (e.g. GNSS) shall be supported. |  | second priority HAPS based scenario providing direct access over unserved/underserved areas to address the challenge of coverage in extremely rural areas. HAPS (transparent & regenerative) is relatively stationary & comparable to GEO. Impact to NR protocols is minimal The on-board antenna system can steer the beams to ensure also fixed beam foot print pattern on Earth. |  |
| CITICSAT | Priority: 1). GEO-based re-generative or transparent payload 2) LEO payload, providing eMBB services. |  | GEO satellite being the first priority, using fixed earth beams to provide high data rate service to the non-3GPP class 3 UE mobile platforms, e.g. the aircrafts and other land or maritime vehicles. |  |
| Nomor | - LEO transparent earth moving beams (non-transparent 2nd priority) - GEO transparent earth fixed beams |  | HAPS are covered by GEO/LEO technologies UEs with and without positioning capabilities Services: eMBB and mIOT for stationary and moving UEs as well as moving relays If Rel.17 specifies NR Broadcast, it should be available for NTN as well |  |
| FirstNet | Fi.scen.nr.1 / 1st priority - Transparent based LEO with Earth fixed beam providing eMBB services directly to mass market devices. |  | The normative phase should consider as a first priority, the scenario Th.scen.nr.1 proposed by Thales which will be able to serve 1st responders needs. |  |
| FirstNet | Fi.scen.nr.2 / 2nd priority – Transparent based GEO providing eMBB services to specific devices. |  | The normative phase should consider in second priority Thales proposed Th.scen.nr.2 scenario enabling to deploy emergency cells in disaster response |  |
| FirstNet | Fi.scen.nr.3 / 3rd priority – Transparent based HAPS with Earth fixed beam providing eMBB services directly to mass market devices. |  | The normative phase should consider in third priority Thales proposed Th.scen.nr.3 scenario to restore service after large disaster |  |
| Ericsson | Our preference is LEO transparent. |  | Here are comments per object listed in 2.1: a) LEO should be prioritized due to the larger market potential. GEO is also relevant to be addressed while HAPS does not need to be treated separately as what is specified for LEO applies for HAPS (see TR 38.821). b) The supported cell size should be based on the conclusion from TR 38.821, and relates to the ongoing study on RACH support for large time-frequency ambiguities. c) Our preference is to support transparent payload option, the benefits of which are listed as follows. - Transparent payloads are less complex and costly compared to regenerative payloads. - With transparent payloads, all gNB functionality resides on the ground, making it easier to build, test, and integrate with terrestrial 5G networks. - With transparent payloads, the terrestrial gNB is more accessible and less expensive to upgrade or modify to support new NR features introduced in later releases. - With transparent payloads, fix or improvement requiring hardware change/modification does not require replacement of satellites. - The time to market will be reduced since already deployed transparent payload satellites can be upgraded to support 5G. d) Our preference is to focus on one option and avoid addressing both steerable beam and fixed beam in one release. e) Our preference is to prioritize UE with location capability (e.g. GNSS) in Rel-17. f) Our preference is to prioritize LEO transparent case for vehicular and stationary (building mounted) usage scenarios as defined in Table B.2-1 in TR 38.821. g) We are open to consider UE types appropriate for NTN systems such as UEs with directional or high gain antennas. |  |
| OPPO | a) The 1st priority is LEO based access, and the 2nd priority is GEO based access. c) Both transparent and regenerative payload are supported. d) Earth fixed beams for LEO based access scenario is preferred. e) Both UE with and without location determination capability (e.g. GNSS) should be supported. |  | a) LEO could provide a lower delay and less propagation loss for eMBB service, while GEO could provide wider coverage on board moving platforms (e.g. Aircrafts, vessels or trains) d) It is easier for mobility management in the earth fixed beam case for LEO |  |

In the following, impacts on NR features for the proposed scenarios are being identified based on the on-going NTN study item (Rel-16) preliminary outcomes. This is pending to the outcomes of the on-going study.

### Question Ph1.2: What are the necessary RAN features to support the proposed NR based NTN scenarios ?

| Q- Ph1.2: Organization | Recommended Scenarios | RAN features impacted at RAN1, RAN2 and RAN3 |
| --- | --- | --- |
| Thales | Th.scen.nr.1: Transparent based LEO with Earth fixed beam providing eMBB services directly to mass market devices | RAN1: - Mandatory: None TBC - Yet to be confirmed: PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control, Enhancement of HARQ to mitigate the larger delay; addition to System information block (E.g. NG-RAN of NTN type, altitude); extension of CQI value range to optimise control loops (power, AMC). RAN2: - Mandatory: MAC/RLC/PDCP timers adaptation, adaptations on mobility (idle, connected mode) procedures to take into account satellite movement and increased latency - Yet to be confirmed: extension of System informations in SIB RAN3: - Mandatory: None - Yet to be confirmed: NTN type RAN4: RRM/RF performance requirements Note 1: Addressing LEO at 600 km will allow the NG-RAN standard to support any NGSO any scenarios at altitude greater than or equal to 600 km in terms of Doppler constraints. Note 2: It is assumed that pre compensation of Doppler shift/variation as well as delay variation is implemented |
| Thales | Th.scen.nr.2: Transparent based GEO providing eMBB services to specific devices | RAN1: - Mandatory: PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control to mitigate larger beam size, HARQ turned off, extension of CQI value range to optimise control loops (power, AMC) - Yes to be confirmed: enhancements to synchronization RAN2: - Mandatory: same as for LEO scenario but with extended latency constraint - Yet to be confirmed: same as for LEO scenario RAN3: - Mandatory: None - Yet to be confirmed: NTN type RAN4: RRM/RF performance requirements Note: It will also allow the NG-RAN standard to support any NGSO based scenarios in terms of Latency constraints. |
| Thales | Th.scen.nr.3: Transparent based HAPS providing eMBB services directly to mass market devices | RAN1: - Yet to be confirmed: None RAN2: - Yet to be confirmed: adaptations on mobility (idle, connected mode) procedures RAN3: - Yet to be confirmed: None RAN4: - Mandatory: RRM/RF performance requirements |
| Nokia | - Mobility for LEO with moving cells, incl RACH access - Feeder link mobility - Positioning | RAN1: Timing Advance, Doppler compensation, HARQ, RACH access, positioning RAN2: Mobility mechanisms and service continuity mechanisms RAN3: some feeder link mobility aspects may have RAN3 impact. Tracking Area management. |
| Samsung | Scenario 1 (high priority): LEO, max cell size up to 500km, both transparent and regenerative, mobile beams, UE with location determination capability as baseline, Targeted usage scenarios: pedestrian, UE type: 3GPP class 3 Scenario 2(high priority): GEO, max cell size up to 1000km, both transparent and regenerative, fixed beams, UE with location determination capability as baseline, targeted use scenarios: stationary, UE type: other | RAN1: frequency and time synchronization, RACH, UL timing, AMC, CSI feedback, power control, beam management, re-transmission RAN2: re-transmission RAN3: |
| Vodafone | Highest priority LEO with fixed-earth beams (stationary LEO cells) | See Thales’s comments |
| SoftBank | - HAPS | RAN2: potentially mobility enhancements for moving beams RAN4: (need further discussion) RRM/RF performance requirements Note: we would keep to the door open to introduce any essential functionality, which is specific for HAPS. This is because HAPS is a very new technology and serious issue may be identified during Rel-17 timeline. |
| Asia Pacific Telecom (APT) | Transparent LEO with moving beam for eMBB and eMTC services | RAN1: UL timing and frequency control, PRACH and RACH procedure including two-step RACH, HARQ, CQI, L1-mobility, power saving and feeder link switch RAN2: mobility, cell selection, RACH, HARQ, feeder link switch RAN3: feeder link switch |
| TNO | See Thales |  |
| Loon | Regenerative HAPs with earth fixed beams (or slowly moving earth fixed beams). Coexistence with Terrestrial | RAN1: - Additional CORESETs to handle multiple beams for coexistence with terrestrial - PCI confusion mitigation mechanisms RAN2: None RAN3: Support for centralized routing across HAPs for IAB based functionality Agree with the note from Softbank on the need to keep the door open for essential functionality specific to HAPs |
| Eutelsat | High priority on transparent based LEO, providing IoT services directly to IoT devices with the same chipsets/device used for Terrestrial and Non terrestrial Network. | RAN 1: Mobility support, new PRACH preamble, UL Timing advance, Doppler compensation. HARQ RAN 2: System Information, Paging, Mobility mechanism, TAU RAN 3: System Information, Paging, TAU RAN 4: RRM/RF performance requirements |
| Fraunhofer | LEO transparent payload with moving beams on earth (1st prio) | RAN1: (PRACH), Doppler compensation, UL synchronization (TA) RAN2: Mobility mechanisms (HO), update timers, HARQ adaptation RAN3: Tracking area management RAN4: Regulatory aspects: inline interference |
| Fraunhofer | LEO processed payload with moving beams on earth (2nd prio) | RAN1: (PRACH), Doppler compensation, UL synchronization (TA) RAN2: Update Timer, Handover, HARQ adaptation RAN3: Tracking Area Management RAN4: Regulatory aspects: inline interference |
| Fraunhofer | GEO transparent payload (1st prio) | RAN1: PRACH, Doppler compensation, UL synchronization (TA) RAN2: Update Timer, HARQ enable/disable RAN3: Tracking Area Management RAN4: Regulatory aspects |
| Sony | Transparent GEOs with earth-fixed beams | All the relevant issues identified during the SIs |
| Sony | Regenerative HAPS with both earth-fixed and moving beams | All the relevant issues identified during the SIs |
| Sony | Regenerative LEOs with both earth-fixed and moving beams | All the relevant issues identified during the SIs |
| Hughes Network Systems (HNS) | HNS.scen.nr / 1st Priority – GEO Transparent As much as possible include all applicable RAN features concluded in Rel-16 studies. | Mandatory RAN1: PRACH and random-access procedure adaptations for uplink timing and frequency control to mitigate larger beam size, HARQ turned off, extension of CQI value range to optimize control loops (power, AMC), techniques to increase spectral efficiency RAN2: MAC/RLC/PDCP timer adaptation, adaptations on mobility (idle, connected mode) procedures to take into consideration RTT and latency. RAN3: Tracking Area Management RAN4: RRM/RF performance requirements and regulatory aspects UEs with position determination capability shall be supported. |
| Hughes Network Systems (HNS) | HNS.scen.nr / 2nd Priority – NGSO Transparent (LEO and MEO). As much as possible include all applicable RAN features concluded in Rel-16 studies. | Mandatory Same as GEO Transparent. |
| Sequans | Generic scenario with preferred assumption listed in the table above | RAN1: to be discussed: modification to synchronisation signals and random access channels; close-loop scheme (power control, AMC, HARQ, MIMO etc…); adaptation of measurements and reporting procedures RAN2: to be discussed: timer adaptation at all layers; new information element in SIB; mobility, DC, paging handover procedures to be adapted, etc… RAN4: performance requirements; new bands to be added |
| MediaTek |  | We think that mandatory RAN features should at least include: RAN1 - New PRACH (format/sequence) due to new requirements for RTD and frequency offset - UL frequency correction indication in Msg2 - UL Timing advance enhancements - HARQ enhancements - Satellite pre-compensation of common Doppler and common propagation delay - Beam/Cell re-selection by idle UE and handover by connected UE RAN2 - Satellite-assisted beam specific system information – i.e. list of next beams, mapping of PCI / SSB for each beam - RRC dedicated signaling to configure enabling / disabling HARQ feedback per HARQ process per UE - Paging, Mobility mechanism, TAU RAN3 - System Information, Paging, TAU RAN 4: - RRM/RF performance requirements Note: CSI/AMC report and PHR / CL UL power control in connected UE is under discussion in SI. |
| Sierra Wireless | - Mobility for LEO with moving cells, incl RACH access | RAN1: Timing Advance, Doppler compensation, HARQ, RACH access, positioning RAN2: Mobility mechanisms and service continuity mechanisms |
| CATT | The prioritized scenarios in R17 are LEO based access and GEO based access with transparent payload. Earth fixed beam or moving beam are considered both. UE with location determination capability (e.g. GNSS) should be taken as baseline. UE type may include the VSAT and handheld type. But the VSAT UE is the first priority. Regarding the service type, in R17, only EMBB is considered. | RAN1: - Timing and Doppler compensation, RACH access - HARQ and reliability enhancement - Beam switching RAN2: - MAC/RLC/PDCP timers adaptation - Extension of System information in SIB - Mobility enhancement for service link and feeder link RAN3: - Feeder link Switch aspects may have RAN3 impact - Tracking Area management and paging |
| Intel Corporation | LEO | RACH procedure, HARQ, mobility, TA, etc. Details are TBD according to the outcome of the ongoing study. |
| Panasonic | 1st priority – Transparent based GEO - Directional antenna device with location determination capability - Main target usage scenario is connectivity for moving platforms, e.g. airplanes, vessels, trains (up to 1200km/h) - Max cell size is 1000km | RAN1: timing advance, RACH, HARQ for long latency RAN2: Timers enhancement, idle mode mobility enhancement for long latency RAN3: None |
| Panasonic | 2nd priority – Regenerative HAPS - Earth fixed beams - 3GPP class 3 terminal, up to 500km/h, without location determination capability | RAN1: None RAN2: None RAN3: None |
| Panasonic | 3rd priority –Transparent LEO - Earth fixed beams - Directional antenna device and 3GPP class 3 terminal with/without location determination capability | RAN1: Timing advance, RACH, Doppler/timing compensation, HARQ for satellite movement and long latency RAN2: feeder link switch, mobility enhancement for satellite movement RAN3: feeder link switch for satellite movement |
| Huawei, HiSilicon | Refer to response of question Ph1.1 | Our view is to refer to the outcome of the on-going study item, i.e. to make conclusions in the on-going study item and to document the conclusions in the TR. Then RAN plenary email discussion can simply refer to the conclusions in the TR. |
| NOVAMINT | 1st Priority: Transparent based LEO with Earth fixed beam providing IoT services | RAN 1: new PRACH preamble, Doppler compensation, UL Timing advance enhancements RAN 2: Satellite-assisted beam specific system Information, Paging, Mobility mechanism, TAU RAN 3: System Information, Paging, TAU RAN 4: RRM/RF performance requirements |
| ZTE | Refers to the Question Ph 1.1 | The recommendation part from the TR for this SI should be considered as the baseline and in general, followings are needed to be done as normative work: RAN1: - PRACH, Timing advanced, scheduling enhancement, CSI enhancement, HARQ process. RAN2: - Corresponding higher layer operation based RAN1 enhancement - Mobility enhancement and service continuity RAN3: Tracking area/impacts due to feeder link switching |
| Mitsubishi Electric | LEO | Timing advance, Doppler compensation, PRACH redesign Mobility mechanisms in both active and idle mode for LEO with fixed beams (moving cells) |
| Inmarsat | Isat_scn_1 / Priority 1 Transparent based GEO with Earth fixed cell providing eMBB services with direct access to special devices: | RAN1: - Support of large cellsize and latency is key. PRACH (format/sequence) and random-access procedure adaptations for uplink timing and frequency control - HARQ enhancements and synchronization - Assess PAPR performance and impact on non-linear Amplifier back-off for single carrier loading and impact of payload BFN. RAN2: - MAC/RLC/PDCP timers’ adaptation for extended latency (idle, connected mode) procedures to take into account extended latency RAN4: - RRM/RF performance requirements |
| Inmarsat | Isat_scn_2 / Priority 2 Transparent/Regenerative HAPS with Earth fixed cells providing eMBB services directly to mass market devices: | RAN1: - None: TBC RAN2: - None: TBC RAN3: - None: TBC RAN4: - RRM/RF performance requirements |
| Intelsat | scenario.1/priority 1: Transparent based GEO providing eMBB services to specific devices | RAN1: • PRACH (format/sequence) and random-access procedure adaptations for uplink timing and frequency control to mitigate larger beam size, Adaptation of NR OFDM to NTN GEO transmission. Assess PAPR performance and impact on nonlinear Amplifier backoff for single carrier loading and impact of payload BFN. RAN2: • MAC/RLC/PDCP timers adaptation for extended latency (idle, connected mode) procedures to take into account extended latency RAN4: RRM/RF performance requirements |
| Intelsat | scenario.2/priority 2: HAPS providing eMBB services directly to mass market devices | RAN1: • None RAN2: • TBC: adaptations on mobility (idle, connected mode) procedures RAN3: • TBC: None RAN4: • Mandatory: RRM/RF performance requirements |
| Nomor |  | RAN1 Physical layer configuration (configuration issues, control channel, uplink feedback configuration) but limited spec impact Limited scheduler impact: Potential enhancements of adaptive modulation/coding, power control, adaptive MIMO etc. Compensate large propagation delay: Uplink timing advance/alignment procedure and adaptation of power control; Modification/disabling of Hybrid ARQ and Channel State Information High Doppler frequency: Doppler compensation by pre-distortion, enhanced time/frequency acquisition; Optimized configuration of synchronization and reference signals RACH: Extended acquisition window to compensate larger delays; Improved RACH design (e.g. sequence, format, message) RAN2 General on protocol: Modify timer range / offset, allow for procedures with longer delay; Adaptation of RLC/PDCP sequence number, window sizes, reordering etc. RRC layer: Definition and configuration of NTN specific parameters; Provisioning of parameters via system information or UE specific signaling MAC layer: Consider required enhancements of resource allocation, link adaptation, timing advance, power control, feedback report, Hybrid ARQ etc. Mobility: Inter-satellites, between satellite and terrestrial gNB, cross country mobility; Idle mode mobility incl. tracking/notification area management, paging etc.; Connected mode mobility incl. handover, beam switching, radio link monitoring NTN specific positioning: GNSS may not always be available e.g. IoT devices RAN3 Some feeder link mobility aspects may have RAN3 impact. Tracking Area management. RAN4 Mandatory: RRM/RF performance requirements |
| Ericsson | LEO transparent with first priority. GEO transparent with 2nd priority. | Our view is that the work should be RAN1 led as physical layer feasibility is dependent on RAN1. RAN1 led: - Introduce PHY layer timing relationship modifications needed for NTN - Introduce frequency correction enhancements for NTN - Introduce new PRACH format for coping with delay and Doppler [RAN1, RAN2] - Introduce PHY layer enhancements for supporting disabling HARQ per UE and per HARQ process RAN2 led: - Identify and possibly specify enhancements for RRM to support connected mode mobility at least including effect of feeder link switch. [RAN2, RAN1, RAN3] - Identify and possibly specify enhancements for idle mode mobility at least including effect of feeder link switch. [RAN2, RAN1] - Identify and possibly specify enhancements for MAC including HARQ, RACH, DRX. [RAN2, RAN1] - Identify and possibly specify enhancements for RLC including timers. [RAN2] - Identify and possibly specify enhancements for PDCP for supporting connected mode mobility. [RAN2] RAN3 led: - Introduce the necessary NGAP and/or XnAP enhancements (as identified during the study phase) to support feeder link switch. - Identify and possibly specify the necessary NGAP and/or XnAP enhancements to support TA handling and paging for NTN. |
| OPPO |  | RAN1: - Uplink timing advance - Doppler frequency shift due to high mobility of satellite - PHY layer per HARQ process and per UE disabling/enabling HARQ machanism - Timing determination for UL/DL transmission & measurement RAN2: - MAC/RLC/PDCP timers’ adaptation for extended latency - Enhancement for UP including SR, HARQ, RACH, DRX - Mobility management for both LEO and GEO RAN4: RRM/RF performance requirements |

## Identification of candidate NB-IoT/LTE-MTC based NTN scenarios and features

During the on Rel-17 NB-IoT and LTE-MTC email discussion, interest on some NTN related subjects were identified and RAN#85 decided to have them treated as part of the Rel-17 NTN email discussion.

- NB-IoT via satellite

  - Support: Eutelsat, MediaTek, Ligado, Sierra Wireless (study), ESA, Fraunhofer, Intelsat, Novamint, Sony, Hughes (in NTN), Sequans, Gemalto

  - Concern: ZTE, Nokia, Deutsche Telekom, Philips (?)

- LTE-MTC via satellite

  - Support: Eutelsat, Ligado, Sierra Wireless (study), ESA, KPN, Intelsat, Novamint, Sony, Hughes, Sequans, Gemalto

  - Concern: Deutsche Telekom, Philips (?), Ericsson,

In addition, the way forward on ” NTN-specific positioning“ in Rel-17 shall also be discussed.

### Question Ph1.3: Which NB-IoT/LTE-MTC based NTN scenarios are to be considered in the Rel-17 NTN activity and with which priority?

| Q- Ph1.3: Organization |  | Recommended Scenarios defined in section 2.1 of this TDOC | Comments |
| --- | --- | --- | --- |
| Thales |  | Th.scen.iot.1 / 1st priority - Transparent based LEO with Earth fixed beam providing eMTC services directly to IoT devices : LEO at 600 km addressing at least 3GPP class 3 UE, transparent payload, 200 km max cell size (edge to edge), Earth fixed beams. Target usage scenarios include also IoT connectivity. At least UEs without position shall be supported. | Same infrastructure able to provide eMBB and eMTC services based on NB-IoT and/or LTE-M. NB-IoT/LTE-M would then be deployed in the same frequency plan as NR-NTN carriers. See scenario Th.scen.nr.1 |
| Thales |  | Th.scen.iot.2 / 2nd priority - Transparent based LEO with Earth moving beam providing eMTC services directly to IoT devices : LEO at 600 km addressing at least 3GPP class 3 UE, transparent payload, 1000 km max cell size (edge to edge), Earth moving beams. IoT connectivity only usage scenario. At least UEs without position shall be supported. | Specific infrastructure providing eMTC services only based on NB-IoT and/or LTE-M |
| Nokia |  | None | We believe that IoT applications can be supported by the NR NTN solution in general, and it is not necessary at the moment to include NB-IoT/eMTC as a concept in the Rel-17 WI. |
| Samsung |  | This scenario should be deprioritized | NB-IoT/LTE-MTC has not been studied or evaluated via SLS and LLS in Rel-16 NTN SI and it is not clear what the spec impacts will be. IN addition, we haven't seen clear use case/business yet. In Rel-17, we should target on the issues that have been thoroughly studied and this can be postponed to future release. |
| Vodafone |  | IoT and NB-IoT applications | Provided the UL and DL link budget are practical, remote monitoring and IoT type applications are well within the scope of the satellite services and should be utilised The services should be in-band, i.e. within the coverage spectrum, agreed by the regulator and not outside the band |
| Asia Pacific Telecom (APT) |  | Transparent LEO with moving beam for eMBB and eMTC services | Rel-17 NTN may support both eMBB and eMTC. |
| TNO |  | 1st priority NB-IoT _for LEO, 2nd priority MTCe for LEO | IoT type applications for e.g. logistics, offshore, maritime, and monitoring remote assets are an attractive new market. It addresses a need for verticals that cellular operators cannot fulfil (e.g. tracking / tracing of containers both on land and at sea) without integrating satellite access. |
| Loon |  | IoT and NB-IoT applications | We believe some tweaks will be required to support HAPs based IOT services |
| Eutelsat |  | NB-IoT and LTE-M for satellite IoT shall be addressed in this release 17. Same Priority for NB-IoT and LTE-M on Transparent LEO at 600km, fixed beams, 1000KM max cell size (edge to edge) providing IoT service directly to IoT Devices. | Infrastructure dedicated to eMTC (NB-IoT and LTE-M) |
| Fraunhofer |  | GEO and LEO scenarios, as mentioned in question 1. UEs with and w/o GNSS capability shall be supported. | We support NB-IoT adaptations for satellite, not the need for eMTC, where the applications can be served with solutions from a NR Rel-17 WI. |
| Sony |  | Support NB-IoT/LTE-MTC based NTN in all scenarios | NB-IoT/LTE-MTC based NTN scenarios for NTN have not been studied during any of the NTN SIs so far. We would suggest to commence with either a Rel-17 WI that includes a study specific to this aspect, or have a separate SI for NB-IoT/LTE-M based NTN. |
| Hughes Network Systems (HNS) |  | HNS.scen.nr / 1st Priority – GEO Transparent GEO satellite can expand the reach of IoT applications & provide extension of coverage for massive NB-IoT or LTE-MTC in rural locations without requiring costly updates to terrestrial network infrastructure. IoT services in general are not sensitive to latency and well suited for GEO. Should also take advantage of that doppler in GEO is almost negligible and therefore easily manageable and therefore less complex for standardization. | The same NTN infrastructure should be able to provide eMBB and eMTC services based on NB-IoT and/or LTE-M. Satellite would allow very fast deployment of NB-IoT or LTE-M services over large geographical areas. Many satellite stakeholders and infrastructure partners have validated the adaptation of LTE-M over GEO satellite and standardization can be easily accomplished. |
|  |  | HNS.scen.nr / 2nd Priority – NGSO Transparent NGSO can expand reach of IoT applications & provide extension of coverage for massive NB-IoT or LTE-MTC in rural locations without | The same NTN infrastructure should be able to provide eMBB and eMTC services based on NB-IoT and/or LTE-M. |
| Sequans |  | Cat-M and NB-IoT should be considered with higher priority. Selection of appropriate technology would depend on the considered use case. Same assumption as in table 1.1 |  |
| MediaTek |  | We agree with Thales proposed scenarios and their priorities. NB-IoT is first priority for satellite IoT | We think 2 deployments should be supported: - NB-IoT with inband deployment in NR carrier re-using specified rel-16 NR coexistence mechanisms (same view as Thales) with fixed-earth beams. - NB-IoT in standalone deployment with moving earth beams. NB-IoT RAT is well suited for typical satellite IoT applications – low data rates for small intermittent packets, long latencies, low power consumption, low cost, low link budget, simpler operations (no CSI, no HO). We believe it is critical to ensure early specification of a specified satellite NB-IoT solution with minimun delta for specification and implementation compare to legacy NB-IoT. |
| ESA |  | Same priority to NB-IoT/LTE-M | As already expressed our view in the dedicated NB-IoT email discussion for Rel. 17, we strongly believe that several satellite IoT applications and services could benefit of a standardized and global solution. The modifications are rather limited, in particular for the GEO case. |
| Sierra Wireless |  | Very important to support both LTE-M and NB-IOT as different parts of the world have deployed different technologies. Supporting NB and LTE-M will provide a much lower cost option vs just NR. We agree with Thales proposed scenarios and their priorities. | Many of the changes needed will be the same for LTE-M and NB-IOT thus the work load increase to support both will be minimal. |
| CATT |  | None | IoT application based on NTN is not urgent in Rel-17 since the business requirement and application scenario are not clear. Actually many technical issues have not been studied so far, for example: - Link budget for IoT - UE power consumption and capability - Mobile beam or fixed beam applicability - Date rate and MCS requirement, etc Only after we have solid knowledge and clear business scenarios, related standardization can be started. |
| Intel Corporation |  | TBD | Rel. 17 NTN work item scope should be focused on the scenarios and use-cases which were considered during the study item. Separate study item for NTN NB-IoT and/or LTE-MTC can be considered if there is enough TUs available. |
| ORANGE |  | if IoT is a selected use case, both LTE-M and NB-IoT should be considered |  |
| Panasonic |  |  | IoT service is one of the important scenario. But, NR based NTN can support it to some extent. Therefore, it would be preferable to focus on NR based NTN in Rel.17. Further optimization using NB-IoT/LTE-MTC could be considered in later release. |
| Huawei, HiSilicon | None for the time being. |  | NB-IoT/eMTC based NTN scenario need to be studied before we can conclude which scenario(s) to be prioritized. |
| NOVAMINT |  | IoT by satellite is a must have in release 17 1st priority is NB-IoT for transparent LEO LTE-M is 2nd priority GEO for both is 3rd priority | There is urgency to deliver IoT by satellite within the release 17 as there is a strong demand for a unified and global solution combining cellular and satellite in one chipset for economy of scale – it is time critical. Main business cases concerned are in particular Asset Tracking (see TR 22.836 for more details and KPIs) The most relevant and representative use case is containers tracking for which it is critical to have standardized solution for low power IoT as soon as possible: - 90% of world trade is carried by container ships - 100-150 million containers transported every year - Representing € 5 Trillion worth of cargo - 5 to 6 million containers crossing the sea at any time The 1st priority is NB-IoT as the most important aspect on the device side is power consumption and it appears it may be easier to address NB-IoT than LTE-M |
| ZTE |  | None and can be postponed | The NB-IoT/LTE-MTC is out of scope for the previous study item, no extensive discussions have been done and corresponding part for enhancements are not clear till now. Postponing the IoT/LTE-M is more preferred. |
| Inmarsat |  | Isat_scn_3 / 1st priority - Transparent based GEO with Earth fixed cell providing eMTC services directly to IoT devices ) UEs without GNSS support should also be supported. | Enhancements of NB-IoT/LTE-M to accommodate satellite delay and to increase spectral efficiency, for fixed and mobile IoT devices. See scenario 1 Ph1.2 |
| DLR |  | NB-IoT and LTE-MTC via satellite |  |
| Ligado |  | Lig.scen.io.1 / 1st priority – Transparent based GEO providing eMTC services to IoT devices : GEO addressing 3GPP class 2 & 3 UE, transparent payload, 2500 km max cell size (edge to edge), Earth fixed beams. IoT connectivity only usage scenario. UEs with position determination capability (e.g. GNSS) shall be supported. | infrastructure providing eMTC services only based on NB-IoT or LTE-M. Feasible to use existing UE chipsets. Ubiquitous reliable GEO coverage supplements terrestrial eMTC deployments. NB-IoT and LTE-M both adaptable to GEO with minimal modifications to standards. |
| Intelsat |  | Scenario 1 / 1st priority – Transparent based GEO providing eMTC services directly to IoT devices: - Transparent payload, - 1000 km max cell size (edge to edge), - Earth fixed beams. - Target usage scenarios IoT connectivity. - UEs without position determination. | Same infrastructure (Ph1.1) providing eMBB and eMTC services based on NB-IoT and LTE-M. |
| Intelsat |  | scenario.2 / 2nd priority – HAPS providing eMTC services directly to IoT devices: - HAPS at 20 km - addressing at least 3GPP class 3 UE, - transparent/regenerative payload, 100 km max cell size (edge to edge), - Earth fixed beams. - Target usage scenarios include also IoT connectivity. - At least UEs without position shall be supported. | Same infrastructure (Ph1.1) providing eMBB and eMTC services based on NB-IoT and LTE-M. |
| Nomor |  | None |  |
| Ericsson |  | We believe that 3GPP should focus on the adaption of NR for NTN in Release 17. If, despite our view, a study on NTN for LTE-MTC and NB-IoT is started in Release 17 then it should prioritize: a) TR 38.821 scenarios: transparent GEO and LEO with earth fixed beams. b) Cell sizes supported by existing physical layer design. c) Transparent architecture. e) UEs with and without GNSS capability. f) We prefer to focus on the use cases/performance metrics agreed in TR 45.820 g) UE types of low complexity. | The NTN Release 15/16 studies have shown that this is a complex task to adapt NR for NTN. Release 17 should focus on the completion of this work, before considering NTN adaptations for NB-IoT/LTE-MTC. In addition to our recommendation in items a) – e) we are not supportive of extending the capabilities of NB-IoT and LTE-MTC to deployment scenarios targeting FR2. |
| OPPO |  |  | We think in Rel-17 NTN we should firstly focus on the Rel-16 SI scope. In later release, we can study the NB-IoT/LTE-MTC use case in NTN. We don’t see the major issue of cost and complexity for IOT devices can be addressed in near term. |

In the following, impacts on RAN features for the proposed scenarios are being identified.

### Question Ph1.4: What are the necessary RAN features to support the candidate NB-IoT/LTE-MTC based NTN scenarios ? Answers shall be distinguished between NB-IoT and LTE-MTC.

| Q- Ph1.4: Organization | Recommended Scenarios | RAN features impacted at RAN1, RAN2 and RAN3 |
| --- | --- | --- |
| Thales | Th.scen.iot.1: 1st priority - Transparent based LEO with Earth fixed beam providing eMTC services to iot devices | RAN1: - Mandatory: - - Yes to be confirmed: PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control, addition to System information block (E.g. NTN type); RAN2: - Mandatory: adaptations on idle mobility procedures to take into account satellite movement and increased latency - Yet to be confirmed: extension of System informations in SIB RAN3: - Mandatory: None - Yet to be confirmed: NTN type RAN4: - Mandatory: RRM/RF performance requirements |
| Thales | Th.scen.iot.2: 2nd priority - Transparent based LEO with Earth moving beam providing eMTC services to mass market devices | RAN1: - Mandatory: PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control to mitigate larger beam size - Yes to be confirmed: addition to System information block (E.g. NTN type); RAN2: - Mandatory: adaptations on idle mobility procedures to take into account satellite movement and increased latency. Adaptations of Tracking Area Update management - Yet to be confirmed: extension of System informations in SIB Mandatory - Yet to be confirmed: same as above RAN3: - Mandatory: Network identities management - Yet to be confirmed: NTN type RAN4: same as above |
| Nokia | None | See Question Ph1.3 |
| Samsung | Please see our general comment | RAN1: RAN2: RAN3: |
| Vodafone | As stated by Thales for IoT services | See comprehensive list from Thales 1st row for IoT applications and services |
| Asia Pacific Telecom (APT) | Transparent LEO with moving beam for eMBB and eMTC services | RAN1: UL timing and frequency control, power saving |
| TNO | See Thales | See Thales impact |
| Eutelsat | For NB-IoT : Transparent based LEO at 600 km with Earth fixed beam providing eMTC services to iot devices | RAN 1 Satellite pre-compensation Adaptation on timing advance and frequency control New RACH preamble format RAN 2 Adaptation Mobility in idle mode Sytem information, Paging, TAU RAN 4 RRM/RF performance requirements |
| Eutelsat | For LTE-M: Transparent based LEO at 600km with Earth fixed beam providing eMTC services to iot devices | RAN 1 Mobility support Satellite pre-compensation Adaptation on timing advance and frequency control New RACH preamble format RAN 2 Adaptation Mobility in idle mode Sytem information, Paging, TAU Mobility support RAN 4 RRM/RF performance requirements |
| Fraunhofer | NB-IoT via GEO | RAN1: PRACH (massive multiple access), UL synchronization (TA, important for power saving) RAN2: Update Timer, HARQ enable/disable RAN3: Tracking Area Management (TBC) |
|  | NB-IoT via LEO | RAN1: PRACH, Doppler drift compensation, UL synchronization (TA) RAN2: Update Timer (TBC), HARQ enable/disable RAN3: Tracking Area Management (TBC) |
| Sony | Premature to answer this question since NB-IoT/LTE-MTC based NTN studies were not included in the previous NTN SIs. | NB-IoT/LTE-MTC based NTN scenarios for NTN have not been studied during any of the NTN SIs so far. We would suggest to commence with either a Rel-17 WI that includes a study specific to this aspect, or have a separate SI for NB-IoT/LTE-M based NTN. |
| Hughes Network Systems (HNS) | There has been broad interest to see NB-IoT and LTE-MTC working over satellite, either geostationary or non-geostationary architectures. NB-IoT and LTE-M via satellite would augment the geographical coverage of terrestrial IoT networks and act as a back-up for terrestrial networks. | For NB-IoT, 2 deployments should be supported: • NB-IoT with in-band deployment in NR carrier • NB-IoT in standalone deployment For both NB-IoT and LTE-MTC: RAN1: PRACH (format/sequence) and random-access procedure adaptations for uplink timing and frequency control to mitigate larger beam size RAN2: Adaptations on idle mobility procedures to consider satellite movement and increased latency. Adaptations of Tracking Area Update management RAN3: Network identities management RAN4: RRM/RF performance requirements |
| Sequans | Generic scenario with preferred assumption listed in table 1.1 above | Similar to table 1.2 with additional consideration on per technology e.g. CE operation in particular with LEO. The details need to be discussed with respect to selected scenario. |
| Mediatek | We want to priotize LEO at 600 km, 1000 km max cell size (edge to edge) , transparent payload.Target use case is NB-IoT LPWA. | We think that mandatory RAN features should at least include: RAN1 - New PRACH (format/sequence) due to new requirements for RTD and frequency offset - UL frequency correction indication in Msg2 - UL Timing advance enhancements - Satellite pre-compensation of common Doppler and common propagation delay - Beam/Cell re-selection by idle UE RAN2 - Satellite-assisted beam specific system information – i.e. list of next beams, mapping of PCI / SSB for each beam - Paging, TAU RAN3 - System Information, Paging, TAU RAN 4: - RRM/RF performance requirements Note: HARQ enhancements are not necessary. Typical satellite applications are not delay critical and low data rates. |
| Sierra Wireless | Agree with Thales | Same list as Thales |
| CATT | None | See the comments in question Ph1.3 |
| Panasonic |  | See Question Ph1.3 |
| Huawei, HiSilicon | None for the time being. | RAN feature impacted for NB-IoT/eMTC based NTN scenario need to be studied. |
| NOVAMINT | LTE-M via transparent LEO at 600 km, earth fixed beam, 1000 km max cell size (edge to edge) | Same as LTE-M + RAN1: Mobility support RAN2: Mobility support |
| ZTE | None and can be postponed | Detailed study is needed. |
| Inmarsat | Isat_scn_3 / Priority 1 Transparent based GEO with Earth fixed cells providing eMTC services direct to IoT devices) | Similar to Ph1.2 (scenario 1: priority 1). In addition to enhancements required for Scenario 1, Ph1.2: - improve battery life - reduce signalling overhead - increase spectral efficiency - RACH design to support large number of UEs in single cell |
| Ligado | Lig.scen.io.1 / 1st priority – Transparent based GEO providing eMTC services to IoT devices | RAN1: - Mandatory: PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control to mitigate larger beam size - Yet to be confirmed: addition to System information block (E.g. NTN type); RAN2: - Mandatory: adaptations on idle mobility procedures to take into increased latency. - Yet to be confirmed: extension of System information in SIB RAN3: - Yet to be confirmed: NTN type RAN4: RRM/RF performance requirements |
| Intelsat | Scenario 1 / 1st priority – Transparent based GEO providing eMTC services to IoT devices | Similar to Ph1.2 (scenario 1: priority 1) |
| Intelsat | Scenario 2/2nd priority – HAPS with Earth fixed beam providing eMTC services to IOT devices | Similar to Ph1.2 (scenario 2: priority 2) |
| Nomor | None |  |
| Ericsson | See our answer in Ph1.3 | LTE-MTC and NB-IoT are, relative to NR, systems of low complexity. To maintain this relation, LTE-MTC and NB-IoT support for NTN should, if introduced, be provided through a minimal set of adaptations on top of the existing RAN features. We are not supportive of any deployment scenarios that contradicts this principle. The aspects that need to be studied to determine the RAN features that needs to be adapted are listed under Ph1.5. |
| OPPO |  | Refer to our reply to Ph1.3 |
| Sierra Wireless | Premature before study. | Same changes will be needed for NB-IOT and LTE-M which may include: NB-IoT/LTE-MTC based NTN scenarios for NTN have not been studied during any of the NTN SIs so far. We would suggest to commence with either a Rel-17 WI that includes a study specific to this aspect, or have a separate SI for NB-IoT/LTE-M based NTN. |

### Question Ph1.5: What are the features/aspects of NR NTN SI recommendations that can be applicable for NB-IoT/LTE-MTC based NTN scenarios or will a study phase be required to confirm this ? Answers shall be distinguished between NB-IoT and LTE-MTC.

| Q- Ph1.5: Organization | Features supporting “NB-IoT/LTE-MTC based scenarios” that can leverage existing studies (e.g. Rel-16 NR NTN SI) | Comments |
| --- | --- | --- |
| Thales | Idle mode mobility procedures Adaptations of MAC/RLC/PDCP timers |  |
| Nokia | None | See Question Ph1.3 |
| Samsung | Please see our general comment | RAN1: RAN2: RAN3: |
| Vodafone | Our preference is IoT services | This may be subject of further discussions, however, to distinguish IoT from eMTC service under a satellite coverage, and in order to signal to different devices in the field, a number of system Information messages could be different. And in order to build a robust system it is worthwhile to have a short study to distinguished signalling messages for IoT and eMTC. |
| Asia Pacific Telecom | UL timing and frequency control |  |
| TNO | See Thales |  |
| Eutelsat | Several feature from Rel-16 NR can be reused: - Idle mode mobility - Satellite-assisted System Information - Satellite pre-compensation for common Doppler, common propagation delay - Satellite UL frequency correction indication in Random Access procedure Timing offset for UL transmission timing |  |
| Fraunhofer | NB-IoT in GEO and LEO scenarios: Uplink synchronization/Timing advance (important for power saving), Adaptation of timers on MAC layer and above (TBC); HARQ (TBC) |  |
| Sony | Premature to answer this question since NB-IoT/LTE-MTC based NTN studies were not included in the previous NTN SIs |  |
| Hughes Network Systems (HNS) | Idle mode mobility procedures Adaptations of MAC/RLC/PDCP timers |  |
| Sequans |  | Same as above. Study phase may be preferable to fine tune the required modification |
| MediaTek |  | There are synergies between essential features to support satellite mode between NR and NB-IoT. Many aspects require Solutions that can be generic to both RATs, with only RAT-specifc details to be specified. RAN1 - UL frequency correction indication in Msg2 - UL Timing advance enhancements (initial and for connected UE) - Satellite pre-compensation of common Doppler and common propagation delay RAN2 - Satellite-assisted beam specific system information – i.e. list of next beams, mapping of PCI / SSB for each beam - Paging, TAU |
| Sierra Wireless | Agree with Thales | Same list as Thales |
| CATT | None | See the comments in question Ph1.3 |
| Panasonic |  | See Question Ph1.3 |
| Huawei, HiSilicon | There are some areas that NB-IoT/eMTC-based NTN can leverage the outcome of the NR NTN SI, e.g. - Channel model and Link budget calculation methodology - TA adjustment mechanism - MAC/PDCP/RLC adaptation - Idle-mode mobility, paging, radio-link monitoring … | To what extent the conclusions of NR NTN SI can be apply to NB-IoT/eMTC based NTN scenario need to be identified/confirmed. |
| NOVAMINT | - Idle Mode mobility - Satellite-assisted System Information - Pre-compensation for common Doppler, common propagation delay, - UL frequency correction indication in Msg2 in RA procedure | It may be worthwhile to have a short study first on IoT to complete the picture – For example, which NB-IoT deployment is preferred (considering verticals point of view) between in Band or standalone. |
| ZTE | None and can be postponed | See Question Ph1.3 |
| Ligado | Timing Advance for GEO delay Adaptations of MAC/RLC/PDCP timers Mobility management enhancements HARQ enhancements |  |
| Nomor | None |  |
| Ericsson | Significant effort has been put in channel models, problem descriptions and evaluation scenarios agreed in TR 38.811 and TR 38.821. This work can be partly reused if LTE-MTC/NB-IoT for NTN will be considered in a later release. A subset of the solutions considered for NR to manage timing and mobility aspects, HARQ and GNSS support can be considered for LTE-MTC/NB-IoT. | We believe it is suitable to first adapt NR for NTN. In a second step 3GPP may consider if and how the NR adaptions introduced in TR 38.821 can be applied on LTE-MTC and NB-IoT. When doing so it is important to remember that NR is a highly advanced technology while LTE-MTC and NB-IoT by design strive for simplicity. The following aspects deserve to be studied in the context of LTE-MTC/NB-IoT: - The impact and necessary modifications of PHY layer timing aspect - Adaptive use of HARQ feedback - Link-budgets - Aspects related to UE GNSS support - Idle and connected mode mobility aspects - Impact from feeder link switch - Impact on timers in all layers of the protocol stack |
| OPPO | None | Refer to our reply to Ph1.3 |

### Question Ph1.6: What are the aspects of NR NTN SI recommendations that are not applicable for NB-IoT/ LTE-MTC based NTN scenarios and will require further study for NB-IoT or LTE-MTC ? Answers shall be distinguished between NB-IoT and LTE-MTC.

| Q- Ph1.6: Organization | Features supporting “NB-IoT/LTE-MTC based NTN scenarios” that require further study | Comments |
| --- | --- | --- |
| Thales | PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control RRM/RF performance requirements Energy management (especially in LEO) |  |
| Nokia | None | See Question Ph1.3 |
| Samsung | Please see our general comment | RAN1: RAN2: RAN3: |
| Vodafone | Agree with the items listed by Thales, however there may be other elements which should be studied further |  |
| Asia Pacific Telecom (APT) | PRACH, RACH procedure, HARQ and mobility |  |
| TNO | Agree with Thales |  |
| Eutelsat | - PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control - RRM/RF performance requirements |  |
| Fraunhofer | PRACH, Physical layer procedures, Paging, Tracking Area Update Link performance of NB-IoT operation over satellite with small SCS. |  |
| Sony | Premature to answer this question since NB-IoT/LTE-MTC based NTN studies were not included in the previous NTN Sis | NB-IoT/LTE-MTC based NTN scenarios for NTN have not been studied during any of the NTN Sis so far. We would suggest to commence with either a Rel-17 WI that includes a study specific to this aspect, or have a separate SI for NB-IoT/LTE-M based NTN. |
| Hughes Network Systems (HNS) | RRM/RF performance requirements |  |
| MediaTek | NB-IoT | The main differences of NB-IoT with NR NTN are: - New PRACH (format/sequence) should be based on rel-13 NB-IoT design with some enhancements. - There is no support of mobility in connected UE. Cell re-selection is done following RLF trigger in legacy NB-IoT. - Simpler Physical control procedures - There is no CQI report in legacy NB-IoT - Only open loop power control is supported in legacy NB-IoT HARQ enhancements are not necessary. Typical satellite applications are not delay critical and low data rates. |
| Sierra Wireless | Agree with Thales |  |
| CATT | None | See the comments in question Ph1.3 |
| Panasonic |  | See Question Ph1.3 |
| Huawei, HiSilicon | - Detailed link budget - HARQ handling - … | NB-IoT/Emtc differ from NR in terms of e.g. channel bandwidth, system bandwidth, RTT, cost constraint. Therefore we believe a detailed study for NB-IoT/Emtc based NTN scenario is needed. |
| NOVAMINT | Same as Thales + Preferred deployment option for NB-IoT | Short study to investigate further some aspects for IoT |
| ZTE | None | See Question Ph1.3 |
| Ligado | PRACH (format/sequence) and random access procedure adaptations for uplink timing and frequency control RRM/RF performance requirements Peak data rate enhancement Multicast / broadcast enhancements | These aspects are not addressed in the on going Rel-16 NTN study item. |
| Intelsat | RRM/RF performance requirements Energy management |  |
| Nomor | None |  |
| Ericsson | We are not supportive of extending LTE-MTC/NB-IoT for NTN in FR2. | NR support for NTN is partly based on NR’s scalable numerology. LTE-MTC/NB-IoT are designed for 15 kHz sub-carrier spacing and the needed adaptations should be based on this support. LTE, including LTE-MTC/NB-IoT, do not support FR2. Extending LTE support to FR2 would imply a considerable effort and impact the strength of NR relative to LTE. We are therefore not supportive of FR2 for LTE-MTC/NB-IoT. |
| OPPO | None | Refer to our reply to Ph1.3 |

## Identification of NTN-specific positioning required standardisation

### Question Ph1.7: What should be done with respect to “NTN-specific positioning” as part of Rel-17 ?

| Q- Ph1.7: Organization | Proposal wrt “NTN-specific positioning” | Comments |
| --- | --- | --- |
| Thales | Feature allowing to locate UE without position determination mechanism (e.g. GNSS) could be defined in 3GPP else it can be handled as an implementation issue. |  |
| Nokia | Network-based positioning (can be UE-assisted) is required for NTN | We should assume non GNSS capable UEs and need to determine the position of a UE in order to fulfil legal requirements (public safety) and country specific policies. The position needs to be verified by the network, as UE based positioning may be faked or erroneous. |
| Samsung | The availability of location information at both the satellite and the UE sides can assumed to be a baseline. | The availability of location information has been discussed in Rel-16. However, there is no discussion for specific positioning schemes, i.e., how to acquire location information. Therefore, our suggestion is not to include positioning scheme discussion in Rel-17. |
| Vodafone | UE with basic on-board ephemeris as well as network, NTN, assisted positioning | Agree with Nokia’s comment regarding the regions’ regulatory environment for location a user |
| Asia Pacific Telecom (APT) | UE without GNSS has been considered in NTN SI. Related features shall be specified in Rel-17. | UE location may not be needed in NTN. Also, PLMN may be fixed on the ground, that may be enough to fulfil legal requirements and country specific policies. |
| Fraunhofer | Solution determination to allow UE location w/o third party localization information, e.g. GNSS | We would like to support UEs w/o GNSS location capabilities. |
| Sony | UE positioning is needed but for first WID, UEs should work with/without positioning capability. For UE positioning-capable functionalities, prioritise UE positioning capability based on GNSS | NTN-specific positioning should adopt progress on NR positioning. In Rel15 & Rel16, gNB does not know the UE position. The framework is still under discussion in RAN3. Existing framework is that UE reports to SMLC. Transparent to gNB. |
| Hughes Network Systems (HNS) | Consider UE with positioning (GNSS). Otherwise consider feature allowing to locate UE without position determination mechanism that could be defined in 3GPP else | Need to determine the position of a UE in order to fulfil legal and emergency requirements especially for public safety. |
| MediaTek | Network-based positioning is not priority | GNSS-based capable Ues should be sufficient in Rel-17. |
| ESA | Not a priority | GNSS-based solutions should be sufficient at least for Rel-17 scoping. |
| Sierra Wireless | Network-based positioning is not priority |  |
| CATT | We share same view with Samsung. NTN network and UE should have the location determination capability. | We assume GNSS capable UEs are the starting point for NTN application, if without GNSS capability is deemed as necessary, it could be defined in future release. How to acquire the location information for the UE without GNSS capability, it is out of R17 scope. |
| Intel Corporation | Do not consider NTN-specific positioning in Rel. 17. | We see some benefits to support NTN-specific positioning. However, there is no urgency to specify this feature in Rel. 17. The NTN-specific positioning can be considered in future releases as part of NR positioning SI/WI. |
| Huawei, HiSilicon | Positioning can be handled by implementation in Rel-17 time frame, e.g. GNSS can be supported by all terminals for example if there is a legal requirement for positioning. |  |
| NOVAMINT | Not a priority for R17 | We should consider for IoT that the UE should work with or without positioning capabilities. The main issue is power consumption. |
| ZTE | Deprioritize the network-based positioning for NTN | No discussion related to this part is done in the study item since the NR based solution is not done yet. Postponing of this part can be considered. |
| Intelsat | UE location without position determination mechanism. |  |
| Nomor | Network-based positioning (can be UE-assisted) is required for NTN | Non GNSS capable devices have to be supported |
| Ericsson | Not part of Rel-17 scope. | The meaning of “NTN-specific positioning” is not clear. Until it is properly clarified, we do not see a need to address NTN specific positioning in Rel-17. |
| OPPO | Not part of Rel-17 scope. | Agree with Ericsson |

## Other related scenarios and features

CMCC recommends add ATG (air-to-ground) deployment scenario into NTN study scope to provide economically efficient in-flight connectivity service for civil aircraft, using ground-based cell towers that send signals up to an aircraft’s antenna(s) of onboard ATG terminal. As a plane travels into different sections of airspace, the onboard ATG terminal automatically connects to the cell with strongest received signal power, just as a mobile phone does on the ground.

Figure 2-1: Illustration of ATG network.

ATG that has been proposed by RP-191217 can be seen as an inverse network of NTN, where for NTN network UE on earth and gNB on space, while for ATG network gNB on ground and UE on sky. Despite the inverse access scenario, some other technique challenges, such as large cell coverage range, high doppler shift, seems similar, which could be resolved by unified design to minimize specification effort.

### Question Ph1.8: Which other scenarios are to be considered in the Rel-17 NTN activity and with which priority? (questions proposed by CMCC)

| Q- Ph1.8: Organization | Recommended Scenarios defined in section 2.5 of this TDOC | Comments |
| --- | --- | --- |
| CMCC | - ATG scenario to be supported - Max cell size: 600 km diameter - - - - - ATG terminal with location determination and global time/frequency synchronization via GNSS is assumed - - - UE type: airplanes with directional antennas - At least TDD mode to be supported with frequency sharing with terrestrial network (e.g., 4.8GHz is considered by CMCC) | - To cover maximum 300km coverage range, maximum 2ms GP is required. In order to limited GP overhead, larger TDD switching period is preferred. For example, 20ms TDD switching period is preferred to limited GP overhead under 10%. Furthermore, 30kHz SCS and 28DL:4S:8UL configuration is considered to support maximum 1200km/h flight speed and DL traffic dominated traffic characteristic. Note that the dedicate frame structure can be configured via two concatenate 10ms uplink-downlink periods, while the first one with all DL slots, and the second one with 8DL:4S:8UL configuration. - - - - - - - Civil aircraft with altitude 6~12km and flight speed 500~1200km/h - Operators are interested to adopt non-disjoint proprietary frequency for deploying both ATG and terrestrial networks to save frequency resource cost. From China Mobile’s point of view, 4.8GHz is an interesting frequency(higherbands maybe also candidates in the future) for deploying both ATG and terrestrial NR network, since the latter network aims to local deployment for hot point enhancement and vertical use cases, which making geography isolating becomes an effective and interesting mechanism to alleviate the mutual interference between ATG and terrestrial networks, as ATG gNBs can be deployed in underpopulated regions like remote mountains. |
| TNO | Prefer not to include ATG, keep it simple. | ATG can already be provided (based on LTE) in Europe. NTN should focus on satellite based access to planes. |
| Eutelsat | Same view as TNO |  |
| Hughes Network Systems (HNS) | Same view as TNO |  |
| Novamint | Prefer not to include ATG for R17 |  |

### Question Ph1.9: What are the necessary RAN features to support the other proposed NR based NTN scenarios ? (questions proposed by CMCC)

| Organization | Recommended other Scenarios | RAN features impacted at RAN1, RAN2 and RAN3 |
| --- | --- | --- |
| Q- Ph1.9: CMCC | ATG scceario | RAN1: Physical layer enhancement to support extreme large coverage (up to 300km), as well as dedicate frame structure with >16 concatenated DL slots, including the following aspects - Potential enhancements to support GNSS-assisted-based pre-compensation mechanism of timing advancing at UE side, if needed - Potential enhancements to >15 slot indication for PDSCH-to-HARQ feedback timing (K1), if needed - Potential enhancements to support >16 HARQ process for PDSCH or enhanced TTI bundling with bigger TBS, if needed RAN3: - Potential enhancements to support conditional handover, if needed RAN4: Core specifications of RF and RRM requirements for coexistence between ATG and terrestrial network - Co-existence evaluation for ATG network (e.g. ACLR, ACS) - Study and identify new power class UE/BS category for ATG network - Study and identify RF requirements for ATG UE/BS - Study and identify RRM core requirements |
| Vodafone | In the initial phase of NTN services, we would prefer to keep the services simple and manageable and the emphasis shuld be on providing additional coverage to Cellular network coverage. In future once the Satellites services have stablised and have matured we could explore more advanced services. | At this stage it is difficult to say. |
| Eutelsat | Share the same view as Vodafone |  |
| Hughes Network Systems (HNS) | Same view as Vodafone |  |
| NOVAMINT | Same as Vodafone |  |

## Phase 1 summary

### Overview

35 organisations provided a response to the survey: Asia Pacific Telecom, CATT, Citicsat, CMCC, DLR, Ericsson, ESA, Eutelsat, Firstnet, Fraunhofer IIS/HHI, HNS, Huawei, Inmarsat, Intel, Intelsat, Ligado, Loon, Mediatek, Mitsubishi, Nokia, Nomor, Novamint, Oppo, Orange, Panasonic, Pivotal communication, Samsung, Sequans, Sierra Wireless, Softbank, Sony, Thales, TNO, Vodafone, ZTE.

|  | NR based NTN scenarios | NB-IoT/LTE-MTC based NTN scenarios | NR based ATG scenarios |
| --- | --- | --- | --- |
| # of organisations having proposed a scenario | 33 | 18 | 1 (CMCC) |
| # of organisations that didn’t propose a scenario | 2 (CMCC, Ligado) | 6 (Pivotal, Softbank, Mitsubishi, Citicsat, Firstnet, CMCC) |  |
| # of organisations that de-prioritise such scenarios | - | 10 (Nokia, Samsung, CATT, Intel, Oppo, Panasonic, Huawei, ZTE, Nomor, Ericsson) | 5 (TNO, Eutelsat, HNS, Novamint, Vodafone) |

Observation 1 (Phase 1): In Rel-17, NR based NTN scenarios are considered to be prioritised over NB-IoT/LTE-M based NTN scenarios

Observation 2 (Phase 1): NR based ATG scenarios are deprioritised

### eMBB - NR based NTN scenarios

| Characteristics | Views |
| --- | --- |
| Orbit (HAPS, LEO, GEO) | - LEO: - 23 propose it as 1st priority. - 5 propose it as 2nd or 3rd priority. - GEO: - 11 propose it as 1st priority. - 13 propose it as 2nd or 3rd priority. - HAPS: - 3 (Softbank, Loon, Pivotal) propose it as 1st priority. - 13 propose it as 2nd or 3rd priority of which 4 consider that HAPS will be supported when addressing LEO. |
| Transparent versus regenerative | - LEO: 20 prioritise transparent of which 4 prioritise both transparent and regenerative payloads. 1 (Vodafone) prioritise regenerative payload - GEO: 9 prioritise transparent. 1 (Citicsat) prioritise also Regenerative payload |
| UE type | - LEO and HAPS: All except two (CATT & Mitsubishi) prioritise the support of 3GPP class 3 - GEO: All except 3 (HNS Sequans, ESA) prioritise the support of other than 3GPP class 3 UE |
| Cell size (edge to edge) | - LEO: 11 prioritise small cell size < 500 km - GEO: All prioritise cell size of 1000 km and greater |
| Earth fixed/moving beam footprint | In case of LEO, 10 prioritise Earth fixed beam footprint. 7 prioritise Earth moving beam footprint. 3 propose to select one of them for the normative phase. |
| UE’s Position determination capability | 17 prioritise the support of at least UE without GNSS capability. While 8 prioritise the support of only UE with GNSS capability. |
| OFDM PAPR assessment | 2 organisations (Inmarsat, Intelsat) proposed to assess PAPR performance and impact on non-linear Amplifier back-off for single carrier loading and impact of payload BFN. |

Observation 3 (Phase 1): In Rel-17, NR based LEO scenarios should be prioritised.

Observation 4 (Phase 1): Transparent payload for LEO and GEO scenarios should be prioritised.

Observation 5 (Phase 1): At least 3GPP class 3 UE should be supported by LEO scenarios.

Observation 6 (Phase 1): At least UE without GNSS should be supported in NTN.

Observation 7 (Phase 1): Both Earth fixed or Earth moving beam footprint may be considered during the normative phase.

Observation 8 (Phase 1): The necessity of explicit work under NTN WI to support HAPS scenarios was not identified through the phase 1 email discussion. The challenges for certain HAPS based scenarios can be solved by the solutions for LEO.

Proposal 1 (Phase 1): In first priority, Rel-17 NR-NTN NWI should include the specification of a transparent payload based LEO scenario addressing at least 3GPP class 3 UE without GNSS capability and with both Earth fixed & moving beamfoot print. This will also support HAPS based scenarios.

Proposal 2 (Phase 1): In second priority, Rel-17 NR-NTN NWI should also include the specification of a transparent payload based GEO scenario addressing other than 3GPP class 3 UE with GNSS capability.

### eMTC - NB-IoT/LTE-MTC based NTN scenarios

| Characteristics | Views |
| --- | --- |
| Orbit (HAPS, LEO, GEO) | - LEO: 8 propose it as 1st priority. 1 (HNS) propose it as 2nd priority. - GEO: 8 propose it as 1st priority. - HAPS: 1 (Loon) proposes it as 1st priority. 1 (Intelsat) propose it as 2nd priority. |
| Transparent versus regenerative payload | In all cases, transparent payload is prioritised |
| NB-IoT versus LTE-M | - 14 proposes to address adaptations of both NB-IoT and LTE-M for the support of NTN - 6 proposes to address only the adaptations of NB-IoT for the support of NTN |
| UE type | In all cases the support of 3GPP class 3 is prioritised |
| Earth fixed/moving beam footprint | In case of LEO, - 5 prioritise Earth fixed beam footprint. - 1 prioritise Earth moving beam footprint as 1st priority but 3 as 2nd priority. |
| UE’s position determination capability | - 8 prioritise the support of at least UE without GNSS capability. - 1 prioritise the support of only UE with GNSS capability. |

Observation 9 (Phase 1): Both adaptations of NB-IoT and LTE-M to support NTN should be considered for NB-IoT/LTE-M based NTN scenarios

Observation 10 (Phase 1): Both transparent payload based GEO and LEO scenarios should be considered for NB-IoT/LTE-M based NTN scenarios

Observation 11 (Phase 1): The support of 3GPP class 3 UE should be considered in priority for NB-IoT/LTE-M based NTN scenarios

Observation 12 (Phase 1): Both types of UE with or without GNSS capability should be supported by NB-IoT/LTE-M based NTN scenarios

Observation 13 (Phase 1): Both Earth fixed or Earth moving beam footprint may be considered for NB-IoT/LTE-M based NTN scenarios

Several organisations have suggested to handle NR based NTN scenarios and NB-IoT/LTE-M based NTN scenarios in separate Work Items

Observation 14 (Phase 1): If NB-IoT/LTE-M based NTN scenarios are addressed during Rel-17, they should be considered in a separate Work Item from the one addressing NR based NTN scenarios

Proposal 3 (Phase 1): NB-IoT/LTE-M based NTN scenarios should be considered as part of a separate Rel-17 NWI  eMTC-NTN

Proposal 4 (Phase 1): Rel-17 eMTC-NTN NWI should include the specification of transparent payload based LEO & GEO scenario addressing 3GPP class 3 UE with/without GNSS capability and with Earth fixed & moving beam footprint.

### NTN-specific positioning

On the need to specify such a scheme which would allow the NTN infrastructure to locate a UE without GNSS capability, 19 organizations expressed their views:

- 9 (Thales, Nokia, Vodafone, Asia Pacific Telecom, Fraunhofer, Sony, HNS, Intelsat, Nomor) organizations consider that this should be defined as part of the Rel-17 NWI NTN, possibly taking into account Rel-16 NR positioning

- 10 (Samsung, Mediatek, ESA, Sierra Wireless, CATT, Intel, Huawei, Novamint, ZTE, Ericsson) organizations do not consider that it is necessary to have it defined as part of the Rel-17 NWI NTN but recognize that it is an important feature

Observation 15 (Phase 1): NTN based positioning is considered an important feature.

Proposal 5 (Phase 1): Rel-17 NR-NTN NWI should include the study of NTN based positioning to prepare a normative phase

# Phase 2 NTN scenarios down-selection and establishment of WID

A tentative ranking of NTN scenarios (including options) will be proposed by the moderator taking into account

•	the number /level of impacts w.r.t. existing  specifications.

•	the stakeholders interest

It should also consider that some feature/adaptations, may support several NTN scenarios.

All organizations will be invited

- to consolidate this ranking of NTN scenarios and contribute to this down-selection.

- to consolidate the NWI objectives for the selected NTN scenarios and related RAN features

## Down selection of NR based NTN scenarios and establishment of WID

Proposal 1 (Phase 1): In first priority, Rel-17 NR-NTN NWI should include the specification of a transparent payload based LEO scenario addressing at least 3GPP class 3 UE without GNSS capability and with both Earth fixed & moving beamfoot print. This will also support HAPS based scenarios.

Proposal 2 (Phase 1): In second priority, Rel-17 NR-NTN NWI should also include the specification of a transparent payload based GEO scenario addressing other than 3GPP class 3 UE with GNSS capability.

Proposal 5 (Phase 1): Rel-17 NR-NTN NWI should include the study of NTN based positioning to prepare a normative phase

### Question Ph2.1: What are the views on the proposed ranking and down selection of NR based NTN scenarios ?

| Organization | Question Ph2.1 : Agree/not agree | Comments |  |
| --- | --- | --- | --- |
| QCOM | Agree (with suggestions) | The first and the second priorities should be combined as one : Rel-17 NR-NTN NWI should include the specification to support the following scenarios: - Transparent based LEO scenario addressing at least 3GPP class 3 UE without GNSS and with both Earth fixed & moving beam foot print. This is expected to also support HAPS based scenarios. - the specification of Transparent based GEO scenario addressing other UE with GNSS. |  |
| Ericsson | Partially agree | We have concerns on the large specification effort required to complete all these in Rel-17. The scenarios need to be further down-scoped. - Our preference is to prioritize UE with location capability (e.g. GNSS) in Rel-17. - Our preference is to focus on one option and avoid addressing both steerable beam and fixed beam in one release. - We do not see a need to address NTN specific positioning in Rel-17. Rather, we should use the valuable TUs in Rel-17 to focus on introducing core features to support NR based NTN. |  |
| ZTE | Agree (with suggestion) | Focusing on the prioritized scenarios is preferred. Proposal-1 for the LEO (1st priority) can be considered for future normative work. Flexibility and scalability should also be highlighted on the introduction of corresponding enhancements to support more scenarios. W.r.t the NTN-specific positioning, postponing is still preferred due to the unclear scope. |  |
| Samsung | Partially agree | Agree that LEO should be prioritized. However, for positioning, even though UE location awareness is an important feature, it is only an assumption and it doesn’t mean we have to study the positioning schemes. |  |
| Panasonic | Agree (with suggestions) | - We agree with Qualcomm that the first and the second priorities should be combined as one. - In addition, proposal 1 should include both without and with GNSS. We are not sure the table and Observation 6 in section 1.2 reflect the situation, because ”UE without GNSS” and ”UE with GNSS” have similar number of votes according to the companies’ responses. |  |
| Huawei/HiSilicon | Partially agree | We are OK with the first part of Proposal 1 “In first priority, Rel-17 NR-NTN NWI should include the specification of Transparent based LEO scenario addressing at least 3GPP class 3 UE”. There are some suggestions: - The support of both earth fixed beam and moving beam seems difficult within a release. Selecting one for normative work would be better. - From specification impact perspective, it seems to us that the work for transparent LEO can enable HAPS-based scenario. If it is not the case, clarification should be provided early has possible. - For Proposal 2, we are in general OK from network perspective. While for UE side, better to check additional specification impact on top of Proposal 1. - We recommend Proposal 5 to be handled by later releases. |  |
| Eutelsat | Agree | 1st Priority LEO transparent, Fixed beam |  |
| Fraunhofer IIS/HHI | Agree (with suggestions) | We agree with proposals 1 and 3. Proposal 2: To enable early deployments of NR over satellites, transparent GEO shall be specified within Rel-17 as well with same priority as LEO transparent. |  |
| Sony | Agree (with suggestions) | Proposal 1 should also include UEs with GNSS capability for the LEO scenario. We think LEOs and HAPS represent the best opportunity for NTN to serve smartphones which, by and large, have GNSS onboard. A positioning capability in UEs will be of enormous help in minimising changes to NR. The use of GNSS instead of NTN-specific positioning will also minimise effort for this first NTN WI. Proposal 2 as second priority is fine. We think the study of NTN network-based positioning as part of the proposed WI will require too many TUs. It should be left to a future WI that follows the proposed Rel17 positioning enhancements WI. |  |
| MediaTek | Agree with proposals | Proposal 1 with highest priority. |  |
| Ligado | Partially agree | We agree that both transparent GEO and transparent LEO priorities should be combined, as Qualcomm suggested above. NTN solutions should address the most challenging use cases. For GEO, timing and HARQ are worst case. For LEO, Doppler and mobility are worst case. NTN work should address the worst cases across both GEO and LEO scenarios. |  |
| Inmarsat | Agree with suggestions | In line with QCOM comments, we propose to combine first and second priority as one. Furthermore, Rel 17 work should focus on transparent payload type only to minimise the effort. In specific NWI should focus on: - Whilst it would be good for the scope to include both steerable and fixed cells. To maximise use of TUs, we could however considering supporting Ericsson proposal to limit it to fixed cells only - Priortise UE with GNSS capability (in line with Ericsson proposal) Inline with Samsung comments, we do not necessarily need to study positioning schemes (proposal 5) |  |
| Thales | Agree (with suggestions) | We agree with Qualcomm suggestions. We also agree with ZTE on LEO (1st priority) and the Flexibility and scalability that should also be highlighted on the introduction of corresponding enhancements to support more scenarios. W.r.t the NTN-specific positioning, it is a necessary feature for the RAN infrastructure to localise UE without GNSS. It can be defined using techniques TDOA/FDOA techniques have proven to be efficient in LEO constellation systems for RAN based location determination and should be considered when addressing trusted and untrusted methods from regulatory services point of view. |  |
| Intelsat | Partially agree | we propose to combine GEO transparent and LEO transparent as one priority. Additionally, to include HAPS transparent and Regenerative as minimal effort is required to address requirements in Release-17. Limit to fixed cells only to minimize the effort. |  |
| SoftBank | Agree (with suggestion and comment) | We would apply “This will also support a certain HAPS based scenario, which have the same technical challenges as LEO” to avoid a misreading such that the functionalities here are necessary & sufficient condition for HAPS based operations. Regenerative is also an important scenario for HAPS. The consequence not having this in Rel-17 should be carefully analysed. |  |
| CTTC | Agree (with suggestion) | Agree on giving priority to LEO. Proposal 1 should consider that GNSS based techniques are applicable at the UE side. |  |
| III | Agree (with suggestion) |  | As MTK’s suggestion. Proposal 1 with highest priority. |
| Nokia | Mainly agree. |  | Proposal 1: We agree, except that explicit support of Earth-fixed beam footprint should not be needed: by focusing on earth-moving beam footprints, sufficient support earth-fixed beams will be available by default; We also think that the proposed rewording of Observation 8 by Softbank describes the situation well regarding HAPS. Hence the proposal can be modified as follows, in order to avoid unnecessary extra work and duplication of solutions: Proposal 1: In first priority, Rel-17 NR-NTN NWI should include the specification of Transparent based LEO scenario addressing at least 3GPP class 3 UE without GNSS with Earth-moving beam footprint. This will also support HAPS based scenarios and Earth-fixed beam footprint without additional specification effort. Proposals 2 & 5: We agree. |
| ESA | Agree |  | Proposal 1 and 2 should be combined. |
| TNO | Agree (with suggestion) | In case further downselecting is needed, we would like to focus on fixed earth beam. Also proposal 5 is of lower prioritity |  |
| Avanti | Partially agree Agree with QCom and Ligado | We support QCom’s proposal to combine GEO transparent and LEO transparent with fixed beams as priority one. Additionally, the proposal to include HAPS transparent and Regenerative as minimal effort is to address requirements in Release-17 is reasonable. |  |
| Hughes Network Systems (HNS), an EchoStar company | Partially agree | As many have suggested above, we would like that both transparent GEO and NGSO be high on priorities, so GEO and LEO transparent should be combined as priority 1. Both GEO and NGSO have their challenges - timing and RTD for GEO and doppler and mobility for NGSO, NTN WI work should address at a minimum, the fundamental needs across both GEO and NGSO scenarios. We agree with Dish that NTN solutions should address the most challenging use cases keeping in mind that many satellite operators deploy GEO. NTN UE positioning is a necessary feature, the capability shall be supported as part of Rel-17. |  |
| DISH Network | Agree with QCom and Ligado | Transparent GEO and transparent LEO should be combined as high priority since many of the satellite operators use GEO. For NTN to be successful, NTN has to enable them to provide basic connectivity using NTN. |  |
| NOVAMINT | Agree | Agree that first priority is LEO transparent. However, fixed beam should be the focus for R17. It should be clear that proposal 5 is of lowest priority |  |
| Mitsubishi Electric | Agree with the merger of Proposal 1 and 2 | We also support the merger of Proposal 1 and 2. |  |
| Loon | Partially agree | We agree with Softbank that certain HAPs based scenarios are similar to LEO satellites. We believe that regenerative payloads and slowly moving beams are particularly relevant to HAPs We believe that the HAPs use cases has not been studied adequately in the Rel-16 NTN study item even though it was included in the objectives of Rel-16. We believe a small study phase dedicated to HAPs NTN scenarios would be useful. |  |
| Intel Corporation | Agree with proposal on NTN scenarios | Description of scenarios should also include considered bands. S-band was assumed for handheld UE and Ka-band was assumed for other type of UE in the SI phase, we propose to clarify that explicitly in the scenario description for the new WI. Regarding NTN-based positioning, we don’t see urgency to consider it in Rel. 17. If NTN-based positioning is introduced, the scope should be included under Rel. 17 NR positioning enhancements item. |  |
| Asia Pacific Telecom (APT) | Agree, but | - For Proposal 1, if further down-scoping is needed, we prefer to focus on transparent LEO-600 km with a 3GPP UE of class 3, GNSS and Earth moving beam scenarios. - For Proposition 5, it is an enhancement, not essential based on our understanding. Its priority shall not be greater than that of Proposal 2. |  |
| Nomor Research GmbH | Agree | Proposal 1 and 2 should be combined. In case further down selection is needed, we would like to focus on fixed earth beams. In that case also proposal 5 should be of lower priority. |  |
| Sequans | Partially agree | Can be agreed with the following modification: 1. transparent based GEO and transparent based LEO are both considered as first priority. |  |
| Leonardo | agrees Proposal 1 as first priority | NR based NTN scenario  LEO & GEO both prioritised  Transparent prioritised  Cell size of 1000 km and greater |  |
| CATT | Partially agree | In the proposal 1, we don’t understand why only 3GPP class 3 UE without GNSS is mentioned as the first priority. In our view, instead, the UE with GNSS positioning capability should be prioritized. Also, we do not see a need to address NTN specific positioning in Rel-17. |  |
| Sierra Wireless | Mostly agree |  |  |

### Question Ph2.1: Synthesis

A total of 33 organisations have responded:

- Qualcomm, Ericsson, ZTE, Samsung, Panasonic, Huawei, Eutelsat, Fraunhofer, Sony, Mediatek, Ligado, Inmarsat, Thales, Intelsat, Softbank, CTTC, III, Nokia, ESA, TNO, Avanti, HNS, Dish network, Novamint, Mitsubishi, Loon, Intel, , Asia Pacific Telecom, Nomor Research, Sequans, Leonardo, CATT, Sierra Wireless

Ranking of NR NTN scenarios: Responses on Proposals 1, 2 and 5 on the are distributed as follow:

- Agree:  5 (Eutelsat, ESA, Novamint, Intel, Sierra Wireless)

- Agree with modifications: 28

- Not agree: -

- Agree + Agree with modifications = 32/32 = 100%

Among the proposed modifications

LEO & GEO: 17 organisations suggested to merge proposal 1 & 2 so that both LEO and GEO be considered in the Rel-17 NWI NTN.

About beams in LEO

- Earth fixed beams: 6 (Eutelsat, Intelsat, TNO, Avanti, Novamint, Thales)

- Earth moving beams: 3 (Nokia, Loon, Asia Pacific Telecom)

- Either Earth fixed or moving beams: 2 (Ericsso, Huwaei)

- Earth fixed and moving beams: 1 (Inmarsat)

About NTN-based positioning

- Agree to prioritise: 4 organisations (Thales, Fraunhofer, HNS, Nokia)

- Postpone: 9 organisations

UE

- without GNSS capability in LEO: 4 (Thales, Qualcomm, Panasonic, Nokia) organisations explicitly raised the importance to consider it in priority

- with GNSS capability in LEO: 7 organisations explicitly raised the importance to consider it in priority

HAPS: 4 organisations (Loon, Softbank, Avanti, Intelsat) sugested to consider HAPS with regenerative payloads in the Rel-17 NWI NTN.

- of them (Loon, Softbank) suggested that a study phase be incorporated in the NWI to address HAPS scenarios which were not properly addressed during Rel-16 NTN SI

Furthermore, 1 organisation (Intel) proposed to clarify the type of targeted UE type (e.g. class 3 UE for FR1 bands and other UE for Ka band

From the above

Observation 1 (Phase 2): Addressing GEO in addition to LEO scenarios (1st priority) will provide the flexibility for the standard to support more NTN scenarios.

Observation 2 (Phase 2): Most organisations propose to address both LEO and GEO scenarios with transparent payload

Observation 3 (Phase 2): Addressing LEO/Earth moving cell scenarios will provide the flexibility for the standard to support LEO & HAPS/Earth fixed cell scenarios & HAPS/Earth moving cell scenarios.

Observation 5 (Phase 2): Most organisations consider that the NTN Network based positioning feature requires further study before the normative phase.

Observation 6 (Phase 2): Addressing UE without GNSS capability in LEO scenarios provide flexibility to support both UE without/with GNSS capability

Observation 7 (Phase 2): Some HAPS scenarios (Regenerative, slow moving beams) require further study before normative phase

Proposal 1 (Phase 2): Rel-17 NR-NTN NWI should include two activities

- Normative activity on NR-NTN to develop specifications to support the following scenarios:

  - Transparent payload based LEO scenario addressing at least 3GPP class 3 UE with & without GNSS capability and with both Earth fixed & moving beam footprint.

    - Note: Addressing LEO will provide the flexibility to also support transparent payload based HAPS based scenarios.

  - Transparent payload based GEO scenario addressing other UE (Non class 3 UE) with GNSS capability.

- Study activity on NTN scenarios addressing

  - Network based UE positioning

  - Regenerative payload based HAPS with Earth moving beam scenarios targeting Class 3 UE

Organisations are invited to review the above proposal and provide comment if any.

| Organization | Comments |
| --- | --- |
| CATT | Regarding the Transparent payload based LEO scenario, we don’t understand why the 3GPP class 3 UE without GNSS capability is highlighted. If for fair selection, all the UE types with and without GNSS capability should be considered. Moreover, in view of less standardization efforts and demonstrated performance, the UE with GNSS capability should be prioritized obviously. In the later stage, we can further optimize the UE without GNSS capability if complete study is finished. |
| Vodafone | We agree with the work indicated in Proposal 1 and in our opinion should be studied first |
| Panasonic | We suggest that transparent LEO should include both without and with GNSS capability, because both don’t have so different number of votes according to the companies’ responses. If it is required to select one due to potential work load restriction, we prefer to prioritize UE with GNSS because less standardization effort is needed. |
| Turkcell | We support the work indicated in Proposal 1. We agree with CATT for the prioritization of UE with GNSS capability. This prioritization may decrease standardization efforts and enhance demonstration performance. |
| OPPO | We don't think the NTN-based positioning is needed in Rel-17 due to the unclear scope. We also don't see a need to study HAPS scenario in Rel-17. |
| CMCC | We think that UE with GNSS capability should have higher priority from the perspective of standard development. The UE without GNSS capability could be considered afterward if time allows before Rel-17 freezing. |
| Inmarsat | We agree with Proposal 1 and that UE with GNSS capability should be high priority from standardisation development perspective. We do want to comment that there is no requirement to associate a satellite payload to a UE category. From standards perspective both LEO and GEO should be able to support Class 3 as well as other non-Class 3 UE – link budget permitting. So in summary: Transparent LEO should support Class 3 and non-Class 3 UEs and Transparent GEO should support Class 3 and non-Class 3 UEs. |
| Intelsat | We agree with Proposal 1 and that UE with GNSS capability. Additionally, we believe Regenerative HAPS requires limited modifications and therefore would support SI followed by NW within the Rel 17. |
| Loon | We mostly agree with proposal. However, we believe that HAPs coexisting on the same frequency as terrestrial gNB and regenerative HAPS can be included as a short study phase leading to normative phase within Rel-17. |
| III | Agree with proposal 1. Transparent payload based LEO/GEO scenarios. UE with GNSS is preferred. |
| Ericsson | For the normative activity, we have concerns on the large specification effort required to complete all these in Rel-17. The scenarios need to be further down-scoped. - Our preference is to prioritize UE with location capability (e.g. GNSS) in Rel-17. - Our preference is to focus on one option and avoid addressing both steerable beam and fixed beam in one release. For the study activity - Regarding NTN-specific positioning, a large number of companies expressed that NTN-specific positioning should not be part of Rel-17. We support this view. We should use the valuable TUs in Rel-17 to focus on introducing core features to support NR based NTN. |
| Intel Corporation | On the “normative activity”, we are fine to consider the listed scenarios. On the “study activity”, we do not see urgency to consider network based UE positioning for NTN in Rel. 17. If introduced, the scope should be included under Rel. 17 NR positioning enhancements item. |
| Nokia | We agree with the proposal. Note that we do not believe that Observation 4 is valid, nor is such an observation pertinent here. |
| Sony | We support a Rel-17 NR-NTN WI with only one activity: Normative activity on NR-NTN to develop specifications based on the scenarios as indicated in Proposal 1. Regarding positioning, UEs with GNSS capability should be prioritized as Rel16 MDT framework can be reused for this. For UEs without GNSS capability, we can support a study of Rel16 positioning methods and any further enhancements under study in RAN3. |
| SoftBank | We agree with the proposal. |
| Eutelsat | We agree on the normative phase |
| Fraunhofer | We agree with proposal 1 |
| Huawei/HiSilicon | From Phase I email discussion, more companies are in favour of fixed beams than companies in favour of moving beams. Taking both companies’ views and standardization impact into consideration, it seems reasonable to focus on earth fixed beam in this release. Likewise, it would seem reasonable to focus on UEs with GNSS support. For Release-17, we did not see the urgency for the listed study activities in Proposal 1 (positioning and regenerative HAPS) |
| Novamint | We agree with normative phase as outlined by proposal 1 |
| ZTE | In the normative work, the recommendation from WGs related to the scenarios defined in proposal-1 should be baseline for WI scoping e.g., support UE with/without GNSS. W.r.t the HAPS, not additional study activity is need according to the existing investigation. After the completion basic framework of NTN solution, additional enhancement can be considered for HAPS if needed. For positioning, corresponding work should be postponed due to the limited study as well as TU consideration for normative phase. |
| Avanti | We support Proposal 1 and the consideration of UE with GNSS capability for GEO (observing that fixed location non class 3 UE may be able to rely on occasional use of external GNSS capability) |
| MediaTek | We agree with proposal 1 |
| Thales | We agree with proposal 1. |
| Orange | We agree with proposal 1 with priority given to transparent payload LEO with direct access to Class 3 UE to address rural mobile broadband with conventional smartphones. |

A total of 24 organisations have responded:

- CATT, Vodafone, Panasonic, Turkcell, Oppo, CMCC, Inmarsat, Intelsat, Loon, III, Ericsson, Intel, Nokia, Sony, Softbank, Eutesat, Fraunhofer, Huawei, Novamint, ZTE, Avanti, Mediatek, Thales, Orange

About their views on Proposal 1 (Phase 2) for Rel-17 NR-NTN NWI

- Agree:  10 (Vodafone, Panasonic, Turkcell, Nokia, Softbank, Fraunhofer, Novamint, Mediatek, Thales, Orange)

- Agree with modifications: 14

- Not agree: 0

- No response:  2 (Qualcomm, HNS)

Among the 14 organisations that agree with modifications to the proposal :

- 11 organisations (CATT, Panasonic, Turkcell, CMCC, Inmarsat, Intelsat, III, Ericsson, Sony, Huawei, Avanti) explicitly prioritise UE with GNSS capability

- 2 organisations (Intelsat, Loon) explicitly asked for study and normative phase on HAPS as part of the NWI

- 2 organisations (Oppo, ZTE) consider thatHAPS study is not needed

- 2 organisations (CATT, Inmarsat) recommend that not only 3GPP class 3 UE be considered

- 1 organisation (Ericsson) recommends that one option among steerable/fixed beams be considered for LEO

- 1 organisation (Huawei) recommends that steerable beams be considered for LEO

About the study on NR-NTN  based network positioning see responses in clause 3.4.1

## Down selection of NB-IoT/LTE-MTC based NTN scenarios and establishment of WID

Proposal 3 (Phase 1): NB-IoT/LTE-M based NTN scenarios should be considered as part of a separate Rel-17 NWI  eMTC-NTN

Proposal 4 (Phase 1): Rel-17 eMTC-NTN NWI should include the specification of transparent payload based LEO & GEO scenario addressing 3GPP class 3 UE with/without GNSS capability and with Earth fixed & moving beam footprint.

### Question Ph2.2: What are the views on the proposed ranking and down selection of NB-IoT based NTN scenarios and of LTE-MTC-based NTN scenarios? Answers shall be distinguished between NB-IoT and LTE-MTC.

| Organization | Question Ph2.2 : Agree/not agree | Comments |
| --- | --- | --- |
| QCOM | Not agree | To be realistic, proposal 3 is equivalent to proposing a Rel-18 WI on NTN based IoT. If the intention is to have both NB-IoT and eMTC, suggest to call it IoT-NTN. To decide on the deployment scenarios to be supported, further study on use cases, requirements, and feasibility is needed. |
| Ericsson | Not agree | A large number of companies expressed not supportive of a Rel-17 WI on NB-IoT/LTE-M based NTN. It is not justified to propose NB-IoT/LTE-M based NTN in a Rel-17 WI. |
| ZTE | Not agree | According to the statistics shown in section 1.1, the deprioritization of NTN based NB-IOT/LTE-M is preferred by the majority. Without justification and solid study on the basic issues, it’s immature to include such part into normative work in Rel-17. |
| Samsung | Not agree | NB-IoT/LTE-M has rarely been addressed in the SI stage. In addtion, we need study two RATs (LTE and NR) if NB-IoT/LTE-M are included, which imposes addtional challenges. Thus we think it should not be included in Rel-17. |
| Panasonic |  | We should take care of how to proceed Rel-17 NR & eMTC-NTN NWI in parallel, even if proposals 3 and 4 are adopted. |
| Huawei/HiSilicon | Not agree | For proposal 3, it is immature to start NB-IoT/eMTC normative work in Rel-17 directly. NB-IoT/LTE-M based NTN scenarios should be studied before any normative work. For Proposal 4, what to be specified depends on the outcome of the possible study item. |
| Eutelsat | Fully -agree | First, more than 18 companies shown interest on having in release 17, NB-IoT and LTE-M via satellite. This has been shown within the email discussion NB-IoT and eMTC enh in August and also during this offline discussion NTN in phase 1. Second, some simulation at least for NB-IoT has been done during the NTN Study Item release 16 phase showing that NB-IoT can easily leverage on the NR results. The conclusion of the NTN NR study item are applicable for NB-IoT and LTE-M. We strongly support to have NB-IoT and LTE-M part of release 17 integrating a small part of study if some companies think that a study is necessary to prepare a normative phase for R17, even if we are convienced that we can easly leverage on NTN NR study conlusion. |
| Fraunhofer IIS/HHI | Agree with modification | Proposal 3: NB-IoT should be prioritized vs. LTE-M Proposal 4: The term eMTC is in our understanding equivalent to LTE-M and might be misleading, because we assume that proposal 4 comprises NB-IoT and LTE-M. The term by Qualcomm IoT-NTN might be better: « Rel-17 IoT-NTN NWI should include… » |
| Sony | Agree with modification | We think at least a study of NB-IoT and LTE-MTC on NTN should be included in the WI. If down-selection is needed, we suggest to prioritise LEO scenarios with moving beams. |
| MediaTek | Agree with proposals | We see NB-IoT NTN as a low hanging fruit with relatively less impact on specifications than NR-NTN. Many aspects of NR _NTN SI have synergies with NB-IoT NTN. |
| Ligado | Fully Agree | There is significant business interest in providing IoT services via satellite. Adaptation of LTE-M and NB-IoT for NTN is technically very feasible. As we suggest for NR, both LEO and GEO scenarios should be addressed to solve the worst-case problems. We support both LTE-M and NB-IoT Work Items. We concur with Eutelsat that the work in the NR NTN Release 16 Study Item is applicable to the adaptation of LTE-M and NB-IoT. We note that the majority of operational satellite constellations today are GEO so there is an immediate business need to address GEO scenarios, especially for satellite IoT services. |
| Inmarsat | Agree with suggestions | We believe the modification required should be manageable if the scope is limited to 1) GEO, 2) fixed cells and NB-IoT only (excluding LTE-M). One possible approach could be to run short SI on NB-IoT in parallel and then the outcome of SI may be progressed into NB-IoT NWI within Rel 17. |
| Thales | Agree (with suggestions) | We believe that the “IoT-NTN” topic may be addressed during Rel-17 starting with a study item. Both NB-IoT & LTE-M may be considered. For “IoT-NTN”, LEO & GEO transparent scenarios should be addressed in priority. |
| Intelsat | Agree (with suggestion) | We believe the effort can be manageable if the scope is focused on transparent GEO, fixed cells and NB-IoT to minimize the work effort. |
| CTTC | Agree (with suggestions) | We propose to give priority to NB-IoT over LTE-M. A more in-depth feasibility study on the application of NB-IoT to NTN should be included in the WI. |
| Nokia | Not agree | If NB-IoT and/or LTE-M based NTN scenarios were to be considered, they should indeed be in a separate WI from NR NTN work. However, considering the substantial extra work to address NB-IoT and/or LTE-M based NTN scenarios, the fact that these have not yet been studied, and the responses from the first phase of the email discussion, it does not seem appropriate to start work on these topics at this stage. |
| ESA | Agree | Fully agree with the Proposal 3. There is a strong business interest and it is confirmed by many 3GPP companies in favour of the inclusion in Release 17. The needed effort is expected to be limited. If still some concern remains due to the lack of a dedicated study phase, we support the choice to have first a short study item followed by the specification stage. |
| TNO | Agree (with suggestions) | Both NB-IoT and LTE-M should be considered. Focus on fixed earth beam (same solution as NR work). IoT seems to be an early opportunity for NTN, with a lot of interest from various market players. |
| Avanti | Agree (with suggestion) | We support the scope where it is focused on transparent GEO, fixed beams and NB-IoT to focus the work effort. |
| Hughes Network Systems (HNS), an EchoStar company | Agree | We concur with many that there has been significant business interest in providing IoT services via satellite. We strongly support both LTE-M and NB-IoT Work Items in Rel-17. Adaptation of LTE-M and NB-IoT for NTN is technically very feasible and we concur that the work in the NR NTN Release 16 Study Item can be applicable to the adaptation of LTE-M and NB-IoT. GEO and NGSO transparent scenarios should be addressed in priority. |
| DISH Network | Not agree | We concur IoT to be in WI of NTN Rel-17 due to notable business interest for satellite operators |
| NOVAMINT | Fully Agree | Many verticals (Transport, logistics, Utilities…) have expressed the importance and the urgency of providing IoT NTN integrated in 3GPP technology. Some use cases around asset Tracking where IoT Sat is particularly relevant (covering sea, white zone, deep rural…) have been already described in TS 22.836. Timing is the essence to serve such needs – already release 17 means readiness of the ecosystem in best case by the end of 2021. Not doing it in Release 17, will make IoT/mMTC NTN only available by 2023 which will be definitely too late and which definitely see adoption of other solutions on a large scale (2021 = key year and likely to see massive constellations launches). 3GPP cannot afford to miss such opportunity as it represents billions of devices. We agree with Mediatek that NB-IoT NTN is a low hanging fruit with relatively less impact on specifications than NR-NTN. Therefore, although it is desired to have both NB-IoT and LTE-M NTN solutions available at the same time, we believe we should focus the effort on NB-IoT for R17 and then LTE-M for R18 (based on what already designed for NB-IoT as it has been done in other contexts). If there is a study phase, it should be short (as at least for NB-IoT many elements are there) in order to achieve completion of the work for NB-IoT at minimum by the end of R17. In term of semantic, we agree with Fraunhofer that eMTC means LTE-M. We would suggest then to use the term mMTC (massive MTC) rather than IoT which also includes home IoT, industrial IoT, NR light… and to rephrase Proposal 3 as such: “NB-IoT/LTE-M based NTN scenarios should be considered as part of a separate Rel-17 NWI mMTC-NTN” |
| Intel Corporation | Not agree | The scope of the eMTC-NTN WI is not clear since NB-IoT and LTE-MTC was not considered in Rel. 15 NTN SI. Rel. 17 SI for NTN-based NB-IoT and LTE-MTC is needed instead of WI. |
| Asia Pacific Telecom (APT) | Not sure | - Not sure we should discuss the working scope for another new WI eMTC-NTN, if established in Rel-17. - In Proposal 3, the companies have excluded the possibility that NTN based on NB-IoT/LTE-M is part of NTN WI's objective. Based on this, further discussion, for example, whether it is a new WI or SI in Rel-17, should be taken in the NB-IoT/LTE-MTC group. |
| Nomor Research GmbH | Not agree | Too much effort to do NR-NTN, NB-IoT- NTN and LTE-M-NTN in a single release. |
| Sequans | Agree w/ modifications | There are many practical applications eager to have IoT NTN support yesterday and we need to find the right place to incorporate this work in Rel-17. it is also quite common view that a study phase is required for IoT NTN. If to place this study phase within the NTN WI, the IoT WIs or a separate WI could be discussed. Availability of IoT experts for the online discussions in RAN1 and RAN2 should be considered for this decision |
| Leonardo | Agrees Proposal 4 as first priority | We prefer Earth fixed rather than moving footprint  Earth fixed beam footprint  Support of UEs with GNSS prioritised |
| CATT | Not agree | In this stage, we don’t see the justification in Rel-17 to promote NTN IoT standardization; on the contrary, there are many open issues to be investigated. |
| Sierra Wireless | Agree with modification | As mentioned above, this work is well supported so should be considered for release 17. Mostly agree but we could start with a short study period. But prioritizing LEO makes sense as this would require far fewer physical layer changes. LTE-M should have same priority as NB-IOT. |

### Question Ph2.2: Synthesis

A total of 23 organisations have responded:

- Qualcomm, CATT, Vodafone, Panasonic, Turkcell, Oppo, CMCC, Inmarsat, Intelsat, Ericsson, Intel, Nokia, Sony, HNS, Eutesat, Fraunhofer, Huawei, Novamint, ZTE, Avanti, Mediatek, Thales

- Qualcomm, Ericsson, ZTE, Samsung, Panasonic, Huawei, Eutelsat, Fraunhofer, Sony, Mediatek, Ligado, Inmarsat, Thales, Intelsat, Softbank, CTTC, III, Nokia, ESA, TNO, Avanti, HNS, Dish network, Novamint, Mitsubishi, Loon, Intel, Asia Pacific Telecom, Nomor Research, Sequans, Leonardo, CATT, Sierra Wireless

Ranking of IoT NTN scenarios: Responses on Proposals 3 and 4 on the are distributed as follow:

- Agree:  5 (Eutelsat, Ligado, ESA, HNS, Novamint)

- Agree with modifications: 12 (Fraunhofer, Sony, Mediatek, Inmarsat, Thales, Intelsat, CTTC, TNO, Avanti, Sequans, Leonardo, Sierra Wireless)

- Not agree: 11 (Qualcomm, Ericsson, ZTE, Samsung, Huawei, Nokia, Dish network, Intel, Nomor Asia Pacific Telecom, CATT)

- No opinion: 1 (Panasonic)

Among organisations that do not agree to to address IoT-NTN scenarios in Rel-17, most indicate that IoT-NTN requires to be studied before entering in a normative phase.

Among the 16 organisations that agree with/without modifications to the proposal to address IoT-NTN scenarios in Rel-17.

- 9 organisations (Sony, Eutelsat, Inmarsat, Thales, ESA, HNS, Novamint, Sequans, Sierra Wireless) explicitly agree that a study phase could be carried out before the normative phase start within Rel-17

- 7 organisations (Franhofer, Mediatek, Intelsat, Inmarsat, CTTC, Avanti, Novamint) explicitly consider that NB-IoT in NTN should be prioritised, while the rest consider that both NB-IoT & LTE-M via satellite scenarios should be considered with equal priority.

- 5 organisations (Inmarsat, Intelsat, TNO, Avanti, leonardo) explicitly prioritise Earth Fixed cell scenario for IoT-NTN scenarios.

- 3 organisations (Intelsat, Inmarsat, Avanti) explicitly prioritise GEO while 1 organisation (Sony) explicitly prioritise LEO

Observation (Phase 2): About whether NB-IoT/LTE-M based NTN scenarios should be considered as part of a normative phase in Rel-17

- 17 organisations agreed (Agree + Agree with modifications) = 52%

- 11 organisations did not agree = 34%

Proposal 2 (Phase 2): Rel-17 should include a study activity on IoT-NTN scenarios (NB-IoT & LTE-M) addressing GEO and LEO which would leverage the Rel-16 NR-NTN SI.

Organisations are invited to review the above proposal and provide comment if any.

| Organization | Comments |
| --- | --- |
| Qualcomm | Regarding to NTN-IoT, Qualcomm now believes that we can have one combined WI to cover both eMBB and IoT. For IoT, NB-IoT is the first priority. |
| CATT | So far IoT-NTN performance and technology enhancements are not clear, if we are ambitious to develop the IoT-NTN, a separate study item can be considered in Rel-17. |
| Vodafone | In our opinion the NTN solution’s first priority should be the provisioning of mobile broadband for sparsely populated areas. The NB-IoT is second priority and should be studies later on, time permitting, once the mobile broadband solution has been delivered through Satellite systems. |
| Panasonic | Specification development for NR-NTN should be prioritized in Rel.17. We prefer to study IoT-NTN in later phase. |
| Turkcell | We agree that a study phase could be carried out before the normative phase start within Rel-17 for IoT-NTN. Our first priority is rural mobile broadband in NTN solution. But the number of user in IoT use cases/services can reach the number of eMBB user in a few years. IoT-NTN study phase should not postpone too much. |
| OPPO | We wonder if there is enough time to study IoT-NTN scenarios in Rel-17. We can study the NB-IoT & LTE-M use case in NTN in the later release. |
| CMCC | We think IoT-NTN is an interesting but unclear scenario for now. We should focus on the normative phase first. |
| Inmarsat | We believe the modification required should be manageable and support NTN IoT. We believe modifications (specifically with GEO) are limited and therefore would support SI followed by NW within the Rel 17. |
| Intelsat | We support NTN IoT. SI for the adaptation of NB-IoT and LTE-M for NTN transitioned into WI in Re-17 and, in particular GEO as the required modifications are minimal. |
| Ericsson | A study activity for IoT NTN must be needed before normative work. Regarding the timeline, our view is that it’s unlikely that there would be room in Rel-17 to accommodate such study activity, considering the large number of Rel-17 topics on the table. |
| Intel Corporation | NB-IoT/LTE-M based NTN scenarios would need to be studied with quite amount of TUs. Given limited amount of TUs, our preference is to prioritize to perform normative works based on Rel-16 NTN SI. |
| Nokia | A separate study of NB-IoT and/or LTE-M over NTN could be considered as a second priority after NR-NTN if the overall time situation for Rel-17 allows. |
| Sony | We agree that IoT-NTN can be studied in Rel17 but because of limited time, we should focus this WI on the NR-NTN normative work only. |
| Hughes | The studies for the adaptation of NB-IoT and LTE-M for NTN can be comfortably transitioned into WI in Re-17 especially for GEO as there need be no concern about mobility and Doppler. As a matter of fact, NB-IoT and LTE-M are tolerant of large latencies. |
| Eutelsat | The market demand and the existing and coming proprietary LPWA solution via satellite show enough that the timing is key here therefore we should have within release 17 IoT NTN, starting by a period of study followed by a normative text in R17. |
| Fraunhofer | We think IoT-NTN is important and should be studied. However, this should be studied in a study item different to NR-NTN. |
| Huawei, HiSilicon | We agree that a study would first be needed for NB-IoT/eMTC based NTN scenario before starting normative work. |
| Novamint | We should address IoT NTN within Release 17 as there is urgency to support many market demands. It should start with a study activity on IoT-NTN scenarios as part of the R-17 WID on NR NTN |
| ZTE | eMBB based solution should be certainly prioritized for normative work. For the NTN IOT part, feasibility/scenarios justification and issues identification should be done before the normative work. And in Rel-17, focusing on the feasibility/scenarios justification with limited SI phase may be one option if there is strong interest and TU budget. |
| Avanti | We support NTN IoT. SI for the adaptation of NB-IoT and LTE-M for NTN transitioned into WI in Re-17 and in particular GEO |
| MediaTek | We agree with proposal 2. We think NB-IoT is first priority for satellite IoT as it is relatively low hanging fruit compare to NR IoT, and there is strong business case for satellite IoT specification as IoT commercial services are mature in legacy satellite systems. A study item phase could be considered first early in Rel-17, with a normative phase later in Rel-17 depending on outcome of SI. We think 2 deployments should be supported: •NB-IoT with inband deployment in NR carrier re-using specified rel-16 NR coexistence mechanisms (same view as Thales) with fixed-earth beams. • NB-IoT in standalone deployment with moving earth beams. NB-IoT RAT is well suited for typical satellite IoT applications – low data rates for small intermittent packets, long latencies, low power consumption, low cost, low link budget, simpler operations (no CSI, no HO). We believe it is critical to ensure early specification of a specified satellite NB-IoT solution with minimun delta for specification and implementation compare to legacy NB-IoT. |
| Thales | We agree with proposal 2. The study should be limited in scope (For example identifying the potential issues associated to the applicability of NR-TN specified features to IoT-NTN). If the outcome of the study results in negligible normative effort, normative effort on IoT-NTN can be considered for Rel-17 |
| Orange | We agree with the proposal to study NTN-IoT (both NB IoT & LTE-M) in Rel.17 to complement the normative work done on Mobile Broadband and prepare future normative NTN-IoT work. |
| Sierra Wireless | Sierra feels a separate SI followed by normative WI for NTN-IoT is warranted and well supported and can have the same priority as NTN-NR. Most of the changes needed for NTN-IoT will be the same for NB-IOT and LTE-M so there is no need to prioritize one over the other – in fact this may slow progress. |

A total of 22 organisations have responded:

- Qualcomm, CATT, Vodafone, Panasonic, Turkcell, Oppo, CMCC, Inmarsat, Intelsat, Ericsson, Intel, Nokia, Sony, Eutesat, Fraunhofer, Huawei, Novamint, HNS, ZTE, Avanti, Mediatek, Thales)

About their views whether NB-IoT/LTE-M based NTN scenarios should be considered as part of Rel-17 NTN NWI

- Agree:  12 (Qualcomm, Inmarsat, Intelsat, Eutelsat, Fraunhofer, Huawei, Novamint, HNS, Avanti, Mediatek, Thales, Orange)

  - Of which 7 (Inmarsat, Intelsat, Eutelsat, Novamint, Avanti, Mediatek, Thales) explicitly propose to consider a study followed by normative activity as part of the Rel-17 if work is small

- Agree but with lower priority: 6 (Vodafone, Turkcell, CMCC, Nokia, Sony, ZTE)

- Not agree: 5 (CATT, Panasonic, Oppo, Ericsson, Intel)

## Rel-17 NTN-related work item(s)?

How to organize the work item(s) for NTN-related work will need to be discussed, especially considering the technical content and magnitude thereof will be different between RATs and the timescales could as a result be different as well.

- RAT-specific NTN work items i.e. distinct work items for NR, NB-IoT and LTE-MTC as necessary.

- Umbrella NTN work item covering NR, NB-IoT and LTE-MTC as necessary. In this case, objectives shall be distinguished between RATs. A separate building block could also be established for each RAT.

### Question Ph2.3: Preference ?

Organisations are invited to provide their opinion.

| Organization | Work item organization: RAT-specific NTN items versus Umbrella NTN work item | Comments |
| --- | --- | --- |
| CATT |  | If other techniques are really needed, it should be separate study item. |
| Panasonic | Prefer a) if IoT-NTN scenarios is studied. | In Rel.17, we prefer to prioritize NR-NTN WI. |
| Turkcell | Prefer a) if IoT-NTN scenarios is studied |  |
| OPPO |  | Please refer to our reply to Ph2.2. |
| CMCC | a) | Let’s focus on normative phase first. |
| Inmarsat |  | Refer to our reply 2.2 |
| Intelsat |  | See response to 2.2 |
| Loon | Umbrella NTN work item is our preference |  |
| Ericsson | RAT-specific NTN | Our view is that Rel-17 NTN WI should be focused on NR. |
| Nokia | Handling multiple RATs in the same item would cause confusion. | As noted above, a study of NB-IoT and/or LTE-M over NTN could be considered as a second priority after NR-NTN if the overall time situation for Rel-17 allows. |
| Sony | Prefer (a) | In Rel.17, prioritize NR-NTN WI. But we are also open to a separate IoT-NTN SI as a second priority. |
| Hughes Network Systems Ltd | Option (a) with flexibility for IoT-NTN. See comments | Distinct items for NR-NTN (NWI). For IoT-NTN package, common views amongst satellite operators – studies for the adaptation of NB-IoT and LTE-M for NTN can be comfortably transitioned into WI in Re-17 especially for GEO as there need be no concern about mobility and Doppler. As a matter of fact, NB-IoT and LTE-M are tolerant of large latencies. |
| Eutelsat | b) agree with modification | We agree to have within the same NTN WI, Building Block 1 for NR NTN, Building block 2 for IoT NTN (NB-IoT/LTE-M) with 2 phases for BB2 : starting by a study and completed within release 17 by a normative phase. |
| Fraunhofer | We prefer option a) |  |
| Huawei, HiSilicon | RAT-specific NTN if IoT-NTN scenarios are studied | The first question is how to scope the work on NTN for Rel-17. How to organize the work is secondary. But It is not 3GPP practice to mix feasibility studies and normative work in the same work item, and it is not 3GPP practice to mix independent work on different RATs in the same item. |
| ZTE | RAT specific is prioritized, final decision will be up to scope discuss | Since the NR eMBB is prioritized, the RAT specific is preferred. |
| MediaTek | b) | We have same view as EutelSat. We agree to have within the same NTN WI, Building Block 1 for NR NTN, Building block 2 for IoT NTN (NB-IoT/LTE-M) with 2 phases for BB2 : starting by a study and completed within release 17 by a normative phase. |
| Sierra Wireless | A – separate NTN-NR and NTN-IOT | Same priorities for NR, LTE-M, and NB-IOT |

Among the organisations that responded to this question

- 8 (CATT, Nokia, Sony, HNS, Fraunhofer, Huawei, ZTE, Sierra Wireless) recommend that the IoT-NTN be treated as part of a separate item within Rel-17

- 3 (Eutelsat, Novamint, Mediatek) recommend that the IoT-NTN be treated as part of same item as NR-NTN within Rel-17

Note: Final selection of work items for Rel-17 will be made at RAN#86

## NTN-specific positioning

### Question Ph2.4: Way forward on NTN-specific positioning ?

| Organization | NTN-specific positioning approach in Rel-17 | Comments |
| --- | --- | --- |
| Panasonic | Not support | We prefer to prioritize GNSS based positioning in Rel.17. NTN specific positioning should be considered in later phase. |
| OPPO |  | Please refer to our reply to Ph2.1. |
| CMCC |  | Refer to our reply to Ph2.1. |
| Inmarsat |  | Refer to our reply to Ph 2.1 |
| Intelsat |  | See response to Phase 2.1 |
| Ericsson | Not part of Rel-17. | A large number of companies expressed that NTN-specific positioning should not be part of Rel-17. We support this view. We should use the valuable TUs in Rel-17 to focus on introducing core features to support NR based NTN. |
| Intel Corporation | No NTN-specific positioning in Rel-17 | Please see our reply to Ph2.1. |
| Nokia | Could commence with a 3-month study phase within the WI before proceeding to normative work. |  |
| Sony | Not support | For Rel17 NTN WI, prioritize GNSS for positioning. For UEs without GNSS capability, we can support a study of Rel16 positioning methods and the further enhancements under study in RAN3. |
| Hughes Network Systems Ltd | Agree with Nokia |  |
| Fraunhofer |  | Positioning features that are required to support NTN for UEs without GNSS capability should be studied in the normative WI on NTN. However, positioning as a service like in the SI positioning should not be considered to be studied, at least for NTN NR WI. |
| Huawei, HiSilicon | Not part of Rel-17 | Positioning can be handled by implementation in Rel-17 time frame, e.g. GNSS can be supported by all terminals for example if there is a legal requirement for positioning |
| Novamint | Not part of Rel-17 | It is nice to have but should be deprioritized for Rel-17. |
| ZTE | Not part of Rel-17 | Deprioritize on this part in R17 is more reasonable. |
| MediaTek | Support | Same view as Nokia |
| Thales | Support for a small/short study | This feature is essential to entrust NTN for some regulatory services. A study could be carried out limited to the identification of possible solutions for network based UE location (with or without GNSS capability) |
| Sierra Wireless | Not support | NTN specific positioning should be considered in later phase/release |

11 organisations (Panasonic, Oppo, CMCC, Inmarsat, Intelsat, Ericsson, Intel, Huawei, Novamint, ZTE, SierraWireless), have explicitely requested that NTN secific positioning be de-prioritised.

6 organisations (Nokia, Sony, HNS, Fraunhofer, Mediatek, Thales), have explicitely recommended to start a SI on the NTN secific positioning as part of the Rel-17 NWI NR-NTN.

## Phase 2 summary

27 organisations provided a response to the survey:

- Qualcomm, CATT, Vodafone, Panasonic, Turkcell, Oppo, CMCC, Inmarsat, Intelsat, Loon, III, Ericsson, Intel, Nokia, Sony, HNS, Softbank, Eutesat, Fraunhofer, Huawei, Novamint, ZTE, Avanti, Mediatek, Thales, Orange, Sierra Wireless

The results can be summarised as follow:

| Candidate topics for Rel-17 NTN NWI normative activities - See Proposal 1 (Phase 2) in clause 3.1.2 | Way forward |
| --- | --- |
| Transparent payload based LEO scenario addressing at least 3GPP class 3 UE with & without GNSS capability and with both Earth fixed & moving beam footprint | Agreement, but with following modifications: - Most recommend UE with GNSS to be prioritised, but several organisations consider the support of UE without GNSS critical - Earth fixed &/or moving beam footprint scenario to be selected (depending SI outcome) |
| Transparent payload based GEO scenario addressing other UE (Non class 3 UE) with GNSS capability | Agreement but not restricted to other UE |

| Candidate topics for Rel-17 NTN NWI study activities - See Proposal 1 (Phase 2) in clause 3.1.2 & Proposal 2 (Phase 2) in clause 3.2.2 | Way forward |
| --- | --- |
| Regenerative payload based HAPS with Earth moving beam scenarios targeting Class 3 UE | Some concerns about the amount of work. The way forward proposal to restrict the study to transparent payload based HAPS scenarios and address only enablers for the spectrum sharing coexistence with cellular system (additional Coresets, PCI confusion mitigation, ..) |
| IoT-NTN scenarios | Strong interest from several organisations on starting a study. A minority have raised a concern that it may consume excessive bandwidth. The way forward proposal may be to restrict the study to the identification of scenarios/requirements & issues associated with the use of NR-NTN spec for NB-IoT/LTE-M to support NTN Most companies suggest to treat this topic in a separate item from NR-NTN |
| Network based UE positioning | Strong interest from a minority of organisations for this topic which will enable to meet regulatry requirements. Most consider that it can be de prioritised. The way forward proposal is to restrict the study to the identification of possible solutions for network based UE location. No new requirements will be considered. |

# Conclusion

On the basis of the responses, the following proposals can be elaborated (final decision at RAN#86):

Proposal (Overall): Rel-17 NR-NTN NWI should include two activities

- Normative activity on NR-NTN to develop specifications to support the following scenarios:

  - Transparent payload based LEO scenario addressing at least 3GPP class 3 UE with and without GNSS capability and both Earth fixed &/or moving cell scenario (as per SI outcome).

    - Note1: Addressing LEO will provide the flexibility to also support transparent payload based HAPS based scenarios.

  - Transparent payload based GEO scenario addressing UE with GNSS capability.

- Study activity on NTN scenarios addressing

  - Transparent payload based HAPS scenarios: Study of enablers for Spectrum coexistence with cellular (additional Coresets, PCI confusion mitigation, ..)

  - NTN-network based location of UE (for regulatory services): identify possible solutions

Proposal 2 (Overall): Rel-17 should include a IoT-NTN study

END

One scenario per row, possibly several row per organization

One scenario per row, possibly several row per organization

One scenario per row, possibly several row per organization

One scenario per row, possibly several row per organization

One scenario per row, possibly several row per organization

One scenario per row, possibly several row per organization