3GPP TSG-RAN WG1 Meeting #98bis	R1-1911364

# Chongqing, China, October 14-20, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1276 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on repetition number of MPDCCH with HARQ-ACK bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For a BL/CE UE in half-duplex FDD operation under CE mode A, if HARQ-ACK bundling is enabled and DCI field HARQ-ACK bundling flag is set to 1, the DCI field ‘DCI subframe repetition number’ is of 0 bit-length. As a result, the DCI subframe repetition number is not specified in this case. In RAN1#87 meeting, the following agreement on this issue was achieved: Agreement: If repetition is used for M-PDCCH or PDSCH, HARQ-ACK bundling is not used The PDSCH part has been captured in 36.213. However, the MPDCCH part has not been captured in 36.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For a BL/CE UE in half-duplex FDD operation under CE mode A and HARQ-ACK bundling enabled, the UE shall assume the number of MPDCCH repetitions as 1 if 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is ambiguity on the repetition number of MPDCCH and thus ambiguity on the starting subframe of PDSCH between eNB and UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

### 9.1.5	MPDCCH assignment procedure

<Unchanged parts in this subclause are omitted>

The number of MPDCCH repetitions is indicated in the 'DCI subframe repetition number' field in the DCI according to the mapping in Table 9.1.5-5. For a BL/CE UE in half-duplex FDD operation, if the UE is configured with CEModeA, and configured with higher layer parameter ce-HARQ-AckBundling, and 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1, the UE shall assume the number of MPDCCH repetitions as 1.

Table 9.1.5-5: Mapping for DCI subframe repetition number

| R | DCI subframe repetition number |
| --- | --- |
|  | 00 |
|  | 01 |
|  | 10 |
|  | 11 |

### < Unchanged parts are omitted >