3GPP TSG-RAN WG1 Meeting #98b	R1-1911366

# Chongqing, China, October 14th – 20th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1278 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to HARQ-ACK delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2019-10-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-14, the feature of HARQ-ACK delay was introduced. This feature is captured in Subclause 7.3.1. Subclause 10.2, however, still has a fix delay between PDSCH and HARQ-ACK. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in Subclause 10.2 that, if HARQ-ACK delay is applied, the HARQ-ACK timing is not N+4 for FDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.2	Uplink HARQ-ACK timing

<Unchanged parts are omitted>

For FDD, a BL/CE UE shall upon detection of a PDSCH intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response using the same  derived according to Subclause 10.1.2.1 in subframe(s) n+ki with i =0,1, …, N-1, where

-	subframe n-k is the last subframe in which the PDSCH is transmitted, where

if the UE is in half-duplex FDD operation and is configured with CEModeA and higher layer parameter ce-HARQ-AckBundling and the 'HARQ-ACK bundling flag' in the corresponding DCI is set to 1, or if the UE is configured with higher layer parameter ce-SchedulingEnhancement

-	k is given by the ‘HARQ-ACK delay’ field in the corresponding DCI, and the HARQ-ACK delay value k is determined based on the higher layer parameters according to Table 7.3.1-2.

otherwise

-	k=4

-	0≤k0<k1<…,kN-1 and the value of and  is provided by higher layer parameter pucch-NumRepetitionCE-format1, if configured, otherwise it is provided by higher layer parameter pucch-NumRepetitionCE-Msg4-Level0-r13, pucch-NumRepetitionCE-Msg4-Level1-r13, pucch-NumRepetitionCE-Msg4-Level2-r13 or pucch-NumRepetitionCE-Msg4-Level3-r13 depending on whether the most recent PRACH coverage enhancement level for the UE is 0, 1, 2 or 3, respectively; and

if N>1

-	subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s) immediately after subframe n-1, and the set of BL/CE UL subframes are configured by higher layers;

otherwise

-	k0 =0

<Unchanged parts are omitted>