3GPP TSG-RAN WG Meeting #99	R1-1913411

# Reno, USA, November 18th – 22nd, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1285 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for PUCCH resource determination with HARQ-ACK bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2019-11-14 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In HARQ-ACK bundling, the PUCCH resource determination is based on the last received MPDCCH. This is clarified for the ECCE determination (number of the first ECCE [...] in the last detected MPDCCH), but not for the final step in the case of 2+4 PRB sets. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the last step of determining the PUCCH resource for 2+4 PRB sets is also based on the last detected MPDCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is ambiguous on what MPDCCH to use to determine the PUCCH resource. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 10.1.2.1	FDD HARQ-ACK procedure for one configured serving cell

<Unchanged parts are omitted>

- for a BL/CE UE, and for a PDSCH on the primary cell where there is not a corresponding MPDCCH detected and subframe  is the last subframe in which the PDSCH is transmitted, the value of  is determined according to higher layer configuration and Table 9.2-2.

- for a PDSCH transmission indicated by the detection of a corresponding MPDCCH, or for an MPDCCH indicating downlink SPS release (defined in Subclause 9.2) where subframe  is the last subframe in which the PDSCH is transmitted, or for HD-FDD HARQ-ACK bundling, subframe  is the last subframe in which the PDSCH is detected, the UE shall use

-	if MPDCCH-PRB-set is configured for distributed transmission

-	if MPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set , or for HD-FDD HARQ-ACK bundling  is the number of the first ECCE (i.e. lowest ECCE index used to construct the MPDCCH) in the last detected MPDCCH used for transmission of the corresponding DCI assignment in MPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding MPDCCH as given in Table 10.1.2.1-1,  for MPDCCH-PRB-set  is configured

-	by the higher layer parameter n1PUCCH-AN-r13, if configured; otherwise:

-	by the higher layer parameter n1PUCCH-AN-InfoList-r13 for the corresponding CE level,

for MPDCCH-PRB-set  is given in Subclause 6.8A.1 in [3] where the same  value is used for each subframe containing a repeat of a MPDCCH transmission,  is determined from the antenna port used for localized MPDCCH transmission which is described in Subclause 6.8A.5 in [3]. When an MPDCCH-PRB-set is 2+4, following procedures is applied.

-	if the last detected MPDCCH is located within 2 PRB set, is obtained by above procedure.

-	if the last detected MPDCCH is located within 4 PRB set, is the sum between and the value obtained by above procedure.

-	if the last detected MPDCCH is MPDCCH format 5, is obtained by the above procedure with  = 0.

<Unchanged parts are omitted>