3GPP TSG RAN WG Meeting #86   	RP-192672

Sitges, Spain, December 9-12, 2019

Agenda Item: 10.1.1

Source: MediaTek Inc.

Title: Rel-17 16QAM support for NB-IoT

Document for: Discussion and Decision

# Introduction

NB-IoT is in many markets replacing GSM/GPRS for IoT usage. Requirements are diverse and while NB-IoT meet some part of requirements well (e.g. for stationary meters), others need enhancement, in particular:

- Tracker application reporting at high rate, e.g. once/s, once/2s.

- File download

- Media applications, e.g. kids toy with audio, smart home VAD (Voice Assistant Device)

NB-IoT often provides worse real time characteristics than GSM GPRS. For interactive embedded UEs this is problematic. For example, sales or payment terminal (a high volume application).  Examples of IoT application that demand higher NB-IoT data rates are given in Table 1. Examples of IoT application that demand lower NB-IoT latency are given in Table 2.

| Applications | Description | Data Rates |
| --- | --- | --- |
| FOTA | Software upgrade | 250 kbps~300 kbps |
| Health/fitness | Heartbeat monitoring | 192 kbps~256 kbps |
| Smart home VAD (Voice Assistant Device) | Voice command for remote control | 200 kbps |

Table 1: IoT applications that demand higher NB-IoT data rate

| Applications | Latency Requirement |
| --- | --- |
| POS (point of sale) Machine | 500 ms ~ few sec |
| Smart Grid Demand Response | 500 ms |
| Smart Door (lock/unlock) | 600 ms |

Table 2: IoT applications that demand lower NB-IoT latency

Support of 16QAM in NB-IoT will likely result in a new UE category Cat NB3 and that fits evolution of NB-IoT product to extend market opportunities. It is expected that the additional hardware complexity will not add significantly to the cost of the NB-IoT UE, and that Cat NB3 could in time be absorbed into the hardware evolution of all vendors.

In this contribution it is proposed to increase the NB-IoT device data rates by use of 16QAM modulation to 248.4 kbps from 126.8 kbps for DL and to 310.5 kbps from 158.5 kbps for UL. The following mechanisms are proposed to be introduced

- New MCS Table

- EVM requirements for 16QAM

# Analysis

The theoretical peak data rate for Rel-14 2 HARQ with TBS 2536 bits for non-anchor carrier in standalone mode for the DL and UL peak data rate is summarized in Table 3.

| R14 stand alone, non-anchor carrier | DL | UL |
| --- | --- | --- |
| Peak data rate | 126.8 kbps | 158.5 kbps |

Table 3: R14 peak data rate for non-anchor carrier in standalone mode

- DL peak data rate analysis for 16QAM

To support 16QAM, the TBS tables for NPDSCH and NPUSCH are extended. Table 4 gives an example to specify the NPDSCH TBS extension by adding TBS index ITBS 14 to ITBS 21 (highlighted in yellow) for 16QAM and to have max DL TBS 4968 bits. Then, without changing the HARQ RTT and scheduling delay timing relationship specified in R14, the DL peak data rate can achieve 248.4 kbps as analysed in Table 5, which almost doubles the DL peak data rate as compared to R14. Note that Rmax=4 and G=2 are assumed, which are the same settings as the R14 peak data rate analysis.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Table 4: Transport block size (TBS) table to support 16QAM for NPDSCH

Table 5: DL peak data rate for 16QAM with max TBS 4968 bits

- UL data rate analysis for 16QAM

Similarly, to extend the NPUSCH TBS by adding TBS index ITBS 14 to ITBS 21 (highlighted in green) to support 16QAM and to have max UL TBS 4968 bits as illustrated in Table 6, the UL peak data rate can achieve 310.5 kbps, without changing the HARQ RTT and scheduling delay timing relationship specified in R14. The peak data rate analysis is given in Table 7.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Table 6: Transport block size (TBS) table to support 16QAM for NPUSCH

Table 7: UL peak data rate for 16QAM with max TBS 4968 bits

In TS 36.101 section 6.5.2F.1, it is stated that the RMS average of the basic EVM measurements for 240/LCtone slots excluding any transient period for the average EVM case, where LCtone = {1, 3, 6, 12} is the number of subcarriers for the category NB1 and NB2 transmission, for the different modulations schemes shall not exceed the values specified in Table 6.5.2.1.1-1 for the parameters defined in Table 6.5.2.1.1-2. For EVM evaluation purposes, both NPRACH formats are considered to have the same EVM requirement as QPSK modulated. Hence, we propose that the EVM requirements for LTE 16QAM as specified in TS 36.101 in Table 6.5.2.1.1-1  can be used for NB-IoT 16QAM.

TS 36.101 Table 6.5.2.1.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8 | 8 |
| 256 QAM | % | 3.5 | 3.5 |

TS 36.101 Table 6.5.2.1.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| UE Output Power for 256 QAM | dBm | -30 |
| Operating conditions |  | Normal conditions |

Proposal 1: Introduce support of higher order modulation 16QAM for PDSCH and PUSCH in NB-IoT by means of

- New TBS table in 36.213

- Not exceeding EVM requirements in TS 36.101 Table 6.5.2.1.1-1 and Table 6.5.2.1.1-2

# Conclusion

In this contribution, we discussed the benefits of higher order modulation 16QAM for IoT application typically requiring higher data rates and proposed the specification of mechanisms to support 16QAM in NB-IoT.

Proposal 1: Introduce support of higher order modulation 16QAM for PDSCH and PUSCH in NB-IoT by means of

- New TBS table in 36.213

- Not exceeding EVM requirements in TS 36.101 Table 6.5.2.1.1-1 and Table 6.5.2.1.1-2

# References

- RP-190337, WID revision: Additional enhancements for NB-IoT, Huawei, TSG RAN#83, March 2019